3GPP TSG RAN WG4 Meeting #96-e	R4-2011592

Online, 17th – 28th August 2020

Agenda item:	10.3.2

Source:	SoftBank Corp.

Title:	TP for TR 37.717-11-11: DC_42_n3

Document for:	Approval

1	Introduction

2B EN-DC of DC_42_n3 was approved in RAN#88 [1]. This TP is to capture the basic aspects for the EN-DC.

2	Band combination and Configurations

The band combination and configurations to be supported are listed in Table 2-1 and 2-2.

Table 2-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_42_n3 | 42 | n3 | DC_42_n3 |

Table 2-2: Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_42A_n3A | DC_42A_n3A | 42A | n3A |
| DC_42C_n3A | DC_42A_n3A DC_42C_n3A | 42C | n3A |

3.  Maximum Output Power

The maximum output power to be supported is in Table 3-1.

Table 3-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_42A_n3A | 23 | +2/-3 |
| DC_42C_n3A | 23 | +2/-3 |

4.  Spurious emission band UE co-existence for DC

The requirements of spurious emission band UE co-existence for band 42 and n3 are captured from 36.101 and 38.101-1, respectively.

Table 4-1: Requirements of spurious emission band UE co-existence for band 42

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| 42 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 11, 18, 19, 20, 21, 25, 26, 27, 28, 31, 32, 33, 34, 38, 40, 41, 44, 45, 50, 51, 65, 66, 67, 68, 69, 72, 73, 74, 75, 76, 87, 88 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. |  |  |  |  |  |  |  |

Table 4-2: Requirements of spurious emission band UE co-existence for band n3

| NR Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n3, n80 | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 39, 40, 41, 43, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73,74, 75, 76. NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 42, 52, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

Then proposal to be captured in the TR is as below:

Table 4-3: Protection table for EN-DC

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_42_n3 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 40, 41, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

5. Delta Tib/Rib

From delta Tib/Rib of LTE CA_3-42, the Delta Tib/Rib proposals for DC_42_n3 are as follows.

Table 5-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA and NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_42_n3 | 42 | 0.8 |
|  | n3 | 0.6 |

Table 5-2: ΔRIB,c

| Inter-band EN-DC configuration | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_42_n3 | 42 | 0.5 |
|  | n3 | 0.2 |

6.  MSD analysis for DC

The frequencies of harmonics and IMD products are captured in Table 6-1.

Table 6-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 3400 | 3600 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 6800 | 7200 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 10200 | 10800 | 5130 | 5355 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 13600 | 14400 | 6840 | 7140 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 17000 | 18000 | 8550 | 8925 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 20400 | 21600 | 10260 | 10710 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 23800 | 25200 | 11970 | 12495 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1615 | 1890 | 5110 | 5385 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 5015 | 5490 | 180 | 170 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8510 | 8985 | 6820 | 7170 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 8415 | 9090 | 1530 | 1955 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3230 | 3780 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 11910 | 12585 | 8530 | 8955 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10220 | 10770 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 3740 | 3240 | 12690 | 11815 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1445 | 2070 | 7380 | 6630 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10240 | 10740 | 15310 | 16185 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11930 | 12555 | 13620 | 14370 |

Based on Table 5-1, it can be seen that

- 2nd order harmonic may fall into Rx frequencies of band 22, 42, 48, 49, n77, n78.

- 3rd order harmonic may fall into Rx frequencies of band 46.

- 2nd order IMD products may fall into Rx frequencies of bands 3, 9, 35, 39, 46.

- 3rd order IMD products may fall into Rx frequencies of bands 46.

- 4th order IMD products may fall into Rx frequencies of bands 2, 3, 9, 22, 24, 25, 33, 35, 36, 37, 39, 42, 43, 48, 49, 52, n77, n78.

- 5th order IMD products may fall into Rx frequencies of bands 2, 3, 9, 11, 21, 22, 24, 25, 32, 33, 34, 35, 36, 37, 39, 42, 43, 45, 48, 49, 50, 52, 70, 74, 75, n77, n78, n92, n94.

Based on above,

- the 2nd order harmonic of Band n3 will fall into Band 42.

- the 2nd, 4th and 5th order IMD may fall into Rx frequencies of band n3.

- the 4th and 5th or der IMD may fall into Rx frequencies of band n42.

Regarding the MSD due to harmonics of Band n3 fall into Band 42, the exceptions are needed. Based on CA_3-42, reference sensitivity exceptions for DC_42_n3 due to UL harmonic are proposed in Table 6-2 with uplink configuration specified in Table 6-3.

Table 6-2: MSD due to UL harmonic for EN-DC in NR FR1

| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n3 | 422, 13 | 27.3 | 24.3 | 22.5 | 21.3 |  |  |  |  |  |  |  |  |
|  | 423 | 1.9 | 1.3 | 1.0 | 0.5 |  |  |  |  |  |  |  |  |
| NOTE 2: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 13: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the EN-DC band combination: ∆FHD = 10 MHz for DC_1_n77, DC_2_n48, DC_2_n77, DC_48_n66, DC_66_n48, DC_66_n77, DC_3_n77, DC_3_n78, DC_11_n28, DC_28_n50 and DC_42_n3 |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6-3: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

|  | E-UTRA or NR Band / Channel bandwidth of the high band |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n3 | 42 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |  |

Regarding the MSD due to 2nd and 4th order IMD, the exceptions are needed. The reference sensitivity exceptions for DC_42_n3 due to IMD are proposed in Table 6-4 based on those of DC_3_n78.

Table 6-4: Reference sensitivity exceptions for PCell due to dual uplink operation for EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_42_n3 | 42 | 3575 | 10 | 50 | 3575 | N/A | TDD | N/A |
|  | n3 | 1740 | 5 | 25 | 1835 | 26 | FDD | 2nd3 |
|  |  |  |  |  |  | 28.74 |  |  |
|  | 42 | 3435 | 10 | 50 | 3435 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 8.0 | FDD | 4th3 |
|  |  |  |  |  |  | 10.74 |  |  |
| NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. |  |  |  |  |  |  |  |  |

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6-5 lists up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6-5: Band 42 and Band n3 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | 4th IMD |
| Galileo | 1559 | - | 1591 | Yes |  | 4th IMD |
| GLONASS | 1591 | - | 1610 | Yes |  | 4th IMD |
| GPS | 1563 | - | 1587 | Yes |  | 4th IMD |
| ISM band (2.4GHz) | 2400 | - | 2483.5 |  | US/Europe |  |
|  | 2400 | - | 2494 |  | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 3rd Harmonic 2nd IMD |
|  | 5150 | - | 5350 | Yes | Europe | 3rd Harmonic 2nd IMD |
|  | 5470 | - | 5725 | Yes |  | 3rd IMD |
|  | 5150 | - | 5825 | Yes | Asia | 3rd Harmonic 2nd IMD |
| 45GHz Unlicensed Bands | 42300 | - | 47000 |  | China |  |
|  | 47200 | - | 48400 |  | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 |  | Europe |  |
|  | 57050 | - | 64000 |  | USA Canada |  |
|  | 57000 | - | 64000 |  | South Korea |  |
|  | 59000 | - | 66000 |  | Japan |  |
|  | 59000 | - | 64000 |  | China |  |
|  | 59400 | - | 62900 |  | Australia |  |

7.  Self-interference analysis

The result of self-interference analysis is captured in Table 7-1.

Table 7-1: Band 42 and Band n3 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_42A_n3A | 42 | a | 0 | b | -1 | 2 | 42 | Harmonic |
|  | n3 | c | 2 | d | 0 |  |  |  |
| DC_42A_n3A | 42 | a | 1 | b | 0 | 2 | n3 | IMD |
|  | n3 | c | -1 | d | -1 |  |  |  |
| DC_42A_n3A | 42 | a | -1 | b | 0 | 4 | n3 | IMD |
|  | n3 | c | 3 | d | -1 |  |  |  |
| DC_42A_n3A | 42 | a | -2 | b | 0 | 5 | n3 | IMD |
|  | n3 | c | 3 | d | 1 |  |  |  |

Equations (1) and (2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 7-1 and CBW stands for channel bandwidth.

(1)

(2)

Formula (3-1) and (3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(3-1)

(3-2)

8.  Conclusion

This paper proposes study results of DC_42_n3.

TP is shown in Annex.

9.  Reference

[1]  RP-201368  New WID on Dual Connectivity (DC) of 1 band LTE (1DL/1UL) and 1 NR band (1DL/1UL), CHTTL

Annex.   Proposed text for TR37.717-11-11:

[Unchanged Parts Skipped]

## 6.1.x			DC_42_n3

### 6.1.x.1		Operating bands for DC

Table 6.1.x.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_42_n3 | 42 | n3 | DC_42_n3 |

### 6.1.x.2		Configuration for DC

Table 6.1.x.2-1: Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_42A_n3A | DC_42A_n3A | 42A | n3A |
| DC_42C_n3A | DC_42A_n3A DC_42C_n3A | 42C | n3A |

### 6.1.x.3		Co-existence studies

Table 6.1.x.3-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_42A_n3A DC_42C_n3A | 23 | +2/-3 |

### 6.1.x.4 		Spurious emission band UE co-existence for DC

Table 6.1.x.4-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_42_n3 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 40, 41, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

### 6.1.x.5		MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.x.5-1

Table 6.1.x.5 -1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 3400 | 3600 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 6800 | 7200 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 10200 | 10800 | 5130 | 5355 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 13600 | 14400 | 6840 | 7140 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 17000 | 18000 | 8550 | 8925 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 20400 | 21600 | 10260 | 10710 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 23800 | 25200 | 11970 | 12495 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1615 | 1890 | 5110 | 5385 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 5015 | 5490 | 180 | 170 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8510 | 8985 | 6820 | 7170 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 8415 | 9090 | 1530 | 1955 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3230 | 3780 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 11910 | 12585 | 8530 | 8955 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10220 | 10770 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 3740 | 3240 | 12690 | 11815 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1445 | 2070 | 7380 | 6630 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10240 | 10740 | 15310 | 16185 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11930 | 12555 | 13620 | 14370 |

Based on Table 6.1.x.5 -1, it can be seen that

- 2nd order harmonic may fall into Rx frequencies of band 22, 42, 48, 49, n77, n78.

- 3rd order harmonic may fall into Rx frequencies of band 46.

- 2nd order IMD products may fall into Rx frequencies of bands 3, 9, 35, 39, 46.

- 3rd order IMD products may fall into Rx frequencies of bands 46.

- 4th order IMD products may fall into Rx frequencies of bands 2, 3, 9, 22, 24, 25, 33, 35, 36, 37, 39, 42, 43, 48, 49, 52, n77, n78.

- 5th order IMD products may fall into Rx frequencies of bands 2, 3, 9, 11, 21, 22, 24, 25, 32, 33, 34, 35, 36, 37, 39, 42, 43, 45, 48, 49, 50, 52, 70, 74, 75, n77, n78, n92, n94.

Based on above,

- the 2nd order harmonic of Band n3 will fall into Band 42.

- the 2nd, 4th and 5th order IMD may fall into Rx frequencies of band n3.

- the 4th and 5th or der IMD may fall into Rx frequencies of band n42.

Regarding the MSD due to harmonics of Band n3 fall into Band 42, the exceptions are needed. Based on CA_3-42, reference sensitivity exceptions for DC_42_n3 due to UL harmonic are proposed in Table 6-1.x.5-2 with uplink configuration specified in Table 6-1.x.5 -3.

Table 6-1.x.5-2: MSD due to UL harmonic for EN-DC in NR FR1

| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n3 | 422, 13 | 27.3 | 24.3 | 22.5 | 21.3 |  |  |  |  |  |  |  |  |
|  | 423 | 1.9 | 1.3 | 1.0 | 0.5 |  |  |  |  |  |  |  |  |
| NOTE 2: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 13: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the EN-DC band combination: ∆FHD = 10 MHz for DC_1_n77, DC_2_n48, DC_2_n77, DC_48_n66, DC_66_n48, DC_66_n77, DC_3_n77, DC_3_n78, DC_11_n28, DC_28_n50 and DC_42_n3 |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6-1.x.5-3: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

|  | E-UTRA or NR Band / Channel bandwidth of the high band |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n3 | 42 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |  |

Regarding the MSD due to 2nd and 4th order IMD, the exceptions are needed. The reference sensitivity exceptions for DC_42_n3 due to IMD are proposed in Table 6-1.x.5-4.

Table 6-1.x.5-4: Reference sensitivity exceptions for PCell due to dual uplink operation for EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_42_n3 | 42 | 3575 | 10 | 50 | 3575 | N/A | TDD | N/A |
|  | n3 | 1740 | 5 | 25 | 1835 | 26 | FDD | 2nd3 |
|  |  |  |  |  |  | 28.74 |  |  |
|  | 42 | 3435 | 10 | 50 | 3435 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 8.0 | FDD | 4th3 |
|  |  |  |  |  |  | 10.74 |  |  |
| NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. |  |  |  |  |  |  |  |  |

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.1.x.5-5 lists up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.1.x.5-5: Band 42 and Band n3 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | 4th IMD |
| Galileo | 1559 | - | 1591 | Yes |  | 4th IMD |
| GLONASS | 1591 | - | 1610 | Yes |  | 4th IMD |
| GPS | 1563 | - | 1587 | Yes |  | 4th IMD |
| ISM band (2.4GHz) | 2400 | - | 2483.5 |  | US/Europe |  |
|  | 2400 | - | 2494 |  | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 3rd Harmonic 2nd IMD |
|  | 5150 | - | 5350 | Yes | Europe | 3rd Harmonic 2nd IMD |
|  | 5470 | - | 5725 | Yes |  | 3rd IMD |
|  | 5150 | - | 5825 | Yes | Asia | 3rd Harmonic 2nd IMD |
| 45GHz Unlicensed Bands | 42300 | - | 47000 |  | China |  |
|  | 47200 | - | 48400 |  | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 |  | Europe |  |
|  | 57050 | - | 64000 |  | USA Canada |  |
|  | 57000 | - | 64000 |  | South Korea |  |
|  | 59000 | - | 66000 |  | Japan |  |
|  | 59000 | - | 64000 |  | China |  |
|  | 59400 | - | 62900 |  | Australia |  |

6.1.x.6	∆TIB and ∆RIB values

For DC_42_n3, the TIB,c and RIB,c values are given in the tables below.

Table 6.x.1.6-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA and NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_42_n3 | 42 | 0.8 |
|  | n3 | 0.6 |

Table 6.x.1.6-2: ΔRIB,c

| Inter-band EN-DC configuration | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_42_n3 | 42 | 0.5 |
|  | n3 | 0.2 |

6.1.x.7	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.1.x.5 except for harmonic mixing and UL harmonic order higher than 5 which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.1.x.7-1.

Table 6.1.x.7-1: Band 42 and Band n3 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_42A_n3A | 42 | a | 0 | b | -1 | 2 | 42 | Harmonic |
|  | n3 | c | 2 | d | 0 |  |  |  |
| DC_42A_n3A | 42 | a | 1 | b | 0 | 2 | n3 | IMD |
|  | n3 | c | -1 | d | -1 |  |  |  |
| DC_42A_n3A | 42 | a | -1 | b | 0 | 4 | n3 | IMD |
|  | n3 | c | 3 | d | -1 |  |  |  |
| DC_42A_n3A | 42 | a | -2 | b | 0 | 5 | n3 | IMD |
|  | n3 | c | 3 | d | 1 |  |  |  |

Equations (1) and (2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.1.x.6-1 and CBW stands for channel bandwidth.

(1)

(2)

Formula (3-1) and (3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(3-1)

(3-2)

[Unchanged Parts Skipped]