3GPP TSG RAN WG4 Meeting #96-e	R4-2011593

Online, 17th – 28th August 2020

Agenda item:	10.4.2

Source:	SoftBank Corp.

Title:	TP for TR 37.717-21-11: EN-DC_3-11_n77

Document for:	Approval

1	Introduction

EN-DC of 2B LTE and 1B NR of DC_3-11_n77 was approved in RAN#88[1]. This TP is to capture the basic aspects for the EN-DC.

2	Combination proposed

Combinations proposed in [1] are as below.

T

Table 2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-11A_n77A | DC_3A_n77A DC_11A_n77A | CA_3A-11A | n77A |
| DC_3A-11A_n77(2A) | DC_3A_n77A DC_11A_n77A | CA_3A-11A | n77(2A) |

3.  Co-existence

When Uplink EN-DC configuration is DC_3A_n77A, IMD4 of (B3 - n77) will fall into Rx band of Band 11. When Uplink EN-DC configuration is DC_11A_n77A, (2) IMD2 and (3) IMD5 of (B11 - n77) will fall into Rx band of Band 3. But considering the current situation that Band 11 is operated only by a certain operators in Japan, the frequency ranges can be limited as follows.

-	Operator X: B11 (UL:1427.9-1437.9 / DL:1475.9-1485.9 MHz), B3 (UL:1750-1765 / DL:1845-1860 MHz), n77 (3400-3440 / 3560-3600 / 3900-4000 MHz)

-	Opeartor Y: B11 (UL:1437.9-1447.9 / DL:1485.9-1495.9 MHz), B3 (UL:1710-1730 / DL:1805-1825 MHz), n77 (3520-3560 / 3700-3800 / 4000-4100 MHz)

Then the own Rx impact can be simplified as below.

IMD4 of (B3-n77) will fall into Rx band of Band 11.

IMD5 of (B11-n77) will fall into Rx band of Band 3.

4.  Delta Tib/Rib

The delta Tib/Rib of subset EN-DC/CA are as follows. EN-DC values are picked.

Table 4-1: ΔTIB of constituent 2 band combo.

| ΔTIB | B3+B11 | B3+n77 | B11+n77 | Proposed Value |
| --- | --- | --- | --- | --- |
| B3 | 0.8 | 0.6 |  | 0.8 |
| B11 | 0.9 |  | 0.4 | 0.9 |
| n77 |  | 0.8 | 0.8 | 0.8 |

Table 4-2: ΔRIB of constituent 2 band combo.

| ΔRIB | B3+B11 | B3+n77 | B11+n77 | Proposed Value |
| --- | --- | --- | --- | --- |
| B3 | 0.3 | 0.2 |  | 0.3 |
| B11 | 0.5 |  | 0 | 0.5 |
| n77 |  | 0.5 | 0.5 | 0.5 |

Then the proposal for this 2LTE-1NR EN-DC is:

Table 4-3: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_3-11_n77 | 3 | 0.8 |
|  | 11 | 0.9 |
|  | n77 | 0.8 |

Table 4-4: ΔRIB,c

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3-11_n77 | 3 | 0.3 |
|  | 11 | 0.5 |
|  | n77 | 0.5 |

5.  REFSENS

As mentioned in section 3, REFSENS exceptions due to IMD4 and IMD 5 are expected. Based on the exceptions of DC_3-21_n77, the following values are proposed.

Table 5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_3A-11A_n77A DC_3A-11A_n77(2A) | 3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n77 | 3675 | 10 | 50 | 3675 | N/A | TDD | N/A |
|  | 11 | 1443 | 5 | 25 | 1491 | 8.8 | FDD | IMD4 |
|  | 11 | 1435.4 | 5 | 25 | 1483.4 | N/A | FDD | N/A |
|  | n77 | 3905 | 10 | 50 | 3905 | N/A | TDD | N/A |
|  | 3 | 1753 | 5 | 25 | 1848 | 3.4 | FDD | IMD5Y |
| Note Y: This band is also subject to IMD2 which is not specified. The frequency range below 3400MHz in n77 is not used for this combination. |  |  |  |  |  |  |  |  |

6.  Conclusion

This paper proposes study results of EN-DC 3-11_n77.

TP is shown in Annex.

7.  Reference

[1]  RP-201369  Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL), Huawei

Annex.   Proposed text for TR37.717-21-11:

[Unchanged Parts Skipped]

5.x	DC_3-11_n77

### 5.x.1	Configurations for DC_3-11_n77

Table 5.x.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-11A_n77A | DC_3A_n77A DC_11A_n77A | CA_3A-11A | n77A |
| DC_3A-11A_n77(2A) | DC_3A_n77A DC_11A_n77A | CA_3A-11A | n77(2A) |

### 5.x.2	Co-existence studies

When Uplink EN-DC configuration is DC_3A_n77A, (1) IMD4 of (B3 - n77) will fall into Rx band of Band 11. When Uplink EN-DC configuration is DC_11A_n77A, (2) IMD2 and (3) IMD5 of (B11 - n77) will fall into Rx band of Band 3.

But considering the current situation that Band 11 is operated only by a certain operators in Japan, the frequency ranges can be limited as follows.

-	Operator X: B11 (UL:1427.9-1437.9 / DL:1475.9-1485.9 MHz), B3 (UL:1750-1765 / DL:1845-1860 MHz), n77 (3400-3440 / 3560-3600 / 3900-4000 MHz)

-	Opeartor Y: B11 (UL:1437.9-1447.9 / DL:1485.9-1495.9 MHz), B3 (UL:1710-1730 / DL:1805-1825 MHz), n77 (3520-3560 / 3700-3800 / 4000-4100 MHz)

Then the own Rx impact can be simplified as below.

- IMD4 of (B3-n77) will fall into Rx band of Band 11.

- IMD5 of (B11-n77) will fall into Rx band of Band 3.

### 5.x.3	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.x.3-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_3-11_n77 | 3 | 0.8 |
|  | 11 | 0.9 |
|  | n77 | 0.8 |

Table 5.x.3-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3-11_n77 | 3 | 0.3 |
|  | 11 | 0.5 |
|  | n77 | 0.5 |

### 5.x.4	Reference sensitivity exceptions

As mentioned above, IMD4 of B3 and n77 to Band11 Rx and IMD5 of B11 and n77 to Band3 Rx need to be addressed for REFSENS relaxation. Based on the exceptions of DC_3-21_n77, the following values are proposed:

Table 5.x.4-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_3A-11A_n77A DC_3A-11A_n77(2A) | 3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n77 | 3675 | 10 | 50 | 3675 | N/A | TDD | N/A |
|  | 11 | 1443 | 5 | 25 | 1491 | 8.8 | FDD | IMD4 |
|  | 11 | 1435.4 | 5 | 25 | 1483.4 | N/A | FDD | N/A |
|  | n77 | 3905 | 10 | 50 | 3905 | N/A | TDD | N/A |
|  | 3 | 1753 | 5 | 25 | 1848 | 3.4 | FDD | IMD5Y |
| Note Y: This band is also subject to IMD2 which is not specified. The frequency range below 3400MHz in n77 is not used for this combination. |  |  |  |  |  |  |  |  |

[Unchanged Parts Skipped]