3GPP TSG-RAN WG4 Meeting #96-e				  											R4-2011663 Online, 17th Aug. 2020 – 28th Aug. 2020

Source: 	Nokia, T-mobile

Title: 		TP for 37.717-11-21 to introduce DC_48A_n48A-n66A

Agenda Item: 			10.7.2 [DC_R17_xBLTE_2BNR_yDL2UL-Core]

Document for:	Approval

# 1	Introduction

This contribution is a TP for TR 37.717-11-21 to introduce DC_48A_n48A-n66A.

# 2	Text Proposal

*********************** Start of the TP ***************************************************

## 6.x	DC_48_n48-n66

### 6.x.1	Operating bands for DC

Table 6.x.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_48_n48-n66 | 48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |
|  | n48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 6.x.2	Configuration for DC

Table 6.X.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_48A_n48A-n66A | DC_48A_n66A | 48A | CA_n48A-n66A |

### 6.x.3	Co-existence studies

Co-existence analysis for DC_48_n66 UL shows that 2nd HAM of band 66 may fall into the lowest 10MHz of NR Band n48 DL. This is already covered in Table 7.3B.2.3.1-1 and Table 7.3B.2.3.1-2 in 38.101-3. Further 4th order IMD might fall in 48 DL However, since 48 and n48 will be operated as synchronous in same UL/DL configuration no IMD issue.

### 6.x.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 6.X.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_48_n48-n66 | 48 | 0.8 |
|  | n48 |  |
|  | n66 | 0.6 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_48_n48-n66 | 48 | 0.4 |
|  | n48 |  |
|  | n66 | 0.3 |

### 6.x.5	MSD

Based on co-existence studies no additional MSD is needed.

*********************** End of the TP ***************************************************

# 3	References