3GPP TSG-RAN4 Meeting #96-e	R4-2011699

# Electronic Meeting, 17th Aug 2020 - 28th Aug 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 5656 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | A-MPR definition for CA_48B |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R16_intra-Core |  |  |  |  |  | Date: |  |  | 2020-08-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | E-UTRA UL CA configuration CA_48B is already specified in clause 5 but A-MPR is missing. This CR is based on simulation results presented in R4-2006493. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A-MPR is introduced for CA_48B |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | E-UTRA UL CA configuration CA_48B needs to be removed from specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.521 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************************** Start of changes ******************************************

### 6.2.4A	UE maximum output power with additional requirements for CA

Additional ACLR, spectrum emission and spurious emission requirements for carrier aggregation can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the CA Power Class as specified in Table 6.2.2A-1.

If for intra-band carrier aggregation the UE is configured for transmissions on a single serving cell, then subclauses 6.2.3 and 6.2 4 apply with the Network Signaling value indicated by the field additionalSpectrumEmission.

For intra-band contiguous aggregation with the UE configured for transmissions on two serving cells, the maximum output power reduction specified in Table 6.2.4A-1 is allowed for all serving cells of the applicable uplink CA configurations according to the CA network signalling value indicated by the field additionalSpectrumEmissionSCell-r10. Then clause 6.2.3A does not apply, i.e. the carrier aggregation MPR = 0dB, unless the value indicated is CA_NS_09 or CA_NS_31. For uplink 64 QAM and 256 QAM, the applied maximum output power reduction is obtained by taking the maximum value of MPR requirements specified in Table 6.2.3A-1 and A-MPR requirements specified in Table 6.2.4A-1.

Table 6.2.4A-1: Additional Maximum Power Reduction (A-MPR) for intra-band contiguous CA

| CA Network Signalling value | Requirements (subclause) | Uplink CA Configuration | A-MPR [dB] (subclause) |
| --- | --- | --- | --- |
| CA_NS_01 | 6.6.3.3A.1 | CA_1C | 6.2.4A.1 |
| CA_NS_02 | 6.6.3.3A.2 | CA_1C | 6.2.4A.2 |
| CA_NS_03 | 6.6.3.3A.3 | CA_1C | 6.2.4A.3 |
| CA_NS_04 | 6.6.2.2A.1, 6.6.3.3A.8, 6.6.3.3A.9 | CA_41C, CA_41D | 6.2.4A.4 |
| CA_NS_05 | 6.6.3.3A.4 | CA_38C | 6.2.4A.5 |
| CA_NS_06 | 6.6.3.3A.5 | CA_7C | 6.2.4A.6 |
| CA_NS_07 | 6.6.3.3A.6 | CA_39C | 6.2.4A.7 |
| CA_NS_08 | 6.6.3.3A.7 | CA_42C | 6.2.4A.8 |
| CA_NS_09 | 6.6.2.2A.2 | CA_66B | N/A |
|  | 6.6.2.2A.3 | CA_66C | N/A |
| CA_NS_10 | 6.6.2.2A.4 6.6.3.3A.10 | CA_48B, CA_48C | 6.2.4A.10 |
| … |  |  |  |
| CA_NS_31 | NOTE 1 | Table 5.6A.1-1 (NOTE 1) | N/A |
| CA_NS_32 | Reserved |  |  |
| NOTE 1: Applicable for uplink CA configurations listed in Table 5.6A.1-1 for which none of the additional requirements in subclauses 6.6.2.2A or 6.6.3.3A apply. NOTE 2: The index of the sequence CA_NS corresponds to the value of additionalSpectrumEmissionSCell-r10. |  |  |  |

****************************** No changes ******************************************

#### 6.2.4A.10	A-MPR for CA_NS_10

If the UE is configured to CA_48C and it receives IE CA_NS_10 the allowed maximum output power reduction applied to transmission on the PCC and the SCC for contiguously aggregated signals is specified in Table 6.2.4A.10-2 or Table 6.2.4A.10-3. Which table is determined by the position of the carrier centre frequency in Table 6.2.4A.10-1.

Table 6.2.4A.10-1: A-MPR regions for CA_48C

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | A-MPR |
| --- | --- | --- |
| 20+5 / 5 + 20 | FL + BWCA/2 ≤ FC < FL + 3*BWCA/2 - 10 MHz FH - 3*BWCA/2 + 10 MHz ≤ FC ≤ FH - BWCA/2 | Table 6.2.4A.10-2 |
|  | FL + 3*BWCA/2 - 10 MHz ≤ FC < FH - 3*BWCA/2 + 10 MHz | Table 6.2.4A.10-3 |
| 20+10 / 10 + 20 | FL + BWCA/2 ≤ FC < FL + 3*BWCA/2 - 10 MHz FH - 3*BWCA/2 + 10 MHz ≤ FC < FH - BWCA/2 | Table 6.2.4A.10-2 |
|  | FL + 3*BWCA/2 - 10 MHz ≤ FC < FH - 3*BWCA/2 + 10 MHz | Table 6.2.4A.10-3 |
| 20+15 / 15 + 20 | FL + BWCA/2 ≤ FC< FL + 3*BWCA/2 - 10 MHz FH - 3*BWCA/2 + 10 MHz ≤ FC < FH - BWCA/2 | Table 6.2.4A.10-2 |
|  | FL + 3*BWCA/2 - 10 MHz ≤ FC < FH - 3*BWCA/2 + 10 MHz | Table 6.2.4A.10-3 |
| 20+20 | FL + BWCA/2 ≤ FC < FL + 3*BWCA/2 - 10 MHz FH - 3*BWCA/2 + 10 MHz ≤ FC < FH - BWCA/2 | Table 6.2.4A.10-2 |
|  | FL + 3*BWCA/2 - 10 MHz ≤ FC < FH - 3*BWCA/2 + 10 MHz | Table 6.2.4A.10-3 |
| NOTE: FL = 3550 MHz, FH = 3700 MHz and BWCA is the combined bandwidth of the contiguous CCs in the CA combination indicated. |  |  |

Table 6.2.4A.10-2: A-MPR regions for CA_48C at the band edge

| BWs [MHz] | RB_start | L_CRB | A-MPR [dB] |
| --- | --- | --- | --- |
|  |  |  | max(mods) |
| 20+5 / 5 + 20 | 0 – 7 and 117 - 124 |  | ≤ 12 |
|  | 5 - 25 | <85 | ≤ 5,5 |
|  |  | >85 | ≤ 6,5 |
|  | 26 - 120 |  | ≤ 5 |
| 20+10 / 10 + 20 | 0 - 13 and 135 - 149 |  | ≤ 11 |
|  | 14 - 33 | <85 | ≤ 5 |
|  |  | >85 | ≤ 7 |
|  | 34 - 134 |  | ≤ 6 |
| 20+15 / 15 + 20 | 0 - 22 and 152 -174 |  | ≤ 11 |
|  | 23 - 42 | <95 | ≤ 5,5 |
|  |  | >95 | ≤ 7 |
|  | 43 - 151 |  | ≤ 6 |
| 20+20 | 0 - 31 and 165 - 199 |  | ≤ 11 |
|  | 31 - 51 | <100 | ≤ 5,5 |
|  |  | >100 | ≤ 7 |
|  | 52 - 164 | <100 | ≤ 4,5 |
|  |  | >100 | ≤ 6 |

Table 6.2.4A.10-3: A-MPR regions for CA_48C at the band center (“range for lower A-MPR”)

| BWs [MHz] | RB_start | L_CRB | A-MPR [dB] |
| --- | --- | --- | --- |
|  |  |  | max(mods) |
| 20+5 / 5 + 20 | 0 – 7 and 117 - 124 |  | ≤ 4 |
|  | 5 - 25 | <85 | ≤ 2 |
|  |  | >85 | ≤ 2,5 |
|  | 26 - 120 |  | ≤ 2 |
| 20+10 / 10 + 20 | 0 - 13 and 135 - 149 |  | ≤ 4,5 |
|  | 14 - 33 | <85 | ≤ 1,5 |
|  |  | >85 | ≤ 2,5 |
|  | 34 - 134 |  | ≤ 1,5 |
| 20+15 / 15 + 20 | 0 - 22 and 152 -174 |  | ≤ 4,5 |
|  | 23 - 42 | <95 | ≤ 1 |
|  |  | >95 | ≤ 2,5 |
|  | 43 - 151 |  | ≤ 1 |
| 20+20 | 0 - 31 and 165 - 199 |  | ≤ 4,5 |
|  | 31 - 51 | <100 | ≤ 1 |
|  |  | >100 | ≤ 1,5 |
|  | 52 - 164 | <100 | ≤ 1 |
|  |  | >100 | ≤ 1 |

If the UE is configured to CA_48C and it receives IE CA_NS_10 the allowed maximum output power reduction applied to transmissions on the PCell and the SCell with non-contiguous resource allocation is defined with both an edge and a center scenario and is determined in Table 6.2.4A.10-4.

Table 6.2.4A.10-4: A-MPR regions for CA_48C

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | A-MPR |
| --- | --- | --- |
| 20+5 / 5 + 20 | FL + BWCA/2 ≤ FC < FL + 3*BWCA/2 - 10 MHz FH - 3*BWCA/2 + 10 MHz ≤ FC ≤ FH - BWCA/2 | Edge |
|  | FL + 3*BWCA/2 - 10 MHz ≤ FC < FH - 3*BWCA/2 + 10 MHz | Center |
| 20+10 / 10 + 20 | FL + BWCA/2 ≤ FC < FL + 3*BWCA/2 - 10 MHz FH - 3*BWCA/2 + 10 MHz ≤ FC < FH - BWCA/2 | Edge |
|  | FL + 3*BWCA/2 - 10 MHz ≤ FC < FH - 3*BWCA/2 + 10 MHz | Center |
| 20+15 / 15 + 20 | FL + BWCA/2 ≤ FC< FL + 3*BWCA/2 - 10 MHz FH - 3*BWCA/2 + 10 MHz ≤ FC < FH - BWCA/2 | Edge |
|  | FL + 3*BWCA/2 - 10 MHz ≤ FC < FH - 3*BWCA/2 + 10 MHz | Center |
| 20+20 | FL + BWCA/2 ≤ FC < FL + 3*BWCA/2 - 10 MHz FH - 3*BWCA/2 + 10 MHz ≤ FC < FH - BWCA/2 | Edge |
|  | FL + 3*BWCA/2 - 10 MHz ≤ FC < FH - 3*BWCA/2 + 10 MHz | Center |
| NOTE: FL = 3550 MHz, FH = 3700 MHz and BWCA is the combined bandwidth of the contiguous CCs in the CA combination indicated. |  |  |

The Edge scenario is defined as follows

A-MPR = CEIL {MA, 0.5}

where MA is defined as follows

MA =      18.00 - 10.00 A; 			0 ≤ A < 0.05

18.50 - 20.00 A;			0.05 ≤ A < 0.2

15.50 - 5.00 A;			0.2 ≤ A < 1

where A = NRB_alloc / NRB_agg.

The Center scenario is defined as follows

A-MPR = CEIL {MA, 0.5}

where MA is defined as follows

MA =      11.50 - 10.00 A; 			0 ≤ A < 0.15

10.88 - 5.88 A;			0.15 ≤ A < 1

where A = NRB_alloc / NRB_agg

For CA_48B contiguous resource allocation when 3560 MHz ≤ Fagg_alloc_low ≤ 3680 MHz

if allocation is inner 1 then A-MPR = 0 dB where inner 1 is defined as

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB_agg – RBStart,Low – LCRB

with following conditions

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB  ≤  ceil(NRB_agg /2)

Inner 1 region exceptions thresholds are

RBstart < 12 and RBend ≥ 92 for BWChannel_CA = 20MHz, and

RBstart < 6 and RBend ≥ 71 for BWChannel_CA = 15MHz,

For which AMPR = 4 dB.

else A-MPR= 4 dB

For CA_48B contiguous resource allocation when Fagg_alloc_low < 3560 MHz

if allocation is inner 3 then A-MPR = 0 dB

Inner 3 region exceptions thresholds are

RBstart < 20 for BWChannel_CA = 20MHz, and

RBstart < 10 for BWChannel_CA = 15MHz,

For which AMPR = 7dB.

where inner 3 is defined as

RBStart = NRB_agg /4

LCRB = NRB_agg/4

RBStart = NRB_agg 3/4  LCRB

with following conditions

NRB_agg /4 < RBStart < NRB_agg 3/4  LCRB  AND LCRB < NRB_agg/4

else A-MPR = 7 dB.

For CA_48B contiguous resource allocation when Fagg_alloc_high > 3690 MHz

if allocation is inner 3 then A-MPR = 0 dB

Inner 3 region exceptions thresholds are

RBstart > 70 for BWChannel_CA = 20MHz, and

RBstart > 62 for BWChannel_CA = 15MHz,

For which AMPR = 7dB.

where inner 3 is defined as

RBStart = NRB_agg /4

LCRB = NRB_agg/4

RBStart = NRB_agg 3/4  LCRB

with following conditions

NRB_agg /4 < RBStart < NRB_agg 3/4  LCRB  AND LCRB < NRB_agg/4

else A-MPR = 7 dB.

For CA_48B non-contiguous resource allocation when 3560 MHz ≤ Fagg_alloc_low ≤ 3680 MHz

A = NRB_alloc / NRB_agg

A-MPR= 			13.00;         			0.00 <= A <= 0.08

13.78 - 9.78 A; 		0.08 < A <= 1.00

For CA_48B non-contiguous resource allocation when Fagg_alloc_low < 3560 MHz or Fagg_alloc_high > 3690 MHz

A-MPR= 		13.00;     			0.00 <= A <= 0.08

14.13  -14.06 A; 		0.08 < A <= 0.40

9.17 – 1.67 A;		0.40 < A <= 1.00

****************************** End of changes ******************************************