TSG-RAN Working Group 4 (Radio) meeting #96-E	R4-2011832

Electronic Meeting, August 17th – 28th 2020

Source:	Ericsson

Title:	WF on adding B24 for M1/M2 and NB1/NB2

Agenda item:	14.7

Document for:	Approval

# Introduction

At RAN4#96-E meeting a way-forward for adding B24 for M1/M2 and NB1/NB2was created based on the discussion [1].

# Way-Forward

## new FCC requirement for A-MPR simulation

The alignment with new WID (modification of B24) for setting the new requirements, the new requirements are summarized in Table 1 below:

Table 1: Additional requirement

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) |  | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- |
|  | 5MHz | 10MHz |  |  |
| 1541 ≤ f ≤ 1559 | -102 |  | 2 kHz | EIRP of discrete emissions of less than 700 Hz bandwidth, averaged over any 2 millisecond active transmission interval |
| 1559≤ f ≤ 1608 |  | -85 |  |  | 700Hz |  |  |  |
| 1608≤ f ≤ 1610 |  | -85 +5/2 (f-1608) |  |  | 700Hz |  |  |  |
| 1610≤ f ≤ 1625 |  | -80+ 66/15 (f-1610) |  |  | 700Hz |  |  |  |
| 1541 ≤ f ≤ 1608 |  | -75 |  |  | 1MHz |  | EIRP and averaged over any 2 millisecond active transmission interval |  |
| 1608≤ f ≤ 1610 |  | -75 + 5/2 (f-1608) |  |  | 1MHz |  |  |  |
| 1610≤ f ≤ 1627.5 |  | -70+ 5 7/17.5 (f-1610) |  |  | 1MHz |  |  |  |
| 1638.5 ≤f ≤ 1645.5 |  | -28 |  |  | 4kHz |  |  |  |
| 1657.5 ≤f ≤ 1660.5 |  | -28 |  |  | 4kHz |  |  |  |

## CAT-M1/M2 A-MPR simulation assumptions

  - PA calibrated in the same way MPR/A-MPR derived for CAT-M1/M2 device together with other parameter setting:

  - Counter-IM3 : 60 dB,

  - IQ image : -25 dB

  - LO leakage of -25 dBc for > 1GHz band and -28 dBc for  <= 1 GHz band

  - Channel BW= 5MHz and BW= 10MHz,

  - Uplink frequency :1627.5MHz – 1637.5 MHz and 1646.5 – 1656.5 MHz

  - PA model:

    - Option 1:  conventional PA model used in LTE UE A-MPR/MPR simulation

  - PA calibrated in the same way MPR/A-MPR derived for CAT-M1/M2 device together with other parameter setting:

      - Counter-IM3 : 60 dB,

      - IQ image : -25 dB

      - LO leakage of -25 dBc for > 1GHz band and -28 dBc for  <= 1 GHz band

    - Option 2: Finalize PA/filter assumptions at the next meeting to  align with new WID[2]

## NB1/NB2 handling on new FCC requirements

New NS signalling notify the B24 NB device without agreed A-MPR table. No need on A-MPR simulation.

# References

- R4-2011575, Email discussion summary for [96e][142] LTE_bands_R17_M1_M2_NB1_NB2, Ericsson

- RP-201356, New WID proposal: Modification of LTE Band 24 specifications to comply with updated regulatory emission limits, Ligado Networks