3GPP TSG-RAN WG4 Meeting # 96-e 												R4-2011857

Electronic Meeting, Aug., 2020

Agenda item:			7.12.1.3, 7.12.1.5, 7.12.1.8

Source:	Moderator (Qualcomm Incorporated)

Title:	Email discussion summary for [96e][117] NR_RF_FR2_req_enh_Part_3

Document for:	Information

# Introduction

This document is intended to capture companies’ views on contributions in the following agenda items.

7.12.1.3	Intra-band non-cont DL CA for aggregated BW larger than 1400 MHz

7.12.1.5	Inter-band DL CA

7.12.1.8	FR2 Beam squint

List of candidate targets for first round of discussion in agenda 7.12.1.3

- CC location restriction

- REFSENS relaxations

- New contig. CA BW classes

List of candidate targets for first round of discussion in agenda 7.12.1.5:

- Scope of work

- Capability framework for feature

List of candidate targets for first round of discussion in agenda 7.12.1.8:

- Formula-based prediction of beam squint

List of candidate targets for first round of discussion in agenda 7.12.1.3

- Refine CRs

List of candidate targets for first round of discussion in agenda 7.12.1.5:

- LS discussion

- CR refinement

List of candidate targets for first round of discussion in agenda 7.12.1.8:

- Continue discussion

# Topic #1: Intra-band DL CA

## Companies’ contributions summary

| T-doc number | Title | Company | Proposals / Observations |
| --- | --- | --- | --- |
| R4-2009753 | CC allocation in intra-band non-cont. DL CA | Intel Corporation | Proposal: In intra-band DL CA, no CC can be scheduled cross the boundary of DL-only spectrum and bi-directional spectrum. |
| R4-2010300 | Release 17 FR2 bandwidth class | Verizon UK Ltd | Proposal: Discuss the following contiguous BW classes in FR2 for Rel-17 NR CA bandwidth class; Aggregated channel bandwidth; Number of contiguous CC; Fallback group. N; 800 MHz < BWChannel_CA ≤ 900 MHz; 9; 5. N1; 900 MHz < BWChannel_CA ≤ 1000 MHz; 10. N2; 1000 MHz < BWChannel_CA ≤ 1100 MHz; 11. N3; 1100 MHz < BWChannel_CA ≤ 1200 MHz; 12. N4; 1200 MHz < BWChannel_CA ≤ 1300 MHz; 13. N5; 1300 MHz < BWChannel_CA ≤ 1400 MHz; 14. N6; 1400 MHz < BWChannel_CA ≤ 1500 MHz; 15. N7; 1500 MHz < BWChannel_CA ≤ 1600 MHz; 16. |

## Open issues summary

### Sub-topic 1-1

CC restriction relative to the boundary of DL-only spectrum and bi-directional spectrum

Issue 1-1: Whether a network can schedule a CC across the boundary of DL-only spectrum and bi-directional spectrum

- Proposals

  - Option 1: Network not allowed to schedule a CC across the boundary of DL-only spectrum and bi-directional spectrum

  - Option 2: (no restriction)

- Recommended WF

  - Agree to option 1

### Sub-topic 1-2

REFSENS relaxation for wide configured DL spectrum

Issue 1-2-1: REFSENS relaxation for 2400 MHz configured DL spectrum

- Proposals

  - Option 1: 1 dB

  - Option 2: other

Issue 1-2-2: Wording of REFSENS relaxation so it applies to both kinds of UEs: Fsd=0 and Fsd!=0

- Proposals

  - Option 1: Change ‘cumulative aggregated BW’ to ‘configured DL spectrum’

  - Option 2: other

### Sub-topic 1-3

Discussion on new contiguous BW classes for FR2 for Rel-17

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| N | 800 MHz < BWChannel_CA ≤ 900 MHz | 9 | 5 |
| N1 | 900 MHz < BWChannel_CA ≤ 1000 MHz | 10 |  |
| N2 | 1000 MHz < BWChannel_CA ≤ 1100 MHz | 11 |  |
| N3 | 1100 MHz < BWChannel_CA ≤ 1200 MHz | 12 |  |
| N4 | 1200 MHz < BWChannel_CA ≤ 1300 MHz | 13 |  |
| N5 | 1300 MHz < BWChannel_CA ≤ 1400 MHz | 14 |  |
| N6 | 1400 MHz < BWChannel_CA ≤ 1500 MHz | 15 |  |
| N7 | 1500 MHz < BWChannel_CA ≤ 1600 MHz | 16 |  |

Issue 1-3: Discussion on new contiguous BW classes for FR2 for Rel-17

## Companies views’ collection for 1st round

### Open issues

| Issue | Options | Company Comments |
| --- | --- | --- |
| Issue 1-1: Whether a network can schedule a CC across the boundary of DL-only spectrum and bi-directional spectrum | Y/N | Intel: No OPPO: Agree with WF, i.e. Option 1: Network not allowed to schedule a CC across the boundary of DL-only spectrum and bi-directional spectrum Samsung: No Nokia: Agree with WF, Option 1 |
| Issue 1-2-1: REFSENS relaxation for 2400 MHz configured DL spectrum | 1 dB | OPPO: OK with 1dB. Apple: This topic has been discussed for several meeting without alternatives proposal on the REFSENS relaxation for this frequency range. For this reason, we think the 1 dB can be agreed. |
|  | other | Samsung: discussion on this issue is not sufficient. In previous discussions, there were two proposals, one is 1dB, the other is 1.5dB. There was also discussion about whether to apply two range between 1400MHz and 1400 MHz, if a whole range for 1400~2400Mhz, 1.5dB is better. Huawei: 1dB is enough for 2400MHz when common LNA is used. Any implementation with separate LNAs cannot fulfill with this 1dB relaxation. |
| Issue 1-2-2: Wording of REFSENS relaxation so it applies to both kinds of UEs: Fsd=0 and Fsd!=0 | Option 1: Change ‘cumulative aggregated BW’ to ‘configured DL spectrum’ | Intel: Clarification question How to apply the relaxation to the following cases 1) DL Fs = 1200MHz, Fsd = 1200MHz 2) DL-Fs = 2400MHz, Fsd = 0MHz. In case 1, there are two separate Rx chains with each chain has Rx BW = 1200MHz, should relaxation be applied to DL-Fs and Fsd independently? OPPO: For clarification, Option 1 only changes the name but definition keeps same, i.e. “is defined as the frequency band from the lowest edge of the lowest CC to the upper edge of the highest CC of all UL and DL configured CCs”, then what is the difference? Samsung: after Fsd is introduced in Rel-16, the meaning of CABW need to be clarified. If CABW only applies to Fs, the CABW wording here need to be changed. Besides ‘configured DL spectrum’, ‘overall frequency separation’ may be also an alternative to emphasize that the configured spectrum may not be contiguous. Qualcomm: To Intel, the same relaxation would be available for CA_REFSENS for both cases. CA_REFSENS is a per CC requirement, and the per CC relaxation would not discriminate between UEs that use one Rx chain or 2 or more parallel Rx chains To Oppo, the idea is to preserve the meaning, but to cut the link between the parameter used to determine CA MPR and the parameter used for CA REFSENS. In Rel-15, CABW was that common link, but in Rel-16, CABW meaning needs to be updated due to creation of DL-only spectrum. |
|  | Option 2: other |  |
| Issue 1-3: Discussion on new contiguous BW classes for FR2 for Rel-17 |  | Apple: We recommend not to use the letter “N” for the BW Class, since the letter “N” has been used to identify the NR bands. Also, we disagree with the numbering after BW Class, such as N1, N2, etc. It has not been used for the BW Classes until now and could cause confusion when defining the CA combinations and violates the band combination rules. This contribution proposes 8 new CA BW Classes in the table, in order to support wider contiguous spectrum in FR2. Our suggestion is to consider 200 MHz instead per carrier, this way the amount of new proposals could be reduced. ZTE: Thanks for the proposal. A question for clarification. Do we really need to establish so many CA BW classes for such a large aggregated bandwidth? If 16 contiguous CCs are applied, it may be difficult to represent so many component carriers in CA configuration table in section 5.5A. If frequency band interval can be larger than 100MHz, the number of CA bandwidth classes will be less. |

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2009754 | Intel: Support adding this restriction |
|  | OPPO: OK |
|  | Samsung: we support this CR Nokia: OK Huawei: “When a UE configures DL-only spectrum, it shall not expect a CC to be configured across the boundary between bidirectional spectrum and DL-only spectrum.” Revise into->“When a UE is configured with DL-only spectrum, it shall not expect a CC to be configured across the boundary between bidirectional spectrum and DL-only spectrum UE can support respectively.” |
| R4-2011451 | Intel: Clarification question How to apply the relaxation to the following cases 1) DL Fs = 1200MHz, Fsd = 1200MHz 2) DL-Fs = 2400MHz, Fsd = 0MHz. In case 1, there are two separate Rx chains with each chain has Rx BW = 1200MHz, should relaxation be applied to DL-Fs and Fsd independently? |
|  | OPPO: For clarification, Option 1 only changes the name but definition keeps same, i.e. “is defined as the frequency band from the lowest edge of the lowest CC to the upper edge of the highest CC of all UL and DL configured CCs”, then what is the difference? |
|  | Qualcomm: To Intel, the same relaxation would be available for CA_REFSENS for both cases. CA_REFSENS is a per CC requirement, and the per CC relaxation would not discriminate between UEs that use one Rx chain or 2 or more parallel Rx chains To Oppo,the idea is to preserve the meaning, but to cut the link between the parameter used to determine CA MPR and the parameter used for CA REFSENS. In Rel-15, CABW was that common link, but in Rel-16, CABW meaning needs to be updated due to creation of DL-only spectrum. |
|  | Samsung: Generally agree that EIS relaxation depends on (Fs+Fsd) |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary | 2nd round topic |
| --- | --- | --- |
| Issue 1-1: Whether a network can schedule a CC across the boundary of DL-only spectrum and bi-directional spectrum | Tentative agreements: Concept is agreeable Recommendations for 2nd round: Refine CR wording |  |
| Issue 1-2-1: REFSENS relaxation for 2400 MHz configured DL spectrum | Agreement possible at 1.5 dB Recommendations for 2nd round: Refine CR wording | Need clarification from Huawei why this is true: Huawei: 1dB is enough for 2400MHz when common LNA is used. Any implementation with separate LNAs cannot fulfill with this 1dB relaxation. |
| Issue 1-3: Discussion on new contiguous BW classes for FR2 for Rel-17 |  | Suggestion to consider fewer channels of wider BW to achieve same frequency coverage, better naming |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2009754 | to be revised |
| R4-2011451 | to be revised |

## Discussion on 2nd round (if applicable)

| CR/TP number | CRs/TPs Status update recommendation | Company comments |
| --- | --- | --- |
| R4-2011739 | Pursue wording refinement of R4-2009754 |  |
| R4-2011740 | R4-2011451 CR seems agreeable with 1.5 dB: Configured DL spectrum (MHz); (dB). ≤ 800; 0.0. > 800 and ≤ 1400; 0.5. > 1400 and ≤ 2400; 1.5. | Comment if you feel otherwise. |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2011739 | Agreeable |
| R4-2011740 | Agreeable |

# Topic #2: Inter-band DL CA

## Companies’ contributions summary

| T-doc number | Title | Company | Proposals / Observations |
| --- | --- | --- | --- |
| R4-2009557 | (withdrawn) |  |  |
| R4-2009755 | FR2 inter-band DL CA | Intel Corporation | Proposal: Introduce a new frequency separation class capability signaling for CBM UEs, such frequency separation class includes frequency separation per band plus the maximum frequency gap between them. |
| R4-2009963 | Beam Management assumptions for inter-band CA | Apple Inc., Qualcomm Incorporated | Observation 1: In our view the introduction of a capability to distinguish between IBM/CBM band pairs is not necessary Proposal 1: The beam management assumption for inter-band CA 28 GHz + 39 GHz is IBM. Proposal 2: The beam management assumption for inter-band CA 28 GHz + 28 GHz shall be CBM only. Proposal 3: The beam management assumption for inter-band CA 39 GHz + 39 GHz is CBM only. Proposal 4: For intra-band CA, the beam management assumption is CBM only. Proposal 5: RAN4 shall define the band combinations as CBM/IBM in the specification. Proposal 6: RAN4 shall capture beam management type assumption per band pair as described in Option 2. |
| R4-2010200 | Discussion on FR2 inter-band DL CA | Samsung | Proposal 1: define UE capability per band pair to distinguish CBM and IBM. Requirements shall apply to CBM and IBM respectively when there are differences. Proposal 2: For CBM band pairs, no spherical coverage requirement will be defined. Proposal 3: For CBM band pairs, the UE shall meet the Peak EIS requirements per band for FR2 DL CA, and it is allowed to re-use the RX beam peak direction obtained from single carrier test for CBM inter-band CA REFSENS test. Proposal 4: CBM and IBM have different EIS relaxation factors and shall be treated separately. Intra-band REFSENS requirement can be a reference and a starting point for CBM REFSENS requirement discussion. Proposal 5: based on proposal 2, beam squint loss can be ignored for FR2 DL CA for both CBM and IBM. Beam squint analysis can be concluded for FR2 DL CA, and beam squint analysis can continue in FR2 UL CA in future. Observation 1: a fixed value of PSD difference requirement is not feasible for inter-band CA OTA test. Proposal 6: Do not specify PSD difference at reference point. It is proposed to minimize PSD difference at baseband for inter-band CA EIS test which is applicable for both IBM and CBM. |
| R4-2010537 | Scope and TP of FR2 Inter-band DL CA in Rel-16 | Nokia, Nokia Shanghai Bell | Proposal 1: Only one CA configuration CA_n260A-n261A is completed in Rel-16. Proposal 2: Only collocated deployment is assumed for CA_n260A-n261A in Rel-16 timeframe, i.e., only a single AoA configuration is required. Proposal 3: IBM is assumed for CA_n260A-n261 by default without UE capability in Rel-16. Proposal 4: The maximum power imbalance is assumed 6.5 dB for CA_n260A-n261A. Proposal 5: Other open issues are to be postponed to Rel-17, such as introduction of CBM requirement (especially for L+L comb), UE capabilities and other band combinations and configurations. |
| R4-2011390 | Inter-band CA UE requirement fragmentation | Qualcomm Incorporated | Proposal 1: Define requirements for requested configurations in [2] assuming IBM Proposal 2: Define requirements for CBM when configurations are requested Proposal 3. Keep the agreement in [6] and define a common spherical coverage requirement for all inter-band CA combinations. Proposal 4: Do not define different MRTD or PSD imbalance requirements for inter-band CA combinations. Proposal 5: No new capabilities that create fragmentation will be defined for Rel-16 and work focused from now on concluding combinations in [2]. |
| R4-2011420 | Inter-band DL CA in FR2: CBM/IBM capability and associated spherical coverage EIS tests | Sony, Ericsson | Observation 1: A CBM UE is assumed to support the co-located deployment scenarios. An IBM UE is assumed to support both co-located and non-co-located deployment scenarios. Configuration of a DL inter-band CA based on these indicated UE capabilities is up to the gNB. Observation 2: To guarantee the UE inter-band CA performance under the co-located deployment scenario, the common spherical coverage area is needed in the EIS test. Observation 3: X = 2.5 on each CC can guarantee the UE to meet the common spherical coverage requirement for 24 GHz + 43 GHz. Observation 4: Y depends on the frequency separation between primary and secondary CCs (see Table I.) Z = 0.5 is sufficient for 24GHz + 30 GHz and 37 GHz+ 43GHz. Proposal 1: Introduce CBM/IBM for inter-band CA as an NR UE per-band combination capability for DL and UL in Rel-16 (regardless if requirements are completed or not). Support of IBM for a band combination implies support of CBM. Proposal 2: Support of co-located and/or non-co-located deployment capability per band pair need not be UE capability from a beam management viewpoint, CBM or IBM indication suffices. Proposal 3: The beam management capability shall not be linked to the discussion on MRTD and timing/synchronization requirements. Proposal 4: The spherical coverage requirements for inter-band CA shall be defined in a way that it can guarantee enough UE minimum performance under the desired deployment scenarios. Proposal 5: Adopt the requirement on solid angles, described by directions that simultaneously meet the inter-band CA spherical coverage for both bands, with the common area of 50 % for both IBM and CBM UEs, with the single AoA test setup. Proposal 6: For IBM inter-band CA spherical coverage, allow X dB relaxation on each CC to enable the UE to form the common spherical coverage. Proposal 7: For CBM inter-band CA spherical coverage, allow Y dB relaxation on the secondary CC due to the beam squint effect. If the common coverage area needs to be built, additional relaxation Z dB on each CC could be added. Proposal 8: Define the PSD imbalance as 0 dB for the CBM inter-band CA DL EIS. Proposal 9: Define the PSD imbalance as 6.5 dB for the IBM inter-band CA DL EIS |
| R4-2011448 | Remaining Details on FR2 Inter-band DL CA | Futurewei Technologies | Proposal 1: Introduce IBM and CBM capability per band pair Observation 1: With the implicit understanding that the CBM band pair shall support co-located deployments and meet corresponding requirements, IBM band-pair shall support both co-located and non-co-located deployments, the other attributes associated with CBM/IBM capabilities are also need to be clarified and defined. Observation 2: The requirements for inter-band DL CA band pairs will be individually defined, so the requirements should be investigated case-by-case if any of the intra-band non-contiguous DL CA requirements apply. In most cases reuse of requirements is possible, but when there is a frequency-dependent RF degradation or different BM process it should be investigated case-by-case. Proposal 2: Adopt requirement on area of sphere (or equivalently, solid angle) where both bands meet their respective single CC EIS spherical coverage requirements. Observation 3: For co-located scenarios, both CBM and IBM could define common coverage Observation 4: For non-co-located scenarios, the “common spherical coverage” requirement is not applicable. Proposal 3: For CBM L+L and H+H PSD difference could be small but this non-zero value should be captured. For L+H and IBM the PSD difference could be large bur for testability point of view 6.5dB can be agreed. Observation 5: If the CBM capability and explicit co-located deployment conditions are established, it would allow inter-band FR2 CA MRTD to be lower than 8us. Proposal 4: It is proposed to explicitly define co-located deployment information to allow parameters such as MRTD to set appropriate requirements. |
| R4-2011483 | On inter band DL CA_FR2 | Huawei, HiSilicon | Proposal 1: Remove FR2 inter-band CA with L+L or H+H combination objective from Rel-16 WID. Observation 1: 3us MRTD is not applicable for inter-band CA CBM if performance loss is not expected. Proposal 2: Remove FR2 inter-band CA CBM type from Rel-16 WID. Proposal 3: RAN4 agrees to define different EIS spherical coverage requirement for inter-band CA L+H combination and indicate the RF requirement capability with single bit: • Bit 0: Spherical coverage for each band determinedly separately without common range definition • Bit 1: common EIS spherical coverage range between the two bands shall be 50% for power class 3 Proposal 4: for inter-band CA L+H combination with IBM type, single polarization for each band is assumed to define the Rx requirement. Proposal 5: 3dB EIS requirement difference is required between single polarization and dual polarization architecture for each Band. Proposal 6: If UE indicates RF requirement capability with common spherical coverage, 10% relaxation on spherical coverage requirement for inter-band L+H IBM CA, where spherical coverage means the common spherical coverage range between the 2 bands. Or 3dB relaxation on spherical coverage requirement for CDF 50% for inter-band L+H IBM CA. Proposal 7: 3dB per band is defined additionally for inter-band L+H IBM CA on min peak EIS. |
| R4-2011510 | PSD for FR2 inter-band CA with IBM | NTT DOCOMO INC. | Proposal: DL power condition for PSD imbalance requirements for FR2 inter-band CA with IBM capability (Bands A+B) should be specified as following: - Test case 1: - DL power of band A = Peak EIS of band A + Multi-band relaxation - DL power of band B = Peak EIS of band A + Multi-band relaxation + Delta - Test case 2: - DL power of band A = Peak EIS of band B + Multi-band relaxation - DL power of band B = Peak EIS of band B + Multi-band relaxation + Delta - Delta = MAX (Delta A-B, Delta B-A) |

## Open issues summary

### Sub-topic 2-1

FR2 inter-band CA feature scope

The primary goal of discussion is to scope for this feature. Options:

- Target completion of feature based on requested CA band combination(s). Note that only n260+n261 has been requested in RP-200672 (‘NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2)’). CR R4-2010538 is consistent with this option

OR

- Target feature completion for a more general set of inter-band combinations, including combinations that have not been requested.

Towards understanding the time-line ramifications of each choice, below table is the space of capabilities proposed over the course of this feature’s work phase and how they may apply to inter-band band pairs. ‘band group’ definition is per usage in agreed WF R4-1916024. If option 2 is chosen, some or all of the listed capabilities would need to be developed, including identifying interdependencies.

| Inter-band CA capabilities proposed: | Applicability to band pair in: |  | Discussion |
| --- | --- | --- | --- |
|  | same band group | different band groups |  |
| IBM |  |  | Does IBM imply CBM capability? |
| CBM |  |  | Does RAN4 need to come up with requirements for CBM for Rel-16, despite no requested combinations from same band group? |
| (Lack of) Common spherical coverage |  |  | How to verify inter-band CA support if a UE declares ‘no’ for common spherical coverage. Can a UE declare CBM as well as ‘no’ for common spherical coverage? |
| MRTD capability |  |  | How does this capability relate to UEs with IBM/CBM |

Issue 2-1-1: Feature scope for Rel-16

- Develop new capabilities or define lean feature

  - Option 1: Define feature only for requested band combination (n260+n261). Assume IBM as default capability for band pairs\. Define CBM capability and requirements in subsequent releases. Existing MRTD values in 38.133 applies, and no discussion needed. See CR R4-2010538.

  - Option 2: Establish combination of capabilities that will be included in the feature

| Options | CBM/IBM | (Lack of) Common Spherical Coverage capability | MRTD capability |
| --- | --- | --- | --- |
|  | (=CBM/IBM defined by specification) | (=all UEs must have common spherical coverage) | (= UE MRTD requirement is determined independent of other inter-band CA capabilities) |
| 2-1-1-2-1 |  |  |  |
| 2-1-1-2-2 |  |  |  |
| 2-1-1-2-3 |  |  |  |
| 2-1-1-2-4 |  |  |  |
| 2-1-1-2-5 |  |  |  |
| 2-1-1-2-6 |  |  |  |
| 2-1-1-2-7 |  |  |  |

- Recommended WF

  - Refine or agree CR R4-2010538

### Sub-topic 2-3

Feature discussion of CR R4-2010538.

Issue 2-3-1: IBM/CBM capability for Rel-16

- Proposals:

  - Option 1: No new capability is defined. IBM is assumed, CBM capability can be defined in a future release

  - Option 2: IBM / CBM capability defined but value hard set to ‘IBM’ until CBM is fully defined

  - Option 3: IBM / CBM capability defined, but without requirements for CBM

## Companies views’ collection for 1st round

### Open issues after GTW of 18 August 2020

| Issue | Options | Company Comments |
| --- | --- | --- |
| Issue 2-3-1: IBM/CBM capability for Rel-16 | Option 1: No new capability is defined. IBM is assumed, CBM capability can be defined in a future release. | Qualcomm: We prefer this method but are willing to compromise to option 2 Apple: In our contribution (R4-2009963) we have shared our view on how the introduction of a capability is not necessary. Additionally, since the Nokia’s CR proposes to down-scope the introduction of inter-band CA to n260+n261 in Rel-16, we think it’s not necessary to discuss this capability in Rel-16, since only IBM will be assumed. |
|  | Option 2: IBM / CBM capability defined but value hard set to ‘IBM’ until CBM is fully defined | Nokia: Option 2 Sony: Both Option 2 and Option 3 are fine to us, but option 2 is more reasonable in our view. IBM/CBM capability is necessary for Rel-16 and shall be defined per band pair. This capability can allow UE vendors can select different RF architectures for inter band CA and also enable operators to deploy the network with both co-located and non-collocated. The Rel-16 BS also need to know such a capability to support the release independent. RAN4 has discussed this capability for a long time and majority view is clear. Delay this capability to Rel-17 would lead to a repeated discussion and complicated the implementation of the capability. NTT DOCOMO, INC: In current our understanding, it would be better to take option 2 so that Rel-16 UE can also support CBM after CBM requirements will be introduced. Qualcomm: in case RAN4 decides to pursue this option, until CBM requirements are agreed (as cat-B CR for 38.101-2), RAN2 specifications (38.133 or 38.306 or both) should mandate UEs to set the capability to IBM. Huawei: for RF requirement, only the requested band combination (n260+n261) is defined. Reuse the current MRTD in TS 38.133 for (n260+n261). So only (n260+n261) with IBM type RF requirement is defined for Rel-16. But IBM/CBM Capability per band combination need to be introduced in Rel-16 |
|  | Option 3: IBM / CBM capability defined, but without requirements for CBM | Qualcomm: We do not agree to this method. CBM capability has significant limitations on what can be expected of the UE (example: MRTD), and significant performance expectations (for example: concurrent common spherical coverage). Without definition of associated requirements, CBM capability cannot be enabled for declaration. Intel: Option 3 Huawei: for RF requirement, only the requested band combination (n260+n261) is defined. Reuse the current MRTD in TS 38.133 for (n260+n261). So only (n260+n261) with IBM type RF requirement is defined for Rel-16. But IBM/CBM Capability per band combination need to be introduced in Rel-16 |

### Open issues before GTW of 18 August 2020 (For reference)

| Issue | Options | Company Comments |
| --- | --- | --- |
| Issue 2-1-1: Feature scope for Rel-16 | Option 1: Define feature only for requested band combination (n260+n261). Assume IBM as default capability band pair. Define CBM capability and requirements in subsequent releases. Existing MRTD values in 38.133 applies, and no discussion needed. See CR R4-2010538. | NTT DOCOMO, INC.: We would like to support the direction of down scoping, but would like to revise the CR R4-2010538. Our request is to capture n257+n259(the highest band combination is n257I+n259M), and to discuss the value of PSD difference and relaxation values of spherical coverage after agreeing down scoping. NOTE: n257+n259 was described in R4-2008056. In our understanding, band combinations should be requested to basket WI after completion of general requirements. That’s why we did not propose the combinations in basket WI. OK with specifying only LB+HB band combinations with IBM as default capability, and existing MRTD values in 38.133 applies, in Rel-16, given that there seems no request from operators for LB+LB or HB+HB so far. Huawei: for RF requirement, only the requested band combination (n260+n261) is defined. Reuse the current MRTD in TS 38.133 for (n260+n261). So only (n260+n261) with IBM type RF requirement is defined for Rel-16. But IBM/CBM Capability per band combination need to be introduced in Rel-16. |
|  | Option 2: Capability combination Options; CBM/IBM; (Lack of) Common Spherical Coverage capability; MRTD capability. (=CBM/IBM defined by specification); (=all UEs must have common spherical coverage); (= UE MRTD requirement is determined independent of other inter-band CA capabilities). 2-1-1-2-1. 2-1-1-2-2. 2-1-1-2-3. 2-1-1-2-4. 2-1-1-2-5. 2-1-1-2-6. 2-1-1-2-7. | Intel: Defines feature for CA_n260A-n261A is quite limited. RAN4 could save significant time if only CA_n260A-n261A was considered at the beginning of Rel-16. RAN4 has taken significant effort on this WID by the end of day. We see completion the complete feature is necessary. On the other hand, band combination is supported in a release independent manner, so don’t see the reason why CBM is not defined in Rel-16. OPPO: 2-1-1-2-4 - CBM/IBM should be covered by spec considering different UE implementations. - If RAN4 only define IBM for inter band group cases in Rel-16 to close this WI, then with clarification in spec and leaving other cases to Rel-17 is also ok. But it means no intra band group is supported in Rel-16, this will have big restriction on FR2 CA. - UE should support Common Spherical Coverage considering the different BS deploy scenarios that UE may confront. - MRTD could be discussed in RRM session. Sony: 2-1-1-2-3 is our preference. To clarify our position for each item here: 1. CBM/IBM capability defined per band pair, 2. all UEs must have common spherical coverage, 3. UE MRTD requirement is also dependent on other inter-band CA properties (not directly link to CBM/IBM). Ericsson: 2-1-1-2-3: we remark that the MRTD also depend on parameters not related to beam management such as the BS TAE. Huawei: We prefer option3, i.e. CBM/IBM is introduced as UE capability per band combination in Rel-16. Spherical coverage can be with different UE capability: Bit 0: Spherical coverage for each band determinedly separately without common range definition Bit 1: common EIS spherical coverage range between the two bands shall be 50% for power class 3 For MRTD requirement, it is better to depend on RRM session. |
| In case Option 1 in 2-1-1 is not chosen, and further WID extension may be necessary to resolve the following: |  |  |
| Issue 2-2-1: New frequency separation class signalling for CBM band pair | Option 1: Yes, introduce a new frequency separation class capability signaling for CBM UEs, such that frequency separation class includes frequency separation per band plus the maximum frequency gap between them | Intel: Option 1. We see UE has limitation to support CBM across entire frequency span between two bands in the same band group. When gNB knows this frequency separation class from UE, then gNB can schedule the inter-band CA within this limit for CBM UE. OPPO: Option 1. CBM restriction should be known to NW to facilitate the CA configuration. Nokia: Can be discussed in REL-17. But we think it is problematic for operators if some UE can work only for some operators but cannot work for others, even if UE support the same band combination. Huawei: Yes, but such separation class capability is defined per band combination for L+L or H+H combination with CBM type |
|  | Option 2: No need to define new type of inter-band nX+nY CA for a UE that cannot support simultaneous configuration of a CC anywhere in nX with a CC anywhere in nY | Apple: Intel’s contribution mentioned their concern on the performance degradation if this capability is not introduced. We think this impact is covered by beam squint discussion, and therefore this capability is not required. |
| Issue 2-2-2: Can UE declare lack of common spherical coverage capability for CBM /IBM band pairs? | Option 1: Both IBM and CBM | OPPO: UE should support Common Spherical Coverage considering the different BS deploy scenarios that UE may confront. For IBM UEs, in some area there are collocated BSs while in other areas there are non-collocated BSs, UE should be able to work in both cases. Nokia: Against all options. Lack of common spherical coverage means that UE cannot support collocated deployment, which is against the earlier agreement. Sony: We see that NO options are feasible here, all UE (both IBM and CBM) need to support common spherical coverage. UE needs to support common spherical coverage regardless of the CBM/IBM capability since the co-located deployment scenario is assumed to be supported by both CBM and IBM UEs based on the WF R4-2005736 agreed in April meeting. Ericsson: no options are feasible, the co-located scenario must be supported by both CBM- and IBM-capable UEs. |
|  | Option 2: Only CBM |  |
|  | Option 3: Only IBM | Intel: Option 3. A IBM UE needs to support non-collocated deployment which common spherical coverage may not be necessary. Huawei: only for IBM is OK for us. This should be introduced in Rel-16. |
| Issue 2-2-3: What requirements apply for a UE band pair with no common spherical coverage? | Option 1: none | Nokia: Support of common coverage shall be mandatory both for CBM and IBM. |
|  | Option 2: RAN4 must determine requirements even if they are not testable in the short term | Intel: Independent coverage may be possible. OPPO: In our understanding, UE should support common spherical coverage and meet corresponding requirements. Sony: A UE band pair with no common spherical coverage is NOT a valid case, see our reply to Issue 2-2-2. Ericsson: support of common spherical cover age mandatory for both CBM and IBM capable UEs. Huawei: Spherical coverage for each band determinedly separately without common range definition. |
| Issue 2-2-4: MRTD capability for UE band pair | Option 1: CBM UEs use intra-band MRTD, IBM UEs use inter-band MRTD in TS38.133 | Intel: Option 1 OPPO: ok with option 1 Apple: For intra-band CA co-located scenario is assumed. It was agreed in RAN4#95e in MRTD discussion that only collocated scenario will be considered for CBM. Thus, we support Option 1 |
|  | Option 2: Capability declaration for CBM, IBM UEs use inter-band MRTD in TS38.133 | Nokia. Partly option 2, IBM uses current inter-band MRTD in TS38.133 and CBM MRTD value can be discussed in REL-17. Huawei: if MRTD requirement for CBM still cannot be in consensus in RRM session, we can see option 2 is the only solution. |
|  | Option 3: No need to discuss | Sony: Option 3. We think MRTD is not directly linked to the CBM/IBM capability. The BM capability is mainly for enabling different UE front end designs and deployment scenarios. Apple: In RRM has been agreed that no CBM related RRM requirements will be specified in R16. |
| Issue 2-2-5: IBM capability for UE band pair implies CBM capability | Option 1: Yes | Intel: Yes OPPO: Option 1, IBM UE should have the capability of CBM. Sony: Option 1 Yes. Ericsson: yes. |
|  | Option 2: No | Nokia: Not necessarily, because CSI-RS is provided in both bands in IBM. For CBM, the CSI-RS may be available only in one band. We are ok that IBM UE does not support CBM configuration. but it is also ok that IBM UE also support CBM configuration. |
|  | Option 3 | Huawei: IBM with common spherical coverage range capability implies CBM IBM without common spherical coverage range capability do not implies CBM |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2010537 | Intel: PSD imbalance is not reflected. |
|  | NTT DOCOMO, INC.: We would like to support the direction of down scoping, but would like to revise the CR R4-2010538. Our request is to capture n257+n259(the highest band combination is n257I+n259M), and to discuss the value of PSD difference and relaxation values of spherical coverage after agreeing down scoping. NOTE: n257+n259 was described in R4-2008056. In our understanding, band combinations should be requested to basket WI after completion of general requirements. That’s why we did not propose the combinations in basket WI. OK with specifying only LB+HB band combinations with IBM as default capability, and existing MRTD values in 38.133 applies, in Rel-16, given that there seems no request from operators for LB+LB or HB+HB so far Nokia: our view is n257+n259 is not urgent due to spectrum availability. We are open to discuss the requirement for the proposed combination in CR drafting session, though there has not been any proposal (TP or CR) until the first round. |
|  | Huawei: 1. Common AOA should be removed from TS 38.101-2, this common AOA only related to RAN5. If UE only applies requirement for common AOA, how could IBM UE support non-collocated in real network. 2. For PSD difference, we proposed to define OBB requirement, and same as IBB level. 3. Delta Rib revised to 3dB per Band Nokia: There was agreement that both IBM and CBM support common AoA for the collocated deployment. IBM can support non-collocated deployment in addition to collocated deployment. It is not only RAN5 issue to verify the common AoA but the core requirement issue. We can further discuss how to make requirement and tests for non-collocated deployment; however, our proposal is to have collocated requirement in Rel-16, as difficult issues are expected to test RF requirement in 2 AoA setting, which emulates the non-collocated deployment. We are open to hear your proposal how make requirement and test for this. |
| R4-2010538 | MODERATOR NOTE after GTW of 18 Aug 2020: Please see folder for draft CR to update with refinements within the parameters of the agreement: - Focus on making CR R4-2010538 agreeable at this meeting https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_96_e/Inbox/Drafts/%5B117%5D%20NR_RF_FR2_req_enh_Part_3/Inter%20DLCA%20feature%20CR NTT DOCOMO, INC.: Same with comments in R4-2010537. NTT DOCOMO, INC (After GTW) If whether release independent approach can apply is FFS, we would like request to add n257+n259(the highest band combination is n257I+n259M) in Rel-16 CR. For the value of PSD difference, this is a controversial topic, but to fix this issue, out compromised suggestion is to focus on the discussion on the testability of PSD difference. But this does not mean 6.5dB of PSD difference, since we can see the possibility of higher PSD difference than 6.5dB based on R4-2000444 from a TE vendor. |
|  | Apple: There is a mistake in Table 5.5A.3-1 for the NR CA configurations for inter-band CA, the SCS is 15 and 30 kHz instead of SCS 60 and 120 kHz. For intra-band CA the proposal for relaxation is 1 dB. We think that the value for the REFSENS relaxation Table 7.3A.2.3-1 and EIS spherical coverage relaxation Table 7.3A.3.3-1 should be larger than 1 dB, when considering LB + HB band group. This discussion should be independent of the MBR framework, which was derived considering single CC, it is not related to CA. We think that the relaxation should be larger than 1 dB for n260 + n261, the additional FE loss increase has to be considered, therefore a higher relaxation should apply for the REFSENS. We should consider the misalignment as well between the beam peak directions of the CCs, since the requirement considers common coverage area. Due to these reasons, we propose [1.5 dB] in square brackets for both Tables. |
|  | Huawei: 1. Common AOA should be removed from TS 38.101-2, this common AOA only related to RAN5. If UE only applies requirement for common AOA, how could IBM UE support non-collocated in real network. 2. For PSD difference, we proposed to define OBB requirement, and same as IBB level. 3. Delta Rib revised to 3dB per Band. 4. Spherical coverage for IBM type can be up to UE capability, which indicates whether common range coverage is support 5. For inter-band CA, the Rx baseline can take 1 polarization as baseline Nokia: There was agreement that both IBM and CBM support common AoA for the collocated deployment. IBM can support non-collocated deployment in addition to collocated deployment. It is not only RAN5 issue to verify the common AoA but the core requirement issue. We can further discuss how to make requirement and tests for non-collocated deployment; however, our proposal is to have collocated requirement in Rel-16, as difficult issues are expected to test RF requirement in 2 AoA setting, which emulates the non-collocated deployment. We are open to hear your proposal how make requirement and test for this. - Relaxation of 3 dB is too large. - Common rage is a must (according to the earlier agreement.) |
|  | Futurewei: Correction and suggested text to CR R4-2010538. Some clarification questions as well. - Correction: Table 5.5A.3-1, the SCS for n261 should be 60kHz and 120kHz (not 15 and 30kHz). - Suggested text: Just above Table 5.2A.2-1, we suggest a sentence like 5.2A.1. Suggested text “NR inter-band carrier aggregation is designed to operate in the operating bands defined in Table 5.2A.2-1, where all operating bands are within FR2.” - Question1: In 7.3A.2.3 the phrase “common AoA for all active component carriers” – does this imply co-located deployment? Since there are no explicit sentences about deployment scenario, can we take this as co-located? Similarly, in 7.3A.3.3, the “intersection set of spherical coverage areas” imply co-located? - Question2: Note1 in Table 5.5.A.3-1 “within 6.5dB”, can this power imbalance be taken as continuation of question1 implicit assumption it is “IBM co-location”? Question3: On two occasions there are references to 7.3.2. In this 7.3.2 where the subsections are according to power class. Should we clarify power class association here in the CR, either by PC3 or specifically pointing to 7.3.2.3? Nokia: There was agreement that both IBM and CBM support common AoA for the collocated deployment. IBM can support non-collocated deployment in addition to collocated deployment. It is not only RAN5 issue to verify the common AoA but the core requirement issue. We can further discuss how to make requirement and tests for non-collocated deployment; however, our proposal is to have collocated requirement in Rel-16, as difficult issues are expected to test RF requirement in 2 AoA setting, which emulates the non-collocated deployment. We are open to hear your proposal how make requirement and test for this. |

## Summary for 1st round

### Open issues

MODERATOR NOTE after GTW of 18 Aug 2020:

Please see folder for draft CR to update with refinements within the parameters of the agreement:

- Focus on making CR R4-2010538 agreeable at this meeting

|  | Status summary |  |
| --- | --- | --- |
| IBM/CBM capability | Option 1: No new capability is defined. IBM is assumed, CBM capability can be defined in a future release. (Apple) Option 2: IBM / CBM capability defined but value hard set to ‘IBM’ until CBM is fully defined (Nokia, Sony, NTT Docomo, Oppo, Samsung-compromise, Intel-compromise, Qualcomm-compromise, Huawei - ambiguous) Option 3: IBM / CBM capability defined, but without requirements for CBM (Huawei-ambiguous) | Status reflects post-DL positions taken by companies towards enabling a compromise solution: Most companies either support or are willing to go with option 2. LS wording is being finalized, TDOC# requested below One company re-iterated that the details of option 2 must be captured in the standard also: in case RAN4 decides to pursue this option, until CBM requirements are agreed (as cat-B CR for 38.101-2), RAN2 specifications (38.133 or 38.306 or both) should mandate UEs to set the capability to IBM. |
| Other capabilities | Not pursued because of focus on CR, per GTW agreement From existing comments, need for many capabilities remain contentious. |  |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | draft LS on UE capability for FR2 inter-band CA | Nokia |

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2010537 | to be revised |
| R4-2010538 | to be revised |

## Discussion on 2nd round (if applicable)

| CR/TP number | LS/CRs/TPs Status update recommendation |
| --- | --- |
| R4-2011472 | Pursue wording refinement in revision of R4-2010537 NTT DOCOMO, INC. The TP should be revised according to the conclusion of CR R4-2011473. |
| R4-2011473 | Pursue wording refinement in revision of CR R4-2010538 on separate thread NTT DOCOMO, INC. We have objection to the CR. As we commented in GTW, we would like to specify higher PSD value than 6.5dB, and would like to propose to test it as Rx requirements. The agreements captured in the latest WF were: - PSD difference between bands in Refsens i.e. peak EIS: - Agree PSD difference is within a range[6.5 – 30] dB and RAN4 aims to agree one number in RAN4#95 - PSD difference between bands in EIS spherical coverage: - Agree a range[6.5 – 30] dB and target to agree one number in RAN4#95 We would like to suggest to use intermediate value between 6.5-30 dB, i.e., 18dB. In addition, we would like to request to add n257+n259, which is LB+HB band combinations with IBM capability as same with n260+n261. Intel: We don’t accept the CR regarding PSD imbalance test. Since it does not follow the WF (see WF quoted in DOCOMO’s comments) We provide here an example for inter band CA n261 + n260 with both 50Mhz BWs, Let’s test band n261 with n260 settled at -73.1dBm during the test which is n260 EIS at 50%-tile according the CR. The criterion for n261 to pass EIS spherical coverage test is P1 = -77.4dBm + allowed ΔMBS,n and ΔRIB,S,n where -74.4dBm is EIS at 50%-tile for n260. Once n260 can get 50%-tile EIS coverage at P1 power level, then n260 can be claimed in the CR to meet the EIS spherical coverage. You can calculate that P1 = -77.4 + ΔMBS,n + ΔRIB,S,n = -77.4 + ~0.7 + 1.5 = -75.2dBm. The real PSD difference is only 75.2 – 73.1 = 2.1dB because n261 is NOT required to pass the EIS spherical test at sensitivity level which is -88.3 dBm. The true PSD stress test is something like to further increase the n260 power level to P1 + required PSD imbalance while n261 pass EIS spherical coverage at P1. For Peak EIS test in current CR, there is even no PSD imbalance test. It apparently does not follow the WF as well. - PSD difference between bands in Refsens i.e. peak EIS: - Agree PSD difference is within a range[6.5 – 30] dB and RAN4 aims to agree one number in RAN4#95 Samsung: About PSD, we have observation in previous discussion that it is not feasible for EIS test which is based on EIS search instead of BER testing with fixed DL signal level. So we prefer not to specify PSD imbalance in EIS test. Accordingly, the EIS spherical coverage specification in the feature CR shall be changed back to the original version. About EIS relaxation spec, [1.5]dB is specified for both REFSENS and EIS spherical coverage in the feature CR. The evaluation on EIS relaxation is not sufficient. [1.5]dB is obtained based on common spherical coverage analysis, and then directly used for REFSENS. We can see that there is not enough dedicated analysis for REFSENS relaxation yet and consequently 1.5dB is not justified. For EIS spherical coverage, we have identified other relaxation factors other than the common coverage relaxation factor, so 1.5dB is not enough. So we prefer keep the value as FFS in the table. Huawei: The requirement is not fulfilled under common AOA, we can say Limited by the potential complexity of testability, it can be verified under common AOA chamber. For PSD issue, we can simply define OBB requirement for inter-band CA. For deta Rib, 1.5dB is not enough. At least 2dB. Apple: We don’t agree that the inter-band CA combination n257-n259 should be included in this CR. The introduction of band combination is frozen in Rel-16. We think that the regular approach should be followed, and that the request for this inter-band CA should take place within the Basket WI in Rel-17. With this approach the combination would be release independent back to Rel-16. Once the band combination is approved, then we can introduce the requirements in the specification. |
| R4-2011471 | Refine wording of LS on separate thread Apple: If the UE feature list will be modified to include the following sentence to reflect the beam management type in the specification. “IBM/CBM indication is in accordance with the beam management type applicable for band combinations specified in 38.101-2.” We have proposed the revision for R4-2011743. For the “Consequence if the feature is not supported by the UE”, we proposed the following wording: UE does not support inter-band CA within FR2. The network cannot configure the UE with inter-band CA within FR2 |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

| CR/TP number | LS/CRs/TPs Status update recommendation |
| --- | --- |
| R4-2011472 | Not pursued |
| R4-2011473 | CR needs further check |
| R4-2011471 | LS is agreeable |

# Topic #3:	FR2 Beam Squint

## Companies’ contributions summary

| T-doc number | Title | Company | Proposals / Observations |
| --- | --- | --- | --- |
| R4-2011442 | Beam squint considerations | FUTUREWEI | Observation: the sensitivity degradation using a formula based on evenly-spaced antenna elements for a linear array (without any implementation margins) is consistent with reported values For future work Proposal: With agreed-upon implementation margins, a formula-based approach can be used to determine sensitivity degradation due to beam squint as a function of carrier frequency, frequency separation, and number of elements. |

## Open issues summary

### Sub-topic 3-1

Adopt formula-based approach to capturing beam squint degradation

Issue 3-1: : With agreed-upon implementation margins, a formula-based approach can be used to determine sensitivity degradation due to beam squint as a function of carrier frequency, frequency separation, and number of elements

- Proposals

  - Option 1: yes

  - Option 2: no

## Companies views’ collection for 1st round

### Open issues

| Issue | Options | Company Comments |
| --- | --- | --- |
| Issue 3-1: formula-based approach to determine sensitivity degradation due to beam squint as a function of carrier frequency, frequency separation, and number of elements | Y/N | Intel: No. This is first time to discuss this formula-based approach, not fully studied. OPPO: In Rel-16, few contributions have discussed on this aspect. Concluding on the exact values could be difficult in the end of Rel-16, but the factors discussed in the formula-based approach (carrier frequency, frequency separation, and number of elements) can be considered in the following discussions. Samsung: based on GTW agreement, in Rel-16 there is no CBM requirements. For IBM, beam squint is not applicable. So beam squint can be concluded in Rel-16 and further discuss beam squint in future release Nokia: Our view is that this relaxation can be included in the inter-band CA delta Tib/Rib of L+L (or H+H) combo (primarily CBM band pair). We proposed to discuss CBM requirement in Rel-17 as we discussed in GTW. We think 0 dB is assumed for the 1.4GHz or less separation. Sony: No strong opinion on the method itself, but the final values should be able to align with other proposed values from previous contributions if it would be specified. Qualcomm: As relevant to Rel-16 requirements, beam squint may no longer be a significant player. As a concept, while elegant, there are significant problems. The formula-based determination would need agreement on array configuration for each power class and other implementation details that will diverge as technology evolves. Also, there is dependence on more sensitive UE implementation details, like beam spatial packing density. Apple: No, the simulations assumptions were agreed several meetings ago in the WF (R4-2005675). According to our simulation results in R4-2004714, for DL intra-band non-contiguous CA with Fs ≤ 1400 the impact is < 0.5 dB captured as a degradation in the 50%-tile EIS CDF performance. For DL intra-band non-contiguous CA with 1400 < Fs ≤ 2400 MHz the impact is < 0.6 dB. These values are 0.4 and 0.5 higher compared to the values shown using the formula-based approach. Huawei: we don’t want to express Yes or No here. We would like to clarify: whether we need beam squint study for CBM UE with frequency separation up to 5~6GHz? If so, how many Rx chains is supposed for this CBM UE? If 2 Rx chains, whether MRTD=0.26us is required for such UE? If MRTD is not 0.26us, whether Beam management can be common between Bands? Is the beam squint analysis related to DL-only separation class introduction? |

### CRs/TPs comments collection

(none)

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 3-1: formula-based approach to determine sensitivity degradation due to beam squint as a function of carrier frequency, frequency separation, and number of elements | Recommendations for 2nd round: (continue discussion) |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

| Issue | Options | Company Comments |
| --- | --- | --- |
| Issue 3-1: formula-based approach to determine sensitivity degradation due to beam squint as a function of carrier frequency, frequency separation, and number of elements | Y/N | Futurewei: We agree that the document should be noted. Thank you all for the comments. We will try to address each. To Intel/OPPO: we understand the concerns about the need to examine the formula. We want to provide a less intensive approach to estimate the magnitude of beam squint. Obviously, simulations based on actual hardware models should determine the magnitude of beam squint. To Samsung/Qualcomm: agree with no need of using beam squint for IBM and based on the results of GTW, beam squint is not needed for this meeting. To Nokia: How this relaxation is captured is a different discussion. For the statement “We think 0 dB is assumed for the 1.4GHz or less separation”, the formula shows less than 0.1 dB beam squint effect for N=4 elements and at 28 and 39 GHz. To Sony: Agree that “the model” should be compared to presented results. There were limited results available. To Qualcomm: The model is agnostic to hardware implementation. Perhaps, augmenting the formula for 2-D arrays and with d/2 can also extend the capability of this model. We welcome the opportunity to address the limitations. To Apple: We suggested that the degradation is the sum of two terms: estimated beam squint plus a component for hardware implementation. In Nokia’s comment, the “We think 0 dB is assumed for the 1.4GHz or less separation” is slightly different than your results. To Huawei: From the results, it seems with N=4 antenna elements and at 28 GHz, the effects of beam squint are small for ~5 GHz separation. The model does not incorporate multiple Rx chains. Beam squint analysis can also be applicable for the UL. It could affect the measured power for a second carrier using CBM. |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# 4	Version Tracking

| Version | File |
| --- | --- |
| V0 | R4-2011550 + 2nd round discussion |
| R4-2011857 | Discussion_117_2nd_v0_FW_DCM_Intel _Samsung_NK_HW Discussion_117_2nd_v0_FW_DCM_Intel _Samsung_NK_Apple |