3GPP TSG-RAN4 Meeting #96-e	R4-2011935

# Online, , 17th Aug 2020 - 28th Aug 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 0438 | rev | 2 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | A-MPR definition for CA_n48B, CA_n41B and CA_n41C |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Skyworks Solution Inc., [Qualcomm Inc.] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1-Core |  |  |  |  |  | Date: |  |  | 2020-08-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A-MPR is missing from CA configurations CA_n41B, CA_n41C and CA_n48B altough it is already in specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of A-MPR for CA_n41B, CA_n41C and CA_n48B. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Uplink CA_n41B, CA_n41C and CA_n48B needs to be removed from specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.3, 6.5A.2.3, 6.5A.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************************** Start of changes ******************************************

### 6.2A.3	UE additional maximum output power reduction for CA

##### 6.2A.3.1.1	UE additional maximum output power reduction for Intra-band contiguous CA

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [7]. Relation between NR CA band and NR frequency band is specified in Table 5.2A.1-1.

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2A.1.5-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2A.2.4. In absense of modulation and waveform types the A-MPR applies to all modulation and waveform types.

Table 6.2A.3.1.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable CA band(s) for each CA_NS value. The mapping of NR CA band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.1.1-2.

Table 6.2A.3.1.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR CA Band | Aggregated channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| CA_NS_01 |  | Table 5.2A.1-1 | All applicaple NR CA bands | All applicaple NR CA configurations | N/A |
| CA_NS_04 | 6.5A.2.3.1.1 6.5A.3.3.1.1 | CA_n41 |  | 6.2A.3.1.2 | 6.2A.3.1.2 |
| CA_NS_27 | 6.5A.2.3.1.2 6.5A.3.3.1.2 | CA_n48 |  | 6.2A.3.1.3 | 6.2A.3.1.3 |

[The CA_NS_01 label with the field additionalPmax [7] absent is default for all NR bands.]

Table 6.2.3.1.1-2: Mapping of network signaling label

| NR CA band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| CA_n41 | CA_NS_01 | CA_NS_04 |  |  |  |  |  |  |
| CA_n48 | CA_NS_01 | CA_NS_27 |  |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [7]. |  |  |  |  |  |  |  |  |

##### 6.2A.3.1.2	A-MPR for CA_NS_04

###### 6.2A.3.1.2.1	Contiguous allocations

For all waveform type, modulations and scs when Fedge, low - BWChannel_CA ≥ 2490.5 MHz, A-MPR = MPR

For all modulations and scs when Fedge, low - BWChannel_CA <  2490.5 MHz

if the RB allocation is an inner allocation as defined in Table 6.2A.2.4-1 then A-MPR = MPR

Except for RBstart ≤ 0.33*BWchannel_CA/0.18MHz, AMPR= max (MPR, AMPRcc).

if the RB allocation is an outer allocation as defined in Table 6.2A.2.4-2,

then A-MPR = MPR+1.5dB for BW Class B A-MPR = MPR for BW class C.

Where

- MPR is the MPR as defined in Table 6.2A.2.4-1 for the respective CA bandwidth class

-	 AMPRcc is defined as the PC3_A1 AMPR in table 6.2.3.2-2.

###### 6.2A.3.1.2.2	Non-contiguous allocations

For intra-band contiguous CA_n41B and CA_n41C and it receives IE CA_ NS_04, the UE determines the allowed Additional Maximum Power Reduction (AMPR) for the maximum output power as specified in this clause. The AMPR is specified by AMPRIM3 to meet -25dBm/MHz when IM3 falls in -25dBm/MHz region of Table 6.5A.2.3.1-1 or Table 6.5A.3.3.1-1. And uses MPR for all other cases.

The UE determines the AMPR type as follows:

For all waveform types, modulations and SCS when Fedge, low - BWChannel_CA ≥ 2490.5 MHz,

if allocation is an inner or outer 1 allocation as defined in Table 6.2A.2.4-2 then A-MPR = MPR

if allocation is an outer 2 allocation as defined in Table 6.2A.2.4-2 then A-MPR = MPR-1dB

For all waveform types, modulations and SCS when Fedge, low - BWChannel_CA <  2490.5 MHz

If AND( MIN(FIM3,low_block,high, SEM-13,low) < Ffilter,low ,  MAX( SEM-13,high, FIM3,high_block,low ) > Ffilter,high )

if RB allocation is an inner or outer 1 allocation as defined in Table 6.2A.2.4-1 then A-MPR = MPR

if RB allocation is an outer 2 allocation as defined in Table 6.2A.2.4-2 then A-MPR = MPR-1dB

Else

A-MPR = A-MPRIM3 defined in Clause 6.2A.3.1.2.2.1

where

-	MPR is the MPR as defined in Table 6.2A.2.4-2 for the respective CA bandwidth class

-    FIM3,low_block,high = (2 * Flow_alloc,high_edge ) – Fhigh_alloc,low_edge

-	FIM3,high_block,low = (2 * Fhigh_alloc,low_edge) – Flow_alloc,high_edge

-	Flow_alloc,low_edge is the lowermost frequency of lower transmission bandwidth allocation.

-	Flow_alloc,high_edge is the uppermost frequency of lower transmission bandwidth allocation.

-	Fhigh_alloc,low_edge is the lowermost frequency of upper transmission bandwidth allocation.

-	Fhigh_alloc,high_edge is the uppermost frequency of upper transmission bandwidth allocation.

-	Ffilter,low = 2480 MHz

-	Ffilter,high = 2745 MHz

-	SEM-13,high = Threshold frequency where upper spectral emission mask for upper channel drops from -13 dBm / 1MHz to -25 dBm / 1MHz, as specified in Clause 6.5A.2.3.1.1

-	SEM-13,low = Threshold frequency where lower spectral emission mask below the lower channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Subclause 6.5A.2.3.1.1

6.2A.3.1.2.2.1	AMPRIM3 to meet -25dBm/MHz

AMPR in this clause is for intra-band contiguous CA_n41B and CA_n41C. The allowed maximum output power reduction is defined as:

AMPRIM3=MA, Where MA is defined as follows

MA = 	13; 	0 ≤ B < 2.16

11.5; 	2.16 ≤ B < 3.24

10.5;       3.24 ≤ B < 5.04

9.5; 	5.04 ≤ B < 10.08

8; 	10.08 ≤ B < 16.56

7;        16.56 ≤ B < 21.96

6; 	      21.96 ≤ B

Where:

B=(LCRB_alloc, 1* 12* SCS1 + LCRB_alloc,2 * 12 * SCS2)/1,000,000

###### 6.2A.3.1.3	A-MPR for CA_NS_27

6.2A.3.1.3.1	Contiguous allocations

For all modulations and scs when Fedge, low - BWChannel_CA ≥ 3540 MHz AND Fedge, high + BWChannel_CA ≤ 3710 MHz

if allocation is inner 1 then A-MPR = 0 dB where inner 1 is defined as

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB_agg – RBStart,Low – LCRB

with following conditions

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB  ≤  ceil(NRB_agg /2)

Inner 1 region exceptions thresholds are

RBstart ≤ 34 and RBstart ≥ 178 for BWChannel_CA = 40MHz, and

RBstart ≤ 28 and RBstart ≥ 164 for BWChannel_CA = 35MHz, and

RBstart ≤ 22 and RBstart ≥ 142 for BWChannel_CA = 30MHz, and

RBstart ≤ 17 and RBstart ≥ 120 for BWChannel_CA = 25MHz, and

RBstart < 12 and RBend ≥  92 for BWChannel_CA = 20MHz, and

RBstart < 6 and RBend ≥ 71 for BWChannel_CA = 15MHz,

For which AMPR = 5dB.

else A-MPR= 5 dB

For all modulations and scs when 3550 MHz ≤ Fedge, low < 3540 MHz + BWChannel_CA

if allocation is inner 3 then A-MPR = 0 dB.

Inner 3 region exceptions thresholds are

RBstart < 63 and RBstart > 168 for BWChannel_CA = 40MHz, and

RBstart < 52 and RBstart > 152 for BWChannel_CA = 35MHz, and

RBstart < 41 and RBstart > 136 for BWChannel_CA = 30MHz, and

RBstart < 30 and RBstart > 116 for BWChannel_CA = 25MHz, and

RBstart < 20 for BWChannel_CA = 20MHz, and

RBstart < 10 for BWChannel_CA = 15MHz,

For which AMPR = 7dB for BWChannel_CA ≤ 20MHz and 11.5dB for BWChannel_CA > 20MHz

where inner 3 is defined as

RBStart = NRB_agg /4

LCRB = NRB_agg/4

RBStart = NRB_agg 3/4  LCRB

with following conditions

NRB_agg /4 < RBStart < NRB_agg 3/4  LCRB  AND LCRB < NRB_agg/4

else when BWagg ≤ 20 MHz, A-MPR = 7 dB or when BWagg > 20 MHz, A-MPR = 11.5dB.

For all modulations and scs when 3710 MHz - BWChannel_CA < Fedge, high ≤ 3700

if allocation is inner 3 then A-MPR = 0 dB.

Inner 3 region exceptions thresholds are

RBend < 36 and RBend > 132 for BWChannel_CA = 40MHz, and

RBend < 27 and RBend > 121 for BWChannel_CA = 35MHz, and

RBend < 18 and RBend > 110 for BWChannel_CA = 30MHz, and

RBend < 9 and RBend > 99 for BWChannel_CA = 25MHz, and

RBstart > 70 for BWChannel_CA = 20MHz, and

RBstart > 62 for BWChannel_CA = 15MHz,

For which AMPR = 7dB for BWChannel_CA ≤ 20MHz and 11.5dB for BWChannel_CA > 20MHz

where inner 3 is defined as

RBStart = NRB_agg /4

LCRB = NRB_agg/4

RBStart = NRB_agg 3/4  LCRB

with following conditions

NRB_agg /4 < RBStart < NRB_agg 3/4  LCRB  AND LCRB < NRB_agg/4

else when BWagg ≤ 20 MHz, A-MPR = 7 dB or when BWagg > 20 MHz, A-MPR = 11.5dB.

6.2A.3.1.3.2	Non-contiguous allocations

For all modulations and scs when Fedge, low - BWChannel_CA ≥ 3540 MHz AND Fedge, high + BWChannel_CA ≤ 3710 MHz

A-MPRCA_IM5=

13; 	0 ≤B<1.08

12; 	1.08 ≤B<2.16

11; 	2.16 ≤B<3.24

10.5; 	3.24 ≤ B < 5.04

9.5; 	5.04≤B< 10.08

8; 		10.08 ≤B< 16.56

7; 		16.56 ≤ B < 21.96

6.5; 		21.96 ≤B

For all modulations and scs when 3550 MHz ≤ Fedge, low < 3540 MHz + BWChannel_CA or 3710 MHz - BWChannel_CA < Fedge, high ≤ 3700

when BWagg ≤ 20 MHz

A-MPRCA_IM5=

13; 	0 ≤B<1.08

12; 	1.08 ≤B<2.16

11; 	2.16 ≤B<3.24

10.5; 	3.24 ≤ B < 5.04

9.5; 	5.04 ≤B< 10.08

8; 		10.08 ≤B< 16.56

7; 		16.56 ≤ B < 21.96

6.5; 	21.96 ≤B

or when BWagg > 20 MHz

A-MPRCA_IM3 =

20; 	0 ≤B<1.08

19.5; 	1.08 ≤B<2.16

19; 	2.16 ≤B<3.24

18.5; 	3.24 ≤ B < 5.04

18; 	5.04 ≤B< 10.08

17; 	10.08 ≤B< 16.56

16; 	16.56 ≤ B < 21.96

13; 	21.96 ≤B

****************************** No changes ******************************************

#### 6.5A.2.3	Additional spectrum emission mask for CA

##### 6.5A.2.3.1	Additional spectrum emission mask for intra-band contiguous CA

###### 6.5A.2.3.1.1	Requirements for network signalled value "CA_NS_04"

When "CA_NS_04" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.2.3.1.1-1.

Table 6.5A.2.3.1.1-1: Additional requirements for "CA_NS_04"

| ΔfOOB MHz | BWChannel_CA (MHz) / Spectrum emission limit (dBm) |  | Measurement bandwidth |
| --- | --- | --- | --- |
|  | ≤50 | >50 |  |
| ± 0 - 1 | -10 |  | 2 % of BWChannel_CA |
|  |  | -10 | 1 MHz |
| ± 1 - 5 | -10 |  | 1 MHz |
| ± 5 - X | -13 |  |  |
| ± X - (BWChannel_CA + 5 MHz) | -25 |  |  |
| NOTE: X is aggregated bandwidth |  |  |  |

###### 6.5A.2.3.1.2	Requirements for network signalled value "CA_NS_27"

When "CA_NS_27" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.2A.2.3.2.1.-1.

Table 6.2A.2.3.2.1-1: Additional requirements for "CA_NS_27"

| Spectrum emission limit (dBm) / measurement bandwidth for each aggregated channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB MHz | Aggregated channel bandwidth of max 40 MHz | Measurement bandwidth |
| ± 0 - 1 | -13 | 1 % of X |
| ± 1 - X | -13 | 1 MHz |
| < – X or > X | -25 |  |
| NOTE 1: X is the aggregated channel bandwidth NOTE 2: The requirements apply only at the frequency range from 3540 MHz to 3710 MHz. |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

****************************** No changes ******************************************

##### 6.5A.3.2.6	Transmit intermodulation for intra-band contiguous CA

For intra-band contiguous carrier aggregation the requirement of transmitting intermodulation is specified in Table 6.5A.3.2.6-1.

Table 6.5A.3.2.6-1: Transmit Intermodulation

| CA bandwidth class(UL) | B and C |  |  |
| --- | --- | --- | --- |
| Interference Signal Frequency Offset | BWChannel_CA | 2*BWChannel_CA |  |
| Interference CW Signal Level | -40dBc |  |  |
| Intermodulation Product | -29dBc | -35dBc |  |
| Measurement bandwidth (NOTE1) | Nominal channel space+MBWACLR,low/2+ MBWACLR,high/2 |  |  |
| Measurement offset from channel center | BWChannel_CA and 2*BWChannel_CA |  | 2*BWChannel_CA and 4*BWChannel_CA |
| NOTE 1: MBWACLR,low and MBWACLR,high are the single-channel ACLR measurement bandwidths specified for channel bandwidths BWchannel(low) and BWchannel(high) in 6.5.2.4.1, respectively. |  |  |  |

#### 6.5A.3.3	Additional spurious emissions for CA

##### 6.5A.3.3.1	Additional spurious emissions for intra-band contiguous  CA

##### 6.5A.3.3.1.1		Requirement for network signalling value "CA_ NS_04"

When "CA_NS04" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.3.3.1.1-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5A.3.1-1 from the edge of the aggregated channel bandwidth.

Table 6.5A.3.3.1.1-1: Additional requirements for "CA_ NS_04"

| Frequency range (MHz) | BWChannel_CA (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 20 to 190 MHz |  |
| 2495 ≤ f < 2496 | -13 | Max(1 % of BWChannel_CA, 1 MHz) |
| 2490.5 ≤ f < 2495 | -13 | 1 MHz |
| 0.009 < f < 2490.5 | -25 | 1 MHz |

##### 6.5A.3.3.1.2		Requirement for network signalling value "CA_ NS_27"

When "CA_NS 27" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.3.3.1.2-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table Table 6.5A.3.1-1 from the edge of the aggregated channel bandwidth.

Table 6.5A.3.3.1.2-1: Additional requirements for "CA_ NS_27"

| Frequency range (MHz) | Spectrum emission limit (dBm) for aggregated channel bandwidth of max 40 MHz | Measurement bandwidth |
| --- | --- | --- |
| 9 kHz – 3530 MHz | -40 | 1 MHz |
| 3530 MHz – 3540 MHz | -25 |  |
| 3710 MHz – 3720 MHz | -25 |  |
| 3720 MHz – 12.75 GHz | -40 |  |

****************************** End of changes ******************************************