3GPP TSG-RAN WG4 Meeting #96-e                 	R4-2012109

Online, August 17th – 28th, 2020

Agenda Item:	7.4.3.2

Source: 	Nokia, Nokia Shanghai Bell

Title:		TP on CA scenarios in Transmit Timing requirements for IAB-MT

Document for:	Approval

Introduction

CA scenarios support for wide area IAB-MT class in Transmit Timing requirements for IAB-MT is introduced in this TP.

Text Proposal

--------------------------------------------------Start of TP------------------------------------------------------

### 12.2.1 IAB-MT transmit timing

Editor notes: The terminology of “downlink” and “uplink” could be revised and aligned with RF conclusion.

#### 12.2.1.1 Introduction

The IAB-MT shall have capability to follow the frame timing change of the reference cell in connected state. The uplink frame transmission takes place  before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. IAB-MT belonging to local area or wide area IAB-MT class as defined in clause 4.4.2 and also capable of carrier aggregation shall use the SpCell as the reference cell for deriving the IAB-MT transmit timing for cells in the PTAG. IAB-MT initial transmit timing accuracy, gradual timing adjustment requirements are defined in the following requirements.

--------------------------------------------------End of TP------------------------------------------------------