3GPP TSG-RAN4 Meeting #96-e	R4-2012134

# Electronic Meeting, 17 – 28 August, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1083 | rev | 1 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for measurement requirements for PRS-RSRP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2020-07-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | PRS-RSRP measurement requirements are not defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define PRS-RSRP measurement requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Core requirements for the WI objective are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.9.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

9.9.3	PRS-RSRP measurements

9.9.3.1	Introduction

The requirements in clause 9.9.3.5 shall apply provided the UE has received a message from LMF via LPP [34] requesting the UE to report PRS-RSRP measurements defined in TS 38.215 [4].

9.9.3.2	Requirements applicability

The requirements in clause 9.9.3 apply, provided:

-	PRS-RSRP related side conditions given in clause 10.1.24 are met for a corresponding Band.

9.9.3.3	Measurement Reporting Requirements

This requirement assumes that the measurement report is not delayed by other LPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration. This measurement reporting delay excludes any delay caused by no UL resources for UE to send the measurement report.

The reported DL RSTD measurement values contained in periodically triggered measurement reports shall be based on the measurement report mapping requirements specified in sections 10.1.24.3.

9.9.3.4	Measurement Period Requirements

When the physical layer receives NR-DL-AoD-ProvideAssistanceData message via LPP [34], the UE shall be able to measure PRS-RSRP from configured PRS resources for configured TRPs on configured PRS frequency layers according to the UE capabilities, within TPRS-RSRP,total ms, where

TPRS-RSRP, total=i=1LTPRS-RSRP,i+L-1*maxTeffect,i.

where i is the index of PRS frequency layer, TPRS-RSRP,i is the measurement period for PRS frequency layer i as defined in the following, L is total number of positioning frequency layers, and Teffect,i is the periodicity of PRS measurement of PRS frequency layer i as defined in the following.

TPRS-RSRP,i=CSSFi*NRxBeam,i*NPRS,islotN'LPRS,iN*Nsample-1*Teffect,i+Tlast

where

CSSFi is scaling factor for gap sharing between PRS layer i and RRM measurements as defined in clause 9.1.5.2,

NRxBeam,i is the scaling factor for Rx beam sweeping, and NRxBeam,i=1 if PRS layer i is in FR1 and NRxBeam,i=8 if PRS layer i is in FR2,

LPRS,i is the time duration spanned by PRS sample of all DL PRS resources of positioning frequency layer i within one PRS period (TPRS). For the purpose of DL PRS processing capability, LPRS,i is calculated by either Type I duration calculation with UE symbol level buffering capability or Type II duration calculation with UE slot level buffering capability as defined in clause 5.1.6.5 of TS 38.214 [26, section 5.1.6.5],

NPRS,islot is the maximum number of DL PRS resources of PRS frequency layer i configured in a slot,

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms processed every T ms for a given maximum bandwidth supported by UE as specified in clause 4.2.7.2 of TS 38.306 [14],

N’ is UE capability for number of DL PRS resources that it can process in a slot as specified in clause 4.2.7.2 of TS 38.306 [14],

Nsample is the number of PRS measurement samples and Nsample= [4],

Teffect,i is periodicity of PRS measurement of PRS frequency layer i:

Teffect,i= TiTavailable_PRS,i*Tavailable_PRS,i

where Tavailable_PRS,i= LCMTPRS,i,MGRPi, and TPRS,i is the maximum PRS resource periodicity among all PRS resources of PRS frequency layer i, MGRPi is the repetition periodicity of the measurement gap applicable for measurement of PRS frequency layer i.

The PRS-RSRP measurement accuracy for all measured PRS resources shall be fulfilled according to the accuracy as specified in the clauses 10.1.24.

If handover occurs while PRS-RSRP measurements are being performed then the UE shall complete the ongoing positioning measurement session. The UE shall also meet the PRS-RSRP measurement and accuracy requirements. However in this case the PRS-RSRP measurement period TPRS-RSRP,total.HO shall be as follows:

TPRS-RSRP, total,HO=TPRS-RSRP, total+K*Teffect+THO   ms

where

K is the number of times handover occurs during TPRS-RSRP,total.HO;

Teffect is the largest Teffect,i among all PRS layers;

THO is the time during which the PRS-RSRP measurement may not be possible due to handover; it can be up to Tinterrupt as defined in clause 6.1.

<End of Change 1>