3GPP TSG-RAN4 Meeting #96-e	R4-2012159

# Electronic Meeting, 17 – 28 August, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 6948 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 36.133 for CGI reading |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_Enh-Core |  |  |  |  |  | Date: |  |  | 2020-07-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Requirements for interruption to LTE serving cells have been endorsed in R4-2008991 for the following cases: - UE in EN-DC, NR GCI reading configured by LTE PCell or NR PSCell - UE in LTE SA, NR CGI reading configured by LTE PCell These changes are re-submitted with corrected section reference. In addiiton, requirements for interruption to LTE serving cells should also be defined for the following cases: - UE in NE-DC, LTE CGI reading configured by NR PCell - UE in NE-DC, NR CGI reading configured by NR PCell |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the above mentioned requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CGI reading requirements in 36.133 are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.32.2.y, 7.37, 7.36.2.x1, 7.36.2.x2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

7.32.2.y	Interruption during NR measurement with autonomous gaps

When a UE is identifying CGI of an NR cell with autonomous gaps as configured by PCell or NR PSCell, the UE is allowed on PCell or any activated SCell in MCG if configured

-	up to K1 interruptions, each with up to X1 interrupted subframes, during MIB decoding time period TMIB specified in clause 9.X of TS 38.133 [50] if the CGI reading is configured by NR PSCell, or TMIB_NR specified in clause 8.17.X1 and 8.17.Y1 if the CGI reading is configured by LTE PCell,

-	up to L1 interruptions, each with up to Y1 interrupted subframes, during SIB1 decoding time period TSIB1 specified in clause 9.X of TS 38.133 [50] if the CGI reading is configured by NR PSCell, or TSIB1_NR specified in clause 8.17.X1 and 8.17.Y1 if the CGI reading is configured by LTE PCell, for SSB and CORESET for RMSI scheduling multiplexing patterns 1,

-	up to L2 interruptions, each with up to Y2 interrupted subframes, during SIB1 decoding time period TSIB1 specified in clause 9.X of TS 38.133 [50] if the CGI reading is configured by NR PSCell, or TSIB1_NR specified in clause 8.17.X1 and 8.17.Y1 if the CGI reading is configured by LTE PCell, for SSB and CORESET for RMSI scheduling multiplexing patterns 2 and 3,

where K1, L1, L2, X1, Y1 and Y2 are as defined in 8.2.1.2.X of TS 38.133 [50] assuming μ=0 for victim cell.

<End of Change 1>

<Start of Change 2>

7.37	Interruptions during NR measurement with autonomous gaps

7.37.1	Introduction

This section contains the requirements for the interruptions on PCell and activated SCells if configured, when performing inter-RAT NR measurement with autonomous gaps cells.

7.37.2	Requirements

When a UE is identifying CGI of an NR cell with autonomous gaps as configured by PCell, the UE is allowed on PCell or any activated SCell if configured

-	up to K1 interruptions, each with up to X1 interrupted subframes, during MIB decoding time period TMIB_NR specified in clause 8.1.2.4.X.2 and 8.1.2.4.Y.2,

-	up to L1 interruptions, each with up to Y1 interrupted subframes, during SIB1 decoding time period TSIB1_NR specified in clause 8.1.2.4.X.2 and 8.1.2.4.Y.2, for SSB and CORESET for RMSI scheduling multiplexing patterns 1,

-	up to L2 interruptions, each with up to Y2 interrupted subframes,  during SIB1 decoding time period TSIB1_NR specified in clause 8.1.2.4.X.2 and 8.1.2.4.Y.2, for SSB and CORESET for RMSI scheduling multiplexing patterns 2 and 3,

where K1, L1, L2, X1, Y1 and Y2 are as defined in 8.2.1.2.X of TS 38.133 [50] assuming μ=0 for victim cell.

<End of Change 2>

<Start of Change 3>

7.36.2.x1	Interruption during E-UTRA measurement with autonomous gaps

When UE is configured by NR PCell to identify CGI of an E-UTRA FDD or E-UTRA TDD cell, UE may cause interruptions on PSCell, and SCell(s) in SCG if configured.

During the time period of Tidentify_CGI, E-UTAN as specified in clause 9.4.7 of TS 38.133 [50], over which the UE identifies the new CGI of the E-UTRA cell, the UE shall transmit at least 60 ACK/NACKs on FDD PSCell and each of activated FDD SCell(s), and at least NACK/NACK ACK/NACKs on TDD PSCell and each of activated TDD SCell(s) where NACK/NACK is specified in Table 7.36.2.x1-1, provided that

-	there is continuous DL data allocation,

-	no DRX and no eDRX_CONN cycle is used,

-	no measurement gaps are configured,

-	only one code word is transmitted in each subframe,

-	no MBSFN subframes are configured in the PCell or each of activated SCell(s).

Table 7.36.2.x1-1: Number of ACK/NACKs transmitted by the UE during the time for identification of CGI of an E-UTRA cell

| UL/DL configuration | Minimum number of transmitted ACK/NACKs |
| --- | --- |
| 0 (Note 1) | 18 |
| 1 | 35 |
| 2 | 43 |
| 3 | 36 |
| 4 | 39 |
| 5 | 42 |
| 6 | 30 |
| Note 1: When a UE is configured with EIMTA-MainConfigServCell via RRC signalling [2] only this requirement shall apply. |  |

7.36.2.x2	Interruption during NR measurement with autonomous gaps

When UE is configured by NR PCell to identify CGI of an NR cell, UE may cause interruptions on PSCell, and SCell(s) in SCG if configured.

-	up to K1 interruptions, each with up to X1 interrupted subframes, during MIB decoding time period TMIB specified in clause 9.X of TS 38.133 [50],

-	up to L1 interruptions, each with up to Y1 interrupted subframes, during SIB1 decoding time period TSIB1 specified in clause 9.X of TS 38.133 [50], for SSB and CORESET for RMSI scheduling multiplexing patterns 1,

-	up to L2 interruptions, each with up to Y2 interrupted subframes, during SIB1 decoding time period TSIB1 specified in clause 9.X of TS 38.133 [50], for SSB and CORESET for RMSI scheduling multiplexing patterns 2 and 3,

where K1, L1, L2, X1, Y1 and Y2 are as defined in 8.2.1.2.X of TS 38.133 [50] assuming μ=0 for victim cell.

<End of Change 3>