3GPP TSG-RAN WG4 Meeting #96-e	R4-2012167

# Electronic Meeting, 17 – 28 August, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1067 | rev | 1 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CSSF for inter-frequency measurement without gap in FR2 inter-band CA sceneario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh-Core |  |  |  |  |  | Date: |  |  | 2020-08-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Correction on inter-frequency measurement without gaps. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Some correction on CSSF for inter-frequency measurement without gaps are made: - when part of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap and interFrequencyConfig-NoGap-r16 is indicated by network, UE performs inter-frequency measurement without gap. The condition “interFrequencyConfig-NoGap-r16 is indicated by network” is missing. - Add the CSSF for inter-frequency measurement without gap in FR2 inter-band CA scenario. - Adding the measurement type for within gap in section 9.1.5.2 : when part of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap and interFrequencyConfig-NoGap-r16 is not indicated by network. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

9.1.5	Carrier-specific scaling factor

This clause specifies the derivation of carrier-specific scaling factor (CSSF) values, which scales the measurement delay requirements given in clause 9.2, 9.3 and 9.4 when UE is configured to monitor multiple measurement objects. The CSSF values are categorized into CSSFoutside_gap,i and CSSFwithin_gap,i, for the measurements conducted outside measurement gaps and within measurement gaps, respectively.

9.1.5.1	Monitoring of multiple layers outside gaps

The carrier-specific scaling factor CSSFoutside_gap,i for measurement object i derived in this chapter is applied to following measurement types:

-	Intra-frequency measurement with no measurement gap in clause 9.2.5, when none of the SMTC occasions of this intra-frequency measurement object are overlapped by the measurement gap.

-	Intra-frequency measurement with no measurement gap in clause 9.2.5, when part of the SMTC occasions of this intra-frequency measurement object are overlapped by the measurement gap.

-	Inter-frequency measurement with no measurement gap in clause 9.3.9, when none of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap, if UE supports interFrequencyMeas-NoGap-r16 and the flag interFrequencyConfig-NoGap-r16 is configured by the Network.

-	Inter-frequency measurement with no measurement gap in clause 9.3.9, when part of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap, if UE supports interFrequencyMeas-NoGap-r16 and the flag interFrequencyConfig-NoGap-r16 is configured by the Network.

UE is expected to conduct the measurement of this measurement object i only outside the measurement gaps.

If the higher layer signaling in TS 38.331 [2] of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, CSSFoutside_gap,i and requirements derived from CSSFoutside_gap,i are not specified.

The UE cell identification and measurement periods derived based on CSSFoutside_gap,i in clauses 9.2.5.1, 9.2.5.2 may be extended for measurement objects of which the cell identification and measurement periods are overlapped with Tmeasure_SFTD1 specified in clause 9.3.8 when no measurement gaps are provided.

The requirements in this clause apply provided that

- 	There are only SCCs in FR2, or

- 	The SMTC on all CCs in FR2 have the same offset, and one of following conditions is met

-	If smtc2 is configured on any FR2 CC,

-	All CCs have the same configuration for smtc1, and

-	All CCs configured with smtc2 have the same configuration for smtc2

-	If smtc2 is not configured on any FR2 CC,

-	The total number of different SMTC periodicities on all serving CCs does not exceed 4

Note:	Longer delays for cell identification and measurement periods derived based on CSSFoutside_gap,i in clauses 9.2.5.1, 9.2.5.2, can be expected, if the UE is configured with more than 4 different SMTC periodicities on FR2 serving carriers. The longer delay applies for the FR2 intra-frequency measurement objects with the longest SMTC periodicity/periodicities.

9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps

For UE configured with the E-UTRA-NR dual connectivity operation, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency and inter-frequency SSB-based measurements performed outside measurements gaps will be as specified in Table 9.1.5.1.1-1.

Table 9.1.5.1.1-1: CSSFoutside_gap,i scaling factor for EN-DC mode

| Scenario | CSSFoutside_gap,i for FR1 PSCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PSCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required Note 2 | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gp |
| --- | --- | --- | --- | --- | --- | --- |
| EN-DC with FR1 only CA | 1 | Number of configured FR1 SCell(s)+Y | N/A | N/A | N/A | Number of configured FR1 SCell(s)+Y |
| EN-DC with FR2 only intra band CA | N/A | N/A | 1 | N/A | Number of configured FR2 SCells+Y | Number of configured FR2 SCells+Y |
| EN-DC with FR2 only inter band CA | N/A | N/A | 1 | 2 Note 3,5 | 2×(Number of configured SCell(s) +Y–1) | 2×(Number of configured SCell(s) +Y–1) |
| EN-DC with FR1 +FR2 CA (FR1 PSCell) Note 1 | 1 | 2×(Number of configured SCell(s)+Y-1) | N/A | 2 Note 3 | 2×(Number of configured SCell(s)+Y-1) | 2×(Number of configured SCell(s)+Y-1) |
| EN-DC with FR1 +FR2 CA (FR2 PSCell) Note 1 | N/A | Number of configured SCell(s)+Y | 1 | N/A | Number of configured SCell(s)+Y | Number of configured SCell(s)+Y |
| Note 1: Only one NR FR1 operating band and one NR FR2 operating band are included for FR1+FR2 inter-band EN-DC. Note 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2. Note 3: CSSFoutside_gap,i =1 if only one SCell is configured and no inter-frequency MO without gap. Note 4: Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG. Note 5: Only two NR FR2 operating band are included for EN-DC with FR2 only inter-band CA. |  |  |  |  |  |  |

9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps

For UE in SA operation mode, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency SSB-based measurements performed outside measurements gaps will be as specified in Table 9.1.5.1.2-1, which shall also be applied for a UE configured with NE-DC operation.

Table 9.1.5.1.2-1: CSSFoutside_gap,i scaling factor for SA mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSF outside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 only CA | 1 | Number of configured FR1 SCell(s)+Y | N/A | N/A | N/A | Number of configured FR1 SCell(s)+Y |
| FR2 only intra band CA | N/A | N/A | 1 | N/A | Number of configured FR2 SCell(s)+Y | Number of configured FR2 SCell(s)+Y |
| FR2 only inter band CA | N/A | N/A | 1 | 2 Note 3,5 | 2×(Number of configured SCell(s)+Y –1) | 2×(Number of configured SCell(s)+Y –1) |
| FR1 +FR2 CA (FR1 PCell) Note 1 | 1 | 2×(Number of configured SCell(s)+Y-1) | N/A | 2 Note 3 | 2×(Number of configured SCell(s) +Y -1) | 2×(Number of configured SCell(s)+Y-1) |
| Note 1: Only one FR1 operating band and one FR2 operating band are included for FR1+FR2 inter-band CA. Note 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2. Note 3: CSSFoutside_gap,i =1 if only one SCell is configured and no inter-frequency MO without gap. Note 4: Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG. Note 5: Only two NR FR2 operating bands are included for FR2 inter-band CA. |  |  |  |  |  |  |

9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps

For UE configured with NR-DC operation, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency and inter-frequency SSB-based measurements performed outside measurements gaps will be as specified in Table 9.1.5.1.3-1.

Table 9.1.5.1.3-1: CSSFoutside_gap,i scaling factor for NR-DC mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PSCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap |
| --- | --- | --- | --- | --- | --- |
| FR1 + FR2 NR-DC (FR1 PCell and FR2 PScell) Note 1 | 1 | 2×(Number of configured SCell(s)+Y) | 2 Note 2 | 2×(Number of configured SCell(s)+Y) | 2×(Number of configured SCell(s)+Y) |
| Note 1: NR-DC in Rel-15 only includes the scenarios where all serving cells in MCG in FR1 and all serving cells in SCG in FR2. Note 2: CSSFoutside_gap,i =1 if no SCell is configured and no inter-frequency MO without gap. Note 3: Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG |  |  |  |  |  |

9.1.5.1.4	NE-DC mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps

For UE configured with NE-DC operation, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency SSB-based measurements performed outside measurements gaps will be as specified in Table 9.1.5.1.4-1.

Table 9.1.5.1.4-1: CSSFoutside_gap,i scaling factor for NE-DC mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap |
| --- | --- | --- | --- | --- | --- | --- |
| NE-DC with FR1 only CA | 1 | Number of configured FR1 SCell(s)+Y | N/A | N/A | N/A | Number of configured FR1 SCell(s)+Y |
| NE-DC with FR2 only intra band CA | N/A | N/A | 1 | N/A | Number of configured FR2 SCell(s)+Y | Number of configured FR2 SCell(s)+Y |
| NE-DC with FR2 only inter band CA | N/A | N/A | 1 | 2 Note 3,5 | 2×(Number of configured SCell(s) +Y–1) | 2×(Number of configured SCell(s) +Y–1) |
| NE-DC with FR1 +FR2 CA (FR1 PCell) Note 1 | 1 | 2×(Number of configured SCell(s)+Y-1) | N/A | 2 Note 3 | 2×(Number of configured SCell(s)+Y-1) | 2×(Number of configured SCell(s)+Y-1) |
| Note 1: Only one FR1 operating band and one FR2 operating band are included for FR1+FR2 inter-band CA. Note 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2. Note 3: CSSFoutside_gap,i =1 if only one SCell is configured and no inter-frequency MO without gap. Note 4: Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG. Note 5: Only two NR FR2 operating band are included for NE-DC with FR2 only inter-band CA. |  |  |  |  |  |  |

9.1.5.2	Monitoring of multiple layers within gaps

The carrier-specific scaling factor CSSFwithin_gap,i for a measurement object i derived in this chapter is applied to following measurement types:

-	Intra-frequency measurement object with no measurement gap in clause 9.2.5, when all of the SMTC occasions of this intra-frequency measurement object are overlapped by the measurement gap.

-	Intra-frequency measurement object with measurement gap in clause 9.2.6.

-	Inter-frequency measurement with no measurement gap in clause 9.3.9, when all of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap, if UE supports interFrequencyMeas-NoGap-r16.

-	Inter-frequency measurement object with measurement gap in clause 9.3.4.

-	E-UTRA Inter-RAT measurement object in clauses 9.4.2 and 9.4.3.

-	E-UTRA Inter-RAT RSTD and E-CID measurements in clauses 9.4.4 and 9.4.5.

-	NR Inter-RAT measurement object configured by the E-UTRAN PCell (TS 36.133 [15] clause 8.17.4).

-	E-UTRAN Inter-frequency measurement object configured by the E-UTRAN PCell (TS 36.133 [15] clause 8.17.3) and by the E-UTRAN PSCell (TS 36.133 [15] clause 8.19.3).

-	E-UTRAN Inter-frequency RSTD measurement configured by the E-UTRAN PCell (TS 36.133 [15] clause 8.17.15).

-	UTRA Inter-RAT measurement object configured by the E-UTRAN PCell (TS 36.133 [15] clauses 8.17.5 to 8.17.12).

-	GSM Inter-RAT measurements configured by the E-UTRAN PCell (TS 36.133 [15] clauses 8.17.13 and 8.17.14).

UE is expected to conduct the measurement of this measurement object i only within the measurement gaps.

If the higher layer signaling in TS 38.331 [2] of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, CSSFwithin_gap,i and requirements derived from CSSFoutside_gap,i are not specified.

9.1.5.2.1	EN-DC mode: carrier-specific scaling factor for SSB-based measurements performed within gaps

The scaling value CSSFwithin_gap,i below has been derived without considering GSM inter-RAT carriers.

When one or more measurement objects are monitored within measurement gaps, the carrier specific scaling factor for a target measurement object with index i is designated as CSSFwithin_gap,i and is derived as described in this clause.

If measurement object i refers to an RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured, CSSFwithin_gap,i=1. Otherwise, the CSSFwithin_gap,i for other measurement objects (including RSTD measurement with periodicity Tprs=160ms) participate in the gap competition are derived as below.

For each measurement gap j not used for an RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured within an arbitrary 160ms period, count the total number of intra-frequency measurement objects and inter-frequency/inter-RAT measurement objects which are candidates to be measured within the gap j.

-	An NR measurement object is a candidate to be measured in a gap if its SMTC duration is fully covered by the MGL excluding RF switching time. For intra-frequency NR carriers, if the higher layer in TS 38.331 [2] signaling of smtc2 is configured, the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc2; otherwise the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc1.

-	An inter-RAT measurement object configured by PSCell is a candidate to be measured in all measurement gaps.

-	An inter-RAT UTRA measurement object configured by E-UTRA PCell [15] is a candidate to be measured in all measurement gaps.

-	An inter-frequency E-UTRA measurement object is a candidate to be measured in all measurement gaps.

-	For UEs which support and are configured with per FR gaps, the counting is done on a per FR basis, and for UEs which are configured with per UE gaps the counting is done on a per UE basis.

-	Mintra,i,j: Number of intra-frequency measurement objects which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mintra,i,j  equals 0.

-	Minter,i,j : Number of NR inter-frequency measurement objects or NR inter-RAT measurement objects configured by E-UTRA PCell, EUTRA inter-frequency measurement objects configured by E-UTRA PCell, UTRA inter-RAT measurement objects configured by E-UTRA PCell which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Minter,i,j  equals 0.

-	Mtot,i,j = Mintra,i,j + Minter,i,j : Total number of intra-frequency, inter-frequency and inter-RAT measurement objects which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mtot,i,j equals 0.

For each measurement gap j used for an RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured within an arbitrary 160ms period, Mintra,i,j = Minter,i,j = Mtot,i,j =0.

The carrier specific scaling factor CSSFwithin_gap,i is given by:

If measGapSharingScheme is equal sharing, CSSFwithin_gap,i= max(ceil(Ri×Mtot,i,j)), where j=0…(160/MGRP)-1

If measGapSharingScheme is not equal sharing and

-	measurement object i is an intra-frequency measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kintra×Mintra,i,j) in gaps where Minter,i,j≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×Mintra,i,j) in gaps where Minter,i,j=0, where j=0…(160/MGRP)-1

-	measurement object i is an inter-frequency or inter-RAT measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kinter×Minter,i,j) in gaps where Mintra,i,j ≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×Minter,i,j) in gaps where Mintra,i,j=0, where j=0…(160/MGRP)-1

Where Ri is the maximal ratio of the number of measurement gap where measurement object i is a candidate to be measured over the number of measurement gap where measurement object i is a candidate and not used for RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured within an arbitrary 1280ms period.

9.1.5.2.2	SA mode: carrier-specific scaling factor for SSB-based measurements performed within gaps

When one or more measurement objects are monitored within measurement gaps, the carrier specific scaling factor for a target measurement object with index i is designated as CSSFwithin_gap,i and is derived as described in this clause.

If measurement object i refers to an RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured, CSSFwithin_gap,i=1. Otherwise, the CSSFwithin_gap,i for other measurement objects (including RSTD measurement with periodicity Tprs=160ms) participate in the gap competition and the CSSFwithin_gap,i are derived as below.

For each measurement gap j not used for an RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured within an arbitrary 160ms period, count the total number of intra-frequency measurement objects and inter-frequency/inter-RAT measurement objects which are candidates to be measured within the gap j.

-	An NR measurement object is a candidate to be measured in a gap if its SMTC duration is fully covered by the MGL excluding RF switching time. For intra-frequency NR measurement objects, if the higher layer in TS 38.331 [2] signaling of smtc2 is configured, the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc2; otherwise the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc1.

-	An inter-RAT measurement object is a candidate to be measured in all meausrement gaps.

-	An inter-frequency SFTD measurement object is a candidate to be measured in all measurement gaps.

-	For UEs which support and are configured with per FR gaps, the counting is done on a per FR basis, and for UEs which are configured with per UE gaps the counting is done on a per UE basis.

-	Mintra,i,j: Number of intra-frequency measurement objects which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mintra,i,j  equals 0.

-	Minter,i,j : Number of NR inter-frequency, EUTRA inter-RAT and UTRA inter-RAT objects which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Minter,i,j  equals 0.

-	Mtot,i,j = Mintra,i,j + Minter,i,j : Total number of intra-frequency, inter-frequency and inter-RAT measurement objects which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mtot,i,j equals 0.

For each measurement gap j used for an RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured within an arbitrary 160ms period, Mintra,i,j = Minter,i,j = Mtot,i,j =0.

The carrier specific scaling factor CSSFwithin_gap,i is given by:

If measGapSharingScheme is equal sharing, CSSFwithin_gap,i= max(ceil(Ri×Mtot,i,j)), where j=0…(160/MGRP)-1

If measGapSharingScheme is not equal sharing and

-	measurement object i is an intra-frequency measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kintra×Mintra,i,j) in gaps where Minter,i,j≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×Mintra,i,j) in gaps where Minter,i,j=0, where j=0…(160/MGRP)-1

-	measurement object i is an inter-frequency or inter-RAT measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kinter×Minter,i,j) in gaps where Mintra,i,j ≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×Minter,i,j) in gaps where Mintra,i,j=0, where j=0…(160/MGRP)-1

Where Ri is the maximal ratio of the number of measurement gap where measurement object i is a candidate to be measured over the number of measurement gap where measurement object i is a candidate and not used for RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured within an arbitrary 1280ms period.

CSSFwithin_gap,k=1 during TDetect, E-UTRAN FDD specified in clause 9.4.4.1.2.2 and TDetect, E-UTRAN TDD specified in clause 9.4.4.2.2.2, where k is the carrier frequency where the UE is performing cell detection of the inter-RAT E-UTRA OTDOA assistance data reference cell when acquiring the subframe and slot timing of the cell according to clause 9.4.4. In this case, the UE cell identification and measurement periods derived based on CSSFwithin_gap,i in clauses 9.2.5.1, 9.2.5.2, 9.2.6.2, 9.2.6.3, 9.3.4, 9.3.5, 9.4.2.2, and 9.4.2.3 may be extended for measurement objects of which the cell identification and measurement periods are overlapped with TDetect, E-UTRAN FDD and TDetect, E-UTRAN TDD.

9.1.5.2.3	NE-DC: carrier-specific scaling factor for SSB-based measurements performed within gaps

When one or more measurement objects are monitored within measurement gaps, the carrier specific scaling factor for a target measurement object with index i is designated as CSSFwithin_gap,i and is derived as described in this clause.

If measurement object i refers to an RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured, CSSFwithin_gap,i=1. Otherwise, the CSSFwithin_gap,i for other measurement objects (including RSTD measurement with periodicity Tprs=160ms) participate in the gap competition are derived as below.

For each measurement gap j not used for an RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured within an arbitrary 160ms period, count the total number of intra-frequency measurement objects and inter-frequency/inter-RAT measurement objects which are candidates to be measured within the gap j.

-	An NR measurement object is a candidate to be measured in a gap if its SMTC duration is fully covered by the MGL excluding RF switching time. For intra-frequency NR measurement objects, if the higher layer in TS 38.331 [2] signaling of smtc2 is configured, the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc2; otherwise the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc1.

-	An inter-RAT measurement object is a candidate to be measured in all meausrement gaps.

-	An inter-frequency E-UTRA measurement object is a candidate to be measured in all measurement gaps.

For UEs which support and are configured with per FR gaps, the counting is done on a per FR basis, and for UEs which are configured with per UE gaps the counting is done on a per UE basis.

If the number of configured interfrequency and interRAT measuerement objects is non-zero and the UE is configured with per UE gaps, or if the UE is configured with per FR gaps:

FR1 and FR2 intrafrequency measurement objects belong to group A

Interfrequency and interRAT measurement objects belong to group B

MgroupA,i,j: Sum of the number of FR1 intra-frequency measurement objects Mintra-FR1,i,j and the number of FR2 intra-frequency measurement objects Mintra-FR2,i,j which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupA,i,j  equals 0.

MgroupBi,j: Number of NR inter-frequency, EUTRA inter-RAT and UTRA inter-RAT measurement objects which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupB,i,j  equals 0.

If the number of configured inter-frequency and inter-RAT measuerement objects is zero and the UE is configured with per UE gaps:

FR1 intrafrequency measurement objects belong to group A

FR2 intrafrequency measurement objects belong to group B

MgroupA,i,j: The number of FR1 intrafrequency measurement objects Mintra-FR1,i,j which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupA,i,j  equals 0.

MgroupBi,j : The number of FR2 intrafrequency measurement objects Mintra-FR2,i,j which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupB,i,j  equals 0.

Mtot,i,j = MgroupA,i,j + MgroupB,i,j : Total number of group A and group B measurement objects which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mtot,i,j equals 0.

For each measurement gap j used for an RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured within an arbitrary 160ms period, Mintra,i,j = Minter,i,j = Mtot,i,j =0.

The carrier specific scaling factor CSSFwithin_gap,i is given by:

If measGapSharingScheme is equal sharing, CSSFwithin_gap,i= max(ceil(Ri×Mtot,i,j)), where j=0…(160/MGRP)-1

If measGapSharingScheme is not equal sharing and

-	measurement object i is a group A measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kintra×MgroupA,i,j) in gaps where MgroupB,i,j≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×MgroupA,i,j) in gaps where MgroupB,i,j=0, where j=0…(160/MGRP)-1

-	measurement object i is an group B measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kinter×MgroupBi,j) in gaps where MgroupA,i,j ≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×MgroupB,i,j) in gaps where MgroupA,i,j=0, where j=0…(160/MGRP)-1

Where Ri is the maximal ratio of the number of measurement gap where measurement object i is a candidate to be measured over the number of measurement gap where measurement object i is a candidate and not used for RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured within an arbitrary 1280ms period.

9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based measurements performed within gaps

When one or more measurement objects are monitored within measurement gaps, the carrier specific scaling factor for a target measurement object with index i is designated as CSSFwithin_gap,i and is derived as described in this clause.

If measurement object i refers to an RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured, CSSFwithin_gap,i=1. Otherwise, the CSSFwithin_gap,i for other measurement objects (including RSTD measurement with periodicity Tprs=160ms) participate in the gap competition and the CSSFwithin_gap,i are derived as below.

For each measurement gap j not used for an RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured within an arbitrary 160ms period, count the total number of intra-frequency measurement objects and inter-frequency/inter-RAT measurement objects which are candidates to be measured within the gap j.

-	An NR measurement object is a candidate to be measured in a gap if its SMTC duration is fully covered by the MGL excluding RF switching time. For intra-frequency NR measurement objects, if the higher layer in TS 38.331 [2] signaling of smtc2 is configured, the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc2; otherwise the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc1.

-	An inter-RAT measurement object is a candidate to be measured in all meausrement gaps.

For UEs which support and are configured with per FR gaps, the counting is done on a per FR basis, and for UEs which are configured with per UE gaps the counting is done on a per UE basis.

If the number of configured interfrequency and interRAT measuerement objects is non-zero and the UE is configured with per UE gaps, or if the UE is configured with per FR gaps:

FR1 and FR2 intrafrequency measurement objects belong to group A

Interfrequency and interRAT measurement objects belong to group B

MgroupA,i,j: Sum of the number of FR1 intra-frequency measurement objects Mintra-FR1,i,j and the number of FR2 intra-frequency measurement objects Mintra-FR2,i,j which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupA,i,j  equals 0.

MgroupBi,j : Number of NR inter-frequency, EUTRA inter-RAT and UTRA inter-RAT measurement objects which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupB,i,j  equals 0.

If the number of configured interfrequency and interRAT measuerement objects is zero and the UE is configured with per UE gaps:

FR1 intrafrequency measurement objects belong to group A

FR2 intrafrequency measurement objects belong to group B

MgroupA,i,j: The number of FR1 intrafrequency measurement objects Mintra-FR1,i,j which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupA,i,j  equals 0.

MgroupBi,j : The number of FR2 intrafrequency measurement objects Mintra-FR2,i,j which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupB,i,j  equals 0.

Mtot,i,j = MgroupA,i,j + MgroupB,i,j : Total number of group A and group B measurement objects which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mtot,i,j equals 0.

For each measurement gap j used for an RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured within an arbitrary 160ms period, Mintra,i,j = Minter,i,j = Mtot,i,j =0.

The carrier specific scaling factor CSSFwithin_gap,i is given by:

If measGapSharingScheme is equal sharing, CSSFwithin_gap,i= max(ceil(Ri×Mtot,i,j)), where j=0…(160/MGRP)-1

If measGapSharingScheme is not equal sharing and

-	measurement object i is a group A measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kintra×MgroupA,i,j) in gaps where MgroupB,i,j≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×MgroupA,i,j) in gaps where MgroupB,i,j=0, where j=0…(160/MGRP)-1

-	measurement object i is an group B measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kinter×MgroupBi,j) in gaps where MgroupA,i,j ≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×MgroupB,i,j) in gaps where MgroupA,i,j=0, where j=0…(160/MGRP)-1

Ri is the maximal ratio of the number of measurement gap where measurement object i is a candidate to be measured over the number of measurement gap where measurement object i is a candidate and not used for RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured within an arbitrary 1280ms period.

<End of Change 1>

<Start of Change 2>

9.3.9	Inter frequency measurements without measurement gaps

9.3.9.1	Inter frequency Cell identification

If UE supports interFrequencyMeas-NoGap-r16 and the flag interFrequencyConfig-NoGap-r16 is configured by the Network, UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured). Otherwise UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_with_index. The UE shall be able to identify a new detectable inter frequency SS block of an already detected cell within Tidentify_inter_without_index. It is assumed that when UE performs inter-frequency measurements without measurement gaps in a TDD bands on FR1 and FR2, the following conditions are met:

-	SFN and frame boundary across serving cell and inter-frequency neighbor cells is aligned, and

-	the timing of SSBs across serving cell and inter-frequency neighbor cells are aligned

Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter) ms

Tidentify_inter_with_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter + TSSB_time_index_inter) ms

Where:

TPSS/SSS_sync_inter: it is the time period used in PSS/SSS detection given in table 9.3.4-1 and table 9.3.4-2.

TSSB_time_index_inter: it is the time period used to acquire the index of the SSB being measured given in table 9.3.4-3.

T SSB_measurement_period_inter: equal to a measurement period of SSB based measurement given in table 9.3.9-1 and table 9.3.9-2.

CSSFinter: it is a carrier specific scaling factor and is determined according to CSSFoutside_gap,i in clause 9.1.5.1 for measurement conducted outside measurement gaps, i.e. when interfrequency SMTC is fully non overlapping or partially overlapping with measurement gaps or according to CSSFwithin_gap,i in clause 9.1.5.2 for measurement conducted within measurement gaps, i.e. when interfrequency SMTC is fully overlapping with measurement gaps.

Mpss/sss_sync_inter: For a UE supporting FR2 power class 1, Mpss/sss_sync_inter = 40 samples. For a UE supporting FR2 power class 2, Mpss/sss_sync_inter = 24 samples. For a UE supporting FR2 power class 3, Mpss/sss_sync_inter = 24 samples. For a UE supporting FR2 power class 4, Mpss/sss_sync = 24 samples.

MSSB_index_inter: For a UE supporting power class 1, MSSB_index_inter = 40 samples. For a vehicle mounted UE supporting power class 2, Mpss/sss_sync_inter = 24 samples. For a UE supporting power class 3, MSSB_index_inter = 24 samples. For a UE supporting power class 4, Mmeas_period_inter = 24 samples.

Mmeas_period_inter: For a UE supporting FR2 power class 1, Mmeas_period_inter =40 samples. For a vehicle mounted UE supporting FR2 power class 2, Mpss/sss_sync_inter=24 samples. For a UE supporting FR2 power class 3, Mmeas_period_inter =24 samples. For a UE supporting FR2 power class 4, Mmeas_period_inter = 24 samples.

When interfrequency SMTC is fully non overlapping with measurement gaps or interfrequency SMTC is fully overlapping with MGs, Kp=1.

When interfrequency SMTC is partially overlapping with measurement gaps, Kp =  1/(1- (SMTC period /MGRP)), where SMTC period < MGRP.

For FR2,

Klayer1_measurement=1,

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by SSB-ToMeasure and RSSI symbols are indicated by SS-RSSI-Measurement;

Klayer1_measurement=1.5, otherwise.

Table 9.3.4-1: Time period for PSS/SSS detection, (FR1)

| DRX cycle | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | max( 600ms, ceil( 5 x Kp) x SMTC period )Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max( 600ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | ceil(5 x Kp) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.3.4-2: Time period for PSS/SSS detection, (FR2)

| DRX cycle | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | max(600ms, ceil(Mpss/sss_sync_inter x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max(600ms, ceil(1.5 x Mpss/sss_sync_inter x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | ceil(Mpss/sss_sync_inter x Kp x Klayer1_measurement) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.3.4-3: Time period for time index detection (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max(120ms, ceil (1.5 x 3 x Kp) x max(SMTC period,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | Ceil(3 x Kp) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

9.3.9.2	Measurement period

The UE physical layer shall be capable of reporting SS-RSRP, SS-RSRQ and SS-SINR measurements to higher layers with measurement accuracy as specified in sub-clauses 10.1.4, 10.1.5, 10.1.9, 10.1.10, 10.1.14 and 10.1.15, respectively, as shown in table 9.3.9-1 and 9.3.9-2, if UE supports inter-frequency measurement without measurement gaps:

Table 9.3.9-1: Measurement period for inter-frequency measurements with gaps (FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | ceil( 5 x Kp ) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.3.9-2: Measurement period for inter-frequency measurements with gaps (FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400ms, ceil(Mmeas_period_inter x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5x Mmeas_period_inter x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | ceil(Mmeas_period_inter xKp x Klayer1_measurement) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

9.3.9.3	Scheduling availability of UE during inter-frequency measurements

If UE supports interFrequencyMeas-NoGap-r16 and the flag interFrequencyConfig-NoGap-r16 is configured by the Network, UE is required to be capable of measuring without measurement gaps when the SSB is completely contained in the active bandwidth part of the UE. When any of the conditions in the following clauses is met, there are restrictions on the scheduling availability; otherwise, there is no scheduling restriction.  Note that the SSB symbols to be measured in the following clauses are the SSB symbols indicated by SSB-ToMeasure [2], if it is configured; otherwise, all L SSB symbols within the SMTC window duration defined in clause 4.1 of TS 38.213 [3] are included.

The scheduling availability requirements when UE performs inter-frequency measurements without measurement gaps in a TDD bands on FR1 and FR2 in clause 9.3.9.3.1~9.3.9.3.3 are valid under the following conditions:

-	SFN and frame boundary across serving cell and inter-frequency neighbor cells is aligned, and

-	the timing of SSBs across serving cell and inter-frequency neighbor cells are aligned

9.3.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1

When UE performs inter-frequency measurements without measurement gaps in a TDD band, the following restrictions apply due to SS-RSRP or SS-SINR measurement

-	UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration.

When UE performs inter-frequency measurements without measurement gaps in a TDD band, the following restrictions apply due to SS-RSRQ measurement

-	UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, RSSI measurement symbols, and on 1 data symbol before each consecutive SSB to be measured/RSSI symbols and 1 data symbol after each consecutive SSB to be measured/RSSI symbols within SMTC window duration.

When TDD intra-band carrier aggregation is performed, the scheduling restrictions due to one serving cell should also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

9.3.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1

For UE which do not support simultaneousRxDataSSB-DiffNumerology-Inter-r16 [14] the following restrictions apply due to SS-RSRP/RSRQ/SINR measurement

-	If UE performs inter-frequency measurements without measurement gaps in a TDD band, UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration.

-	If UE performs inter-frequency measurements without measurement gaps in a FDD band, UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on all symbols within SMTC window duration.

When intra-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell should also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

9.3.9.3.3	Scheduling availability of UE performing measurements on FR2

The following scheduling restriction applies due to SS-RSRP or SS-SINR measurement on an FR2 inter-frequency cell

The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration.

The following scheduling restriction applies to SS-RSRQ measurement on an FR2 inter-frequency cell

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured, RSSI measurement symbols, and on 1 data symbol before each consecutive SSB to be measured/RSSI symbols and 1 data symbol after each consecutive SSB to be measured/RSSI symbols within SMTC window duration.

When intra-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell should also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

If following conditions are met:

-	The UE has been notified about system information update through paging,

-	The gap between the UE’s reception of PDCCH that UE monitors in the Type 2-PDCCH CSS set that notifies system information update, and the PDCCH that UE monitors in the Type0-PDCCH CSS set, is greater than 2

For the SSB and CORESET for RMSI scheduling multiplexing patterns 3, the UE is expected to receive the PDCCH that the UE monitors in the Type0-PDCCH CSS set, and the corresponding PDSCH, on SSB symbols to be measured; and

For the SSB and CORESET for RMSI scheduling multiplexing patterns 2, the UE is expected to receive PDSCH that corresponds to the PDCCH that the UE monitors in the Type0-PDCCH CSS set, on SSB symbols to be measured.

9.3.9.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA

There are no scheduling restrictions on FR1 serving cell(s) due to measurements performed on FR2 serving cell frequency layer.

There are no scheduling restrictions on FR2 serving cell(s) due to measurements performed on FR1 serving cell frequency layer.

<End of Change 2>