3GPP TSG-RAN WG4 Meeting # 96-e	R4-2012171

# Electronic Meeting, 17 Aug. – 28 Aug., 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0996 | rev | 1 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on introduction, applicability and capability for CSI-RS inter-frequency measurement requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CSIRS_L3meas-Core |  |  |  |  |  | Date: |  |  | 2020-08-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduce the defintion of inter-frequency measurement for L3 CSI-RS based measurements, and specify requirement applicability and number of cells/beams to be measured for inter-frequency measurement |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture the agreements on CSI-RS inter-frequency measurement in RAN4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CSI-RS based inter-frequency measurement is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.x.y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of change 1>>

9.x.y	CSI-RS based Inter-frequency measurements

9.x.y.1	Introduction

A measurement is defined as an CSI-RS based inter-frequency measurement provided it is not defined as an intra-frequency measurement according to clause 9.x.z.

If a UE is configured with the higher layer parameter CSI-RS-Resource-Mobility and the higher layer parameter associatedSSB is configured, the UE shall be able to identify inter-frequency cells indicated for measurment and perform CSI-RSRP, CSI-RSRQ, and CSI-SINR measurements of identified inter-frequency cells.

When measurement gaps are needed, the UE is not expected to detect the associated SSB on an inter-frequency measurement object which start earlier than the gap starting time + switching time, nor perform measurement of the CSI-RS resource configured in CSI-RS-Resource-Mobility which ends later than the gap end – switching time. When the inter-frequency cells are in FR2 and the per-FR gap is configured to the UE in EN-DC, SA NR, NE-DC and NR-DC, or the serving cells are in FR2, the inter-frequency cells are in FR2 and the per-UE gap is configured to the UE in SA NR and NR-DC, the switching time is 0.25ms. Otherwise the switching time is 0.5ms.

9.x.y.2	Requirements applicability

Unless otherwise stated in the clause 9.x.y, if the associated SSB of the CSI-RS is counted as one seperate SSB layer in addtion to the SSB layers for SSB based measurements, as specified in subclause 9.1.3, the requirements for this associated SSB layer follows the same requirements as SSB based measurements defined in 9.3.

The requirements in clause 9.x.y apply, provided:

-	all CSI-RS resources in the same MO is configured with the same csi-rs-MeasurementBW.

-	The associated SSB of the cell being identified or measured is detectable. The associated SSB shall be considered detectable when it meets the side condition of SSB_RP and SSB Ês/Iot according to Annex B.2.3 for a corresponding Band.

-	The CSI-RS for measurement is QCL-ed to the associated SSB for FR2.

-    CSI-RS resources for measurements and the associated SSB for cell identification are configured within measurement gap.

-	CSI-RSRP related side conditions given in clauses 10.1.x and 10.1.x for FR1 and FR2, respectively, for a corresponding Band,

-	CSI-RSRQ related side conditions given in clauses 10.1.x and 10.1.x for FR1 and FR2, respectively, for a corresponding Band,

-	CSI-SINR related side conditions given in clauses 10.1.x and 10.1.x for FR1 and FR2, respectively, for a corresponding Band,

-	CSI-RS_RP and CSI-RS Ês/Iot according to Annex B.2.x for a corresponding Band.

9.x.y.3	Number of cells and number of CSI-RS resources

9.x.y.3.1	Requirements for FR1

For each inter-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing CSI-RSRP, CSI-RSRQ, and CSI-SINR measurements for at least:

-	[24/14] CSI-RSs with different CSI-RS index and/or PCI on the inter-frequency layer.

-    The cells to be monitored based on CSI-RS can be the same set or a subset of the cells monitored based on the [layer of the associated] SSB.

9.x.y.3.2	Requirements for FR2

For each inter-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing CSI-RSRP, CSI-RSRQ, and CSI-SINR measurements for at least:

-    24 CSI-RSs with different CSI-RS index and/or PCI on the inter-frequency layer.

-   The cells to be monitored based on CSI-RS can be the same set or a subset of the cells monitored based on the [layer the associated] SSB.

<< End of change 1>>