3GPP TSG-RAN WG4 Meeting # 96-e 												R4-2012204

Electronic Meeting, 17 – 28 August., 2020

Agenda item:			5.3, (6.5.3)

Source:	Moderator (Ericsson.)

Title:						Email discussion summary for [96e][203] LTE_RRM_maintenance

Document for:	Information

# Introduction

This document is the email discussion summary for [96e][203] LTE_RRM_maintenance with the following topics covered

- Topic 1:	euCA maintenance

- Topic 2:	CE Mode A maintenance

- Topic 3:	NB-IoT maintenance

No document was submitted under AI 6.5.3 (LTE R16 maintenance - RRM)

Documents to be treated

| R4-2009534 | CR | LTE_MTCe2_L1-Perf | Agreement | Rel-13 | Correction to intra-frequency event triggered reporting test case in CEModeA | ANRITSU LTD |
| --- | --- | --- | --- | --- | --- | --- |
| R4-2009535 | CR | LTE_MTCe2_L1-Perf | Agreement | Rel-14 | Correction to intra-frequency event triggered reporting test case in CEModeA | ANRITSU LTD |
| R4-2009536 | CR | LTE_MTCe2_L1-Perf | Agreement | Rel-15 | Correction to intra-frequency event triggered reporting test case in CEModeA | ANRITSU LTD |
| R4-2009537 | CR | LTE_MTCe2_L1-Perf | Agreement | Rel-16 | Correction to intra-frequency event triggered reporting test case in CEModeA | ANRITSU LTD |
| R4-2009891 | CR | TEI | Agreement | Rel-15 | CR on TS36.133 for measurement capability of IDLE mode CA measurement | MediaTek inc. |
| R4-2009892 | CR | TEI | Agreement | Rel-16 | CR on TS36.133 for measurement capability of IDLE mode CA measurement | MediaTek inc. |
| R4-2010561 | discussion | LTE_euCA-Core | Approval | Rel-15 | Search threshold applicability in euCA | Nokia, Nokia Shanghai Bell |
| R4-2010562 | CR | LTE_euCA-Core | Approval | Rel-15 | CR clarifying S-measure thresholds for EMR carriers | Nokia, Nokia Shanghai Bell |
| R4-2010563 | CR | LTE_euCA-Core | Approval | Rel-16 | CR clarifying S-measure thresholds for EMR carriers | Nokia, Nokia Shanghai Bell |
| R4-2010564 | discussion | LTE_euCA-Core | Approval | Rel-15 | Measurement Performance Requirements test for euCA | Nokia, Nokia Shanghai Bell |
| R4-2010565 | CR | LTE_euCA-Core | Approval | Rel-15 | CR on performance requirements tests for euCA | Nokia, Nokia Shanghai Bell |
| R4-2010566 | CR | LTE_euCA-Core | Approval | Rel-16 | CR on performance requirements tests for euCA | Nokia, Nokia Shanghai Bell |
| R4-2011097 | CR | NB_IOTenh2-Perf | Agreement | Rel-15 | CR on NB-IoT Intra frequency with serving cell measurement relaxation test case 4.2.38 R15 | Huawei, Hisilicon |
| R4-2011098 | CR | NB_IOTenh2-Perf | Agreement | Rel-16 | CR on NB-IoT Intra frequency with serving cell measurement relaxation test case 4.2.38 R16 (Cat A) | Huawei, Hisilicon |

# Topic #1: euCA maintenance

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| Document | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2009891 | MediaTek inc. | CR : Add “In addition to the requirements defined in section 4.2.2.9 and 4.2.2.9a,” in section 4.9.2.2 to clarify that measurement capabilities in section 4.2.2.9, 4.2.2.9a and section 4.9.2.2 should be simultaneously followed. |
| R4-2009892 | MediaTek inc. | Shadow CR |
| R4-2010561 | Nokia, Nokia Shanghai Bell | Proposal 1: Clarify in 36.133 that the fields s-NonIntraSearch in SystemInformationBlockType3 do not affect measurements of inter-frequency CA candidate cells. |
| R4-2010562 | Nokia, Nokia Shanghai Bell | CR : Implement proposal in R4-2010561 |
| R4-2010563 | Nokia, Nokia Shanghai Bell | Shadow CR |
| R4-2010564 | Nokia, Nokia Shanghai Bell | Proposal 1: Define measurement performance test cases for RSRP and RSRQ measurement accuracies for euCA. Proposal 2: Define intra-frequency measurement performance test cases for RSRP and RSRQ. Proposal 3: Define inter-frequency measurement performance test cases for RSRP and RSRQ. |
| R4-2010565 | Nokia, Nokia Shanghai Bell | CR : Implement proposals in R4-2010564 |
| R4-2010566 | Nokia, Nokia Shanghai Bell | Shadow CR |

Moderator: CRs are moved to Section 1.3.2

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1 Search threshold

Issue 1-1-1 : Whether s-NonIntraSearch affects measurement of interfrequency CA candidate cells

- Proposals

  - Option 1: NonIntraSearch in SystemInformationBlockType3 do not affect measurements of inter-frequency CA candidate cells (Nokia)

  - Option 2: Others

- Recommended WF:

  - Collect feedback on option 1 and decide how to proceed

### Sub-topic 1-2 Accuracy test cases

Issue 1-2-1 : Whether additional accuracy tests are specified for idle mode measurements in euCA

- Proposals

  - Option 1: Define accuracy tests

  - Option 2: Do not define accuracy tests

- Recommended WF:

Further discussion of the proposal needed

Issue 1-2-2 : Testing scope if accuracy tests are specified

- Proposal

  - Discuss test case list from the following candidate test case list proposed by the propoenent

    - Test 1 : Intra-frequency RSRP accuracy test for euCA

    - Test 2 : Intra-frequency RSRQ accuracy test for euCA

    - Test 3 : Inter-frequency RSRP accuracy test for euCA

    - Test 4 : Inter-frequency RSRQ accuracy test for euCA

- Recommended WF:

Depending on the outcome of issue 1-2-1 discuss the test case list

Issue 1-2-3 : CRs

- Recommended WF:

  - Provide feedback in first round for R4-2009891 in section 1.3.2

  - Provide feedback in first or second round for R4-2010562 and R4-2010565 once issue 1-2-1 and issue 1-2-2 are progressed.

## Companies views’ collection for 1st round

### Open issues

Issue 1-1-1 : Whether s-NonIntraSearch affects measurement of interfrequency CA candidate cells

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | Based on the RAN4 discussion during the development of the euCA feature it was agreed (in Chongqing meeting) to align the RAN4 euCA requirements with the RAN2 decision stating that the inter-frequency search thresholds (S-measure) are not applied for carriers configured for early measurement reporting. The CR was endorsed in [R4-1912845]. In the following meeting in Reno (RAN4#93), RAN4 did have discussion concerning whether RAN4 should clarify the RAN4 specification to clearly reflect the RAN2 agreement that the fields s-NonIntraSearch in SystemInformationBlockType3 do not affect measurements of inter-frequency CA candidate cells. However, there was no time to discuss and agree during the meeting. And then Rel-15 was not on the agenda in Athens meeting. For clarity we suggest including such clarification in the RAN4 specification. We support option 1. |
| Qualcomm | If this is agreeable, then the accuracy requirements in 9.1.3B should also remove references to these thresholds. |
| Ericsson | Support clarification of the RAN4 spec |
| Huawei | We do not agree with option 1. Similar as NR EMR discussion in Rel-16 MR-DC WI, we understand the measurement procedure is not affected by the threshold, but the measurement requirements is, and this is aligned with current spec after R4-1912845 which states that overlapping carrier follows existing reselection requirements which are depending on threshold. |

Issue 1-2-1 : Whether additional accuracy tests are specified for idle mode measurements in euCA

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | For Rel-15 LTE early measurement reporting scheme was standardized such that UE will perform the idle mode measurements in idle mode. UE shall only report the EMR measurement results for the cells which at least meet the configured threshold (qualityThreshold). Hence, RAN4 need to define the measurement performance test case to ensure that RSRP/Q measurement accuracy is within the specified limits. We support option 1. |
| Qualcomm | We think this was discussed in euCA work item. The accuracy of measurements in idle mode is already verified via cell reselection test cases. Why would new test cases be necessary? We prefer option 2. |
| Ericsson | Support the addition of accuracy tests, especially considering that UEs could use different measurement approach (number of samples, sampling rate etc) for non overlapping carriers compared to reselection measurements. If companies feel it is too late to add in R15 euCA then a TEI16 or TEI17 test could be considered |
| Huawei | If this has already been discussed before as QC mentioned, we agree that we should not re-open the discussion. Otherwise, we are open to define some accuracy test cases. For the timeline, we think maybe TEI16 or TEI17 should be considered as Ericsson mentioned. |

Issue 1-2-2 : Testing scope if accuracy tests are specified

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | The test case list proposes to introduce test cases for the measurements defined for idle mode reporting. This includes serving cell measurements and LTE inter-frequency measurements. The CR is initial draft and all numbers should be in []. |
| Ericsson | Since reselection testing implicitly tests intrafrequency measurements and interfrequency overlapping carrier measurements, we think it would be sufficient to have a single type of accuracy test for inter-frequency non overlapping carriers. In addition, we think there should be FDD and TDD variants of the test (CR currently looks to be for FDD). |
| Huawei | We share similar view as Ericsson if the test cases are to be defined. |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2009891 (Mediatek, TS36.133) | Nokia: It is not clear to us what ‘in addition to the requirements….’ means. During the release 15 discussion it was clear that the UE measurement capability as defined in section 4.2.2.9 remain unchanged for a UE supporting euCA. The change in this CR seems to indicate that for the UE measurement capability in section 4.2.2.9, the UE supporting euCA, shall additionally support idle mode CA measurements of at least 1 non-overlapping inter-frequency carrier and 1 overlapping inter-frequency carrier. Hence, it reads as if the measurement capability is increased. With this understanding, this CR is not aligned with Rel-15 agreement and not agreeable. |
|  | Huawei: We understand the motivation of the CR and support to make clarification. On the other hand, we think the issue (potential misunderstanding) pointed above by Nokia needs to be addressed with a better wording. |
| R4-2010562 (Nokia, TS36.133) | Huawei: This is pending on Issue 1-1-1. |
|  | Company B |
| R4-2010565 (Nokia, TS36.133) | Ericsson : See comments on 10564 |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Issue 1-1-1 : Whether s-NonIntraSearch affects measurement of interfrequency CA candidate cells Two companies support that NonIntraSearch does not affect measurements of inter-frequency CA candidate cells. One company supports that overlapping carrier follows existing reselection requirements which are depending on threshold. Corresponding discussion is ongoing in MR-DC for NR, although this is a release 16 work item which should in principle not affect rel15 euCA. Tentative agreements: Further discussion needed in 2nd round Candidate options: Option 1 : sNonIntraSearch does not affect measurement of interfrequency CA candidate cells Option 2 : Overlapping interfrequency carriers are not measured when serving cell is above sNonIntraSearch Option 3 : Others Issue 1-2-1 : Whether additional accuracy tests are specified for idle mode measurements in euCA Issue 1-2-2 : Testing scope if accuracy tests are specified Two companies supported the addition of accuracy tests, one company thought it was discussed previously and reselection tests implicitly verify the accuracy Tentative agreements For discussion in 2nd round : Is it agreeable to introduce only the tests for inter-frequency non-overlapping carriers only as a compromise approach? If so, which release is suitable? Recommendations for 2nd round: |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2009891 | To be revised |
| R4-2010562 | To be revised based on the outcome of discussion 1-1-1 |
| R4-2010565 | To be revised based on the outcome of discussion 1-2-1 and 1-2-2 |

## Discussion on 2nd round (if applicable)

Issue 1-1-1 : Whether s-NonIntraSearch affects measurement of interfrequency CA candidate cells

Two companies support that NonIntraSearch does not affect measurements of inter-frequency CA candidate cells. One company supports that overlapping carrier follows existing reselection requirements which are depending on threshold. Corresponding discussion is ongoing in MR-DC for NR, although this is a release 16 work item which should in principle not affect rel15 euCA.

Candidate options:

Option 1 : sNonIntraSearch does not affect measurement of interfrequency CA candidate cells

Option 2 : Overlapping interfrequency carriers are not measured when serving cell is above sNonIntraSearch

Option 3 : Others

Second round : Please provide further comments, Nokia to provide revised CR if a suitable way to address the issue is identified

Issue 1-2-1 : Whether additional accuracy tests are specified for idle mode measurements in euCA

Issue 1-2-2 : Testing scope if accuracy tests are specified

Two companies supported the addition of accuracy tests, one company thought it was discussed previously and reselection tests implicitly verify the accuracy

Tentative agreements

For discussion in 2nd round : Is it agreeable to introduce only the tests for inter-frequency non-overlapping carriers only as a compromise approach? If so, which release is suitable?

Second round : Please provide further comment on the tentative agreement after first round, Nokia to provide revised CR if a suitable way to address the issue is identified

| Company | Comments |
| --- | --- |
| Qualcomm | Issue 1-1-1: We can support option 2. Issue 1-2-1 and 1-2-2: we can support the compromise proposal from Ericsson based on TEI16 or TEI17; but not R15. |
| Ericsson | Issue 1-1-1 : Our view here is that if the interpretation of RAN4 specs that overlapping carriers do not need to be measured (at all) when Srxlev>s-Non-Intra-Search this is in conflict with a RAN2 procedure that the inter-frequency search thresholds (S-measure) are not applied for carriers configured for early measurement reporting. Not measuring such carriers at all is to us, clearly a procedural matter and not just a requirement matter since it would cause an interoperability issue in case the network assumed that the UE would report the EMR candidates and the UE stopped measuring. .In this situation, it is likely as a workaround that network implementation would just not configure s-non-Intra-Search which is clearly harmful also for UEs not supporting LTE EMR. On the other hand, we can understand concerns with measuring such frequencies continuously when that is not needed for reselection purpose, and we agree with Huawei that it is still up to RAN4 to define what the requirements are in such case, and they do not need to be the same requirements as for reselection purpose.. So to try to suggest a constructive way forward we propose Option 3 : When serving cell measurements exceed Snon-intra-search overlapping carriers are measured with similar requirements to non-overlapping carriers. In practice, this means that the requirement would be something like For non-overlapping carriers, and also for overlapping carriers when the serving cell exceeds SnonIntraSearchP or SnonIntraSearchQ, at least prior to transmission of the idle mode measurement report, the UE shall perform at least a single measurement on detected cells on the non-overlapping inter-frequency carrier(s) configured to be measured for early measurement reporting. Clearly, when the serving cell does not exceed SnonIntraSearchP or SnonIntraSearchQ, (or thresholds are not configured) the overlapping EMR measurements should follow reselection measurement requirements as they do today. Issue 1-2-1 / 1-2-2 : We are Ok to define accuracy tests for a the non overlapping case (interfrequency) only and from R16 onwards. |
| Huawei | Issue 1-1-1 : Whether s-NonIntraSearch affects measurement of interfrequency CA candidate cells We are OK with option 3 suggested by Ericsson above. Issue 1-2-1 : Whether additional accuracy tests are specified for idle mode measurements in euCA Issue 1-2-2 : Testing scope if accuracy tests are specified We are OK with Ericsson suggestion above. |
| Nokia | Issue 1-1-1 : Whether s-NonIntraSearch affects measurement of interfrequency CA candidate cells: Option 1. Concerning option 2: this seems a bit contradictive to the fact that RAN4 initially did develop requirements assuming option 2 and then later removed to dependency on sNonIntraSearch thresholds in R4-1912845. Can Huawei clarify what they mean when saying that measurement procedure? Additionally, it would be good to get clarification regarding how measurement requirements are impacted by the search thresholds when search thresholds are not impacting the EMR measurements? Our view is that the measurement requirements are defined in 4.2.2. Section 4.2.2.4 defines the measurement requirements for inter-frequency measurements. And the measurement requirements for EMR inter-frequency requirements are not impacted by search thresholds. Hence, search threshold-based measurement requirements in 4.2.2.4 do not apply for measurement performed for EMR measurements. Rel-15 euCA should be as baseline for Rel-16 MR-DC EMR as described in the WID. As for the compromise proposal from Ericsson, we still see this being against the RAN2 assumption. Meaning not measuring at all or partly or frequently based on the search threshold and the serving cell quality – still all means applying search thresholds to EMR measurements. One reason for not applying search thresholds to EMR carriers is to enable use of the thresholds independently from EMR measurement is – as pointed out by Ericsson – to enable use of thresholds for UEs 1) not supporting the feature, 2) supporting the feature but T331 has expired and 3) non-EMR carriers would not need to be measured when in good conditions. All these factors have positive impact on the UE power consumption. An important aspect of the EMR measurements is that they are measured recently and that they are up to date when reported. Hence, performing the measurements more frequent in idle mode while T331 timer is running was/is the way to guarantee this. We would have some concerns applying same approach as for non-overlapping carriers due to this. For euCA RAN4 has already reduced the number of EMR carriers to 2 to address the UE measurement burden. One question to the Ericsson proposal: would the accuracy requirements would remain? Alternative is to not change the current Rel-15 specification and leave it for UE implementation how to measure the EMR carriers as long as the UE fulfills the accuracy requirements. Issue 1-2-1 and 1-2-2: Whether additional accuracy tests are specified for idle mode measurements in euCA Agree that this was discussed in Rel-15 timeframe. Current will only test relative. However, when RAN5 discussed introducing the tests it turns out that is problematic unless accuracy rest tests. Additionally, UE should only report from carriers with quality better than set threshold. This would also lead to a need for having defined accuracy tests. If Rel-15 change is late we can also agree on introducing this in Rel-16 as proposed. |
| Huawei2 | Response to Nokia comments above, we understand the EMR measurement procedure is not affected by the search threshold in that whether UE performs EMR measurement for a carrier or not does not depend on the threshold and the carrier priority for mobility. Then how UE performs EMR measurement for a carriers, i.e. the measurement requirements, can be up to RAN4 to decide, and it can be based on the threshold and the carrier priority. In euCA the measurement requirements for overlapping carriers are specified to follow those defined in section 4.2.2.4. We understand the requirements in section 4.2.2.4 are depending on the search threshold, as stated in the beginning of the section and copied below. If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2. If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-frequency layers of higher, equal or lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority layers shall be the same as that defined below.. Technically, we have analysed the issues in defining EMR requirements independently of the search threshold in section 2.1 of our paper R4-2011145. Although it is for Rel-16 MR-DC WI, the same observations also apply for euCA. On the potential benefits for not applying search thresholds to EMR carriers Nokia mentioned above, we understand the use of the search thresholds can still be enabled for the 3 types of UEs even it also applies to the EMR requirements. Hope this clarifies a bit our understanding about the issue. The last alternative suggested by Nokia is also fine for us. |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

Moderator summary of conclusions at end of email discussion period

Issue 1-1-1 : Whether s-NonIntraSearch affects measurement of interfrequency CA candidate cells

Discussion of two possible compromise options took place in the 2nd round

- When serving cell measurements exceed Snon-intra-search overlapping carriers are measured with similar requirements to non-overlapping carriers.

- Not change the current Rel-15  specification and leave it for UE implementation how to measure the EMR carriers as long as the UE fulfills the accuracy requirements.

No updated CR was discussed or provided before the deadline, so the Tdoc for the updated CR is recommended to be withdrawn and the original CR postponed. Moderator also notes that the 2nd compromise approach discussed would not require any rel-15 specification change, so it is up to the proponent and other interested companies whether to bring contributions on this in future meetings.

Issue 1-2-1 : Whether additional accuracy tests are specified for idle mode measurements in euCA

No company expressed an objection in principle to adding additional accuracy tests.

Issue 1-2-2 : Testing scope if accuracy tests are specified

All companies active in the discussion could agree to rel-16 scope. The detailed test list was not agreed with the proponent clarifying their view that they wanted to ensure that absolute accuracy of idle measurements is verified.

Based on this status, no updated CR was discussed or provided before the deadline, so the Tdoc for the updated CR is recommended to be withdrawn and the original CR postponed.

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2012083 | Widthdawn. Status of original CR in R4-2010562 updated to “Postponed” |
| R4-2010563 | Withdrawn (rel-16 shadow CR) |
| R4-2010284 | Widthdawn. Status of original CR in R4-2010565 updated to “Postponed” |
| R4-2010566 | Withdrawn (rel-16 shadow CR) |

# Topic #2: CE Mode-A maintenance

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| Document | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2009534 | Anritsu | In SchedulingRequest-Config-DEFAULT tables of tests A.8.1.28 the sr-ConfigIndex changed from 10 (10 ms periodicity) to 30 for FDD, which are the default SR_Index values used by RAN5 in TS 36.508 |
| R4-2009535 | Anritsu | Shadow CR |
| R4-2009536 | Anritsu | Shadow CR |
| R4-2009537 | Anritsu | Shadow CR |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Comment on CR directly in section 2.3.2

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX |  |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2009534 (Anritsu), 36.133 | Ericsson: OK with this CR. |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2009534 | Agreeable |

## Discussion on 2nd round (if applicable)

Not applicable

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

Not applicable

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #3: NB-IoT maintenance

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| Document | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2011097 | Huawei | CR on NB-IoT Intra frequency with serving cell measurement relaxation test case 4.2.38 R15 Change SSearchDeltaP to 6 dB Change the ES/Noc and ES/Io to 23 dB. Thus NRSRP in T1 is -75 dB, and the realexed monitoring criteria is no longger fulfilled at T2 with 4 dB margin. |
| R4-2011098 | Huawei | Shadow CR |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Comment on CR directly in section 3.3.2

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX |  |
| Qualcomm | We agree with the point. But our concern is very high SNR of cell1 in T1. Isn’t it better to reduce cell2 SNR a bit more and increase cell1 SNR a bit less to make it more reasonable? |
| Huawei | Thanks to QC for the comments. We agree with the concern. We think it is better to keep the SNR condition of nCell1 in T1 unchanged, and reduce the SNR of nCell1 in T2 (13dB to 7dB) and nCell2 in T2 (17 dB to 11 dB), thus it won’t exceed the maximum IO/BW in nCell1 in T1. |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2011097 (Huawei), 36.133 |  |
|  | Company B |
|  | Qualcomm: please see comments in section 3.3.1. |
|  | Huawei: We could revised the CR if the proposed changes in our comments in 3.3.1 if it is acceptable. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2011097 | To be revised |

## Discussion on 2nd round (if applicable)

Second round : Huawei, please provide a draft of the updated CR and discuss under the following sub-thread so that an agreeable wording can be found during 2nd round

[96e][203] LTE_RRM_maintenance – NBIoT CR

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2012085 (Revision of R4-2011097) | Agreeable |