3GPP TSG-RAN WG4 Meeting #96-e	R4-2012236

# Electronic Meeting, 17th – 28th Aug., 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0950 | rev | 1 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on TS38.133 for dual active protocol stack handover (Section 6.1.3) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh-Core |  |  |  |  |  | Date: |  |  | 2020-08-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Clarify that UE is not required to retune its RF to conduct the SSB measurement during the DAPS handover procedure - For asynchronous intra-frequency DAPS handover and asynchronous intra-band inter-frequency DAPS handover, demodulation performance degradation might happen on at least one symbol of a slot. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify requirements only apply if - the UE indicates ‘no-gap’ via intraFreq-needForGap for intra-frequency measurement of source cell and intra-frequency measurement of target cell, or - the SSB of source cell is completely contained in the active DL BWP of the source cell, and the SSB of target cell is completely contained in the active DL BWP of the target cell, or - the initial DL and UL BWP of source cell is confined within the active DL and UL BWP of the source cell respectively, and the initial DL and UL BWP of target cell is confined within the active DL and UL BWP of the target cell respectively. - Clarify that demodulation performance degradation might happen on at least one symbol of a slot for asynchronous intra-frequency DAPS handover and asynchronous intra-band inter-frequency DAPS handover. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - More interruption will be expected, if the SSB of target cell is not confined within the active DL BWP of the target cell or the SSB of source cell is not confined within the active DL BWP of the source cell. - Whether UE is allowed to have demodulation performance degradation on at least one symbol of a slot for asynchronous intra-frequency DAPS handover and asynchronous intra-band inter-frequency DAPS handover is not clear. If performance degradation is not allowed, UE will not be able to support asynchronous intra-frequency DAPS handover and asynchronous intra-band inter-frequency DAPS handover. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2009896 |  |  |  |  |  |  |  |  |

-------------------------------------------------------- Beginning of Change ------------------------------------------------------------

### 6.1.3	NR DAPS Handover

#### 6.1.3.1	Introduction

The requirements in this clause 6.1.3 are applicable to DAPS handover to change the NR PCell to another NR cell.

Note:	requirements only apply if

- the UE indicates ‘no-gap’ via intraFreq-needForGap for intra-frequency measurement of source cell and intra-frequency measurement of target cell, or

- the SSB of source cell is completely contained in the active DL BWP of the source cell, and the SSB of target cell is completely contained in the active DL BWP of the target cell, or

- the initial DL and UL BWP of source cell is confined within the active DL and UL BWP of the source cell respectively, and the initial DL and UL BWP of target cell is confined within the active DL and UL BWP of the target cell respectively.

#### 6.1.3.2	NR FR1 - NR FR1 DAPS Handover

The requirements in this clause are applicable to both intra-frequency and inter-frequency handovers from NR FR1 cell to NR FR1 cell. A DAPS handover is intra-frequency if the centre frequency of the SSB of the source cell and the centre frequency of the SSB of the target cell are the same, and 	the subcarrier spacing of the two SSBs are also the same.

Note:	For intra-frequency DAPS handover, no requirement applies if active DL and UL BWP of target cell is not confined within the active DL and UL BWP of the source cell respectively.

Note:	For inter-frequency DAPS handover, no requirement applies if the BWP of target cell is overlaped with the BWP of source cell in frequency domain.

An FR1 DAPS handover is synchronous if it meets the conditions in table  6.1.3.2-1, otherwise it is asynchronous

Table  6.1.3.2-1: Sync conditions for FR1 DAPS handover

| Type of handover | Maximum receive timing difference between source and taget cell (µs) for sync DAPS handover | Maximum transmit timing difference between source and taget cell (µs) for sync DAPS handover |
| --- | --- | --- |
| Intra-frequencyNote 1,2,3 | 6µs | 7.6 µs |
| Intra-band inter-frequency Note 1,2,3 | 6µs | 7.6 µs |
| Inter-band inter-frequency | 33 µs | 34.6 µs |
| Note 1 : For sync DAPS handover, if the receive time difference exceeds the cyclic prefix length of that SCS, demodulation performance degradation is expected for the first symbol of the slot. For async DAPS handover, demodulation performance degradation might happen on at least one symbol of a slot. Note 2: For DAPS handover on a TDD band, a UE is not expected to transmit in the uplink earlier than NRX-TX after the end of the last received downlink symbol in the same cell where NRX-TX=26500Tc. Note 3: For DAPS handover on a TDD band, a UE is not expected to receive in the downlink earlier than NTX-RX after the end of the last transmitted uplink symbol in the same cell where NTX-RX=26500Tc. |  |  |

##### 6.1.3.2.1	DAPS handover delay

Procedure delays for the procedure that can command a DAPS handover are specified in TS 38.331 [2].

When the UE receives a RRC message implying handover, the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover1 seconds from the end of the last TTI containing the RRC command when UE is configured with dual active protocol stack handover.

Dhandover1 = TRRC_procedure + Tsearch + TIU + Tprocessing + T∆ + Tmargin ms

Where:

TRRC_procedure is the maximum RRC procedure delay as specified in clause 12 in TS 38.331 [2].

Tsearch, TIU, Tprocessing, T∆ and Tmargin are defined in clause 6.1.1.2.2.

After successful RACH procedure of the target cell, when the UE receives an RRC message implying source cell release command, the UE shall accomplish the release actions specified in TS 38.331 [2] within Dhandover2.

Dhandover2 = TRRC_procedure+ Tinterrupt2

Where:

TRRC_procedure is the RRC procedure delay as specified in clause 12 in TS 38.331 [2].

Tinterrupt2 is defined in clause 6.1.3.2.2.

##### 6.1.3.2.2	Interruption time

During Dhandover1, the UE is allowed an interruption of up to Tinterrupt1 on source cell.

For FR1-to-FR1 intra-frequency handover, Tinterrupt1 is specified in Table 6.1.3.2.2-1.

Table 6.1.3.2.2-1: Tinterrupt1 for FR1-to-FR1 intra-frequency DAPS HO

|  | NR Slot length (ms) | Interruption length Tinterrupt1 (slotsNote 1), synchronous DAPS HO | Interruption length Tinterrupt1 (slotsNote 1), asynchronous DAPS HO |
| --- | --- | --- | --- |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 2 | 3 |
| 2 | 0.25 | 4 | 5 |
| Note 1: The same SCS of source cell and target cell is assumed. Note 2: It is assumed that the BWP of target cell is not larger than the BWP of source cell. It is assumed that the CBW of target cell is not larger than the CBW of source cell Note 3: Void |  |  |  |

For FR1-to-FR1 intra-band inter-frequency handover, Tinterrupt1 is specified in Table 6.1.3.2.2-2.

Table 6.1.3.2.2-2: Tinterrupt1 for FR1-to-FR1 intra-band inter-frequency DAPS HO

|  | NR Slot length (ms) | Tinterrupt1 (slotsNote 1), synchronous DAPS HO | Tinterrupt1 (slotsNote 1), asynchronous DAPS HO |
| --- | --- | --- | --- |
| 0 | 1 | 1 + TSMTC_duration | 2+ TSMTC_duration |
| 1 | 0.5 | 2 + TSMTC_duration | 3+ TSMTC_duration |
| 2 | 0.25 | 4 + TSMTC_duration | 5+ TSMTC_duration |
| Note 1: The same SCS of source cell and target cell is assumed. Note 2: TSMTC_duration is the longest SMTC duration between source cell and target cell. Note 3: Void |  |  |  |

For FR1-to-FR1 inter-band handover, Tinterrupt1 is specified in Table 6.1.3.2.2-3.

Table 6.1.3.2.2-3: Tinterrupt1 for FR1-to-FR1 inter-band DAPS HO

|  | NR Slot length (ms) of source cell | Tinterrupt1 (slots) |  |
| --- | --- | --- | --- |
|  |  | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 2 | 3 |
| 2 | 0.25 | 5 | 5 |

During Dhandover2, the UE is allowed an interruption of up to Tinterrupt2 on target cell.

For FR1-to-FR1 intra-frequency handover, Tinterrupt2 equals to 2ms when the BWP of target cell is smaller than the BWP of source cell, and Tinterrupt2 is specified in Table 6.1.3.2.2-4 when the same BWP is used for target cell and source cell.

Table 6.1.3.2.2-4: Tinterrupt2 for FR1-to-FR1 intra-frequency DAPS HO

|  | NR Slot length (ms) | Interruption length X (slotsNote 1) | Tinterrupt2 (slotsNote 1) for asynchronous DAPS HO |
| --- | --- | --- | --- |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 2 | 3 |
| 2 | 0.25 | 4 | 5 |
| Note 1: The same SCS of source cell and target cell is assumed. Note 2: It is assumed that the BWP of target cell is the same as the BWP of source cell. Note 3: Void |  |  |  |

For FR1-to-FR1 intra-band inter-frequency handover, Tinterrupt2 is specified in Table 6.1.3.2.2-5.

Table 6.1.3.2.2-5: Tinterrupt2 for FR1-to-FR1 intra-band inter-frequency DAPS HO

|  | NR Slot length (ms) | Tinterrupt2 (slotsNote 1) for synchronous DAPS HO | Tinterrupt2 (slotsNote 1) for asynchronous DAPS HO |
| --- | --- | --- | --- |
| 0 | 1 | 1 + TSMTC_duration | 2+ TSMTC_duration |
| 1 | 0.5 | 2 + TSMTC_duration | 3+ TSMTC_duration |
| 2 | 0.25 | 4 + TSMTC_duration | 5+ TSMTC_duration |
| Note 1: The same SCS of source cell and target cell is assumed. Note 2: TSMTC_duration is the longest SMTC duration between source cell and target cell. Note 3: Void. |  |  |  |

For FR1-to-FR1 inter-band handover, Tinterrupt2 is specified in Table 6.1.3.2.2-6.

Table 6.1.3.2.2-6: Tinterrupt2 for FR1-to-FR1 inter-band DAPS HO

|  | NR slot length (ms) of target cell | Tinterrupt2 (slots) |  |
| --- | --- | --- | --- |
|  |  | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 2 | 3 |
| 2 | 0.25 | 5 | 5 |

#### 6.1.3.3	NR FR2- NR FR1 DAPS Handover

The requirements in this clause are applicable to inter-frequency handovers from NR FR2 cell to NR FR1 cell.

An FR2-FR1 DAPS handover is synchronous if it meets the conditions in table  6.1.3.3-1, otherwise it is asynchronous

Table  6.1.3.3-1: Sync condition for FR2-FR1 DAPS handover

| Frequency Range of the pair of carriers | Maximum receive timing difference between source and taget cell (µs) for sync DAPS handover | Maximum transmit timing difference between source and taget cell (µs) for sync DAPS handover |
| --- | --- | --- |
| Between FR1 and FR2 | 25 | 26.1 |

##### 6.1.3.3.1	DAPS handover delay

Procedure delays for the procedure that can command a DAPS handover are specified in TS 38.331 [2].

When the UE receives a RRC message implying handover, the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover1 ms from the end of the last TTI containing the RRC command when UE is configured with dual active protocol stack handover.

Dhandover1 = TRRC_procedure + Tsearch + TIU + Tprocessing + T∆ + Tmargin ms

Where:

TRRC_procedure is the maximum RRC procedure delay as specified in clause 12 in TS 38.331 [2].

Tsearch, TIU, Tprocessing, T∆ and Tmargin are defined in clause 6.1.1.3.2.

After successful RACH procedure of the target cell, when the UE receives an RRC message implying source cell release command, the UE shall accomplish the release actions specified in TS 38.331 [2] within Dhandover2.

Dhandover2 = TRRC_procedure+ Tinterrupt2

Where:

TRRC_procedure is the RRC procedure delay as specified in clause 12 in TS 38.331 [2].

Tinterrupt2 is defined in clause 6.1.3.3.2.

##### 6.1.3.3.2	Interruption time

During Dhandover1, the UE is allowed an interruption of up to Tinterrupt1 on source cell.

For FR2-to-FR1 inter-band handover, Tinterrupt1 is specified in Table 6.1.3.3.2-1.

Table 6.1.3.3.2-1: Tinterrupt1 for FR2-to-FR1 inter-band DAPS HO

|  | NR slot length (ms) of source cell | Tinterrupt1 (slots) |  |
| --- | --- | --- | --- |
|  |  | Sync | Async |
| 2 | 0.25 | 5 | 5 |
| 3 | 0.125 | 9 | 9 |

During Dhandover2, the UE is allowed an interruption of up to Tinterrupt2 on target cell.

For FR2-to-FR1 inter-band handover, Tinterrupt2 is specified in Table 6.1.3.3.2-2.

Table 6.1.3.3.2-2: Tinterrupt2 for FR2-to-FR1 inter-band DAPS HO

|  | NR slot length (ms) of target cell | Tinterrupt2 (slots) |  |
| --- | --- | --- | --- |
|  |  | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 2 | 3 |
| 2 | 0.25 | 5 | 5 |

#### 6.1.3.4	NR FR1- NR FR2 DAPS Handover

The requirements in this clause are applicable to inter-frequency handovers from NR FR1 cell to NR FR2 cell.

An FR1-FR2 DAPS handover is synchronous if it meets the conditions in table  6.1.3.4-1, otherwise it is asynchronous

Table  6.1.3.4-1, : Sync condition for FR1-FR2  DAPS handover

| Frequency Range of the pair of carriers | Maximum receive timing difference between source and taget cell (µs) for sync DAPS handover | Maximum transmit timing difference between source and taget cell (µs)Note 1 sync DAPS handover |
| --- | --- | --- |
| Between FR1 and FR2 | 25 | 26.1 |

##### 6.1.3.4.1	DAPS handover delay

Procedure delays for the procedure that can command a DAPS handover are specified in TS 38.331 [2].

When the UE receives a RRC message implying handover, the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover1 ms from the end of the last TTI containing the RRC command when UE is configured with dual active protocol stack handover.

Dhandover1 = TRRC_procedure + Tsearch + TIU + Tprocessing + T∆ + Tmargin ms

Where:

TRRC_procedure is the maximum RRC procedure delay as specified in clause 12 in TS 38.331 [2].

Tsearch, TIU, Tprocessing, T∆ and Tmargin are defined in clause 6.1.1.5.2.

After successful RACH procedure of the target cell, when the UE receives an RRC message implying source cell release command, the UE shall accomplish the release actions specified in TS 38.331 [2] within Dhandover2.

Dhandover2 = TRRC_procedure+ Tinterrupt2

Where:

TRRC_procedure is the RRC procedure delay as specified in clause 12 in TS 38.331 [2].

Tinterrupt2 is defined in clause 6.1.3.4.2.

##### 6.1.3.4.2	Interruption time

During Dhandover1, the UE is allowed an interruption of up to Tinterrupt1 on source cell.

For FR1-to-FR2 inter-band handover, Tinterrupt1 is specified in Table 6.1.3.4.2-1.

Table 6.1.3.4.2-1: Tinterrupt1 for FR1-to-FR2 inter-band DAPS HO

|  | NR slot length (ms) of source cell | Tinterrupt1 (slots) |  |
| --- | --- | --- | --- |
|  |  | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 2 | 3 |
| 2 | 0.25 | 5 | 5 |

During Dhandover2, the UE is allowed an interruption of up to Tinterrupt2 on target cell.

For FR1-to-FR2 inter-band handover, Tinterrupt2 is specified in Table 6.1.3.4.2-2.

Table 6.1.3.4.2-2: Tinterrupt2 for FR1-to-FR2 inter-band DAPS HO

|  | NR slot length (ms) of target cell | Tinterrupt2 (slots) |  |
| --- | --- | --- | --- |
|  |  | Sync | Async |
| 2 | 0.25 | 5 | 5 |
| 3 | 0.125 | 9 | 9 |

------------------------------------------------------------ End of Change ----------------------------------------------------------------