3GPP TSG-RAN WG4 Meeting #96-e	R4-2012284

# Online Meeting, 17 – 28 August, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0943 | rev | 2 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of UE Rx-Tx time difference measurement requirements for NR positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2020-08-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Per work split agreement in RAN4#94-e-Bis, the measurement requirements for UE Rx-Tx time difference in NR positioning is introduced with this CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define UE Rx-Tx time difference measurement requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR positioning core requirements will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.9.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Start of change >

### 9.9.4	UE Rx-Tx time difference measurements

### 9.9.4.1 Introduction

The requirements in this clause shall apply, provided the UE has received nr-Multi-RTT-RequestLocationInformation message from LMF via LPP [34] requesting the UE to measure one or more UE Rx-Tx time difference measurements defined in TS 38.215 [4].

### 9.9.4.2 Requirements Applicability

The requirements in clause 9.9.4 apply, provided:

- UE Rx-Tx time difference measurement related side conditions given in clause 10.1.25 are met for a corresponding band.

### 9.9.4.3 Measurement Capability

### UE Rx-Tx time difference measurement capability is as reported by the UE according to TS 37.355 [34].

### 9.9.4.4 Measurement Reporting Requirements

This requirement assumes that the measurement report is not delayed by other LPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration. This measurement reporting delay excludes any delay caused by no UL resources for UE to send the measurement report.

The reported UE Rx-Tx time difference measurement values reported shall be based on the measurement report mapping requirements specified in clause 10.1.25.

Editor’s note: the measurement reporting requirements for aperiodic reports are FFS.

The UE Rx-Tx time difference measurement accuracy for all measured DL PRS resources shall be fulfilled according to the accuracy requirements specified in clause 10.1.25.

### 9.9.4.5 Measurement Period Requirements

When physical layer receives last of NR-Multi-RTT-ProvideAssistanceData message and NR-Multi-RTT-RequestLocationInformation message from LMF via LPP [34], UE shall be able to measure multiple (up to the UE capability specified in clause 9.9.4.3) UE Rx-Tx time difference measurements as defined in TS 38.215 [4] in configured positioning frequency layers within the measurement period TUERxTx,Total ms.

If measurement gaps and processing time T have overlap between different frequency positioning frequency layers,

TUERxTx, Total=i=1LTUERxTx,i+L-1*maxTeffect,i.

Editor’s Note: FFS the UE Rx-Tx time difference measurement period when measurement gaps and processing time T do not have overlap between different positioning frequency layers.

where i is the index of PRS frequency layer,

TUERxTx,i is the measurement period for PRS frequency layer i as further defined in this section,

L is total number of positioning frequency layers, and

Teffect,i is the periodicity of the UE Rx-Tx time difference measurement in PRS frequency layer i

TUERxTx,i=CSSFi*NRxBeam,i*NPRS,islotN'LPRS,iN*Nsample-1*Teffect,i+Tlast

Where

CSSFi is scaling factor for NR positioning measurements as defined in clause 9.1.5.2,

NRxBeam,i is the scaling factor for Rx beam sweeping, and NRxBeam,i=1 if PRS layer i is in FR1 and NRxBeam,i=8 if PRS layer i is in FR2,

LPRS,i is the time duration specified in clause 5.1.6.5 of TS 38.214 [26, section 5.1.6.5]

NPRS,islot is the maximum number of DL PRS resources of frequency layer i configured in a slot,

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms processed every T ms for a given maximum bandwidth supported by UE as specified in clause 4.2.7.2 of TS 38.306 [14],

N’ is UE capability for number of DL PRS resources that it can process in a slot as specified in clause 4.2.7.2 of TS 38.306 [14],

Nsample is the number of UE Rx-Tx time difference measurement samples and Nsample= [4],

Teffect,i is periodicity of UE Rx-Tx time difference measurement in frequency layer i:

Teffect,i= TiTavailable_PRS,i*Tavailable_PRS,i

where Tavailable_PRS,i= LCMTPRS,i,MGRPi, the least common multiple between TPRS,i and MGRPi

TPRS,i is the maximum PRS resource periodicity among all PRS resources in frequency layer i,

MGRPi is the repetition periodicity of the measurement gap applicable for measurement in frequency layer i.

If the frequency layer i has more than one DL PRS resource sets with different PRS periodicities, the maximum PRS periodicity among DL PRS resource sets is used to derive the measurement period of that positioning frequency layer.

The time TUERxTx,i starts from the first MG instance aligned with DL PRS resources of positioning frequency layer i closest in time after both the NR-Multi-RTT-RequestLocationInformation message and NR-Multi-RTT-ProvideAssistanceData message from LMF via LPP [34] are delivered to the physical layer of UE.

The UE Rx-Tx time difference measurement period is restarted if HO occurs during the measurement period and after SRS reconfiguration on the target cell is complete.

Editors Note: The applicability of requirements if the time duration of a DL PRS resource exceeds the UE capability N.

# < End of change >