3GPP TSG-RAN WG4 Meeting #96-e	R4-2012537

Electronic Meeting, 17 – 28 August 2020

Agenda item:			6.1.3

Source:	Moderator (Ericsson)

Title:	Email discussion summary for [96e][314] LTE_eMTC5_Demod

Document for:	Information

# Introduction

This email discussion targets to set the UE demodulation/CSI reporting requirements for Rel-16 eMTC.

Candidates target of email discussion for 1st round and 2nd round:

- 1st round:

  - Collect companies’ simulation results

  - Collect comments for CRs for MPDCCH demodulation and CSI-RS based PMI reporting test

- 2nd round:

  - Set the SNR test points for MPDCCH demodulation requirements

  - Agree with CRs.

# Topic #1: Rel-16 eMTC MPDCCH and PMI reporting requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2010471 | Ericsson | Simulation results only |
| R4-2010964 | Huawei, HiSilicon | Simulation results only |
| R4-2010475 | Ericsson | Summary of simulation results |
| R4-2010472 | Ericsson | Observation: Throughput ratio of 1.4 is achieved with 70% of maximum throughput with follow PMI. Proposal: Confirm the throughput ratio of follow PMI over random PMI (γ) to 1.2, for the CSI-RS based PMI reporting test for non-BL UE at SNR where 70% of the maximum throughput is achieved with follow PMI, for both FDD and TDD. |
| R4-2010965 | Huawei, HiSilicon | Simulation results |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1

Issue 1-1: Set the final requirements

- Recommended WF

  - Collect the impairment simulation results for MPDCCH demodulation.

  - Set MPDCCH demodulation requirements based on the summary.

### Sub-topic 1-2

Issue 1-2: Requirements of PMI reporting tests for non-BL UE

- Proposal

  - Set the throughput ratio of follow PMI over random PMI (γ) to 1.2, for the CSI-RS based PMI reporting test for non-BL UE at SNR where 70% of the maximum throughput is achieved with follow PMI, for both FDD and TDD.

- Recommended WF

  - Agree with the proposal above

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Issue 1-1: Impairment results have been added into the summary. Issue 1-2: Based on our simulation results, we confirm that throughput ratio of more than 1.4 can be achieved with 70% of maximum throughput with follow PMI. Thus, we agree with recommended WF. |
| Ericsson | Issue 1-1: Support the recommended WF. Issue 1-2: Support the recommended WF. |

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2010473 | Company A |
|  | Company B |
|  | Huawei: There are typo on table numbering for section 8.11.2.2.5: First ‘8.11.2.2.5-1’ should be ‘8.11.2.2.5-2’. Second ‘8.11.2.2.5-1’ should be ‘8.11.2.2.5-3’. |
| R4-2010474 | Ericsson: New FRC name R.106 FDD/TDD is conflicted with R4-2010967. Propose to revise R4-2010474 so that FRC name is changed from R.106 to R.108. |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Agreements: Set enhanced MPDCCH demodulation requirements based on R4-2010475. Required SNR. FDD/HD-FDD; MPDCCH; DMRS+CRS, CEModeA, AL16, Rep16, EPA5 2x1; 1% pm-dsg; -3.4. DMRS+CRS, CEModeB, AL24, Rep32, ETU1 2x1; 1% pm-dsg; -9.5. TDD; MPDCCH; DMRS+CRS, CEModeA, AL16, Rep16, EPA5 2x1; 1% pm-dsg; -4.1. DMRS+CRS, CEModeB, AL24, Rep32, ETU1 2x1; 1% pm-dsg; -9.7. Recommendations for 2nd round: Discuss the revision of CR R4-2010473. |
| Sub-topic#2 | Agreements: - Set the throughput ratio of follow PMI over random PMI (γ) to 1.2, for the CSI-RS based PMI reporting test for non-BL UE at SNR where 70% of the maximum throughput is achieved with follow PMI, for both FDD and TDD. Recommendations for 2nd round: Discuss the revision of CR R4-2010474. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | No WFs are required |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2010473 | To be revised. - Fix typos - Set MPDCCH demodulation requirements in []. |
| R4-2010474 | To be revised. - Rename FRC number from R.106 to R.108. |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |