3GPP TSG-RAN WG4 Meeting # 96-e 												R4-200xxxx

Electronic Meeting, 17 – 28 August, 2020

Agenda item:			6.2.3

Source:	Moderator (Huawei, HiSilicon)

Title:	Email discussion summary for [96e][315] NB_IOTenh3_Demod

Document for:	Information

Introduction

During last RAN4#95-e meeting, R4-2008759 WF on LTE UE and BS performance requirements for additional enhancements of NB-IoT was approved with the following agreements:

- NPDSCH - Define NPDSCH performance requirements with multi-TB scheduled transmission by using the following simulation assumptions Parameter; Value. System bandwidth; 200 kHz. Operation mode; Stand alone. Antenna configuration; 1T1R. Channel model; ETU 1Hz. Frequency error; 0 Hz. Timing error; 0μs. Performance target; SNR@70% of maximum throughput. NSF; 10. Repetition number; 32. Transmission mode; Interleaved. HARQ process; 2. Carrier Type; Non-anchor. TB Size; 1032. Duplex mode; HD-FDD. - Interested companies are welcome to provide simulation results in the next meeting - NPUSCH format 1 - Define NPUSCH format 1 performance requirements with multi-TB scheduled transmission by using the following simulation assumptions Parameter; Value. Number of tones; 12. SCS; 15kHz. Antenna configuration; 1T2R. Channel model; ETU 1Hz. Frequency error; 0 Hz. Timing error; 0μs. Performance target; SNR@70% of maximum throughput. NRU; 5. Repetition number; 64. HARQ process; 2. Transmission mode; Interleaved. TB Size; 1032. - Interested companies are welcome to provide simulation results in the next meeting

This is for the 1st round discussion based on the agreements reached in last meeting.

Further discussions are summarized in the following as per the submitted contributions.

Topic #1: NPDSCH performance requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2010476 | Ericsson | Simulation results and proposal: Proposal: Introduce NPDSCH demodulation requirements with interleaved multi-TB transmission for UE capable of 1) Cat-NB2, 2) multiTB-DL, and 3) multiTB-DL-Interleaving. |
| R4-2010969 | Huawei, HiSilicon | Simulation results only |
| R4-2010970 | Huawei, HiSilicon | Proposal 1: No change to Noc level Proposal 2: Use following test parameters in Table 4 Table 4 Test parameters Parameter; Unit; Value. DCI format; DCI format N1. scheduling delay field (); 1. (ack-NACK-NumRepetitions-r13); 1. ACK/NACK resource field; 0. Repetition number of NPDCCH; 128. Rmax; 128. G; 1.5. 0. |
| R4-2010971 | Huawei, HiSilicon | CR for introduction of NPDSCH performance requirements for multi-TB interleaved into TS 36.101 and RFC definition |
| R4-2010975 | Huawei, HiSilicon | Summary of simulation results for LTE NPDSCH with multi-TB interleaved transmission. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1-1 Test parameters

Issue 1-1-1: Noc level

- Proposals

  - Option 1: Use =-93dBm/15kHz,  = -99dBm/15kHz (Huawei)

  - Option 2:

- Recommended WF

  - TBA

Issue 1-1-2: Other parameters

- Proposals

  - Option 1: (Huawei)

| Parameter | Unit | Value |
| --- | --- | --- |
| DCI format |  | DCI format N1 |
| scheduling delay field () |  | 1 |
| (ack-NACK-NumRepetitions-r13) |  | 1 |
| ACK/NACK resource field |  | 0 |
| Repetition number of NPDCCH |  | 128 |
| Rmax |  | 128 |
| G |  | 1.5 |
|  |  | 0 |

  - Option 2:

- Recommended WF

  - TBA

Sub-topic 1-2 Test applicability

Issue 1-2-1: Test applicability

- Proposals

  - Only applicable for UE supporting:

    - ue-Category-NB: NB2

    - multiTB-DL-r16: support multiple TB scheduling in downlink

    - multiTB-DL-Interleaving-r16: support interleaved transmission when multiple TBs

- Recommended WF

  - Capture the above test applicability rule in TS 36.101

Sub-topic 1-3: Final performance requirements

Issue 1-3-1: Final performance requirements for NPDSCH with multi-TB interleaved transmission

- Proposals

  - Collect both the ideal and impairment results during this meeting, and set the final performance requirements based on the averaged impairment results.

- Recommended WF

  - TBA

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Ericsson | Sub topic 1-1: Test parameters Issue 1-1-1: Noc level We are fine if the intension is to set Noc1 = -99dBm/15kHz for NPDSCH transmission and set -93dBm/15kHz for NPDCCH transmission. Issue 1-1-2: Other parameters G=1.5 seems too small. Since one TB transmits with 32 repetitions and N_SF=10, effective transmission length is 320 per TB. Because of 2 TB transmission, NPDSCH is scheduled every 640 subframes. Since NPDCCH repetition is 128, G could be 8. Sub topic 1-2: Test applicability Agree with the recommended WF. Sub topic 1-3: Final performance requirements [20200818] For Issue 1-1-2, we understand your intention. G=1.5 is fine with us. |
| Huawei, HiSilicon | Sub topic 1-1: Test parameters Issue 1-1-1: Noc level Yes, Noc1 = -99dBm/15kHz is for NPDSCH transmission and Noc2 = -93dBm/15kHz is for NPDCCH transmission as per CR R4-2010971. Issue 1-1-2: Other parameters NPDCCH repetition number is 128 (Rmax), if we use G=1.5, and based on the total transmission duration including NPDCCH+NPDSCH+NPUSCH format 2 is 789ms as per the proposed value for scheduling delay (IDelay=1) and number of ack-NACK-NumRepetitions =1, there will be additional 171ms prolongation for one transmission duration, but if we choose G=8, then there will be additional 235ms prolongation for one transmission duration. To minimize the transmission gap impact for the performance, maybe G=1.5 is feasible. Sub topic 1-2: Test applicability Agree with the recommended WF. Sub topic 1-3: Final performance requirements Agree with the proposal |

CRs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR number | Comments collection |
| --- | --- |
| R4-2010971 | Ericsson: Add the applicability rule. |
|  | Huawei: we will capture the applicability rule in the revised CR. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1-1 Test parameters | Issue 1-1-1: Noc level Tentative agreements: Use =-93dBm/15kHz for NPDSCH transmission, = -99dBm/15kHz for NPDCCH transmission. Issue 1-1-2: Other parameters Tentative agreements: Use following test parameters for NPDSCH performance requirements: Parameter; Unit; Value. DCI format; DCI format N1. scheduling delay field (); 1. (ack-NACK-NumRepetitions-r13); 1. ACK/NACK resource field; 0. Repetition number of NPDCCH; 128. Rmax; 128. G; 1.5. 0. Recommendations for 2nd round: All tentative agreements are captured in the revised CR R4-2010971, further comments can be based on the revised CR if needed. |
| Sub-topic 1-3: Test applicability | Issue 1-2-1: Test applicability Tentative agreements: Capture the following test applicability rule in TS 36.101: - The performance requirements are only applicable for UE supporting: - ue-Category-NB: NB2 - multiTB-DL-r16: support multiple TB scheduling in downlink - multiTB-DL-Interleaving-r16: support interleaved transmission when multiple TBs Recommendations for 2nd round: Tentative agreements are captured in the revised CR R4-2010971, further comments can be based on the revised CR if needed. |
| Sub-topic 1-3: Final performance requirements | Issue 1-3-1: Final performance requirements for NPDSCH with multi-TB interleaved transmission Tentative agreements: According to the summary of simulation results R4-2010975, the averaged impairment results can be added to TS 36.101. Recommendations for 2nd round: Averaged impairments results are captured in the revised CR R4-2010971, company can double check them in the revised CR. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2010971 | To be revised |

Discussion on 2nd round (if applicable)

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Topic #2: NPUSCH format 1 performance requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2010276 | Samsung | Ideal and impairment simulation results, and Observation: Observation 1: About 0.64 dB gain can be achieved with multi-TB interleaving scheduling compared with continuous scheduling for NPUSCH |
| R4-2010477 | Ericsson | Simulation results and proposals: Proposal 1: Introduce NPUSCH format 1 demodulation requirements with interleaved multi-TB transmission for BS supporting multiTB-UL and multiTB-UL-Interleaving. Proposal 2: The performance requirements for NPUSCH format 1 demodulation requirements with interleaved multi-TB transmission are optional. |
| R4-2010968 | Huawei, HiSilicon | Simulation results only |
| R4-2010972 | Huawei, HiSilicon | CR: Introduce NPUSCH format 1 performance requirements for multi-TB interleaved transmission for TS 36.104 |
| R4-2010973 | Huawei, HiSilicon | CR: Introduce NPUSCH format 1 test requirements for multi-TB interleaved transmission for TS 36.141 |
| R4-2010974 | Huawei, HiSilicon | Summary of simulation results for LTE NPUSCH format 1 with multi-TB interleaved transmission. |
| R4-2011504 | Nokia, Nokia Shanghai Bell | Simulation results and Observation: Observation 1: For interleaved multi-TB transmission, the gain versus non-interleaved transmission is FFS. |
| R4-2011505 | Nokia, Nokia Shanghai Bell | Revision with simulation results. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 2-1 UL transmission gap

As per TS 36.211 section 10.1.3.6: After transmissions and/or postponements due to NPRACH of time units, for frame structure type 1, a gap of time units shall be inserted where the NPUSCH transmission is postponed.

Issue 2-1-1: UL transmission gap

- Proposals

  - Option 1: It is implicitly reflected in the simulation considering that it is mandatory behaviour and not configurable.

  - Option 2:

- Recommended WF

  - TBA

Sub-topic 2-2 Test applicability

Issue 2-2-1: Test applicability

- Proposals

  - Only applicable for BS supporting 2 HARQ processes, multiTB-UL-r16 and multiTB-UL-Interleaving-r16.

- Recommended WF

  - Add the test applicability rule in TS 36.141: The NPUSCH format 1 Only applicable for BS supporting 2 HARQ processes, multiTB-UL-r16 and multiTB-UL-Interleaving-r16

Sub-topic 2-3: Final performance requirements

Issue 2-3-1: Performance requirements definition

- Proposals

  - Collect both the ideal and impairment results during this meeting, and set the final performance requirements based on the averaged impairment results.

- Recommended WF

  - TBA

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Ericsson | Sub topic 2-1-1: We have also same understanding as option 1. Sub topic 2-2-1: Support the recommended WF. Sub topic 2-3: |
| Samsung | Sub topic 2-1: We are fine with option1. It is similar understanding with Rel-15 NPRACH format2, UL transmission gap is existed, and there is no need to explicitly reflect. Meanwhile, the performance impact is limited Sub topic 2-2: We are fine with recommended WF |
| Huawei, HiSilicon | Issue 2-1-1: We have same understanding as option 1. Issue 2-2-1: Support the recommended WF Issue 2-3-1: Support the proposal. |
| Nokia | Issue 2-1-1: We agree proceeding according option 1. We will take into account UL transmission gap of 40 ms in our evaluation and will resubmit revised figures within next 3 working days. In our understanding the performance impact is rather limited, though. Can the moderator please allocate a Tdoc number for revision of R4-2011505. Issue 2-2-1: We support the recommended WF. Issue 2-3-1: We included our impairment results as well to the NPUSCH performance summary sheet. Both our ideal and impairment results may need to be revised according the above. |

CRs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR number | Comments collection |
| --- | --- |
| R4-2010972 | Ericsson: Need applicability rule. Also need to mention this is optional. |
|  | Samsung: similar view with Ericsson, the requirement for NPUSCH format1 with multi-TB interleaved scheduling is optional. |
|  | Huawei: thanks for the comments, we will capture the test applicability rule in the revised CR and indicate it is optional. |
|  | Nokia: We agree with above views on test applicability rule and optional indication. In Table 8.5.1.1.1-4 it is stated in Note 1: “This requirement is applicable to UE support interleaved transmissions …”. However, it is a requirement for eNB here. Thus, we propose following rewording: “This requirement is only applicable if UE supports interleaved transmissions …”. |
|  | Huawei: thanks for the comments. The comments has been captured in Revised CR v1. To Nokia, the requirement in this CR has been revised based on your updated impairment results. |
| R4-2010973 | Ericsson: Same comment as 10972. |
|  | Samsung: similar view with Ericsson, the requirement for NPUSCH format1 with multi-TB interleaved scheduling is optional. |
|  | Huawei: thanks for the comments, we will capture the test applicability rule in the revised CR and indicate it is optional. |
|  | Nokia: We have the same comments as for R4-2010972. |
|  | Huawei: thanks for the comments. The comments has been captured in Revised CR v1. To Nokia, the requirement in this CR has been revised based on your updated impairment results. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#2-1 | Issue 2-1-1: UL transmission gap Tentative agreements: Companies have common understanding that UL gap is implicitly reflected in simulation and not configurable, it has limited impact on performance. Recommendations for 2nd round: No further discussion is needed. |
| Sub-topic#2-2 | Issue 2-2-1: Test applicability Tentative agreements: Add the following test applicability into TS 36.104 and TS 36.141: the performance requirements for NPUSCH format 1 with multi-TB interleaved transmission are only applicable for BS supporting 2 HARQ processes, multiTB-UL-r16 and multiTB-UL-Interleaving-r16. Recommendations for 2nd round: Agreements are captured in the revised CR R4-2010972 and R4-2010973, further comments can be based on the revised CRs if needed. Companies can check if it is necessary to capture it in TS 36.104. |
| Sub-topic# 2-3 | Issue 2-3-1: Performance requirements definition Tentative agreements: According to the summary of simulation results R4-2010974, the averaged impairment results can be captured in TS 36.104 and TS 36.141 with TT. Recommendations for 2nd round: Averaged impairments results are captured in the revised CRs R4-2010972 and R4-2010973, company can double check them in the revised CRs. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2010972 | To be revised |
| R4-2010973 | To be revised |

Discussion on 2nd round (if applicable)

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |