3GPP TSG-RAN4 Meeting #96-e	R4-20xxxxx

# Electronic Meeting, 17th Aug 2020 - 28th Aug 2020

Agenda item:			6.4.1

Source:	Moderator (Qualcomm)

Title:	Email discussion summary for [96e][316] LTE_terr_bcast_Demod

Document for:	Information

# Introduction

Simulation assumption was agreed in RAN4#95e. In this meeting, the plan is to finalize PMCH demod SNR requirement and close the work item.

# Topic #1: PMCH Demod Requirement

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2010966 | Huawei | Proposal 1: Use MBMS UE category ≥ 2 for both MBSFN RS type 1 and 2 for PMCH 0.37kHz SCS. Proposal 2: For PMCH cases with 0.37kHz SCS, change the Doppler to [5Hz] for propagation conditions |
| R4-2011384 | Qualcomm | Simulation result collection |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### PMCH demod requirement

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 1-1: UE category for MBSFN RS type 1 and 2 for PMCH 0.37kHz SCS

- Proposals

  - Option 1: UE category ≥ 2

- Recommended WF

  - Agree option 1

Issue 1-2: Doppler spread in demod requirement for PMCH 0.37kHz SCS

- Proposals

  - Option 1: Follow original agreement, 0Hz

  - Option 2: [5]Hz

- Recommended WF

  - Need further discussion

Issue 1-3: PMCH SNR requirement

- Finalize 0.37kHz SCS requirement after agreement on issue 1-2 is reached

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Sub topic 1-1: Ok to the recommended WF Sub topic 1-2: Option 1 is Ok for us after double checking our simulation. Issue 1-3: Our PMCH simulation results: PMCH rooftop scenario: Test number; Cell; Bandwidth (MHz); MCS; Antenna configuration; MBSFN RS type; Number of MBSFN subframes per 40ms; 1% PMCH BLER. 1; Dedicated; 10; 21; 1x1; 1; 13; 14.7. 2; Dedicated; 10; 21; 1x1; 2; 13; 15.7. PMCH mobility scenario: Test number; Cell; Bandwidth (MHz); MCS; Antenna configuration; Number of MBSFN subframes per 40ms; 1% PMCH BLER. 3; Dedicated; 10; 12; 1x2 low; 39; 9.5. |
| QC | Issue 1-1: Support recommended WF Issue 1-2: Support option 1 Issue 1-3: our results are listed below: SNR. Rooftop RS1 type 1; 15.6. Rooftop RS1 type 2; 17.9. Mobility; 10.3. |

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2010967 | Ericsson: R4-2010474 (CR for eMTC PMI reporting requirements by Ericsson) will introduce new FRC R.106 FDD/TDD, which conflicts with FRC in this CR. Ericsson is ok to revise our CR (R4-2010474) to use another number. |
|  | Huawei: Thanks for figuring out this confliction and revision of your CR. We will keep to use the FRC number in our CR. |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: UE category for MBSFN RS type 1 and 2 for PMCH 0.37kHz SCS UE category ≥ 2 Doppler spread in demod requirement for PMCH 0.37kHz SCS 0Hz Candidate options: Recommendations for 2nd round: Provide impairment results and finalize SNR requirement |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 Simulation result collection | Summary of alignment and impairment results for 5G broadcast | QC |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2010967 | To be revised according to 1st round agreement and later impairment results collection |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |