3GPP TSG-RAN4 Meeting #96-e	 R4-2012586

# Online, 17th Aug – 28th Aug 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | 0232 | rev | 1 | Current version: | 15.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.104: Annex B and C clarification on equlisation calculation (B.6, C.6) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies, Rohde & Schwarz, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2020-08-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In Annex section describing equlisation, clarification needed where to apply exceptional case of moving average calculation. Current text which originally copied from LTE specification describes exceptional case at or near the edge of the channel. The same needs to be applied for the case at or near the edge of allocated resources because NR test model 2 doesn’t not have contiguously allocated reference signal subcarriers in the frequency domain. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following modifications made: - Description of where to apply reduced window size, added the case when the available DM-RS subcarrier within a set of contiguously allocated RBs is smaller than the moving average window. - Added additional description text for the case of smaller allocation size in figure (figure replaced) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without this clarificaition, unnecessary poor EVM measurement result calculated due to unnecessary combined amplitude and phase characterisctics which largely separated in frequency. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.6, C.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.141-1 CR0149 38.141-2 CR0219 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Changed text which describes the method using more number of subcarriers for averaging for the case of reduced moving avearge window size |  |  |  |  |  |  |  |  |

# B.6	Estimation of TX chain amplitude and frequency response parameters

The equalizer coefficients and  are determined as follows:

1.	Calculate the complex ratios (amplitude and phase) of the post-FFT acquired signal  and the post-FFT ideal signal , for each reference signal, over 10ms measurement interval. This process creates a set of complex ratios:

Where the post-FFT ideal signal  is constructed by the measuring equipment according to the relevant TX specifications, using the following parameters: i.e. nominal demodulation reference signals, (all other modulation symbols are set to 0 V), nominal carrier frequency,  nominal amplitude and phase for each applicable subcarrier, nominal timing.

2.	Perform time averaging at each reference signal subcarrier of the complex ratios, the time-averaging length is 10ms measurement interval. Prior to the averaging of the phases  an unwrap operation must be performed according to the following definition: The unwrap operation corrects the radian phase angles of  by adding multiples of 2*PI when absolute phase jumps between consecutive time instances ti are greater than or equal to the jump tolerance of PI radians. This process creates an average amplitude and phase for each reference signal subcarrier (i.e. every second subcarrier).

Where N is the number of reference signal; time-domain locations ti from Z't,f for each reference signal subcarrier .

3.	The equalizer coefficients for amplitude and phase  and  at the reference signal subcarriers are obtained by computing the moving average in the frequency domain of the time-averaged reference signal subcarriers, i.e. every second subcarrier. The moving average window size is 19 and averaging is over the DM-RS subcarriers in the allocated RBs. For DM-RS subcarriers at or near the edge of the channel, or when the number of available DM-RS subcarriers within a set of contiguously allocated RBs is smaller than the moving average window size,  the window size is reduced accordingly as per figure B.6-1.

4.	Perform linear interpolation from the equalizer coefficients  and  to compute coefficients ,  for each subcarrier.

The subsequent 7 subcarriers are averaged over 5, 7 .. 17 subcarriers

From the 10th subcarrier onwards the window size is 19 until the upper edge of the channel is reached and the window size reduces back to 1

The first reference subcarrier is not averaged

The second reference subcarrier is the average of the first three subcarriers

Reference subcarriers

Reference subcarriers

The first, second and third reference subcarriers are the average of the five subcarriers on left

- Moving averaging at channel edge

- Moving averaging for the case when the number of available reference subcarriers within a set of contiguously allocated RBs is smaller than the moving average window size

The first, second and third reference subcarriers are the average of the five subcarriers on right

Figure B shows an example of 1RB allocation using a reduced window size of five subcarriers for averaging. The same method applies for RB allocations with fewer than 19 subcarriers available for the moving average size. For the case of 2 and 3 RB allocations, 11 and 17 are the window sizes, respectively.

Figure B.6-1: Reference subcarrier smoothing in the frequency domain

<unchanged text omiitted>

# C.6	Estimation of TX chain amplitude and frequency response parameters

The equalizer coefficients and  are determined as follows:

1.	Calculate the complex ratios (amplitude and phase) of the post-FFT acquired signal  and the post-FFT ideal signal , for each reference signal, over 10ms measurement intervals. This process creates a set of complex ratios:

Where the post-FFT ideal signal  is constructed by the measuring equipment according to the relevant TX specifications, using the following parameters:

-	nominal demodulation reference signals and nominal PT-RS if present (all other modulation symbols are set to 0 V),

-	nominal carrier frequency,

-	nominal amplitude and phase for each applicable subcarrier,

-	nominal timing.

2.	Perform time averaging at each reference signal subcarrier of the complex ratios, the time-averaging length is 10ms measurement interval. Prior to the averaging of the phases  an unwrap operation must be performed according to the following definition: The unwrap operation corrects the radian phase angles of  by adding multiples of 2*PI when absolute phase jumps between consecutive time instances ti are greater than or equal to the jump tolerance of PI radians. This process creates an average amplitude and phase for each reference signal subcarrier (i.e. every second subcarrier).

af=i=1Nati,fN
φf=i=1Nφti,fN

Where N is the number of reference signal time-domain locations ti from Z't,f for each reference signal subcarrier .

3.	The equalizer coefficients for amplitude and phase  and  at the reference signal subcarriers are obtained by computing the moving average in the frequency domain of the time-averaged reference signal subcarriers, i.e. every second subcarrier. The moving average window size is 19 and averaging is over the DM-RS subcarriers in allocated RBs. For DM-RS subcarriers at or near the edge of the channel, or when the number of available DM-RS subcarriers within a set of contiguously allocated RBs is smaller than the moving average window size, the window size is reduced accordingly as per figure C.6-1.

4.	Perform linear interpolation from the equalizer coefficients  and  to compute coefficients ,  for each subcarrier. To account for the common phase error (CPE) experienced in millimetre wave frequencies, φ(f), in the estimated coefficients contain phase rotation due to the CPE, θ, in addition to the phase of the equalizer coefficient φf, that is

φf=φf+θ(t)

For OFDM symbols where PT-RS does not exist, θ(t) can be estimated by performing linear interpolation from neighboring symbols where PT-RS is present.

In order to separate component of the CPE, θ, contained in, φ(f), estimation and compensation of the CPE needs to follow. θ(t) is the common phase error (CPE), that rotates all the subcarriers of the OFDM symbol at time t.

Estimate of the CPE, θ(t), at OFDM symbol time, t, can then be obtained from using the PT-RS employing the expression

θ(t)=argf∈fptrsZ't,fIptrst,f afe-jφf

In the above equation, fptrs is the set of subcarriers where PT-RS are mapped, t∈tptrs where  tptrs is the set of OFDM symbols where PT-RS are mapped while Z't,f and Iptrst,f are is the post-FFT acquired signal and the ideal PT-RS signal respectively. That is, estimate of the CPE at a given OFDM symbol is obtained from frequency correlation of the complex ratios at the PT-RS positions with the conjugate of the estimated equalizer complex coefficients. The estimated CPE can be subtracted from φf to remove influence of the CPE, and obtain estimate of the complex coefficient’s phase

φf=φf-θ(t)

The subsequent 7 subcarriers are averaged over 5, 7 .. 17 subcarriers

From the 10th subcarrier onwards the window size is 19 until the upper edge of the channel is reached and the window size reduces back to 1

The first reference subcarrier is not averaged

The second reference subcarrier is the average of the first three subcarriers

Reference subcarriers

Reference subcarriers

The first, second and third reference subcarriers are the average of the five subcarriers on left

- Moving averaging at channel edge

- Moving averaging for the case when the number of available reference subcarriers within a set of contiguously allocated RBs is smaller than the moving average window size

The first, second and third reference subcarriers are the average of the five subcarriers on right

Figure B shows an example of 1RB allocation using a reduced window size of five subcarriers for averaging. The same method applies for RB allocations with fewer than 19 subcarriers available for the moving average size. For the case of 2 and 3 RB allocations, 11 and 17 are the window sizes, respectively.

Figure C.6-1: Reference subcarrier smoothing in the frequency domain