3GPP TSG-RAN WG4 Meeting #96-e                                                       R4-2012632

Electronic Meeting, 17th Aug 2020 - 28th Aug 2020

Agenda Item:	7.4.2.2.2

Source: 	Ericsson

Title:	TP to TR 38.809: ACS and IBB

Document for:	Approval

- Introduction

After RAN4#94bis-e the IAB TR [1] is email approved. Section 2 provides text proposal on 10 OTA ACS for the IAB TR 38.xxx (Integrated access and backhaul radio transmission and reception).

The comments received in 1st round discussion [2] during RAN4#96e:

| CR/TP number | Comments collection |
| --- | --- |
| R4-2011036 | Huawei: The conclusion under the table seems to contradict the agreement (i.e. to use the BS requirements). The statement about traditionally using 99.99% is less correct than it was, for AAS and hence NR we have been using lower probabilities. |

# Text Proposal

--------------------------------------------------Start of TP------------------------------------------------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	38.817-02, General aspects for Base Station (BS) Radio Frequency (RF) for NR

#### 10.5	OTA in-band selectivity and blocking

## 10.5	OTA in-band selectivity and blocking

Detailed structure of the subclause is TBD.

#### 10.5.1 ACS for IAB MT of IAB type 2-O

The ACS requirement for IAB-MT is agreed as 24 dB in 24.25 – 33.4 GHz BS and 23 dB in 37 – 52.6 GHz. IAB-MT is expected to have better adjacent carrier selectivity than UE ACS due to the limited physical separation distance to the aggressor BS. Generically BS has higher ACS selectivity compared to UE thus BS type of ACS number can be used for IAB-MT.

#### 10.5.2 In-band blocking for IAB-MT

Traditionally for UTRA the in-band blocker is set according to the 99.99% probability of interferer cdf curve. It is recommended in [2] that between 99% and 99.9% could be considered due to the NR OFDMA scheme does not suffer greatly as for WCDMA.

As the interfere level to IAB-MT receiver relates to the distance between victim IAB and aggressor BS, coexisting simulation is done assuming the 40m, 50m , 60m and 80m and companies result is captured in Table 10.5.3-1.

Table 10.5.3-1: Coexisting simulation result for In-band blocking level comparision

| Company |  | Blocker level @99 Percentile point (dBm) |  | Blocker level @ 99.9 perncentile point (dBm) |  | Physical distance to Agressor BS (m) |  | Simulation scenario (Acc. to R4-1907825) |  | RAN4 contribution |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ericsson | -67.5 | -65.5 | 40 | Layout 2 for FR2 | R4-2001873 |
|  | -75 | -66 | 50 |  |  |
|  | -81 | -75 | 60 |  |  |
|  | -55 | -48 | 40 | Layout 2 for FR1 |  |
| Nokia | -52 | -45 | 40 | Layout 2 for FR2 | R4-2001432 |
|  | -55 | -54 | Max grid shift |  |  |
| Qualcommn incopration | -45 |  | 40 | Layout 2 for FR2 | R4-2001282 |
|  | -49 |  | 60 |  |  |
|  | -53 |  | 80 |  |  |
| Huawei | -64 |  | 40 | Layout 2 for FR2 (element OTA result) | R4-1914757 |
|  | -69 |  | 60 |  |  |
| ZTE | -42 |  | 40 | Layout 2 for FR1(element OTA result) | R4-2000977 |

It is concluded from the coexisting simulation results in Table 10.5.3-1 that IAB-MT can reuse the OTA BS blocking requirement with CP-OFDM interferer signal. Therefore, the inband blocking level for wide area IAB-MT of type 2-O is specified with 33 dB higher than OTA REFSENS power level with inteferer signal of CP-OFDM waveform. The inband blocking level for local area IAB-MT of type 2-O is specified with 35.5 higher than OTA REFSENS power level with inteferer signal of CP-OFDM waveform for bands n257, n258, n261. The inband blocking level for local area IAB-MT of type 2-O is specified with 34.5 higher than OTA REFSENS power level with CP-OFDM waveform of inteferer signal for bands n260.

--------------------------------------------------End of TP------------------------------------------------------

- References

- TR38.809,

- R4-2012548,	Email discussion summary for [96e][310] NR_IAB_RF_Part_3, samsung