3GPP TSG-RAN WG4 Meeting #96-e	R4-2012658

Electronic Meeting, August 17th – 28th 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | - | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TS 38.141-2: Introduction of performance requirements of PUSCH repetition type A and PUSCH mapping type B for URLLC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_L1enh_URLLC-Perf |  |  |  |  |  | Date: |  |  | 2020-08-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR introduces performance requirements of PUSCH repetition Type A and PUSCH mapping type B with low number of symbols for URLLC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add new section 8.2.7 for PUSCH performance requirements with aggregation factor configured for FR1. - Add new section 8.2.8 for PUSCH performance requirements for mapping type B with non-slot transmission for FR1. - Add test tolerance for PUSCH performance requirements for repetition Type A and mapping type B with low number of symbols for FR1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Performance of PUSCH repetition Type A for FR1 and PUSCH mapping type B with non-slot transmission for FR1 are not ensured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.7, 8.2.8, C.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.104 |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.141-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R4-2011396 |  |  |  |  |  |  |  |  |

--------------Start of text proposal-------------

8.2.7	Performance requirements for PUSCH repetition Type A

8.2.7.1	Definition and applicability

The performance requirement of PUSCH repetition Type A is determined by block error probability (BLER). The performance is measured by the required SNR at block error probability of PUSCH data not exceeding 1 %. The performance requirements assume HARQ re-transmissions.

Which specific test(s) are applicable to BS is based on the test applicability rules defined in subclause 8.1.2.1.

8.2.7.2	Minimum Requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2], subclause 11.2.1.7.

8.2.7.3	Test purpose

The test shall verify the receiver's ability to achieve target block error probability of PUSCH repetition Type A under multipath fading propagation conditions for a given SNR.

8.2.7.4	Method of test

8.2.7.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier: M, see subclause 4.9.1.

RF channels to be tested for carrier aggregation: MBW Channel CA; see subclause 4.9.1.

Direction to be tested: OTA REFSENS receiver target reference direction (see D.54 in table 4.6-1).

8.2.7.4.2	Procedure

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.3.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.3. Each of the demodulation branch signals should be transmitted on one polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A, and according to additional test parameters listed in table 8.2.7.4.2-1.

Table 8.2.7.4.2-1: Test parameters for testing PUSCH repetition Type A

| Parameter |  | BS type 1-O |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | 0 |
|  | DM-RS sequence generation | NID0=0, nSCID=0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH aggregation factor | 15kHz SCS: n2 for FDD and n8 for TDD 30kHz SCS: n2 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Note 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. Note 2: The intention of this configuration is to have two effective transmissions of the transport block. To achieve this for the standard TDD pattern captured in this table, a value of n8 is necessary, while for FDD a value of n2 is necessary. |  |  |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in subclause 8.2.7.5.1 for BS type 1-O, and that the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 8.2.7.4.2-2.

Table 8.2.7.4.2-2: AWGN power level at the BS input

| BS type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 | 5 | -86.5 - ΔOTAREFSENS dBm / 4.5 MHz |
|  |  | 10 | -83.3 - ΔOTAREFSENS dBm / 9.36 MHz |
|  | 30 | 10 | -83.6 - ΔOTAREFSENS dBm / 8.64 MHz |
|  |  | 40 | -77.2 - ΔOTAREFSENS dBm / 38.16 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.53 in table 4.6-1 and subclause 7.1. |  |  |  |

8)	For reference channels applicable to the BS, measure the BLER.

8.2.7.5	Test Requirement

8.2.7.5.1	Test requirement for BS type 1-O

The block error probability (BLER) measured according to subclause 8.2.7.4.2 shall not exceed 1% for the SNR levels specified in table 8.2.7.5.1-1 to table 8.2.7.5.1-8 for 1Tx.

Table 8.2.7.5.1-1: Test requirements for PUSCH, Type A, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Target BLER | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 1% (Note 1) | G-FR1-A3A-1 | pos1 | TBD |
| Note 1: BLER is defined as residual BLER; i.e. ratio of incorrectly received transport blocks / sent transport blocks, independently of the number HARQ transmission(s) for each transport block. |  |  |  |  |  |  |  |

Table 8.2.7.5.1-2: Test requirements for PUSCH, Type A, 10 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Target BLER | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 1% (Note 1) | G-FR1-A3A-2 | pos1 | TBD |
| Note 1: BLER is defined as residual BLER; i.e. ratio of incorrectly received transport blocks / sent transport blocks, independently of the number HARQ transmission(s) for each transport block. |  |  |  |  |  |  |  |

Table 8.2.7.5.1-3: Test requirements for PUSCH, Type A, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Target BLER | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 1% (Note 1) | G-FR1-A3A-3 | pos1 | TBD |
| Note 1: BLER is defined as residual BLER; i.e. ratio of incorrectly received transport blocks / sent transport blocks, independently of the number HARQ transmission(s) for each transport block. |  |  |  |  |  |  |  |

Table 8.2.7.5.1-4: Test requirements for PUSCH, Type A, 40 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Target BLER | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 1% (Note 1) | G-FR1-A3A-4 | pos1 | TBD |
| Note 1: BLER is defined as residual BLER; i.e. ratio of incorrectly received transport blocks / sent transport blocks, independently of the number HARQ transmission(s) for each transport block. |  |  |  |  |  |  |  |

Table 8.2.7.5.1-5: Test requirements for PUSCH, Type B, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Target BLER | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 1% (Note 1) | G-FR1-A3A-1 | pos1 | TBD |
| Note 1: BLER is defined as residual BLER; i.e. ratio of incorrectly received transport blocks / sent transport blocks, independently of the number HARQ transmission(s) for each transport block. |  |  |  |  |  |  |  |

Table 8.2.7.5.1-6: Test requirements for PUSCH, Type B, 10 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Target BLER | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 1% (Note 1) | G-FR1-A3A-2 | pos1 | TBD |
| Note 1: BLER is defined as residual BLER; i.e. ratio of incorrectly received transport blocks / sent transport blocks, independently of the number HARQ transmission(s) for each transport block. |  |  |  |  |  |  |  |

Table 8.2.7.5.1-7: Test requirements for PUSCH, Type B, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Target BLER | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 1% (Note 1) | G-FR1-A3A-3 | pos1 | TBD |
| Note 1: Calculate the target BLER after all HARQ transmission(s) for one TB. |  |  |  |  |  |  |  |

Table 8.2.7.5.1-8: Test requirements for PUSCH, Type B, 40 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Target BLER | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 1% (Note 1) | G-FR1-A3A-4 | pos1 | TBD |
| Note 1: Calculate the target BLER after all HARQ transmission(s) for one TB. |  |  |  |  |  |  |  |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

8.2.8	Performance requirements for PUSCH mapping type B with non-slot transmission

8.2.8.1	Definition and applicability

The performance requirement of PUSCH mapping type B with low number of symbols is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex A.

Which specific test(s) are applicable to BS is based on the test applicability rules defined in subclause 8.1.2.1.

8.2.8.2	Minimum Requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2], subclause 11.2.1.8.

8.2.8.3	Test purpose

The test shall verify the receiver's ability to achieve throughput of PUSCH mapping type B with low number of symbols under multipath fading propagation conditions for a given SNR.

8.2.8.4	Method of test

8.2.8.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier: M, see subclause 4.9.1.

RF channels to be tested for carrier aggregation: MBW Channel CA; see subclause 4.9.1.

Direction to be tested: OTA REFSENS receiver target reference direction (see D.54 in table 4.6-1).

8.2.8.4.2	Procedure

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.3.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.3. Each of the demodulation branch signals should be transmitted on one polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A, and according to additional test parameters listed in table 8.2.1.4.2-1.

Table 8.2.8.4.2-1: Test parameters for testing PUSCH for mapping type B with non-slot transmission

| Parameter |  | BS type 1-O |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 1 |
|  | RV sequence | 0 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos0 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | 0 |
|  | DM-RS sequence generation | NID0=0, nSCID=0 |
| Time domain resource assignment | PUSCH mapping type | B |
|  | Start symbol | 0 |
|  | Allocation length | 2 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| TPMI index for 2Tx two layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| Note 1: The same requirements are applicable to FDD and TDD with different UL-DL patterns for BS type 1-O. |  |  |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in subclause 8.2.1.5.1 for BS type 1-O, and that the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 8.2.1.4.2-2.

Table 8.2.8.4.2-2: AWGN power level at the BS input

| BS type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 | 5 | -86.5 - ΔOTAREFSENS dBm / 4.5 MHz |
|  |  | 10 | -83.3 - ΔOTAREFSENS dBm / 9.36 MHz |
|  | 30 | 10 | -83.6 - ΔOTAREFSENS dBm / 8.64 MHz |
|  |  | 40 | -77.2 - ΔOTAREFSENS dBm / 38.16 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.53 in table 4.6-1 and subclause 7.1. |  |  |  |

8)	For reference channels applicable to the BS, measure the throughput.

8.2.8.5	Test Requirement

8.2.8.5.1	Test requirement for BS type 1-O

The throughput measured according to subclause 8.2.8.4.2 shall not be below the limits for the SNR levels specified in table 8.2.8.5.1-1 to table 8.2.8.5.1-4 for 1Tx.

Table 8.2.8.5.1-1: Test requirements for PUSCH, Type B, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLC300-100 Low | 70% | G-FR1-A3B-1 | pos0 | TBD |

Table 8.2.8.5.1-2: Test requirements for PUSCH, Type B, 10 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLC300-100 Low | 70% | G-FR1-A3B-2 | pos0 | TBD |

Table 8.2.8.5.1-3: Test requirements for PUSCH, Type B, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLC300-100 Low | 70% | G-FR1-A3B-3 | pos0 | TBD |

Table 8.2.8.5.1-4: Test requirements for PUSCH, Type B, 40 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLC300-100 Low | 70% | G-FR1-A3B-4 | pos0 | TBD |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

--------------Omitted unchanged sections-------------

C.3	Measurement of performance requirements

Table C.3-1: Derivation of test requirements (FR1 OTA performance tests)

| Test | Minimum Requirement in TS 38.104 [2] | Test Tolerance (TTOTA) | Test requirement in the present document |
| --- | --- | --- | --- |
| 8.2.1 Performance requirements for PUSCH with transform precoding disabled | See subclause 11.2.1.1 | 0.6 dB | Formula: SNR + TTOTA T-put limit unchanged |
| 8.2.2 Performance requirements for PUSCH with transform precoding enabled | See subclause 11.2.1.2 | 0.6 dB | Formula: SNR + TTOTA T-put limit unchanged |
| 8.2.3 Performance requirements for UCI multiplexed on PUSCH | See clause 11.2.1.3 | 0.6 dB | Formula: SNR + TTOTA BLER limit unchanged |
| 8.2.4 Performance requirements for PUSCH for high speed train | SNRs as specified | 0.3 dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.7 Performance requirements for PUSCH repetition Type A | See subclause 11.2.1.7 | 0.6 dB | Formula: SNR + TTOTA BLER limit unchanged |
| 8.2.8 Performance requirements for PUSCH mapping type B with low number of symbols | See subclause 11.2.1.8 | 0.6 dB | Formula: SNR + TTOTA T-put limit unchanged |
| 8.3.1 Performance requirements for PUCCH format 0 | See subclause 11.3.1.2 | 0.6 dB | Formula: SNR + TTOTA False ACK limit unchanged Correct ACK limit unchanged |
| 8.3.2 Performance requirements for PUCCH format 1 | See subclause 11.3.1.3 | 0.6 dB | Formula: SNR + TTOTA False ACK limit unchanged False NACK limit unchanged Correct ACK limit unchanged |
| 8.3.3 Performance requirements for PUCCH format 2 | See subclause 11.3.1.4 | 0.6 dB | Formula: SNR + TTOTA False ACK limit unchanged Correct ACK limit unchanged UCI BLER limit unchanged |
| 8.3.4 Performance requirements for PUCCH format 3 | See subclause 11.3.1.5 | 0.6 dB | Formula: SNR + TTOTA UCI BLER limit unchanged |
| 8.3.5 Performance requirements for PUCCH format 4 | See subclause 11.3.1.6 | 0.6 dB | Formula: SNR + TTOTA UCI BLER limit unchanged |
| 8.3.6 Performance requirements for multi-slot PUCCH | See clause 11.3.1.7 | 0.6 dB | Formula: SNR + TTOTA False ACK limit unchanged False NACK limit unchanged Correct ACK limit unchanged |
| 8.4.1 PRACH false alarm probability and missed detection | See subclause 11.4.1 | 0.6 dB for fading cases 0.3 dB for AWGN cases | Formula: SNR + TTOTA PRACH False detection limit unchanged PRACH detection limit unchanged |
| 8.2.5 Performance requirements for UL timing adjustment | See subclause 11.2.1.5 | 0.3 dB for AWGN cases | Formula: SNR + TTOTA T-put limit unchanged |
| NOTE: TT values are applicable for normal condition unless otherwise stated. |  |  |  |

Table C.3-2: Derivation of test requirements (FR2 OTA performance tests)

| Test | Minimum requirement in TS 38.104 [2] | Test Tolerance (TTOTA) | Test requirement in the present document |
| --- | --- | --- | --- |
| 8.2.1 Performance requirements for PUSCH with transform precoding disabled | See subclause 11.2.2.1 | 0.6 dB | Formula: SNR + TTOTA T-put limit unchanged |
| 8.2.2 Performance requirements for PUSCH with transform precoding enabled | See subclause 11.2.2.2 | 0.6 dB | Formula: SNR + TTOTA T-put limit unchanged |
| 8.2.3 Performance requirements for UCI multiplexed on PUSCH | See clause 11.2.2.3 | 0.6 dB | Formula: SNR + TTOTA BLER limit unchanged |
| 8.3.1 Performance requirements for PUCCH format 0 | See subclause 11.3.2.2 | 0.6 dB | Formula: SNR + TTOTA False ACK limit unchanged Correct ACK limit unchanged |
| 8.3.2 Performance requirements for PUCCH format 1 | See subclause 11.3.2.3 | 0.6 dB | Formula: SNR + TTOTA False ACK limit unchanged False NACK limit unchanged Correct ACK limit unchanged |
| 8.3.3 Performance requirements for PUCCH format 2 | See subclause 11.3.2.4 | 0.6 dB | Formula: SNR + TTOTA False ACK limit unchanged Correct ACK limit unchanged UCI BLER limit unchanged |
| 8.3.4 Performance requirements for PUCCH format 3 | See subclause 11.3.2.5 | 0.6 dB | Formula: SNR + TTOTA UCI BLER limit unchanged |
| 8.3.5 Performance requirements for PUCCH format 4 | See subclause 11.3.2.6 | 0.6 dB | Formula: SNR + TTOTA UCI BLER limit unchanged |
| 8.4.1 PRACH false alarm probability and missed detection | See subclause 11.4.2 | 0.6 dB for fading cases 0.3 dB for AWGN cases | Formula: SNR + TTOTA PRACH False detection limit unchanged PRACH detection limit unchanged |
| NOTE: TT values are applicable for normal condition unless otherwise stated. |  |  |  |

--------------End of text proposal-------------