3GPP TSG-RAN WG4 Meeting #96-e	R4-2012696

# E-meeting, 17th – 28nd August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | - | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on FRC for Normal NR CA demodulation requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_perf_enh-Perf |  |  |  |  |  | Date: |  |  | 2020-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Definition of FRCs for Normal CA requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added FRCs for FR1 FDD with 15 kHz SCS, FR1 TDD with 15 kHz SCS, FR1 TDD with 30 kHz SCS and FR2 TDD with 120 kHz SCS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FRCs for Normal CA requirements are not defined |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.2.1.1, A.3.2.2.1, A.3.2.2.2, A.3.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

## A.3.2	Reference measurement channels for PDSCH performance requirements

For PDSCH reference channels if more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit).

### A.3.2.1	FDD

#### A.3.2.1.1	Reference measurement channels for SCS 15 kHz FR1

<SKIP UNCHANGED PART>

Table A.3.2.1.1-9: PDSCH Reference Channel for FDD CC and CA scenario

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-9.1 FDD | R.PDSCH.1-9.2 FDD | R.PDSCH.1-9.3 FDD | R.PDSCH.1-9.4 FDD | R.PDSCH.1-9.5 FDD |
| Channel bandwidth | MHz | 5 | 15 | 20 | 25 | 30 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 |
| Number of allocated resource blocks | PRBs | 25 | 79 | 106 | 133 | 160 |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per 2 frames | Slots | 19 | 19 | 19 | 19 | 19 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 13 | 13 | 13 | 13 | 13 |
| Modulation |  | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Target Coding Rate |  | 0.48 | 0.48 | 0.48 | 0.48 | 0.48 |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 | 2 |
| Number of DMRS REs |  | 12 | 12 | 12 | 12 | 12 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | Bits | 12552 | 39936 | 53288 | 67584 | 79896 |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | Bits | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | CBs | 2 | 5 | 7 | 9 | 10 |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 10, 11 | Bits | 25200 | 79632 | 106848 | 134064 | 161280 |
| For Slots i =1,…, 9, 12, …, 19 | Bits | 26400 | 83424 | 111936 | 140448 | 168960 |
| Max. Throughput averaged over 2 frames | Mbps | 11.924 | 37.939 | 50.624 | 64.205 | 75.901 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.1.1-10: PDSCH Reference Channel for FDD CC and CA scenario

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-10.1 FDD | R.PDSCH.1-10.2 FDD |  |  |  |
| Channel bandwidth | MHz | 40 | 50 |  |  |  |
| Subcarrier spacing | kHz | 15 | 15 |  |  |  |
| Number of allocated resource blocks | PRBs | 216 | 270 |  |  |  |
| Number of consecutive PDSCH symbols |  | 12 | 12 |  |  |  |
| Allocated slots per 2 frames | Slots | 19 | 19 |  |  |  |
| MCS table |  | 64QAM | 64QAM |  |  |  |
| MCS index |  | 13 | 13 |  |  |  |
| Modulation |  | 16QAM | 16QAM |  |  |  |
| Target Coding Rate |  | 0.48 | 0.48 |  |  |  |
| Number of MIMO layers |  | 2 | 2 |  |  |  |
| Number of DMRS REs |  | 12 | 12 |  |  |  |
| Overhead for TBS determination |  | 0 | 0 |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |  |  |  |
| For Slots i = 1,…, 19 | Bits | 108552 | 135296 |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |  |  |  |
| For Slots i = 1,…, 19 | Bits | 24 | 24 |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A |  |  |  |
| For Slots i = 1,…, 19 | CBs | 13 | 17 |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |  |  |  |
| For Slots i = 10, 11 | Bits | 217728 | 272160 |  |  |  |
| For Slots i =1,…, 9, 12, …, 19 | Bits | 228096 | 285120 |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 103.124 | 128.531 |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

<SKIP UNCHANGED PART>

### A.3.2.2	TDD

#### A.3.2.2.1	Reference measurement channels for SCS 15 kHz FR1

<SKIP UNCHANGED PART>

Table A.3.2.2.1-2: PDSCH Reference Channel for TDD CC with UL-DL pattern FR1.15-1 and CA scenario

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-2.1 TDD | R.PDSCH.1-2.2 TDD | R.PDSCH.1-2.3 TDD | R.PDSCH.1-2.4 TDD | R.PDSCH.1-2.5 TDD |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 | 25 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 |
| Number of allocated resource blocks | PRBs | 25 | 52 | 79 | 106 | 133 |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} |  | 8 | 8 | 8 | 8 | 8 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} |  | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per 2 frames | Slots | 15 | 15 | 15 | 15 | 15 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 13 | 13 | 13 | 13 | 13 |
| Modulation |  | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Target Coding Rate |  | 0.48 | 0.48 | 0.48 | 0.48 | 0.48 |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 | 2 |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} |  | 12 | 12 | 12 | 12 | 12 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} |  | 12 | 12 | 12 | 12 | 12 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | Bits | 8064 | 16896 | 25608 | 33816 | 43032 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} | Bits | 12552 | 26120 | 39936 | 53288 | 67584 |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | Bits | 24 | 24 | 24 | 24 | 24 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} | Bits | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | CBs | 1 | 3 | 4 | 5 | 6 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} | CBs | 2 | 4 | 5 | 7 | 9 |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 10, 11 | Bits | 25200 | 52416 | 79632 | 106848 | 134064 |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | Bits | 16800 | 34944 | 53088 | 71232 | 89376 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,9,12,…,19} | Bits | 26400 | 54912 | 83424 | 111936 | 140448 |
| Max. Throughput averaged over 2 frames | Mbps | 8.516 | 17.745 | 27.086 | 36.072 | 45.778 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.2.1-3: PDSCH Reference Channel for TDD CC with UL-DL pattern FR1.15-1 and CA scenario

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-3.1 TDD | R.PDSCH.1-3.2 TDD | R.PDSCH.1-3.3 TDD |  |  |
| Channel bandwidth | MHz | 30 | 40 | 50 |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 |  |  |
| Number of allocated resource blocks | PRBs | 160 | 216 | 270 |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} |  | 8 | 8 | 8 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} |  | 12 | 12 | 12 |  |  |
| Allocated slots per 2 frames | Slots | 15 | 15 | 15 |  |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |  |
| MCS index |  | 13 | 13 | 13 |  |  |
| Modulation |  | 16QAM | 16QAM | 16QAM |  |  |
| Target Coding Rate |  | 0.48 | 0.48 | 0.48 |  |  |
| Number of MIMO layers |  | 2 | 2 | 2 |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} |  | 12 | 12 | 12 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} |  | 12 | 12 | 12 |  |  |
| Overhead for TBS determination |  | 0 | 0 | 0 |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | Bits | 51216 | 69672 | 86040 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} | Bits | 79896 | 108552 | 135296 |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | Bits | 24 | 24 | 24 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} | Bits | 24 | 24 | 24 |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | CBs | 7 | 9 | 11 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} | CBs | 10 | 13 | 17 |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A |  |  |
| For Slots i = 10, 11 | Bits | 161280 | 217728 | 272160 |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | Bits | 107520 | 145152 | 181440 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,9,12,…,19} | Bits | 168960 | 228096 | 285120 |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 54.186 | 73.638 | 91.621 |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

#### A.3.2.2.2	Reference measurement channels for SCS 30 kHz FR1

<SKIP UNCHANGED PART>

Table A.3.2.2.2-13: PDSCH Reference Channel for TDD CC with UL-DL pattern FR1.30-1 and CA scenario

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-13.1 TDD | R.PDSCH.2-13.2 TDD | R.PDSCH.2-13.3 TDD | R.PDSCH.2-13.4 TDD | R.PDSCH.2-13.5 TDD |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 | 25 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 | 30 |
| Allocated resource blocks | PRBs | 11 | 24 | 38 | 51 | 65 |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 4 | 4 | 4 | 4 | 4 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per 2 frames |  | 31 | 31 | 31 | 31 | 31 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 13 | 13 | 13 | 13 | 13 |
| Modulation |  | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Target Coding Rate |  | 0.48 | 0.48 | 0.48 | 0.48 | 0.48 |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 | 2 |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 6 | 6 | 6 | 6 | 6 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 12 | 12 | 12 | 12 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 1800 | 3840 | 6144 | 8192 | 10504 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 5504 | 12040 | 18960 | 25608 | 32776 |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 16 | 24 | 24 | 24 | 24 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | 1 | 1 | 1 | 1 | 2 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | CBs | 1 | 2 | 3 | 4 | 4 |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 20, 21 | Bits | 11088 | 24192 | 38304 | 51408 | 65520 |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 3696 | 8064 | 12768 | 17136 | 21840 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 11616 | 25344 | 40128 | 53856 | 68640 |
| Max. Throughput averaged over 2 frames | Mbps | 7.790 | 17.022 | 26.825 | 36.209 | 46.348 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.2.2-14: PDSCH Reference Channel for TDD CC with UL-DL pattern FR1.30-1 and CA scenario

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-14.1 TDD | R.PDSCH.2-14.2 TDD | R.PDSCH.2-14.3 TDD | R.PDSCH.2-14.4 TDD | R.PDSCH.2-14.5 TDD |
| Channel bandwidth | MHz | 30 | 50 | 60 | 80 | 90 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 | 30 |
| Allocated resource blocks | PRBs | 78 | 133 | 162 | 217 | 245 |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 4 | 4 | 4 | 4 | 4 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per 2 frames |  | 31 | 31 | 31 | 31 | 31 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 13 | 13 | 13 | 13 | 13 |
| Modulation |  | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Target Coding Rate |  | 0.48 | 0.48 | 0.48 | 0.48 | 0.48 |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 | 2 |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 6 | 6 | 6 | 6 | 6 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 12 | 12 | 12 | 12 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 12552 | 21504 | 26120 | 34816 | 38936 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 38936 | 67584 | 81976 | 110632 | 122976 |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 24 | 24 | 24 | 24 | 24 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | 2 | 3 | 4 | 5 | 5 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | CBs | 5 | 9 | 10 | 14 | 15 |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 20, 21 | Bits | 78624 | 134064 | 163296 | 218736 | 246960 |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 26208 | 44688 | 54432 | 72912 | 82320 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 82368 | 140448 | 171072 | 229152 | 258720 |
| Max. Throughput averaged over 2 frames | Mbps | 55.074 | 95.539 | 115.892 | 156.316 | 173.805 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.2.2-15: PDSCH Reference Channel for TDD CC with UL-DL pattern FR1.30-1 and CA scenario

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-15.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 100 |  |  |  |  |
| Subcarrier spacing | kHz | 30 |  |  |  |  |
| Allocated resource blocks | PRBs | 273 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 4 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 |  |  |  |  |
| Allocated slots per 2 frames |  | 31 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 13 |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |
| Target Coding Rate |  | 0.48 |  |  |  |  |
| Number of MIMO layers |  | 2 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 6 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 44040 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 139376 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 24 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | 6 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | CBs | 17 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slots i = 20, 21 | Bits | 275184 |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 91728 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 288288 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 196.966 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

<SKIP UNCHANGED PART>

#### A.3.2.2.5	Reference measurement channels for SCS 120 kHz FR2

<SKIP UNCHANGED PART>

Table A.3.2.2.5-9: PDSCH Reference Channel for TDD CC with UL-DL pattern FR2.120-1 and CA scenario

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-9.1 TDD | R.PDSCH.5-9.2 TDD | R.PDSCH.5-9.3 TDD | R.PDSCH.5-9.4 TDD |  |
| Channel bandwidth | MHz | 50 | 100 | 200 | 400 |  |
| Subcarrier spacing | kHz | 120 | 120 | 120 | 120 |  |
| Allocated resource blocks | PRBs | 32 | 66 | 132 | 264 |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 9 | 9 | 9 | 9 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} |  | 13 | 13 | 13 | 13 |  |
| Allocated slots per 2 frames |  | 127 | 127 | 127 | 127 |  |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM |  |
| MCS index |  | 10 | 10 | 10 | 10 |  |
| Modulation |  | 16QAM | 16QAM | 16QAM | 16QAM |  |
| Target Coding Rate |  | 0.33 | 0.33 | 0.33 | 0.33 |  |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 12 | 12 | 12 | 12 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} |  | 12 | 12 | 12 | 12 |  |
| Overhead for TBS determination |  | 6 | 6 | 6 | 6 |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 7680 | 15880 | 31752 | 63528 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | Bits | 11784 | 24072 | 48168 | 96264 |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 24 | 24 | 24 | 24 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | Bits | 24 | 24 | 24 | 24 |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | CBs | 1 | 2 | 4 | 8 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | CBs | 2 | 3 | 6 | 12 |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |  |
| For Slots i = 80, 81 | Bits | 33920 | 69960 | 139920 | 279840 |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 23680 | 48840 | 97680 | 195360 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,79,84,…,159} | Bits | 35456 | 73128 | 146256 | 292512 |  |
| Max. Throughput averaged over 2 frames | Mbps | 68.262 | 139.750 | 279.601 | 558.899 |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

END OF CHANGE