3GPP TSG-RAN WG4 Meeting #96-e	R4-2012735

# Electronic Meeting, 17th - 28th Aug, 2020

Agenda item:			7.8.1.2

Source:	Moderator (Huawei, HiSilicon)

Title:	Email discussion summary for [96e][319] NR_L1enh_URLLC_Demod_Part2

Document for:	Information

Introduction

The discussions in this thread include URLLC UE and BS demodulation requirements for high reliability with higher BLER and low latency and UE CQI reporting requirements. The discussion about UE and BS demodulation requirements for high reliability with BLER 10^-5 and confidence level 99.999% will happen in another thread RAN4 [96e][318] NR_L1enh_URLLC_Demod_Part1. The work plan for URLLC WI will be discussed in Topic #1, and issues of URLLC UE will be discussed from Topic #2 to Topic #7. Issues about URLLC BS will be discussed from Topic #8 to Topic #11.

Besides, there are some CRs which contain contents of both thread [318] and [319]. In this thread, the common issues (such as FRCs etc.) will be discussed. In thread [318], the test methodology and ultra-low BLER will be discussed. Please comment the following issues in this summary:

- CR structures for UE and BS.

- Discuss CRs for all common issues (such as FRCs etc.)

- Discuss CRs for the PDSCH repetition, mapping type B and processing capability 2 and pre-emption.

- Discuss CRs for the PUSCH repetition type A and low latency (i.e. type B) requirements.

Topics in this summary:

- Topic #1: Work plan for URLLC WI

URLLC UE:

- Topic #2: UE demodulation requirements for high reliability with higher BLER

- Topic #3: UE demodulation requirements for mapping Type B and processing capability 2.

- Topic #4: UE demodulation requirements for pre-emption.

- Topic #5: CQI reporting requirements for supporting CQI table 3.

- Topic #6: URLLC UE Rel-16 features.

- Topic #7: Test applicability, release independent for URLLC UE.

URLLC BS:

- Topic #8: BS demodulation requirements for high reliability.

- Topic #9: BS demodulation requirements for low latency.

- Topic #10: URLLC BS Rel-16 features

- Topic #11: BS demod CR work split for FR2.

Topic #1: Work plan for URLLC WI

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2010994 | Huawei, HiSilicon | Work plan for Physical layer enhancements for NR ultra-reliable and low latency communication |

Open issues summary

Sub-topic 1-1: Work plan for URLLC WI

The URLLC WI RP-191584 was agreed during RAN #84. Its objectives are listed below：

Phase 1:

- Study the test methodology for both BS and UE [RAN4]

  - Test methodology for the test metric of 99.999% reliability with testing time into consideration

  - Test methodology for low latency requirements

Phase 2:

Specify the following performance requirements based on Rel-15 URLLC functionalities [RAN4]

- Investigate and specify the RLM test cases

- Study and specify the US/BS demodulation performance and UE CQI reporting requirements for high reliability

  - The following candidate features related to high reliability should be further identified and prioritized

    - PDSCH repetitions over multiple slots

    - PUSCH repetitions over multiple slots

    - 4-bit CQI Table 3

    - MCS index table 3

    - Other features are not precluded

- Study and specify the UE/BS demodulation performance and UE CQI reporting requirements for low latency

  - The following candidate features related to low latency should be further identified and prioritized

    - PDSCH processing capability 2

    - Self-contained slot and/or non slot for DL

    - PDSCH and PUSCH mapping type A/B

    - Pre-emption indication for DL

    - Other features are not precluded

Specify the following performance requirements based on Rel-16 URLLC functionalities [RAN4]

- Base station demodulation performance requirements

- UE demodulation performance requirements

- Base station conformance testing

According to the WI, the progress has been delayed. In R4-2010994, a detailed work plan was proposed for the following three meetings.

Issue 1-1-1: Work plan for URLLC WI

- Proposals

RAN4#96:

- Agree on work plan.

- Discuss on Rel-16 URLLC features.

- Agree on remained open issues for Rel-15 URLLC features.

- Simulation results alignment on Rel-15 test cases.

- Discussion and agree on Rel-15 CRs which have been assigned.

RAN4#97:

- Agree on Rel-16 URLLC test cases and simulation assumptions.

- Agree on Rel-16 CR split.

- Agree on Rel-15 URLLC CRs

RAN4#98:

- Simulation results alignment on Rel-16 test cases.

- Discussion and agree on Rel-16 CRs

- Recommended WF

  - During #96 meeting, we recommend to follow the work plan for RAN4 #96.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| XXX | Issue 1-1-1 |
| Nokia, Nokia Shanghai Bell | Issue 1-1-1: Work plan for URLLC WI We agree with following the workplan for RAN4#96e. However, we caution that the Rel-16 feature discussion is unlikely to result in agreed CRs until RAN4#97. Even the agreement to Rel-15 CRs until RAN4#97 seems doubtful for some controversial issues (e.g., FR2) and more extensive CRs (e.g., statistical testing). |
| Ericsson | Issue 1-1-1: There are still some results and impairment incomplete for FR1 Rel-15 requirements, so it may take another meeting cycle to finalize results. We also need to agree on FR2 parameters and so FR2 simulations and FRC; this is not included in the work plan. |
| Samsung | Generally, we are fine this work plan Based on the WID of Rel-16 URLLC, the performance requirements, including UE/BS demodulation requirements, will be specified. Considering there are still some open issues existed based on Rel-15 URLLC feature, we prefer to deprioritize the requirement specified for Rel-16 as least in Q3. As agreed in previous meeting, there is no impact on RRM core requirement based on Rel-15 URLLC functionalities. As indicated in the WID, although there is no objective to specify the URLLC core requirements based on Rel-16 URLLC functionalities. We wonder whether there is impact on RRM core requirement should be clarified. Compared with Rel-15, the high reliability requirement and lower latency requirement are further tighter, e.g., higher reliability (up to 10^-6) and short latency in the order of 0.5 to 1ms. Whether we should define the high reliability requirement with BLER 10^-6? If RAN4 agree to introduce the requirement with BLER 10^-6, from the test methodology perspective, whether the existed methodology for 99.999% can be applied for 99.9999% should be further studied? |
| Qualcomm | We understand that this is probably the only possible work plan , given the timeline agreed in plenary. However, we don’t think we can finish Rel-16 requirements that quickly, when we haven’t even started discussing them properly. 1e-6 BLER alone may need a lot of discussion. |
| Apple | We don’t have a conclusion on CQI reporting testcases yet and results alignment will take another meeting, once we conclude on the testcase definition. The work plan needs to be adjusted for that for Rel-15 requirements. If we have additional testcases for Rel-16, we would need at least 2 meetings to finalize requirements. |
| Intel | Taking into account that focus of this meeting is finalization of URLLC requirements for Rel-15 features, it is rather hard to have comprehensive discussion on Rel-16 URLLC features. At least, we can list all Rel-16 feature and companies can provide view in the next RAN4 meeting. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Work plan for URLLC WI: Considering we are still discuss the CQI test and FR2 parameters, the work plan will not be continue to discuss in 2nd round. Candidate options: Recommendations for 2nd round: |

Topic #2: UE performance requirements for high reliability

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2009612 | Apple | Proposal #1: Define requirements with PDSCH slot aggregation with MCS 13 Proposal #2: Do no define requirements in FR2 for high reliability for URLLC |
| R4-2009725 | Intel Corporation | Proposal 1: Use the following assumptions for FR1 High reliability PDSCH requirements: - Use 2 HARQ process for FDD mode and 4 HARQ process for TDD mode - MCS 13 Proposal 3: Define FR2 demodulation requirements to verify that UE supports PDSCH repetitions. |
| R4-2010981 | Huawei, HiSilicon | Proposal 1: We propose to use MCS13 for this case. Proposal 2: We prefer option 3 that select the MCS higher or equal to -4 dB for final 4 Rx requirement definition (average ideal SNR alignment result + IM). Proposal 3: Do not define UE FR2 URLLC requirements for high reliability. |
| R4-2010982 | Huawei, HiSilicon | Simulation results on URLLC UE demodulation requirements with higher BLER |
| R4-2010986 | Huawei, HiSilicon | CR to TS38.101-4 Applicability rules for URLLC UE demodulation requirements |
| R4-2010987 | Huawei, HiSilicon | CR to TS 38.101-4 Performance requirements for URLLC PDSCH repetitions over multiple slots. |
| R4-2011046 | NTT DOCOMO, INC. | Proposal 4: Define the FR2 requirements for high reliability with higher BLER |
| R4-2011371 | Ericsson | Proposal 2: Configure MCS 19 for slot aggregation test. Observations 1: The maximum throughput needs to be adjusted according to aggregation factor in table 3-1 and table 3-2 in [2]. Proposal 1: Select an MCS which gives higher or equal to -4 dB for final 4 Rx requirement definition (average ideal SNR alignment result + IM) Proposal 6: Specify FR2 Slot aggregation test case Proposal 7: Configure FR2 Slot aggregation with TDLA30-300, MCS19, TDD pattern DDSU, 100MHz/120kHz |
| R4-2011445 | Qualcomm Incorporated | Proposal 1: Do not define URLLC high reliability high BLER test cases for FR2. Proposal 2: Define high reliability high BLER tests with MCS 19 in Low SE MCS Table. |

Open issues summary

During the last meeting, most of the test parameters for FR1 were agreed. In this section, the remained open issues will be discussed.

From the approved WF R4-2008807 in RAN4 #95 e-meeting, following were agreed:

Agreements of #95-e

      - Scheduling for PDSCH:

  - FDD: No scheduling in slots 0 and 1 (or 19) within 20ms.

  - TDD: No scheduling in D slots i, where mod(i, 10) = 0, and S slots

In this section, the following open issues will be discussed:

Open issues of #95-e:

      - MCS:

  - Option 1: MCS13

  - Option 2: MCS16

  - Option 3: MCS19

- Methodology for MCS selection

  - Option 1: Higher or equal to -6 dB for average ideal 4 Rx SNR alignment results.

  - Option 2: Higher or equal to -3 dB for average ideal 4 Rx SNR alignment results

  - Option 3: Higher or equal to -4 dB for final 4 Rx requirement definition (average ideal SNR alignment result + IM)

      - FR2 requirements for High reliability

  - FFS whether to define FR2 requirements

  - Interested companies are encouraged to bring more information and analysis for the deployment/usage scenarios

Sub-topic 2-1: UE demodulation requirements for high reliability for FR1

Open issues and candidate options before e-meeting:

Issue 2-1-1: MCS (table 3)

- Proposals

  - Option 1: MCS13 (Apple, Intel, Huawei)

  - Option 2: MCS16

  - Option 3: MCS19 (Ericsson, QC)

- Recommended WF

  - Before make an agreement of this issue, companies should align the BLER calculation method in Issue 2-1-1A.

Issue 2-1-1A: BLER calculation method

For PDSCH high reliability test case, the test metrics is agreed as 1% BLER (calculated after all transmission). As the simulation results are quite divergent, the purpose of this issue is to ensure we have the same understanding of the BLER calculation. For example, the maximum number of HARQ is 4，if an information block is transmitted and succeed with HARQ index of 3, the BLER is

Method 1: BLER =0, because the TB is successfully transmitted through the transmissions are failed for three times.

Method 2: BLER=3/4. Counting the BLER residue for each HARQ transmission. Here, BLER= 3/4, as the TB is failed for three times and it is transmitted for 4 times.

- Proposals

  - Option 1: Method 1. (Intel)

  - Option 2: Method 2 (Apple)

  - Option 3: Other method which is not precluded.

- Recommended WF

  - TBD

Issue 2-1-2: Methodology for MCS selection

- Proposals

  - Option 1: Higher or equal to -6 dB for average ideal 4 Rx SNR alignment results.

  - Option 2: Higher or equal to -3 dB for average ideal 4 Rx SNR alignment results

  - Option 3: Higher or equal to -4 dB for final 4 Rx requirement definition (average ideal SNR alignment result + IM) (Huawei, Ericsson, QC, Intel)

- Recommended WF

  - Option 3

Issue 2-1-3: HARQ process number

- Proposals

  - Option 1: 2 for FDD and 4 for TDD (Intel, Ericsson)

  - Option 2:

- Recommended WF

  - Option 1

Sub-topic 2-2: UE demodulation requirements for high reliability for FR2

Issue 2-2-1: Whether to define URLLC high reliability requirements for FR2

- Proposals

  - Option 1: Yes (Intel, DoCoMo, Ericsson)

  - Option 2: No (Apple, Huawei, QC)

  - Option 3: Yes with test applicability rule.

- Recommended WF

  - TBD

Issue 2-2-2: Test applicability rule for FR2 (only if FR2 is defined)

- Proposals

  - Option 1: The performance requirements are only applicable for UE supporting FR2 operating bands. (Ericsson, Huawei, Intel)

  - Option 2:

- Recommended WF

  - TBD

Issue 2-2-3: Test applicability rule for FR1 and FR2 if UE support both (only if FR2 is defined)

- Proposals

  - Option 1: UE should be tested only for FR1 if UE support both FR1 and FR2.

  - Option 2: UE should be tested for both FR1 and FR2 if UE support FR1 and FR2 (DoCoMo, Intel)

- Recommended WF

  - TBD

Issue 2-2-4: Channel model

- Proposals

  - Option 1: TDLA30-300 (Ericsson)

  - Option 2:

- Recommended WF

  - TBD

Issue 2-2-5: TDD pattern

- Proposals

  - Option 1: DDSU (Ericsson)

  - Option 2: DDDSU (DoCoMo)

- Recommended WF

  - TBD

Issue 2-2-7: MCS

- Proposals

  - Option 1: MCS19 (Ericsson)

  - Option 2: Reuse MCS from FR1 (Intel)

- Recommended WF

  - TBD

Issue 2-2-8: Bandwidth

- Proposals

  - Option 1: 100MHz (Ericsson, DoCoMo)

  - Option 2:

- Recommended WF

  - TBD

Issue 2-2-9: SCS

- Proposals

  - Option 1: 120kHz (Ericsson, DoCoMo)

  - Option 2:

- Recommended WF

  - TBD

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| XXX | Issue 1-1-1: Issue 1-1-2: Issue 1-1-3: |
| Ericsson | Issue 2-1-1: We should agree issue 2-1-2 first and then come back to look at the results in order to select the MCS. The results seem quite divergent and need some more discussion (e.g. if everyone has the same understanding of 1% BLER). Issue 2-1-2: Option 1 and 3 are equal if we would assume 2dB implementation margin. We propose option 3 as it relates to actual SNR setting. Issue 2-1-3: OK for option 1 Issue 2-2-1: We do not believe that ultra-low BLER in FR2 is realistic, but we think that there are high reliability cases for less stringent latency/BLER that are relevant. Issue 2-2-2: For sure requirements for FR2 should only be appliable for UEs that support FR2. Issue 2-2-6: What is the difference to 2-2-4 ? |
| Qualcomm | Issue 2-1-1: We prefer MCS19 based on our simulation results to avoid defining requirements at very low SNR. Issue 2-1-2: Prefer Option 3. Issue 2-2-1: Prefer Option 2. In practice, it will be difficult to achieve high reliability for FR2. Any temporary blockage such as human tissue will result in failure of high reliability. Issue 2-2-2 to 2-2-9: We can comment after we decide on Issue 2-2-1. |
| Apple | Issue 2-1-1: Prefer MCS13 targeting smaller TBS for URLLC. Based on our results the SNR seems reasonable for MCS of 13, but there is a large span in results which needs to be looked into. Perhaps we need to discuss how BLER is calculated as Ericsson suggested Issue 2-1-2: option 1 or 3 result in the same criteria. Since we have results we have results from companies we could decide MCS based on results (perhaps after clarification on BLER calculation) Issue 2-2-1: Option 2, we don’t see the necessity to introduce requirements for high reliability in FR2 as there is no practical use case. Update 08/19 On Issue 2-1-1A: BLER calculation method: Our results are based on calculation method 2 |
| NTT DOCOMO | Issue 2-2-1: We prefer Option 1. We prefer to introduce FR2 high reliability requirements to ensure the function of MCS table 3 and PDSCH repetition under FR2 condition. From the BS side, RAN4 agreed to introduce the FR2 high reliability requirements with higher BLER. Thus, RAN4 should align BS and UE requirements to have more meaningful specification. Issue 2-2-3: We prefer Option 2. If we choose Option 1, FR2 requirements apply to UE supporting only FR2. Since UEs that do not support FR1 are difficult to envision, FR2 requirements is rarely applied to UEs. Issue 2-2-5: We slightly prefer DDDSU since this TDD pattern is the most typical pattern. Issue 2-2-8: We prefer Option 1. Issue 2-2-9: We prefer Option 1. |
| Huawei | Issue 2-1-2: option 3 is more straightforward. Issue 2-1-3: option 1 is ok. Issue 2-2-1: we prefer do not define the requirements for high reliability. And the parameters can be discussed after this issue has an agreement. |
| Intel | Issue 2-1-1A: BLER calculation method For our analysis we consider Method 1 Issue 2-1-2: Methodology for MCS selection Support Option 3 Issue 2-2-1: Whether to define URLLC high reliability requirements for FR2 We think that it is rather important to verify that FR2 devices support PDSCH repetition which is covered by this test. Issue 2-2-2: Support Option 1 Issue 2-2-3: Test applicability rule for FR1 and FR2 if UE support both (only if FR2 is defined) Support Option 2 Issue 2-2-7: MCS MCS can be reused from agreement for FR1. |

CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2010987 (Huawei) CR to TS 38.101-4 Performance requirements for URLLC PDSCH repetitions over multiple slots. | Company A |
|  | Company B |
| R4-2010986 (Huawei) CR to TS38.101-4 Applicability rules for URLLC UE demodulation requirements | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Sub-topic 2-1: UE demodulation requirements for high reliability for FR1 - Methodology for MCS selection: Higher or equal to -4 dB for final 4 Rx requirement definition (average ideal SNR alignment result + IM) - HARQ process number: 2 for FDD and 4 for TDD Candidate options: - Recommendations for 2nd round Sub-topic 2-1: UE demodulation requirements for high reliability for FR1 - BLER calculation method: companies should align the BLER calculation method before deciding the MCS. - MCS (table 3) Sub-topic 2-2: UE demodulation requirements for high reliability for FR2 - Whether to define URLLC high reliability requirements for FR2: We have a GTW session in the 2nd round. - FR2 parameters: FR2 parameters will be discussed after FR2 performance requirements are agreed to be defined. CRs: In 2nd round, the section number and name can be decided. Companies are encouraged to comment on these CRs. CRs can be revised in 2nd round. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| Tdoc # | Way forward on NR URLLC UE performance requirements | Intel |

    - CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2010987 | to be revised |
| R4-2010986 | to be revised |

Discussion on 2nd round

Sub-topic 2-5-1: UE demodulation requirements for high reliability for FR1

Issue 2-5-1-1: BLER calculation method

For PDSCH high reliability test case, the test metrics is agreed as 1% BLER (calculated after all transmission). As the simulation results are quite divergent, the purpose of this issue is to ensure we have the same understanding of the BLER calculation. For example, the maximum number of HARQ is 4，if an information block is transmitted and succeed with HARQ index of 3, the BLER is

Method 1: BLER =0, because the TB is successfully transmitted through the transmissions are failed for three times.

Method 2: BLER=3/4. Counting the BLER residue for each HARQ transmission. Here, BLER= 3/4, as the TB is failed for three times and it is transmitted for 4 times.

- Proposals

  - Option 1: Method 1. (Intel, Huawei, Ericsson, Qualcomm)

  - Option 2: Method 2 (Apple)

  - Option 3: Other method which is not precluded.

- Recommended WF

  - Option 1. The method 1 is also used for BS side.

Issue 2-5-1-2: MCS (table 3)

Before make an agreement of MCS, companies should firstly align the BLER calculation method.

- Proposals

  - Option 1: MCS13 (Apple, Intel, Huawei)

  - Option 2: MCS16

  - Option 3: MCS19 (Ericsson, QC)

- Recommended WF

  - TBD

Sub-topic 2-5-2: UE demodulation requirements for high reliability for FR2

Issue 2-5-2-1: Whether to define URLLC high reliability requirements for FR2

This issue will be treated on GTW session on Tuesday, companies are still encouraged to provide their opinions in this summary, along with the FR2 parameters.

- Proposals

  - Option 1: Yes (Intel, DoCoMo, Ericsson)

  - Option 2: No (Apple, Huawei, QC)

  - Option 3: Yes with test applicability rule.

- Recommended WF

  - Option 1

Issue 2-5-2-2: Test applicability rule for FR2 (only if FR2 is defined)

- Proposals

  - Option 1: The performance requirements are only applicable for UE supporting FR2 operating bands. (Ericsson, Huawei, Intel)

  - Option 2:

- Recommended WF

  - Companies have consensus that FR2 performance requirements only applicable for UE supporting FR2, no need to define the applicability rule.

  - Option 1

Issue 2-5-2-3: Test applicability rule for FR1 and FR2 if UE support both (only if FR2 is defined)

- Proposals

  - Option 1: UE should be tested only for FR1 if UE support both FR1 and FR2.

  - Option 2: UE should be tested for both FR1 and FR2 if UE support FR1 and FR2 (DoCoMo, Intel)

- Recommended WF

  - Option 2

Issue 2-5-2-4: Channel model

- Proposals

  - Option 1: TDLA30-300 (Ericsson)

  - Option 2:

- Recommended WF

  - TBD

Issue 2-5-2-5: TDD pattern

- Proposals

  - Option 1: DDSU (Ericsson)

  - Option 2: DDDSU (DoCoMo)

- Recommended WF

  - TBD

Issue 2-5-2-6: MCS

- Proposals

  - Option 1: MCS19 (Ericsson)

  - Option 2: Reuse MCS from FR1 (Intel)

- Recommended WF

  - TBD

Issue 2-5-2-7: Bandwidth

- Proposals

  - Option 1: 100MHz (Ericsson, DoCoMo)

  - Option 2:

- Recommended WF

  - TBD

Issue 2-5-2-8: SCS

- Proposals

  - Option 1: 120kHz (Ericsson, DoCoMo)

  - Option 2:

- Recommended WF

  - TBD

Issue 2-5-2-9: Mapping type

- Proposals

  - Option 1: Type A (Ericsson, Intel)

  - Option 2:

- Recommended WF

  - TBD

Issue 2-5-2-10: Aggregation factor

- Proposals

  - Option 1: n2 (Ericsson, Intel)

  - Option 2:

- Recommended WF

  - TBD

Issue 2-5-2-11: Maximum HARQ transmission

- Proposals

  - Option 1: 4 (Ericsson, Intel)

  - Option 2:

- Recommended WF

  - TBD

Issue 2-5-2-12: Antenna configuration

- Proposals

  - Option 1: 2x2 (Ericsson, Intel)

  - Option 2:

- Recommended WF

  - TBD

Issue 2-5-2-13: Metric

- Proposals

  - Option 1: 1% (Ericsson, Intel)

  - Option 2:

- Recommended WF

  - TBD

Companies views’ collection for 2nd round

Open issues

| Company | Comments |
| --- | --- |
| Ericsson | Issue 2-5-1-1: We use option 1. We continue to support introduction of an FR2 requirement. Regarding the parameters, we propose that the following are also captured in the WF: - PDSCH mapping type A - PDSCH aggregation level 2 - Max HARQ transmissions 4 - Antenna configuration: 2x2 - Target BLER 1% |
| Qualcomm | Issue 2-5-1-1: We use Option 1. [Update on 8/25] Issue 2-5-2-4: As it is high reliability test, prefer to use TDA30-75. Issue 2-5-2-5: Prefer DDSU since aggregation factor is 2 and with DDDSU, we will have longer test time with no grant on DSU part. Issue 2-5-2-6: Depends on simulation results. Prefer to keep it FFS in this meeting. Issue 2-5-2-7/8/9/10/11/12/13: Ok with Option 1. |
| Apple | Issue 2-5-1-1: BLER calculation method ACK/NACK 0 1 0 1 0 1 ACK/NACK ACK/NACK Rep Index (DL) We use option 2. The UE transmits ACK/NACK on the 2nd transmission (for slot aggregation) of every PDSCH, hence we should calculate BLER over all the subframes there is a ACK/NACK transmission. By only considering counting BLER on the last HARQ transmission we are calculating residual BLER and not BLER in our understanding. Issue 2-5-2-1: Whether to define URLLC high reliability requirements for FR2 We propose not to introduce requirements with high reliability for FR2. Update 08/25 Issue 2-5-1-1: BLER calculation method We are not sure if method 1 is the correct approach to calculate BLER. We should be counting BLER on every ACK/NACK transmission and not after all HARQ transmission. If we were to calculate the TP we would count all ACKs transmitted (for rep index 1) and not just after all HARQ transmission. Issue 2-5-2-4 Prefer lower doppler for high reliability as proposed by Qualcomm Issue 2-5-2-5 Option 1 DDSU pattern Issue 2-5-2-6 MCS depends on simulation results. Should we have same options as FR1: 13, 1, 19 to evaluate? For other parameters fine with proposals from Intel and Ericsson. |
| Intel | Issue 2-5-1-1: BLER calculation method @Apple We think that method 1 is correct approach because it allows to reflect impact of HARQ and repetitions on packet error rate. As for TP calculation, we assume that it is also slightly different in comparison to calculation for method 2, because TP = {Number of Tx with ACK and with rep idx 1}*TBS/{total test time}, where {total test time} includes impact from all transmission (i.e. Rep Idx 0 and 1). Same time, as far as we correctly understand Method 2, we have the following equation for Method 2: BLER = {Number of Tx with NACK and with rep idx 1}/{Total number of Tx with rep idx 1}. |
| docomo | Issue 2-5-2-5 We can compromise with DDSU. |

WF/Simulation assumptions comments collection

All the comments for UE WF will be addressed in this section:

| WF number | Comments |
| --- | --- |
| R4-2012648 Intel Way forward on NR URLLC UE performance requirements | Apple: Slide #3: BLER calculation method We propose to add clarification on how repetition transmission of the same HARQ idx should be treated. Option 2 doesn’t capture correctly the method we used. |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2012650 (from R4-2010987) (Huawei) CR to TS 38.101-4 Performance requirements for URLLC PDSCH repetitions over multiple slots. | Company A |
|  | Company B |
|  | Ericsson: Is the intention to approve these CRs in this meeting ? [Moderator]: CRs have been commented from the 1st round and the spec numbering and structures have been agreed for UE and BS specs. Companies are encourage to upload the revised CR with correction sepc numbering and title. It is better to be approved during this meeting. For BS side, the FRC structure is still discussed. Let’s see the progress that whether the related CR can be approved or not. |
|  | Intel: We have not discussed “The number of slots between PDSCH and corresponding HARQ-ACK information” for scenarios with PDSCH repetition. Therefore, we suggest to keep it as TBD. |
|  | QC: Test metric should be clarified in the description that the target BLER of 1% is after all retransmissions. |
|  | Apple: The BLER calculation method should be captured after its agreed. |
|  | Huawei: To Apple: Regarding to the BLER calculation: this issue is also aligned with BS side, companies are all use method 1. We discussed the target BLER for this case for several meetings. Agreements and comments can be found in R4-2002520 Issue 1-1-1/1-1-1a/4-1-1/4-1-1a. The BLER calculation was also discussed for several meetings and companies agreed to calculate the target BLER after all transmission. Here, we just want to align the understanding of “calculate the target BLER after all transmission”. Then companies can provide the simulation results with the same BLER calculation method. Intel: “The number of slots between PDSCH and corresponding HARQ-ACK information” has not been discussed is because this is not the latency case, no processing capability 2 is required. Then we think the existed parameter should be used here. For this CR, we recommended to be endorsed. |
|  | Intel: Don’t quit understand how discussion on K1 values are related to whether it is low latency test or not. We just think that more discussion on exact values is needed, because K1 value probably can be different for scenarios without and with PDSCH repetitions and suggest to double check. Also, for TDD test, I assume that value “2” is typo, because K1 is different for different slots and defined in section with TDD patterns for Rel-15 requirements. |
|  | QC: Same comments as Intel on K1 value for TDD. It can’t be 2 for all slots and may need more discussion whether we can reuse the same K1 as in other normal demod tests. Also, we didn’t have any discussion on number of HARQ processes for TDD. We may need to double check if 4 processes are good enough. So, may be it should be marked as TBD in this meeting. Also, in TDD tables for minimum performance, another column should be added for TDD UL DL configuration, similar to existing TDD tests. |
| R4-2012651 (from R4-2010986) (Huawei) CR to TS38.101-4 Applicability rules for URLLC UE demodulation requirements | Company A |
|  | Apple: To clarify do requirements with PDSCH slot aggregation apply to UE supporting CQI Table 3 as well in addition to MCS table3 and PDSCH slot aggregation? |
|  | Intel: Comment to original version. - We suggest to list only one feature in the first column: New 64QAM MCS table for PDSCH (dl-64QAM-MCS-TableAlt); FR1 FDD; PDSCH; Clause 5.2.2.1.5 Clause 5.2.3.1.5 Clause 5.2.2.1.6 Clause 5.2.3.1.6. FR1 TDD; PDSCH; Clause 5.2.2.2.5 Clause 5.2.3.2.5 Clause 5.2.2.2.6 Clause 5.2.3.2.6. PDSCH repetitions over multiple slots (pdsch-RepetitionMultiSlots); FR1 FDD; PDSCH; Clause 5.2.2.1.6 Clause 5.2.3.1.6. FR1 TDD; PDSCH; Clause 5.2.2.2.6 Clause 5.2.3.2.6. UE PDSCH processing capability #2 (pdsch-ProcessingType2); FR1 FDD; PDSCH; Clause 5.2.2.1.7 Clause 5.2.3.1.7. FR1 TDD; PDSCH; Clause 5.2.2.2.7 Clause 5.2.3.2.7. Pre-emption indication for DL (pre-EmptIndication-DL); FR1 FDD; PDSCH; Clause 5.2.2.1.8 Clause 5.2.3.1.8. FR1 TDD; PDSCH; Clause 5.2.2.2.8 Clause 5.2.3.2.8. - In the previous meeting it was agreed that Ultra-low BLER requirements are applicable for UEs that support both MCS Table 3 (dl-64QAM-MCS-TableAlt) and CQI Table 3 (cqi-TableAlt). Therefore, we suggest to include this applicability for Clause 5.2.2.1.5 etc. - Clauses 5.2.2.1.7, Clause 5.2.3.1.7, Clause 5.2.2.2.7, Clause 5.2.3.2.7 should be added to the Table 5.1.1.4-1 in row with PDSCH mapping type B |
|  | Huawei: To Intel: We suggest to list only one feature in the first column: The purpose of listing all the features is to clearly show that UE need to do the test only if it supports all the features listed here. If only list one feature, is this ok for UE? Clauses 5.2.2.1.7, Clause 5.2.3.1.7, Clause 5.2.2.2.7, Clause 5.2.3.2.7 should be added to the Table 5.1.1.4-1 in row with PDSCH mapping type B The mapping Type B case is involved with processing capability 2. If we list these clauses in table 5.1.1.4-1 for PDSCH mapping type B. Does this mean all the cases need to be passed if a UE is testing PDSCH mapping type B? |
|  | Intel: These tables with applicability show connection between UE feature/capability and exact test. - To pass test in clause Clause 5.2.2.1.6, UE should support New 64QAM MCS table for PDSCH (dl-64QAM-MCS-TableAlt) and PDSCH repetitions over multiple slots (pdsch-RepetitionMultiSlots). - To pass test in clause Clause 5.2.2.1.7, UE should support UE PDSCH processing capability #2 (pdsch-ProcessingType2) and PDSCH mapping type B We think that if for each test we will list the set of features, which are covered by each test, then test applicability section becomes rather huge. Answer on second question: No, if UE does not support UE PDSCH processing capability #2 (pdsch-ProcessingType2). |

Summary for 2nd round

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#2-5-1/2-5-2 | Agreements: FR1 High reliability - Methodology for MCS selection - Higher or equal to -4 dB for final 4 Rx requirement definition (average ideal SNR alignment result + IM) - BLER calculation method - BLER = NpacketFail/NpacketTx, where NpacketFail is the number of packets with CRC fail after all transmissions (initial and retransmissions), NpacketTx is the total number of packets transmitted during the test. FR2 High reliability - Introduce test cases with PDSCH repetition in FR2 with 1% BLER as test metric - Test applicability rule for FR2 - The performance requirements are only applicable for UE supporting FR2 operating bands - Test applicability rule for FR1 and FR2 if UE support both - UE should be tested for both FR1 and FR2 if UE support FR1 and FR2 Open issues for next meeting: FR1 High reliability - HARQ process number: 2 for FDD and 4 for TDD - MCS: - Option 1: MCS13 - Option 2: MCS16 - Option 3: MCS19 FR2 High reliability - Companies are encouraged to provide view on detailed test parameters in the next RAN4 meeting: - Aggregation factor, MCS, Channel bandwidth, SCS, Channel model, TDD pattern, PDSCH Mapping type etc. |

WF and CR status

| WF number | Status summary |
| --- | --- |
| R4-2012648 WF for NR UE URLLC performance requirements | Agreeable |

| CR number | Status summary |
| --- | --- |
| R4-2012650 (revised from R4-2010987) | To be endorsed |
| R4-2012651 (revised from R4-2010986) | To be noted |

| Summary of simulation results | Status summary |
| --- | --- |
| R4-2011280 Simulation results for UE | To be noted |

Topic #3: UE demodulation requirements for PDSCH mapping Type B and processing capability 2

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2009612 | Apple | Proposal #3: Define requirements with 2 HARQ processes Proposal #4: Define requirements with MCS 4 for URLLC low latency feature test cases |
| R4-2009725 | Intel Corporation | Proposal 2: Use the following assumptions for FR1 requirements with mapping Type B and PDSCH processing capability 2: - HARQ processes and PDSCH scheduling with HARQ index 3 - MCS 4 Proposal 4: Use the following assumptions for FR2 PDSCH requirements with mapping Type B: - CBW/SCS: 100 MHz/120 kHz - TDD pattern: DDDSU with S = 10D:2G:2U - PDSCH configuration: Mapping Type B, Start symbol 1, Duration 2 - PDSCH scheduling: No PDSCH in slot 0 within 20 ms - MCS 4 |
| R4-2010981 | Huawei, HiSilicon | Proposal 4: HARQ process is 2 for TDD. Proposal 5: Only define MCS4 to verify mapping Type B and processing capability 2. |
| R4-2010982 | Huawei, HiSilicon | Simulation results |
| R4-2011046 | NTT DOCOMO, INC. | Proposal 1: Following TDD configuration should be supported for FR2 low latency requirements ・120kHz SCS: DDDSU, S=10D:2G:2U Proposal 2: For the symbol length of mapping type B, 2 or 4 symbol should be supported. Proposal 3: For antenna configuration, 2x2/2x4 should be supported. |
| R4-2011371 | Ericsson | Proposal 3: Configure both MCS4 and MCS17 for PDSCH mapping type B transmission and PDSCH capability 2 tests, to cover variable realistic deployment scenarios, for both FDD and TDD. Observation 4: There is no processing capability 2 for FR2, thus there is no need to restrict data scheduling to the special slot. Proposal 7: Specify PDSCH Type B mapping in FR2 |
| R4-2011445 | Qualcomm Incorporated | Proposal 3: Define the test for low latency feature with only MCS 4. |
| R4-2011403 | Qualcomm Incorporated | Draft CR on FR1 PDSCH Mapping Type B and Processing Capability 2 Requirements |

Open issues summary

In this section, test for URLLC UE low latency: PDSCH mapping Type B and processing capability 2 is discussed.

Open issues of 95-e:

- PDSCH symbol length for FDD and TDD: 2os only

- Test metrics: 70% throughput

- FR2 requirements for PDSCH mapping Type B

  - Introduce test cases for FR2, FFS whether test applicable rules will be introduced.

Open issues of 95-e:

- Number of HARQ process for TDD:

  - Option 1: 2

  - Option 2: 4 processes and PDSCH scheduling with HARQ index 3

- MCS:

  - Option 1: Only MCS 4

  - Option 2: Only MCS 17

  - Option 3: MCS 4 and MCS 17

- FR2 requirements for PDSCH mapping Type B

  - Introduce test cases for FR2, FFS whether test applicable rules will be introduced.

Sub-topic 3-1: PDSCH mapping Type B and processing capability 2 for FR1

In this section, the open issues of PDSCH mapping Type B and processing capability 2 will be discussed,

Open issues and candidate options before e-meeting:

Issue 3-1-1: MCS

- Proposals

  - Option 1: Only MCS4 in Table 1. (Apple, Intel, Huawei, Ericsson)

  - Option 2: Both MCS4 and MCS17 in Table 1. (Ericsson)

- Recommended WF

  - Option 1

Issue 3-1-2: Number of HARQ process for TDD

- Proposals

  - Option 1: 2 (Apple, Huawei, Intel)

  - Option 2: 4 HARQ processes and PDSCH scheduling with HARQ index 3 (Intel)

- Recommended WF

  - Option 1

Sub-topic 3-2: PDSCH mapping Type B for FR2

Issue 3-2-1: Test applicability rule for FR2

- Proposals

  - Option 1: The performance requirements are only applicable for UE supporting FR2 operating bands.

  - Option 2: No need. (Apple, QC, Intel)

- Recommended WF

  - This is default that FR2 performance requirements should only be applicable if the UE supports FR2. No need to define the applicability rule.

  - Remove this issue

Issue 3-2-3: PDSCH scheduling

- Proposals

  - Option 1: No PDSCH in slot 0 within 20 ms. (Intel, Ericsson, QC)

  - Option 2:

- Recommended WF

  - Option 1

Issue 3-2-4: MCS

- Proposals

  - Option 1: MCS4. (Intel, Ericsson, QC)

  - Option 2:

- Recommended WF

  - Decide the MCS depends on the simulation results to ensure a proper SNR value.

Issue 3-2-5: Antenna configuration

- Proposals

  - Option 1: 2x2 and 2x4. (DoCoMo)

  - Option 2: 2x2 (Ericsson? Huawei, Apple, Intel)

- Recommended WF

  - For FR2, the maximum number of Rx antenna is 2.

  - Option 2?

Issue 3-2-6: TDD pattern

- Proposals

  - Option 1: DDDSU with S = 10D:2G:2U (Intel, DoCoMo, QC, Huawei)

  - Option 2:

- Recommended WF

  - Option 1

Issue 3-2-7: SCS

- Proposals

  - Option 1: 120 kHz (Intel, DoCoMo, Ericsson, QC, Huawei)

  - Option 2:

- Recommended WF

  - Option 1

Issue 3-2-8: CBW

- Proposals

  - Option 1: 100 MHz for 120 kHz (Intel, Ericsson, QC, Huawei)

  - Option 2:

- Recommended WF

  - Option 1

Issue 3-2-9: Start symbol

- Proposals

  - Option 1: 1 (Intel, Ericsson, QC, Huawei)

  - Option 2:

- Recommended WF

  - Option 1

Issue 3-2-10: Symbol length

- Proposals

  - Option 1: 2 (Intel, DoCoMo, QC)

  - Option 2: 4 (DoCoMo)

- Recommended WF

  - TBD

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| XXX | Issue 3-1-1: Issue 3-1-2: Issue 3-1-3: |
| Ericsson | Issue 3-1-1: We still believe that fewer PRBs could be scheduled together with higher MCS (to get the same transport block size) and this would be more efficient. But we can compromise to option 1. Issue 3-2-1: We think the FR2 performance requirements should only be applicable if the UE supports FR2 (with or without FR1) Issue 3-2-2: We think both should be tested as FR2 differs a bit to FR1; it does not have PDSCH processing capability 2 defined. Issue 3-2-3, 4: OK Issue 3-2-5: FR2 has to be OTA tested and it is not possible to test greater than 2RX. Issue 3-2-7/8/9: OK |
| Qualcomm | Issue 3-1-1: Prefer Option 1. We prefer to define only one test. As smaller MCS is more suitable for URLLC, we prefer MCS4. Issue 3-1-2: No preference. Issue 3-2-1: We are not sure what this applicability rule means. If UE doesn’t support FR2, then these requirements cannot be tested anyway. Issue 3-2-2: As there is no processing capability 2 defined for FR2, this will be just PDSCH Type B test case. Since the motivation here is to have low latency test case, we don’t see any gain in testing both FR1 and FR2. Therefore, we prefer Option 1. Issue 3-2-3 to 3-2-9: In general, ok with assumptions except 4Rx. However, since FR2 does not have any processing capability 2, it may be possible to schedule PDSCH on all slots. If that is the case, MCS should be chosen based on simulation results so that required SNR is not too low. Issue 3-2-10: Prefer to use 2 symbol grant, same as FR1. |
| Apple | Issue 3-1-1: Option 1. We prefer to have one test with MCS=4, small TBS for URLLC Issue 3-1-2: Option 1 for ease of specifying test parameters. Issue 3-2-1: No need to specify this applicability rule. Issue 3-2-2: Prefer option 1. But the tests are not the same between FR1 and FR2 as FR2 doesn’t have PDSCH processing capability 2. So not sure of the applicability rule. Issue 3-2-3 – 3-2-10: We should decide on parameters based on simulation results. But prefer to use same setup as FR1 as much as possible. Issue 3-2-5: Requirements should be defined with 2RX in FR2. |
| NTT DOCOMO | Issue 3-2-2: Test applicability rule for FR1 and FR2 If we choose Option 1, FR2 requirements apply to UE supporting only FR2. Since UE that do not support FR1 are difficult to envision, FR2 requirements is rarely applied to UE. As for the latency reduction, FR2 would be feasible than FR1 since it is beneficial to achieve shorter OFDM symbol duration by using higher subcarrier spacing in FR2. In this sense, we cannot guarantee the low latency performance of FR2 even if only FR1 low latency is tested. Issue 3-2-5: Antenna configuration If there are no concerns, the number of antennas used with FR1 low latency requirements should be reused. Issue 3-2-6: TDD pattern We prefer Option 1 since this TDD pattern is the most typical pattern. Issue 3-2-7: SCS We prefer Option 1. Issue 3-2-8: CBW We prefer Option 1. Issue 3-2-10: Symbol length We prefer Option 1. |
| Huawei | Issue 3-1-1: We prefer to only define MCS4. Small MCS is more reasonable for URLLC high reliability test. Issue 3-2-1: FR2 test is only applicable for UE supports FR2. No test applicability rule is needed. Issue 3-2-3---3-2-10: We prefer to use the same parameters with FR1 as much as possible. Issue 3-2-4: The MCS should be decided after the simulation. Issue 3-2-7/8: We prefer 120kHz/100MHz. Issue 3-2-9: we prefer option 1. |
| Intel | Issue 3-1-1: MCS Support Option 1. We think that using of one MCS is sufficient from test coverage point of view. Issue 3-1-2: Number of HARQ process for TDD Option 1 is fine for us. Issue 3-2-1: Test applicability rule for FR2 Ok with recommended WF Issue 3-2-5: Antenna configuration Support Option 2. At current stage, only 2 Rx chains are supported for FR2. |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2011403 (QC) Draft CR on PDSCH Mapping Type B and Processing Capability 2 Requirements | Company A |
|  | Company B |
|  | Ericsson: Should be category B. Will need to align section header name with the test specification discussion in Section 7.2.2. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Sub-topic 3-1: PDSCH mapping Type B and processing capability 2 for FR1 - MCS: Only MCS4 in Table 1 - Number of HARQ process for TDD: 2 Sub-topic 3-2: PDSCH mapping Type B for FR2 - Test applicability rule for FR2: This is default that FR2 performance requirements should only be applicable if the UE supports FR2. No need to define the applicability rule. - Test applicability rule for FR1 and FR2: the test cases are different for FR1 and FR2, no test applicability rule is needed. - PDSCH scheduling: No PDSCH in slot 0 within 20 ms - MCS: [MCS4] from table 1. Confirming the MCS depends on the simulation results to ensure a proper SNR value. - Antenna configuration: 2x2 - TDD pattern: DDDSU with S = 10D:2G:2U - SCS/CBW: 120 kHz/100 MHz - Start symbol: 1 Candidate options: - Recommendations for 2nd round Sub-topic 3-1: PDSCH mapping Type B and processing capability 2 for FR1 - Ideal SNR has been added in the summary of simulation results, companies are encouraged to add their impairments results in the 2nd round. Sub-topic 3-2: PDSCH mapping Type B for FR2 - Symbol length - Option 1: 2 (Intel, DoCoMo, QC) - Option 2: 4 (DoCoMo) - Other parameters not precluded CRs: In 2nd round, the section number and name can be decided. Companies are encouraged to comment on these CRs. CRs can be revised in 2nd round. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2011403 | to be revised |

Discussion on 2nd round

Sub-topic 3-5-1: PDSCH mapping Type B and processing capability 2 for FR1

Ideal SNR has been added in the summary of simulation results, companies are encouraged to add their impairments results to make the alignment.

Sub-topic 3-5-2: PDSCH mapping Type B for FR2

Issue 3-5-2-1: Symbol length

- Proposals

  - Option 1: 2 (Intel, DoCoMo, QC, Huawei, Ericsson, Apple)

  - Option 2: 4 (DoCoMo)

- Recommended WF

  - Option 1

Companies views’ collection for 2nd round

Open issues

| Company | Comments |
| --- | --- |
| XXX | Issue: |
| Huawei | Issue 3-5-2-1: We are ok with symbol length of 2. Keep same with FR1. |
| Ericsson | We are OK with either option; we can decide option 1 if it assists to make a decision in this meeting |
| Apple | Issue 3-5-2-1: Symbol length Option 1, to be consistent with FR1 requirements. |
| Qualcomm | Issue 3-5-2-1: Prefer Option 1 same as FR1. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2012652 (from R4-2011403) (QC) Draft CR on PDSCH Mapping Type B and Processing Capability 2 Requirements | Company A |

Summary for 2nd round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#3-5-1/3-5-2 | Agreements: FR1 PDSCH mapping Type B and processing capability 2 - Number of HARQ process for TDD: 2 - MCS: Only MCS 4 FR2 PDSCH mapping Type B - Test applicability rule for FR2: No need to define the applicability rule. - Test applicability rule for FR1 and FR2: No test applicability rule is needed. - SCS/CBW: 120 kHz/100 MHz - TDD pattern: DDDSU with S = 10D:2G:2U - PDSCH Configuration - Scheduling: No PDSCH in slot 0 within 20 ms - MCS: [MCS4] from table 1. Confirming the MCS depends on the simulation results to ensure a proper SNR value. - Start symbol: 1 - Symbol length: 2 - Max number of HARQ transmissions: 4​ - Number of HARQ process: 8 - Antenna configuration: 2x2, ULA low - Test metrics: 70% throughput Open issues for next meeting: FR2 PDSCH mapping Type B - Channel model: - Option 1: TDLA30-75 - Other options are not precluded |

WF and CR status

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR number | Status summary |
| --- | --- |
| R4-2012652 (Revised from R4-2011403) | To be endorsed |

Topic #4: UE demodulation requirements for pre-emption

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2009725 | Intel Corporation | Proposal 5: Choose one of the following options for Pre-emption indication requirements: Option 1: Test metric 70 % of Max-throughput, Pre-emption probability 20%, MCS 13 Option 2: Test metric 90 % of Max-throughput, Pre-emption probability 10%, MCS 13 |
| R4-2010720 | MediaTek inc. | Observation 1: Small SNR loss at 70% throughput between the case without HARQ flushing and with HARQ flushing in both MCS 4 and MCS 13. Proposal 1: Discuss more about the HARQ buffer flushing behavior of eMBB UE when pre-empted. |
| R4-2010981 | Huawei, HiSilicon | Proposal 6: We propose to use MCS13 for the pre-emption verification. Proposal 7: We propose 10% probability for pre-emption periodicity. |
| R4-2010982 | Huawei, HiSilicon | Simulation results |
| R4-2011371 | Ericsson | Observation 2: For FDD, the SNR operating points for 70% maximum throughput with MCS4 and 4Rx are too low to be defined as PDSCH demodulation requirements. Observation 3: For TDD, the SNR operating points for 70% maximum throughput with MCS4 and 4Rx are too low to be defined as PDSCH demodulation requirements. Proposal 4: Only configure 10% pre-emption probability for pre-emption tests Proposal 5: Configure MCS13 for pre-emption tests. |
| R4-2011445 | Qualcomm Incorporated | Proposal 4: Define DL pre-emption test for eMBB with 10% pre-emption probability, fixed scheduling and MCS 4. |

From the approved WF R4-2008807 in RAN4 #94 e-meeting, following were agreed for the Pre-emption:

Open issues of 95-e:

- Pre-emption scheduling: Fixed scheduling

- Use the following parameters for eMBB PDSCH configuration

Open Issues:

- Pre-emption probability

  - Option 1: 10% within 1 radio frame

  - Option 2: 20% within 1 radio frame

- eMBB MCS

  - Option 1: MCS13 in Table 1

  - Option 2: MCS4 in Table 1

- Companies are encouraged to prepare comparison analysis of UE with and without HARQ buffer flushing of pre-empted bits to decide on options above

- Test metric

  - Option 1: 70% of max T-put

  - Other options are not precluded

Open issues summary

Sub-topic 4-1: PDSCH pre-emption

In this sub-topic, the parameters for pre-emption indication will be discussed.

Open issues and candidate options before e-meeting:

Issue 4-1-1: Pre-emption periodicity

- Proposals

  - Option 1: 10% probability with fixed scheduling within 1 radio frame. (Huawei, Ericsson)

  - Option 2: 20% probability with fixed scheduling within 1 radio frame with test metric 70% of max t-put. (Intel)

  - Option 3: 10% probability with fixed scheduling within 1 radio frame with test metric 90% of max t-put. (Intel)

- Recommended WF

  - Companies should have an agreements on 4-1-3A firstly.

Issue 4-1-2: MCS

- Proposals

  - Option 1: MCS13 in Table 1. (Intel, Huawei, Ericsson)

  - Option 2: MCS4 (QC)

- Recommended WF

  - Companies should have an agreements on 4-1-3A firstly.

Issue 4-1-3: Whether to discuss about the assumption of UE behaviour for buffer flushing and decoding?

- Proposal

  - Option 1 : Yes (MediaTek)

- Recommended WF

  - [Moderator] MediaTek proposed this issues in R4-2010720. The UE buffer behaviours have been discussed in previous meetings, please check issues 2-2-3 of R4-2008888.

Issue 4-1-3A: The assumption of UE behaviours for buffer flushing and decoding

- Proposal

  - Option 1: If UE cannot decode the PDCSH correctly, UE flushes the buffer and then decodes the PDSCH immediately again and feeds back the 2nd decoding result to gNB. (QC)

  - Option 2: If UE cannot decode the PDCSH correctly, UE feeds back NACK to gNB. Then UE flushes the buffer and waits for the next re-transmission for LLR combing to decode the PDSCH. (Ericsson, Huawei, Intel with revised paper R4-2012564)

- Recommended WF

  - TBD

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| MediaTek | Issue 4-1-3: We would like to discuss the detail for the assumption of UE behaviour for buffer flushing and decoding. For example, there might be two cases after UE receives the pre-emption indicator. Case 1: If UE cannot decode the PDCSH correctly, UE flushes the buffer and then decodes the PDSCH immediately again and feeds back the 2nd decoding result to gNB. Case 2: If UE cannot decode the PDCSH correctly, UE feeds back NACK to gNB. Then UE flushes the buffer and waits for the next re-transmission for LLR combing to decode the PDSCH. We understand that the UE buffer behaviour had been discussed in the previous meeting and we do share the same view as all companies that buffer is default to be flushed after pre-empted. However, the simulation results provided by companies, including Intel, Ericsson, Huawei and MediaTek, are quite diverse for both MCS4 and MCS13. Especially for MCS13, the SNR loss between with/without buffer flushing is small in the results from Ericsson and MediaTek and large in the results from Intel and Huawei. Besides, with buffer flushing, it seems that only Intel can achieve the same throughput as no pre-emption at high SNR for both pre-emption probability 10% and 20%. We think the first priority is to discuss the assumption about UE behaviour for buffer flushing and decoding. Then, we can align the simulation results and figure out the metric and requirements for the test cases. |
| Ericsson | Issue 4-1-3: Companies should state their assumption on UE behavior so that simulations can be aligned. Our approach is similar to case 2. |
| Qualcomm | Issue 4-1-1: Prefer Option 1. Issue 4-1-2: Prefer Option 2. Based on our results, we see 1.4dB of degradation even with MCS4. Issue 4-1-3: It seems that simulation assumptions are not aligned between companies. We would like to clarify how other companies are modeling URLLC interference and at what SNR. In our case, we are modeling URLLC interference by transmitting random garbage data in the pre-empted resources at the same SNR as the eMBB UE and then we look at performance with HARQ buffer flushing and without flushing for those resources. For ACK/NACK, we are assuming Case 1 in MediaTek comment. |
| Huawei | We are testing the UE pre-emption performance. The SNR loss is achieved between with and without buffer flushing. Our simulation assumption is similar to case 2 in MediaTek. After eMBB UE receives the PI, the buffer will be flushed and waiting for re-transmission for LLR combing, then decode the PDSCH. |
| Intel | Issue 4-1-3A: The assumption of UE behaviours for buffer flushing and decoding Our original results in R4-2009725 are prepared under assumption of Option 1. Same time, we realized that Option 1 looks like advanced processing and Option 2 should be considered as baseline. In our revised paper R4-2012564, we presented results for Option 2. In case Option 2 is considered, based on our analysis, we cannot verify correct HARQ buffer flushing for MCS4. Same time, maximum throughput cannot be reached for MCS 13 (such case is also not convenient for requirements definition). Therefore, we think that, first, we can try to conclude on UE behaviour in this meeting and continue analysis to define proper test setup. |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: No Candidate options: Recommendations for 2nd round: Before make an alignment of the simulation results, companies should make an agreement of the UE behaviors for buffer flushing and decoding and what to be transmitted on pre-empted resources: The assumption of UE behaviours for buffer flushing and decoding - Option 1: If UE cannot decode the PDCSH correctly, UE flushes the buffer and then decodes the PDSCH immediately again and feeds back the 2nd decoding result to gNB. - Option 2: If UE cannot decode the PDCSH correctly, UE feeds back NACK to gNB. Then UE flushes the buffer and waits for the next re-transmission for LLR combing to decode the PDSCH. (Ericsson, Huawei, Intel with revised paper R4-2012564) What to be transmitted on pre-empted resources: - Option 1: Some random data - Option 2: URLLC interference - Option 3: Options not precluded SNR of URLLC interference compared to eMBB transmission (if URLLC interference is transmitted on pre-empted resources): - Option 1: Same - Option 2: Options not precluded Then we can continue to align the simulation results and make agreements on pre-emption periodicity and MCS: Pre-emption periodicity - Option 1: 10% probability with fixed scheduling within 1 radio frame. - Option 2: 20% probability with fixed scheduling within 1 radio frame with test metric 70% of max t-put. - Option 3: 10% probability with fixed scheduling within 1 radio frame with test metric 90% of max t-put. MCS - Option 1: MCS13 in Table 1. - Option 2: MCS4 CRs: In 2nd round, the section number and name can be decided. Companies are encouraged to comment on these CRs. CRs can be revised in 2nd round. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round

Sub-topic 4-5-1: PDSCH pre-emption

Before make an alignment of the simulation results, companies should make an agreement of the UE behaviors for buffer flushing, decoding and what to be transmitted on pre-empted resources:

Issue 4-5-1-1: The assumption of UE behaviours for buffer flushing and decoding:

- Proposal

  - Option 1: If UE cannot decode the PDCSH correctly, UE flushes the buffer and then decodes the PDSCH immediately again and feeds back the 2nd decoding result to gNB.

  - Option 2: If UE cannot decode the PDCSH correctly, UE feeds back NACK to gNB. Then UE flushes the buffer and waits for the next re-transmission for LLR combing to decode the PDSCH. (Ericsson, Huawei, Intel with revised paper R4-2012564, MTK, QC, Apple)

- Recommended WF

  - Option 2.

Issue 4-5-1-2: How to model the URLLC interference (what is transmitted on pre-empted resources):

- Proposal

  - Option 1: Some random data (MTK, Huawei, Ericsson, QC, Intel)

  - Option 2: Other

- Recommended WF

  - Option 1

Issue 4-5-1-3: SNR of the modelled URLLC interference compared to eMBB transmission:

- Proposal

  - Option 1: Same with eMBB transmission (MTK, Huawei, Ericsson, QC)

  - Option 2: others

- Recommended WF

  - Option 1

Then we can continue to align the simulation results and make agreements on pre-emption periodicity and MCS:

Issue 4-5-1-4: Pre-emption periodicity

- Proposal

  - Option 1: 10% probability with fixed scheduling within 1 radio frame. (Huawei, Ericsson)

  - Option 2: 20% probability with fixed scheduling within 1 radio frame with test metric 70% of max t-put. (Intel)

  - Option 3: 10% probability with fixed scheduling within 1 radio frame with test metric 90% of max t-put. (Intel)

- Recommended WF

  - TBD

Issue 4-5-1-5: MCS

- Proposal

  - Option 1: MCS13 in Table 1. (Intel, Huawei, Ericsson)

  - Option 2: MCS4 (QC)

- Recommended WF

  - According to the agreements of issues 4-5-11/2/3, it is better for companies to check their simulation and propose MCS and periodicity in next meeting.

Companies views’ collection for 2nd round

Open issues

| Company | Comments |
| --- | --- |
| MediaTek | Issue 4-5-1-1: Our simulation results are based on Option 2. That is, UE flushes the buffer and waits for the next re-transmission for LLR combing to decode the PDSCH if UE cannot decode the PDCSH correctly. We agree with Intel that Option 1 looks like advanced processing and Option 2 should be considered as baseline. Issue 4-5-1-2: Our simulation results are based on Option 1. We model the URLLC interference with random data. Issue 4-5-1-3: Our simulation results are based on Option 1. The SNR of the modelled URLLC interference is the same with eMBB transmission. |
| Huawei | Issue 4-5-1-2/3: we model the UELLC interference by transmitting some random data with same SNR with eMBB data. |
| Intel | Issue 4-5-1-1: The assumption of UE behaviours for buffer flushing and decoding: Usually, minimum performance requirements are defined under assumption of non-advanced receive processing. Therefore, we suggest to consider Option 2 as baseline receive processing for requirements definition. Issue 4-5-1-2: How to model the URLLC interference In the previous RAN4 meeting the following was agreed for URLLC modelling as a part of simulation assumptions (R4-2008808) - PDSCH configuration - Scheduling - Option 1: 10% (slot 1 within 10 ms) - Option 2: 20% (slots 1 and 2 within 10 ms) - Mapping Type B, Start symbol 3, Duration 2 - PRB allocation: Full (52 PRBs for FDD and 106 PRBs for TDD) - FRC: Rank 1 with QPSK modulation Therefore, we’ve assumed explicit URLLC transmission modelling for analysis. Issue 4-5-1-3: SNR of the modelled URLLC interference compared to eMBB transmission: Same SNR for eMBB and URLLC transmissions is considered in our analysis. |
| Ericsson | Issue 4-5-1-1: Actually, we are not absolutely clear what does “If the UE cannot decode the PDSCH correctly” mean. We assume it to mean “If the UE has received PI information but still attempt to decode without flushing first”. Is that the general understanding? So the difference between option 1 and option 2 is that with option 1, once it receives PI the UE still attempts to decode without flushing and decode again with flushing if the first attempt fails (then sends ACK/NACK), whereas with option 2, the UE as soon as it receives PI sends NACK, flushes the buffer, and only re-attempts decoding after the retransmission is received ? With those assumptions, we do option 2. Issue 4-5-1-2: We do option 1 Issue 4-5-1-3: We do option 1 |
| Qualcomm | Issue 4-5-1-1: Ok with Option 2. Issue 4-5-1-2/3: We use Option 1 in our simulations. |
| Apple | Issue 4-5-1-1: We support option 2 as that is the assumed baseline. Option 1 seems to be behavior with advanced processing. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Summary for 2nd round

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | Status summary |
| --- | --- |

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#4-5-1/4-5-2 | Agreements: Pre-emption indication - The assumption of UE behaviours for buffer flushing and decoding - If UE cannot decode the PDCSH correctly, UE feeds back NACK to gNB. Then UE flushes the buffer and waits for the next re-transmission for LLR combing to decode the PDSCH. - URLLC interference modelling - SNR: Same as for eMBB transmission - Structure: Some random data Open issues for next meeting: Pre-emption indication - Pre-emption probability - Option 1: 10% within 1 radio frame - Option 2: 20% within 1 radio frame - eMBB MCS - Option 1: MCS13 in Table 1 - Option 2: MCS4 in Table 1 - Test metric - Option 1: 70% of max T-put - Other options are not precluded - Companies are encouraged to prepare comparison analysis of UE with and without HARQ buffer flushing of pre-empted bits to decide on options above |

Topic #5: CQI reporting requirements for supporting CQI table 3

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2009726 | Intel Corporation | Proposal 1: efine wideband CQI requirements for fading conditions to verify CQI Table 3 using the following test metrics: A CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least α% of the time The ratio of the throughput with follow CQI vs median CQI shall be ≥ γ |
| R4-2010985 | Huawei, HiSilicon | CR to TS 38.101-4 Applicability rules for URLLC CSI requirements. |
| R4-2011372 | Ericsson | Observation 1: CQI table3 is designed for 10^-5 BLER target according to RAN1 specification, therefore the CQI reporting cannot be done nor verified with other BLER targets than 10^-5. Proposal #1: Define the UE CQI reporting performance tests for CQI table 3 with 10^-5 BLER target. Proposal #2: Define the UE CQI reporting performance tests for CQI table 3 with AWGN channel model. |
| R4-2011446 | Qualcomm Incorporated | Observation 1: With CQI reporting test under fading, proponents had proposed to remove the BLER criteria. In that case, we cannot fulfil the purpose of CQI reporting tests. Observation 2: With CQI reporting under AWGN condition with higher BLER criteria, we will not be able to test the UE for 1e-5 BLER, which was the purpose of having CQI Table 3. Proposal 1: Define CQI reporting tests with 1e-5 BLER under AWGN condition with 99.999% confidence level. |
| R4-2009612 | Apple | Proposal 5: Define CQI reporting test in fading channel with the following test metric: - A CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least α% of the time - The ratio of the throughput with follow CQI vs median CQI shall be ≥ γ |

Open issues summary

In this section, the remained open issues for CQI reporting test will be discussed.

From the approved WF R4-2008807 in RAN4 #95 e-meeting, remaining open issues are listed as follows:

Open issues:

- Propagation channel for CQI reporting

  - Option 1: AWGN

  - Option 2: Fading channel

- Target BLER

  - Option 1: 10^-3

  - Option 2: 10^-2

  - Option 3: 10^-5

  - Option 4: No BLER metric in fading channel

- Test metric in case AWGN conditions will be used

  - Option 1: Reuse Rel-15 CQI test metric

  - Option 2: Reuse Rel-15 CQI test metric and define a lower bound for median reported CQI in the CQI reporting tests

  - Other options are not precluded

- Test metric in case Fading conditions will be used

  - Option 1:

    - A CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least α% of the time

    - The ratio of the throughput with follow CQI vs median CQI shall be ≥ γ

  - Other options are not precluded

- In case AWGN conditions with target BLER 10^-5 will be used, FFS feasibility to define CQI reporting test case and FMCS case at the same SNR and define an applicability rule between CQI reporting test and FMCS test

Sub-topic 5-1 CQI reporting requirements for support of CQI table 3

Open issues and candidate options before e-meeting:

Issue 5-1-1: Propagation channel for CQI reporting

- Proposals

  - Option 1: AWGN (Huawei, QC, Ericsson)

  - Option 2: Fading channel (Intel, Apple)

- Recommended WF

  - TBD

Issue 5-1-2: Target BLER

- Proposals

  - Option 1: 10^-3 or higher

  - Option 2:

- Recommended WF

  - Define CQI test with 10^-5, and discuss this issue in email thread 318.

Issue 5-1-3: Test metric

- Proposals

  - Option 1:

    - The reported CQI value shall be in the range of ±1 of the reported median more than 90% of the time. If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to the target BLER, the BLER using the transport format indicated by the (median CQI + 1) shall be greater than the target BLER. If the PDSCH BLER using the transport format indicated by the median CQI is greater than the target BLER, the BLER using transport format indicated by (median CQI – 1) shall be less than or equal to the target BLER.

  - Option 2: (QC)

    - The reported CQI value shall be in the range of ±1 of the reported median more than 90% of the time. If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to the target BLER, the BLER using the transport format indicated by the (median CQI + 1) shall be greater than the target BLER. If the PDSCH BLER using the transport format indicated by the median CQI is greater than the target BLER, the BLER using transport format indicated by (median CQI – 1) shall be less than or equal to the target BLER.

    - Define a lower bound for median reported CQI in the CQI reporting tests for 99.999% reliability.

  - Option 3: (Apple, Intel)

    - A CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least α% of the time

    - The ratio of the throughput with follow CQI vs median CQI shall be ≥ γ

- Recommended WF

  - TBD

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| XXX | Issue 5-1-1: Issue 5-1-2: Issue 5-1-3: |
| Ericsson | Issue 5-1-1: AWGN; discussed in thread 318 for 10^-5 BLER Issue 5-1-2: We propose 10^-5 BLER; discussed in thread 318. We do not see a use in creating a requirement for 10^-3. (Option 2: No higher BLER CQI requirement) |
| Qualcomm | Issue 5-1-1: Prefer Option 1 Issue 5-1-2: Prefer 10^-5 BLER as discussed in thread 318. Issue 5-1-3: Prefer Option 2, otherwise UE can pass the test by always transmitting CQI 0. |
| Apple | Issue 5-1-1: Option 2. We prefer to avoid long test for CQI reporting. Issue 5-1-2: We cannot define CQI reporting with CQI Table 3 with target BLER other than 10-5 Issue 5-1-3: Option 3 |
| Intel | Issue 5-1-1: Propagation channel for CQI reporting At current stage, we support Option 2. Same time, if test methodology with feasible testing time for AWGN will be defined, based on discussion in e-mail thread #318, then we can accept definition of requirements for AWGN conditions. Issue 5-1-3: Test metric @QC: Could you please clarify how UE which always report CQI0 can pass the test? Based on our understanding, in case UE always reports CQI0, BLER for median CQI will be less then target and probably BLER for median CQI+1 will be also less then target. In result, UE will fail the test. We think that definition of CQI requirements for different SNR points allows to avoid passing of requirements with always reporting of CQI0. |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2010985 (Huawei) CR to TS 38.101-4 Applicability rules for URLLC CSI requirements. | Company A |
|  | Company B |
|  | Ericsson: Use '4-bit CQI Table 3 [12]' instead of 'New CQI table ' Use 'MCS index table 3 for PDSCH [12]' instead of 'New 64QAM MCS table for PDSCH' |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: No Candidate options: Recommendations for 2nd round: The CQI reporting test with 10^-5 is discussed in email thread 318. If a higher BLER is decided, then we come back to this email thread. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2010985 | to be revised |

Discussion on 2nd round

Sub-topic 5-5-1 CQI reporting requirements for support of CQI table 3

The CQI reporting requirements will be discussed in GTW session on Tuesday, and CQI test with 10^-5 is discussing in email thread 318. If a higher BLER will be defined, then we come back to this email thread.

Issue 5-5-1-1: Propagation channel for CQI reporting

- Proposals

  - Option 1: AWGN (Huawei, QC, Ericsson)

  - Option 2: Fading channel (Intel, Apple)

- Recommended WF

  - TBD

Issue 5-5-1-2: Target BLER

- Proposals

  - Option 1: 10^-3 or higher

  - Option 2:

- Recommended WF

  - TBD

Issue 5-5-1-3: Test metric

- Proposals

  - Option 1:

    - The reported CQI value shall be in the range of ±1 of the reported median more than 90% of the time. If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to the target BLER, the BLER using the transport format indicated by the (median CQI + 1) shall be greater than the target BLER. If the PDSCH BLER using the transport format indicated by the median CQI is greater than the target BLER, the BLER using transport format indicated by (median CQI – 1) shall be less than or equal to the target BLER.

  - Option 2: (QC)

    - The reported CQI value shall be in the range of ±1 of the reported median more than 90% of the time. If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to the target BLER, the BLER using the transport format indicated by the (median CQI + 1) shall be greater than the target BLER. If the PDSCH BLER using the transport format indicated by the median CQI is greater than the target BLER, the BLER using transport format indicated by (median CQI – 1) shall be less than or equal to the target BLER.

    - Define a lower bound for median reported CQI in the CQI reporting tests for 99.999% reliability.

  - Option 3: (Apple, Intel)

    - A CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least α% of the time

    - The ratio of the throughput with follow CQI vs median CQI shall be ≥ γ

- Recommended WF

  - TBD

Companies views’ collection for 2nd round

Open issues

| Company | Comments |
| --- | --- |
| Qualcomm | Same views as in 1st round. |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2012653 (from R4-2010985) (Huawei) CR to TS 38.101-4 Applicability rules for URLLC CSI requirements. | Company A |
|  | Company B |
|  | Intel: CSI requirements are still under discussion. Therefore, we suggest to postpone this CR to the next meeting. |

Summary for 2nd round

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR number | Status summary |
| --- | --- |
| R4-2012653 (Revised from R4-2010985) | To be noted |

Topic #6: URLLC UE Rel-16 features

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2010976 | Huawei, HiSilicon | Proposal 2: RAN4 needs further discuss this feature and decide whether the new performance requirements is needed for PDCCH. Proposal 3: Not to define PDSCH performance requirements for URLLC single-DCI multi-TRP transmission. |

Open issues summary

Sub-topic 6-1: Rel-16 URLLC UE features

Issue 6-1-1: Rel-16 features need to be discussed

- Proposals

  - PDCCH enhancement (Huawei, Ericsson)

  - Other features not precluded.

- Recommended WF

  - Multi-TRP has been discussed in e-MIMO

Issue 6-1-2: Whether to define performance requirements for PDCCH enhancement.

- Proposals

  - Option 1: Yes

  - Option 2: No (QC, Apple)

- Recommended WF

  - TBD

Issue 6-1-3: Whether to define performance requirements for multi-TRP transmission for URLLC

- Proposals

  - Option 1: Yes

  - Option 2: No, has been discussed in e-MIMO (Ericsson, QC, Apple, Intel)

- Recommended WF

  - Option 2

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| XXX | Issue |
| Nokia, Nokia Shanghai Bell | Other: The question of whether or not to include Rel-16 features at all, should be discussed for both UE and BS at the same time. |
| Ericsson | In our understanding, the new configurable PDCCH DCI format 0_2 and 1_2, PDCCH monitoring capability enhancements could be candidates for performance requirements and should be considered. We do not see an urgency to complete the requirements in Rel-16 in RAN4 though. Multi-TRP should be discussed in e-MIMO |
| Qualcomm | We don’t think we have enough time to discuss and finalize requirements for Rel-16 features because we have only 2 meetings left and discussion has not even started yet. Issue 6-1-2: Prefer Option 2. Issue 6-1-3: Prefer Option 2. It is already being discussed in eMIMO thread. |
| Apple | Issue 6-1-2: Option 2. We are not sure of how critical it would be to introduce requirements with PDCCH enhancements for URLLC. Also with limited time left this doesn’t seem to be a priority. Issue 6-1-3: This is being discussed in eMIMO thread. |
| Intel | Issue 6-1-1: Rel-16 features need to be discussed We suggest to keep open discussion on requirements for PDCCH enhancements. Multi-TRP transmission is already covered by Rel-16 eMIMO WI. |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Rel-16 features need to be discussed - Rel-16 features need to be discussed: Multi-TRP has been discussed in e-MIMO, no discussion in URLLC thread. Candidate options: Recommendations for 2nd round: Most of the companies would like to concentrate on finishing Rel-15 features, as Rel-16 features are included in the URLLC WI, companies are encouraged to list the Rel-16 features that they would like to discuss in the next meeting in the 2nd round. Rel-16 features need to be discussed - PDCCH enhancement - Other features not precluded. |

Discussion on 2nd round

Sub-topic 6-5-1: Rel-16 URLLC UE features

In URLLC WI RP-191584, the agreed objectives include specifying the performance requirements based on Rel-15 and Rel-16 URLLC functionalities. The candidate features for Rel-15 were listed in the WI but did not list for Rel-16. In this section, companies are encouraged to propose the candidate features they would like to discuss for Rel-16 in the next meeting.

Issue 6-5-1-1: Rel-16 features need to be discussed

- Proposals

  - PDCCH enhancement (Huawei, Ericsson)

  - Other features not precluded.

- Recommended WF

  - Discuss in next meeting.

Issue 6-5-1-2: Whether to define performance requirements for PDCCH enhancement.

- Proposals

  - Option 1: Yes

  - Option 2: No (QC, Apple)

- Recommended WF

  - TBD

Companies views’ collection for 2nd round

Open issues

| Company | Comments |
| --- | --- |
| XXX |  |
| Intel | Issue 6-5-1-1/6-5-1-2 We suggest to list all potential Rel-16 features for requirements definition and discuss in the next meeting on whether to define requirements. |

Summary for 2nd round

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#6-1 | Agreements: Requirements for Rel-16 URLCC feature - Requirements for Multi-TRP URLLC transmission schemes are covered by Rel-16 e-MIMO WI, no discussion in URLLC thread. - Rel-16 features need to be discussed - PDCCH enhancement - Other features not precluded. |

Topic #7: Test applicability, release independent for URLLC UE

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2010981 | Huawei, HiSilicon | Proposal 8: UE URLLC requirements for Rel-15 features are release independent from Rel-15. Proposal 9: No need to introduce RAN4 features and capability for URLLC requirements for Rel-15 features. |

From the approved WF R4-2008807 in RAN4 #95 e-meeting, remaining open issues are listed as follows:

Open issues:

- FFS whether UE URLLC requirements for Rel-15 features are release independent from Rel-15

- FFS necessity of introduction of RAN4 features and UE applicability for URLLC requirements for Rel-15 features

Open issues summary

Sub-topic 7-1: Release independence, Test applicability

Issue 7-1-1: UE URLLC requirements for Rel-15 features release independent from Rel-15

- Proposals

  - Option 1: Yes (Huawei, DoCoMo)

  - Option 2: No (Apple)

- Recommended WF

  - TBD

QC: We prefer not to apply the requirements in release independent manner. For UE feature list, we believe no need to additional feature since Rel-15 feature list already there.

Apple: similar view as QC.

China Telecomm: we prefer to apply the requirements starting from Rel-15 as these are rel-15 features. NO new capability needed.

E///: We have similar view as China Telecomm, applied from Rel-15.

Huawei: These features still optional for Rel-15 UEs. We prefer option 1 applied from Rel-15.

NTT DoCoMo: share similar view as China Telecomm.

Apple: Requirements not introduced in Rel-16 even the feature introduced in Rel-15, how we can guarantee UE can pass these requirements since we don’t plan to introduce additional capability signalling.

Samsung:

QC: Share similar concern as Apple. For already commercial UE, how to ensure these UEs can pass such requirements.

Intel: We are fine with both options. If applied with Rel-15, no need new features. We have similar discussion in performance enhancement WI, we already agree some requirements introduced in Rel-16 for Rel-15 features will be applied from Rel-15.

DoCoMo:

Huawei: The requirements applied for new Rel-15 UE, not applied for the Rel-15 UE which already in the market.

E///: Share same understanding as Huawei.

QC: For performance enhancement WI, these features we already requirements in Rel-15 with some extension. Our concern is for UE which on developing and planned to support these features.

Intel: From performance enhancement, we mentioned for PMI requirements with larger 8 Tx ports.

Agreement:

Tentative agreement: No additional features and capability needed for URLLC Demod and CSI requirements introduced for Rel-15 feature under Rel-16 URLLC WI.

- Intel: Want to further check and confirm before Friday.

- [Moderator]: Intel checked and confirmed the tentative agreement is fine.

For the requirement release independent, need to further discussed, whether and how to apply the requirements for Rel-15 UEs.

- Will further check the status in 2nd round

QC: For Rel-16 UEs should pass requirements they declared to support. The only issue for Rel-15 legacy UE.

Intel: If Rel-15 UE implemented these features and indicated supported these features, what’s gNB assumption? If we decide not to apply RAN4 demod requirements for these UEs.

QC: gNB can always follow the capability signalling no matter whether such features already by RAN4 requirements, this is general approach always can applied.

Apple: If additional Rel-16 feature/capability introduced does mean such new feature/capability apply for Rel-15 UEs?

Why we need these additional signalling, since they can resolve the issue.

- Recommended WF

  - Tentative agreement from GTW: For the requirement release independent, need to further discussed, whether and how to apply the requirements for Rel-15 UEs.

Will further check the status in 2nd round

Issue 7-1-2: Necessity of introduction of RAN4 features and UE applicability for URLLC requirements for Rel-15 features

- Proposals

  - Option 1: No need. (Huawei, DoCoMo, Intel)

  - Option 2:

- Recommended WF

  - Tentative agreement from GTW: No additional features and capability needed for URLLC Demod and CSI requirements introduced for Rel-15 feature under Rel-16 URLLC WI.

Sub-topic 7-2: Structure for specifications

In #95 e-meeting, we have agreements on the specification layout:

Agreements:

How to capture URLLC features in specifications and structure for test cases capturing:

- Introduce new sections within the existing PDSCH requirements for each separate feature when incorporate URLLC requirements into the specification.

- Structure for test cases capturing: Reuse existing Rel-15 eMBB test case structure to capture new demodulation requirements from URLLC WI.

In this sub-topic, the section numbers will be discussed and allocated. According to the agreements, the new features should be added following the existing requirements:

Issue 7-2-1: Section numbers and title for TS38.101-4

- Proposals

2RX:

For FDD:

Option 1: (Intel)

5.2.2.1.5 Minimum requirements for PDSCH  0.001% BLER

5.2.2.1.6 Minimum requirements for PDSCH repetitions over multiple slots

5.2.2.1.7 Minimum requirements for PDSCH mapping Type B and processing capability 2

5.2.2.1.8 Minimum requirements for PDSCH pre-emption

Option 2:

5.2.2.1.5 Minimum requirements for PDSCH for higher reliability

5.2.2.1.5.1 Minimum requirements for PDSCH  0.001% BLER

5.2.2.1.5.2 Minimum requirements for PDSCH repetitions over multiple slots

5.2.2.1.6 Minimum requirements for PDSCH for lower latency

5.2.2.1.6.1 Minimum requirements for mapping Type B and processing capability 2

5.2.2.1.3.2 Minimum requirements for PDSCH pre-emption

Option 3:

5.2.2.1.5 Minimum requirements for PDSCH for high reliability URLLC features

5.2.2.1.5.1 Minimum requirements for PDSCH  0.001% BLER

5.2.2.1.5.2 Minimum requirements for PDSCH repetitions over multiple slots

5.2.2.1.6 Minimum requirements for PDSCH for low latency URLLC features

5.2.2.1.6.1 Minimum requirements for mapping Type B and processing capability 2

5.2.2.1.3.2 Minimum requirements for PDSCH pre-emption

[Moderator]: Companies provide views for 2Rx FDD, then 2Rx TDD and 4Rx FDD/TDD will follow the agreements. For section numbers, the moderator is aligning them with HST-UE and an agreement will be made later. HST-UE has not decided how many sections it needs, if we can have an agreement of issue 7-2-1, then we can start from 5.2.2.1.5.

For TDD:

5.2.2.2.5 Minimum requirements for PDSCH 0.001% BLER

5.2.2.2.7 Minimum requirements for PDSCH repetitions over multiple slots

5.2.2.2.8 Minimum requirements for PDSCH mapping Type B and processing capability 2

5.2.2.2.9 Minimum requirements for PDSCH pre-emption

4Rx:

For FDD:

5.2.3.1.6 Minimum requirements for PDSCH  0.001% BLER

5.2.3.1.7 Minimum requirements for PDSCH repetitions over multiple slots

5.2.3.1.8 Minimum requirements for PDSCH mapping Type B and processing capability 2

5.2.3.1.9 Minimum requirements for PDSCH pre-emption

For TDD:

5.2.3.2.6 Minimum requirements for PDSCH  0.001% BLER

5.2.3.2.7 Minimum requirements for PDSCH repetitions over multiple slots

5.2.3.2.8 Minimum requirements for PDSCH mapping Type B and processing capability 2

5.2.3.2.9 Minimum requirements for PDSCH pre-emption

- Recommended WF

  - TBD

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| XXX | Issue |
| Nokia, Nokia Shanghai Bell | Other: While not strictly necessary, we might want to aim for aligning the section naming schemes between UE and BS (within reason). On the BS side, we are proposing to introduce subsections in “PxSCH” called “for lower latency” and “for higher reliability”, which hold all current (and future) features/requirements pertaining to the respective goal. Please consider, if such an approach would make sense also for the UE side. |
| Ericsson | Issue 7-2-1: As for the BS, we should avoid the term “ultra-low BLER” in the specifications as it would cause a problem with naming if a requirement with even lower BLER would be introduced in the future. Alternative possibilities are “0.001% BLER” or “low BLER” (other suggestions welcome). |
| Qualcomm | Issue 7-1-1: Ok with Option 2. Issue 7-1-2: This is not clear. We already have capability bits for different UE features. So, we are not sure which capability it is referring to. Issue 7-2-1: We need to discuss this numbering along with new section for HST-SFN requirements as CRs for HST-SFN seem to also assume Section 5.2.3.1.5 for defining HST-SFN requirements. |
| Apple | Issue 7-1-1: We support Option 2. There is a risk with making it release independent for the Rel-15 UEs under development and supporting the features and not meeting requirements as it was never tested with the requirements during development. Issue 7-1-2: We need to further discuss the need to introduce feature for URLLC demod requirements. Issue 7-2-2:We are fine with the proposed structure. |
| NTT DOCOMO | Issue 7-1-1: We prefer Option 1. Issue 7-1-2: We prefer Option 1. |
| Samsung | Issue 7-1-1: UE URLLC requirements for Rel-15 features release independent from Rel-15 We are fine both option 1 and option2, while further clarification is needed. In Rel-16 URLLC, RAN4 introduce the requirement with ulta-low BLER targeting 10^-5 and high reliability BLER targeting 10^-2, We wonder whether the release independent should be applied for all the requirement? The test feasibility and test procedure with 10^-5 BLER is discussed and introduced in Rel-16. Even the requirement defined is based on Rel-15 URLLC core requirement, we wonder whether Rel-15 commercial UE can pass this requirement 10^-5 BLER with given confidence level? |
| Intel | Issue 7-1-2: Necessity of introduction of RAN4 features and UE applicability for URLLC requirements for Rel-15 features As we commented before: If requirements are defined from Rel-16 then we suggest to define one Rel-16 RAN4 feature and corresponding capability signalling to indicate that UE supports URLLC requirements for Rel-15 URLLC features. Introduction of such capability will ensure a clear indication that UE supporting certain URLLC related features was verified and we can guaranty that UE correctly operates under these conditions. Taking into account, that most of companies think that this is not the issue, we are fine to mark tentative agreement as agreement. Issue 7-2-1: Section numbers and title for TS38.101-4 Prefer Option 1, because terminology “higher reliability” and “lower latency” is rather confusing from specification reading point of view (it is not clear higher and lower in comparison to what?). Also, the following terminology probably better if Option 2 will be considered 5.2.2.1.5 Requirements for PDSCH for high reliability URLLC features 5.2.2.1.6 Requirements for PDSCH for low latency URLLC features |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Necessity of introduction of RAN4 features and UE applicability for URLLC requirements for Rel-15 features [Moderator]We discussed this issue on the GTW session and made a tentative agreement. Intel wanted to further check and confirm. After the session, Intel checked and confirmed the tentative agreement is fine. Tentative agreement from GTW: No additional features and capability needed for URLLC Demod and CSI requirements introduced for Rel-15 feature under Rel-16 URLLC WI. Candidate options: Recommendations for 2nd round: UE URLLC requirements for Rel-15 features release independent from Rel-15: We discussed this issue on the GTW session and companies would like to further check in 2nd round. Sub-topic 7-2: Structure for specifications: continue to discuss in 2nd round. |

Discussion on 2nd round

Sub-topic 7-5-1: Release independence, structure for specification

Issue 7-5-1-1: UE URLLC requirements for Rel-15 features release independent from Rel-15

This issue has been discussed on the GTW in 1st round, no agreement was reached. Companies encouraged to continue provide their views on why or why not release independent from Rel-15. The status will be checked on GTW in 2nd round.

- Proposals

  - Option 1: Yes (Huawei, DoCoMo, Ericsson, Samsung)

  - Option 2: No (Apple, QC, Samsung)

- Recommended WF

  - TBD

Sub-topic 7-5-2: Structure for specifications

Companies provide views for 2Rx FDD, then 2Rx TDD and 4Rx FDD/TDD will follow the agreements. The section numbers should be aligned with HST-UE. As HST-UE has not decided how many sections it needs, so if we can have an agreement on issue 7-5-2-1, then we can start from 5.2.2.1.5. Companies are encouraged to provide their opinions ASAP to make progress for CRs.

Issue 7-5-2-1: Section numbers and title for TS38.101-4

- Proposals

2RX:

For FDD:

  - Option 1: (Intel, Huawei, Samsung, Ericsson, QC, Apple)

5.2.2.1.5 Minimum requirements for PDSCH 0.001% BLER

5.2.2.1.6 Minimum requirements for PDSCH repetitions over multiple slots

5.2.2.1.7 Minimum requirements for PDSCH Mapping Type B and UE processing capability 2

5.2.2.1.8 Minimum requirements for PDSCH pre-emption

  - Option 2:

5.2.2.1.5 Minimum requirements for PDSCH for higher reliability

5.2.2.1.5.1 Minimum requirements for PDSCH 0.001% BLER

5.2.2.1.5.2 Minimum requirements for PDSCH repetitions over multiple slots

5.2.2.1.6 Minimum requirements for PDSCH for lower latency

5.2.2.1.6.1 Minimum requirements for mapping Type B and processing capability 2

5.2.2.1.3.2 Minimum requirements for PDSCH pre-emption

  - Option 3:

5.2.2.1.5 Minimum requirements for PDSCH for high reliability URLLC features

5.2.2.1.5.1 Minimum requirements for PDSCH 0.001% BLER

5.2.2.1.5.2 Minimum requirements for PDSCH repetitions over multiple slots

5.2.2.1.6 Minimum requirements for PDSCH for low latency URLLC features

5.2.2.1.6.1 Minimum requirements for mapping Type B and processing capability 2

5.2.2.1.3.2 Minimum requirements for PDSCH pre-emption

- Recommended WF

  - Option 1

Sub-topic 7-5-3: CR work split for UE

Issue 7-5-3-1: Work split for UE

| Specifacation | Requirements title | Frequency range | CR work | CR Responsibility |
| --- | --- | --- | --- | --- |
| UE demodulation and CSI (38.101-4) | Test applicability for all cases | FR1 | Test applicability | Huawei, HiSilicon |
|  |  | FR2 | Test applicability | Huawei, HiSilicon |
|  | FRC for all cases | FR1 | FRC | [Intel] |
|  |  | FR2 | FRC | Intel |
|  | Requirements for PDSCH with ultra-low BLER target (10-5) | FR1 | Requirements | [Ericsson] |
|  |  | FR2 | FFS | FFS |
|  | Requirements for PDSCH with aggregation factor configured | FR1 | Requirements | Huawei, HiSilicon |
|  |  | FR2 | FFS | FFS |
|  | Requirements for PDSCH for mapping Type B and processing capability 2 | FR1 | Requirements | [Qualcomm] |
|  | Requirements for PDSCH for mapping Type B | FR2 | Requirements | Ericsson |
|  | Requirements for PDSCH for pre-emption indication | FR1 | Requirements | [Ericsson] |
|  |  | FR2 | N/A | N/A |
|  | CQI reporting test for table 3 | FR1 | Requirements | [Apple] |
|  |  | FR2 | FFS | FFS |

Companies views’ collection for 2nd round

Open issues

| Company | Comments |
| --- | --- |
| Huawei | Issue 7-5-2-1: We prefer option 1. For option 2 and 3, it is confuse to declare what is “higher” and “lower”. |
| Intel | Issue 7-5-2-1: Section numbers and title for TS38.101-4 Prefer Option 1. Option 3 is also fine for us. Wording “higher” and “lower” for Option 2 is confusing. Issue 7-5-3-1: Work split for UE Intel volunteers to prepare FRC for FR2 requirements. |
| Samsung | Issue 7-5-1: we are option 1 and option2, As mentioned, further clarification is needed. In Rel-16 URLLC, RAN4 introduce the requirement with ulta-low BLER targeting 10^-5 and high reliability BLER targeting 10^-2, We wonder whether the release independent should be applied for all the requirement? The test feasibility and test procedure with 10^-5 BLER is discussed and introduced in Rel-16. Even the requirement defined is based on Rel-15 URLLC core requirement, we wonder whether Rel-15 commercial UE can pass this requirement 10^-5 BLER with given confidence level? |
| Ericsson | Issue 7-5-1-1: We prefer option 1. For a network to be able to confidently deploy high reliability it needs to be clear that the UEs meet minimum requirements so Rel-15 UEs declaring support for these features without meeting requirements would not be useful. Issue 7-5-2-1: Prefer option 1. We volunteer for FR2 PDSCH mapping type B requirements |
| Qualcomm | Issue 7-5-1-1: Prefer Option 2. Issue 7-5-2-1: Ok with Option 1. One minor comment: For 5.2.2.1.7, we prefer the title as “Minimum requirements for PDSCH Mapping Type B and UE processing capability 2” to align with previous titles and make it more clear. |
| Apple | Issue 7-5-1-1: UE URLLC requirements for Rel-15 features release independent from Rel-15 Option 2: Do not introduce requirements for REl-15 URLLC features as release independent from Rel-15. Same reason as stated earlier: There is a risk with making it release independent for the Rel-15 UEs under development and supporting the features and not meeting requirements as it was never tested with the requirements during development. Issue 7-5-2-1: Section numbers and title for TS38.101-4 Option 1 or 3 is fine with slight preference for option 1, as it reduces the levels introduced Update 08/25 Issue 7-5-3-1: Work split for UE We volunteer to take up CR preparation for FR2 PDSCH slot aggregation requirements |

Summary for 2nd round

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#7-5-1/7-5-2/7-5-3 | Agreements: Test applicability - FFS whether UE URLLC requirements for Rel-15 features are release independent from Rel-15 - No additional features and capability needed for URLLC Demod and CSI requirements introduced for Rel-15 feature under Rel-16 URLLC WI. - Work split: |

Topic #8: BS demodulation requirements for high reliability

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2009701 | NTT DOCOMO, INC. | Proposal 1: For high-reliability test, configure aggregation level n2 for FDD 15kHz SCS and n8 for TDD 15kHz SCS (Option 3). Proposal 2: Regarding FR2 SCS/CBW for high reliability tests, adopt option 2 or 2a: Option 2: 50/100MHz for 60 kHz SCS, 50/100MHz for 120kHz SCS Option 2a: 50MHz for 60 kHz SCS, 50/100MHz for 120kHz SCS Proposal 3: Introduce both no additional DM-RS (pos0) and one additional DM-RS (pos1) for high-reliability requirements. Proposal 4: High-reliability tests shall be done for both FR1 and FR2 if BS declares support both, and all supported SCS will be tested for FR1 and only 1 SCS need to be tested for FR2 (Option 3). |
| R4-2009727 | Intel Corporation | Proposal 1: Configure n2 for FDD and n8 for TDD for FR1 BS High reality requirements. Proposal 2: Use the following assumptions for FR2 BS High reliability requirements: - CBS/SCS: 60 KHz for 50/100 MHz and 120 KHz for 50/100 MHz with test applicability rule - TDD UL/Dl pattern and aggregation factor - Option 1: DDDSU with S=10D:2G:2U and PUSCH aggregation factor = n8 - Option 1: DSUU with S=102:2G and PUSCH aggregation factor = n2 - PUSCH mapping Type A and B with test applicability rule - MCS 5 from Table 3 - Channel model: TDLA30-300 |
| R4-2009857 | Nokia, Nokia Shanghai Bell | Observation 1: Optimized implementations might use the aggregation for performance improvement (over HARQ), even in cases where the coherence is low between the repetitions. Proposal 1: RAN4 to choose to configure n2 for FDD and n8 for TDD with note. Proposal 2: RAN4 to take a recorded majority decision on to inclusion of a safety statement clarification. Observation 2: FR2 and high reliability do not make a very practical use case. |
| R4-2010282 | Samsung | Proposal 3: The RV sequence with 4 HARQ transmission is 0 3 0 3 under full buffer scenario. Observation 1: Configured PUSCH aggregation factor with n8 is not a typical scenario to meet the URLLC requirement with high reliability and low latency for 15 KHz SCS with TDD patter DDDSU. Proposal 4: No PUSCH aggregation requirement for TDD with 15 KHz SCS Observation 2: The RV value for effective UL transmission with PUSCH aggregation level n8 is same as TDD 30 KHz with PUSCH aggregation level n2. Proposal 5: If the slot aggregation requirement with n8 for TDD with 15KHz SCS is introduced, adding a note as The requirement with PUSCH aggregation level n8 for TDD with 15 KHz SCS can be applied with FDD 15KHz SCS or TDD 30 KHz SCS with PUSCH aggregation level n2. Proposal 6: URLLC tests for requirement of high reliability with high BLER shall be done for either FR1 or FR2 based on BS declaration if BS declares to support both FR1 and FR2. Proposal 7: Only define FR2 URLLC requirement with 60 KHz SCS for 50MHz, and 120 KHz SCS for 50MHz |
| R4-2010838 | Ericsson | Proposal 1: Define the requirement and test for 15kHz based on TDD, but equally applicable for FDD (as for other requirements) Proposal 2: To clarify the safety statement add the note on requirement implications according to option 1b from [2] |
| R4-2010839 | Ericsson | Simulation results for URLLC “high BLER” requirements |
| R4-2010837 | Ericsson | Draft CR to 38.141-1 on test methodology and FRCs for URLLC and test requirements for 0.001% BLER |
| R4-2010990 | Huawei, HiSilicon | CR to TS38.141-1 Test applicability for URLLC BS demodulation requirements |
| R4-2010983 | Huawei, HiSilicon | Proposal 1: Only configure n2 for FDD and no requirement for TDD. Proposal 2: No need to specify any safety statements in specification. Proposal 3: Tests shall be done for both, and only 1 SCS will be tested for each frequency band with test applicability rule. |
| R4-2010984 | Huawei, HiSilicon | Simulation results on BS demodulation requirements with higher BLER |
| R4-2010988 | Huawei, HiSilicon | CR to TS38.104 Performance requirements for URLLC PUSCH repetition Type A |
| R4-2010989 | Huawei, HiSilicon | CR to TS38.141-1 Performance requirements for URLLC BS demodulation requirements with higher BLER |
| R4-2010991 | Huawei, HiSilicon | CR to TS38.141-2 FRC for URLLC BS performance requirements |
| R4-2011396 | NTT DOCOMO, INC. | Draft CR for TS 38.141-2: Introduction of performance requirements of PUSCH repetition type A and PUSCH mapping type B for URLLC |

Open issues summary

During the last meeting, most of the test parameters for FR1 were agreed. In this section, the remaining open issues will be discussed.

Agreements:

- SCS/CBW: 15 KHz for 5/10 MHz, 30 KHz for 10/40 MHz with test applicability rule.

- Whether to define requirements for FR2：Yes with test applicability rule.

  - 3a: The performance requirements are only applicable for BS only supporting FR2 or supporting both FR1 and FR2

Open issues:

- PUSCH aggregation factor for 15 kHz SCS:

  - Option 1: No requirement for 15 kHz SCS.

  - Option 2: Only configure n2 for FDD and no requirement for TDD.

  - Option 3: Configure n2 for FDD and n8 for TDD with note.

- Note: The intention of this configuration is to have two effective transmissions of the transport block. To achieve this for the standard TDD pattern captured in this table, a value of n8 is necessary, while for FDD a value of n2 is necessary.

- Whether to clarify the safety statement

  - Option 1: No need to specify any safety statements in specification

  - Option 2: Yes

    - Option 1a: Since the URLLC features of 5G NR will potentially be used in safety critical applications, the ultimately chosen statistical testing methodology for testing of these features must be verified by an independent body of experts/statisticians, before requirements and test can be used as basis for safety critical implementations. All statistical analysis and discussions provided in this meeting are to be taken as a best effort and is not to be taken as due diligence

    - Option 1b: If the URLLC features of 5G NR would be used in safety or mission critical applications, the ultimately chosen statistical testing methodology for testing of these features must be verified by an independent body of experts/statisticians. It is also important to bear in mind that the demodulation requirements do not take account of all aspects of system operation (for example RF, transmitter, internal interfaces, higher layer protocol software etc.).

- Test applicability rule for FR1 and FR2 if BS declare to support both FR1 and FR2:

  - Option 1: Tests shall be done for either FR1 or FR2

  - Option 2: Tests shall be done for both , and only 1 SCS will be tested for each frequency band with test applicability rule.

  - Option 3: Tests shall be done for both, all supported SCS will be tested for FR1 and only 1 SCS need to be tested for FR2.

- SCS/BW for FR2 (only if FR2 is defined)

  - 60 KHz:

    - Option 1: 50 MHz

    - Option 2: 50 MHz and 100 MHz

  - 120 KHz

    - Option 1: 100 MHz

    - Option 2: 50 MHz and 100 MHz In this paper, the remained open issues are discussed and our views are provided.

Sub-topic 8-1: BS demodulation requirements of high reliability for FR1

In this sub-topic, the open issues for FR1 will be discussed:

Open issues and candidate options before e-meeting:

Issue 8-1-1: PUSCH aggregation factor for TDD 15 kHz SCS with pattern DDDSU:

- PUSCH aggregation factor for 15 kHz SCS:

  - Option 1: No requirement for 15 kHz SCS.

  - Option 2: Only configure n2 for FDD and no requirement for TDD. (Samsung, Huawei)

  - Option 3: Configure n2 for FDD and n8 for TDD with note. (DoCoMo, Intel, Ericsson, Nokia)

    - Note: The intention of this configuration is to have two effective transmissions of the transport block. To achieve this for the standard TDD pattern captured in this table, a value of n8 is necessary, while for FDD a value of n2 is necessary.

- Recommended WF

  - Option 2?

Issue 8-1-2: RV sequence with 4 HARQ transmission:

- Proposal:

  - Option 1: 0 3 0 3 under full buffer scenario. (Samsung, Intel)

  - Option 2:

- Recommended WF

  - TBD

Issue 8-1-3: Applicability rule for FDD and TDD (if n8 for TDD 15 kHz is introduced):

- PUSCH aggregation factor for 15 kHz SCS:

  - Option 1: The requirement with PUSCH aggregation level n8 for TDD with 15 KHz SCS can be applied with FDD or TDD 30 KHz SCS with PUSCH aggregation level n2. (Samsung, Nokia, Intel)

  - Option 2:

- Recommended WF

  - TBD

Issue 8-1-4: Whether to clarify the safety statement in specification

- Proposals

  - Option 1: No need to specify any safety statements in specification (Huawei)

  - Option 2: Yes (Nokia)

    - Option 1a: Since the URLLC features of 5G NR will potentially be used in safety critical applications, the ultimately chosen statistical testing methodology for testing of these features must be verified by an independent body of experts/statisticians, before requirements and test can be used as basis for safety critical implementations. All statistical analysis and discussions provided in this meeting are to be taken as a best effort and is not to be taken as due diligence

    - Option 1b: (Ericsson)

If the URLLC features of 5G NR would be used in safety or mission critical applications, the ultimately chosen statistical testing methodology for testing of these features must be verified by an independent body of experts/statisticians. It is also important to bear in mind that the demodulation requirements do not take account of all aspects of system operation (for example RF, transmitter, internal interfaces, higher layer protocol software etc.).

- Recommended WF

  - TBD

Sub-topic 8-2: BS demodulation requirements of high reliability for FR2

Open issues and candidate options before e-meeting:

Issue 8-2-1: Test applicability rule for FR1 and FR2

- Proposals

  - Option 1: If BS declare to support both FR1 and FR2, the tests shall be done for either FR1 or FR2 (Samsung)

  - Option 2: If BS declare to support both FR1 and FR2, the tests shall be done both (DoCoMo, Ericsson, Huawei, Intel)

- Recommended WF

  - TBD

Issue 8-2-2: Test applicability rule for FR2 for different SCS

- Proposals

  - Option 1: Only 1 SCS need to be tested. (DoCoMo, Huawei, Nokia, Samsung, Intel)

  - Option 2:

- Recommended WF

  - Option 1

Issue 8-2-3: Antenna configuration

- Proposals

  - Option 1: 1x2, ULA low (Ericsson, Nokia, Samsung, Huawei, DoCoMo, Intel)

  - Option 2:

- Recommended WF

  - Option 1

Issue 8-2-4: SCS/BW for FR2

- Proposals

  - 60 KHz:

    - Option 1: 50 MHz (DoCoMo, Samsung, Ericsson, Nokia, Intel)

    - Option 2: 50 MHz and 100 MHz (DoCoMo, Intel)

  - 120 KHz

    - Option 1: 50 MHz (Samsung, Ericsson, Nokia, , Intel)

    - Option 2: 50 MHz and 100 MHz (DoCoMo, Intel)

- Recommended WF

  - Option 1 for both?

Issue 8-2-5: TDD pattern

- Proposals

  - Option 1: DDDSU, S=10:2:2 (Intel, Ericsson, Nokia, DoCoMo, Huawei)

  - Option 2: DSUU, S=12:2 (Samsung)

- Recommended WF

  - TBD

Issue 8-2-6: Aggregation factor for TDD

- Proposals

  - Option 1: n8 for DDDSU (Intel, Ericsson, DoCoMo)

  - Option 2: n2 for DSUU (Samsung)

- Recommended WF

  - Following the decision from FR1 if TDD pattern “DDDSU” is configured. (Nokia)

Issue 8-2-7: Mapping type

- Proposals

  - Option 1: Type A and Type B with test applicability rule (Intel, Nokia)

  - Option 2: Type B (Ericsson, Nokia, Samsung, DoCoMo, Huawei, Intel)

- Recommended WF

  - Option 2?

Issue 8-2-8: MCS

- Proposals

  - Option 1: MCS5 from table 3 (Intel, Nokia, Samsung)

  - Option 2: MCS2 from table 3 (Ericsson, Samsung)

- Recommended WF

  - TBD

Issue 8-2-9: Channel model

- Proposals

  - Option 1: TDLA30-300 Low (Intel, Ericsson, Samsung, DoCoMo)

  - Option 2: TDLA30-75 (Nokia)

- Recommended WF

  - TBD

Issue 8-2-10: Start symbol

- Proposals

  - Option 1: 0 (Ericsson, Nokia, Samsung, Huawei, DoCoMo)

  - Option 2:

- Recommended WF

  - Option 1

Issue 8-2-11: Symbol length

- Proposals

  - Option 1: 10 (Ericsson, Samsung, DoCoMo, Intel)

  - Option 2: 14 (Nokia)

- Recommended WF

  - TBD

Issue 8-2-12: DM-RS

- Proposals

  - Option 1: 1+0 and 1+1. (DoCoMo)

  - Option 2: 1+1 (Ericsson, Nokia, Huawei, Samsung, Intel)

- Recommended WF

  - TBD

Issue 8-2-13: DM-RS Type

- Proposals

  - Option 1: Type 1 (Ericsson, Nokia, Huawei, Samsung, DoCoMo, Intel)

  - Option 2:

- Recommended WF

  - Option 1

Issue 8-2-14: DM-RS duration

- Proposals

  - Option 1: Single-symbol DM-RS (Ericsson, Nokia, Huawei, Samsung, DoCoMo, Intel)

  - Option 2:

- Recommended WF

  - Option 1

Issue 8-2-15: Bandwidth allocation

- Proposals

  - Option 1: Full bandwidth (Ericsson, Nokia, Huawei, Samsung, DoCoMo, Intel)

  - Option 2:

- Recommended WF

  - Option 1

Issue 8-2-16: Maximum number of HARQ re-transmissions

- Proposals

  - Option 1: 4 (Ericsson, Nokia, Huawei, Samsung, DoCoMo, Intel)

  - Option 2:

- Recommended WF

  - Option 1

Issue 8-2-17: Test metric

- Proposals

  - Option 1: 10^-2 (Calculate the residual BLER after all transmission) (Ericsson, Nokia, Huawei, Samsung, DoCoMo, Intel)

  - Option 2:

- Recommended WF

  - Option 1

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| XXX | Issue |
| Nokia, Nokia Shanghai Bell | Issue 8-1-1: PUSCH aggregation factor for TDD 15 kHz SCS with pattern DDDSU It is possible for receiver implementation to make use of slot repetition, even if this repetition is already 5 slots “old”. Even in cases with low coherence between the repetitions (i.e., large Doppler spread) gains in addition to soft combination should be observable. Hence, we prefer option 3. Issue 8-1-2: RV sequence with 4 HARQ transmission We recognize the potential issue raised by Samsung. However, we need to clarify what the HARQ RV sequence, as captured in the test configuration, exactly means in case of slot repetition. It was our understanding that this RV sequence is the resulting/effective sequence. Since BS demod testing does not have a real trigger of PUSCH (via DCI with RV for example), we assumed that the theoretical trigger was chosen such that the ultimately observed RV sequence is 0,2,3,1. It is understood that this trigger sequence should conform to 0,3,0,3 to achieve this goal. A common decision on what the captured RV sequences means (effective vs. triggered) should be made and a corresponding note added to the configuration table for slot repetition (type A). Nokia support the “effective” RV sequence interpretation. Issue 8-1-3: Applicability rule for FDD and TDD (if n8 for TDD 15 kHz is introduced): Agree with Option 1, as per our position in issue 8-1-1. Issue 8-1-4: Whether to clarify the safety statement in specification We are fine with both alternatives for option 2. The text itself is not so critical for us, but rather that all companies voice their opinion on yes/no. Issue 8-2-2 - Test applicability rule for FR2 for different SCS: We agree with option 1. Issue 8-2-3 - Antenna configuration: 1x2, since OTA does not support otherwise. Issue 8-2-4 - SCS/BW for FR2: 50MHz only for either seems like a good compromise. Issue 8-2-5 - TDD pattern: In the interest of reducing test implementation effort, it is preferable to not include a new TDD pattern. I.e., option 1. Issue 8-2-6 - Aggregation factor for TDD: Follow the decisions from FR1. Issue 8-2-7 - Mapping type: No strong opinion. Issue 8-2-8 - MCS: Follow the FR1 decision. Issue 8-2-9 - Channel model: Use TDLA30-75. Mobility/Doppler impact should be minimized in high reliability testing. Issue 8-2-10 - Start symbol: Follow the FR1 decision. Issue 8-2-11 - Symbol length: Follow the FR1 decision. Issue 8-2-12 - DM-RS: Follow the FR1 decision. Issue 8-2-13 - DM-RS Type: Follow the FR1 decision. Issue 8-2-14 - DM-RS duration: Follow the FR1 decision. Issue 8-2-15 - Bandwidth allocation: Follow the FR1 decision. Issue 8-2-16 - Maximum number of HARQ re-transmissions: Follow the FR1 decision. Issue 8-2-17 - Test metric: Follow the FR1 decision. |
| Ericsson | Issue 8-1-1: As indicated, we support option 3. We would prefer if the test were configured with the same TDD pattern so that it is common with other tests, but a note states that the same requirement applies for FDD with n2. Actually though it would also be fine to have a note that the requirement is equivalent for FDD and TDD with n2/8 and that the test may be done with either FDD or TDD though. Issue 8-1-2: We would like to clarify that the 0 3 0 3 refers to the RV at the beginning of a HARQ retransmission of a series of aggregated slots. If this is the case, we are OK with the option 1. Issue 8-1-3: We are OK with option 1; presumably this means that testing can be done with either FD n2 or TDD n8 ? Issue 8-2-1. 8-2-2: A BS supporting both FR seems quite unusual, but we believe that in such a case both FR should be tested. There may be some RX digital processing functionality in the RF (or even the RF itself may impact the ability to meet BB demod in this case). It is also not given that the same baseband would be applied for both. We think that it is sufficient to test one SCS per FR in this case. Issue 8-2-7: Note that the existing rel-15 PUSCH requirements for FR2 are defined with just type B. Issue 8-2-12: Testing with 2 DM-RS would double the testing with no obvious reason for a performance difference. Is an applicability rule proposed ? |
| Samsung | Issue 8-1-1: PUSCH aggregation factor for TDD 15 kHz SCS with pattern DDDSU: We are fine with option2. The motivation to configure n8 for TDD pattern DDDSU with 15 KHz is to guarantee two effective transmissions of the transport block. While the shortage for this configuration, the transmission delay will be increased in order to complete the transmission, due to the unavailable UL slots with TDD. Since the purpose of URLLS is designed to high reliability and low latency, in that sense, it seems that configured PUSCH aggregation factor with n8 is not a typical scenario to meet the URLLC requirement for 15 KHz SCS with TDD pattern DDDSU. Therefore, we prefer to no requirement for TDD with 15 KHz SCS. Meanwhile, based on the RV allocation for each transmission, it can be calculated that the RV for effective UL transmission, for PUSCH aggregation level n8 with TDD 15 KHz, is the same as TDD 30 KHz SCS with PUSCH aggregation level n2, where RV value is calculated with mod (n, 4) operation for nth transmission From the demodulation requirements perspective, the performance with PUSCH aggregation level n8 is not expected to show difference with PUSCH aggregation level n2 for FDD, considering the number of effective UL transmission is 2. Therefore, it is no need to define the URLLC requirement with PUSCH aggregation level n8 for TDD with 15 KHz. Issue 8-1-2: RV sequence with 4 HARQ transmission: We are fine with option1. As agreed, both HARQ and slot aggregation are enable for requirement for high BLER. The RV value for 4 HARQ transmission without aggregation {0 2 3 1}. With enabling slot aggregation, the RV allocation based {0 2 3 1} for HARQ retransmission is {0 2 2 3 3 1 1 0}. To guarantee all the possible RV at least with 2 re-transmission, we prefer to define the RV sequence with 4 HARQ transmission as {0 3 0 3} under full buffer scenario RTT Issue 8-1-3: Applicability rule for FDD and TDD (if n8 for TDD 15 kHz is introduced): As analysed in issue 8-1-1, we do not see the performance difference with PUSCH aggregation level n8 for TDD with 15KHz compared with PUSCH aggregation level n2 for FDD and TDD with 30KHz. Therefore, if the slot aggregation requirement with n8 for TDD with 15 KHz is introduced to guarantee the BS with supporting TDD 15 KHz for URLLC feature can be tested, we prefer to add the note as: The requirement with slot aggregation level n8 for TDD with 15 KHz SCS can be applied with FDD or TDD 30KHz SCS with slot aggregation level n2. Issue 8-1-4: Whether to clarify the safety statement in specification Issue 8-2-1: Test applicability rule for FR1 and FR2 We prefer option1. Although both FR1 and FR2 are the targeting scenario for URLLC, as indicated in the specification 38.824, most of URLLC use cases (i.e. Rel-15 enabled use case, factory automation, transport industry and electrical power distribution), are evaluated under FR1, such as 700MHz and 4GHz. In that sense, FR1 should be the practical deployment for URLLC in current stage. Therefore, we prefer to perform the URLLC tests for requirements of high reliability with high BLER either FR1 or FR2 based on BS declaration to reduce the test effort if BS declares to support both FR1 and FR2 Issue 8-2-2: Test applicability rule for FR2 for different SCS As mentioned in Issue 8-2-1, We prefer the tests shall be done for either FR1 or FR2, if BS declare to support both FR1 and FR2 For each FR, only 1SCS need to be tested Issue 8-2-3: Antenna configuration We are fine with option1 Issue 8-2-4: SCS/BW for FR2 We prefer option 1 In eMBB, RAN4 has defined the requirement with different SCS and CBW Meanwhile the related test applicability for SCS and BW combination are defined. To reduce the test effort and fulfil the BS requirement with any supported BW, we prefer to only define requirement with 60 KHz SCS for 50MHz, and 120 KHz for 50MHz. Issue 8-2-5: TDD pattern We are fine option 2 , f slot aggregation level requirement with FR2 is needed. Issue 8-2-6: Aggregation factor for TDD We are fine with option 2, if slot aggregation level requirement with FR2 is needed. Issue 8-2-7: Mapping type We are fine with option 2 similar as eMBB in FR2 Issue 8-2-8: MCS Either option 1 or option 2 is fine for us, no strong view. Issue 8-2-9: Channel model We are fine with option 1 Issue 8-2-10: Start symbol We are fine with option 1 Issue 8-2-11: Symbol length We are fine with option 1 Issue 8-2-12: DM-RS We are fine with option 2,,no need to replicate the test cases from eMBB Issue 8-2-13: DM-RS Type We are fine with option 1 Issue 8-2-14: DM-RS duration We are fine with option 1 Issue 8-2-15: Bandwidth allocation We are fine with option 1 Issue 8-2-16: Maximum number of HARQ re-transmissions We are fine with option 1 Issue 8-2-17: Test metric We are fine with option 1 |
| NTT DOCOMO | Issue 8-1-1: PUSCH aggregation factor for TDD 15 kHz SCS with pattern DDDSU We still prefer option 3 to ensure the functionality of PUSCH repetition in TDD 15kHz SCS. In our understanding, BSs can support TDD patterns other than DDDSU, and such BSs need to be tested with supported TDD patterns which may not be DDDSU. The concern with Option 2 is that the test does not apply to BSs that support 15kHz SCS and TDD, even if the BS supports TDD patterns other than DDDSU. Issue 8-1-3: Applicability rule for FDD and TDD (if n8 for TDD 15 kHz is introduced): We would like to understand the intention of option 1 correctly. For further clarification, the following 4 cases can be considered: Case 1) TDD (DDDSU) 15kHz SCS with PUSCH aggregation level n8 Case 2) FDD 15kHz SCS with PUSCH aggregation level n2 Case 3) TDD (7D1S2U) 30kHz SCS with PUSCH aggregation level n2 Case 4) FDD 30kHz SCS with PUSCH aggregation level n2 In our understanding, the requirement for PUSCH aggregation level n8 for TDD with 15kHz SCS can be applied with Case 1 and 2, and the requirement for PUSCH aggregation level n2 for TDD with 30kHz SCS can be applied with Case 3 and 4. If option 1 means above, we agree with option 1. Note: Regarding the applicability for SCS, it was already agreed to reuse the test applicability rules from Rel-15. Agreement in RAN4#94bis (R4-2005528) Test applicability for different SCS and channel bandwidth: - Reuse the test applicability rules defined for NR Rel-15 PUSCH performance requirements Issue 8-2-1: Test applicability rule for FR1 and FR2 There is no such kind of rule according to Rel-15 applicability rule, and we don’t see technical reasons to skip either one. We prefer to test both FR1 and FR2 if BS supports both. Issue 8-2-3: Antenna configuration We agree with option 1. Issue 8-2-5: TDD pattern RAN4 has used DDDSU for other BS demodulation requirements as the test parameter of TDD pattern for FR2. In addition, other TDD patterns can be tested based on the NOTE in the test parameter table (i.e., NOTE1: The same requirements are applicable to FDD and TDD with different UL-DL pattern.). There seems to be no concern on reusing TDD pattern from Rel-15. Therefore, we prefer option 1 (DDDSU). Issue 8-2-6: Aggregation factor for TDD As mentioned in Issue 8-2-5, we prefer to assume DDDSU for FR2. Based on this TDD pattern, it is reasonable to configure n8 which can combine two effective slots to verify the functionality of PUSCH repetition. Thus, we prefer Option 1 (n8 for DDDSU). Issue 8-2-7: Mapping type In Rel-15, only mapping type B has been defined for PUSCH performance requirements. With the same approach as Rel-15, we could prioritize to define the requirements for PUSCH mapping type B. Issue 8-2-9: Channel model We agree with option 1. Issue 8-2-10/11: Start symbol, Symbol length We agree with option 1 since 10 symbol length is the same assumption as Rel-15. Issue 8-2-12: DM-RS In Rel-15, both 1+0 and 1+1 have been defined for FR2 PUSCH requirements. We don’t need to remove 1+0 from URLLC requirements taking into account of the Rel-15 test coverage, Issue 8-2-13/14/15/16/17: We agree with option 1. |
| Huawei | 8-2-1: We support option 2. Both should be tested. 8-2-2: option 1. Only 1 SCS need to be tested. 8-2-3/5: option 1. 8-2-6: We propose to follow the agreements from FR1 if DDDSU is configured. 8-2-7: We propose mapping type B to be configured for FR2. 8-2-8/9/10: Option 1. 8-2-12/13/14/15/16/17: Align with FR1, option 1. |
| Intel | Issue 8-1-1: PUSCH aggregation factor for TDD 15 kHz SCS with pattern DDDSU: Prefer Option 3, because scenarios with non-contiguous transmission of PUSCH transmission (15 kHz TDD) is rather important from functionality verification and test coverage point of view. Issue 8-1-2: RV sequence with 4 HARQ transmission: Option 1 is fine for us Issue 8-1-3: Applicability rule for FDD and TDD (if n8 for TDD 15 kHz is introduced): Support Option 1 Issue 8-2-1: Test applicability rule for FR1 and FR2 Support Option 2 Issue 8-2-2: Test applicability rule for FR2 for different SCS Ok with recommended Wf Issue 8-2-3: Antenna configuration Ok with recommended WF Issue 8-2-4: SCS/BW for FR2 One CBW per SCS is also fine for as (i.e. Option 1) Issue 8-2-5: TDD pattern and Issue 8-2-6: Aggregation factor for TDD DSUU pattern with n2 was proposed to avoid extensive discussion which we have for FR1 15 kHz TDD. Same time, taking into account that this option is not supported by many companies, we prefer only option 1 for both issues. Issue 8-2-7: Mapping type Option 2 is also fine for us. Issue 8-2-11: Symbol length Support Option 1 to align with Rel-15 assumptions Issue 8-2-12: DM-RS Support Option 2. One DMRS configuration is enough for testing. Issue 8-2-13/14/15/16/17 Fine with Option 1. |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2010837 Ericsson Draft CR to 38.141-1 on test methodology and FRCs for URLLC and test requirements for 0.001. [only comment FRCs in this summary] | Huawei: - In table A.2A-1, the payload size and code block size including CRC of G-FR1-A2A-4 may incorrectly. The number should be 2976 and 2960. Please double check the results. - Regarding to the FRC numbering in Annex A, a new issue is added in issues 11-2-4. TS38.141-1 should follow the agreements of TS38.104. |
|  | Company B |
| R4-2010988 Huawei, HiSilicon CR to TS38.104 Performance requirements for URLLC PUSCH repetition Type A | Company A |
|  | Company B |
| R4-2010989 Huawei, HiSilicon CR to TS38.141-1 Performance requirements for URLLC BS demodulation requirements with higher BLER | Company A |
|  | Company B |
| R4-2010990 Huawei, HiSilicon CR to TS38.141-1 Test applicability for URLLC BS demodulation requirements |  |
| R4-2010991 Huawei, HiSilicon CR to TS38.141-2 FRC for URLLC BS performance requirements | Company A |
|  | Company B |
|  | Ericsson: We should align the Annex section numbers and FRC numbers between the different CRs. |
| R4-2011396 NTT DOCOMO, INC Draft CR for TS 38.141-2: Introduction of performance requirements of PUSCH repetition type A and PUSCH mapping type B for URLLC | Company A |
|  | Company B |
|  | Ericsson: Table 8.2.8.4.2-1 states that the DM-RS setting is pos1, but there should only be 1 DM-RS symbol NTT DOCOMO: Thank you for your comment. Both pos0 and pos1 can be allowed for 2 symbol PUSCH as additional DM-RS configurations. Is it better to capture both? |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Sub-topic 8-1: BS demodulation requirements of high reliability for FR1 - PUSCH aggregation factor for TDD 15 kHz SCS with pattern DDDSU: - Configure n2 for FDD and n8 for TDD with note. - Note: The intention of this configuration is to have two effective transmissions of the transport block. To achieve this for the standard TDD pattern captured in this table, a value of n8 is necessary, while for FDD a value of n2 is necessary. Sub-topic 8-2: BS demodulation requirements of high reliability for FR2 - Test applicability rule for FR2 for different SCS: Only 1 SCS need to be tested - Antenna configuration: 1x2, ULA low - SCS/BW for FR2: 60 kHz/50MHz, 120 kHz/ 50MHz - Mapping type: Type B - Start symbol: 0 - DM-RS Type: Type 1 - DM-RS duration: Single-symbol DM-RS - Bandwidth allocation: Full bandwidth - Maximum number of HARQ re-transmissions: 4 - Test metric: 1% BLER (Calculated after all re-transmissions) Candidate options: Recommendations for 2nd round: Sub-topic 8-1: BS demodulation requirements of high reliability for FR1 - Applicability rule for FDD and TDD (if n8 for TDD 15 kHz is introduced) - Option 1: The requirement with PUSCH aggregation level n8 for TDD with 15 KHz SCS can be applied with FDD or TDD 30 KHz SCS with PUSCH aggregation level n2. - RV sequence with 4 HARQ transmission - Option 1: 0 3 0 3 under full buffer scenario. - Whether to clarify the safety statement in specification - Option 1: No need to specify any safety statements in specification - Option 2: Yes - Option 1a: Since the URLLC features of 5G NR will potentially be used in safety critical applications, the ultimately chosen statistical testing methodology for testing of these features must be verified by an independent body of experts/statisticians, before requirements and test can be used as basis for safety critical implementations. All statistical analysis and discussions provided in this meeting are to be taken as a best effort and is not to be taken as due diligence - Option 1b: If the URLLC features of 5G NR would be used in safety or mission critical applications, the ultimately chosen statistical testing methodology for testing of these features must be verified by an independent body of experts/statisticians. It is also important to bear in mind that the demodulation requirements do not take account of all aspects of system operation (for example RF, transmitter, internal interfaces, higher layer protocol software etc.). Sub-topic 8-2: BS demodulation requirements of high reliability for FR2 - Test applicability rule for FR1 and FR2 - Option 1: If BS declare to support both FR1 and FR2, the tests shall be done for either FR1 or FR2 - Option 2: If BS declare to support both FR1 and FR2, the tests shall be done both - TDD pattern - Option 1: DDDSU, S=10:2:2 - Option 2: DSUU, S=12:2 - Aggregation factor for TDD - Option 1: n8 for DDDSU - Option 2: n2 for DSUU - MCS - Option 1: MCS5 from table 3 - Option 2: MCS2 from table 3 - Channel model - Option 1: TDLA30-300 Low - Option 2: TDLA30-75 - Symbol length - Option 1: 10 - Option 2: 14 - DM-RS - Option 1: 1+0 and 1+1. - Option 2: 1+1 CRs: In 2nd round, the section number and name can be decided. Companies are encouraged to comment on these CRs. CRs can be revised in 2nd round. Simulation results alignments: Only three companies uploaded their simulation results. More inputs from other companies are encouraged. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| Tdoc # | Way forward for NR URLLC BS performance requirements | Huawei, HiSilicon |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2010837 | to be revised |
| R4-2010988 | to be revised |
| R4-2010989 | to be revised |
| R4-2010990 | to be revised |
| R4-2010991 | to be revised |
| R4-2011396 | to be revised |

Discussion on 2nd round

Sub-topic 8-5-1: BS demodulation requirements of high reliability for FR1

Simulation results alignments:

Only three companies uploaded their simulation results. More inputs from other companies are encouraged.

Issue 8-5-1-1: RV sequence with 4 HARQ transmission:

Nokia and Ericsson have some concern of this issue, can Samsung confirm the concerns?

[Nokia]It was our understanding that this RV sequence is the resulting/effective sequence. Since BS demod testing does not have a real trigger of PUSCH (via DCI with RV for example), we assumed that the theoretical trigger was chosen such that the ultimately observed RV sequence is 0,2,3,1. It is understood that this trigger sequence should conform to 0,3,0,3 to achieve this goal. A common decision on what the captured RV sequences means (effective vs. triggered) should be made and a corresponding note added to the configuration table for slot repetition (type A).

Nokia support the “effective” RV sequence interpretation.

[Ericsson] clarify that the 0 3 0 3 refers to the RV at the beginning of a HARQ retransmission of a series of aggregated slots. If this is the case, we are OK with the option 1.

- Proposal:

  - Option 1: 0 3 0 3 under full buffer scenario. (Samsung, Intel, Huawei, Ericsson)

  - Option 2: Effective RV sequence with 4 HARQ transmission: {0 2 3 1} (Nokia)

- Recommended WF

  - TBD

Issue 8-5-1-2: Applicability rule for PUSCH aggregation testing in FDD and TDD:

Could Samsung explain more about option 1? Does option 1 mean 15 kHz TDD with n8 requirement can be tested with a 30 kHz SCS?

- PUSCH aggregation factor for 15 kHz SCS:

  - Option 1: The requirement with PUSCH aggregation level n8 for TDD with 15 KHz SCS can be applied with FDD or TDD 30 KHz SCS with PUSCH aggregation level n2. (Samsung, Nokia, Intel)

  - Option 2: The requirements for PUSCH with aggregation for 15kHz can be tested either by configuring n8 and the DDDSU TDD pattern or by configuring FDD with aggregation level n2. (Ericsson)

- Recommended WF

  - TBD

Issue 8-5-1-3: Whether to clarify the safety statement in specification

This issue has been discussed for several meetings, more opinions are encouraged from companies apart from Huawei, Nokia and Ericsson.

- Proposals

  - Option 1: No need to specify any safety statements in specification (Huawei)

  - Option 2: Yes (Nokia)

    - Option 1a: Since the URLLC features of 5G NR will potentially be used in safety critical applications, the ultimately chosen statistical testing methodology for testing of these features must be verified by an independent body of experts/statisticians, before requirements and test can be used as basis for safety critical implementations. All statistical analysis and discussions provided in this meeting are to be taken as a best effort and is not to be taken as due diligence

    - Option 1b: (Ericsson)

If the URLLC features of 5G NR would be used in safety or mission critical applications, the ultimately chosen statistical testing methodology for testing of these features must be verified by an independent body of experts/statisticians. It is also important to bear in mind that the demodulation requirements do not take account of all aspects of system operation (for example RF, transmitter, internal interfaces, higher layer protocol software etc.).

- Recommended WF

  - TBD

Issue 8-5-1-4: BLER calculation method

This issue is raised from the UE side. For PUSCH high reliability test case, the test metrics is agreed as 1% BLER (calculated after all transmission). The purpose of this issue is to ensure we have the same understanding of the BLER calculation. For example, the maximum number of HARQ is 4，if an information block is transmitted and succeed with HARQ index of 3, the BLER is

Method 1: BLER =0, because the TB is successfully transmitted through the transmissions are failed for three times.

Method 2: BLER=3/4. Counting the BLER residue for each HARQ transmission. Here, BLER= 3/4, as the TB is failed for three times and it is transmitted for 4 times.

- Proposals

  - Option 1: Method 1. (Huawei, Intel, Samsung, Ericsson, Nokia)

  - Option 2: Method 2

  - Option 3: Other method which is not precluded.

- Recommended WF

  - Option 1

Sub-topic 8-5-2: BS demodulation requirements of high reliability for FR2

Issue 8-5-2-1: Test applicability rule for FR1 and FR2

- Proposals

  - Option 1: If BS declare to support both FR1 and FR2, the tests shall be done for either FR1 or FR2

  - Option 2: If BS declare to support both FR1 and FR2, the tests shall be done both (DoCoMo, Ericsson, Huawei, Intel, Samsung)

- Recommended WF

  - Option 2. Both of FR1 and FR2 should be tested if BS declare to support both. No technical reasons to skip either one.

Issue 8-5-2-2: TDD pattern

- Proposals

  - Option 1: DDDSU, S=10:2:2 (Intel, Ericsson, Nokia, DoCoMo, Huawei)

  - Option 2: DSUU, S=12:2 (Samsung)

- Recommended WF

  - TBD

Issue 8-5-2-3: Aggregation factor for TDD

- Proposals

  - Option 1: n8 for DDDSU (Intel, , DoCoMo)

  - Option 2: n2 for DSUU (Samsung)

  - Option 3: n8 for DDDSU with note (Ericsson)

    - Note: The testing can be performed with a different TDD pattern

- Recommended WF

  - Following the decision from FR1 if TDD pattern “DDDSU” is configured. (Nokia, DoCoMo)

Issue 8-5-2-4: MCS

- Proposals

  - Option 1: MCS5 from table 3 (Intel, Nokia, Samsung, Huawei, Ericsson, Samsung)

  - Option 2: MCS2 from table 3 (Ericsson, Samsung)

- Recommended WF

  - Option 1

Issue 8-5-2-5: Channel model

- Proposals

  - Option 1: TDLA30-300 Low (Intel, Ericsson, Samsung, DoCoMo, Huawei)

  - Option 2: TDLA30-75 (Nokia)

- Recommended WF

  - TBD

Issue 8-5-2-6: Symbol length

- Proposals

  - Option 1: 10 (Ericsson, Samsung, DoCoMo, Intel, Huawei, Nokia)

  - Option 2: 14 (Nokia)

- Recommended WF

  - Option 1. Mapping type B was agreed in 1st round, we can configured the symbol length align with eMBB test.

Issue 8-5-2-7: DM-RS

- Proposals

  - Option 1: 1+0 and 1+1. (DoCoMo)

  - Option 2: 1+1 (Ericsson, Nokia, Huawei, Samsung, Intel)

- Recommended WF

  - Option 2? One DM-RS test is enough for performance test.

Companies views’ collection for 2nd round

Open issues

| Company | Comments |
| --- | --- |
| Huawei | Issue 8-5-2-4/5/6: We are ok with option 1. Issue 8-5-2-7: We are fine with the WF. |
| Intel | Issue 8-5-1-4: BLER calculation method We assume Option 1. |
| Samsung | Issue 8-5-1: Option1 means for the RV trigger of PUSCH transmission (via DCI). For eMBB, since there is no PUSCH aggregation, with 4 HARQ transmission, the RV selection should be {0, 2, 3, 1}. With PUSCH aggregation (n2 for example) with FDD, if the value of RV for 1st transmission via DCI is 0, then the RV for these transmission is {0, 2}. With 4 HARQ transmission, the following RV should be {0, 2, 2, 3, 3, 1, 1, 0} if the RV indicated by DCI for 4 HARQ transmission is {0, 2, 3, 1}. Base on the assumption of option1, all the possible RV based on HARQ IR combination at least with 2 HARQ transmission. Issue 8-5-2: Based on the RV allocation for each transmission, rvid indicated by the DCI scheduling the PUSCH; rvid to be applied to nth transmission occasion. n mod 4 = 0; n mod 4 = 1; n mod 4 = 2; n mod 4 = 3. 0; 0; 2; 3; 1. 2; 2; 3; 1; 0. 3; 3; 1; 0; 2. 1; 1; 0; 2; 3. if the initial RV for 1st transmission is 0, then it can be calculated that the RV for effective UL transmission, for PUSCH aggregation level n8 with TDD (DDDSU) 15 KHz as {0 x x x x 1 x x}, x is not valid due to DL or the special subframe, so the RV for effective transmission with 1st transmission is {0,2}, where RV value is calculated with mod (n, 4) operation for nth transmission As for FDD with PUSCH aggregation level n2 or TDD (7D1S2U) 30KHz, the RV value for these two transmission is also {0, 2}. Therefore, from the demodulation requirements perspective, the performance with PUSCH aggregation level n8 is not expected to show difference with PUSCH aggregation level n2 for FDD, considering the number of effective UL transmission is 2. So, the requirement (targeting SNR) of TDD with n8 can be applied for FDD. There is no need to define new requirement with TDD 15KHz with n8. Issue 8-5-4: we are ok with option 1, similar metric for eMBB scenario Issue 8-5-2-1: We can compromise with option1 Issue 8-5-2-2: TDD pattern The TDD pattern depends on the whether PUSCH with aggregation factor should be considered with TDD in FR2. Option 1(DDDSU) is same with eMBB in FR2. Issue 8-5-2-3: Aggregation factor for TDD If we go TDD pattern with DSUU, then PUSCH with 2 consecutive slots is valid. While with n8 for 3D1S1U, the effective transmission is only 2 transmission. Issue 8-5-2-4/5/6: we are ok with option 1 Issue 8-5-2-7: we are ok with option 2 Update 20200826 We have replied the concerns for issue 8-5-1-1: RV sequence with 4 HARQ transmission: As mentioned, {0, 3, 0, 3} is RV for each re-transmission if configured 4 HARQ. While for each transmission within slot aggregation level n2, the RV can be calculated with as {0 2 3 1 0 2 3 1}. My understanding similar as with NB-IOT with repetition, the RV is selected cycling within repetition. If we go option 2 as {0, 2, 3, 1}, firstly, it should indicate the RV for each HARQ transmission? With slot aggregation level, the RV is {0 2} for 1st HARQ, and {2, 3} for 2nd HARQ, {3, 1} for 3rd HARQ, and {1, 0} for 4th HARQ. .At least 3 HARQ transmission can guarantee ultimately observed RV sequence. |
| Ericsson | 8-5-1-1: We’re OK with option 1, understanding that this is the RV at the start of each HARQ transmission. 8-5-1-2: We propose to reword the option as follows: - Applicability rule for PUSCH aggregation testing in FDD or TDD: The requirements for PUSCH with aggregation for 15kHz can be tested either by configuring n8 and the DDDSU TDD pattern or by configuring FDD with aggregation level n2. Our understanding is that there only needs to be one requirement, and then it can be tested either way. Issue 8-5-1-4: Option 1/Method 1. 8-5-2-1: We propose to use the same TDD pattern and n8, but add a note that testing can be performed with a different TDD pattern. Issue 8-5-2-4: We are also OK for MCS5. Issue 8-5-2-5: The TDLA30-75 channel is used for 64QAM in other requirements, and TDLA30-300 for other modulation orders. Why would be change here ? Issue 8-5-2-6: 10 symbols is used for all other PUSCH requirements, although one might ask why not use all of the symbols if repetition is needed. |
| Nokia, Nokia Shanghai Bell | Issue 8-5-1-1: RV sequence with 4 HARQ transmission: For FDD with n2, we agree with the initial proposal of “RV sequence indicated by the DCI scheduling the PUSCH = {0, 3, 0, 3}” to result in an “effective RV sequence = {0, 2, 3, 1}”. We fail to follow the {0, 2} example given by Samsung above. Given the current misalignment in understanding, we propose to either leave the RV sequence (both the name and the value) in [] and to discuss again after doing further studies until the next meeting, or to rename the RV sequence parameter to “Effective RV sequence” and leave it at {0, 2, 3, 1}. Issue 8-5-1-2: Applicability rule for FDD and TDD (if n8 for TDD 15 kHz is introduced): We fail to follow the n8 with RV {0, 2} example given by Samsung above. Given the current misalignment in understanding, we propose to either leave the RV sequence (both the name and the value) in [] and to discuss again after doing further studies until the next meeting, or to rename the RV sequence parameter to “Effective RV sequence” and leave it at {0, 2, 3, 1}. Concerning the options offered in 8-5-1-2, we don’t see support of any other option than option 1. Hence, we can agree to it. Issue 8-5-1-3: Whether to clarify the safety statement in specification We would like to see this issue being brought to the online session, where each company can be asked their support for the options and a majority decision can be taken. Issue 8-5-1-4: BLER calculation method Option 1 is inline with the “BLER calculated after all transmission” agreement. We note that this BLER_after cannot be used to find the TPUT simply as 1-BLER_after, this definition only holds for BLER per transmission attempt as in option 2: TPUT = 1 - BLER_per_tx. Issue 8-5-2-1: Test applicability rule for FR1 and FR2 Nokia agreed with option 1 in the first round, but we are ready to follow majority opinion here. Issue 8-5-2-2: TDD pattern In the interest of reducing test implementation effort, it is preferable to not include a new TDD pattern. I.e., option 1 Issue 8-5-2-3: Aggregation factor for TDD Agree with recommended WF. Issue 8-5-2-4: MCS MCS 5 to align with FR1. Issue 8-5-2-5: Channel model We would ask all companies to reconsider the speed a 300Hz (UL) Doppler corresponds to at 28GHz. In our calculation 300 Hz corresponds to 6kph (jogging speed). High reliability in FR2 is not a use case with “active” mobility, the devices should be stationary. Issue 8-5-2-6: Symbol length Option 1 is fine. Issue 8-5-2-7: DM-RS Considering the reliability goal, and the possibility to have 1+1 DM-RS in 10/14 symbols, we prefer to go with option 2 to save testing cost. |
| NTT DOCOMO | Issue 8-5-2-1: Test applicability rule for FR1 and FR2 We agree with the recommended WF (i.e., option 2). Issue 8-5-2-2: TDD pattern We support option 1 to reuse the TDD pattern from Rel-15. In the practical testing, the other TDD pattern can be allowed. Issue 8-5-2-3: Aggregation factor for TDD We agree with the recommended WF. Issue 8-5-2-4: MCS We slightly prefer Option 1 to align with FR1. Issue 8-5-2-5: Channel model We prefer Option 1. We think it is not necessary to change the channel model from Rel-15, and it is noted that TDL30-300 Low has been used for QPSK performance requirements. Issue 8-5-2-6: Symbol length We agree with the recommended WF, Issue 8-5-2-7: DM-RS In Rel-15, both 1+0 and 1+1 were introduced for FR2 performance requirements. Why we need to narrow down the configurations on DMRS from Rel-16? BS vendor can declare 1+0 or/and 1+1, which means that it can be allowed and expected to support only 1+0 in some cases. We believe that there is no need to narrow down DMRS configurations from Rel-15. Please clarify the reason why RAN4 needs to narrow down the DMRS configuration. |

WF/Simulation assumptions comments collection

The BS WF should be commented in this section:

| WF number | Comments |
| --- | --- |
| R4-2012649 Huawei Way forward on NR URLLC BS performance requirements |  |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2012654 (from R4-2010837) Ericsson Draft CR to 38.141-1 on test methodology and FRCs for URLLC and test requirements for 0.001. [only comment FRCs in this summary] | Huawei: Table A.3B-1 NOTE1: the number of DM-RS CDM groups without data is 1. 1 or 2 should be aligned. We think the number does not affect the FRC calculation and the common used number is 2, should we align with 2? |
|  | Company B |
| R4-2012655 (from R4-2010988). Huawei, HiSilicon CR to TS38.104 Performance requirements for URLLC PUSCH repetition Type A | Company A |
|  | Company B |
|  | Ericsson: - The CR does not introduce OTA requirements - Regarding the note “Note 1: Calculate the target BLER after all HARQ transmission(s) for one TB”, to improve clarity we propose “Note 1: BLER is defined as residual BLER; i.e. ratio of incorrectly received transport blocks / sent transport blocks, independently of the number HARQ transmission(s) for each transport block” |
| R4-2012656 (from R4-2010989) Huawei, HiSilicon CR to TS38.141-1 Performance requirements for URLLC BS demodulation requirements with higher BLER | Company A |
|  | Company B |
|  | Ericsson: Same comment on note as for 2655 |
| R4-2012661 (from R4-2010990) Huawei, HiSilicon CR to TS38.141-1 Test applicability for URLLC BS demodulation requirements |  |
| R4-2012657 (from R4-2010991) Huawei, HiSilicon CR to TS38.141-2 FRC for URLLC BS performance requirements | Company A |
|  | Company B |
|  | Ericsson: There is a minor difference to the Ericsson and Nokia CRs on FRCs for the 2 symbol FRC; whether to write “no additional DM-RS” or “DM-RS position = pos0”. It should be aligned either way. |
| R4-2012658 (from R4-2011396) NTT DOCOMO, INC Draft CR for TS 38.141-2: Introduction of performance requirements of PUSCH repetition type A and PUSCH mapping type B for URLLC | Company A |
|  | Company B |
|  | Ericsson: The CR does not introduce OTA requirements Huawei: - As Ericsson comments for R4-2012655: Regarding the note “Note 1: Calculate the target BLER after all HARQ transmission(s) for one TB”, to improve clarity we propose “Note 1: BLER is defined as residual BLER; i.e. ratio of incorrectly received transport blocks / sent transport blocks, independently of the number HARQ transmission(s) for each transport block” - Table 8.2.8.4.2-1 title: “with low number of symbols” change to with non-slot transmission |

Summary for 2nd round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#8-5-1/8-5-2 | Agreements: - BS demodulation requirements of high reliability for FR1 - PUSCH aggregation factor for TDD 15 kHz SCS with pattern DDDSU: Configure n2 for FDD and n8 for TDD with note. - Note: The intention of this configuration is to have two effective transmissions of the transport block. To achieve this for the standard TDD pattern captured in this table, a value of n8 is necessary, while for FDD a value of n2 is necessary. - RV sequence with 4 HARQ transmission：{0,3,0,3} with note - Note: The effective RV sequence is {0,2,3,1} with slot aggregation - BS demodulation requirements of high reliability for FR2 - Test applicability rule for FR2 for different SCS: Only 1 SCS need to be tested - Test applicability rule for FR1 and FR2: If BS declare to support both FR1 and FR2, the tests shall be done both. - Antenna configuration: 1x2, ULA low - SCS/BW for FR2: 60 kHz/50MHz, 120 kHz/ 50MHz - Mapping type: Type B - MCS: MCS5 from table 3 - Start symbol: 0 - Symbol length: 10 - DM-RS Type: Type 1 - DM-RS duration: Single-symbol DM-RS - Bandwidth allocation: Full bandwidth - Maximum number of HARQ re-transmissions: 4 - Test metric: 1% BLER (Calculated after all re-transmissions) Open issues for next meeting: BS demodulation requirements of high reliability for FR1 - Applicability rule for FDD and TDD - Option 1: The requirement with PUSCH aggregation level n8 for TDD with 15 KHz SCS can be applied with FDD or TDD 30 KHz SCS with PUSCH aggregation level n2. - Option 2: The requirements for PUSCH with aggregation for 15 kHz can be tested either by configuring n8 and the DDDSU TDD pattern or by configuring FDD with aggregation level n2. - Whether to clarify the safety statement - Option 1: No need to specify any safety statements in specification - Option 2: Yes - Option 1a: Since the URLLC features of 5G NR will potentially be used in safety critical applications, the ultimately chosen statistical testing methodology for testing of these features must be verified by an independent body of experts/statisticians, before requirements and test can be used as basis for safety critical implementations. All statistical analysis and discussions provided in this meeting are to be taken as a best effort and is not to be taken as due diligence - Option 1b: If the URLLC features of 5G NR would be used in safety or mission critical applications, the ultimately chosen statistical testing methodology for testing of these features must be verified by an independent body of experts/statisticians. It is also important to bear in mind that the demodulation requirements do not take account of all aspects of system operation (for example RF, transmitter, internal interfaces, higher layer protocol software etc.). BS demodulation requirements of high reliability for FR2 - TDD pattern - Option 1: DDDSU, S=10:2:2 - Option 2: DSUU, S=12:2 - Aggregation factor for TDD - Option 1: n8 for DDDSU - Option 2: n2 for DSUU - Option 3: n8 for DDDSU with note - Note: The testing can be performed with a different TDD pattern - Channel model - Option 1: TDLA30-300 Low - Option 2: TDLA30-75 - DM-RS - Option 1: 1+0 and 1+1. - Option 2: 1+1 |

WF and CR status

| WF number | Status summary |
| --- | --- |
| R4-2012649 WF for NR BS URLLC performance requirements | Agreeable |

| CR number | Status summary |
| --- | --- |
| R4-2012654 (Revised from R4-2010837) | To be endorsed |
| R4-2012655 (Revised from R4-2010988). | To be endorsed |
| R4-2012656 (Revised from R4-2010989) | To be endorsed |
| R4-2012661 (Revised from R4-2010990) | To be endorsed |
| R4-2012657 (Revised from R4-2010991) | To be endorsed |
| R4-2012658 (Revised from R4-2011396) | To be noted |

| Summary of simulation results | Status summary |
| --- | --- |
| R4-20112279 Simulation results for BS | To be noted |

Topic #9: BS demodulation requirements for low latency

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2009701 | NTT DOCOMO, INC. | Proposal 5: Regarding FR2 SCS/CBW for low-latency tests, adopt option 2 or 2a: Option 2: 50/100MHz for 60 kHz SCS, 50/100MHz for 120kHz SCS Option 2a: 50MHz for 60 kHz SCS, 50/100MHz for 120kHz SCS Proposal 6: Introduce both no additional DM-RS (pos0) and one additional DM-RS (pos1) for low-latency requirements. Proposal 7: Low-latency tests shall be done for both FR1 and FR2 if BS declares support both, and all supported SCS will be tested for FR1 and only 1 SCS need to be tested for FR2 (Option 3). |
| R4-2009727 | Intel Corporation | Proposal 3: Use the following assumptions for FR2 PUSCH mapping Type B requirements: CBS/SCS: 60 KHz for 50/100 MHz and 120 KHz for 50/100 MHz with test applicability rule TDD UL/Dl pattern: DDDSU with S=10D:2G:2U PUSCH configuration: Mapping Type B, Start symbol 0, Duration 2 or 4. MCS 10 from Table 3 Channel model: TDLA30-300 |
| R4-2009857 | Nokia, Nokia Shanghai Bell | Proposal 3: RAN4 to introduce low latency requirements for both FR1 and FR2, along with applicability rule that only one SCS will be tested for each frequency band with test applicability rule. |
| R4-2011340 | Nokia, Nokia Shanghai Bell | draftCR for 38.104: High reliability and low latency BS demod requirements |
| R4-2010282 | Samsung | Observation 3: 4 or 7 OS is assumption for baseline performance evaluation for most of URLLC use cases. Observation 4: Mini-slot repetition with 4OS is the typical scenario in RAN1 discussion to supporting dynamic switch between mini-slot repetition and multi-segments Observation 5: Minor performance difference existed for DMRS configuration with 1 and 1+1 for eMBB in FR2 Proposal 8: Non-slot scheduling with 4 symbols can be considered for the lower latency requirement for FR2 Proposal 9: URLLC tests for low latency requirement shall be done for either FR1 or FR2 based on BS declaration if BS declares to support both FR1 and FR2. Proposal 10: Only define FR2 URLLC requirement with 60 KHz SCS for 50MHz, and 120 KHz SCS for 50MHz |
| R4-2010838 | Ericsson | Proposal 3: Adopt the proposed FR2 parameters according to sections 3 and 4 |
| R4-2010839 | Ericsson | Simulation results for URLLC “high BLER” requirements |
| R4-2010837 | Ericsson | Draft CR to 38.141-1 on test methodology and FRCs for URLLC and test requirements for 0.001% BLER |
| R4-2010983 | Huawei, HiSilicon | Proposal 4: Tests shall be done for both, and only 1 SCS will be tested for each frequency band with test applicability rule. |
| R4-2010984 | Huawei, HiSilicon | Simulation results on BS demodulation reuqirements with higher BLER |
| R4-2010991 | Huawei, HiSilicon | CR to TS38.141-2 FRC for URLLC BS performance requirements |
| R4-2011396 | NTT DOCOMO, INC. | Draft CR for TS 38.141-2: Introduction of performance requirements of PUSCH repetition type A and PUSCH mapping type B for URLLC |

Open issues summary

In this section, the parameters for defining the requriements of URLLC PUSCH mapping Type B wil be discussed.

From the approved WF R4-2002429 in RAN4 #94 e-meeting, following were agreed:

Agreements:

- Test metrics: 70% throughput

- SCS/CBW: 15 KHz for 5/10 MHz, 30 KHz for 10/40 MHz with test applicability rule.

- Whether to define requirements for FR2：Yes with test applicability rule.

  - 3a: The performance requirements are only applicable for BS only supporting FR2 or supporting both FR1 and FR2.

- Symbol length: 2os with higher [MCS10] from table 3.

Open issues:

- Test applicability rule for FR1 and FR2 (only if FR2 is agreed)

  - Option 1: Tests shall be done for either FR1 or FR2

  - Option 2: Tests shall be done for both , and only 1 SCS will be tested for each frequency band with test applicability rule.

  - Option 3: Tests shall be done for both, all supported SCS will be tested for FR1 and only 1 SCS need to be tested for FR2.

- SCS/CBW for FR2 (only if FR2 is agreed)

  - 60 KHz:

    - Option 1: 50 MHz

    - Option 2: 50 MHz and 100 MHz

  - 120 KHz

    - Option 1: 100 MHz

    - Option 2: 50 MHz and 100 MHz

Sub-topic 9-1: BS demodulation requirements of low latency for FR2

In this section, the open issues relate to the FR2 will be discussed.

Open issues and candidate options before e-meeting:

Issue 9-1-1: Test applicability rule for FR1 and FR2 if both are supported by BS

- Proposals

  - Option 1: Tests shall be done for either FR1 or FR2 (Samsung)

  - Option 2: Tests shall be done for both, and only 1 SCS will be tested for each frequency band with test applicability rule. (Nokia, Huawei, Ericsson, DoCoMo, Intel)

  - Option 3: Tests shall be done for both, all supported SCS will be tested for FR1 and only 1 SCS need to be tested for FR2. (DoCoMo)

- Recommended WF

  - TBD

Issue 9-1-2: SCS/CBW for FR2

- Proposals

  - 60 KHz:

    - Option 1: 50 MHz (DoCoMo, Samsung, Huawei, Ericsson, Nokia, Intel)

    - Option 3: 50 MHz and 100 MHz with test applicability rule (Intel, DoCoMo)

  - 120 KHz

    - Option 1: 50 MHz (Samsung, Ericsson, Huawei, Nokia, Intel)

    - Option 3: 50 MHz and 100 MHz with test applicability rule (Intel, DoCoMo)

- Recommended WF

  - Option 1 for both?

Issue 9-1-3: TDD pattern

- Proposals

  - Option 1: DDDSU, S=10:2:2 (Intel, Ericsson, Nokia, Huawei, Samsung, DoCoMo)

  - Option 2:

- Recommended WF

  - Option 1

Issue 9-1-4: Aggregation factor for TDD

- Proposals

  - Option 1: n1 (Ericsson, Nokia, Huawei, Samsung, DoCoMo, Intel)

  - Option 2:

- Recommended WF

  - Option 1

Issue 9-1-5: MCS

- Proposals

  - Option 1: MCS10 from table 3 (Intel, Nokia, Huawei, DoCoMo)

  - Option 2: MCS 5 or MCS 2 from table 3 (Samsung))

- Recommended WF

  - TBD

Issue 9-1-6: Channel model

- Proposals

  - Option 1: TDLA30-300 (Intel, Ericsson, Nokia, Huawei, Samsung, DoCoMo, Intel)

  - Option 2:

- Recommended WF

  - Option 1

Issue 9-1-7: Antenna configuration

- Proposals

  - Option 1: 1x2, ULA low (Ericsson, Nokia, Huawei, Samsung, DoCoMo, Intel)

  - Option 2:

- Recommended WF

  - Option 1

Issue 9-1-8: Bandwidth allocation

- Proposals

  - Option 1: Full bandwidth (Ericsson, Nokia, Huawei, Samsung, DoCoMo, Intel)

  - Option 2:

- Recommended WF

  - Option 1

Issue 9-1-9: Maximum number of HARQ re-transmissions

- Proposals

  - Option 1: 1 (Ericsson, Huawei, Samsung, DoCoMo, Intel)

  - Option 2:

- Recommended WF

  - Option 1

Issue 9-1-10: DM-RS Type

- Proposals

  - Option 1: Type 1 (Ericsson, Nokia, Huawei, Samsung, DoCoMo, Intel)

  - Option 2:

- Recommended WF

  - Option 1

Issue 9-1-11: DM-RS duration

- Proposals

  - Option 1: Single-symbol DM-RS (Ericsson, Nokia, Huawei, Samsung, DoCoMo, Intel)

  - Option 2:

- Recommended WF

  - Option 1

Issue 9-1-12: DM-RS

- Proposals

  - Option 1: 1+0 and 1+1. (DoCoMo)

  - Option 2: 1+0 (Ericsson, Nokia, Huawei, Samsung, Intel)

  - Option 3: 1+1 if symbol length larger than 4 (Ericsson)

- Recommended WF

  - TBD

Issue 9-1-13: Start symbol

- Proposals

  - Option 1: 0 (Intel, Ericsson, Nokia, Huawei, Samsung)

  - Option 2:

- Recommended WF

  - Option 1

Issue 9-1-14: Symbol length

- Proposals

  - Option 1: 2 (Intel, Ericsson, Nokia, Huawei, DoCoMo)

  - Option 2: 4 (Intel, Samsung, DoCoMo)

- Recommended WF

  - TBD

Issue 9-1-15: Test metric

- Proposals

  - Option 1: 70% TP (Ericsson, Nokia, Huawei, Samsung, DoCoMo, Intel)

  - Option 2:

- Recommended WF

  - Option 1

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| XXX | Issue |
| Nokia, Nokia Shanghai Bell | Issue 9-1-1: Test applicability rule for FR1 and FR2 if both are supported by BS Issue 9-1-2: SCS/CBW for FR2 50MHz only for either seems like a good compromise. Issue 9-1-3: TDD pattern: See no reason to change from the standard DDDSU. Issue 9-1-4: Aggregation factor for TDD: n1 as for FR1. Issue 9-1-5: MCS: Follow the FR1 decision (MCS10 table 3). Issue 9-1-6: Channel model: Follow the FR1 decision. Issue 9-1-7: Antenna configuration: 1x2, since OTA does not support otherwise. Issue 9-1-8: Bandwidth allocation: Follow the FR1 decision. Issue 9-1-9: Maximum number of HARQ re-transmissions: Issue 9-1-10: DM-RS Type: Follow the FR1 decision. Issue 9-1-11: DM-RS duration: Follow the FR1 decision. Issue 9-1-12: DM-RS: Follow the FR1 decision. Issue 9-1-13: Start symbol: Follow the FR1 decision. Issue 9-1-14: Symbol length: 2, i.e., same as in FR1. Issue 9-1-15: Test metric: Follow the FR1 decision. |
| Ericsson | Option 2 in our view. A BS supporting both seems unusual, but in case it does then there could be some digital processing in the RF or even RF aspects that impact the demod that differ between the FR for this case. It is sufficient to test one SCS per FR in our view. Issue 9-1-12: Supporting 1+1 DM-RS would depend on agreeing on the number of symbols being larger than 4. Issue 9-1-14: If relatively small fixed payloads are to be transmitted in a system with analogue beamforming such that the whole bandwidth is used. Then a single symbol may be sufficient to accommodate the payload. Also if we test 2 symbols then we have requirements at the two extremes. |
| Samsung | Issue 9-1-1: Test applicability rule for FR1 and FR2 if both are supported by BS We prefer option1 Although both FR1 and FR2 are the targeting scenario for URLLC, as indicated in the specification 38.824, most of URLLC use cases (i.e. Rel-15 enabled use case, factory automation, transport industry and electrical power distribution), are evaluated under FR1, such as 700MHz and 4GHz. In that sense, FR1 should be the practical deployment for URLLC in current stage. Therefore, we prefer to perform the URLLC tests for requirements of high reliability with high BLER either FR1 or FR2 based on BS declaration to reduce the test effort if BS declares to support both FR1 and FR2 Issue 9-1-2: SCS/CBW for FR2 In eMBB, RAN4 has defined the requirement with different SCS and CBW Meanwhile the related test applicability for SCS and BW combination are defined. To reduce the test effort and fulfil the BS requirement with any supported BW, we prefer to only define requirement with 60 KHz SCS for 50MHz, and 120 KHz for 50MHz. Issue 9-1-3: TDD pattern We are fine with option1, if no aggregation is considered. Issue 9-1-4: Aggregation factor for TDD We are fine with option1, Issue 9-1-5: MCS We prefer to use MCS 5 or MCS2 from table 3 In Rel-15, RAN1 has designed some features, such as slot-based repetition, low efficient MCS table, which aims to guarantee the high reliability transmission. To some extent, with configured low MCS index can be benefit considering achievable coding gain, also it can reduce the possibility for re-transmission, which will further reduce the transmission latency. Meanwhile, for low latency requirement, there is no HARQ transmission in FR1, where the gain coming from HARQ operation is disappear. To guarantee the reliable transmission, it is meaningful to configure low MCS index for coding gain. Therefore, we prefer to use the low MCS index MCS5, We propose Option2: MCS5 or MCS2 from table 3 Issue 9-1-6: Channel model We are fine with option1, Issue 9-1-7: Antenna configuration We are fine with option1 Issue 9-1-8: Bandwidth allocation We are fine with option1 Issue 9-1-9: Maximum number of HARQ re-transmissions We are fine with option1 Issue 9-1-10: DM-RS Type We are fine with option1 Issue 9-1-11: DM-RS duration We are fine with option1 Issue 9-1-12: DM-RS We are fine with option1 The issue is related with symbol length. Based our preferred, only 1 DMRS configured, Issue 9-1-13: Start symbol We are fine with option1 Issue 9-1-14: Symbol length We prefer option 2 with 4 symbols In terms of requirement, we think RAN4 should focus on the typical scenario with possible network scheduling. Firstly, as indicated in the specification 38.824, most of URLLC use cases (i.e. Rel-15 enabled use case, factory automation, transport industry and electrical power distribution), 4 or 7 OS is assumption for baseline performance evaluation Secondly, form the RAN1 physical design, in current Rel-16 URLLC feature with mini-slot repetition, different options based on 4 OS mini-slot repetition to investigate the scheme with supporting dynamic switch between mini-slot repetition and multi-segments Thirdly, in the last meeting, 2OS with MCS 10 was agreed to specify low latency requirement for FR1 Meanwhile, the PT-RS can be configured in FR2. It is not a typical scenario that network will configure less number of OFDM symbol for URLLC transmission. Under this scheduling, the number of available REs for data transmission is reduced, which will result in increasing coding rate, for given targeting information bits From test coverage aspect, 2 OS is already covered in FR1, we prefer to define with different symbol length for coverage, either 4OS or 7OS can be fulfilled the test coverage. As for 7OS symbol length, in current Rel-15 BS demodulation requirement for eMBB, RAN4 has already defined with 10 symbols requirement with type B in FR2. In terms for performance, we do not think there is too much different between 7OS and 10OS. Meanwhile, both 1 DMRS and 1+1 DMRS are configured for requirement for mapping type B in FR2. As indicated, the requirement between 1 DMRS and 2 DMRS configuration is minor. Issue 9-1-15: Test metric We are fine with option1 |
| NTT DOCOMO | Issue 9-1-1: Test applicability rule for FR1 and FR2 if both are supported by BS Originally, we proposed option 3 to follow the Rel-15 applicability rule. However, if it is necessary to reduce the number of test, we can compromise on option 2 since we think at least one SCS need to be tested since the performance impact due to high frequency on FR2 compared to FR1. The SCS to be tested can be chosen and declared. Issue 9-1-2: SCS/CBW for FR2 In our understanding, the same applicability rule on CBW can be applied for URLLC. If this understanding is correct, option 2 and option 3 are the same meaning. Issue 9-1-3/4/5/6/7/8/9/10/11: We agree with Option1. Issue 9-1-14: Symbol length Both option 1 and 2 can be acceptable for us. Issue 9-1-15: Test metric We agree with Option 1. |
| Huawei | Issue 9-1-1: We prefer option 2, FR1 and FR2 should be tested if BS support both. It does not make sense to only test one frequency band. The performance cannot be guaranteed for the other frequency band. Issue 9-1-2: Option 1 for both 60 kHz and 120 kHz Issue 9-1-3: Reuse the exiting TDD pattern. Issue 9-1-4: Follow the agreement of FR1, no slot aggregation for low latency. Issue 9-1-5: We prefer option 1: reuse the agreements of FR1. Issue 9-1-6/7/8/9/10/11/13/15: Option 1. Use agreements of FR1. Issue 9-1-12/14: We prefer symbol length of 2 for low latency, and 1+0 for DM-RS configuration. |
| Intel | Issue 9-1-1: Test applicability rule for FR1 and FR2 if both are supported by BS Support Option 2 Issue 9-1-2: SCS/CBW for FR2 Option 1 for both SCS is also fine for us Issue 9-1-4: Aggregation factor for TDD Support Option 1 Issue 9-1-6/7/8/9/10/11 Ok with Option 1 Issue 9-1-12: DM-RS Support Option 2 (at current stage only options with 2 and 4 PUSCH duration are under discussion) Issue 9-1-15: Test metric Support Option 1 |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2011340 Nokia, Nokia Shanghai Bell DraftCR for 38.104: High reliability and low latency BS demod requirements | Huawei: - Regarding to the FRC numbering in Annex A, A.2A and A.3A were used in this CR. We think A.3Aand A.3Bare more reasonable. But in the latest version of 38.104, 16QAM with R=434/1024 has been numbered as A.7. This issue has been listed in issue 11-2-4, we can follow the agreement later. - In table 8.2.X.1-1, we recommend to separately introduce requirements of BLER=0.001% and BLER=0.01% in spec as listed in sub-topic 11-2 in this documents. - In table 8.2.X.1-1 and table 8.2.Y.1-1. The parameter of “PUSCH aggregation factor” is missing. - In table A.2A-1, the payload size and code block size including CRC of G-FR1-A2A-4 may incorrectly. The number should be 2976 and 2960. Please double check the results. |
|  | Company B |
|  | Ericsson: Section A.3.A, the DM-RS position indicates ops1, but there is only 1 DM-RS symbol Regarding section numbering, we prefer to separate 0.001% (ultra-low) BLER and higher reliability, since the BS may support one but not both of these. We should discuss whether separate tables is sufficient or separate headings would be better. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: BS demodulation requirements of low latency for FR2 - SCS/CBW for FR2: 60kHz/50 MHz, 120 kHz/ 50 MHz - TDD pattern: DDDSU, S=10:2:2 - Aggregation factor for TDD: n1 - Channel model：TDLA30-300 - Antenna configuration：1x2, ULA low - Bandwidth allocation：Full bandwidth - Maximum number of HARQ re-transmissions: 1 - DM-RS Type: Type 1 - DM-RS duration: Single-symbol DM-RS - Start symbol: 0 - Test metric: 70% TP Candidate options: Recommendations for 2nd round: BS demodulation requirements of low latency for FR2 - Test applicability rule for FR1 and FR2 if both are supported by BS - Option 1: Tests shall be done for either FR1 or FR2 - Option 2: Tests shall be done for both, and only 1 SCS will be tested for each frequency band with test applicability rule. - Option 3: Tests shall be done for both, all supported SCS will be tested for FR1 and only 1 SCS need to be tested for FR2. - MCS - Option 1: MCS10 from table 3 - Option 2: MCS 5 or MCS 2 from table 3 - DM-RS - Option 1: 1+0 and 1+1. - Option 2: 1+0 - Option 3: 1+1 if symbol length larger than 4 - Symbol length - Option 1: 2 - Option 2: 4 CRs: In 2nd round, the section number and name can be decided. Companies are encouraged to comment on these CRs. CRs can be revised in 2nd round. Simulation results alignments: Only four companies uploading their simulation results. More inputs from other companies are encouraged. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2011340 | to be revised |

Discussion on 2nd round

Sub-topic 9-5-1: BS demodulation requirements of low latency for FR1

Simulation results alignments:

Only four companies uploading their simulation results. More inputs from other companies are encouraged.

Sub-topic 9-5-2: BS demodulation requirements of low latency for FR2

Issue 9-5-2-1: Test applicability rule for FR1 and FR2 if both are supported by BS

- Proposals

  - Option 1: Tests shall be done for either FR1 or FR2 (Samsung)

  - Option 2: Tests shall be done for both, and only 1 SCS will be tested for each frequency band with test applicability rule. (Nokia, Huawei, Ericsson, DoCoMo, Intel)

- Recommended WF

  - Option 2. Both of FR1 and FR2 should be tested if BS declare to support both. No technical reasons to skip either one.

Issue 9-5-2-2: MCS

- Proposals

  - Option 1: MCS10 from table 3 (Intel, Nokia, Huawei, DoCoMo)

  - Option 2: MCS 5 or MCS 2 from table 3 (Samsung))

- Recommended WF

  - TBD

Issue 9-5-2-3: Symbol length

- Proposals

  - Option 1: 2 (Intel, Ericsson, Nokia, Huawei, DoCoMo)

  - Option 2: 4 (Intel, Samsung, DoCoMo)

  - Option 3: 7 (Samsung)

- Recommended WF

  - TBD

Issue 9-5-2-4: DM-RS

- Proposals

  - Option 1: 1+0 and 1+1. (DoCoMo)

  - Option 2: 1+0 (Ericsson, Nokia, Huawei, Samsung, Intel)

  - Option 3: 1+1 if symbol length larger than 4 (Ericsson, Samsung)

- Recommended WF

  - Depends on the symbol length.

Companies views’ collection for 2nd round

Open issues

| Company | Comments |
| --- | --- |
| XXX | Issue: |
| Samsung | Issue 9-5-2-2: MCS We are not ok with option 1 and recommend WF. MCS 2 is feasible based our prefer length of OFDM symbols with 4OS, MCS 5 is feasible based on our prefer length of OFDM symbols with 7OS We don't think high MCS is not meaningful for URLLC, which is targeting reliability transmission with low latency. Without achievable coding gain, the BLER will be high in low SNR region, which re-transmission is needed, low latency cannot be guaranteed. Also, with FR2, with the potential overhead of RS, such as PT-RS, which results in the effect code rate is further increasing. Issue 9-5-2-3: Symbol length We are not ok with option 1 and recommend WF As mentioned,2 OS is not typical scenario for URLLC, in RAN1 evaluation for Rel-15 URLLC use cases, most of test cases are 4OS or 7OS, Meanwhile, we don't think high MCS is not meaningful for URLLC, which is targeting reliability transmission with low latency. Without achievable coding gain, the BLER will be high in low SNR region, which re-transmission is needed, low latency cannot be guaranteed. Also, with FR2, with the potential overhead of RS, such as PT-RS, which results in the effect code rate is further increasing. Again, 2OS requirement has already defined in FR1, we prefer to differentiate the test case with cover the possible non-slot transmission scheme. Similar with eMBB, full symbol length in FR1, and 10 symbols in FR2. We have provided the details technical reason of option 2 in 1st round Based on the 1st round, I do not see any technical reason for 4OS is not feasible We still prefer option 2, we also consider 7OS is feasible Issue 9-5-2-4: DM-RS We can also support 2 DMRS with 7OS , If companies have concerns with channel estimation performance of 4OS |
| Nokia, Nokia Shanghai Bell | Issue 9-5-2-1: Test applicability rule for FR1 and FR2 if both are supported by BS Recommended WF is acceptable. Issue 9-5-2-2: MCS The MCS value was chosen to fit a small payload in only one symbol (2 OS TDRA with one DM-RS). We don’t see an issue with MCS10, which is far away from a high coding rate (modulation order 2 with 308/1024 CR). Issue 9-5-2-3: Symbol length No reason to change from FR1. Thus option 1. Issue 9-5-2-4: DM-RS Option 2 is the only choice for TDRA <=4OS. |
| NTT DOCOMO | Issue 9-5-2-1: Test applicability rule for FR1 and FR2 if both are supported by BS We agree with the recommended WF (i.e., Option 2). Issue 9-5-2-4: DM-RS For DMRS, if Option 1 or 2 is agreed for Issue 9-5-2-3: symbol length, the channel mapping is the same between 1+0 and 1+1. Both configurations (i.e., 1+0 and 1+1) can be captured in the specification? Or is it better to capture only one configuration (i.e.,1+0)? |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going Wis, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2012659 (from R4-2011340) Nokia, Nokia Shanghai Bell DraftCR for 38.104: High reliability and low latency BS demod requirements | Huawei: The parameter of number of DM-RS CDM groups without data is not aligned in Table 8.2.6.1-1/8.2.8.1-1/A.3A-1 (NOTE 1)/ A.3B-1 (NOTE 1)/, we think the number 1 or 2 does not affect the FRC calculation and the common used number is 2 in spec. should we align the number with 2? |
|  | Company B |

Summary for 2nd round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#9-5-1/9-5-2 | Agreements: PUSCH demodulation requirements for low latency for FR2 - Test applicability rule for FR1 and FR2 if both are supported by BS: Tests shall be done for both, and only 1 SCS will be tested for each frequency band with test applicability rule. - SCS/CBW for FR2: 60kHz/50 MHz, 120 kHz/ 50 MHz - TDD pattern: DDDSU, S=10:2:2 - Aggregation factor for TDD: n1 - Channel model：TDLA30-300 - Antenna configuration：1x2, ULA low - Bandwidth allocation：Full bandwidth - Maximum number of HARQ re-transmissions: 1 - DM-RS Type: Type 1 - DM-RS duration: Single-symbol DM-RS - Start symbol: 0 - Test metric: 70% TP Open issues for next meeting: PUSCH demodulation requirements for low latency for FR2 - MCS - Option 1: MCS10 from table 3 - Option 2: MCS 5 or MCS 2 from table 3 - DM-RS - Option 1: 1+0 and 1+1. - Option 2: 1+0 - Option 3: 1+1 if symbol length larger than 4 - Symbol length - Option 1: 2 - Option 2: 4 - Option 3: 7 |

WF and CR status

| CR number | Status summary |
| --- | --- |
| R4-2012659 (Revised from R4-2011340) | To be endorsed |

Topic #10: URLLC BS Rel-16 features

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2010978 | Huawei, HiSilicon | Proposal 3: PUSCH repetition type B should be further discussed in RAN4. Proposal 4: RAN4 needs to further study inter-UE multiplexing that whether the performance requirements are needed. |
| R4-2010282 | Samsung | Proposal 11: Prioritize the performance requirements specification based on Rel-15 URLLC feature. Proposal 12: FFS on the test methodology for test metric of 99.9999% and low latency requirements (0.5ms~1ms), FFS on the performance requirements based on Rel-16 URLLC functionalities. |

Open issues summary

Sub-topic 10-1: Rel-16 URLLC BS features

Issue 10-1-1: Features need to be discussed

- Proposals

  - PUSCH repetition type B

  - Inter-UE multiplexing

  - Other features not precluded.

- Recommended WF

  - TBD

Issue 10-1-2: Whether to define performance requirements for PUSCH repetition type B

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - TBD

Issue 10-1-3: Whether to define performance requirements for Inter-UE multiplexing

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - TBD

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| XXX | Issue |
| Nokia, Nokia Shanghai Bell | Issue 10-1-1: Features need to be discussed Given the time it took for RAN4 demod to come to an agreement concerning Rel-15 features, we see it infeasible to add further features to this WI at this point. It seems promising to add this task to the RAN plenary discussion concerning the content of Rel-17 further enhancement demod WI (moderator: CTC). Issue 10-1-2: Whether to define performance requirements for PUSCH repetition type B Not in this WI. See 10-1-1. Issue 10-1-3: Whether to define performance requirements for Inter-UE multiplexing Not in this WI. See 10-1-1. |
| Ericsson | In our understanding, PUSCH repetition type B and UL pre-emption could be candidates for performance requirements and should be considered. We do not see an urgency to complete the requirements in Rel-16 in RAN4 though. |
| Samsung | Based on the WID of Rel-16 URLLC, the performance requirements, including UE/BS demodulation requirements, will be specified. Considering there are still some open issues existed based on Rel-15 URLLC feature, we prefer to deprioritize the requirement specified for Rel-16 as least in Q3. |
| Huawei | Rel-16 features are included in URLLC WI. It should be done within this WI. |
| Intel | We suggest to list all Rel-16 features in this meeting and discuss definition of requirements in the next meeting, taking into account that focus of discussion for this meeting is finalization of requirements for Rel-15 features. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: No Candidate options: Recommendations for 2nd round: As Rel-16 features are included in the URLLC WI, companies are encouraged to list the Rel-16 features that they would like to discuss in the next meeting in 2nd round. - Rel-16 features need to be discussed - PUSCH repetition type B - Inter-UE multiplexing - Other features not precluded. |

Discussion on 2nd round

Sub-topic 10-5-1: Rel-16 URLLC BS features

In URLLC WI RP-191584, the agreed objectives include specifying the performance requirements based on Rel-15 and Rel-16 URLLC functionalities. The candidate features for Rel-15 were listed in the WI but did not list for Rel-16. In this section, companies are encouraged to propose the candidate features they would like to discuss for Rel-16 in the next meeting.

Issue 10-5-1-1: Rel-16 features need to be discussed

- Proposals

  - PUSCH repetition type B (Ericsson, Huawei, Samsung)

  - UL pre-emption (Ericsson, Huawei, Samsung)

  - Other features not precluded.

- Recommended WF

  - TBD

Issue 10-5-1-2: Whether to define performance requirements for PUSCH repetition type B

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - TBD

Issue 10-5-1-3: Whether to define performance requirements for Inter-UE multiplexing

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - TBD

Companies views’ collection for 2nd round

Open issues

| Company | Comments |
| --- | --- |
| Huawei | In our contribution R4-2010978, we proposed to discuss PUSCH repetition type B and inter-UE multiplexing. |
| Samsung | We are ok to list the candidate option of PUSCH repetition type B and inter-UE in the next meeting, if there is no more issue related Rel-15 URLLC. |
| Nokia, Nokia Shanghai Bell | Issue 10-5-1-1: Rel-16 features need to be discussed Given the time it took for RAN4 demod to come to an agreement concerning Rel-15 features, we see it infeasible to add further features to this WI at this point. We don’t see the need to discuss any Rel-16 features at this point. [Moderator]: In URLLC WI RP-191584, Rel-15 features and Rel-16 features were agreed to be specified: Phase 1: - Study the test methodology for both BS and UE [RAN4] - Test methodology for the test metric of 99.999% reliability with testing time into consideration - Test methodology for low latency requirements Phase 2: Specify the following performance requirements based on Rel-15 URLLC functionalities [RAN4] - Investigate and specify the RLM test cases - Study and specify the US/BS demodulation performance and UE CQI reporting requirements for high reliability - The following candidate features related to high reliability should be further identified and prioritized - PDSCH repetitions over multiple slots - PUSCH repetitions over multiple slots - 4-bit CQI Table 3 - MCS index table 3 - Other features are not precluded - Study and specify the UE/BS demodulation performance and UE CQI reporting requirements for low latency - The following candidate features related to low latency should be further identified and prioritized - PDSCH processing capability 2 - Self-contained slot and/or non slot for DL - PDSCH and PUSCH mapping type A/B - Pre-emption indication for DL - Other features are not precluded Specify the following performance requirements based on Rel-16 URLLC functionalities [RAN4] - Base station demodulation performance requirements - UE demodulation performance requirements - Base station conformance testing RAN 4 can only follow the agreements from RAN meeting. Your concern is out of RAN4’s scope and it cannot be discussed in RAN 4 meeting, but should be discussed in RAN meeting. |

Summary for 2nd round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#10-5-1 | Agreements: Features need to be discussed - PUSCH repetition type B - Inter-UE multiplexing - Other features not precluded. Open issues for next meeting: Rel-16 URLLC BS features: - Whether to define performance requirements for PUSCH repetition type B - Option 1: Yes - Option 2: No - Whether to define performance requirements for Inter-UE multiplexing - Option 1: Yes - Option 2: No |

WF and CR status

| CR number | Status summary |
| --- | --- |
| R4-2012659 (Revised from R4-2011340) |  |

Topic #11: BS demod CR work split for FR2

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2010282 | Samsung | Proposal 13: Samsung can voluntarily take some CR drafting work for FR2 URLLC requirement |

Open issues summary

Sub-topic 11-1: CR work split for BS (FR2)

Issue 11-1-1: FR2 CR work split:

  - Ericsson volunteers to take some CRs for 38.141-2 FR2

  - Intel volunteers to cover CR for any FR2 requirements

| Specifacation | Requirements title | CR Specifacation | Frequency range | CR work | CR Responsibility |
| --- | --- | --- | --- | --- | --- |
| BS demodulation (38.104/38.141-1/38.141-2) | FRC for all test cases | 38.104 | FR1 | FRC | Nokia |
|  |  |  | FR2 | FFS | Samsung |
|  |  | 38.141-1 | FR1 | FRC | Ericsson |
|  |  | 38.141-2 | FR1 | FRC | Huawei |
|  |  |  | FR2 | FFS | Samsung |
|  | Test methodology | 38.104? | N/A | Test methodology | Nokia |
|  |  | 38.141-1 | N/A | Test methodology | Ericsson |
|  |  | 38.141-2 | N/A | Test methodology | Nokia/Huawei |
|  | Test applicability for all test cases | 38.141-1 | FR1 | Test applicability | Huawei |
|  |  | 38.141-2 | FR1 | Test applicability | Ericsson |
|  |  |  | FR2 | FFS | FFS |
|  | Requirements for PUSCH with ultra-low BLER target (10-5) | 38.104 | FR1 | Requirements | Nokia |
|  |  |  | FR2 | FFS | Samsung |
|  |  | 38.141-1 | FR1 | Requirements/Measurement of Performance requirements Annex C.3 / Measurement system set-up Annex D | Ericsson |
|  |  | 38.141-2 | FR1 | Requirements / Measurement of Performance requirements Annex C.3 | Ericsson |
|  |  |  | FR2 | FFS | FFS |
|  | Requirements for PUSCH with aggregation factor configured | 38.104 | FR1 | Requirements | Huawei |
|  |  |  | FR2 | FFS | Samsung |
|  |  | 38.141-1 | FR1 | Requirements/Measurement of Performance requirements Annex C.3 / Measurement system set-up Annex D | Huawei |
|  |  | 38.141-2 | FR1 | Requirements / Measurement of Performance requirements Annex C.3 | DoCoMo |
|  |  |  | FR2 | FFS | FFS |
|  | Requirements for PUSCH for mapping Type B with low number of symbols | 38.104 | FR1 | Requirements | Nokia |
|  |  |  | FR2 | FFS | Samsung |
|  |  | 38.141-1 | FR1 | Requirements/Measurement of Performance requirements Annex C.3 / Measurement system set-up Annex D | Huawei |
|  |  | 38.141-2 | FR1 | Requirements / Measurement of Performance requirements Annex C.3 | DoCoMo |
|  |  |  | FR2 | FFS | FFS |

- Recommended WF

  - Agree on the current work split by considering no CR is assigned to Samsung in FR1. FFS FR2 CRs for 38.141-2 and split to volunteering companies in 2nd round.

Sub-topic 11-2: Structure for specifications

In #95 e-meeting, we have agreements on the specification layout:

Agreements:

How to capture URLLC features in specifications and structure for test cases capturing:

- Introduce new sections within the existing PUSCH requirements for each separate feature when incorporate URLLC requirements into the specification.

In this sub-topic, the section numbers will be discussed and allocated. According to the agreements, the new features should be added following the existing requirements:

Issue 11-2-1: Section numbers and title for TS38.104

[Moderator]: Companies provide views for issue 11-2-1: TS 38.104, then TS 38.141-1/2 will follow the agreements. The section numbers have been aligned with HST-BS and that we can start from 8.2.6. For TS 38.141-1, section 8.2.5 is reserved for UL timing adjustment.

- Proposals

Option 1 (Intel)

8.2.6 Requirements for PUSCH 0.001% BLER

8.2.7 Requirements for PUSCH repetition Type A

8.2.8 Requirements for PUSCH mapping Type B with low number of symbols

Option 2 (Nokia):

8.2.X	Requirements for PUSCH for higher reliability
8.2.Y	Requirements for PUSCH for lower latency

Option 3:

8.2.6 Requirements for PUSCH for higher reliability

8.2.6.1 Requirements for PUSCH 0.001% BLER

8.2.6.2 Requirements for PUSCH repetition Type A

8.2.7 Requirements for PUSCH for lower latency

8.2.7.1 Requirements for PUSCH mapping Type B with low number of symbols

Option 4: (Intel)

8.2.6 Requirements for PUSCH for high reliability URLLC features

8.2.6.1 Requirements for PUSCH 0.001% BLER

8.2.6.2 Requirements for PUSCH repetition Type A

8.2.7 Requirements for PUSCH for low latency URLLC features

8.2.7.1 Requirements for PUSCH mapping Type B with low number of symbols

- Recommended WF

  - TBD

Issue 11-2-4: FRC numbers in Annex A for TS38.104

- Proposals

  - Option 1: (DoCoMo: change A.7 to A.4A)

    - A.3A Fixed Reference Channels for performance requirements (QPSK, R=99/1024)

    - A.3B Fixed Reference Channels for performance requirements (QPSK, R=308/1024)

  - Option 2 (Nokia, Intel):

    - A.8 Fixed Reference Channels for performance requirements (QPSK, R=99/1024)

    - A.9 Fixed Reference Channels for performance requirements (QPSK, R=308/1024)

- Recommended WF

  - TS38.141-1 and TS38.141-2 will follow the agreements of this issue.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| XXX |  |
| Nokia, Nokia Shanghai Bell | Issue 11-1-1: FR2 CR work split: No change for Nokia. We are fine with this work split. Issue 11-2-1: Section numbers and title for TS38.104 We would prefer to introduce just two new subsections. One for “lower latency” and one for “higher reliability”, which hold all current (and future) features/requirements pertaining to the respective goal. This structure would allow us to accommodate future low latency/high reliability features, without exploding the number of PUSCH subsection. Furthermore we would like to avoid titles such as “ultra-low”, as it would force us to come up with ever more outlandish superlatives in the future. I.e., the proposal from our draftCR is: 8.2.X Requirements for PUSCH for higher reliability 8.2.Y Requirements for PUSCH for lower latency In particular, we would like to capture both 1% and 1e-5 BLER in the same subsection. We have no preference, if this should result in one or two configuration tables. However, the minimum requirement tables should be titled with the exact feature tested (e.g., “2 symbols”, or “PUSCH repetition type A”, or 1% BLER, or 0.001% BLER) Issue 11-2-2: Section numbers and title for TS38.141-1 Same as 11-2-1. Issue 11-2-3: Section numbers and title for TS38.141-2 Same as 11-2-1. Issue 11-2-4: FRC numbers in Annex A for TS38.104 We had previously overlooked the existence of A.7 (434/1024). In order to align with the current spec structure, we would now agree with option 2. |
| Ericsson | Issue 11-1-1: We volunteer to cover one or two of the FR2 CRs to 38.141-2 (depending on the number of other volunteers) for higher BLER. Note that it is not yet agreed to create requirements for FR2 ultra-low BLER. Issue 11-2-1-3: This split of section headings makes sense and is OK for us. We prefer to separate 10^-5 and high reliability requirements as the BS may support one but not the other. If that could be clarified with separate tables then it is also OK. One thing though is whether to write “ultra-low BLER” in the specifications. If we write ultra-low BLER for 10^-5 BLER, then it is difficult to name a section in case there are requirements at even lower BLER in the future. Alternatives could be “0.001% BLER” or “low BLER”. |
| NTT DOCOMO | Issue 11-1-1: FR2 CR work split: We are fine with the current CR work split. Issue 11-2-2: Section numbers and title for TS38.141-1 In our understanding, section 8.2.5 needs to be reserved for UL timing adjustment. Issue 11-2-4: FRC numbers in Annex A for TS38.104 If option 1 is adopted, section number A.7 should be changed to A.4A. |
| Huawei | Issue 11-2-1: We prefer option 1 or 3. Issue 11-2-4: Option 1 is more reasonable. Although A.7 with 16QAM has been archived, we can change it to A.4A. But the change may need to be happened later as some performance requirements are referring the A.7. |
| Intel | Issue 11-1-1: FR2 CR work split: We volunteer to cover CR with definition of any FR2 requirements. Issue 11-2-1: Section numbers and title for TS38.104 At current stage, we prefer Option 1, because terminology “higher reliability” and “lower latency” is rather confusing from specification reading point of view (it is not clear higher and lower in comparison to what?). One of potential modification: 8.2.6 Requirements for PUSCH for high reliability URLLC features 8.2.7 Requirements for PUSCH for low latency URLLC features Issue 11-2-4: FRC numbers in Annex A for TS38.104 Probably Option 2 is better from spec structure point of view. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: CR work split for BS (FR2) - Agree on the current work split. FFS FR2 CRs for 38.141-2 and split to voluntary companies in 2nd round. Candidate options: Recommendations for 2nd round: FFS FR2 CRs for 38.141-2 Structure for specifications: Continue to discuss in 2nd round |

Discussion on 2nd round

Sub-topic 11-5-1: CR work split for BS (FR2)

Issue 11-5-1-1: FR2 CR work split:

| Specifacation | Requirements title | CR Specifacation | Frequency range | CR work | CR Responsibility |
| --- | --- | --- | --- | --- | --- |
| BS demodulation (38.104/38.141-1/38.141-2) | FRC for all test cases | 38.104 | FR1 | FRC | Nokia |
|  |  |  | FR2 | FFS | Samsung |
|  |  | 38.141-1 | FR1 | FRC | Ericsson |
|  |  | 38.141-2 | FR1 | FRC | Huawei |
|  |  |  | FR2 | FRC | Samsung |
|  | Test methodology | 38.104? | N/A | Test methodology | Nokia |
|  |  | 38.141-1 | N/A | Test methodology | Ericsson |
|  |  | 38.141-2 | N/A | Test methodology | Nokia/Huawei |
|  | Test applicability for all test cases | 38.141-1 | FR1 | Test applicability | Huawei |
|  |  | 38.141-2 | FR1 | Test applicability | Ericsson |
|  |  |  | FR2 | Test applicability | Ericsson |
|  | Requirements for PUSCH with ultra-low BLER target (10-5) | 38.104 | FR1 | Requirements | Nokia |
|  |  |  | FR2 | FFS | Samsung |
|  |  | 38.141-1 | FR1 | Requirements/Measurement of Performance requirements Annex C.3 / Measurement system set-up Annex D | Ericsson |
|  |  | 38.141-2 | FR1 | Requirements / Measurement of Performance requirements Annex C.3 | Ericsson |
|  |  |  | FR2 | N/A | N/A |
|  | Requirements for PUSCH with aggregation factor configured | 38.104 | FR1 | Requirements | Huawei |
|  |  |  | FR2 | FFS | Samsung |
|  |  | 38.141-1 | FR1 | Requirements/Measurement of Performance requirements Annex C.3 / Measurement system set-up Annex D | Huawei |
|  |  | 38.141-2 | FR1 | Requirements / Measurement of Performance requirements Annex C.3 | DoCoMo |
|  |  |  | FR2 | Requirements / Measurement of Performance requirements Annex C.3 | Huawei |
|  | Requirements for PUSCH for mapping Type B with low number of symbols | 38.104 | FR1 | Requirements | Nokia |
|  |  |  | FR2 | FFS | Samsung |
|  |  | 38.141-1 | FR1 | Requirements/Measurement of Performance requirements Annex C.3 / Measurement system set-up Annex D | Huawei |
|  |  | 38.141-2 | FR1 | Requirements / Measurement of Performance requirements Annex C.3 | DoCoMo |
|  |  |  | FR2 | Requirements / Measurement of Performance requirements Annex C.3 | Intel |

- Recommended WF

  - TBD

Sub-topic 11-5-2: Structure for specifications

Issue 11-5-2-1: Section numbers and title for TS38.104

Companies provide views for issue 11-5-2-1: TS 38.104, then TS 38.141-1/2 will follow the agreements. The section numbers have been aligned with HST-BS and that we can start from 8.2.6. For TS 38.141-1, section 8.2.5 is reserved for UL timing adjustment.

- Proposals

  - Option 1 (Intel, Huawei, Samsung, Ericsson)

8.2.6 Requirements for PUSCH 0.001% BLER

8.2.7 Requirements for PUSCH repetition Type A

8.2.8 Requirements for PUSCH mapping Type B with non-slot transmission (Samsung, Nokia)

  - Option 2 (Nokia):

8.2.6 Requirements for PUSCH for higher reliability

8.2.7 Requirements for PUSCH for lower latency

  - Option 3:

8.2.6 Requirements for PUSCH for higher reliability

8.2.6.1 Requirements for PUSCH 0.001% BLER

8.2.6.2 Requirements for PUSCH repetition Type A

8.2.7 Requirements for PUSCH for lower latency

8.2.7.1 Requirements for PUSCH mapping Type B with low number of symbols

  - Option 4: (Intel)

8.2.6 Requirements for PUSCH for high reliability URLLC features

8.2.6.1 Requirements for PUSCH 0.001% BLER

8.2.6.2 Requirements for PUSCH repetition Type A

8.2.7 Requirements for PUSCH for low latency URLLC features

8.2.7.1 Requirements for PUSCH mapping Type B with low number of symbols

- Recommended WF

  - Option 1

Issue 11-5-2-2: FRC numbers in Annex A for TS38.104

TS38.141-1 and TS38.141-2 will follow the agreements of this issue.

- Proposals

  - Option 1: (Huawei, DoCoMo:change A.7 to A.4A, Samsung, Ericsson)

    - A.3A Fixed Reference Channels for performance requirements (QPSK, R=99/1024)

    - A.3B Fixed Reference Channels for performance requirements (QPSK, R=308/1024)

  - Option 2 (Nokia, Intel):

    - A.8 Fixed Reference Channels for performance requirements (QPSK, R=99/1024)

    - A.9 Fixed Reference Channels for performance requirements (QPSK, R=308/1024)

- Recommended WF

  - Option 1 is more reasonable. We suppose more FRCs will be added in the future, categorising FRC based on different modulation order make the specifications look more organised. Although A.7 with 16QAM has been archived, we can change it to A.4A. But the change may need to be happened later as some performance requirements are referring the A.7.

  - Option 1? Keep FRCs more organised according to the modulation types and regardless the archived A.7. It is better to change A.7 to A.4A in the future, if it cannot be changed then leave it.

Companies views’ collection for 2nd round

Open issues

| Company | Comments |
| --- | --- |
| XXX | Issue: |
| Huawei | Issue 11-5-2-1: we prefer option 1. It may make confuse to state “higher” and “lower” in spec. and issues should be aligned with UE spec. Issue 11-5-2-2: we agree with the recommend WF. |
| Intel | Issue 11-5-1-1: FR2 CR work split: We are fine with suggested work split Issue 11-5-2-1: Section numbers and title for TS38.104 Prefer Option 1. Option 4 also can be considered as compromised solution between Options 1,2,3. |
| Samsung | Issue 11-5-2-1: generally, we are ok option 1. Since the lower latency is targeting with non-slot transmission , we prefer the section tile to avoid the title as “low number of symbols” 8.2.6 Requirements for PUSCH 0.001% BLER 8.2.7 Requirements for PUSCH repetition Type A 8.2.8 Requirements for PUSCH mapping Type B with non-slot transmission Issue 11-5-2-2: No strong view, option 1 is reasonable in future improve manner |
| Ericsson | Issue 11-5-2-1: Prefer option 1 Issue 11-5-2-2: Prefer option 1 to keep the modulation types in order. OK with the work split |
| Nokia, Nokia Shanghai Bell | Issue 11-5-1-1: FR2 CR work split: No changes for Nokia, so we are fine with it. Issue 11-5-2-1: Section numbers and title for TS38.104 Option 2 and 3 are equal to us and are preferred. Since most companies don’t seem to have any view, this question seems to be of little relevance. So, we can also agree to Samsung’s proposal of 8.2.6 Requirements for PUSCH 0.001% BLER 8.2.7 Requirements for PUSCH repetition Type A 8.2.8 Requirements for PUSCH mapping Type B with non-slot transmission Issue 11-5-2-2: FRC numbers in Annex A for TS38.104 It is no longer practically feasible to change A.7 to A.4A. Hence, we would prefer to follow the path laid out by A.7. However, it is more important to achieve an acceptable agreement this meeting, than to obtain the optimal solution. |

Summary for 2nd round

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#11-5-1/11/5/2 | Agreements: Structure for specifications - Section numbers and title for TS38.104: (TS 38.141-1/2 will follow the agreements.) - 8.2.6 Requirements for PUSCH 0.001% BLER - 8.2.7 Requirements for PUSCH repetition Type A - 8.2.8 Requirements for PUSCH mapping Type B with non-slot transmission - FRC numbers in Annex A for TS38.104: (TS38.141-1 and TS38.141-2 will follow the agreements.) - A.3A Fixed Reference Channels for performance requirements (QPSK, R=99/1024) - A.3B Fixed Reference Channels for performance requirements (QPSK, R=308/1024) CR work split for BS (FR2) Specifacation; Requirements title; CR Specifacation; Frequency range; CR work; CR Responsibility. BS demodulation (38.104/38.141-1/38.141-2); FRC for all test cases; 38.104; FR1; FRC; Nokia. FR2; FFS; Samsung. 38.141-1; FR1; FRC; Ericsson. 38.141-2; FR1; FRC; Huawei. FR2; FRC; Samsung. Test methodology; 38.104?; N/A; Test methodology; Nokia. 38.141-1; N/A; Test methodology; Ericsson. 38.141-2; N/A; Test methodology; Nokia/Huawei. Test applicability for all test cases; 38.141-1; FR1; Test applicability; Huawei. 38.141-2; FR1; Test applicability; Ericsson. FR2; Test applicability; Ericsson. Requirements for PUSCH with ultra-low BLER target (10-5); 38.104; FR1; Requirements; Nokia. FR2; FFS; Samsung. 38.141-1; FR1; Requirements/Measurement of Performance requirements Annex C.3 / Measurement system set-up Annex D; Ericsson. 38.141-2; FR1; Requirements / Measurement of Performance requirements Annex C.3; Ericsson. FR2; N/A; N/A. Requirements for PUSCH with aggregation factor configured; 38.104; FR1; Requirements; Huawei. FR2; FFS; Samsung. 38.141-1; FR1; Requirements/Measurement of Performance requirements Annex C.3 / Measurement system set-up Annex D; Huawei. 38.141-2; FR1; Requirements / Measurement of Performance requirements Annex C.3; DoCoMo. FR2; Requirements / Measurement of Performance requirements Annex C.3; Huawei. Requirements for PUSCH for mapping Type B with low number of symbols; 38.104; FR1; Requirements; Nokia. FR2; FFS; Samsung. 38.141-1; FR1; Requirements/Measurement of Performance requirements Annex C.3 / Measurement system set-up Annex D; Huawei. 38.141-2; FR1; Requirements / Measurement of Performance requirements Annex C.3; DoCoMo. FR2; Requirements / Measurement of Performance requirements Annex C.3; Intel. |