3GPP TSG-RAN WG4 Meeting # 96e 												R4-2012736

Electronic Meeting, Aug. 17 – Aug .28, 2020

Agenda item:			7.9.3

Source:	Moderator (Samsung)

Title:	Email discussion summary for [96e][320] NR_eMIMO_Demod

Document for:	Information

Introduction

As agreed in previous RAN4 meeting, the overall impact on RAN4 Performance requirements on Rel-16 eMIMO WI as summarized in below table:

| Sub-items |  | BS Demod | UE Performance |  |
| --- | --- | --- | --- | --- |
|  |  |  | UE Demod | CSI |
| Item 1: CSI enhancement for Type II codebook |  | No | NO | Yes |
| Item 2: Multi-TRP/Panel transmission | Single DCI based transmission schemes | No | Yes | No |
|  | Multi-DCI based transmission scheme | No | Yes | No |
| Item3:Beam management enhancement | Item3a: L1-SINR measurement | No | No | No |
|  | Item3b: BFR for Scell | No | No | No |
|  | Item3c: DL/UL beam indication with reduced latency and overhead | No | No | No |
| Item4: Full TX power UL transmission |  | No | No | No |
| Item5: Low PAPR RS |  | NO | NO | No |

The scope of this email discussion mainly focuses to identify the test scope of performance requirements include demodulation and CSI, decide the test set-up including detailed test parameters and introduce corresponding test cases into specifications.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: Discuss and identify the potential impact on UE performance requirement based on RAN1 feature, and discuss the details test parameters to facilitate the test case setup for requirements. In order to make the discussion more concentrative, some open issues suggested to be deferred into 2nd round.

  - For detailed test set-up, suggest to focused on FR1 first  in 1st round

- 2nd round: Further discuss the left open issue, pending on the progress on 1st round.

Topic #1: PDSCH demodulation requirements (Multi-Panel/TRP transmission schemes)

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2009613 | Apple | Proposal 1: Use RX FFT timing based on TCI state #0 from TRP#1 for multi-TRP transmission. Proposal 2: PDSCH demodulation requirements with multi-DCI transmission scheme are introduced with non-overlapping PDSCH allocation Proposal #3: Introduce PDSCH demodulation requirements with multi-DCI with 2TX transmission from each TRP. Proposal #4: Do not introduce demodulation requirements in FR2 for multi-TRP |
| R4-2009738 | Intel | Observation 1: Performance benefits of single wide Rx beam reception from multi-Panel Tx for cell-edge UEs as well for cell-center UEs are not clear and require further study Observation 2: To define performance requirements for FR2 single-DCI based multi-TRP TDMShemeA and inter-slot TDM schemes changing of current OTA test methodology is required Proposal 1: Do not define FR2 performance requirements for reception for multi-TRP/Panel in Rel-16 Observation 3: Typical time offset distribution in multi-TRP configuration - Strategy 1 - Less than 12% and 8% UEs have TO less than -0.5us for ISD 500m and 200m, respectively. - Less than 3% UEs have TO higher than 2us for ISD 500m and all UEs in scenario with ISD 200m have TO not higher than 1.3us - Strategy 2 - No negative TO values - Less than 4% UEs have TO higher than 2us for ISD 500m and all UEs in scenario with ISD 200m have TO not higher than 1.3us Observation 4: Link-level analysis show that - With proper TO compensation 2us TO leads to negligible performance degradation for 15 kHz SCS and rather limited loss (<1dB) for 30 kHz SCS - With proper TO compensation which assumes fixed timing shift with respect to FFT window demodulation performance loss due to negative TO can be fully avoided Proposal 2: Use 2us and -0.5us TO values for requirement definition for both 15 kHz and 30 kHz SCS. Proposal 3: Deprioritize multi-DCI based multi-TRP/panel scheme with fully and partially overlapped resource allocations. Proposal 4: Consider only 2Tx antenna configuration per each TRP for requirements definition. Observation 5: Single-DCI repetition schemes are features to increase transmission reliability Proposal 6: Define performance requirements for each single-DCI based multi-TRP repetition Tx scheme. Proposal 7: Use parameters from Table 2 and Annex B as simulation parameters for single-DCI based multi-TRP Tx repetition schemes. Proposal 8: Use 1% BLER as a test metric for single-DCI based multi-TRP repetition Tx schemes performance requirements. Proposal 9: Introduce the following test cases and define applicability rules between them in accordance to tables 3~6: - FO + positive TO: - Single-DCI SDM - Single-DCI FDMSchemeA - Single-DCI FDMSchemeB - Single-DCI TDMSchemeA - Single-DCI inter-slot TDM - FO + negative TO: - Multi-DCI non-overlapped |
| R4-2010068 | CMCC | Simulation Results |
| R4-2010140 | Samsung | Test scope: Proposal 1: Introducing PDSCH demodulation requirements for transmission schemes related to URLLC operation - For detailed scheme, further down-selected with 1 or 2 scheme(s) from {FDM scheme A, FDM scheme B, TDM scheme A, inter-slot TDM scheme} ; - We preferred to at least introduce test case covering TDM scheme Proposal 2: Introduce PDSCH demodulation requirements with Multi-Panel/TRP transmission schemes with below two cases in FR2 - Single Tx/Rx beam with same QCL Type –D for multi-TRP transmission (eMBB scheme) - Multi-TRP transmission with multi-TX beams in TDM manner (URLLC TDM scheme) Evaluation results General observation: It’ s feasible to use the agree test parameters to introduce performance requirements Observation 1: Without power scaling, 3 dB power offset observed among full-overlapping and non-overlapping cases as show in figure A.1-1 Observation 2: Performance with time offset is more sensitive with/without time offset compensation under “TDLA-30” compared to “TDLC-300” fading channel as received path more concentrated as show in figure A.1-2 Observation 3: Performance with frequency offset is more sensitive with/without frequency offset compensation under low Doppler spread case “TDLA-30-10Hz” compared to “TDLC-300-100Hz” as show in figure A.1-3. Observation 4: For frequency offset, with 200Hz for FR1 FDD, 300Hz for FR1 TDD: - There is enough performance gap to discriminate different UE behavior with and w/o time/frequency compensation - The performance loss compared to ideal case (without TO/FO) less than 0.5 dB with proper compensation Observation 5: For positive time offset, 2us for FR1 FDD, 1us for FR1 TDD - Enough performance gap observed to discriminate UE behaviour - <0.5dB performance loss compared to ideal case with proper compensation Observation 6: For positive time offset, 2us for FR1 FDD, 1us for FR1 TDD - Performance gap among different UE behaviour around 1~2 dB - <1 dB performance loss observed compared ideal case with proper compensation Observation 7: For FR2 - Performance gap around 1.6 dB with 600Hz frequency offset - Performance gap around 2.4 dB with 0.25us time offset - Performance gap around 0.2 dB with -0.0625us time offset Test Parameters Proposal 3: Using TP1 is Reference to define timing and frequency offset - Timing offset = time offset among TP2 and TP1 - Frequency offset = frequency offset among TP2 and TP1 Proposal 4: Define performance requirements in receiver agonistic manner - No need to align the receiver assumption, FFT window adjustment strategy up to UE implementation as well as proper performance ensured by requirements Proposal 5: Introduce both negative and positive time offset among two TPs to ensure proper UE performance considering UE mobility Proposal 6: Introduce time offset as ∆t=2^(-μ) ∆t_1, ∆t_1 = [-0.5, 2] μs - The time offset value scaled with SCS - FR1 FDD 15kHz: {2,-0.5} us - FR1 TDD 30kHz: {1,-0.25} us Proposal 7: For FR2, further evaluate below candidate values: - Frequency offset: {1400Hz,2800Hz} which is 0.05~ 0.1 ppm of 28GHz - Positive time offset: {0.25us, 0.375us, 0.5us} which is 1/8 ~ 1/4 of 2su - Negative time offset: {-0.0625us, -0.09375us, -0.125 us} which is 1/8 ~ 1/4 of -0.25us Proposal 8: Using only 2Tx per TRP as antenna configuration to introduce test cases. Proposal 9: Only introduce non-overlapping cases for multi-DCI based on PDSCH requirements. Overall Test cases Test case 1: Single-DCI based on PDSCH requirements with fully overlapping resource (eMBB) - Test 1a: Single-DCI with frequency offset and negative time offset - Test 1b: Single-DCI with positive time offset Test case 2: Multi-DCI based on PDSCH requirements with non- overlapping resource (eMBB) - Test 2a: Multi-DCI with frequency offset and negative time offset - Test 2b: Multi-DCI with positive time offset Test case 3: Single-DCI based on PDSCH requirements (URLLC) - Test 3a: Single-DCI based FDM scheme A with frequency offset and negative time offset - Test 3b: Single-DCI based inter-slot TDM with positive time offset |
| R4-2010481 | Ericsson | Observation 1: No performance degradation for both mDCI-based FDM (SDM with non-overlapped) and sDCI-based SDM due to the frequency offset. Observation 2: Significant performance impact for both mDCI-based FDM (SDM with non-overlapped) and sDCI-based SDM due to the larger negative time offset. Proposal 1: UE receiver for multi-TRP PDSCH demodulation requirements is up to UE implementation. For alignment purpose, RAN4 should assume that UE always sets the FFT timing based on TCI state #0 (TRP1). ​Proposal 2: Set timing offset to the value in the range Δt=2-μΔt1 with Δt1 = [-0.5, 2] μs. Proposal 3: TRS from different TRP should be non-collided. The signals/channels from all the TRPs should use the same Cell ID. Proposal 4: Configure 2T2R and 2T4R per TRP for multi-TRP PDSCH demodulation requirements. Proposal 5: For frequency offset and timing offset: - Set frequency offset (200Hz for FDD and 300kHz for TDD) and positive time offset (2us for FDD SCS=15kHz and 1us for TDD SCS=30kHz) for mDCI-based SDM. - Set frequency offset (200Hz for FDD and 300kHz for TDD) and negative time offset (-0.5us for FDD SCS=15kHz and -0.25us for SCS=30kHz) for sDCI-based SDM. Proposal 6: Not to introduce test case(s) for multi-panel/TRP transmission schemes in FR2. Proposal 7: RAN4 defines PDSCH demodulation requirements for sDCI-based FDM Scheme A. Proposal 8: If UE is capable of two CORESTPoolIndex reception and passes mDCI-based SDM tests, UE can skip sDCI-based FDM Scheme A. Proposal 9: RAN4 uses the 70% of maximum throughput as the test metric of PDSCH demodulation requirements for single-DCI based multi-TRP transmission. |
| R4-2010719 | MTK | Observation 1: Without compensation, the performance is degraded with 200Hz frequency offset. Observation 2: The performance gap is small for timing offset ∆t = -0.5 μs and ∆t = 2 μs |
| R4-2011012 | Huawei, HiSilicon | Proposal 1: Not to define any test case for single-DCI based multi-panel/TRP transmission schemes (URLLC) Proposal 2: Not to define any FR2 test cases for multi-panel/TRP transmission Observation 1: Taking TCI state #0 as the reference TP (TRP1) is not strictly clear especially when switching happened. Proposal 3: FFT timing based on TRP with the highest RSRP on sync signals + fixed timing shift Proposal 4: Setting timing offset by scaled with SCS Proposal 5: Using only 2us for timing offset value Proposal 6: Not to configure TRS/CSI-RS colliding in multi-panel/TRP transmission test cases Proposal 7: Define multi-DCI based PDSCH test cases with only non-overlapping for resource allocation Proposal 8: Only use 2T2R, 2T4R for antenna configuration Proposal 9: Propose option 2 and option 3 for the number of test cases for multi-DCI scheduled PDSCH requirements: - Option 2: - Test 1 Multi- DCI with frequency offset and Non-overlapping scheduling - Test 2 Multi DCI with positive time offset and Non-overlapping scheduling - Option 3: - Test 1 Multi- DCI with frequency offset, positive time offset and Non-overlapping scheduling Proposal 10: Propose option 2 and option 3 for the number of test cases for single-DCI scheduled PDSCH requirements: - Option 2: - Test 1 Single-DCI with frequency offset and overlapping scheduling - Test 2 Single-DCI with positive time offset and overlapping scheduling - Option 3: - Test 1 Single-DCI with frequency offset, positive time offset and overlapping scheduling |
| R4-2011421 | Qualcomm | Proposal 1: Do not define requirements for URLLC multi-TRP schemes Proposal 2: Do not define multi-TRP requirements for FR2 Proposal 3: Assume the UE always fixes its FFT timing based on TCI state #0 (TP1) to define RAN4 performance requirements for multi-TRP schemes Proposal 4: Define a single test case with both timing offset and frequency offset to limit number of test cases for multi-TRP schemes. Proposal 5: Define multi-DCI multi-TRP requirements only for non-overlapping PDSCH scheduling. Proposal 6: Define multi-DCI multi-TRP requirements only for 2Tx at each TRP. |

Open issues summary

Last RAN4 meeting agreements in WF R4-2008813 and R4-2008814 as captured in Annex.

List of open issues:

- Sub-Topic 1-1: Test Scope

  - Issue 1-1-1: Necessity of introducing test case(s)  for single DCI-based multi-panel/TRP transmission schemes (URLLC)

  - Issue 1-1-2: Necessity of introducing test case(s)  for multi-panel/TRP transmission schemes  in FR2

- Sub-Topic 1-2: Generic test set-up

  - Issue 1-2-1: Reference for timing offset/frequency offset set-up

  - Issue 1-2-2: Baseline receiver assumption for FFT window timing

  - Issue 1-2-3: Timing offset among  multi-panel/TRP (FR1 only)

  - Issue 1-2-4: TRS/CSI-RS configuration

  - Issue 1-2-5: Timing offset among  multi-panel/TRP for FR2 (Postpone to 2nd round)

  - Issue 1-2-6: Frequency offset among multi-panel/TRP for FR2 (Postpone to 2nd round)

- Sub-Topic 1-3: Test parameters for Multi-DCI based multi-TRP/Panel transmission schemes (eMBB)

  - Issue 1-3-1: Resource allocation

  - Issue 1-3-2: Antenna configuration per each TRP

  - Issue 1-3-3: Number of Test cases

- Sub-Topic 1-4: Test parameters for Single-DCI based multi-TRP/Panel transmission schemes (eMBB)

  - Issue 1-4-1: Number of Test cases

- Sub-Topic 1-5: Test parameters for Single-DCI based multi-TRP/Panel transmission schemes (URLLC)

  - Issue 1-5-1: Transmission schemes

  - Issue 1-5-2: Test metric

  - Issue 1-5-3: Test applicability (Postpone to 2nd round)

  - Issue 1-5-4: Number of Test cases (Postpone to 2nd round)

  - Issue 1-5-5: PDSCH configuration for single-DCI based multi-TRP repetition schemes

Sub-topic 1-1: Test Scope

Issue 1-1-1: Necessity of introducing test case(s) for single DCI-based multi-panel/TRP transmission schemes (URLLC)

Note: detailed selection of transmission schemes will be discussed under sub-topic 1-5.

- Proposals

  - Option 1: Yes (Intel, Samsung, Ericsson)

  - Option 2: No (Huawei, Qualcomm, MTK, Apple)

- Recommended WF

  - Companies’ view quite diverse  for  whether to introduce PDSCH requirements for URLLC single DCI based transmission schemes  and the detailed selection of transmission schemes; suggest to discuss and identify the difference from UE processing aspect compared to existing URLLC test cases (URLLC WI) and eMBB operation multi-panel/TRP transmission scheme test cases

Issue 1-1-2: Necessity of introducing test case(s) for multi-panel/TRP transmission schemes in FR2

- Proposals

  - Option 1: No (Huawei, Intel, Ericsson, Apple, Qualcomm, MTK)

  - Option 2 (Samsung): Introduce PDSCH demodulation requirements with Multi-Panel/TRP transmission schemes in FR2

- Case 1: Single Tx/Rx beam with same QCL Type-D for multi-TRP transmission (eMBB).

- Case 2: Multi-TRP transmission with multi-Tx beams in TDM manner (URLLC TDM scheme)

- Recommended WF

  - For case 1: further discuss: Whether the scenario with simultaneous transmission from mTRPs with single Tx/Rx beam direction (only one QCI type-D), where two panels implemented in the same site or two TRPs located in different sites is valid scenario for FR2?

  - For case 2: considering current OTA Test limitation for demodulation test cases, no test cases for case 2 in Rel-16; and further discuss in future release with consideration of test ability issue

Sub-topic 1-2: Generic test set-up

Issue 1-2-1: Reference for timing offset/frequency offset set-up

- Proposals

  - Option 1: Using TP1 is Reference to define timing and frequency offset (Samsung, Ericsson, MTK, Apple, Qualcomm, Huawei)

- Timing offset = time offset among TP2 and TP1

- Frequency offset  = frequency offset among TP2 and TP1

- Recommended WF

  - Agree option 1

Issue 1-2-2: Baseline receiver assumption for FFT window timing

In Previous meeting, the assumption for UE receiver implementation is agreed in RAN4#94bis-e meeting

The test case design should be ensure receiver implementation agnostic with assumption of single FFT operation

- Proposals

  - Option 1: Using TP1 is Reference to define timing and frequency offset (Samsung, Ericsson)

  - Option 2: Define performance requirements in receiver agonistic manner (Samsung)

- No need to align the receiver assumption, FFT window adjustment strategy up to UE implementation as well as proper performance ensured by requirements

  - Option 3: Assume the UE always fixes its FFT timing based on TCI state #0 (TP1) to define RAN4 performance requirements for multi-TRP schemes (Qualcomm, Apple)

  - Option 4: UE receiver for multi-TRP PDSCH demodulation requirements is up to UE implementation. For alignment purpose, RAN4 should assume that UE always sets the FFT timing based on TCI state #0 (TRP1) (Ericsson)

- Recommended WF

  - It’s up to UE implementation for FFT windowing timing adjustment strategy; Define performance requirements in receiver agonistic manner. Meanwhile define RAN4 performance requirements based on the assumption of UE fixed FFT timing based on TCI state #0 (TP1 i.e. SSB only transmitted from TP1).

Issue 1-2-3: Timing offset among multi-panel/TRP (FR1 only)

- Proposals

  - Option 1: Set timing offset of first path from two TRPs by scaled with SCS: ∆t=2-μ∆t1, ∆t1 = [-0.5, 2]μs (Samsung, Ericsson, Huawei)

  - Option 2: Not consider negative timing offset, and set timing offset by scaled with SCS ∆t=2-μ∆t1, only TO = 2us for 15 kHz SCS, and TO = 1us for 30 kHz SCS (Huawei)

  - Option 3: Set timing offset TO =2us and TO=-0.5us for both 15KHz and 30KHz SCS, not scaled with SCS (Intel)

- Recommended WF

  - Positive time offset

- For FR1 FDD with 15kHz : 2us

- For FR1 TDD with 30kHz: further down-select among 2us/1us

  - Negative time offset:

- Issue 1: further discuss whether negative time offset is valid in real deployment scenario?  And whether need to introduce test cases covering negative time offset?

- Issue 2: if introduced, what’s the proper values

- For FR1 FDD with 15kHz: -0.5 us if introduced

- For FR1 TDD with 30kHz: further down-select among -0.5/-0.25us if introduced

Issue 1-2-4: TRS/CSI-RS configuration

In Previous meeting, the assumption for generating different scrambling sequence for PDSCH scheduled by multi-DCI is agreed in RAN4#94bis-e meeting

- Configure different scrambling sequences for PDSCH scheduled by Multi-DCI

- Proposals

  - Option 1: Non-collided for TRS/CSI-RS from different TRP (Ericsson, Huawei, Qualcomm)

- Recommended WF

  - Agree to introduce the test cases with non-colliding TRS/CSI-RS in multi-TRP/panel.

Issue 1-2-5:  timing offset among multi-panel/TRP for FR2 (deferred to 2nd round)

- Proposals

  - Option 1: Introduce time offset as  ∆t=2^(-μ) ∆t_1, candidate values for further evaluation

- Positive time offset : ∆t_1 ={0.25us,0.375us,0.5us}, which is 1/8 ~1/4 of 2us

- Negative time offset: ∆t_1 ={-0.0625us, -0.09375us, -0.125 us }, which is 1/8 ~1/4 of -0.25us

- Recommended WF

  - Postpone to 2nd round pending on discussion status on issue 1-1-2:

Issue 1-2-6:  Frequency offset among multi-panel/TRP for FR2 (deferred to 2nd round)

- Proposals

  - Option 1: Candidate values for further evaluation

- Frequency offset: {1400Hz, 2800Hz}, which is 0.05~0.1 ppm of 28GHz

- Recommended WF

  - Postpone to 2nd round pending on discussion status on issue 1-1-2:

Sub-topic 1-3: Test parameters for Multi-DCI based multi-TRP/Panel transmission schemes (eMBB)

Issue 1-3-1: Resource allocation

- Proposals

  - Option 1: Candidate values for further evaluation Option 1: Only non-overlapping cases (Huawei, Qualcomm, MTK, Samsung, Apple)

  - Option 2: Deprioritize multi-DCI based multi-TRP/panel scheme with fully and partially overlapped resource allocation (Intel)

- Recommended WF

  - Agree option 1: Only non-overlapping cases

Issue 1-3-2: Antenna configuration per each TRP

- Proposals

  - Option 1: Only 2T2R, 2T4R (Intel, Huawei, Apple, Ericsson, Samsung)

- Recommended WF

  - Agree option 1: Only 2T2R, 2T4R

Issue 1-3-3: Number of test cases

- Proposals

  - Option 1: 2 test cases per duplex mode (Samsung)

- Test 1a Multi- DCI with frequency offset and negative time offset and non-overlapping scheduling

- Test 1b Multi DCI with positive time offset and non-overlapping scheduling

  - Option 2: 2 test cases per duplex mode (Huawei)

- Test 1a Multi- DCI with frequency offset and Non-overlapping scheduling

- Test 1b Multi- DCI with positive time offset and Non-overlapping scheduling

  - Option 3: 1 test case per duplex mode (Huawei, Ericsson, Qualcomm?)

- Test1  Multi- DCI with frequency offset, positive time offset and Non-overlapping scheduling

  - Option 4: 1 test case per duplex mode (Intel, Qualcomm?, Huawei)

- Test1  Multi- DCI with frequency offset, negative time offset and Non-overlapping scheduling

- Recommended WF

  - Introduce one test case per duplex mode cover both frequency offset and time offset, the values  of time offset (positive or negative) pending on the decision on issue 1-2-2 (time offset);  meanwhile further discussion the necessity  to introduce one additional test case per duplex mode cover only the impact of time offset (negative or positive), separately

  - Companies’ feedback needed for above proposal, also pending on decision on issue: 1-2-2 (time offset)

Sub-topic 1-4: Test parameters for Single-DCI based multi-TRP/Panel transmission schemes (eMBB)

Issue 1-4-1: Number of test cases

- Proposals

  - Option 1: 2 test cases per duplex mode (Samsung)

- Test 1a Single DCI with frequency offset, negative time offset and overlapping scheduling (Ericsson, Qualcomm?)

- Test 1b Single DCI with positive time offset and overlapping scheduling

  - Option 2: 2 test cases per duplex mode (Huawei)

- Test 1a Single DCI with frequency offset and overlapping scheduling

- Test 1b Single DCI with positive offset and overlapping scheduling

  - Option 3: 1 test case per duplex mode (Huawei, Intel, Qualcomm?)

- Test 1 Single DCI with frequency offset, positive time offset and overlapping scheduling

- Recommended WF

  - Introduce one test case per duplex mode cover both frequency offset and time offset, the values  of time offset (positive or negative) pending on the decision on issue 1-2-2 (time offset);  meanwhile further discussion the necessity  to introduce one additional test case per duplex mode cover only the impact of time offset (negative or positive), separately

  - Companies’ feedback needed for above proposal, also pending on decision on issue: 1-2-2 (time offset)

Sub-topic 1-5: Test parameters for Single-DCI based multi-TRP/Panel transmission schemes (URLLC)

Issue 1-5-1: Transmission schemes

- Proposals

  - Option 1: Define performance requirements for all the single-DCI based multi-TRP Tx repetition schemes (FDM Scheme A, FDMSchemeB, TDMSchemeA, Inter-slot TDM)(Intel)

  - Option 2: Single-DCI based FDM Scheme A  (Ericsson)

  - Option 3: Down-selected with 1 or 2 scheme(s) from { FDM Scheme A, FDM SchemeB, TDMSchemeA, Inter-slot TDM } , at least covering TDM scheme (Samsung)

- Recommended WF

  - Related to discussion on issue 1-1-2, suggest to discuss and identify any new behaviour from UE processing aspect for each transmission schemes which not verified by existing URLLC test cases (URLLC WI) and eMBB operation multi-panel/TRP transmission scheme test cases

Issue 1-5-2: Test Metric

- Proposals

  - Option 1: 70% TP (Ericsson)

  - Option 2: 1% BLER (Intel)

- Recommended WF

  - Companies’ feedback needed for above proposal

Issue 1-5-3: Test applicability rule

- Proposals

  - Option 1(Intel)

|  | Only single-DCI SDM | Only single-DCI repetition scheme(s) | Only multi-DCI without over-ing | Both single-DCI SDM and multi-DCI | Both single-DCI SDM and repetition scheme(s) | Both single-DCI repetition scheme(s) and multi-DCI | Both single-DCI repetition scheme(s), SDM and multi-DCI |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Single-DCI SDM: FO + positive TO | Yes |  |  | Yes |  |  | Yes |
| Multi-DCI Non-Overlapped: FO + negative TO |  |  | Yes | Yes |  |  | Yes |
| Single-DCI repetition scheme: FO + positive TO |  | Yes (Note 1) |  |  | Yes (Note 1) | Yes (Note 1) |  |
| Note 1: In case of supporting both single DCI based FDM scheme(s) and TDM scheme(s) UE can be tested only for TDM scheme. |  |  |  |  |  |  |  |

- Only TDM scheme can be tested if UE supporting both single DCI based FDM scheme(s) and TDM scheme(s)

- Only FDMSchemeB can be tested if UE supporting both single DCI based FDMSchemeA and FDMSchemeB

- Only TDMSchemeA can be tested if UE supporting both TDMScheme A and inter-slot TDMrepetition schemes

- Only TDM can be tested if UE supporting both FDM and TDM repetition schemes

  - Option 2: If UE is capable of two CORE CORESTPoolIndex reception and passes mDCI-based SDM tests, UE can skip sDCI-based FDM Scheme A (Ericsson)

- Recommended WF

  - Companies’ feedback needed for above proposal, also pending on decision on issue: 1-5-1 (transmission scheme)

Issue 1-5-4: Number of test cases

- Proposals

  - Option 1(Samsung): 2 test cases per duplex mode

- Test 1a (Samsung, Ericsson): Single-DCI based  FDM scheme A with frequency offset and negative time offset

- Test 1b: Single-DCI based inter-slot TDM with positive time offset

  - Option 2 (Intel): 4 test cases per duplex mode

- Test 1a: Single-DCI based FDMScheme A with frequency offset and positive time offset

- Test 1b: Single-DCI based FDMScheme B with frequency offset and positive time offset

- Test 1c: Single-DCI based TDMScheme A with frequency offset with positive time offset

- Test 1b: Single-DCI based inter-slot TDM with frequency offset with positive time offset

- Recommended WF

  - Companies’ feedback needed for above proposal, also pending on decision on issue: 1-2-2, 1-5-1 (resource allocation) and 1-2-2 (time offset)

Issue 1-5-5: PDSCH configuration for single-DCI based multi-TRP repetition schemes

- Proposals

  - Option 1(Intel):

- Reused the parameters (TDD configuration, SSB and CSI-RS configuration, PDCCH setup, Propagation conditions and TO/FO) agreed for single-DCI based Tx scheme for each repetition scheme

- MCS 13

- Resource configuration

|  | FDMSchemeA | FDMSchemeB | TDMSchemeA | Inter-slot TDM |
| --- | --- | --- | --- | --- |
| Mapping type | Type A | Type A | Type A | Type A |
| Resource allocation type | Type 1 | Type 1 | Type 1 | Type 1 |
| DMRS configuration | Type 1, 1+1 | Type 1, 1+1 | Type 1, No additional symbols | Type 1, 1+1 |
| Antenna port index | {1000} for TCI#1 and {1001} for TCI#2 | {1000} for TCI#1 and {1001} for TCI#2 | {1000} for TCI#1 and {1001} for TCI#2 | {1000} for TCI#1 and {1001} for TCI#2 |
| Start symbol and time duration | PDSCH#1: 2, 12 PDSCH#1: 2, 12 | PDSCH#1: 2, 12 PDSCH#1: 2, 12 | PDSCH#1: 2, 6 PDSCH#1: 8, 12 | PDSCH#1: 2, 12 PDSCH#1: 2, 12 |
| FDD PRB allocation | PDSCH#1: PRB#0 to PRB#25 PDSCH#2: PRB#26 to PRB#51 | PDSCH#1: PRB#0 to PRB#25 PDSCH#2: PRB#26 to PRB#51 | PDSCH#1: PRB#0 to PRB#51 PDSCH#2: PRB#0 to PRB#51 | PDSCH#1: PRB#0 to PRB#51 PDSCH#2: PRB#0 to PRB#51 |
| TDD PRB allocation | PDSCH#1: PRB#0 to PRB#52 PDSCH#2: PRB#53 to PRB#105 | PDSCH#1: PRB#0 to PRB#52 PDSCH#2: PRB#53 to PRB#105 | PDSCH#1: PRB#0 to PRB#105 PDSCH#2: PRB#0 to PRB#105 | PDSCH#1: PRB#0 to PRB#105 PDSCH#2: PRB#0 to PRB#105 |

- Recommended WF

  - Companies’ feedback needed for above proposal, also pending on decision on issue: 1-2-1, 1-5-1 (resource allocation) and 1-2-2 (time offset)

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| CompanyA | Sub-Topic 1-1: Test Scope - Issue 1-1-1: Necessity of introducing test case(s) for single DCI-based multi-panel/TRP transmission schemes (URLLC) - Issue 1-1-2: Necessity of introducing test case(s) for multi-panel/TRP transmission schemes in FR2 Sub-Topic 1-2: Generic test set-up - Issue 1-2-1: Reference for timing offset/frequency offset set-up - Issue 1-2-2: Baseline receiver assumption for FFT window timing - Issue 1-2-3: Timing offset among multi-panel/TRP (FR1 only) - Issue 1-2-4: TRS/CSI-RS configuration - Issue 1-2-5: Timing offset among multi-panel/TRP for FR2 (Postpone to 2nd round) - Issue 1-2-6: Frequency offset among multi-panel/TRP for FR2 (Postpone to 2nd round) Sub-Topic 1-3: Test parameters for Multi-DCI based multi-TRP/Panel transmission schemes (eMBB) - Issue 1-3-1: Resource allocation - Issue 1-3-2: Antenna configuration per each TRP - Issue 1-3-3: Number of Test cases Sub-Topic 1-4: Test parameters for Single-DCI based multi-TRP/Panel transmission schemes (eMBB) - Issue 1-4-1: Number of Test cases Sub-Topic 1-5: Test parameters for Single-DCI based multi-TRP/Panel transmission schemes (URLLC) - Issue 1-5-1: Transmission schemes - Issue 1-5-2: Test metric - Issue 1-5-3: Test applicability (Postpone to 2nd round) - Issue 1-5-4: Number of Test cases (Postpone to 2nd round) - Issue 1-5-5: PDSCH configuration for single-DCI based multi-TRP repetition schemes |
| Intel | Sub-Topic 1-1: Test Scope - Issue 1-1-1: Necessity of introducing test case(s) for single DCI-based multi-panel/TRP transmission schemes (URLLC) To define performance requirements the usual practice is based on two main reasons: - Specific demodulation processing As was discussed on the previous meeting all multi-TRP Tx schemes correspond to different UE features. In this case there is a possibility that particular UE may support, for instance, FDMSchemeA and do not support other Tx schemes like SDM which is more complex. In this case demodulation performance will not be guaranteed if RAN4 define requirements only for multi-DCI based and single-DCI based SDM Tx schemes. The common essential difference between all schemes is a resource allocation approach which is completely different for each scheme. Beside that there are specific receive processing difference for each scheme: “FDMSchemeA”: It is a simplest Tx scheme in terms of receive processing since it operates with only one codeword and comparing to SDM scheme it does not deal with inter-layer interference. In this case it is more likely that simplest URLLC capable device will support this scheme - not SDM, especially considering RAN1 system-level observations that SDM scheme provides less reliability than FDM/TDM schemes and more suitable for eMBB rather than for URLLC use cases [R1-1905064]. “FDMSchemeB”: In order to provide reliable performance UE needs to properly combine Tx occasions with different RVs. Potentially it can impact soft buffer implementation since in normal conditions UE does not apply HARQ combining in slots with first HARQ indices and do not combine different codewords. “TDMSchemeA”: Specific resource allocation approach may impact overall demodulation processing implementation. Some companies mentioned that this scheme is similar to mini-slot repetition in single-TRP Tx scenario. Same time they correspond to different UE capabilities and demodulation flow between scenarios with Single-TRP Tx and multi-TRP Tx is completely different. “Inter-slot TDM”: Specific resource allocation approach may impact overall demodulation processing implementation. Some companies mentioned that this scheme is similar to slot-aggregation in single-TRP Tx scenario. Same time they correspond to different UE capabilities and demodulation flow between scenarios with Single-TRP Tx and multi-TRP Tx is completely different. - Performance gains Based on RAN1 link-level analysis “FDM” schemes outperforms SDM scheme in terms of demodulation performance. About 1~2 dB difference can be observed due to inter-layer interference and less accurate channel estimation in SDM scheme.[ R1-1905026, R1-1905030, R1-1904313] Considering defined UE features separation and provided receive processing different for each scheme as well as observed performance gains for repetition Tx schemes comparing to SDM scheme we support requirements definition for repetition schemes (Option 1) Issue 1-1-2: Necessity of introducing test case(s) for multi-panel/TRP transmission schemes in FR2 Case 1: From system design perspective both options (two panels implemented in the same site or two TRPs located in different sites with single Rx beam reception) are valid. Same time performance benefits of such operation for FR2 are not clear. Scenario with single Tx TRP/panel with single narrow beam reception is more preferable due to higher antenna Rx gain comparing to wide Rx beam reception in FR2. At this stage we do not see any performance benefits that these scenarios may bring. Therefore, we think that these scenarios will not be used in real field. In this case there is no necessity to define corresponding requirements. Case 2: Agree with recommended WF for case 2. Sub-Topic 1-2: Generic test set-up - Issue 1-2-1: Reference for timing offset/frequency offset set-up Agree with recommended WF. - Issue 1-2-2: Baseline receiver assumption for FFT window timing This issue strictly related to SSB configuration during the test procedure. In real field there are several options on how SSB can be transmitted in multi-TRP Tx scenario: Option1: SSB is transmitted only from one TRP; Option 2: SSB is transmitted from both TRPs with same SSB index (SFN manner); Option3: SSB is transmitted from both TRPs with different SSB indices but all of them correspond to same physical cell-Id. Considering QCL relationship between SSB and transmitted TRSs options 1 and 2 potentially may lead to large performance degradation since propagation conditions on TRSs might be completely different comparing to SSB propagation conditions in terms of delay and Doppler characteristic. Same time option 3 allows to accurately estimate propagation conditions from each TRP. However, from test methodology perspective option 3 leads to increased test complexity since in this case it is not clear how to set TO between two TRPs since during the test UE may set its timing based on both TRPs. To simplify the test option1 might be considered especially considering the agreement on Issue 1-2-1 in which TO/FO are added to the second TRP. In result, we agree with recommended WF but would like to slightly change the wording: It’s up to UE implementation for FFT windowing timing adjustment strategy; Define performance requirements in receiver agnostic manner. Meanwhile define RAN4 performance requirements based on the assumption of UE sets FFT timing based on TCI state #0 (TP1 i.e. SSB only transmitted from TP1). Regarding SSB configuration we support option when SSB is transmitted only from one TRP. - Issue 1-2-3: Timing offset among multi-panel/TRP (FR1 only) Positive TO: Based on our link-level simulation results 2us TO leads to rather limited performance degradation (< 1dB) for 30 kHz SCS comparing to scenario without TO. Considering obtained reliable performance we suggest using same 2us TO for both 15 kHz and 30 kHz SCSs which will mean that from performance requirements perspective RAN4 does not restrict the multi-TRP deployments for 30 kHz SCS (ISD) and 15 kHz and 30 kHz might be deployed in same cells. Negative TO: Based on our system level simulation results depending on UE synchronization strategy both positive and negative TO might be present in multi-TRP deployments. Due to large scale channel model parameters, potential blockages negative TO might appear in real field. In this case we think that it is necessary to have requirements not only with positive but also with negative TO. Based on provided link-level simulation results there is no need to scale negative TO with SCS for FR1 since there is no performance difference between scenarios with -0.5us TO and without TO for 30 kHz SCS. In total we propose to define requirements for both positive and negative TO and do not apply scaling regarding SCS. - Issue 1-2-4: TRS/CSI-RS configuration Agree with recommended WF with slightly modified wording which reflect previous discussions: “Agree to introduce the test cases with only non-colliding TRS/CSI-RS in multi-TRP/panel” Sub-Topic 1-3: Test parameters for Multi-DCI based multi-TRP/Panel transmission schemes (eMBB) - Issue 1-3-1: Resource allocation Agree with recommended WF - Issue 1-3-2: Antenna configuration per each TRP Agree with recommended WF - Issue 1-3-3: Number of Test cases To reduce work efforts one test case per Duplex mode is a reasonable assumption. In this case we are fine with first part of the proposed WF. Same time we do not see necessity to continue discussion regarding separate test case introduction to test only TO compensation. Nothing new will be tested comparing to test with both TO and FO. Sub-Topic 1-4: Test parameters for Single-DCI based multi-TRP/Panel transmission schemes (eMBB) - Issue 1-4-1: Number of Test cases Same view as on Issue 1-3-3: One test case per Duplex mode is enough and there is no need to introduce additional test case to cover only TO impact. Beside that we would like to capture that if both positive and negative TO will be agreed to introduce, then different TO should be used in multi-DCI and single-DCI based test cases. |
| Qualcomm | Sub-Topic 1-1: Test Scope - Issue 1-1-1: Necessity of introducing test case(s) for single DCI-based multi-panel/TRP transmission schemes (URLLC) Prefer Option 2 as mentioned in our paper that URLLC schemes can be covered by m-DCI, SDM and URLLC slot aggregation requirements. - Issue 1-1-2: Necessity of introducing test case(s) for multi-panel/TRP transmission schemes in FR2 Having m-TRP with single Tx/Rx beam seems like an artificial setup for the sake of testing it. So, we prefer not to have these requirements for Case 1. Ok with recommended WF for Case 2. Sub-Topic 1-2: Generic test set-up - Issue 1-2-1: Reference for timing offset/frequency offset set-up Ok with recommended WF. - Issue 1-2-2: Baseline receiver assumption for FFT window timing Recommended WF seems ok. Need more time to confirm this. - Issue 1-2-3: Timing offset among multi-panel/TRP (FR1 only) Ok with recommended WF, - Issue 1-2-4: TRS/CSI-RS configuration Ok with recommended WF, Sub-Topic 1-3: Test parameters for Multi-DCI based multi-TRP/Panel transmission schemes (eMBB) - Issue 1-3-1: Resource allocation Ok with recommended WF, - Issue 1-3-2: Antenna configuration per each TRP Ok with recommended WF, - Issue 1-3-3: Number of Test cases Prefer Option 3 or Option 4. Sub-Topic 1-4: Test parameters for Single-DCI based multi-TRP/Panel transmission schemes (eMBB) - Issue 1-4-1: Number of Test cases Prefer Option 3. Sub-Topic 1-5: Test parameters for Single-DCI based multi-TRP/Panel transmission schemes (URLLC) Prefer to discuss this after we have decided on Issue 1-1-1. |
| Apple | Sub-Topic 1-1: Test Scope - Issue 1-1-1: Necessity of introducing test case(s) for single DCI-based multi-panel/TRP transmission schemes (URLLC) Option 2. Based on UE processing for different single DCI URLLC transmission schemes, we don’t see a huge impact from different schemes compared to already agreed testcases. Given the limited time and already a significant number of testcases and issues to resolve for multi TRP requirements, we propose to down prioritize this for now. - Issue 1-1-2: Necessity of introducing test case(s) for multi-panel/TRP transmission schemes in FR2 Option 1: No necessity to introduce requirements in FR2 for multiTRP transmission. We don’t agree with the recommended WF. Sub-Topic 1-2: Generic test set-up - Issue 1-2-1: Reference for timing offset/frequency offset set-up Option 1. - Issue 1-2-2: Baseline receiver assumption for FFT window timing Option 3. Option 4 seems to be same as option 3. - Issue 1-2-3: Timing offset among multi-panel/TRP (FR1 only) - Issue 1-2-4: TRS/CSI-RS configuration We support the recommended WF. - Issue 1-2-5: Timing offset among multi-panel/TRP for FR2 (Postpone to 2nd round) - Issue 1-2-6: Frequency offset among multi-panel/TRP for FR2 (Postpone to 2nd round) Sub-Topic 1-3: Test parameters for Multi-DCI based multi-TRP/Panel transmission schemes (eMBB) - Issue 1-3-1: Resource allocation Option 1. We propose to only introduce requirements with non overlapping resource allocation . We support the recommended WF. - Issue 1-3-2: Antenna configuration per each TRP We support the recommended WF. - Issue 1-3-3: Number of Test cases We support introducing 1 testcases with both time and frequency offset per duplex mode. Additional test is not needed with other time offset alone. Sub-Topic 1-4: Test parameters for Single-DCI based multi-TRP/Panel transmission schemes (eMBB) - Issue 1-4-1: Number of Test cases Option 3. 1 test per duplex mode with frequency offset, timeoffset and overlapping scheduling. Sub-Topic 1-5: Test parameters for Single-DCI based multi-TRP/Panel transmission schemes (URLLC) - Issue 1-5-1: Transmission schemes - Issue 1-5-2: Test metric - Issue 1-5-3: Test applicability (Postpone to 2nd round) - Issue 1-5-4: Number of Test cases (Postpone to 2nd round) - Issue 1-5-5: PDSCH configuration for single-DCI based multi-TRP repetition schemes |
| MediaTek | Sub-Topic 1-1: Test Scope - Issue 1-1-1: Necessity of introducing test case(s) for single DCI-based multi-panel/TRP transmission schemes (URLLC) We support option 2. - Issue 1-1-2: Necessity of introducing test case(s) for multi-panel/TRP transmission schemes in FR2 We support option 1. Sub-Topic 1-2: Generic test set-up - Issue 1-2-1: Reference for timing offset/frequency offset set-up OK with the recommended WF. - Issue 1-2-2: Baseline receiver assumption for FFT window timing OK with the recommended WF. - Issue 1-2-3: Timing offset among multi-panel/TRP (FR1 only) OK with the recommended WF, - Issue 1-2-4: TRS/CSI-RS configuration OK with the recommended WF. Sub-Topic 1-3: Test parameters for Multi-DCI based multi-TRP/Panel transmission schemes (eMBB) - Issue 1-3-1: Resource allocation We support the recommended WF to only introduce requirements with non-overlapping resource allocation. - Issue 1-3-2: Antenna configuration per each TRP We support the recommended WF. - Issue 1-3-3: Number of Test cases OK with option 3. Sub-Topic 1-4: Test parameters for Single-DCI based multi-TRP/Panel transmission schemes (eMBB) - Issue 1-4-1: Number of Test cases OK with option 3. |
| Huawei, HiSilicon | Issue 1-1-1: Prefer option 2. RAN4 does not define test case for every single UE feature. For this issue, If we want, we can always find differences between each URLLC transmission scheme, otherwise there is no meaning for RAN1 to define it. However, what we focus is whether the major demodulation process or feature that has been covered and verified by already defined test cases. If so, we can skip this feature to reduce the overall number of test cases. As we mentioned before, the major demodulation feature of those transmission schemes for URLLC are well covered by exist or agreed-to-introduce test cases. Therefore, we don’t see the need for define new requirements for those schemes. Issue 1-1-2: For case 1, single wide Rx beam reception might have problem with the reception angle. UE at the cell edge has few chances to be on one side of two TRPs. And if one UE is in the middle of two TRPs, then one Rx beam is struggling to be wide enough to cover two transmission beams. Thus, we think case 1 is not realistic. Thus, we prefer not have requirements for case 1. For case 2, we are fine for further discuss this issue in future release, but for now we cannot agree on anything on this issue. Issue 1-2-1, 1-2-2: OK with recommended WF. It is straightforward to use TRP with TCI state #0 as the reference. What we concern is the criteria for deciding TCI state #0, like highest RSRP or nearest distance etc. But since these criteria might related to the UE specific implementation, RAN4 should not define such restrictions. Issue 1-2-3: Based on our comments for issue 1-2-1 and 1-2-2, positive and negative timing offset are both theoretically possible. But we are not sure in practice, how many chances that a negative timing offset could be met. Since the feature of Multi-TRP mainly serves those edge UEs, how many chances that they are in a scenario with two TRPs on one side. But if all companies think negative timing offset is necessary, we are fine to consider it. For positive timing offset value, we prefer to select 2us for 15kHz SCS and scaling for 30kHz SCS. Without scaling, 2us is a little bit risky for 30kHz SCS. Issue 1-2-4: Agree recommended WF. Issue 1-3-1: Agree recommended WF. Issue 1-3-2: Agree recommended WF. Issue 1-3-3: For limiting the number of test cases, we are fine to define one test covers both frequency offset and one selected time offset. If both positive timing offset and negative timing offset are introduced, then two different value can be applied on multi-DCI based test case and single-DCI based test case respectively. Thus, we are ok with option 3 or 4. Issue 1.4-1: Same comments as issue 1-3-3. Issue 1-5-1~1-5-5: Prefer not to discuss this since the decision of issue 1-1-1 has not been made. |
| Ericsson | Sub-Topic 1-1: Test Scope - Issue 1-1-1: Necessity of introducing test case(s) for single DCI-based multi-panel/TRP transmission schemes (URLLC) We think there is no difference from UE demodulation point. Therefore our motivation to define FDM Scheme 1 is the case if UE does not support multi-DCI. Also the metric is same as eMBB, i.e., 70% of max Tput with HARQ retransmission enabled. - Issue 1-1-2: Necessity of introducing test case(s) for multi-panel/TRP transmission schemes in FR2 We prefer Option 1. We have concern Option 2 because the UE performance in this setup is affected the OTA test environment. We are wondering FR2 UE can keep the same reception performance regardless of OTA test environment. Sub-Topic 1-2: Generic test set-up - Issue 1-2-1: Reference for timing offset/frequency offset set-up Agree with the recommended WF. - Issue 1-2-2: Baseline receiver assumption for FFT window timing Agree with the recommended WF. - Issue 1-2-3: Timing offset among multi-panel/TRP (FR1 only) We are ok to set negative time offset larger than -0.5us for SCS=15kHz and -0.25us for SCS=30kHz However it does not mean the number of test cases are increased just to test the negative time offset. If RAN4 specify 2 test cases, for example, one for multi-DCI SDM and another for single-DCI SDM, then we can set +2us for one case and -0.5 for another case. From our simulation it is observed the significant performance degradation with -0.5us for SCS=30kHz. We therefore propose to set -0.25us for SCS=30kHz. - Issue 1-2-4: TRS/CSI-RS configuration Agree with the recommended WF. - Issue 1-2-5: Timing offset among multi-panel/TRP for FR2 (Postpone to 2nd round) - Issue 1-2-6: Frequency offset among multi-panel/TRP for FR2 (Postpone to 2nd round) Sub-Topic 1-3: Test parameters for Multi-DCI based multi-TRP/Panel transmission schemes (eMBB) - Issue 1-3-1: Resource allocation Agree with option 1. - Issue 1-3-2: Antenna configuration per each TRP Agree with the recommended WF. - Issue 1-3-3: Number of Test cases We don’t want to increase the test cases just to test for frequency offset/time offset configuration. Since we set one test case for multi-DCI SDM (applicable for FDD/TDD and 2Rx/4Rx) and one test case for single-DCI SDM, one case sets frequency offset + positive time offset and another case sets frequency offset and negative time offset. We are open which case uses positive/negative time offset. Sub-Topic 1-4: Test parameters for Single-DCI based multi-TRP/Panel transmission schemes (eMBB) - Issue 1-4-1: Number of Test cases Same comments as 1-3-3. Since we set one test case for multi-DCI SDM (applicable for FDD/TDD and 2Rx/4Rx) and one test case for single-DCI SDM, one case sets frequency offset + positive time offset and another case sets frequency offset and negative time offset. We are open which case uses positive/negative time offset. Sub-Topic 1-5: Test parameters for Single-DCI based multi-TRP/Panel transmission schemes (URLLC) - Issue 1-5-1: Transmission schemes We think there is no difference from UE demodulation point. Therefore our motivation to define FDM Scheme 1 is the case if UE does not support multi-DCI. Also the metric is same as eMBB, i.e., 70% of max Tput with HARQ retransmission enabled. - Issue 1-5-2: Test metric Option 1. - Issue 1-5-3: Test applicability (Postpone to 2nd round) - Issue 1-5-4: Number of Test cases (Postpone to 2nd round) Issue 1-5-5: PDSCH configuration for single-DCI based multi-TRP repetition schemes |
| Samsung | Sub-Topic 1-1: Test Scope - Issue 1-1-1: Necessity of introducing test case(s) for single DCI-based multi-panel/TRP transmission schemes (URLLC) The major debate point for the necessity of introducing test cases of URLLC transmission schemes is whether UE behaviour already covered by the agreed Multi-TRP transmission test cases and/or test cases introduced under URLLC WI. URLLC based on transmission schemes and eMBB based on transmission schemes target to different deployment and usage scenarios In our view, at least the following aspects cannot be covered in current test case for eMBB or Rel-16 URLLC- -Compared to URLLC test cases, these transmission schemes also required to support multi-TCI states with different QCL information associated with same TBS which transmitted from different TRPs -Meanwhile, the time/frequency non-synchronization transmission cannot be verified in current Rel-16 single TRP URLLC test -Compared to current eMBB test cases, the special scheduling and processing, such as mini-slot scheduling, CW combing within slots and across slots for URLLC operation cannot be verified in current eMBB test From UE feature perspective, single DCI-based multi-panel/TRP with different transmission schemes belongs different UE feature, UE maybe or maybe not to support FDM or TDM schemes, Based on above analysis, the UE function/behaviour can be partially verified in the already agreed URLLC test cases or multi-TRP transmission test cases for eMBB transmission from UE processing aspect; meanwhile there are still delta compared what already agreed test cases. As a whole independent feature itself, the UE behaviour hasn’t been covered by existing test cases. - Issue 1-1-2: Necessity of introducing test case(s) for multi-panel/TRP transmission schemes in FR2 The debate point in RAN4 whether there are realistic deployment scenarios to schedule UE which only capable of single Rx beam for multi-Panel/TRP transmission schemes in FR2. We see at least two potential scenarios for multi-panel/TRP transmission with same TX/Rx beam direction: - FR2 scheduling scenario A: two panels implemented in the same site with same transmitted beam to UE - FR2 scheduling scenario B: two TRPs located in different sites with the same transmitted beam direction to UE For case 2, Considering the current OTA test ability issue, we are fine with recommend WF Sub-Topic 1-2: Generic test set-up - Issue 1-2-1: Reference for timing offset/frequency offset set-up We are fine with recommend WF. - Issue 1-2-2: Baseline receiver assumption for FFT window timing We are fine with recommend WF. - Issue 1-2-3: Timing offset among multi-panel/TRP (FR1 only) We still prefer option 1 - Positive time offset • For FR1 FDD with 15kHz : 2us • For FR1 TDD with 30kHz : 1us - Negative time offset • For FR1 FDD with 15kHz : -0.5 • For FR1 TDD with 30kHz : -0.25 For time offset, taking LTE CoMP transmission scheme experience; positive and negative time offset according to FFT window has unsymetric performance impact in receiver side. Reasonable UE implementation will employ some strategy to reduce the probability of negative time offset refer to FFT window i.e. adjusting FFT window based on the earliest received signal path, nearest TRP with strongest received power plus fixed timing offset. On the other hand, there is always a probability that received signals arrive before or lag behind FFT window in reality considering UE mobility and FFT window adjustment cycle i.e. UE move to the canter area among two TPs. Based on our initial results, we show the different performance gap with Therefore, we think it is necessary to introduce both postive and negative offset among two TPs to ensure proper UE implemantion and performance. As for value of timing offset with 30KHz. in our view, it should be depended on the ISD of deployment. 30KHz is mainly applied in TDD operation, where carrier frequency is high, the cell size is not expected with too large, In our view, around 200ISD should be enough. Based on Intel’ contribution, In case of ISD=200m, around 85% UE have TO within the range of [-0.25, 1] us. Given current deployment, option 3 is out of CP range. Therefore, it make sense to introduce the test cases with timing offset within the range for 30KHz. Meanwhile, the motivation of introduce timing/frequency is to verify the proper UE behaviour with TO/FO compensation. Based on our initial results, less than 0.5 dB performance loss observed compared to ideal case with proper compensation for positive timing offset , and less than 1dB performance loss observed compared ideal case with proper compensation for negative timing offset. In our view, the existed performance gap is enough to discriminate different UE behaviour with and without time compensation. - Issue 1-2-4: TRS/CSI-RS configuration We are fine with the recommend WF following the previous agreement Sub-Topic 1-3: Test parameters for Multi-DCI based multi-TRP/Panel transmission schemes (eMBB) - Issue 1-3-1: Resource allocation We are fine with the recommend WF - Issue 1-3-2: Antenna configuration per each TRP We are fine with the recommend WF - Issue 1-3-3: Number of Test cases We still prefer option 1. To compact the test case, we are fine to introduce one test case per duplex mode to cover both frequency offset and time offset. As mention, both positive and negative time offset can appear among 2 TRP transmission scenario. Considering the performance different between positive and negative time offset, we think it is needed to define the test cases to cover both positive and negative timing offset to verify UE behaviour. If companies have the concerns the number of test cases, we can define the test related applicability rule. Single-DCI and multi-DCI are different UE feature. Most likely, UE supported multi-DCI based transmission can also support single-DCI based transmission, then if UE pass the multi-DCI based transmission tested, it can skip the single-DCI test. Sub-Topic 1-4: Test parameters for Single-DCI based multi-TRP/Panel transmission schemes (eMBB) - Issue 1-4-1: Number of Test cases To compact the test case, we are fine to introduce one test case per duplex mode to cover both frequency offset and time offset. As mention, both positive and negative time offset can appear among 2 TRP transmission scenario. Considering the performance different between positive and negative time offset, we think it is needed to define the test cases to cover both positive and negative timing offset to verify UE behaviour. If companies have the concerns the number of test cases, we can define the test related applicability rule. Single-DCI and multi-DCI are different UE feature. Most likely, UE supported multi-DCI based transmission can also support single-DCI based transmission, then if UE pass the multi-DCI based transmission tested, it can skip the single-DCI test. Sub-Topic 1-5: Test parameters for Single-DCI based multi-TRP/Panel transmission schemes (URLLC) - Issue 1-5-1: Transmission schemes We prefer option 1. As mentioned, single-DCI based transmission with different schemes belong different UE feature. FDMschemeB requirement additional signalling for UE to support CW combination with different RV. TDMschemeA is similar with Rel-16 URLLC feature with intra-slot repetition. Whether to define the requirement of Rel-16 URLLC feature is not decided yet So, we suggest to down selected with 1 or 2 schemes from {FDM scheme A, FDM scheme B, TDM scheme A, Inter-slot TDM}. FDM scheme A and inter-slot TDM is preferred. |

CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |
| Sub-topic#1-1 | Issue 1-1-1: Necessity of introducing test case(s) for single DCI-based multi-panel/TRP transmission schemes (URLLC) Candidate options： - Option 1: Yes (Intel, Samsung, Ericsson) - Option 2: No (Huawei, Qualcomm, MTK, Apple) Recommendations for 2nd round: - Proponents of each option should identify what is the difference from UE processing aspect compared to existing URLLC test cases (URLLC WI) and eMBB operation multi-panel/TRP transmission scheme test cases - Issue 1: whether multi-TCI states with different QCI information associated with same TBS transmitted from different TRPs can be covered in current Rel-16 URLLC test cases (only one TCI state) - Issue 2: whether the time/frequency non-synchronization multi-TRP transmission can be covered in current Rel-16 URLLC test cases (no time/frequency tracking assumption)? - Issue 3: whether special scheduling and processing, such as mini-slot scheduling, CW (with different RV) combing within slots and across slots for URLLC operation can be covered in current eMBB operation multi-panel/TRP transmission scheme test cases (only 1 CW) Issue 1-1-2: Necessity of introducing test case(s) for multi-panel/TRP transmission schemes in FR2 Tentative agreements: No PDSCH demodulation requirements with multi-Panel/TRP transmission schemes under single Tx/Rx beam with same QCI Type-D in FR2 for Rel-16 Considering current OTA Test limitation for demodulation test cases, no test cases for multi-TRP transmission with multi-Tx beams in TDM manner (URLLC TDM scheme) in Rel-16; and further discuss in future release with consideration of test ability issue Recommendations for 2nd round: N.A |
| Sub-topic#1-2 | Issue 1-2-1: Reference for timing offset/frequency offset set-up Tentative agreements: - Option 1: Using TP1 is Reference to define timing and frequency offset (Samsung, Ericsson) - Timing offset = time offset among TP2 and TP1 - Frequency offset = frequency offset among TP2 and TP1 Recommendations for 2nd round: N.A Issue 1-2-2: Baseline receiver assumption for FFT window timing Tentative agreements: It’s up to UE implementation for FFT windowing timing adjustment strategy; Define performance requirements in receiver agnostic manner. Meanwhile define RAN4 performance requirements based on the assumption of UE sets FFT timing based on TCI state #0 (TP1 i.e. SSB only transmitted from TP1). Recommendations for 2nd round: N.A Issue 1-2-3: Timing offset among multi-panel/TRP (FR1 only) Tentative agreements: - FR1 FDD with 15KHz - Positive timing offset: 2us - Negative timing offset : -0.5us Candidate options： - FR2 TDD with 30KHz - Positive timing offset : - Option 1: 2us (Intel) - Option 2: 1us (Samsung, Huawei, Ericsson, Qualcomm? Apple?) - Negative timing offset - Option 1: -0.5us (Intel) - Option 2: -0.25us (Samsung, Huawei, Ericsson, Qualcomm? Apple?) Recommendations for 2nd round: - Given the ISD with targeting high carrier frequency for 30KHz is small , less UE have timing offset out of [-0.25,1us], whether proponent of option 1 can compromise to option 2? Issue 1-2-4: TRS/CSI-RS configuration Tentative agreements: introduce the test cases with only non-colliding TRS/CSI-RS in multi-TRP/panel Recommendations for 2nd round: N.A |
| Sub-topic#1-3 | Issue 1-3-1: Resource allocation Tentative agreements: - Option 1: Only non-overlapping cases Recommendations for 2nd round: N.A Issue 1-3-2: Antenna configuration per each TRP Tentative agreements: - Option 1: Only 2T2R, 2T4R Recommendations for 2nd round: N.A Issue 1-3-3: Number of test cases Candidate options： - Option 1: 2 test cases per duplex mode with test applicability rule (Samsung) - Test 1a Multi- DCI with frequency offset and negative time offset and non-overlapping scheduling - Test 1b Multi DCI with positive time offset and non-overlapping scheduling - FFS on test applicability rule - Option 2: 1 test case per duplex mode (Huawei, Qualcomm, Intel, Ericsson, Apple) - Test 1a Multi- DCI with frequency offset, [negative time offset] or [positive time offset] and non-overlapping scheduling - Option 3: 1 test case per duplex mode (Ericsson) - Test 1a Multi DCI with frequency offset, negative time offset and non-overlapping scheduling - Test 1b Single DCI with frequency offset , positive time offset and overlapping Recommendations for 2nd round: - Companies’ feedback needed for about proposals, down select one option |
| Sub-topic#1-4 | Issue 1-4-1: Number of test cases Candidate options: - Option 1: 2 test cases per duplex mode with test applicability rule (Samsung) - Test 1a Single- DCI with frequency offset , negative time offset and overlapping scheduling - Test 1b Single- DCI with positive time offset and overlapping scheduling - FFS on test applicability rule - Option 2: 1 test case per duplex mode (Huawei, Qualcomm, Intel, Ericsson, Apple) - Test 1a Single- DCI with frequency offset, [negative time offset] or [positive time offset] and overlapping scheduling - Option 3: 1 test case per duplex mode (Ericsson) - Test 1a Multi DCI with frequency offset, negative time offset and overlapping scheduling - Test 1b Single DCI with frequency offset , positive time offset and overlapping Recommendations for 2nd round: - Companies’ feedback needed for about proposals, down select one option |
| Sub-topic#1-5 | Issue 1-5-1: Transmission schemes Candidate options: - Option 1: Define performance requirements for all the single-DCI based multi-TRP Tx repetition schemes (FDM Scheme A, FDMSchemeB, TDMSchemeA, Inter-slot TDM)(Intel) - Option 2: Single-DCI based FDM Scheme A (Ericsson) - Option 3: Down-selected with 1 or 2 scheme(s) from { FDM Scheme A, FDM SchemeB, TDMSchemeA, Inter-slot TDM } , at least covering TDM scheme (Samsung) Recommendations for 2nd round: - Related to discussion on issue 1-1-2 Issue 1-5-2: Test Metric Candidate options: - Option 1: 70% TP (Ericsson) - Option 2: 1% BLER (Intel) Recommendations for 2nd round: - Related to discussion on issue 1-1-2 Issue 1-5-3: Test applicability rule - Option 1(Intel) Only single-DCI SDM; Only single-DCI repetition scheme(s); Only multi-DCI without over-ing; Both single-DCI SDM and multi-DCI; Both single-DCI SDM and repetition scheme(s); Both single-DCI repetition scheme(s) and multi-DCI; Both single-DCI repetition scheme(s), SDM and multi-DCI. Single-DCI SDM: FO + positive TO; Yes; Yes; Yes. Multi-DCI Non-Overlapped: FO + negative TO; Yes; Yes; Yes. Single-DCI repetition scheme: FO + positive TO; Yes (Note 1); Yes (Note 1); Yes (Note 1). Note 1: In case of supporting both single DCI based FDM scheme(s) and TDM scheme(s) UE can be tested only for TDM scheme.. - Only TDM scheme can be tested if UE supporting both single DCI based FDM scheme(s) and TDM scheme(s) - Only FDMSchemeB can be tested if UE supporting both single DCI based FDMSchemeA and FDMSchemeB - Only TDMSchemeA can be tested if UE supporting both TDMScheme A and inter-slot TDMrepetition schemes - Only TDM can be tested if UE supporting both FDM and TDM repetition schemes - Option 2: If UE is capable of two CORE CORESTPoolIndex reception and passes mDCI-based SDM tests, UE can skip sDCI-based FDM Scheme A (Ericsson) Recommendations for 2nd round: - Related to discussion on issue 1-1-2 Issue 1-5-4: Number of test cases - Option 1(Samsung): 2 test cases per duplex mode - Test 1a (Samsung, Ericsson): Single-DCI based FDM scheme A with frequency offset and negative time offset - Test 1b: Single-DCI based inter-slot TDM with positive time offset - Option 2 (Intel): 4 test cases per duplex mode - Test 1a: Single-DCI based FDMScheme A with frequency offset and positive time offset - Test 1b: Single-DCI based FDMScheme B with frequency offset and positive time offset - Test 1c: Single-DCI based TDMScheme A with frequency offset with positive time offset - Test 1b: Single-DCI based inter-slot TDM with frequency offset with positive time offset Recommendations for 2nd round: - Related to discussion on issue 1-1-2 Issue 1-5-5: PDSCH configuration for single-DCI based multi-TRP repetition schemes - Option 1(Intel): - Reused the parameters (TDD configuration, SSB and CSI-RS configuration, PDCCH setup, Propagation conditions and TO/FO) agreed for single-DCI based Tx scheme for each repetition scheme - MCS 13 - Resource configuration FDMSchemeA; FDMSchemeB; TDMSchemeA; Inter-slot TDM. Mapping type; Type A; Type A; Type A; Type A. Resource allocation type; Type 1; Type 1; Type 1; Type 1. DMRS configuration; Type 1, 1+1; Type 1, 1+1; Type 1, No additional symbols; Type 1, 1+1. Antenna port index; {1000} for TCI#1 and {1001} for TCI#2; {1000} for TCI#1 and {1001} for TCI#2; {1000} for TCI#1 and {1001} for TCI#2; {1000} for TCI#1 and {1001} for TCI#2. Start symbol and time duration; PDSCH#1: 2, 12 PDSCH#1: 2, 12; PDSCH#1: 2, 12 PDSCH#1: 2, 12; PDSCH#1: 2, 6 PDSCH#1: 8, 12; PDSCH#1: 2, 12 PDSCH#1: 2, 12. FDD PRB allocation; PDSCH#1: PRB#0 to PRB#25 PDSCH#2: PRB#26 to PRB#51; PDSCH#1: PRB#0 to PRB#25 PDSCH#2: PRB#26 to PRB#51; PDSCH#1: PRB#0 to PRB#51 PDSCH#2: PRB#0 to PRB#51; PDSCH#1: PRB#0 to PRB#51 PDSCH#2: PRB#0 to PRB#51. TDD PRB allocation; PDSCH#1: PRB#0 to PRB#52 PDSCH#2: PRB#53 to PRB#105; PDSCH#1: PRB#0 to PRB#52 PDSCH#2: PRB#53 to PRB#105; PDSCH#1: PRB#0 to PRB#105 PDSCH#2: PRB#0 to PRB#105; PDSCH#1: PRB#0 to PRB#105 PDSCH#2: PRB#0 to PRB#105. Recommendations for 2nd round: - Related to discussion on issue 1-1-2 |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | WF for general and PDSCH requirements with Single-DCI SDM scheme and Multi-DCI transmission schemes (eMBB) | Huawei |
| #2 | WF for PDSCH requirements with Single-DCI based multi-TRP/Panel transmission schemes (URLLC) | Intel |
| #3 | Simulation assumption for PDSCH requirements with Single-DCI SDM scheme and Multi-DCI transmission schemes | Ericsson |

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Issue 1-1-1: Necessity of introducing test case(s) for single DCI-based multi-panel/TRP transmission schemes (URLLC)

Candidate options：

  - Option 1: Yes (Intel, Samsung, Ericsson)

  - Option 2: No (Huawei, Qualcomm, MTK, Apple)

Recommendations for 2nd round:

  - Proponents of each option should identify what is the difference from UE processing aspect compared to existing URLLC test cases (URLLC WI) and eMBB operation multi-panel/TRP transmission scheme test cases

- Issue 1: whether multi-TCI states with different QCI information associated with same TBS transmitted from different TRPs can be covered in current Rel-16 URLLC test cases (only one TCI state)

- Issue 2: whether the time/frequency non-synchronization multi-TRP transmission can be covered in current Rel-16 URLLC test cases (no time/frequency tracking assumption)?

- Issue 3: whether special scheduling and processing, such as mini-slot scheduling, CW (with different RV)  combing within slots and across slots for URLLC operation can be covered in current eMBB operation multi-panel/TRP transmission scheme test cases (only 1 CW)

Issue 1-2-3: Timing offset among multi-panel/TRP (FR1 only)

Candidate options：

  - FR2 TDD with 30KHz

- Positive timing offset :

- Option 1: 2us (Intel)

- Option 2: 1us (Samsung, Huawei, Ericsson, Qualcomm? Apple?)

- Negative timing offset

- Option 1: -0.5us (Intel)

- Option 2: -0.25us (Samsung, Huawei, Ericsson, Qualcomm? Apple?)

Recommendations for 2nd round:

  - Given the ISD with targeting high carrier frequency for 30KHz is small , less UE have timing offset out of [-0.25,1us], whether proponent of option 1 can compromise to option 2?

Issue 1-3-3: Number of test cases for Multi-DCI

Candidate options：

  - Option 1: 2 test cases per duplex mode with test applicability rule  (Samsung)

- Test 1a Multi- DCI with frequency offset and negative time offset and non-overlapping scheduling

- Test 1b Multi DCI with positive time offset and non-overlapping scheduling

- FFS on test applicability rule

  - Option 2: 1 test case per duplex mode (Huawei, Qualcomm, Intel, Ericsson, Apple)

- Test 1a Multi- DCI with frequency offset, [negative time offset] or [positive time offset] and non-overlapping scheduling

  - Option 3: 1 test case per duplex mode (Ericsson)

- Test 1a Multi DCI with frequency offset,  negative time offset and non-overlapping scheduling

- Test 1b Single DCI with frequency offset , positive time offset and overlapping

Recommendations for 2nd round:

  - Companies’ feedback needed for about proposals, down select one option

Issue 1-4-1: Number of test cases for single-DCI

Candidate options:

  - Option 1: 2 test cases per duplex mode with test applicability rule (Samsung)

- Test 1a Single- DCI with frequency offset , negative time offset and overlapping scheduling

- Test 1b Single- DCI with positive time offset and overlapping scheduling

- FFS on test applicability rule

  - Option 2: 1 test case per duplex mode (Huawei, Qualcomm, Intel, Ericsson, Apple)

- Test 1a Single- DCI with frequency offset, [negative time offset] or [positive time offset] and overlapping scheduling

  - Option 3: 1 test case per duplex mode (Ericsson)

- Test 1a Multi DCI with frequency offset,  negative time offset and overlapping scheduling

- Test 1b Single DCI with frequency offset , positive time offset and overlapping

Recommendations for 2nd round:

  - Companies’ feedback needed for about proposals, down select one option

Issue 1-5-1: Transmission schemes

Candidate options:

  - Option 1: Define performance requirements for all the single-DCI based multi-TRP Tx repetition schemes (FDM Scheme A, FDMSchemeB, TDMSchemeA, Inter-slot TDM)(Intel)

  - Option 2: Single-DCI based FDM Scheme A  (Ericsson)

  - Option 3: Down-selected with 1 or 2 scheme(s) from { FDM Scheme A, FDM SchemeB, TDMSchemeA, Inter-slot TDM } , at least covering TDM scheme (Samsung)

Recommendations for 2nd round:

  - Related to discussion on issue 1-1-2

Issue 1-5-2: Test Metric

Candidate options:

  - Option 1: 70% TP (Ericsson)

  - Option 2: 1% BLER (Intel)

Recommendations for 2nd round:

  - Related to discussion on issue 1-1-2

Issue 1-5-3: Test applicability rule

  - Option 1(Intel)

|  | Only single-DCI SDM | Only single-DCI repetition scheme(s) | Only multi-DCI without over-ing | Both single-DCI SDM and multi-DCI | Both single-DCI SDM and repetition scheme(s) | Both single-DCI repetition scheme(s) and multi-DCI | Both single-DCI repetition scheme(s), SDM and multi-DCI |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Single-DCI SDM: FO + positive TO | Yes |  |  | Yes |  |  | Yes |
| Multi-DCI Non-Overlapped: FO + negative TO |  |  | Yes | Yes |  |  | Yes |
| Single-DCI repetition scheme: FO + positive TO |  | Yes (Note 1) |  |  | Yes (Note 1) | Yes (Note 1) |  |
| Note 1: In case of supporting both single DCI based FDM scheme(s) and TDM scheme(s) UE can be tested only for TDM scheme. |  |  |  |  |  |  |  |

- Only TDM scheme can be tested if UE supporting both single DCI based FDM scheme(s) and TDM scheme(s)

- Only FDMSchemeB can be tested if UE supporting both single DCI based FDMSchemeA and FDMSchemeB

- Only TDMSchemeA can be tested if UE supporting both TDMScheme A and inter-slot TDMrepetition schemes

- Only TDM can be tested if UE supporting both FDM and TDM repetition schemes

  - Option 2: If UE is capable of two CORE CORESTPoolIndex reception and passes mDCI-based SDM tests, UE can skip sDCI-based FDM Scheme A (Ericsson)

Recommendations for 2nd round:

  - Related to discussion on issue 1-1-2

Issue 1-5-4: Number of test cases

  - Option 1(Samsung): 2 test cases per duplex mode

- Test 1a (Samsung, Ericsson): Single-DCI based  FDM scheme A with frequency offset and negative time offset

- Test 1b: Single-DCI based inter-slot TDM with positive time offset

  - Option 2 (Intel): 4 test cases per duplex mode

- Test 1a: Single-DCI based FDMScheme A with frequency offset and positive time offset

- Test 1b: Single-DCI based FDMScheme B with frequency offset and positive time offset

- Test 1c: Single-DCI based TDMScheme A with frequency offset with positive time offset

- Test 1b: Single-DCI based inter-slot TDM with frequency offset with positive time offset

Recommendations for 2nd round:

  - Related to discussion on issue 1-1-2

Issue 1-5-5: PDSCH configuration for single-DCI based multi-TRP repetition schemes

  - Option 1(Intel):

- Reused the parameters (TDD configuration, SSB and CSI-RS configuration, PDCCH setup, Propagation conditions and TO/FO) agreed for single-DCI based Tx scheme for each repetition scheme

- MCS 13

- Resource configuration

|  | FDMSchemeA | FDMSchemeB | TDMSchemeA | Inter-slot TDM |
| --- | --- | --- | --- | --- |
| Mapping type | Type A | Type A | Type A | Type A |
| Resource allocation type | Type 1 | Type 1 | Type 1 | Type 1 |
| DMRS configuration | Type 1, 1+1 | Type 1, 1+1 | Type 1, No additional symbols | Type 1, 1+1 |
| Antenna port index | {1000} for TCI#1 and {1001} for TCI#2 | {1000} for TCI#1 and {1001} for TCI#2 | {1000} for TCI#1 and {1001} for TCI#2 | {1000} for TCI#1 and {1001} for TCI#2 |
| Start symbol and time duration | PDSCH#1: 2, 12 PDSCH#1: 2, 12 | PDSCH#1: 2, 12 PDSCH#1: 2, 12 | PDSCH#1: 2, 6 PDSCH#1: 8, 12 | PDSCH#1: 2, 12 PDSCH#1: 2, 12 |
| FDD PRB allocation | PDSCH#1: PRB#0 to PRB#25 PDSCH#2: PRB#26 to PRB#51 | PDSCH#1: PRB#0 to PRB#25 PDSCH#2: PRB#26 to PRB#51 | PDSCH#1: PRB#0 to PRB#51 PDSCH#2: PRB#0 to PRB#51 | PDSCH#1: PRB#0 to PRB#51 PDSCH#2: PRB#0 to PRB#51 |
| TDD PRB allocation | PDSCH#1: PRB#0 to PRB#52 PDSCH#2: PRB#53 to PRB#105 | PDSCH#1: PRB#0 to PRB#52 PDSCH#2: PRB#53 to PRB#105 | PDSCH#1: PRB#0 to PRB#105 PDSCH#2: PRB#0 to PRB#105 | PDSCH#1: PRB#0 to PRB#105 PDSCH#2: PRB#0 to PRB#105 |

Recommendations for 2nd round:

  - Related to discussion on issue 1-1-2

| Issue | Comments |
| --- | --- |
| Issue 1-1-1: Necessity of introducing test case(s) for single DCI-based multi-panel/TRP transmission schemes (URLLC) | Ericsson: Option 1. However the idea is to test FDMSchemeA only in case UE is not capable of multi-DCI based transmission. Issues 1: We are not sure how important to verify the multiple TCI states associated with one TBS for both ‘eMBB’ and ‘URLLC’. From UE demodulation algorithm point of view, we don’t see any difference between eMBB and URLLC. Since there is not UE categories or capabilities to indicate ‘eMBB UE’ or ‘URLLC UE’, it is enough to verify this feature either with eMBB scenario or URLLC scenario. Issue 2/3: Same comment as Issue 1. Email discussion comment in [96e][320] NR_eMIMO_Demod #2 demod requirements for URLLC schemes Samsung Qualcomm We still have the same position as before. Our preference is not to define the requirements for URLLC schemes since demodulation performance is already covered by Multi-DCI, SDM scheme and URLLC slot aggregation, as we commented in the 1st round. So, we don’t think it is reasonable to say “Define performance requirements only for FDM scheme A and inter-slot TDM scheme” when four companies were against defining the requirements for URLLC schemes. It can be listed as one of the options, but it should not be called the agreement until we have consensus As a compromise, we would suggest to have following applicability rule: “FDM scheme is skipped if UE passes the multi-DCI based multi-TRP transmission and TDM scheme is skipped if UE passes URLLC slot aggregation requirements.” Intel We do not want to again restart technical discussion and copy previous comments considering remaining time before the deadline. After first-round comments we have different views on this issue but based on the agreement from the previous meeting we should make conclusion now hence we cannot just list companies views. From our side, we see a one possible compromise, as define requirements only for two schemes instead of 4. Also, to address companies concerns on test efforts we are fine to introduce reasonable applicability rule. If you still do not agree with such compromise, please suggest another which can address views from both camps The proposed applicability rule is fine for us with additional statement for UEs which support only TDM scheme: “FDM scheme is skipped if UE passes the multi-DCI based multi-TRP Tx and TDM scheme is skipped if UE passes URLLC slot aggregation requirements and anyone of the other multi-TRP Tx requirements” Regarding MCS value, we are fine to not preclude other options even that we have one meeting left and too many options will lead to more complicated discussion. Regarding other options for particular scheme, we think it is not possible to make such agreement, because it means that companies need to prepare simulation results for all 4 schemes and beside that for different TO values and MCSs. As we see, additional simulation efforts were one of the main concerns to define corresponding requirements. Considering upcoming deadline, we think it is impossible to revert previous agreement which was proposed 3 days ago. The 2 test cases are mean between zero and 4, and at least in this case it is a compromise and I hope that agreed applicability rule can address your concerns We were open to discuss particular schemes for requirements definition and on this we had ~3 days but there were no any objections and now you are trying to revert agreement just 10 minutes before the deadline. Considering that all other companies are agree with WF and we have 3 meetings discussions and only one meeting left to complete WI, we think that page 4 should be acceptable. Otherwise we will not finalize requirements on time Huawei For URLLC schemes, we have discussed for the past three meetings and with no consensus. Actually, from demodulation perspective, we think the essential UE behaviors have been already covered by existed or agreed to be introduced test cases. While, considering the urgent situation for only 1 meeting left, we are trying to consider compromise solutions with define limited test case(s). But we need more time to check those schemes. In this case, we suggest to add ‘other options not precluded’ on page 4. Also, please add ‘other options not precluded’ on page 6 for MCS selection We’ve already give our compromise to define some requirements. We just need more time to check those schemes and do the selection, which is not against any previous agreements. Regarding to the workload you’ve mentioned, we think following sentence can be added with left schemes selection open: ‘companies are encouraged to bring simulation results for the schemes they want to define |
| Issue 1-2-3: Timing offset among multi-panel/TRP (FR1 only) | Samsung: we are ok with option 2 and recommend WF. The purpose is to verify the proper timing offset tracking behavior. Based on the current simulation results, the performance gap existed between ideal time offset and timing offset compensation is enough to discriminate the UE behavior. Ericsson: This should be FR1 TDD with 30kHz. We support moderator’s recommendation. TDD 30kHz is used for mid-band (freq > 2.5GHz) and ISD is small. It makes sense to consider smaller time offset compared with FDD used in low-band (2us). We also concern the performance loss with larger time offset according to our simulation results. Email discussion comment in 96e][320] NR_eMIMO_Demod #1 demod requirement Intel Regarding TO values for requirements definition, considering results from Ericsson which assumes another UE implementation comparing to us, major companies view and also remaining time, we can compromise to option 2 to move forward |
| Issue 1-3-3: Number of test cases for Multi-DCI | Samsung During the 1st round discussion, the attentive agreement both negative time offset and positive time offset should be introduced to define the related PDSCH requirement. Considering the performance diverse with positive and negative time offset, as well as test coverage, it is needed to define the proper test case to verify UE receiver behavior of time tracking. Meanwhile, in the last meeting, it is agree to define the PDSCH requirement for both single-DCI and multi-DCI. From UE feature list, single-DCI and multi-DCI belongs to two different UE feature. UE maybe or maybe not support these feature. Since companies have concerns about the number of test cases, we propose to discuss the number of test for multi-TCI and single-TCI together with test applicability rule for make move forward. Issue 1-3-3: Number of test cases for single-DCI/Multi-DCI - Option 4: 4 test cases per duplex mode with test applicability rule (Samsung) - Test 1a single- DCI with frequency offset and negative time offset and overlapping scheduling - Test 1b single- DCI with positive time offset and overlapping scheduling - Test 2a Multi- DCI with frequency offset and negative time offset and non-overlapping scheduling - Test 2b Multi DCI with positive time offset and non-overlapping scheduling - Test applicability rule - If UE only supports single-DCI based multi-TRP transmission for eMBB, it should be tested with test case 1a and test case 1b - If UE only supports multi-DCI based on multi-TRP transmission for eMBB, not support single –DCI, it should be tested test 2a and test 2b. Maybe this UE is not valid. - If UE can support both single-DCI and multi-DCI for eMBB, it should be tested test 2a and either test 2b or test 1b Furthermore, in order to further compact the test cases for UE can support both single-DCI and multi-DCI based on multi-TRP transmission as - Option 5: 3 test cases per duplex mode with test applicability rule (Samsung) - Test 1a single- DCI with frequency offset and negative time offset and overlapping scheduling - Test 1b single- DCI with positive time offset and overlapping scheduling - Test 2a Multi- DCI with frequency offset and negative time offset and non-overlapping scheduling - Test applicability rule - If UE only supports single-DCI based multi-TRP transmission for eMBB, it should be tested with test case 1a and test case 1b - If UE can support both single-DCI and multi-DCI for eMBB, it should be tested test 2a and test 1b In that sense, for each UE, although 3 cases are introduced, only 2 cases need to be tested, which can fulfil the test coverage and UE feature. Ericsson: Option 2. Email discussion comment in 96e][320] NR_eMIMO_Demod #1 demod requirement Samsung: Regarding the number of test cases, there are two issues need to be considered, one is the test coverage, another is the UE capability. Based on the 1st round discussion, companise agree to introduce the requirment with postive and nagative time offset.. Therefore, it is necessary to design the proper test case to verify the UE behaviour for time tracking/frequency tracking with nagative and positive time offset. Meanwhile, single-DCI and mult-DCI belongs two different UE feature with capability signalling based on RAN1 feature. In that sense, UE maybe or maybe not support single-DCI or Multi-DCI. If we go option3, in case of UE only support single-DCI, it can not guarantee both nagative time offset and postive time offset can be tested simultaneously, Therefore, we think it is need to define the test case with cover both nagative and positive for each feature seperately. If companies have cocerns about number of test cases, we can go option 2 for compromise. With option 2, my understanding the total number of test is same with option 3 with 2 cases need to be tested for each UE. There is no additional test effort increasing. Based on above analysis, we wonder whether company can accept option 2 for compromise for moving forward we agree with you the purpose of mDCI/sDCI-based SDM transmission test is to verify RAN1 feature. While from the network deployment point of view, both negative and positive time offset can avaible. Based on our results, the performance with nagative or positve is different. Considering the UE mobility, UE may adjust the FFT timing stragety. Therefore, it is essential to guarantee the UE receiver performance with different scenario under negative time offset and postive time offset, and without impact on whole system performance. Similar with LTE CoMP, the requirement with both positive and nagative has also been specified. To some extent, the proper network performance can be guaranteed with these requirement. Regarding the number of tests, as mentioned, option 2 and option 3 have the same number, there is no additional test effort increasing Ericsson We support Option 3. We are wondering how important to verify both positive offset or negative offset cases. Our proposal [2us, -0.5]us for SCS=15kHz and [1us, -0.25us] for SCS=30kHz are based on our simulation results so that the performance is not degraded if UE sets FFT timing based on TCI state #0. So we don’t expect performance changes with positive or negative time offsets. In our understanding the purpose of mDCI/sDCI-based SDM transmission test to verify the RAN1 features. On the other hand, we can say the time/frequency offset is the test condition. We don’t want to increase the number of tests only for the test condition. Intel 1. As a compromise we can try to merge Option 2 and 3 that this meeting agree to define the test case 1b, 2a and further discuss 1a and corresponding applicability rule. In this case we propose compromise option 4 to move forward – hope it can address companies concerns: Number of test cases for single-DCI/multi-DCI  Test 1b Single- DCI with positive time offset and overlapping scheduling.  Test 2a Multi- DCI with frequency offset and negative time offset and non-overlapping scheduling  FFS on Test 1a Single-DCI with frequency offset and negative time offset and overlapping scheduling. Applicability rule:  Option 1 (If Test 1a will be defined):  If UE only supports single-DCI based multi-TRP transmission for eMBB, it should be tested with test case 1a and test case 1b  If UE can support both single-DCI and multi-DCI for eMBB, it should be tested test 2a and test 1b Interested companies are encouraged to provide simulation results next meeting for tests 1a, 1b and 2a. QC We prefer Option 3. We also understand Samsung’s intention to define more tests but only test the UE for two of them. But, we will still have to define all those test cases, i.e. more simulation effort. So, as a compromise, we are also ok with Option 4 as proposed by Intel and we can decide based on simulation results in the next meeting Huawei Now the question seems whether to introduce another test case just for testing a different timing offset. From our view, we don’t see the need for that. Accordingto the simulation results provided by other companies, with compensation on, there will be negligible difference among performance of implementing different timing offsets. Thus, one test case for a specific timing offset is enough for verifying the rightprogress of compensation of UE. But for moving forward, we are fine with option 4 as a compromise, though we prefer option 3. |
| Issue 1-4-1: Number of test cases for single-DCI | Samsung Similar comment as Issue 1-3-3 Ericsson: Option 2. |
| Issue 1-5-1: Transmission schemes for single-DCI based multi-TRP repetition schemes | Ericsson: Option2. Idea is to test FDMSchemeA only in case UE is not capable of multi-DCI based transmission. |
| Issue 1-5-2: Test Metric for single-DCI based multi-TRP repetition schemes | Samsung: we are no strong prefer, since 1% BLER is already considered in URLLC WI for high reliability transmission without HARQ. We prefer to differentiate the test metric for URLLC. Meanwhile, HARQ transmission is enable in the test cases. Therefore, 70% TP should be enough similar with eMBB Ericsson: Option 1: We don’t see difference between eMBB and URLLC, from UE demodulation algorithm point. Email discussion comment in 96e][320] NR_eMIMO_Demod #2 demod requirements for URLLC schemes Samsung Generally, we are fine with both options. To move forward, we can compromise as option1, same with current PDSCH requirement with slot aggreagtion for high reliablity Intel Even that multi-TRP repetition schemes are discussed in eMIMO WI it is a feature to improve reliability. For high reliability features 1% BLER is considered in RAN4 for performance requirements. In this case we do not see any motivation to chose eMBB test metric |
| Issue 1-5-3: Test applicability rule | Ericsson: Option 2. |
| Issue 1-5-4: Number of test cases for single-DCI based multi-TRP repetition schemes | Ericsson: Prefer to use the same condition as multi-DCI based SDM transmission. |
| Issue 1-5-5: PDSCH configuration for single-DCI based multi-TRP repetition schemes | Company A Email discussion comment in 96e][320] NR_eMIMO_Demod #1 demod requirements for URLLC schemes Samsung we are fine with the resouce configuration. Regarding the MCS, for eMBB, MCS 19 is selected with requirement for eMBB with sDCI and mDCI. Considering the addtional gain can be achieved with CW combining, we donot think MCS19 have any limitation While we are open for further discssion, My suggest we can define MCS19 as baseline, and encourage interested companies to provide the simulation results to check the feasibility Intel We fully agree that there will not be any performance issues with same MCS 19, and even higher performance will be observed with agreed propagation conditions. Our intention regarding MCS 13 was to select MCS with CR less than 0.5. Potentially, in real field some blockages may appear for one of the Tx occasion which in results will lead to non-self-decodable transmission if we consider CR higher than 0.5 because only one received TX occasion has higher than 1 CR. This situation was comprehensively analyzed in RAN1 as a possible in real conditions. In this case we can suppose that it is typically to configure MCS13 rather than MCS19 in real deployments. We suggest collecting more companies views before agreeing on baseline MCS value. |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2012662 (PDSCH requirement for eMBB) | Agreeable, except for page 5 with minor typo - FR1 TDD with 30kHz - Positive timing offset: 1us - Negative timing offset: -0.25us Recommend to capture the modification in the Chairman note as agreement |
| R4-2012663 (PDSCH requirement for URLLC) | Agreeable, except for page 4, Moderator suggest to modify page 4 as - Define performance requirement for URLLC transmission schemes with test applicability rule - Option 1: Only FDM scheme A and inter-slot TDM scheme as baseline - FDM scheme is skipped if UE passes the multi-DCI based multi-TRP Tx requirements and TDM scheme is skipped if UE passes URLLC slot aggregation requirements and anyone of the other multi-TRP Tx requirements - Other options are not precluded - Interest companies are encouraged to provide the simulation results for URLLC transmission schemes in the next meeting at least for baseline transmission schemes (FDM scheme A and inter-slot TDM scheme) considering the PDSCH configuration in page 6 and test metric in page 7. Recommend to capture the modification in the Chairman note as agreement |
| R4-2012664 (simulation assumption) | Agreeable |

Topic #2: CSI requirements(Rel-16 TypeII codebook)

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2011366 | Ericsson | Observation 1: A SU-MIMO test cannot be used for Type II CSI reporting since the performance benefit of Type II feedback is not visible. This is due to that SU-MIMO doesn’t take advantage of the rich channel feedback of Type II reporting Observation 2: In MU-MIMO scenario with rich channel environment (CDL) employing ZF precoding with Type II CSI feedback provides the Gnb with sufficient information to schedule multiples Ues close to each other with high MCS and rank. Observation 3: Type I CSI feedback does not provide sufficient information for the ZF algorithm to correctly calculate the most suitable precoders to achieve FRC maximum throughput. Observation 4: Zero-forcing algorithm is needed to properly cancel out interference in between the two scheduled Ues. Proposal: Configure paramCombination-r16 = 6, R = 1. Proposal: Use same parameters as Rel-15 Type II MU-MIMO PMI test case design where applicable. |
| R4-2009614 | Apple | Observation 1: For link level assessment, no performance improvement would be observed with MU-MIMO compared to SU-MIMO test setup Observation 2: There is no impact on UE PMI reporting based on no knowledge of co-scheduled UE and baseline receiver as MMSE-IRC with MU-MIMO setup compared to SU-MIMO Observation 3: MU-MIMO setup is more complicated compared to SU-MIMO, involving aligning scheduling mode with co-scheduled UE. Proposal 1: Use SU-MIMO test setup for requirements for PMI reporting with enhanced Type II codebook. Proposal #2: For enhanced Type II codebook, introduce requirements with SU-MIMO test setup with the following assumptions: - ParamCombination-r16: 6, with L =4, pν =1/2, β=1/2. - Codebook parameter configuration, paramCombination-r16: 6 - Number of PMI sub-bands per CQI sub-band, R:1 - Channel Model: TDLA 30-50Hz - Antenna correlation: XP-Medium - Subband Size: 4 |
| R4-2009858 | Nokia | Observation 1: eType II PMI is primarily intended to enhance MU-MIMO throughput by providing a much more accurate representation of the strongest channel eigenvectors than Type I SP PMI. This allows the Gnb to steer the beams of co-schedule Ues in each other’s null space with less residual interference. Observation 2: SU-MIMO throughput is less sensitive than MU-MIMO to PMI inaccuracies because MU-MIMO throughput is limited by interference between co-scheduled Ues. Hence, a DUT could pass an SU-MIMO test for eType II without fulfilling the eType II PMI requirements, because the throughput difference between Type I SP and eType II is not large enough for SU-MIMO transmission. Observation 3: In the MU-MIMO test case, with both Option 1 and 2 for the co-scheduled UE channel generation, Xb, the ZF precoder calculation can be simplified as W=WaXbH where Wa is the orthogonalized and normalised Wa and XbH is the normalized projection of the co-scheduled PMI on the null space of Wa XbH=I-WaWaHWb diagq-1/2 Proposal 1: RAN4 to select Option 2 (MU-MIMO) as test case for eType II PMI reporting as it is the only test setup that can guarantee appropriate UE PMI reporting requirements for MU-MIMO, given a suitable test metric. Proposal 2: RAN4 to adopt ZF precoder as precoder calculation for the MU-MIMO test case. If complexity is an issue for TE vendors, the precoder calculation can be simplified by applying the ZF principle of orthogonality only to the co-scheduled UE (details in Observation 3). Proposal 3: RAN4 to adopt throughput ratio between following PMI and random PMI as test metric for the MU-MIMO test case of eType II. Relative throughput between following PMI for Type I SP and eType II can also be considered. |
| R4-2010141 | Samsung | Proposal 1: Introducing R16 Type II codebook PMI requirements with MU-MIMO Set-up only if RAN4 can reach consensus on test feasibility and detailed test set-up for MU-MIMO set-up in RAN4#96e; otherwise, introducing Type II codebook PMI requirements in Rel-16 with SU-MIMO Set-up and further evaluate and introducing Type II codebook PMI requirements with MU-MIMO set-up in future release. Proposal 2-Number of ports: introduce Rel-16 Type II codebook PMI test cases with 16 Tx ports considering test complexity and test coverage. Proposal 3- Number of PMI sub-bands per CQI band: R =1 considering supporting R=2 is a separate optional feature. Proposal 4-codebook parameter: using below configuration - ParamCombination-r16: 6, with L =4, pν =1/2, β=1/2. - Sub-band Size: - 4 for FDD with 15kHz SCS, 10MHz CBW - 8 for TDD with 30kHz SCS, 40MHz CBW Proposal 5-Beam steering: Introduce a generic beam steering model into specification in a future proof manner which the number of beams configurable. Proposal 6-Propagation condition: Introduce test case with MIMO correlation –XP Medium and TDLA30-5 Proposal 7-MCS&Rank: It’s feasible to use MCS20 (64QAM), Rank2 for introducing test cases. |
| R4-2010805 | R&S | Observation 1: Further clarifications on the exact requirements for MU-MIMO based testing are needed to fully judge the test system impact Proposal 1: RAN4 agrees to follow Option 1 and defines requirements with up to 16Tx Ports Proposal 2: RAN4 agrees on Option1 and defines TCs for SU-MIMO Proposal 3: RAN4 agrees on Option 2 and reuses the definition as specified in B.2.3B.4A of TS 36.101. |
| R4-2011013 | Huawei, HiSilicon | Observation 1: For SU-MIMO test setup, the performance of Follow PMI for Type II has an obvious gain over Follow PMI for Type I single panel Proposal 1: Using SU-MIMO test setup for Rel-15/Rel-16 Type II codebook PMI reporting test Proposal 2: Consider having a MU-MIMO setup based demodulation test with test metric of either follow PMI based or random PMI based Throughput Proposal 3: Take either option 1 or 2 for Sub-band Size configuration for SU-MIMO Proposal 4: Introduce beam steering model and specify using generic number of beams Proposal 5: Take either XP High or XP Medium but slightly prefer XP Medium for MIMO correlation configuration for SU-MIMO |
| R4-2011421 | Qualcomm | Proposal 7: Define PMI reporting test cases for Enhanced Type II codebook under similar assumptions as that of the test cases for Rel-15 Type II Codebook. Observation 1: UE reports the same precoder for both SU-MIMO and MU-MIMO test setups. Proposal 8: Use SU-MIMO test setup for defining Enhanced Type II PMI reporting tests. Proposal 9: Do not extend the beam steering model beyond 2 clusters and reuse the dual cluster beam steering defined in 36.101. Proposal 10: Define enhanced Type II PMI reporting tests only for 16 Tx ports. Proposal 11: Use smaller sub-band size, i.e., 4 for FDD 10MHz and 8 for TDD 40MHz, for defining PMI reporting tests for enhanced Type II codebook. Proposal 12: Use XP High correlation to define PMI reporting requirements for enhanced Type II codebook. Proposal 13: Use R = 1 in PMI reporting requirements for enhanced Type II codebook. |

Open issues summary

Last RAN4 meeting agreements captured in WF R4-2008816 which also summarized in Annex.

List of open issues:

- Sub-Topic 2-1: MU-MIMO scheduling model

  - Issue 2-1-1: Transmitted signal modelling

  - Issue 2-1-2: Precoder generation in TE for DUT and co-scheduled Ues

  - Issue 2-1-3: Beam steering model

- Sub-Topic 2-2: Test set-up SU-MIMO VS. MU-MIMO

  - Issue 2-2-1: SU-MIMO VS MU-MIMO Setup

- Sub-Topic 2-3: Test parameters for SU-MIMO option

  - Issue 2-3-1: Number of ports

  - Issue 2-3-2: Number of PMI Sub-bands per CQI Sub-band

  - Issue 2-3-3: paramCombination-r16

  - Issue 2-3-4: Sub-band Size

  - Issue 2-3-5: Beam steering model: how to specify beam steering model in to specification

  - Issue 2-3-6: Channel Model

  - Issue 2-3-7: MIMO Correlation

  - Issue 2-3-8: MCS and Rank

- Sub-Topic 2-4: Test parameters for MU-MIMO option

  - Issue 2-4-1: Number of ports (MU-MIMO only)

  - Issue 2-4-2: Number of PMI Sub-bands per CQI Sub-band

  - Issue 2-4-3: paramCombination-r16

  - Issue 2-4-4: Sub-band Size

  - Issue 2-4-5: Channel Model

  - Issue 2-4-6: MIMO Correlation

  - Issue 2-4-7: MCS and Rank

  - Issue 2-4-8: Test metric

Sub-topic 2-1: MU-MIMO scheduling model

Issue 2-1-1: Transmitted signal modeling

- Proposals

  - Option 1 : y1=h1w1x1+h1w2x2+n1 (Qualcomm, Ericsson ,Nokia, Samsung)

- Recommended WF

  - FFS

Issue 2-1-2: Precoder generation in TE for DUT and co-scheduled Ues

- Proposals

  - Option 1: Using ZF Precoder for DUT and co-scheduled UE (Ericsson, Samsung,Nokia)

- W = XH*(X*XH + Λi)-1, where - X = [Xa; Xb], is the DL channel estimate for the two co-scheduled Ues, where - Xa is the channel estimated for DUT i.e., Xa= WaH from reported type II PMI - Xb is the channel of co-scheduled UE - FFS whether Xb= WbH (from a PMI corresponding to the artificial UE), or if Xb is actual generated channel Hb. - W = [Wa Wb] is the resulting precoder for the two co-scheduled Ues - Wa is the DUT precoder - Wb is the co-scheduled UE precoder

  - Option 2: Using ZF to generate co-schedule UE precoder (Nokia, Ericsson, Samsung)

W=WaXbH where Wa is the orthogonalized and normalised Wa and XbH is the normalized projection of the co-scheduled PMI on the null space of Wa XbH=I-WaWaHWb diagq-1/2

  - Option 3: (Xa, Xb) = (PMIa, fixed PMIb) , fixed PMIb could be generated once and be kept fixed throughout the simulation (Huawei)

- Recommended WF

  - N.A

Issue 2-1-3: Beam steering model

- Proposals

  - Option 1: No beam steering model applied in MU-MIMO test set-up (Ericsson, Nokia)

- Recommended WF

  - N.A

Sub-topic 2-2: Test setup (SU-MIMO vs MU-MIMO)

Issue 2-2-1: SU-MIMO VS MU-MIMO Setup

- Proposals

  - Option 1: SU-MIMO Set-up (Apple, R&S, Huawei, Qualcomm)

  - Option 2: MU-MIMO Set-up (Ericsson, Nokia, Samsung,  Vodafone, Orange)

  - Option 3: Using SU-MIMO set-up to Introduce PMI test cases meanwhile  a MU-MIMO setup based demodulation test with test metric of either follow PMI based or random PMI based throughput can be introduced (Huawei)

- Recommended WF

  - Further discuss the details of MU-MIMO scheduling , including how to generate Precoder  in TE side, and how to get the channel of co-scheduled UE; how to generate channel model for co-scheduled UE; what’s beam steering assumption;

  - Also TE venders feedback are encouraged for the test feasibility especially for Tx signal generation with precoder for DUT and generated UE

Sub-topic 2-3: Test parameters for SU-MIMO option

Issue 2-3-1: Number of ports

- Proposals

  - Option 1: 16 ports with (N1,N2) = (4,2) and (O1,O2)=(4,4) (Samsung, Apple, R&S, Qualcomm, Huawei)

- Recommended WF

  - Option 1, introduce requirements with 16 ports

Issue 2-3-2: Number of PMI Sub-bands per CQI Sub-band

- Proposals

  - Option 1: R=1 (Samsung, Apple, Qualcomm, Huawei, Ericsson )

- Recommended WF

  - Agree Option 1, introduce requirements with R=1

Issue 2-3-3: paramCombination-r16

- Proposals

  - Option 1: paramCombination-r16: 6, with L =4, pν =1/2, β=1/2 (Samsung, Apple, Ericsson, Qualcomm. Huawei )

- Recommended WF

  - Agree option 1

Issue 2-3-4: Sub-band Size

- Proposals

  - Option 1: (Samsung, Apple, Huawei, Qualcomm, Ericsson)

- 4 for FDD with 15kHz SCS, 10MHz CBW

- 8 for TDD with 30kHz SCS, 40MHz CBW

  - Option 2: (Huawei)

- 8 for FDD with 15kHz SCS, 10MHz CBW

- 16 for TDD with 30kHz SCS, 40MHz CBW

- Recommended WF

  - Agreed option 1.

Issue 2-3-5: Beam steering model: how to specify beam steering model in to specification

- Proposals

  - Option 1: Same as specified in B.2.3B.4A of TS 36.101 (Qualcomm, R&S, Apple)

  - Option 2: Specify using generic number of beams (Huawei, Samsung)

- Recommended WF

  - Introduce beam steering model into specification with configurable number of beams in a future proof manner (i.e. L can configured as 1, 2 or >2)

Issue 2-3-6: Channel Model

- Proposals

  - Option 1: TDLA30-5 (Apple, Qualcomm, Huawei, Samsung)

- Recommended WF

  - Agreed option 1. 4 companies provide the initial results based on TDLA30-5.  One company shows better performance can be achieved with TDLA30-5 channel model compared with TDLC300-5 channel model.

Issue 2-3-7: MIMO Correlation

- Proposals

  - Option 1: XP High (Huawei, Qualcomm)

  - Option 2: XP Medium (Samsung, Apple, Huawei, Qualcomm)

- Recommended WF

  - Agreed option 2. 4 companies provide the initial results, 3 companies results show that with XP medium can achieve better performance compared with XP High under TDLA30-5 channel model. 1 company result shows XP High correlation provides better throughput ratio compared to XP Medium correlation with TDLA30-10 channel model.

Issue 2-3-8: MCS and Rank

- Proposals

  - Option 1: MCS 20 (64QAM Table), Rank 2 (Samsung, Apple, Huawei, Qualcomm)

- Recommended WF

  - Agreed option 1.

Sub-topic 2-4: Test parameters for MU-MIMO option

Issue 2-4-1: Number of ports

- Proposals

  - Option 1: 32 ports with (N1,N2) = (4,4) and (O1,O2)=(4,4) (Ericsson, Nokia)

  - Option 2: 16 ports with (N1,N2) = (4,2) and (O1,O2)=(4,4) (Samsung, Ericsson, Nokia, Huawei)

- Recommended WF

  - FFS

Issue 2-4-2: Number of PMI Sub-bands per CQI Sub-band

- Proposals

  - Option 1: R=1 (Samsung, Ericsson, Nokia, Huawei)

- Recommended WF

  - Agree Option 1, introduce requirements with R=1

Issue 2-4-3: paramCombination-r16

- Proposals

  - Option 1: paramCombination-r16: 6, with L =4, pν =1/2, β=1/2 (Samsung, Apple, Ericsson, Nokia, Huawei )

- Recommended WF

  - Agree option 1

Issue 2-4-4: Sub-band Size

- Proposals

  - Option 1: (Samsung, Apple?, Huawei, Qualcomm, Ericsson, Nokia)

- 4 for FDD with 15kHz SCS, 10MHz CBW

- 8 for TDD with 30kHz SCS, 40MHz CBW

  - Option 2: (Huawei)

- 8 for FDD with 15kHz SCS, 10MHz CBW

- 16 for TDD with 30kHz SCS, 40MHz CBW

- Recommended WF

  - Agree option 1

Issue 2-4-5: Channel Model

- Proposals

  - Option 1: TDLC300-5 (Ericsson, Nokia)

  - Option 2: TDLA30-5 (Samsung)

  - Other options are not precluded

- Recommended WF:

  - Note: the details proposal of option 1 is provided in Tdoc R4-2011365, which belong Agenda 7.16.1.2

  - Companies’ feedback needed for above proposal, also pending on the issue of 2-1-1

Issue 2-4-6: MIMO Correlation

- Proposals

  - Option 1: XP High (Huawei, Qualcomm)

  - Option 2: XP Medium (Samsung, Apple, Huawei, Nokia)

- Recommended WF:

  - Agreed option 2?

Issue 2-4-7: MCS and Rank

- Proposals

  - Option 1: MCS 7 (64QAM Table), Rank 1 (Ericsson, Nokia)

  - Option 2 other options not preclude (Samsung, Huawei)

- Recommended WF:

  - Companies’ feedback needed for above proposal

Issue 2-4-8: Test metric

- Proposals

  - Option 1: The ratio of following PMI and random PMI (Nokia, Samsung)

  - Option 2 :  The ratio of following PMI with following PMI with enhanced type II codebook and type I single panel codebook (Ericsson, Nokia, Samsung, Huawei)

- Recommended WF:

  - Companies’ feedback needed for above proposals

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub-Topic 2-1: MU-MIMO scheduling model - Issue 2-1-1: Transmitted signal modeling - Issue 2-1-2: Precoder generation in TE for DUT and co-scheduled Ues - Issue 2-1-3: Beam steering model Sub-Topic 2-2: Test set-up SU-MIMO VS. MU-MIMO - Issue 2-2-1: SU-MIMO VS MU-MIMO Setup Sub-Topic 2-3: Test parameters for SU-MIMO option - Issue 2-3-1: Number of ports - Issue 2-3-2: Number of PMI Sub-bands per CQI Sub-band - Issue 2-3-3: paramCombination-r16 - Issue 2-3-4: Sub-band Size - Issue 2-3-5: Beam steering model: how to specify beam steering model in to specification - Issue 2-3-6: Channel Model - Issue 2-3-7: MIMO Correlation - Issue 2-3-8: MCS and Rank Sub-Topic 2-4: Test parameters for MU-MIMO option - Issue 2-4-1: Number of ports - Issue 2-4-2: Number of PMI Sub-bands per CQI Sub-band - Issue 2-4-3: paramCombination-r16 - Issue 2-4-4: Sub-band Size - Issue 2-4-5: Channel Model - Issue 2-4-6: MIMO Correlation - Issue 2-4-7: MCS and Rank - Issue 2-4-8: Test metric |
| Rohde & Schwarz | Issue 2-1-2: Precoder generation in TE for DUT and co-scheduled Ues From our current understanding, the proposed options to generate precoding based on channel conditions are not something that we can see as feasible TE implementations. This is also one of the the reasons, why we propose in our paper to utilize SU-MIMO testing for now. Issue 2-3-5: Beam steering model: We still think we should reuse the existing LTE model, due to the increased complexity an open ended model brings to TE implementation. But we can further discuss. |
| Qualcomm | Sub-Topic 2-2: Test set-up SU-MIMO VS. MU-MIMO - Issue 2-2-1: SU-MIMO VS MU-MIMO Setup Based on analysis provided in our paper, the reported PMI by UE will not change based on the test setup. There are also concerns from TE vendors. Therefore, we prefer to use SU-MIMO test setup. Sub-Topic 2-3: Test parameters for SU-MIMO option - Issue 2-3-1: Number of ports Ok with recommended WF. - Issue 2-3-2: Number of PMI Sub-bands per CQI Sub-band Ok with recommended WF. - Issue 2-3-3: paramCombination-r16 Ok with recommended WF. - Issue 2-3-4: Sub-band Size Ok with recommended WF. - Issue 2-3-5: Beam steering model: how to specify beam steering model in to specification Prefer Option 1. As we have already agreed to configure only 2 beams, there is no incentive to extend the beam steering model to generic number of beams. We can reuse the LTE model which has two independent beams. Otherwise, it may give an impression to an outsider that it is feasible to test with any number of beams while that is not the case. Also, it is not practical to assume that UE can receive signal from more than 2 independent beams with same power. - Issue 2-3-6: Channel Model Ok with recommended WF. - Issue 2-3-7: MIMO Correlation Prefer XP High. But ok to compromise to XP Medium. - Issue 2-3-8: MCS and Rank Ok with recommended WF. Sub-Topic 2-4: Test parameters for MU-MIMO option Prefer to discuss after the decision on Issue 2-2-1. |
| Apple | Sub-Topic 2-1: MU-MIMO scheduling model - Issue 2-1-1: Transmitted signal omplexit - Issue 2-1-2: Precoder generation in TE for DUT and co-scheduled Ues - Issue 2-1-3: Beam steering model No comment on these issues as our preference is to use SU-MIMO test setup. Sub-Topic 2-2: Test set-up SU-MIMO VS. MU-MIMO - Issue 2-2-1: SU-MIMO VS MU-MIMO Setup Option 1. We recommend to define requirements with SU-MIMO test setup. Results shown in Ericsson’s paper with MU-MIMO in CDL channel and the comparison is performance with MU-MIMO set up when Type II and Type I are used. This doesn’t justify that MU-MIMO setup is better to test Type II. Also, the evaluation results from Ericsson with MU-MIMO setup are for lower MCS and Rank 1 which results in much lower TP than the SU-MIMO set up we have been using, so that doesn’t show that MU-MIMO setup can achieve better performance with Type II codebook. Sub-Topic 2-3: Test parameters for SU-MIMO option - Issue 2-3-1: Number of ports We support the recommended WF. - Issue 2-3-2: Number of PMI Sub-bands per CQI Sub-band We support the recommended WF. - Issue 2-3-3: paramCombination-r16 We support the recommended WF. - Issue 2-3-4: Sub-band Size Option 1. We support the recommended WF. - Issue 2-3-5: Beam steering model: how to specify beam steering model in to specification We support option 1. - Issue 2-3-6: Channel Model We support the recommended WF. - Issue 2-3-7: MIMO Correlation We support the recommended WF. - Issue 2-3-8: MCS and Rank We support the recommended WF. |
| Huawei, HiSilicon | Issue 2-1-1: Need more investigations. Issue 2-1-2: For simplicity perspective, another option of (Xa, Xb) = (PMIa, fixed PMIb) can be considered. The fixed PMIb could be generated once and be kept fixed throughout the simulation. Issue 2-2-1: Throughout the discussion for the past several meetings on this issue, these two setup of SU-MIMO and MU-MIMO both have their advantages. For SU-MIMO, testing is simple and more straightforward. For MU-MIMO, testing can take advantage of the rich channel feedback of Type II reporting, and more obvious gain can be observed. We are fine for further discuss on both options. Since there is limited timeslot left for completing this WI, and for moving forward, an alternative option of adding a demodulation test with test metric of either follow PMI based or random PMI based throughput. Companies can consider whether this option can be seen as a way of testing MU-MIMO. Issue 2-3-1~2-3-6: Agree with recommended WF. Issue 2-3-7: OK with recommended WF. Issue 2-3-8: Agree with recommended WF. Issue 2-4-1: Prefer Option 2. Issue 2-4-2~2-4-4: Agree with recommended WF. Issue 2-4-5~2-4-7: Need more investigations. Issue 2-4-8: Prefer option 2, to avoid the potential performance ratio (follow vs random) degradation brought by advanced receiver. |
| Ericsson | Issue 2-1-1: Transmitted signal modeling The signal seen from the DUT would be according to Option 1. Issue 2-1-2: Precoder generation in TE for DUT and co-scheduled Ues We are ok with Option 2 since it is aligned with our proposed setup. We did not include the normalization factor diag(q)^-1/2 in our equation from the previous WF. We did however simulate with this model in mind to normalize the precoders according to option 2. Furthermore, we believe Nokia proposed another simplified model for generating the precoder for the co-scheduled UE using the Gram-Schmidt orthogonalization method which is not captured here in this summary. We think those are the two options that should be listed here. I.e., We support Option 2, but also would like to discuss the other method proposed by Nokia as well. Issue 2-2-1: SU-MIMO VS MU-MIMO Setup We’re proposing an MU-MIMO based test setup based on the intended deployment scenario for Type II codebooks. SU-MIMO has seen marginal to no gains when comparing Type I codebook with Type II codebook. Furthermore, we agree that from a UE perspective there is no difference in how the PMI will be reported irrespective of an SU-MIMO, or MU-MIMO test setup. With these two points in mind this leaves room for a poor UE implementation where a Type I CSI report could be reported with Type II format and still pass RAN4 testing, whereas in a MU-MIMO test case we’ve shown in our simulations that proper Type II CSI reporting is needed to achieve maximum theoretical throughput. The SU-MIMO test furthermore does not fulfil the purpose of ensuring that the channel state information reported to the BS enables selection of orthogonal MU-MIMO beam parings. Without a proper test, the network will not know whether UE implementations vary and will not be able to ensure that MU-MIMO orthogonality is achievable, and the MU-MIMO feature, which is an important building block of NR would risk to become a paper feature with little ability to provide gain. Justifiably we think RAN4 should implement test setups which would share similarities for the intended use case. In this case Type II PMI codebook are inherently designed for an MU-MIMO operation in a live network. Thus, it does not make sense to simply reuse the test method from SP Type I requirements since the two codebooks are designed for different operation. Issue 2-4-1: Number of ports We think that supporting Option 1 makes more sense since this would implicitly verify that a correct implementation of Type II codebook is supported for 16Tx ports as well. However, we are open to discuss the feasibility of testing with different number of Tx ports based on input from TE vendors. Issue 2-4-2, 2-4-3, 2-4-4: Agree with recommended WF. Issue 2-4-5: Channel Model We prefer a channel model with a large delay spread to get frequency selectivity across the subband size. In RAN4 we have defined TDLC with 300ns delay spread which is the largest delay spread currently defined. Issue 2-4-6: MIMO Correlation We support option 2 given that medium correlation has shown more benefit in simulations. Issue 2-4-8: Test metric For the purpose of defining performance requirements we support Option 2 comparing Type II with Type I. Type II codebook was developed to extend the MU-MIMO support of NR. Employing a Random PMI does not make sense since this will never be a precoding scheme employed at the Gnb side. Thus, for benchmarking purposes the logical reference performance would be Type I codebook with a similar configuration. |
| Vodafone | Issue 2-2-1: SU-MIMO VS MU-MIMO Setup Option 2. Still agree with Ericsson’s views here. Our concern is that SU-MIMO throughput is not a validation of successful type II reporting. It confirms a CSI feedback mechanism is working, but not that type II CSI of a sufficient quality has been received. This leaves the door open for poor UE implementation that could cripple MU-MIMO in deployed networks. |
| Orange | Sub-Topic 2-2: Test set-up SU-MIMO VS. MU-MIMO - Issue 2-2-1: SU-MIMO VS MU-MIMO Setup We support Option 2 in order to ensure the right performance for MU-MIMO with codebook type II, i.e., to ensure that the UE feedback is the closest to the eigenvectors of the MIMO channel. |
| Samsung | Sub-Topic 2-1: MU-MIMO scheduling model - Issue 2-1-1: Transmitted signal modelling Generally, we are fine with this modelling. It is the common understanding for MU-MIMO signal modelling for data part. While the details of RS configuration should be further clarified, such as whether CRS-RS/TRS colliding for two co-located UE will be considered, whether NZP CSI-RS, IMR should be configured? - Issue 2-1-2: Precoder generation in TE for DUT and co-scheduled Ues In our understanding, either option 1 or option 2 feasible for MU-MIMO setup, we think the impact on the UE receiver processing is minor. While the details for precoder generation should be clarified? For option 1: how to decide the regularization factor? The motivation of adding regularization factor is to guarantee matrix invertible, the setting should be based on the condition number of matrix, whether applied the same value of regularization factor for the different strategies for channel generation for co-located UE, such as PMI corresponding to the artificial UE or actual generated channel In option 2: we agree the interference of coming from co-located can be null with Gram-Schmid method, which is benefit the TP achieved by DUT UE, From the test UE, it is fine.while for co-located UE, the interference coming from DUT is still existed, How can guarantee the whole MU-MIMO performance achieved by these two Ues, if these two Ues are beam pairing in the real network? In terms of test applicability , we need the confirmation from TE vendor about the feasibility of Tx signal generation with precoder for DUT and channel model for co-located UE - Issue 2-1-3: Beam steering model We think more clarification is needed. With MU-MIMO setup, if there is no beam steering model for DUT and co-located UE, how can guarantee the different beam directions for each UE? Sub-Topic 2-2: Test set-up SU-MIMO VS. MU-MIMO - Issue 2-2-1: SU-MIMO VS MU-MIMO Setup We are fine with MU-MIMO setup. Rel-16 type II codebook can provide more accurate representation of actual channel, Gnb can achieve better separation, so it can provide more benefit in MU-MIMO operations compared with type I codebook. With SU-MIMO setup, it is not close to the real network scenario of scheduling type II codebook, maybe not reflect the advantage of type II codebook over than Type I codebook under multi-user co-scheduling scenario. Therefore, it make sense to consider the MU-MIMO setup for type II codebook To achieve comparable performance of MU-MIMO in real network, the test parameters should be properly design, such as scheduling, MU-MIMO beam pairing, UE receiver type. Based on the test procedure of MU-MIMO setup, we do see some details set-up is unclear, it needs to be further clarified to make it feasible. Regarding the UE receiver type, as agreed in last RAN4 meeting, the baseline receiver assumption is UE without interference cancellation capability with/without co-scheduled UE. What is the baseline receiver assumption , MMSE or MMSE-IRC？ In case of MMSE receiver, what is the practical noise/interference estimation assumption? In case of MMSE IRC, how can DUT aware the interference of co-located UE? If not, what is the interference/noise estimation assumption? Meanwhile, the feasibility of MU-MIMO, related with PMI calculation and generation for DUT and co-located UE, the channel model generation for co-located UE should be confirmed from TE vendor. On the other hand, no matter SU-MIMO test sup or MU-MIMO setup, the purpose is to verify the gain of type II codebook compared with type I codebook Based on our initial results, we do see some gain of Rel-16 type II codebook, around 4Db SNR gain can be achieved with FDD 16X2 XP medium condition under SU-MIMO setup. In that sense, type II is also feasible for SU-MIMO test up. Meanwhile, from the test feasibility aspect, it has been widely used in existing LTE and NR PMI test cases, the feasibility already been verified. So, the poor UE implementation can be avoided by meeting the requirement of type II codebook Sub-Topic 2-3: Test parameters for SU-MIMO option - Issue 2-3-1: Number of ports We are fine with recommend WF. - Issue 2-3-2: Number of PMI Sub-bands per CQI Sub-band We are fine with recommended WF. - Issue 2-3-3: paramCombination-r16 We are fine with recommended WF. - Issue 2-3-4: Sub-band Size We are fine with recommended WF. - Issue 2-3-5: Beam steering model: how to specify beam steering model in to specification We are fine with recommended WF and option 2. Although only two beams is configured in beam steering model for Rel-16 type II test cases, from future proof manner, we think it is more proper to use a generic beam steering approach, which can be applied for PMI test cases with different codebook types i.e. single beam direction (Type I codebook), dual beam directions (Rel-15 Type II codebook) and multi-beam directions (Rel-16 Type II codebook). We think there is no impact on current test for type II codebook. - Issue 2-3-6: Channel Model We are fine with recommended WF. - Issue 2-3-7: MIMO Correlation We are fine with recommended WF and option 2. Based on results, with XP medium can achieve better performance compared with XP high condition. - Issue 2-3-8: MCS and Rank We are fine with recommended WF. Sub-Topic 2-4: Test parameters for MU-MIMO option - Issue 2-4-1: Number of ports We prefer option 2. NR Rel-15 Type II codebook and Rel-16 advanced Type II codebook are the extension of LTE Rel-14 advanced codebook which they share similar codebook structure. It’s better we can align the number of Tx ports with PMI test case of LTE Efd-MIMO advanced codebook to provide comparable performance. The test complexity especially the number of required individual MIMO channel faders also needs to be considered With MU-MIMO test setup, additional independent MIMO faders should be also generated for co-located UE, The test complexity will be further increased. The purpose is to verify the UE PMI feedback behavior, we think 16 Tx port should be enough to show the gain of type II. Also 32 Tx port requirement has already introduced in type I codebook, the test coverage for different number of Tx port can be fulfilled. Meanwhile, in order to verify the type II codebook One candidate test metric is considered with ratio of following PM with enhanced type II codebook and type I single panel codebook. Both 16 Tx and 32 Tx ports requirement are introduced for type I single panel codebook, where 16 Tx port is defined for sub-band PMI, and 32Tx port is defined for wideband PMI. Since only sub-band PMI is supported for Rel-16 enhanced type II, if we apply with test metric of following PMI with enhanced type II codebook and type I single panel codebook, in that sense, only 16 Tx port can be available. - Issue 2-4-2: Number of PMI Sub-bands per CQI Sub-band We are fine with recommend WF. Since R=2 is a UE optional feature. - Issue 2-4-3: paramCombination-r16 We are fine with recommend WF. - Issue 2-4-4: Sub-band Size We are fine with recommend WF and option 1. - Issue 2-4-5: Channel Model We propose with TDLA30-5Hz, following the assumption of Type I codebook requirement Issue 2-4-6: MIMO Correlation We prefer option 2 - Issue 2-4-7: MCS and Rank We are fine with option1 with Rank1, while the MCS, we suggest with high MCS, which is similar assumption with type I codebook requirement. The current MCS is too low, it cannot guarantee the benefit of MU-MIMO compared with SU-MIMO test up. |
| Nokia/NSB | Sub-Topic 2-1: MU-MIMO scheduling model - Issue 2-1-1: Transmitted signal omplexit Option 1 is a correct model for the signal seen by the DUT with 1 rx antenna and rank-1 precoder for both the DUT and co-scheduled UE - Issue 2-1-2: Precoder generation in TE for DUT and co-scheduled Ues Option 1 is the ideal option, we also support Option 2 as a way of simplifying the test. As pointed out by Ericsson, both Option 1 and 2 are aligned in the intention of generating a ZF precoder for the co-scheduled UE, so we support both options. Option 2 is intended to simplify the ZF precoder implementation by observing that we may not need to steer the precoder for the DUT in the null space of the co-scheduled UE because we do not measure the throughput of the co-scheduled UE. This simplification would avoid the matrix inversion. We are also open to consider a further simplification proposed by Huawei in their comments on this issue, i.e., in fixing the PMI of the co-scheduled UE throughout the simulation, provided that the precoder of the co-scheduled UE is maintained orthogonal to the PMI reported by the DUT. - Issue 2-1-3: Beam steering model We support Option 1 Sub-Topic 2-2: Test set-up SU-MIMO VS. MU-MIMO - Issue 2-2-1: SU-MIMO VS MU-MIMO Setup We support Option 2. Rel-16 Type II codebooks were designed by RAN1 specifically to enhance support for MU-MIMO operations. Evaluation methodology and performance metrics for the design of Rel-16 Type II assumed MU-MIMO in RAN1 so we think it is not possible to test performance requirements in RAN4 with an SU-MIMO test. In Option 1, there is no cross-layer interference from co-scheduled Ues, good beamforming gain can be achieved with Type I, and there is not big margin of improvement with eType II. However, in Option 2, which is an interference-limited MU-MIMO test, accurate representation of the channel eigenvectors is crucial and eType II significantly outperforms Type I. Therefore, the performance gain of eType II relative to Type I can only be verified with Option 2. Although the PMI calculation and demodulation at the DUT is the same in SU- or MU-MIMO setup, there is a significant difference in throughput at the DUT between the two setups because of the presence of co-scheduled Ues. In our view, this test should be designed to guarantee performance in the latter case, i.e. with a co-scheduled UE. We also suggested ways to simplify the MU-MIMO test to ease the TE implementation (see comments on subtopic 2-1-2). Sub-Topic 2-4: Test parameters for MU-MIMO option - Issue 2-4-1: Number of ports Fine prefer Option 2 (16 ports) but are ok with Option 1 (32 ports) - Issue 2-4-2, 2-4-3, 2-4-4 Agree with recommended WF - Issue 2-4-5: Channel Model Option 1 is ok - Issue 2-4-6: MIMO Correlation Preference for Option 2 - Issue 2-4-7: MCS and Rank Option 1 is ok - Issue 2-4-8: Test metric Initially we supported both Options but we agree with Ericsson’s argument that Option 2 is a more accurate metric for this test case. |

CRs/TPs comments collection

Major close to finalize Wis and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going Wis, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |
| Sub-topic 2-1: | Issue 2-1-1: Transmitted signal modeling GTW Agreement on Thursday 20200820 Agreement: Taking Option 1 (this mode only assumes Rank1 transmission) as the assumption for MU-MIMO set-up - it’s not exclude to further extend the model which can be applied for rank2 transmission if needed Further discuss for the details, NZP-CSI configuration/DMRS configuration assumption Recommendations for 2nd round: - Further discuss the details of details, NZP-CSI configuration/DMRS configuration assumption based on GTW agreement Issue 2-1-2: Precoder generation in TE for DUT and co-scheduled Ues GTW Agreement on Thursday 20200820 Irrespective of SU-MI MO test set-up or MU-MIMO set-up: The test purpose of such requirements was to verify UE PMI reporting accuracy following NW configuration with RAN1 feature: enhanced type II codebook - There is no restriction for Gnb scheduling with such requirements. - RAN4 need to ensure UE reporting PMI follow Type II codebook other than Type I codebook under proper test set-up either with MU-MIMO set-up or SU-MIMO set-up. - We need to ensure the performance requirements with proper test set-up as receiver implementation agonistic manner i.e. no pushniment for advanced receiver with inference cancellation capability. Agreement: FFS check the feasibility and details among below candidate options for precoder generation in TE - Option 1( further provide the details from proponent company E///) - Option2 How to generate Xb (channel for co-scheduled UE) - Xb is the channel of co-scheduled UE - Option 1: PMI reporting from co-scheduled UE - Option 2: Random PMI - Option 3: Fixed PMI - Option 4: True Channel We will comeback during 2nd round for above open issues and SU-MIMO vs MU-MIMO set-up. Proponents for SU-MIMO set-up and MU-MIMO set-up need to clarify how to meet the agreed test purpose. Recommendations for 2nd round: - Further discuss the details of procoder generation based on GTW agreement Issue 2-1-3: Beam steering model Candidate options - Option 1: No beam steering model applied in MU-MIMO test set-up (Ericsson, Nokia) - Option 2: Yes Recommendations for 2nd round: - Companies’ feedback needed for about proposals, Proponents of option1 should be further clarified the motivation of not introduce beam steering model, which is different with existed PMI test cases |
| Sub-topic 2-2: | Issue 2-2-1: SU-MIMO VS MU-MIMO Setup Candidate options - Option 1: SU-MIMO Set-up (Apple, R&S, Huawei, Qualcomm) - Option 2: MU-MIMO Set-up (Ericsson, Nokia, Samsung, Orange , Vodafone) - Option 3: Using SU-MIMO set-up to Introduce PMI test cases meanwhile a MU-MIMO setup based demodulation test with test metric of either follow PMI based or random PMI based throughput can be introduced (Huawei) Recommendations for 2nd round: - Further discuss the details of MU-MIMO scheduling , including how to generate Precoder in TE side, and how to get the channel of co-scheduled UE; how to generate channel model for co-scheduled UE; what’s beam steering assumption; - Also TE venders feedback are encouraged for the test feasibility especially for Tx signal generation with precoder for DUT and generated UE |
| Sub-topic 2-3 | Issue 2-3-1: Number of ports Tentative agreements: - Option 1: 16 ports with (N1,N2) = (4,2) and (O1,O2)=(4,4) Recommendations for 2nd round: N.A Issue 2-3-2: Number of PMI Sub-bands per CQI Sub-band Tentative agreements: - Option 1: R=1 Recommendations for 2nd round: N.A Issue 2-3-3: paramCombination-r16 Tentative agreements: - Option 1: paramCombination-r16: 6, with L =4, pν =1/2, β=1/2 Recommendations for 2nd round: N.A Issue 2-3-4: Sub-band Size Tentative agreements: Option 1: - 4 for FDD with 15kHz SCS, 10MHz CBW - 8 for TDD with 30kHz SCS, 40MHz CBW Recommendations for 2nd round: N.A Issue 2-3-5: Beam steering model: how to specify beam steering model in to specification Candidate options: - Option 1: Same as specified in B.2.3B.4A of TS 36.101 (Qualcomm, R&S ,Apple) - Option 2: Specify using generic number of beams (Huawei, Samsung) Recommendations for 2nd round: - Considering potential test complexity of TE with an open ended model, whether proponents of option2 can compromise option 1? Issue 2-3-6: Channel Model Tentative agreements: - Option 1: TDLA30-5 Recommendations for 2nd round: N.A Issue 2-3-7: MIMO Correlation Tentative agreements: - Option 2: XP Medium Recommendations for 2nd round: N.A Issue 2-3-8: MCS and Rank Tentative agreements: - Option 1: MCS 20 (64QAM Table), Rank 2 Recommendations for 2nd round: N.A |
| Sub-topic 2-4 | Issue 2-4-1: Number of ports Status: companies prefer 16 ports Tx , 2 companies also fine with option 1, and open to discuss the feasibility of testing with different number of Tx ports based on input from TE vendors Candidate options: - Option 1: 32 ports with (N1,N2) = (4,4) and (O1,O2)=(4,4) (Ericsson, Nokia) - Option 2: 16 ports with (N1,N2) = (4,2) and (O1,O2)=(4,4) (Samsung, Nokia, Huawei, Ericsson, R&S) Recommendations for 2nd round: - Based on test complexity of TE, can proponents of option1 compromise 16Tx Issue 2-4-2: Number of PMI Sub-bands per CQI Sub-band Tentative agreements: - Option 1: R=1 Recommendations for 2nd round: N.A Issue 2-4-3: paramCombination-r16 Tentative agreements: - Option 1: paramCombination-r16: 6, with L =4, pν =1/2, β=1/2 Recommendations for 2nd round: N.A Issue 2-4-4: Sub-band Size Tentative agreements: - 4 for FDD with 15kHz SCS, 10MHz CBW - 8 for TDD with 30kHz SCS, 40MHz CBW Recommendations for 2nd round: N.A Issue 2-4-5: Channel Model Status: 3 companies prefer to option 1, 1 company prefers reuse the test parameters of SU-MIMO , 1 company needs more time checking Candidate options: - Option 1: TDLC300-5 (Ericsson, Nokia) - Option 2: TDLA30-5 (Samsung) - Other options are not concluded Recommendations for 2nd round: - Companies’ feedback needed for about proposals Issue 2-4-6: MIMO Correlation Status: 3 companies prefer to option 1, 1 company prefers reuse the test parameters of SU-MIMO , 1 company needs more time checking Candidate options: - Option 1: XP Medium (Samsung, Ericsson, Nokia) - Other options are not precude Recommendations for 2nd round: - Companies’ feedback needed for about proposals Issue 2-4-7: MCS and Rank Status: Companies agree option1, 2 companies think MCS and rank is too low, which will result in lower TP, cannot guarantee MU-MIMO setup can achieve better performance with type II codebook Candidate options: - Option 1: MCS 7 (64QAM Table), Rank 1 (Ericsson) - Option 2: Other options are not precluded Recommendations for 2nd round: - Companies’ feedback needed for about proposals Issue 2-4-8: Test metric Candidate options: - Option 1: The ratio of following PMI and random PMI (Nokia, Samsung) - Option 2 : The ratio of following PMI with following PMI with enhanced type II codebook and type I single panel codebook (Ericsson, Nokia, Huawei) Recommendations for 2nd round: - Companies’ feedback needed for about proposals |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | Way forward on PMI reporting requirement for NR Emimo | Qualcomm, Ericsson |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Issue 2-1-1: Transmitted signal modeling

Further discuss for the details, NZP-CSI configuration/DMRS configuration assumption

Recommendations for 2nd round:

  - Further discuss the details of details, NZP-CSI configuration/DMRS configuration assumption based on GTW agreement

  - Issue 1: DMRS configuration for each UE

  - Issue 2: NZP-CSI configuration for each UE, whether it is colliding or not?

  - Issue 3: Baseline UE receiver, MMSE or MMSE-IRC?

  - Issue 4: Without UE advanced receiver with interference cancellation capability, what is the interference estimation assumption?

Issue 2-1-2: Precoder generation in TE for DUT and co-scheduled Ues

Agreement: FFS check the feasibility and details among below candidate options for precoder generation in TE

- Option 1( further provide the details from proponent company E///)

- Option2

How to generate Xb (channel for co-scheduled UE)

- Xb is the channel of co-scheduled UE

  - Option 1: PMI reporting from co-scheduled UE

  - Option 2: Random PMI

  - Option 3: Fixed PMI

  - Option 4: True Channel

We will comeback during 2nd round for above open issues and SU-MIMO vs MU-MIMO set-up.

Proponents for SU-MIMO set-up and MU-MIMO set-up need to clarify how to meet the agreed test purpose.

Issue 2-1-3: Beam steering model

Candidate options

  - Option 1: No beam steering model applied in MU-MIMO test set-up (Ericsson, Nokia)

  - Option 2: Yes

Recommendations for 2nd round:

  - Companies’ feedback needed for about proposals, Proponents of option1 should further clarify the motivation of not introduce beam steering model, which is different with existed PMI test cases

Issue 2-2-1: SU-MIMO VS MU-MIMO Setup

Candidate options

  - Option 1: SU-MIMO Set-up (Apple, R&S, Huawei, Qualcomm)

  - Option 2: MU-MIMO Set-up (Ericsson, Nokia, Samsung, Orange , Vodafone)

  - Option 3: Using SU-MIMO set-up to Introduce PMI test cases meanwhile  a MU-MIMO setup based demodulation test with test metric of either follow PMI based or random PMI based throughput can be introduced (Huawei)

Recommendations for 2nd round:

  - Further discuss the details of MU-MIMO scheduling , including how to generate Precoder  in TE side, and how to get the channel of co-scheduled UE; how to generate channel model for co-scheduled UE; what’s beam steering assumption;

  - Also TE venders feedback are encouraged for the test feasibility especially for Tx signal generation with precoder for DUT and generated UE

Issue 2-3-5: Beam steering model: how to specify beam steering model in to specification

Candidate options:

  - Option 1: Same as specified in B.2.3B.4A of TS 36.101 (Qualcomm, R&S ,Apple)

  - Option 2: Specify using generic number of beams (Huawei, Samsung)

Recommendations for 2nd round:

  - Considering potential test complexity of TE with an open ended model, whether proponents of option2 can compromise option 1?

Issue 2-4-1: Number of ports for MU-MIMO setup

Candidate options:

  - Option 1: 32 ports with (N1,N2) = (4,4) and (O1,O2)=(4,4) (Ericsson, Nokia)

  - Option 2: 16 ports with (N1,N2) = (4,2) and (O1,O2)=(4,4) (Samsung, Nokia, Huawei, Ericsson, R&S)

Recommendations for 2nd round:

  - Based on test complexity of TE, can proponents of option1 compromise 16Tx

Issue 2-4-5: Channel Model

Candidate options:

  - Option 1: TDLC300-5 (Ericsson, Nokia)

  - Option 2: TDLA30-5 (Samsung)

  - Other options are not concluded

Recommendations for 2nd round:

  - Companies’ feedback needed for about proposals

Issue 2-4-6: MIMO Correlation

Candidate options:

  - Option 1: XP Medium (Samsung, Ericsson, Nokia)

  - Other options are not precude

Recommendations for 2nd round:

  - Companies’ feedback needed for about proposals

Issue 2-4-7: MCS and Rank

Candidate options:

  - Option 1: MCS 7 (64QAM Table), Rank 1 (Ericsson)

  - Option 2: Other options are not precluded

Recommendations for 2nd round:

  - Companies’ feedback needed for about proposals

Issue 2-4-8: Test metric

Candidate options:

  - Option 1: The ratio of following PMI and random PMI (Nokia, Samsung)

  - Option 2 :  The ratio of following PMI with following PMI with enhanced type II codebook and type I single panel codebook (Ericsson, Nokia, Huawei)

Recommendations for 2nd round:

  - Companies’ feedback needed for about proposals

| Issue | Comments |
| --- | --- |
| Issue 2-1-1: Transmitted signal modeling | Samsung: As discussed in GTW session, this model is valid for PDSCH data channel. In order better to align the assumption, details of RS configuration should be further discussed. What is assumption for DMRS configuration for each UE, for example port {0} for UE1, and port {2} for UE2 under rank1 assumption? If my understanding is correctly , for each UE, there is no interference existed within the DMRS RE Based on previous agreement, the baseline receiver assumption is UE without interference cancellation capability. Therefore, we wonder what is the baseline receiver assumption, MMSE or MMSE-IRC? If the baseline receiver is MMSE, so whether UE can apply the interference estimation based on the adjacent DMRS? If no interference estimation, what is the different UE receiver compared with SU-MIMO setup including both detection and PMI calculation? If the baseline receiver is MMSE-IRC, what is the interference estimation assumption? How can UE aware the interference existed coming from other UE? Network signaling? Meanwhile, whether the IMR is configured? If not, each UE will only relay the CSI-RS for PMI calculation Ericsson: Issue 1: We’ve configured port 1000 for DUT and 1002 for co-scheduled UE with rank 1. For rank 2 we’ve configured port 1000, 1001 for DUT and 1002, 1003 for co-scheduled UE. Issue 2: The CSI-RS from the Gnb in our simulations is Non-colliding since there is only one TRP. Given in this test setup we assume that the UE pairing would be done with two Ues with the same number of CSI-RS ports configured there is only one set of NZP-CSI-RS which is common to both Ues. Issue 3: In our simulations we run with MMSE receiver. There is no different UE receiver assumption from the SU-MIMO test. The point of the test is to validate a correct UE type II implementation, which we don’t think is verifiable in the SU-MIMO test Issue 4: Given that we do not assume interference cancellation receiver, there should not be a difference for a good UE when it comes to passing an SU-MIMO setup compared to an MU-MIMO setup. However, a UE with a bad type II implementation will pass the SU-MIMO setup, but won’t be able to pass the MU-MIMO setup. Since the Gnb will not be able to do fast fading null-forming to the victim UE based on bad CSI feedback. A UE with a poor implementation will not be able to provide rich enough CSI feedback for co-scheduling multiple Ues in an SDM way. The point of the PMI test is not to verify that the UE can do advanced interference cancellation, the point is to verify accurate type II PMI reporting. Nokia/NSB: Issue 1. We are fine with Ericsson’s proposal. Issue 2. CSI-RS configuration is the same as for the SU-MIMO test case for a given number of ports, because the same set of CSI-RS ports are configured for DUT and co-scheduled UE. Hence, there is no collision between CSI-RS. Issue 3. The baseline UE receiver is MMSE, same as for SU-MIMO test. The UE implementation is the same for SU- and MU-MIMO because the network choice between these two MIMO modes is transparent to the Ues. Issue 4. There is no interference estimation assumption at the UE, same as for SU-MIMO test. The purpose of the test is to verify accurate reporting of eType II PMI by the DUT, which can only be achieved by the MU-MIMO setup for which the codebook was designed. This is because it is the higher accuracy of eType II PMI that allows the Gnb to spatially separate co-scheduled Ues. As a consequence, only an MU-MIMO test can achieve a large enough throughput gain against Type I reporting to guarantee a reliable test. Email discussion comment in [96e][320] NR_Emimo_Demod: WF on eType2 PMI Reporting Samsung based on the GTW session agreement, further discuss the details on NZP-CSI configuration and DMRS configuration we suggest to we can add the open list for discussion -NZP-CSI configuration for each UE : colliding or not colliding -IMR configuration? -DMRS configuration for each UE -Rank assumption for each UE -baseline UE receiver: MMSE or MMSE-IRC? -Interference estimation assumption Huawei Based on our understanding, if we configure parameters like DMRS in a non-colliding way, there will be no interference or even difference with SU-MIMO test setup. But if we consider the colliding way, the absolute throughput will be degradation for both UE and the whole system comparing to the SU-MIMO test setup. For the receiver for simulation, as companies agreed in the last meeting, it should be without interference cancellation capability. So MMSE seems the only choice. Regarding to the IMR, it is intended to measure the interference and whiten it so that the channel used for CSI estimation is the whitened channel, which will lead to more precise results for PMI reporting. But since we are agreed not to consider interference cancellation, IMR should not be configured. |
| Issue 2-1-2: Precoder generation in TE for DUT and co-scheduled Ues | Samsung Regarding to how to generate the PMI, there are 4 candidate options listed in 1st round. On top of that, based on the formula of ZF as W = XH*(X*XH + Λi)-1, where regulation factor λ is selected as 0.001. So, where the different PMI generation strategies will apply the same value regulation factor λ, how to choose the proper value of λ? As for details of options, we do see some details need to be aligned Option 1: In case of no IMR configuration, each UE will feedback the PMI calculation based on CSI-RS. For Gnb side, Gnb will select the proper UE for beam pairing based on PMI feedback to MU-MIMO transmission. For each UE, how can UE aware the CSI-RS configuration of co-located UE? Option 2: random PMI, what is the random strategy? Random all the parameters {i11, i12, i2}, or some codebook subset should be selected? Also, How to guarantee the selected PMI has the different main beam direction to limit the interference Option 3: fix PMI, In order to align the simulation assumption, and design the proper test cases, what is the principle to choose the fixed PMI? How to guarantee the selected PMI has the different main beam direction to limit the interference Meanwhile, based on initial result, no big difference on performance for co-scheduled UE Xb generation among option 2 and option 4. With Option 3, the performance degraded. In terms of test ability, we also need the feedback of TE vendor for above options Ericsson: @Samsung, Option 1: Why would the DUT need to be aware of the CSI-RS for the co-scheduled UE? With only one set of NZP-CSI-RS with e.g. 16 ports, each UE will estimate its own channel and feedback a type II CSI report. Based on the Type II feedback the Gnb will calculate appropriate precoders for each UE. For λ value, we’ve simulated with some different options, where 0.001 was found to be a good value, indeed this can be subject to discussion based on which Xb is selected. But for the regularized ZF precoder λ should not be a big value as this would bias the precoder to becoming a matched filter. Option 2: Given the entire set of type II PMI reporting there is a very slim chance that the PMI reported by random PMI would be the same PMI reported from the DUT. Limiting interference in this scenario is not the point. When a completely random PMI is selected the correlation between the DUT PMI and the co-scheduled UE PMI will most likely be quite low which will not exercise the codebook performance of Type II. Thus, this would not be our preferred option for generating the co-scheduled UE PMI because it would be not be a sensitive test. As a consequence, with this method employed we’ve seen that both Type I and Type II will be able to achieve higher theoretical throughput than with the case of Option 1. We may be open to explore options where the main beam direction (wideband PMI) is kept the same for the DUT, and the co-scheduled UE but the subband PMI components would be randomized. Thus the signal h1w2x2 would not be as orthogonal and would better verify a good type II codebook implementation. Option 3: In this setup the TE would generate a single PMI for the co-scheduled UE and store it in memory as precalculated Xb which would then be used each time a new PMI is reported by the DUT, thus consequently used in the calculation for the precoder selected for the DUT and the co-scheduled UE. Option 4: In this setup the genie channel for the co-scheduled UE would be employed, keeping the location fixed for the co-scheduled UE would only need the TE to store Xb in memory and use the same value each time. An option for this scenario could be to drift the location (Xb) of the co-scheduled UE thus giving rise to different interference coming from the h1w2x2 term in the signal received by the DUT. Consequently, challenging the DUT to report PMI which would give better performance. |
| Issue 2-1-3: Beam steering model | Company A Ericsson: Overall, the different Xb options listed above provide different approaches which would cause the DUT PMI to differ for each CSI report sent. This is one of the reasons we propose to not have a beam-steering modeled for the MU-MIMO scenario. Since we’re already challenging the DUT with interference coming from a co-scheduled UE there is no need for additional interference. Nokia/NSB. Agree with Ericsson Email discussion comment in [96e][320] NR_Emimo_Demod: WF on eType2 PMI Reporting Samsung As comment in 1st round, the principle to select the regulation value lamda =0.001 should be further complexit. Whether the value can applied for different precoder generation stragety? As commnent in 2nd round, We do see some omplex should be aligned for MU-MIMO setup -option 1: In case of no IMR configuration, each UE will feedback the PMI calculation based on CSI-RS. For Gnb side, Gnb will select the proper UE for beam pairing based on PMI feedback to MU-MIMO transmission. For each UE, how can UE aware the CSI-RS configuration of co-located UE? -option 2: What is the random PMI stragety, random all parameters , or some codebook subset should be selected, how to guarantee the selected PMI has the different main beam direction compared with DUT -Option 3: In order to align the simulation assumption, and design the proper test cases, what is the principle to choose the fixed PMI,? How to guarantee the selected PMI has the different main beam direction compared with DUT Huawei Option 1 seems not easy in real testing since the co-scheduled UE is not actually a UE. For option 2, random PMI is not a real implementation in Type II codebook. So choosing this option is only for testing purpose. We are slightly prefer option 3, since the PMI for the co-scheduled UE is generated at the very beginning and is kept fixed throughout the whole testing, which is a little bit easier comparing with other options. But anyway, further studies and discussions on how to choose this specific fixed PMI is needed |
| Issue 2-2-1: SU-MIMO VS MU-MIMO Setup | Company A Ericsson: We prefer Option 2. Some clarification on why an SU-MIMO test is not appropriate: The Type II codebook is given by the table below (from TS 38.214). Table 5.2.2.2.3-5: Codebook for 1-layer and 2-layer CSI reporting using antenna ports 3000 to 2999+PCSI‑RS Layers. where , and the mappings from to , , , , , and , and from to , , and are as described above, including the ranges of the constituent indices of and .. The Type II feedback contains reporting amplitudes per each of L>1 DFT beams. Now, if the UE deliberately skip the detailed amplitude reporting and, in the CSI feedback of Type II CSI, is implemented to select only one of the L beams. Hence, the use these beam amplitudes - pl,0(1)=1 and - pl,i(1)=0 for i=1,2,..,L-1, and in addition - pl,i(2)=1 for i=0,1,2,..,L-1, then that means that the UE is using this component in the Type II codebook (assuming the strongest coefficient is in the first polarization, without loss of generality). Wq1,q2,n1,n2,pl(1),pl(2),cll=1N1N2vm1(0),m2(0)vm1(0),m2(0)φl,L, l=1,2 This implies that the rank 1 the codebook is Wq1,q2,n1,n2,p1(1),p1(2),c1(1)=1N1N2vm1(0),m2(0)vm1(0),m2(0)φ1,L, l=1,2 Where a co-phasing of the two 2D DFT beams is performed using the scalar φ1,L. If instead the strongest coefficient were in the second polarization, it would be possible to arrive at a similar expression where a scalar φ1,0 co-phases the two polarizations. This should then be compared with the Type I codebook for rank 1: Wl,m,n(1)=1PCSI-RSvl,mφnvl,m Where the co-phasing φn is taken from a QPSK alphabet. This means, the Type I codebook is a subset of the Type II codebook, the “Type I subset” Some further notes: - The network may configure φ1,L to be taken from a 8-PSK or QPSK constellation. However, the UE may in implementation choose to only select from the QPSK even if configured the 8-PSK constellation (if UE still pass the Type II RAN4 test) - The network may configure subbandAmplitude=TRUE. However, UE may in implementation feedback all subband coefficients equal to one (if UE still pass the Type II SU-MIMO test), which would correspond to Type I reporting. - For eType II reporting: - The UE can similarly feedback only the strongest FD basis component, which would be equivalent to Type I wideband PMI reporting (if UE still pass the Type II SU-MIMO test) It has been shown that the Type I codebook is a subset of the Type II codebook. In addition, in the case of wideband PMI reporting, the Type I codebook is a subset of the eType II codebook. Therefore, if the SU-MIMO test for Type II and eType II doesn’t identify whether a UE is reporting using the “Type I subset” or the full Type II codebook, there is no incentive for the UE to implement type II, the relatively lower complexity Type I is sufficient to implement. Moreover, this means for testing in RAN4, that a UE that is configured with Type II/eType II CSI reporting, but tested only using a SU-MIMO scenario, where the performance of Type I and Type II/eType II is similar, can make the implementation shortcut and always report from the “Type I subset” codebook. Such an implementation shortcut destroys the gNBs ability to properly select precoders for MU-MIMO pairing of multiple Ues during real operation, and thus ultimately, MU-MIMO performance in networks where Type II / eType II reporting is used (e.g. FDD), will be severely reduced/non-existing. Furthermore, some companies may show some performance gains with eTypeII codebook e.g. Samsung showed 4Db gain under ideal conditions. Yet Huawei showed ~2Db gain under similar scenario. From our SU-MIMO ideal simulation we’ve observed ~1Db gain with eTypeII codebook. If we agree to a minimum gain requirement for type II codebooks, there will be implementation margins, on top of the gain shown by companies which means that the performance gain observed under ideal conditions will be eaten up by impairment margins, and beam-steering models which will make the minimum requirements passable by a Type I UE as we showed above. Additionally, by using the test metric of follow type II codebook versus random type II codebook we further cannot guarantee a performance given that there is no clear definition on how a random type II feedback will perform. Different companies may see gains by following type II PMI. But the metric is flawed seeing that we cannot guarantee a performance under random circumstances, thus we see diverging results and gain requirements from companies. Leading to even more uncertain performance requirements. Whereas if we set gain requirements based on Type II versus Type I we can at least guarantee a performance under the benchmark metric; Type I in this case. Random feedback is also never something that will be employed by neither the UE, nor used as the Gnb precoder selected, which further diverges the RAN4 testing from real world deployment. Therefore, the proposed SU-MIMO setup will only verify that the UE is able to report PMI feedback in a type II format, not that it can guarantee that the type II codebook is implemented correctly. We are open to explore options in a SU-MIMO scenario which would address these issues listed above, but so far we have not seen any concrete proposals which would satisfy those issues. In conclusion: - Type I PMI can be formatted in a Type II CSI report since Type I is a subset of Type II as explained above. - The proposed SU-MIMO test setup is not sensitive enough to distinguish Type II CSI reporting from Type I CSI reporting given the proposed test setup. - Setting a minimum performance gain versus random PMI feedback leaves further implementation margins for the UE to pass Type II performance requirements with a Type I implementation - A UE passing the SU-MIMO test will therefore not guarantee that the UE can be scheduled in an MU-MIMO scenario when deployed in a live network. R&S: We have further checked on the test feasibility of the MU-MIMO setup as described. From the Options to generate Xb we don’t think Option1 and Option4 are feasible in a test system, since the co-scheduled UE is not actually available in the test system to provide any feedback. It is therefore not possible to have any knowledge on the reporting of this DUT or the channel conditions for it. The second issue we see is the generation of the Precoding matrix via a ZF algorithm as proposed. This will have major impact on the test system complexity and it needs to be further evaluated, if and to what extend such algorithm could be applied. Taking these open issues into account, we propose to use SU-MIMO setup in Rel-16 timeframe and to further study and evaluate MU-MIMO test setups for future releases. Nokia/NSB: We agree with Ericsson’s analysis and would like to elaborate more on the reasons why an MU-MIMO test succeeds in testing the accuracy of an eType II report where an SU-MIMO test would fail. Let us consider a simple example where the channel measured by a UE1 is roughly flat in frequency and has two dominant eigenvectors with nearly equal eigenvalues in each polarisation. Let us assume that these two eigenvectors are well represented by two orthogonal vectors v1, v2 of the oversampled DFT codebook. UE1, configured to report eType II PMI with paramCombination-r16=2, and rank 1, will accurately report a PMI that is a linear combination of v1 and v2 with linear combination coefficients of equal amplitude for all the subbands, i.e. (omitting power scaling): WeTypeIIUE1=v1+ϕ2,1v2ϕ1,2v1+ϕ2,2v2 where ϕ2,1,ϕ1,2,ϕ2,2 are phase factors. The same UE1, configured to report Type I SP with rank 1, will report only one of the two beams, i.e., for example (omitting power scaling) WeTypeI-SPUE1=v1ϕ2v1 If the Gnb schedules a rank-1 SU-MIMO transmission to UE1, the throughput achieved by eType II PMI and Type I PMI will be about the same because the channel gain in the two beams is about the same. Therefore, an SU-MIMO test would not be able to detect if UE1 inaccurately reported Type I PMI in eType II format to avoid the more complex eType II implementation, because the UE would pass the test. Conversely, an MU-MIMO test would be able to detect this inaccurate PMI report, provided that the PMI of a co-scheduled UE2 is not orthogonal to the accurate eType II PMI of UE1. In fact, let us consider a second UE experiencing a similar channel condition to UE1, but with dominant eigenvectors represented by two orthogonal DFT vectors: v2 and v3. Hence, an accurate eType II PMI for UE2 reads (omitting power scaling) WeTypeIIUE2=v2+ϕ3,1v3ϕ2,2v2+ϕ3,2v3 If UE1 reports the accurate eType II PMI, WeTypeIIUE1, the MU-MIMO precoder for rank-1 transmission to UE1 and UE2 is given by (omitting power scaling) WpassMU=v1ϕ3,1v3ϕ1,2v1ϕ3,2v3 and the throughput measured at UE1 would be comparable to that of the SU-MIMO test because the Gnb was able to steer the transmission of the co-scheduled UE2 in the null space of UE1. Hence the UE would pass the test. Conversely, if UE1 reports the inaccurate Type I PMI, WeTypeI-SPUE1, the MU-MIMO precoder may be given by WfailMU=v1v2ϕ2v1ϕ2,2v2 and the throughput measured at UE1 would be near 0 because the transmission to UE1 is destroyed by the interfering co-scheduled UE. |
| Issue 2-3-5: Beam steering model: how to specify beam steering model in to specification | Samsung: we can compromise to option 1 for moving forward, which reuse current LTE beam steering mode Email discussion comment in [96e][320] NR_Emimo_Demod: WF on eType2 PMI Reporting Samsung We can compromise option 1 for move forward Huawei We do see benefits in specify beam steering model using generic number of beams. But for moving forward, we are fine to compromise |
| Issue 2-4-1: Number of ports | Email discussion comment in [96e][320] NR_Emimo_Demod: WF on eType2 PMI Reporting Samsung NR Rel-15 Type II codebook and Rel-16 advanced Type II codebook are the extension of LTE Rel-14 advanced codebook which they share similar codebook structure. It’s better we can align the number of Tx ports with PMI test case of LTE Efd-MIMO advanced codebook to provide comparable performance. The test complexity especially the number of required individual MIMO channel faders also needs to be considered. With MU-MIMO test setup, additional independent MIMO faders should be also generated for co-located UE, The test complexity will be further increased. The purpose is to verify the UE PMI feedback omplexit, we think 16 Tx port should be enough to show the gain of type II. Also 32 Tx port requirement has already introduced in type I codebook, the test coverage for different number of Tx port can be fulfilled. Based on the TE concerns about complexity , we think 16 Tx port should be enough Huawei We support using 16 Tx ports. |
| Issue 2-4-5: Channel Model | Samsung: based on our initial results, with TDLA channel, more gain can be achieved compared with TDLC |
| Issue 2-4-6: MIMO Correlation | Samsung: we are ok with option 1 |
| Issue 2-4-7: MCS and Rank | Samsung: Base our initial results, with MCS 7 rank1 transmission, Type II codebook can reach maximum throughput. While with MCS 13 rank1 transmission can’t reach peak throughput. If we apply option 1, The MCS is too low, it cannot guarantee the benefit of MU-MIMO compared with SU-MIMO test up Ericsson: If we can align a MU-MIMO setup more clearly then we can discuss the MCS selection. There’s diverging views coming from different assumptions. @Samsung, do you use ideal channel estimation/CSI-RS estimation in your simulation results? |
| Issue 2-4-8: Test metric | Samsung: we suggest to list option 1 as baseline, even we cannot reach consensus for MU-MIMO test setup, and companies are encouraged to provide the initial simulation results to check the proper gain for next meeting Ericsson: As one of the main reasons we listed above in Issue 2-2-1 we think that random PMI is not clearly defined and does not provide an appropriate metric to showcase the gains of type II codebook. Therefore, we prefer Option 2. Nokia/NSB. With Option 1, and without having decided first between SU- and MU- test, there is an added risk, on top of that elaborated in Issue 2-2-1, that the test metric requirement would not guarantee the test objective because of the additional uncertainty of the random PMI definition. For this reason, we prefer Option 2. Email discussion comment in [96e][320] NR_Emimo_Demod: WF on eType2 PMI Reporting Samsung we suggest to list option 2 as baseline, even we cannot reach consensus for MU-MIMO test setup, if we can align the simulation assumption, interested companies can provide the initial simulation results to check the performance with type II under MU-MIMO test setup for next meeting Huawei We cannot agree on option 1. As we mentioned in our contribution, using test metric of TP ratio by follow PMI over random PMI will have potential impact on specific Ues that implementing advanced receiver. In MU-MIMO test setup, there is indeed interference between DUT and co-scheduled UE no matter UE treat it or not. While using the advanced receiver, it is intended to cancel the interference so that in testing it will make great compensate on the throughput of random PMI, which will lead to a lower TP ratio compared with that of Ues without advanced receiver. Why we mention this now rather than other PMI tests is because MU-MIMO test setup brings interference. Moreover, not like Type I SP, random PMI is not a realistic implementation for type II codebook. Therefore, we cannot agree on option 1 |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommandation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2012665->revised to R4-2012761 (eType II) | The revision R4-2012761 after the GTW session is agreeable |

- Annex

Agreements for PDSCH requirements in last RAN4 meeting:

Performance requirements for multi-panel/TRP in FR1 - Reference for timing offset/frequency offset - Option 1 - Using TP which carry on SSB transmission with default TCI state #0 as the reference TP (TP1) - Timing offset = time offset among TP2 and TP1 - Frequency offset = frequency offset among TP2 and TP1 - Other options not precluded - Baseline receiver assumption for FFT window timing - Option 1: Assuming UE always fix FFT timing based on TCI state #0 (TP1) as baseline assumption to define RAN4 performance requirements - Option 2: FFT timing based on TRP with the highest RSRP on sync signals + fixed timing shift - Option 3: FFT timing based on nearest TRP - Other options not precluded - Timing Offset among multi-panel/TRP - Timing offset values - Option 1: ∆t=2-μ∆t1, ∆t1 = [-0.5, 2]μs - Option 2: ∆t=2-μ∆t1, ∆t1 = 2μs - Option 3: FFS on Introducing timing offset which scaled with SCS ∆t=2^(−μ) ∆t1, Candidate values for simulation purpose: {-1, -0.5, 1, 3} - Values for requirements definition should be derived based on performance analysis and analysis on typical TO distributions. Final values should ensure that reasonable UE implementations can meet the requirements - Frequency Offset among multi-panel/TRP - 200Hz for FR1 FDD 15kHz, 300Hz for FR1 TDD 30kHz - TRS/CSI-RS configuration - Taking non-colliding TRS/CSI-RS in multi-TRP/panel as baseline assumption meanwhile interested companies are encouraged to bring more analysis and evaluation results for non-colliding and colliding cases - Multi-DCI based PDSCH requirements - Resource allocation - Option 1: Only non-overlapping cases - Option 2: Both non-overlapping and full-overlapping cases - CRS rate-matching - Do not define performance requirements for multi-DCI based multi-TRP with UE rate-matching around configured CRS pattern - PDCCH configuration - K0 = 0, AL=8 - PDSCH configuration for each TRP - PDSCH resource mapping type: Type A - Resource allocation type: Type 1 - DM-RS configuration: Type 1 with single-symbol DM-RS: 1+1 - Antenna ports indexes: {1000,1001} and {1002,1003}, i.e. different CDM groups for two TRPs - Starting symbol (S): 2 - Time duration (L): 12 - Frequency domain: half of the maximum bandwidth by indicating the start resource block , the allocated resource blocks - Antenna configuration: - Option1: Only 2T2R, 2T4R - Option2: Both 2T2R, 2T4R and 4T2R, 4T4R - Number of test cases - Option 1: 3 test cases per duplex mode - Test 2a Multi- DCI with frequency offset and Non-overlapping scheduling - Test 2b Multi DCI with positive time offset and Non-overlapping scheduling - Test 2c Multi DCI with negative time offset and overlapping scheduling - Other options are not preclude - Single-DCI based multi-TRP/Panel - Introduce PDSCH demodulation requirements for Single-DCI based SDM scheme with full-overlapping resource allocation - Layer combination: 1+1 for both 2Rx and 4Rx - Number of TCI state: Two TCI states configuration - Number of test cases - Option 1: 3 test cases per duplex mode - Test 1a Single DCI with frequency offset and overlapping scheduling - Test 1b Single DCI with positive time offset and overlapping scheduling - Test 1c Single DCI with negative time offset and overlapping scheduling - Other options are not preclude Performance requirements for multi-panel/TRP in FR2 - Necessity of introducing test case(s) for multi-panel/TRP transmission schemes in FR2 - Option 1: No - Option 2: Do not define FR2 requirements for simultaneous reception from multi-TRP/Panel (eMBB) and Study testability for FR2 single-DCI based multi-TRP schemes 3 and 4 - Option 3: Further discuss technical details and relevance of single wide Rx beam reception in FR2 - TCI state configuration for FR2 - Option 1: Single Type D - Other options are not preclude PDSCH requirements with Single-DCI based multi-TRP/Panel transmission schemes (URLLC) - Candidate Schemes - URLLC single-DCI based multi-TRP/panel transmission schemes - SDM scheme (1a) - Repetition schemes - FDM scheme A (2a) - FDM scheme B (2b) - TDM scheme A (3) - Inter-slot TDM scheme (4) - Necessity of PDSCH requirements - Whether already agreed requirements for multi-DCI based and single-DCI based multi-TRP transmission scheme and URLLC single-TRP requirements can cover single DCI based URLLC multi-TRP transmission schemes - Option 1: Yes. FDM schemes can be verified by multi-DCI based and single-DCI based SDM multi-TRP transmission schemes. TDM schemes can be verified by single-DCI based SDM multi-TRP transmission scheme and URLLC single-TRP slot aggregation - Option 2: No. Each scheme is a separate UE feature. Different resource allocation approach comparing to single-DCI based SDM multi-TRP transmission scheme. Different CDM group and TBS calculation assumptions comparing to multi-DCI based and single-DCI based SDM multi-TRP transmission schemes. Non-synchronized (with TO/FO) transmission comparing to single TRP URLLC slot aggregation scenario. Specific sub-features for some schemes as: CW combining, number of repetitions. - Option 3: Cannot cover at least FDM scheme A. - PDSCH requirements introduction - Whether already agreed requirements for multi-DCI based and single-DCI based multi-TRP transmission scheme and URLLC single-TRP requirements can cover single DCI based URLLC multi-TRP transmission schemes - Option 1: Not need to define - Option 2a: Yes, for schemes 2a, 2b, 3 and 4 - Option 2b: Yes, only for one scheme from FDM schemes and one from TDM schemes - Make decision on whether to introduce test cases in Q3 2020 - Interested companies are encouraged to provide simulation assumptions on the next meeting in order to have sufficient time for requirements definition if they will be agreed to introduce on the next meeting - Further discuss test applicability rule (If requirements will be agreed to define) - Option 1: FDM scheme is skipped if UE passes the multi-DCI based multi-TRP transmission and/or TDM scheme is skipped if UE passes URLLC slot aggregation requirements - Other options are not precluded - Test Metric for requirement definition - Option 1: 1 % BLER which is more suitable for URLLC service - Option 2: 70%@max throught

Agreements for PMI test cases in last RAN4 meeting:

- Test setup: - Option 1: Only use SU-MIMO test setup, i.e., one tested UE - Option 2: MU-MIMO based test setup, i.e., one tested UE + one co-scheduled UE (generated by TE) - The baseline receiver assumption is UE without interference cancellation capability with/without co-scheduled UE. - Under the baseline UE receiver assumption, the PMI calculation processing will not change with and without co-scheduled UE. - TE vendors are encouraged to provide feedback for the test feasibility of MU-MIMO test setup. - Proponents for each option need to provide technical analysis for how the test set-up can guarantee UE PMI reporting requirements with enhanced type II codebook for its intended purpose. Detailed test set-up for SU-MIMO and MU-MIMO - Number of CSI-RS ports - Option 1: 16 ports with (N1,N2) = (4,2) and (O1,O2)=(4,4) - Option 2 : 32 ports with (N1,N2) = (4,4) and (O1,O2)=(4,4) - Number of PMI Sub-bands per CQI Sub-band - Option 1: R = 1 as baseline - Option 2: R = 2 - Codebook parameter configuration - Option 1: paramCombination-r16: 6, with L =4, pν =1/2, β=1/2 as baseline - Other options not precluded - Sub-band Size - Option 1: - 4 for FDD with 15kHz SCS, 10MHz CBW - 8 for TDD with 30kHz SCS, 40MHz CBW - Option 2: - 8 for FDD with 15kHz SCS, 10MHz CBW - 16 for TDD with 30kHz SCS, 40MHz CBW - Beam-Steering Model - Configure only two beams in beam steering model for Rel-16 type II test cases - FFS how to specify beam steering model into specification - Option 1: Same as specified in B.2.3B.4A of TS 36.101 - Option 2: Specify using generic number of beams - FFS if beam steering model needs to be modified to account for co-scheduled UE - Channel Model - TDLA30-5 as baseline - Other options not precluded - MIMO Correlation - Option 1: XP High - Option 2: XP Medium - Down-select to one option based on simulation results in the next meeting - MCS and Rank - MCS 20 (64QAM Table), Rank 2 as baseline - Other options not precluded - For initial simulations: - Use the parameters listed in previous slides. - The remaining parameters will be same as for Rel-15 Type II codebook simulation assumptions in R4-2008847 Test metric - Test Metric for MU-MIMO - Option 1: Relative throughput ratio between following PMI for Rel-16 enhanced Type II and Rel-15 Type II codebook - Option 2: Relative throughput ratio between following PMI and random PMI - Other options not precluded Scheduling Parameters for MU-MIMO - Channel model for co-scheduled UE - Option 1: Possibility for spatially separated from DUT - Other options not precluded - Scheduling mode - Option 1: Zero-forcing - Other options not precluded - DCI value for antenna mapping (Table 7.3.1.2.2-2 from 38.212): - Option 1: DUT = 3 with DMRS seed=0, co-scheduled UE=5 with random DMRS seed (Rank 1, DMRS antenna port mapping 1000 for DUT, 1002 for co-scheduled UE) - Option 2: DUT = 7 with DMRS seed=0, co-scheduled UE=8 with random DMRS seed, (Rank 2, DMRS antenna port mapping [1000, 1001] for DUT, [1002, 1003] for co-scheduled UE) - Other options not precluded