3GPP TSG-RAN4 Meeting #96-e	 R4-2012754

# Online, 17th Aug – 28th Aug, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | 0220 | rev | 1 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.141-2: Annex L clarification on equlisation calculation (L.6) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies, Rohde & Schwarz, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2020-08-27 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In Annex section describing equlisation, clarification needed where to apply exceptional case of moving average calculation. Current text which originally copied from LTE specification describes exceptional case at or near the edge of the channel. The same needs to be applied for the case at or near the edge of allocated resources because NR test model 2 doesn’t not have contiguously allocated reference signal subcarriers in the frequency domain. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following modifications made: - Description of where to apply reduced window size, added the case when the available DM-RS subcarrier within a set of contiguously allocated RBs is smaller than the moving average window. - Added additional description text for the case of smaller allocation size in figure (figure replaced) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without this clarificaition, unnecessary poor EVM measurement result calculated due to unnecessary combined amplitude and phase characterisctics which largely separated in frequency. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | L.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | 38.104 CR 0233 |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.141-1 CR 0150 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Changed text which desccribes the method using more number of subcarriers for averaging for the case of reduced moving average window size |  |  |  |  |  |  |  |  |

# L.6	Post-FFT equalisation

Perform 14Ndl FFTs on z'ν, one for each OFDM symbol within 10 ms measurement interval with the FFT window timing to produce an array of samples, 14Ndl in the time axis t by FFT size in the frequency axis f.

For the example in the annex, 1120 FFTs are performed on z'ν. The result is an array of samples, 1120 in the time axis by 4096 in the frequency axis.

The equalizer coefficients af and φf are determined as follows:

1.	Calculate the complex ratios (amplitude and phase) of the post-FFT acquired signal Z't,f and the post-FFT ideal signal I2t,f, for each demodulation reference signal, over 10 ms measurement interval. This process creates a set of complex ratios:

at,fejφt,f=Z't,fI2t,f

2.	Perform time averaging at each demodulation reference signal subcarrier of the complex ratios, the time-averaging length is 10 ms measurement interval. Prior to the averaging of the phases  φti,f  an unwrap operation must be performed according to the following definition:

-	The unwrap operation corrects the radian phase angles of  φti,f by adding multiples of 2 * π when absolute phase jumps between consecutive time instances ti are greater then or equal to the jump tolerance of π radians.

-	This process creates an average amplitude and phase for each demodulation reference signal subcarrier (i.e. every second subcarrier).

af=i=1Nati,fN

φf=i=1Nφti,fN

Where N is the number of demodulation reference signal time-domain locations ti from Z't,f for each demodulation reference signal subcarrier f.

3.	The equalizer coefficients for amplitude and phase af  and φf at the demodulation reference signal subcarriers are obtained by computing the moving average in the frequency domain of the time-averaged demodulation reference signal subcarriers. The moving average window size is 19 and averaging is over the DM-RS subcarriers in the allocated RBs. For DM-RS subcarriers at or near the edge of the channel, or when the number of available DM-RS subcarriers within a set of contiguously allocated RBs is smaller than the moving average window size, the window size is reduced accordingly as per figure L.6-1.

4.	Perform linear interpolation from the equalizer coefficients af and φf to compute coefficients af, φf for each subcarrier.

The subsequent 7 subcarriers are averaged over 5, 7 .. 17 subcarriers

From the 10th subcarrier onwards the window size is 19 until the upper edge of the channel is reached and the window size reduces back to 1

The first reference subcarrier is not averaged

The second reference subcarrier is the average of the first three subcarriers

Reference subcarriers

Reference subcarriers

The first, second and third reference subcarriers are the average of the five subcarriers on left

- Moving averaging at channel edge

- Moving averaging for the case when the number of available reference subcarriers within a set of contiguously allocated RBs is smaller than the moving average window size

The first, second and third reference subcarriers are the average of the five subcarriers on right

Figure B shows an example of 1RB allocation using a reduced window size of five subcarriers for averaging. The same method applies for RB allocations with fewer than 19 subcarriers available for the moving average size. For the case of 2 and 3 RB allocations, 11 and 17 are the window sizes, respectively.

Figure L.6-1: Reference subcarrier smoothing in the frequency domain

a)	In case of FR2 EVM, to account for the common phase error (CPE) experienced in millimetre wave frequencies, φ(f), in the estimated coefficients contain phase rotation due to the CPE, θ, in addition to the phase of the equalizer coefficient φf, that is:

φf=φf+θ(t)

For OFDM symbols where PT-RS does not exist, θ(t) can be estimated by performing linear interpolation from neighboring symbols where PT-RS is present.

In order to separate component of the CPE, θ, contained in, φ(f), estimation and compensation of the CPE needs to follow. θ(t) is the common phase error (CPE), that rotates all the subcarriers of the OFDM symbol at time t.

Estimate of the CPE, θ(t), at OFDM symbol time, t, can then be obtained from using the PT-RS employing the expression:

θ(t)=argf∈fptrsZ't,fIptrst,f afe-jφf

In the above equation, fptrs is the set of subcarriers where PT-RS are mapped, t∈tptrs where tptrs is the set of OFDM symbols where PT-RS are mapped while Z't,f and Iptrst,f are is the post-FFT acquired signal and the ideal PT-RS signal respectively. That is, estimate of the CPE at a given OFDM symbol is obtained from frequency correlation of the complex ratios at the PT-RS positions with the conjugate of the estimated equalizer complex coefficients. The estimated CPE can be subtracted from φf to remove influence of the CPE, and obtain estimate of the complex coefficient's phase:

φf=φf-θ(t)