3GPP TSG-RAN WG4 Meeting #96e		R4-2012765

Electronic Meeting, August 17th – August 28th, 2020

Agenda item:	7.16.1.2

Source:	Ericsson

Title:	Simulation assumptions for NR PMI reporting requirements for more than 8 Tx ports

Document for:	Approval

# Introduction

In this paper we provide detailed simulation assumptions for PMI ports larger than 8 and up to 32 ports. Based on the WF found in [1].

# Simulation assumptions for type-I single panel

## 16Tx ports FDD test

| Parameter |  | Unit | Test 1.1 | Test 1.2 |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |
| Subcarrier spacing |  | kHz | 15 |  |
| Duplex Mode |  |  | FDD |  |
| Propagation channel |  |  | TDLC300-5 |  |
| Antenna configuration |  |  | High XP 16 x 2 (N1,N2) = (4,2) | High XP 16 x 4 N1,N2) = (4,2) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |  |
|  | CSI-RS interval and offset | slot | Not configured |  |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |  |
|  | Number of CSI-RS ports (X) |  | 16 |  |
|  | CDM Type |  | CDM4 (FD2, TD2) |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 12, (2, 4, 6, 8) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5,-) |  |
|  | CSI-RS interval and offset | slot | Not configured |  |
|  | aperiodicTriggeringOffset |  | 0 |  |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |  |
|  | CSI-IM RE pattern |  | Pattern 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |  |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |  |
| ReportConfigType |  |  | Aperiodic |  |
| CQI-table |  |  | Table 1 |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |  |
| cqi-FormatIndicator |  |  | Wideband |  |
| pmi-FormatIndicator |  |  | Subband |  |
| Sub-band Size |  | RB | 8 |  |
| csi-ReportingBand |  |  | 1111111 |  |
| CSI-Report interval and offset |  | slot | Not configured |  |
| Aperiodic Report Slot Offset |  |  | 5 |  |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |
| reportTriggerSize |  |  | 1 |  |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |  |
|  | Codebook Mode |  | 1 |  |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) |  |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |  |
|  | CodebookSubsetRestriction |  | 0x FFFF |  |
|  | RI Restriction |  | 00000010 |  |
| Physical channel for CSI report |  |  | PUSCH |  |
| CQI/RI/PMI delay |  | ms | 8 |  |
| Maximum number of HARQ transmission |  |  | 4 |  |
| Measurement channel |  |  | R.PDSCH.1-6.3 FDD |  |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |  |

## 16Tx ports TDD test

| Parameter |  | Unit | Test 2.1 | Test 2.2 |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |  |
| Subcarrier spacing |  | kHz | 30 |  |
| Duplex Mode |  |  | TDD |  |
| TDD DL-UL configurations |  |  | FR1.30-1 as specified in Annex A |  |
| Propagation channel |  |  | TDLC300-5 |  |
| Antenna configuration |  |  | High XP 16 x 2 (N1,N2) = (4,2) | High XP 16 x 4 (N1,N2) = (4,2) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1) |  | Row 5, (4,-) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |  |
|  | CSI-RS interval and offset | slot | Not configured |  |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |  |
|  | Number of CSI-RS ports (X) |  | 16 |  |
|  | CDM Type |  | CDM4 (FD2, TD2) |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 12, (2, 4, 6, 8) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5,-) |  |
|  | CSI-RS interval and offset | slot | Not configured |  |
|  | aperiodicTriggeringOffset |  | 0 |  |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |  |
|  | CSI-IM RE pattern |  | Pattern 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |  |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |  |
| ReportConfigType |  |  | Aperiodic |  |
| CQI-table |  |  | Table 1 |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForIChannelMeasurements |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |  |
| cqi-FormatIndicator |  |  | Wideband |  |
| pmi-FormatIndicator |  |  | Subband |  |
| Sub-band Size |  | RB | 16 |  |
| csi-ReportingBand |  |  | 1111111 |  |
| CSI-Report interval and offset |  | slot | Not configured |  |
| Aperiodic Report Slot Offset |  |  | 8 |  |
| CSI request |  |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |  |
| reportTriggerSize |  |  | 1 |  |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |  |
|  | Codebook Mode |  | 1 |  |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) |  |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |  |
|  | CodebookSubsetRestriction |  | 0x FFFF |  |
|  | RI Restriction |  | 00000010 |  |
| Physical channel for CSI report |  |  | PUSCH |  |
| CQI/RI/PMI delay |  | ms | 6.5 |  |
| Maximum number of HARQ transmission |  |  | 4 |  |
| Measurement channel |  |  | R.PDSCH.2-8.3 TDD |  |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |  |

## 32Tx ports FDD test

| Parameter |  | Unit | Test 3.1 | Test 3.2 |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |
| Subcarrier spacing |  | kHz | 15 |  |
| Duplex Mode |  |  | FDD |  |
| Propagation channel |  |  | TDLA30-5 |  |
| Antenna configuration |  |  | High XP 32 x 2 (N1,N2) = (4,4) | High XP 32 x 4 N1,N2) = (4,4) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |  |
|  | CSI-RS interval and offset | slot | Not configured |  |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |  |
|  | Number of CSI-RS ports (X) |  | 32 |  |
|  | CDM Type |  | CDM4 (FD2, TD2) |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17, (2, 4, 6, 8) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5,12) |  |
|  | CSI-RS interval and offset | slot | Not configured |  |
|  | aperiodicTriggeringOffset |  | 0 |  |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |  |
|  | CSI-IM RE pattern |  | Pattern 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |  |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |  |
| ReportConfigType |  |  | Aperiodic |  |
| CQI-table |  |  | Table 1 |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |  |
| cqi-FormatIndicator |  |  | Wideband |  |
| pmi-FormatIndicator |  |  | Wideband |  |
| Sub-band Size |  | RB | 8 |  |
| csi-ReportingBand |  |  | 1111111 |  |
| CSI-Report interval and offset |  | slot | Not configured |  |
| Aperiodic Report Slot Offset |  |  | 5 |  |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |
| reportTriggerSize |  |  | 1 |  |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |  |
|  | Codebook Mode |  | 1 |  |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,4) |  |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |  |
|  | CodebookSubsetRestriction |  | 0x FFFF |  |
|  | RI Restriction |  | 00000010 |  |
| Physical channel for CSI report |  |  | PUSCH |  |
| CQI/RI/PMI delay |  | ms | 8 |  |
| Maximum number of HARQ transmission |  |  | 4 |  |
| Measurement channel |  |  | R.PDSCH.1-6.3 FDD |  |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |  |

## 32Tx ports TDD test

| Parameter |  | Unit | Test 2.1 | Test 2.2 |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |  |
| Subcarrier spacing |  | kHz | 30 |  |
| Duplex Mode |  |  | TDD |  |
| TDD DL-UL configurations |  |  | FR1.30-1 as specified in Annex A |  |
| Propagation channel |  |  | TDLA30-5 |  |
| Antenna configuration |  |  | High XP 32 x 2 (N1,N2) = (4,4) | High XP 32 x 4 (N1,N2) = (4,4) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |  |
|  | CSI-RS interval and offset | slot | Not configured |  |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |  |
|  | Number of CSI-RS ports (X) |  | 32 |  |
|  | CDM Type |  | CDM4 (FD2, TD2) |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17, (2, 4, 6, 8) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5,12) |  |
|  | CSI-RS interval and offset | slot | Not configured |  |
|  | aperiodicTriggeringOffset |  | 0 |  |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |  |
|  | CSI-IM RE pattern |  | Pattern 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |  |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |  |
| ReportConfigType |  |  | Aperiodic |  |
| CQI-table |  |  | Table 1 |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForIChannelMeasurements |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |  |
| cqi-FormatIndicator |  |  | Wideband |  |
| pmi-FormatIndicator |  |  | Wideband |  |
| Sub-band Size |  | RB | 16 |  |
| csi-ReportingBand |  |  | 1111111 |  |
| CSI-Report interval and offset |  | slot | Not configured |  |
| Aperiodic Report Slot Offset |  |  | 8 |  |
| CSI request |  |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |  |
| reportTriggerSize |  |  | 1 |  |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |  |
|  | Codebook Mode |  | 1 |  |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,4) |  |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |  |
|  | CodebookSubsetRestriction |  | 0x FFFF |  |
|  | RI Restriction |  | 00000010 |  |
| Physical channel for CSI report |  |  | PUSCH |  |
| CQI/RI/PMI delay |  | ms | 6.5 |  |
| Maximum number of HARQ transmission |  |  | 4 |  |
| Measurement channel |  |  | R.PDSCH.2-8.3 TDD |  |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |  |

# Simulation assumptions for Type II codebook

## Type II test setup and test metric

### UE assumptions

The baseline receiver assumption is UE without interference cancellation capability with/without co-scheduled UE.

Under the baseline UE receiver assumption, the PMI calculation processing will not change with and without co-scheduled UE.

The test purpose of such requirements is to verify UE PMI reporting accuracy following NW configuration with RAN1 feature: enhanced type II codebook

There is no restriction for gNB scheduling with such requirements.

RAN4 need to ensure UE reporting PMI follow Type II codebook other than Type I codebook under proper test set-up either with MU-MIMO set-up or SU-MIMO set-up.

We need to ensure the performance requirements with proper test set-up as receiver implementation agonistic manner i.e. no punishment for advanced receiver with inference cancellation capability.

### Test case design

- Option 1: Only use SU-MIMO test setup, i.e., one tested UE

  - TP ratio between following PMI and random PMI

  - Detailed simulation assumptions in section 3.2 and 3.3.

- Option 2: MU-MIMO based test setup, i.e., one tested UE + one co-scheduled UE (generated by TE)

  - Scheduling parameters for MU-MIMO based test setup:

    - Transmitted signal modeling

      - y1=h1w1x1+h1w2x2+n1

        - Applicable for Rank1

    - Generation of Xb (channel for co-scheduled UE)

      - Option 1: Random PMI

      - Option 2: Fixed PMI

    - Scheduling mode:

      - Option 1: Zero-forcing (more details in Annex 3.8)

    - NZP-CSIRS configuration for each UE : not colliding

    - IMR configuration: NO IMR  configuration

    - DMRS/Rank configuration for each UE (Table 7.3.1.2.2-2 from 38.212):

      - DUT = 3 with DMRS seed=0, co-scheduled UE=5 with random DMRS seed (Rank 1, DMRS antenna port mapping 1000 for DUT, 1002 for co-scheduled UE)

  - codebook construction:

    - Option 1: 32Tx ports (N1, N2) = (4,4), (O1, O2) = (4,4)

    - Option 2: 16Tx ports (N1, N2) = (4,2), (O1, O2) = (4,4)

  - Npsk (phaseAlphabetSize): 8

  - subbandAmplitude: True

  - Propagation model

    - Option 1: TDLC300-5

    - Option 2: TDLA30-5

    - Other options are not precluded

  - MIMO correlation

    - XP Medium

  - Test metric

    - Option 1: TP ratio between following PMI and random PMI

    - Option 2: TP ratio between following Type II codebook and following SP Type I codebook

    - Other options are not precluded

  - MCS and Rank

    - Option 1: Rank1 MCS7

    - Other options not precluded

- Option 3: Using SU-MIMO set-up to Introduce PMI test cases. Meanwhile a MU-MIMO setup based demodulation test with test metric of either follow PMI based or random PMI based throughput can be introduced

## SU-MIMO FDD test

| Parameter |  | Unit | Test 5.1 | Test 5.2 |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |
| Subcarrier spacing |  | kHz | 15 |  |
| Duplex Mode |  |  | FDD |  |
| Propagation channel |  |  | TDLA30-5 |  |
| Antenna configuration |  |  | Medium or High XP 16 x 2 (N1,N2) = (4,2) | Medium or High XP 16 x 4 (N1,N2) = (4,2) |
| Beamforming Model |  |  | As specified in Annex B.2.3B.4A of TS36.101 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |  |
|  | CSI-RS interval and offset | slot | Not configured |  |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |  |
|  | Number of CSI-RS ports (X) |  | 16 |  |
|  | CDM Type |  | CDM4 (FD2, TD2) |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 12, (2, 4, 6, 8) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5,-) |  |
|  | CSI-RS interval and offset | slot | Not configured |  |
|  | aperiodicTriggeringOffset |  | 0 |  |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |  |
|  | CSI-IM RE pattern |  | Pattern 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |  |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |  |
| ReportConfigType |  |  | Aperiodic |  |
| CQI-table |  |  | Table 1 |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |  |
| cqi-FormatIndicator |  |  | Wideband |  |
| pmi-FormatIndicator |  |  | Option 1: Wideband Option 2: Subband |  |
| Sub-band Size |  | RB | 4 or 8 |  |
| csi-ReportingBand |  |  | 1111111 |  |
| CSI-Report interval and offset |  | slot | Not configured |  |
| Aperiodic Report Slot Offset |  |  | 5 |  |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |
| reportTriggerSize |  |  | 1 |  |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |
| Codebook configuration | Codebook Type |  | TypeII |  |
|  | L (numberOfBeams) |  | 2 |  |
|  | NPSK (phaseAlphabetSize) |  | 8 |  |
|  | subbandAmplitude |  | Option 1: False Option 2: True |  |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) |  |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |  |
|  | CodebookSubsetRestriction |  | 0x FFFF |  |
|  | RI Restriction |  | 00000010 |  |
| Physical channel for CSI report |  |  | PUSCH |  |
| CQI/RI/PMI delay |  | ms | 8 |  |
| Maximum number of HARQ transmission |  |  | 4 |  |
| Measurement channel |  |  | R.PDSCH.1-6.3 FDD |  |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |  |

## SU-MIMO TDD test

| Parameter |  | Unit | Test 6.1 | Test 6.2 |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |  |
| Subcarrier spacing |  | kHz | 30 |  |
| Duplex Mode |  |  | TDD |  |
| TDD DL-UL configurations |  |  | FR1.30-1 as specified in Annex A |  |
| Propagation channel |  |  | TDLA30-5 |  |
| Antenna configuration |  |  | Medium or High XP [16 x 2 (N1,N2) = (4,2)] | Medium or High XP [16 x 4 (N1,N2) = (4,2)] |
| Beamforming Model |  |  | As specified in Annex B.2.3B.4A of TS36.101 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |  |
|  | CSI-RS interval and offset | slot | Not configured |  |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |  |
|  | Number of CSI-RS ports (X) |  | 16 |  |
|  | CDM Type |  | CDM4 (FD2, TD2) |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | (Row 12, (2, 4, 6, 8) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5,-) |  |
|  | CSI-RS interval and offset | slot | Not configured |  |
|  | aperiodicTriggeringOffset |  | 0 |  |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |  |
|  | CSI-IM RE pattern |  | Pattern 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |  |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |  |
| ReportConfigType |  |  | Aperiodic |  |
| CQI-table |  |  | Table 1 |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForIChannelMeasurements |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |  |
| cqi-FormatIndicator |  |  | Wideband |  |
| pmi-FormatIndicator |  |  | Option 1: Wideband Option 2: Subband |  |
| Sub-band Size |  | RB | 8 or 16 |  |
| csi-ReportingBand |  |  | 1111111 |  |
| CSI-Report interval and offset |  | slot | Not configured |  |
| Aperiodic Report Slot Offset |  |  | 8 |  |
| CSI request |  |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |  |
| reportTriggerSize |  |  | 1 |  |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |
| Codebook configuration | Codebook Type |  | TypeII |  |
|  | L (numberOfBeams) |  | 2 |  |
|  | NPSK (phaseAlphabetSize) |  | 8 |  |
|  | subbandAmplitude |  | Option 1: False Option 2: True |  |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) |  |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |  |
|  | CodebookSubsetRestriction |  | 0x FFFF |  |
|  | RI Restriction |  | 00000010 |  |
| Physical channel for CSI report |  |  | PUSCH |  |
| CQI/RI/PMI delay |  | ms | 6.5 |  |
| Maximum number of HARQ transmission |  |  | 4 |  |
| Measurement channel |  |  | R.PDSCH.2-8.3 TDD |  |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |  |

## MU-MIMO FDD test

| Parameter |  | Unit | Test 5.1 | Test 5.2 |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |
| Subcarrier spacing |  | kHz | 15 |  |
| Duplex Mode |  |  | FDD |  |
| Propagation channel |  |  | Option 1: TDLC300-5 Option 2: TDLA30-5 Other options are not precluded |  |
| Antenna configuration |  |  | Medium XP 16 x 2 (N1,N2) = (4,2) or 32 x 2 (N1,N2) = (4,4) | Medium XP 16 x 4 (N1,N2) = (4,2) or 32 x 4 (N1,N2) = (4,4) |
| Beamforming Model |  |  | – Option 1: Not introducing impairment model – Option 2: Yes |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |  |
|  | CSI-RS interval and offset | slot | Not configured |  |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |  |
|  | Number of CSI-RS ports (X) |  | TBD 16 or 32 |  |
|  | CDM Type |  | CDM4 (FD2, TD2) |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | TBD (Row 12, (2, 4, 6, 8) or Row 17, (2, 4, 6, 8)) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | TBD ((5,-) or (5,12)) |  |
|  | CSI-RS interval and offset | slot | Not configured |  |
|  | aperiodicTriggeringOffset |  | 0 |  |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |  |
|  | CSI-IM RE pattern |  | Pattern 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |  |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |  |
| ReportConfigType |  |  | Aperiodic |  |
| CQI-table |  |  | Table 1 |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |  |
| cqi-FormatIndicator |  |  | Wideband |  |
| pmi-FormatIndicator |  |  | Subband |  |
| Sub-band Size |  | RB | 4 |  |
| csi-ReportingBand |  |  | 1111111 |  |
| CSI-Report interval and offset |  | slot | Not configured |  |
| Aperiodic Report Slot Offset |  |  | 5 |  |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |
| reportTriggerSize |  |  | 1 |  |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |
| Codebook configuration | Codebook Type |  | TypeII |  |
|  | L (numberOfBeams) |  | 2 or 4 |  |
|  | NPSK (phaseAlphabetSize) |  | 8 |  |
|  | subbandAmplitude |  | True |  |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) or (4,4) |  |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |  |
|  | CodebookSubsetRestriction |  | 0x FFFF |  |
|  | RI Restriction |  | 00000001 |  |
| Physical channel for CSI report |  |  | PUSCH |  |
| CQI/RI/PMI delay |  | ms | 8 |  |
| Maximum number of HARQ transmission |  |  | 4 |  |
| Measurement channel |  |  | TBD |  |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |  |

## MU-MIMO TDD test

| Parameter |  | Unit | Test 6.1 | Test 6.2 |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |  |
| Subcarrier spacing |  | kHz | 30 |  |
| Duplex Mode |  |  | TDD |  |
| TDD DL-UL configurations |  |  | FR1.30-1 as specified in Annex A |  |
| Propagation channel |  |  | Option 1: TDLC300-5 Option 2: TDLA30-5 Other options are not precluded |  |
| Antenna configuration |  |  | Medium XP 16 x 2 (N1,N2) = (4,2) or 32 x 2 (N1,N2) = (4,4) | Medium XP 16 x 4 (N1,N2) = (4,2) or 32 x 4 (N1,N2) = (4,4) |
| Beamforming Model |  |  | Option 1: Not introducing impairment model – Option 2: Yes |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |  |
|  | CSI-RS interval and offset | slot | Not configured |  |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |  |
|  | Number of CSI-RS ports (X) |  | TBD 16 or 32 |  |
|  | CDM Type |  | CDM4 (FD2, TD2) |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | TBD (Row 12, (2, 4, 6, 8) or Row 17, (2, 4, 6, 8)) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | TBD ((5,-) or (5,12)) |  |
|  | CSI-RS interval and offset | slot | Not configured |  |
|  | aperiodicTriggeringOffset |  | 0 |  |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |  |
|  | CSI-IM RE pattern |  | Pattern 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |  |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |  |
| ReportConfigType |  |  | Aperiodic |  |
| CQI-table |  |  | Table 1 |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForIChannelMeasurements |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |  |
| cqi-FormatIndicator |  |  | Wideband |  |
| pmi-FormatIndicator |  |  | Subband |  |
| Sub-band Size |  | RB | 8 |  |
| csi-ReportingBand |  |  | 1111111 |  |
| CSI-Report interval and offset |  | slot | Not configured |  |
| Aperiodic Report Slot Offset |  |  | 8 |  |
| CSI request |  |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |  |
| reportTriggerSize |  |  | 1 |  |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |
| Codebook configuration | Codebook Type |  | TypeII |  |
|  | L (numberOfBeams) |  | 2 or 4 |  |
|  | NPSK (phaseAlphabetSize) |  | 8 |  |
|  | subbandAmplitude |  | True |  |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) or (4,4) |  |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |  |
|  | CodebookSubsetRestriction |  | 0x FFFF |  |
|  | RI Restriction |  | 00000001 |  |
| Physical channel for CSI report |  |  | PUSCH |  |
| CQI/RI/PMI delay |  | ms | 6.5 |  |
| Maximum number of HARQ transmission |  |  | 4 |  |
| Measurement channel |  |  | TBD |  |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |  |

# References

- R4-2012762 “Way forward on PMI reporting requirements for Tx ports larger than 8 and up to 32”, Ericsson, Samsung, RAN4#96e, August 2020.

# Annex

## SU-MIMO FDD FRC

Table A.1: PDSCH Reference Channel for FDD PMI reporting requirements

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-6.1 FDD | R.PDSCH.1-6.2 FDD | R.PDSCH.1-6.3 FDD |  |  |
| Channel bandwidth | MHz | 10 | 10 | 10 |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 |  |  |
| Number of allocated resource blocks | PRBs | 52 | 52 | 52 |  |  |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 |  |  |
| Allocated slots per 2 frames | Slots | 15 | 15 | 15 |  |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |  |
| MCS index |  | 13 | 13 | 20 |  |  |
| Modulation |  | 16QAM | 16QAM | 64QAM |  |  |
| Target Coding Rate |  | 0.48 | 0.48 | 0.55 |  |  |
| Number of MIMO layer |  | 1 | 2 | 2 |  |  |
| Number of DMRS REs (Note 3) |  | 24 | 24 | 24 |  |  |
| Overhead for TBS determination |  | 0 | 0 | 0 |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A |  |  |
| For CSI Slots i, if mod (i,5) =1, i={0,…,19} |  | N/A | N/A | N/A |  |  |
| For Non CSI-RS Slot i, if mod (i,5) ={0,2,3,4}, i={1,..19} | Bits | 12040 | 24072 | 40976 |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A |  |  |
| For CSI Slots i, if mod (i,5) =1, i={0,…,19} |  | N/A | N/A | N/A |  |  |
| For Non CSI-RS Slot i, if mod (i,5) ={0,2,3,4}, i={1,..19} | Bits | 24 | 24 | 24 |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A | N/A |  |  |
| For CSI Slots i, if mod (i,5) =1, i={0,…,19} |  | N/A | N/A | N/A |  |  |
| For Non CSI-RS Slot i, if mod (i,5) ={0,2,3,4}, i={1,..,19} | CBs | 2 | 3 | 5 |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A |  |  |
| For CSI Slots i, if mod (i,5) =1, i={0,…,19} |  | N/A | N/A | N/A |  |  |
| For Slots i = 10 | Bits | 23712 | 47424 | 71136 |  |  |
| For Non CSI-RS Slot i, if mod (i,5) ={0,2,3,4}, i={1,..9,11,…,19} | Bits | 24960 | 49920 | 74880 |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 9.030 | 18.054 | 30.732 |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data |  |  |  |  |  |  |

## SU-MIMO TDD FRC

Table A.2: PDSCH Reference Channel for TDD PMI reporting requirements with UL-DL pattern FR1.30-1

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-8.1 TDD | R.PDSCH.2-8.2 TDD | R.PDSCH.2-8.3 TDD |  |  |
| Channel bandwidth | MHz | 40 | 40 | 40 |  |  |
| Subcarrier spacing | kHz | 30 | 30 | 30 |  |  |
| Allocated resource blocks | PRBs | 106 | 106 | 106 |  |  |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 |  |  |
| Allocated slots per 2 frames |  | 23 | 23 | 23 |  |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |  |
| MCS index |  | 13 | 13 | 20 |  |  |
| Modulation |  | 16QAM | 16QAM | 64QAM |  |  |
| Target Coding Rate |  | 0.48 | 0.48 | 0.55 |  |  |
| Number of MIMO layers |  | 1 | 2 | 2 |  |  |
| Number of DMRS REs (Note 3) |  | 24 | 24 | 24 |  |  |
| Overhead for TBS determination |  | 0 | 0 | 0 |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A |  |  |
| For CSI-RS Slot i, if mod(i,10) =1 for i from {0,…,39} | Bits | N/A | N/A | N/A |  |  |
| For Slot i = 20 | Bits | 24576 | 49176 | 83976 |  |  |
| For Slot i, if mod(i, 10) = {0,2,3,4,5,6} for i from {1,…,19,22,…,39} |  | 24576 | 49176 | 83976 |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A |  |  |
| For CSI-RS Slot i, if mod(i,10) =1 for i from {0,…,39} |  | N/A | N/A | N/A |  |  |
| For Slot i = 20 | Bits | 24 | 24 | 24 |  |  |
| For Slot i, if mod(i, 10) = {0,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 24 | 24 | 24 |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | CBs | N/A | N/A | N/A |  |  |
| For CSI-RS Slot i, if mod(i,10) =1 for i from {0,…,39} |  | N/A | N/A | N/A |  |  |
| For Slot i = 20 | CBs | 3 | 6 | 10 |  |  |
| For Slot i, if mod(i, 10) = {0,2,3,4,5,6} for i from {1,…,19,22,…,39} | CBs | 3 | 6 | 10 |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A |  |  |
| For CSI-RS Slot i, if mod(i,10) =1 for i from {0,…,39} | Bits | N/A | N/A | N/A |  |  |
| For Slot i = 20 | Bits | 48336 | 96672 | 145008 |  |  |
| For Slot i, if mod(i, 10) = {0,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 50880 | 101760 | 152640 |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 28.2624 | 56.5524 | 96.5724 |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data |  |  |  |  |  |  |

Figure 1 Test setup proposal for MU-MIMO based transmission

## Zero Forcing precoder

W=WaXbH where Wa is the orthogonalized and normalised Wa and XbH is the normalized projection of the co-scheduled PMI on the null space of Wa XbH=I-WaWaHWb diagq-1/2