3GPP TSG-RAN WG4 #96-e	R4-2010553

Online, 17th – 28th August, 2020

Source:	Huawei, HiSilicon, Etisalat, CMCC

Title:	TP to TR 37.717-00-00 DC_28A_SUL_n41A-n83A

Agenda item:			xxxx

Document for:	Approval

# 1. Introduction

In this contribution, we propose a text proposal for TR 37.717-00-00 for SUL band combinations in Rel-17 to introduce DC_28A_SUL_n41A-n83A.

# 2. Text Proposal

----- Unchanged sections omitted -----

## 7.x	DC_28A-SUL_n41A-n83A

### 7.x.1	Operating bands

Table 7.x.1-1: EN-DC band combination

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_28_SUL_n41-n832 | 1 | SUL_n41-n83 | N/A |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier can be up to 140us and placed in SUL resources. NOTE 2: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. |  |  |  |

### 7.x.2	Configuration

Table 7.x.2-1: Inter-band EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_28A-SUL_n41A-n83A | DC_28A_n41A DC_28A_n83A_ULSUP-TDM_n41, | 28A | SUL_n41A-n83A |

### 7.x.3	Maximum output power

Table 7.x.3-1: Maximum output power for inter-band EN-DC

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_28A_n41A DC_28A_n83A_ULSUP-TDM_n41, | 23 | +2/-3 |

### 7.x.4	Spurious emission band UE co-existence

Table 7.x.4-1: Spurious emissions for inter-band EN-DC

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_28_n41 DC_28_n83_ULSUP-TDM_n41, | E-UTRA Band 4, 10, 12, 13, 14, 17, 18, 19, 20, 26, 27, 29, 39, 42, 43, 50, 51, 52, 65, 66, 71, 73, 85 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |
|  | E-UTRA Band 2, 3, 5, 8, 24, 25, 30, 31, 34, 40, 44, 48, 70, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 9: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 10: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 11: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 14: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. NOTE 17: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. |  |  |  |  |  |  |  |

### 7.x.5	MSD

For DC_28_SUL_n41-n83, there are neither harmonic and harmonic mixing products generated by Band 28/n83 that may fall into the RX band of Band n41 nor IMD products produced by Band 28 and n41 that impact the reference sensitivity of Band 28. Therefore, MSD requirements are not needed for this configuration.

### 7.x.6	∆TIB and ∆RIB values

For DC_28_SUL_n41-n83, the TIB,c and RIB values are given in the tables below.

# Table 7.x.6-1: ΔTIB,c

| EN-DC Band combination | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28-SUL_n41-n83 | 28 | 0.3 |
|  | n41 | 0.3 |
|  | n83 | 0.3 |

# Table 7.x.6-2: ΔRIB,c

| EN-DC Band combination | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_28-SUL_n41-n83 | 28 | 0 |
|  | n41 | 0 |
|  | n83 | 0 |

----- Unchanged sections omitted -----

# Reference