3GPP TSG-RAN4 Meeting #97-e	R4-2014683

# Online,  2nd Nov – 13th Nov 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 0370 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UL output power for spurious response and general Rx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Corporation, Apple Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-10-23 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Closely associated to the previously agreed CR to OoBB requirements (R4-2011936/2010047), same definitions of UL output power need to be applied also to the following spurious response requirements: - 7.7B.3 Inter-band EN-DC within FR1 - 7.7B.3a Inter-band NE-DC within FR1 - Related to above, there is an inconsistency that the current definitions of 7.7B.3a spurious response for inter-band NE-DC within FR1 are not aligned with 7.6B.3.3a (OoBB) Inter-band NE-DC within FR1. - Similar output power setting also needs to be updated for intra-band non-contiguous EN-DC Rx requirements in clause 7.1. - Incorrect clause referencing numbers for inter-band EN-DC/NE-DC combinations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change UL power setting at spurious response for the lower UL power carrier (either E-UTRA or NR) from minimum output power to 29 dB below PCMAX_L. - Align the description of 7.7B.3a with 7.6B.3.3a. - Change UL power setting in clause 7.1 for intra-band non-contiguous EN-DC Rx requirements to ensure anchor resources do not interfere NR operation. - Change referencing clause number from 5.5.B.4.1 to 5.5B.4.1 for inter-band EN-DC combinations within FR1. Correct the referencing clause number from 5.5.B.4a.1 to 5.5B.4a.1 for inter-band NE-DC combinations within FR1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testability issue due to the ADC dyanmic range limitation on the tester will remain in the Rx spec. Incorrect clause referencing remains in the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1, 7.6B.3.3, 7.6B.3.3a, 7.7B.3, 7.7B.3a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Associated CR agreed at RAN4 #96-e: R4-2011936 / R4-2010047 Associated discussion paper : R4-2010045 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchaged sections skipped>>

<<Start of change 1>>

7.1	General

Unless otherwise stated the receiver characteristics are specified at the antenna connector(s) of the UE for the bands operating on frequency range 1 and over the air of the UE for the bands operating on frequency range 2. The requirements for frequency range 1 and frequency range 2 can be verified separately. For the carrier in frequency range 1, requirements can be verified with NR FR2 link disabled. For the carrier in frequency range 2, requirements can be verified in OTA mode with E-UTRA connecting to the network by OTA without calibration.

The requirements defined in this clause are the extra requirements compared with the single carrier requirements defined in TS 38.101-1 [2] and TS 38.101-2 [3].

Unless otherwise stated, the UL and DL reference measurement channels are the same with the configurations specified in TS 38.101-1 [2] and TS 38.101-2 [3].

Unless otherwise stated, requirements for NR receiver written in TS 38.101-1 [2] and TS 38.101-2 [3] apply and are assumed anchor agnostic. Requirements are verified under conditions where anchor resources do not interfere NR operation.

For intra-band non-contiguous EN-DC, the output power is configured as follows:

-	One E-UTRA uplink carrier with the output power set to 29 dB below PCMAX_L and the NR band whose downlink is being tested has its uplink carrier output power set to 4 dB below PCMAX_L,f,c.

-	One NR uplink carrier with the output power set to 29 dB below PCMAX_L,f,c and the E-UTRA band whose downlink is being tested has its uplink carrier output power set to 4 dB below PCMAX_L,c.

For the additional requirements for intra-band non-contiguous EN-DC of two sub-blocks, an in-gap test refers to the case when the interfering signal is located at a negative offset with respect to the assigned lowest channel frequency of the highest sub-block and located at a positive offset with respect to the assigned highest channel frequency of the lowest sub-block.

For the additional requirements for intra-band non-contiguous EN-DC of two sub-blocks, an out-of-gap test refers to the case when the interfering signal(s) is (are) located at a positive offset with respect to the assigned channel frequency of the highest carrier frequency or located at a negative offset with respect to the assigned channel frequency of the lowest carrier frequency.

For the additional requirements for intra-band non-contiguous EN-DC of two sub-blocks with channel bandwidth larger than or equal to 5 MHz, the existing adjacent channel selectivity requirements, in-band blocking requirements (for each case), and narrow band blocking requirements apply for in-gap tests only if the corresponding interferer frequency offsets with respect to the two measured carriers satisfy the following condition in relation to the sub-block gap size Wgap for at least one of the E-UTRA or NR sub-blocks, so that the interferer frequency position does not change the nature of the core requirement tested:

Wgap ≥ 2∙|FInterferer (offset)| – BWChannel

For the E-UTRA sub-block, the FInterferer (offset), for a sub-block with a single component carrier is the interferer frequency offset with respect to carrier as specified in clause 7.5.1, clause 7.6.1 and clause 7.6.3 for the respective requirement in TS 36.101 [4] and BWChannel. FInterferer (offset) for the E-UTRA sub-block with two or more contiguous component carriers is the interference frequency offset with respect to the carrier adjacent to the gap is specified in clause 7.5.1A, 7.6.1A and 7.6.3A in TS 36.101 [4].

For the NR sub-block, the FInterferer (offset), for a sub-block with a single component carrier is the interferer frequency offset with respect to carrier as specified in clause 7.5.1, clause 7.6.1 and clause 7.6.3 for the respective requirement in TS 38.101-1 [2] and BWChannel.

The interferer frequency offsets for adjacent channel selectivity, each in-band blocking case and narrow-band blocking shall be tested separately with a single in-gap interferer at a time.

For sub-clauses with suffix A or B: the minimum requirements for band combinations including Band n41 also apply for the corresponding band combinations with Band n90 replacing Band n41 but with otherwise identical parameters. For brevity the said band combinations with Band n90 are not listed in the tables below but are covered by this specification.

<<End of change 1>>

<<Unchaged sections skipped>>

<<Start of change 2>>

7.6B.3.3	Inter-band EN-DC within FR1

Out-of band blocking requirements for E-UTRA single carrier and CA operation specified in clauses 7.6.2.1 and 7.6.2.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.6.3 and 7.6A.3 of TS 38.101-1 [2] apply for lowest level EN-DC fallbacks (two bands) in clause 5.5B.4.1 with following conditions

one E-UTRA uplink carrier with the output power set to 4 dB below PCMAX_L,c and the NR band whose downlink is being tested has its uplink carrier output power set to 29 dB below PCMAX_L,f,c.

one NR uplink carrier with the output power set to 4 dB below PCMAX_L,f,c on the NR band with both E-UTRA and NR downlinks being tested with E-UTRA output power set to 29 dB below PCMAX_L,c.

If CW interferer falls in a gap between FDL_high of the E-UTRA or NR band and FDL_low of the NR or EUTRA band, where the corresponding OOB ranges 1 and 2 overlap, then the lower level interferer limit of the overlapping OOB ranges applies.

If FDL_high of the lower E-UTRA or NR band is greater than or equal to the FDL_low of the upper NR or E-UTRA band as in overlapping RX frequency ranges, then the OOB range shall start from the FDL_low of the lower E-UTRA or NR band, and from the FDL_high of the upper NR or E-UTRA band.

For EN-DC combination listed in Table 7.6B.3.3-1 under the first test condition above, exceptions to the requirement specified in Table 7.6B.3.3-2 are allowed when the second order intermodulation product of the lower frequency band UL carrier and the CW interfering signal fully or partially overlaps with the higher frequency band DL carrier.

Table 7.6B.3.3-1: EN-DC combination with exceptions allowed

EN-DC combination

DC_5_n78

DC_8_n77

DC_8_n78

DC_8_n79

DC_11_n77

DC_18_n77

DC_18_n78

DC_18_n79

DC_19_n77

DC_19_n78

DC_19_n79

DC_20_n77

DC_20_n78

DC_21_n77

DC_26_n77

DC_26_n78

DC_26_n79

DC_28_n77

DC_28_n78

DC_28_n79

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -441 |
| NOTE 1: The requirement applies when fInterferer± fULLB- fDLHB≤(BWULLB+ BWDLHB)/2, where fULLB and fDLHB are the carrier frequencies for lower frequency band UL and higher frequency band DL, respectively. BWULLB and BWDLHB are the channel bandwidths configured for lower frequency band UL carrier and higher frequency band DL carrier in MHz, respectively. |  |  |

For each of the two test cases in clauses 7.6.2.1 and 7.6.2.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.6.3 and 7.6A.3 of TS 38.101-1 [2] for all interferer frequency ranges a maximum of

exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a step size of  MHz with NRB the number of resource blocks in the downlink transmission bandwidth configuration, CBW the bandwidth of the frequency channel in MHz and n = 1, 2, 3 for SCS = 15, 30, 60 kHz, respectively. For these exceptions, the requirements in clause 7.7 apply.

#### 7.6B.3.3a	Inter-band NE-DC within FR1

Out-of band blocking requirements for E-UTRA single carrier and CA operation specified in clauses 7.6.2.1 and 7.6.2.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.6.3 and 7.6A.3 of TS 38.101-1 [2] apply for lowest level NE-DC fallbacks (two bands) in clause 5.5B.4a.1 with following conditions

one E-UTRA uplink carrier with the output power set to 4 dB below PCMAX_L,c and the NR band whose downlink is being tested has its uplink carrier output power set to 29 dB below PCMAX_L,f,c.

one NR uplink carrier with the output power set to 4 dB below PCMAX_L,f,c on the NR band with both E-UTRA and NR downlinks being tested with E-UTRA output power set to 29 dB below PCMAX_L,c.

<<End of change 2>>

<<Start of change 3>>

7.7B.3	Inter-band EN-DC within FR1

Spurious response requirement for E-UTRA single carrier and CA operation specified in clauses 7.7.1 and 7.7.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.7 and 7.7A of TS 38.101-1 [2] apply for lowest level EN-DC fallbacks (two bands) in clause 5.5B.4.1 with following conditions

-	one E-UTRA uplink carrier with the output power set to 4 dB below PCMAX_L and the NR band whose downlink is being tested has its uplink carrier output power set to 29 dB below PCMAX_L,f,c.

-	one NR uplink carrier with the output power set to 4 dB below PCMAX_L,f,c on the NR band with both E-UTRA and NR downlinks being tested with E-UTRA output power set to 29 dB below PCMAX_L,c.

### 7.7B.3a	Inter-band NE-DC within FR1

Spurious response requirement for E-UTRA single carrier and CA operation specified in clauses 7.7.1 and 7.7.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.7 and 7.7A of TS 38.101-1 [2] apply for lowest level NE-DC fallbacks (two bands) in clause 5.5B.4a.1 with following conditions

one E-UTRA uplink carrier with the output power set to 4 dB below PCMAX_L,c and the NR band whose downlink is being tested has its uplink carrier output power set to 29 dB below PCMAX_L,f,c.

one NR uplink carrier with the output power set to 4 dB below PCMAX_L,f,c on the NR band with both E-UTRA and NR downlinks being tested with E-UTRA output power set to 29 dB below PCMAX_L,c.

<<End of change 3>>