3GPP TSG-RAN WG4 Meeting #97-e	 R4-2016988

Electronic Meeting, 2-13 Nov., 2020

Title:	Draft LS on simultaneous Rx/Tx capability

Response to:

Release:	Rel-15

Work Item:	NR_newRAT

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

Name: Liu Ye

E-mail Address: leo.liuye@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

1. Overall Description:

Simultaneous Rx/Tx capability for TDD-TDD and TDD-FDD inter-band CA, SUL and EN-DC band combinations has been discussed in RAN4. It is identified that there are some ambiguity on the applicability of simultaneous Rx/Tx condition for an inter-band combination, especially for the combination having more than two bands. As an example, for CA_n39-n41-n79, if UE supports simultaneous Rx/Tx capability for CA_n41-n79 but not for CA_n39-n41, the simultaneous Rx/Tx capability shall not be reported for CA_n39-n41-n79. However, since the capability of the fallback mode is different from the higher order band combination, simultaneous Rx/Tx capability for CA_n41-n79 shall be reported additionally.

For a band combination with different simultaneous Rx/Tx capability for the fallback mode, RAN4’s understanding is that the network shall also consider the fallback mode capability to decide the UL/DL scheduling among all bands for this band combination. It’s not clear whether the current RAN2 specification supports this kind of understanding. If not or if it can only be derived implicitly, RAN4 would like to see some explicit clarification in the RAN2 specification.

In addition, it is RAN4 understanding that absence of the simultaneous Rx/Tx capability for TDD-TDD and TDD-FDD inter-band CA, SUL and EN-DC band combinations C means that simultaneous RX/TX is not supported for the band combination, otherwise, if simultaneous Rx/Tx capability is supported, the capability indication must be set to “supported”.

For the question raised by RAN2 on whether the simultaneous RxTx UE capability is needed for inter-band NR-DC (for TDD-TDD and TDD-FDD band combinations), RAN4 thinks that the capability is needed, and same principles used for simultaneousRxTxInterBandCA as well as clarification consideration above shall also be applied for NR-DC.

2. Actions:

To RAN2:

ACTION: RAN4 respectfully asks RAN2 to take the above RAN4 consideration on simultaneous Rx/Tx capability into account.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN4 Meeting#98 	 1st – 5th March 2020	Athens, Greece