3GPP TSG-RAN WG4 Meeting #97-e	R4-2017250

# Electronic Meeting, Nov 2 – 13, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | 1 | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test cases for inter-RAT cell identification in connected mode for HST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST-Perf |  |  |  |  |  | Date: |  |  | 2020-11-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The following test cases for high speed are needed to be specified: 1. NR-EUTRA inter-RAT event triggered reporting test under DRX in FR1 2. EUTRA-NR inter-RAT event triggered reporting for FR1 with SSB time index detection when DRX is used |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specifying the following test cases for high speed: - NR-EUTRA inter-RAT event triggered reporting test under DRX in FR1 - DRX cycle: 320ms - SMTC period: 20ms - MGRP: 40ms - MGL: 6ms 2. EUTRA-NR inter-RAT event triggered reporting for FR1 with SSB time index detection when DRX is used - DRX cycle: 320ms - SMTC period: 20ms - MGRP: 40ms - MGL: 6ms |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New A.6.6.3.3; A.8.4.2.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change 1 ------------------------------------------------------------

A.6.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for UE configured with highSpeedMeasFlag-r16

A.6.6.3.3.1	Test Purpose and Environment

The purpose of this set of tests is to verify that the UE makes correct event-triggered reporting of inter-RAT E-UTRAN measurements for UE configured with highSpeedMeasFlag-r16 in standalone (SA) operation with PCell in FR1 when DRX is used. This test shall partly verify the cell search and measurement requirements in Clauses 9.4.2 and 9.4.3.

In the test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN inter-RAT neighbour cell. In the measurement control information from the PCell it is indicated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) is to be used. Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1. During T1, the UE shall not have any information on Cell 2.

The UE shall be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore the UE shall be allocated with PUSCH resource at every DRX cycle.

Supported test configurations are shown in table A.6.6.3.3.1-1. General test parameters are provided in Table A.6.6.3.3.1-2 below. Test parameters for Cell 1 and Cell 2, valid for both time duration T1 and T2, are provided in Tables A.6.6.3.3.1-3 and A.6.6.3.3.1-4, respectively.

Table A.6.6.3.3.1-1: Supported test configurations in SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1 for UE configured with highSpeedMeasFlag-r16

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.6.3.3.1-2: General test parameters for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1 for UE configured with highSpeedMeasFlag-r16

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 2 | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in Tables A.6.6.3.3.1-2 and A.6.6.3.3.1-3. |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 | As specified in Clause Table 9.1.2-1. Per-UE gap pattern. |
| NR measurement quantity |  | SS-RSRP | Measurement quantity for Cell 1 |
| Inter-RAT E-UTRAN measurement quantity |  | RSRP | Measurement quantity for Cell 2 |
| b2-Threshold1 | dBm | Note 1 | SS-RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| b2-Threshold2EUTRA | dBm | -97 | E-UTRAN RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | DRX.6 | DRX cycle configurations DRX.6 is defined in Table A.3.3.1-6. |
| T1 | s | 5 |  |
| T2 | s | 5 |  |
| Note 1: Values are defined in Table A.6.6.3.3.1-3 |  |  |  |

Table A.6.6.3.3.1-3: PCell specific test parameters for SA inter-RAT E-UTRA event triggered reporting in DRX with PCell in FR1 for UE configured with highSpeedMeasFlag-r16

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  |  | 1, 2, 3 | FDD |  |
|  |  |  |  | 4, 5, 6 | TDD |  |
| TDD Configuration |  | SCS=15 KHz |  | 2, 5 | TDDConf.1.1 |  |
|  |  | SCS=30 KHz |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |
|  |  |  |  | 2, 5 | 10: NRB,c = 52 (TDD) |  |
|  |  |  |  | 3, 6 | 40: NRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |
| CORSET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  |  | 3, 6 | SSB.2 FR1 |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | -98 |  |
|  |  |  |  | 3, 6 | -95 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -106 |  |
| NocNote2 |  |  | dBm/SCS | 1, 2, 4, 5 | -106 |  |
|  |  |  |  | 3, 6 | -103 |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 18 | -2 |
| Ês/IotNote3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 18 | -2 |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -108 |
|  |  |  |  | 3, 6 | -85 | -105 |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -108 |
|  |  |  |  | 3, 6 | -85 | -105 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | -59.98 | -75.92 |
|  |  |  | dBm/38.16 MHz | 3, 6 | -53.88 | -69.82 |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.6.3.3.1-4: E-UTRAN neighbour cell specific test parameters for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1 for UE configured with highSpeedMeasFlag-r16

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -106 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 19 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 19 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| IoNote5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -78.22+10log (NRB,c /50) | -59.16+10log (NRB,c /50) |
| Propagation Condition Note6 |  | 1, 2, 3, 4, 5, 6 | AWGN1944 |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |

A.6.6.3.3.2	Test Requirements

In the test, the UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 4.8s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

------------------------------------------------------------- End of change 1 ------------------------------------------------------------

----------------------------------------------------- Beginning of Change 2 ------------------------------------------------------------

A.8.4.2.9	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16

A.8.4.2.9.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the NR inter-RAT cell search requirements in clause 8.1.2.4.21of TS 36.133 [15] for E-UTRAN FDD-NR measurements and clause 8.1.2.4.22 of TS 36.133 [15] for E-UTRAN TDD-NR measurements when UE is configured with highSpeedInterRAT-NR-r16.

In this test, there are two cells: E-UTRA cell 1 as PCell on E-UTRA RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 1. The test parameters are given in Tables A.8.4.2.9.1-1, A.8.4.2.9.1-2, A.8.4.2.9.1-3 and A.8.4.2.9.1-4.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) [16] is used. In the measurement configuration the UE shall be indicated to report the SSB index of the identified NR cell. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Table A.8.4.2.9.1-1: NR inter-RAT event triggered reporting tests with SSB index reading for FR1 for UE configured with highSpeedInterRAT-NR-r16

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.8.4.2.9.1-2: General test parameters for NR inter-RAT event triggered reporting for FR1 with SSB time index detection for UE configured with highSpeedInterRAT-NR-r16

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 | One E-UTRA carrier frequency is used. |
| NR RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 | One FR1 NR carrier frequency is used. |
| Active cell |  | 1, 2, 3, 4, 5, 6 | E-UTRA cell 1 (PCell) | E-UTRA cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | 1, 2, 3, 4, 5, 6 | NR cell 2 | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2, 3, 4, 5, 6 | 0 | As specified in clause Table 8.1.2.1-1 of TS 36.133 [15]. |
| Measurement gap offset |  | 1, 2, 3, 4, 5, 6 | 39 | As specified in TS 36.331 [16]. |
| b2-Threshold1 | dBm | 1, 2, 3, 4, 5, 6 | Note 1 | E-UTRA RSRP threshold for E-UTRA RSRP measurement on cell 1 for event B2 [16] |
| b2-Threshold2NR | dBm | 1, 2, 3, 4, 5, 6 | Note 2 | SS-RSRP threshold for SS-RSRP measurement on cell 2 for event B2 [16] |
| Hysteresis | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| CP length |  | 1, 2, 3, 4, 5, 6 | Normal |  |
| TimeToTrigger | s | 1, 2, 3, 4, 5, 6 | 0 |  |
| Filter coefficient |  | 1, 2, 3, 4, 5, 6 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3, 4, 5, 6 | DRX.6 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | 1, 4 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2, 3, 5, 6 | 3s | Synchronous cells. |
| T1 | s | 1, 2, 3, 4, 5, 6 | 5 |  |
| T2 | s | 1, 2, 3, 4, 5, 6 | 5 |  |
| Note 1: The value of b2-Threshold1 is defined in Table A.8.4.2.9.1-3 Note 2: The value of b2-Threshold2NR is defined in Table A.8.4.2.9.1-4 |  |  |  |  |

Table A.8.4.2.9.1-3: E-UTRAN PCell specific test parameters for NR inter-RAT event triggered reporting with NR neigbour cell in FR1 with SSB time index detection for UE configured with highSpeedInterRAT-NR-r16

| Parameter | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| b2-Threshold1 | dBm | 1, 2, 3, 4, 5, 6 | -79 |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | 6 | 17 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | 6 | 17 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -98 | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -98 | -87 |
| IoNote5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -69.25+10log (NRB,c /50) | -59.13+10log (NRB,c /50) |
| Propagation Condition Note6 |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |

Table A.8.4.2.9.1-4: NR neighbour cell specific test parameters for NR inter-RAT event triggered reporting for FR1 with SSB time index detection for UE configured with highSpeedInterRAT-NR-r16

| Parameter | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| NR RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 4 | FDD |  |
|  |  | 2, 3, 5, 6 | TDD |  |
| TDD configuration |  | 2, 5 | TDDConf.1.1 |  |
|  |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel | MHz | 1, 2, 4, 5 | 10: NRB,c = 52 |  |
|  |  | 3, 6 | 40: NRB,c = 106 |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  | 1, 4 | SMTC.2 |  |
|  |  | 2, 3, 5, 6 | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1, 2, 4, 5 | 15 |  |
|  |  | 3, 6 | 30 |  |
| b2-Threshold2NR | dBm/SCS | 1, 2, 4, 5 | -99 |  |
|  |  | 3, 6 | -96 |  |
| EPRE ratio of PSS to SSS |  | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Note2 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -98 |  |
| Note2 | dBm/SCS | 1, 2, 4, 5 | -98 |  |
|  |  | 3, 6 | -95 |  |
| SS-RSRP Note 3 | dBm/SCS | 1, 2, 4, 5 | -Infinity | -91 |
|  |  | 3, 6 | -Infinity | -88 |
|  | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 |
|  | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 |
| IoNote3 | dBm/9.36MHz | 1, 2, 4, 5 | -Infinity | -65.38 |
|  | dBm/38.16MHz | 3, 6 | -Infinity | -61.06 |
| Propagation Condition |  | 1, 2, 4, 5 | AWGN1944 |  |
|  |  | 3,6 | AWGN3334 |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |

A.8.4.2.9.2	Test Requirements

The UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than 4.8s from the beginning of time period T2. The UE is required to read the neighbour cell SSB index and report the acquired SSB index in this test.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

------------------------------------------------------------- End of change 2 ------------------------------------------------------------