3GPP RAN WG4 Meeting #97-e		R4-2017598

Electronic meeting, 2nd – 13th November 2020

Agenda item:	13.1.1

Source:	Apple Inc., Keysight Technologies, Rohde & Schwarz, MVG Industries

Title:	TP to TR38.884 on High DL and Low UL power test cases

WI/SI:	FS_FR2_enhTestMethods

Release:	Rel-17

Document for:	Discussion

# 1	Introduction

The TR38.884 [1] can be updated with a number of intermediate agreements reached as part of the discussion of the high DL / low UL power test cases, as captured in [2] and [3].  The technical details of these agreements are based on company analyses submitted to 3GPP in [4], [5], [6], [7].

This contribution is revised from R4-2014919 to capture company contributions and comments received during the RAN4 #97e meeting [8], [9], [10], [11], [12].

Proposal 1:	It is proposed to approve the text proposal related to the high DL and low UL power test cases objective.

# 2	References

- R4-1913071, “TR38.884 skeleton for FR2 test method enhancement,” Apple Inc., 3GPP RAN4 #92bis, October 2019

- R4-1913070, “WF on FR2 test methodology enhancement,” Apple Inc., CAICT, 3GPP RAN4 #92bis, October

- R4-2012713, “WF on the high DL power and low UL power test cases objective,” Apple, 3GPP RAN4 #96e, August 2020

- R4-2010856, “Summary and Further Results on Impact of phase variation on beam pattern for NF test method,” MVG Industries, Sony

- R4-2011218, “On Test methodology for high DL power and low UL power test cases,” Keysight Technologies

- R4-2011281, “Views on test methods for high DL power and low UL power TCs,” Rohde & Schwarz

- R4-2011456, “FR2 testability enhancement for UE emissions,” Qualcomm Incorporated

- R4-2014267, “Impact on beam management due to spherical wavefront in DL,” Qualcomm Incorporated, 3GPP RAN4 #97e, November 2020

- R4-2016213, “On Test methodology for high DL power and low UL power test cases,” Keysight Technologies, 3GPP RAN4 #97e, November 2020

- R4-2016377, “Impact of phase variation,” MVG Industries, Sony, 3GPP RAN4 #97e, November 2020

- R4-2016562, “Views on test methods for high DL power and low UL power TCs,” Rohde & Schwarz, 3GPP RAN4 #97e, November 2020

- R4-2017631, “Email discussion summary for [97e][331] FR2_enhTestMethods,” Moderator (Apple), 3GPP RAN4 #97e, November 2020

# Text Proposal

<< begin text proposal >>

## 5.1	High DL power and low UL power

### 5.1.1	General

The investigation of high DL power and low UL power enhancements to the FR2 test methodology includes the following aspects:  scope of test cases with high DL power and low UL power issues, enhanced test systems, including the investigation of non-permitted systems, enhancements to permitted methods, manufacturer declarations, beam management sensitivity of the DUT in near-field test system environments, and path loss comparison across system types.

Table 5.1.1-1 below provides a summary of the test cases and testability issues.

Table 5.1.1-1: Summary of test cases and testability issues

| Clause | Requirement | Testability issue | Test Metric |
| --- | --- | --- | --- |
| 6.3.1 | Minimum output power | Low UL power | EIRP (Link=TX beam peak direction, Meas=Link angle). |
| 6.3.2 | Transmit OFF power | Low UL power | TRP (Link=TX beam peak direction, Meas=TRP grid) |
| 6.5.1 | Occupied bandwidth | Low UL power | OBW (Link=TX beam peak direction, Meas=Link angle) |
| 6.5.2.3 | Adjacent channel leakage ratio | Low UL power | TRP (Link=TX beam peak direction, Meas=TRP grid). |
| 6.5.3.2 | Additional spurious emissions | Low UL power | TRP (Link=TX beam peak direction, Meas=TRP grid). |
| 7.4 | Maximum input power | Hidh DL power | EIS (Link=RX beam peak direction, Meas=Link angle). |
| 7.5 | Adjacent channel selectivity (case 1) | High DL power | EIS (Link=RX beam peak direction, Meas=Link angle) |
| 7.5 | Adjacent channel selectivity (case 2) | High DL power | EIS (Link=RX beam peak direction, Meas=Link angle) |
| 7.6.2 | In-band blocking | High DL power | EIS (Link=RX beam peak direction, Meas=Link angle) |
| 7.9 | Receiver spurious emissions | Low UL power | TRP (Link=TX beam peak direction, Meas=TRP grid). |

The investigation of test methodology enhancements to strive to reduce the testability issues which were identified includes study of the feasibility of enhancing test systems which are permitted in TR38.810 [reference TBD] as well as test systems which are not permitted.  Non-permitted test systems according to TR38.810 [reference TBD] are not required to verify all requirements in TS38.101-2 [reference TBD].  The candidate test systems are limited to near-field (NF) based solutions and include the following solutions:

-	Direct near-field (DNF) system, where a beam peak search is necessary to perform all applicable test case procedures

-	Combined far-field/near-field (CFFNF) system utilizing a transform-based approach to correct the incurred path loss, where beam peak direction and UE beamlock function (UBF) activation are performed based on the far-field method and then test case procedures are performed based on the near-field method.

-	Combined far-field/direct-near-field (CFFDNF) system utilizing a correction for the array displacement from the center of the DUT, where beam peak direction and UE beamlock function (UBF) activation are performed based on the far-field method and then test case procedures are performed based on the direct near-field method. Applicability to EIRP measurements is FFS.

### 5.1.2	Beam management sensitivity study of NF based solutions

#### 5.1.2.1	Simulation assumptions

For NF based solutions, where beam peak search is necessary to perform all applicable test case procedures, an evaluation of UE beam management sensitivity to magnitude/phase variation of the DL signal is needed.  Two assumptions are made about the NF based system:

-	Beam peak search is performed in the NF (i.e. DNF system); OR

-	Beam peak search is first performed in the FF/IFF and test case is executed in the NF (i.e. CFFNF system).

Using the spherical coverage measurement grid assumptions shown in Table 5.1.2.1-1, evaluations were performed of the UE beam management sensitivity in terms of simulated radiated performance metrics for each of the assumptions.

Table 5.1.2.1-1: Beam management sensitivity simulation assumptions

| Parameter | Value | Notes |
| --- | --- | --- |
| Spherical coverage Measurement Grids baseline assumption | Annex G.1.1 in TR38.810 |  |
| Antenna array | - 8x2 and 4x1 - Antenna element HPBW: {260/130, 90/90} deg | Element near-field assumption is implementation specific |
| Simulated DUT | Two antenna arrays are integrated in the UE for the spherical coverage analyses - Antenna panels are studied with Nz x Ny with Nz>Ny, e.g., 8x2 corresponds to Nz = 8 and Ny = 2 - The implementation loss for the antenna near the front is 0dB less than that for the antenna near the back - The antenna in the back is on the opposite side of the UE (mirrored around (0,0,0)). | See Figure 5.1.2.1-1 for example positions of two antenna arrays |
| Beam steering | - In the xy plane, assume 45º beam steering granularity (AZ from -45º to +45º) - In the xz plane, assume 22.5o beam steering granularity (EL from -90º to 90º) |  |
| Offsets | - Various antenna offsets (yoffset, zoffset) beyond 7.5cm in radius (12.5cm max) - For TRP analysis, model random antenna offsets anywhere within the 30cm spherical QZ | Offset is defined with respect to the center of antenna array |
| Range Lengths | - 30cm, 20m (more range lengths are not precluded) - Goal is to eventually determine min. range length and MU for performing spherical coverage tests in DNF | Defined as distance between centre of QZ/positioning axes and measurement probe |
| Test methodology | - DNF (while taking path loss offsets into account) - CFFNF |  |
| Sampling grid | Study finer than 7.5deg step size for constant-step size grids | Parametric studies to show convergence for the selected assumption |

Figure 5.1.2.1-1 below illustrates example positions of two antenna arrays in the simulated DUT.

Figure 5.1.2.1-1: Simulated DUT antenna assumptions for beam management sensitivity study

#### 5.1.2.2	Simulation results

Table 5.1.2.2-1 below summarizes the results from simulations of beam management sensitivity of a DNF system (i.e. beam peak search is performed in the NF).

Table 5.1.2.2-1: Beam management sensitivity results of a DNF system

| Company label | Swept parameters | Beam management performance maximum ∆ relative to reference (dB) |  |  | Notes |
| --- | --- | --- | --- | --- | --- |
|  |  | Beam peak | 50% CDF | TRP |  |
| Company A | Array: 8x2 Range: {0.2, 0.4, 0.8} m Offset: {0, 0.05, 0.10} m HPBW: {90/90} | 2.5 | Not analyzed | Not analyzed | There is approximately 2.5 dB of BP error when range length is reduced to 0.2m in presence of the module offset mentioned above. There is also significant perturbation of the CDF curve. CDF statistics start to converge when the range length is at least 4 times the offset. |
| Company B | Array: 8x2, 4x1 Range: {0.25, 0.3, 0.45, 20} m Offset: {0.125 in y, 0.125 in z, 0.09 in y & z} m HPBW: {260/130} | 7.0 | 1.0 | TRP analyzed separately | The EIRP beam peak (100%-ile EIRP) and direction cannot be measured accurately with the direct NF methodology |
| Company B | Array: 8x2 Range: 0.2 m Offset: 0.15 m in x, y, z HPBW: {260/130} |  |  | 0.66 dB systematic 0.46 dB RSS’ed | Large uncertainties can be observed for TRP for measurements performed in the NF utilizing the black back box approach |
| Company C | Array: 4x1 Range: {100, 4.2, 0.9, 0.45, 0.3} m Offsets: not specified Full phone model (including the PCB and phone house) has been considered | 0.3 | 0 | Not clear whether 0.1 or 0.4 | Figure of merits such as EIRP, TRP, and Spherical Coverage are not influenced dramatically from range length |
| Company B | Array: {4x1, 8x2} Range: 0.25 m Offset: {0, 0.125, 0.9} m in y, z HPBW: {90/90} | 4.2 |  |  | UE selected different beam between NF beam peak direction and FF beam peak direction |
| Company B | Array: {4x1, 8x2} Range: 0.25 m Offset: {0, 0.53, 0.75} m in x, y, z HPBW: {90/90} | 10.4 |  |  | UE select the same beam in the NF as in the FF more often, we still see concerning trends with the peak EIRP deltas |
| Company B | Reuse assumptions used by Company A: Array: 8x2 Range: {0.2, 0.4, 0.8} m Offset: {0, 0.05, 0.10} m HPBW: {90/90} | 2.5 | 1.2 |  | Simulations were performed to establish alignment with another company |

Table 5.1.2.2-2 below summarizes the results from simulations of beam management sensitivity of a CFFNF system (i.e. beam peak search is first performed in the FF/IFF and test case is executed in the NF).

Table 5.1.2.2-2: Beam management sensitivity results of a CFFNF system

| Company and reference | Swept parameters | Beam management performance maximum ∆ relative to reference (dB) |  |  | Notes |
| --- | --- | --- | --- | --- | --- |
|  |  | Beam peak | 50% CDF | TRP |  |
| Company B (“Black box with transform approach”) | Array: 8x2, 4x1 Range: {0.22 – 0.30} m Offset: {0, 0.50, 0.10, 0.125} m | Max µ = 0.2 Max σ = 0.3 | Not analyzed | Not analyzed | These results were obtained using a transform-based approach to correct the incurred path loss. Feedback from industry is requested whether to continue efforts in terms of simulations and empirical investigations on this enhanced NF methodology with transform utilizing black-box approach |
| Company B (“Black & White box with transform approach”) | Array: 8x2, 4x1 Range: {0.22 – 0.30} m Offset: {0, 0.50, 0.10, 0.125} m | Max µ = 0.1 Max σ = 0.3 | Not analyzed | TRP analyzed separately | These results were obtained using a transform-based approach to correct the incurred path loss. Feedback from industry is requested whether to continue efforts in terms of simulations and empirical investigations on this enhanced NF methodology with transform utilizing the white&black-box approach |
| Company B (“TRP with compensation for antenna offset”) | Array: 8x2 Range: 0.2 m Offset: 0.15 m in x, y, z HPBW: {260/130} |  |  | 0.02 dB systematic 0.21 dB RSS’ed | These results were obtained using the DNF methodology with declared offset; alternatively, these results could be obtained using a transform based approach to estimate the phase center offset. With the offset of the antenna array known, e.g., estimated with the enhanced NF methodology introduced in this contribution, very accurate TRP measurements in the NF can be made with a TRP offset compensation approach |
| Company C | Array: 4x1 Range: {100, 4.2, 0.9, 0.45, 0.3} m Offsets: not specified Full phone model (including the PCB and phone house) has been considered | 0.3 | 1.0 | 0.8 | These results were obtained using the DNF methodology. Figure of merits such as EIRP, TRP, and Spherical Coverage are not influenced dramatically from range length Full phone model (including the PCB and phone house) has been considered |

While it has always been argued that TRP can be tested in the near-field due to conservation of power, no clear measurement uncertainty analyses have been presented to quantify the errors. In this section, we briefly present our findings for measurement uncertainties when testing TRP in the near field.

An analysis of the impact on measurement uncertainty by testing TRP in the NF was performed according to the assumption for TRP offsets in Table 5.1.2.1-1.  In this analysis, near-field effects of the antenna pattern were taken into account. Figure 5.1.2.2-1 below illustrates the differences in the 8x2 antenna pattern at the 2D2/λ distance (a) and at 1/8th of that distance (b).

a)b)

Figure 5.1.2.2-1: Radiation pattern of the 8x2 antenna array at 2D2/λ FF distance (a) and in NF at 1/8th of FF distance (b)

Table 5.1.2.2-3 below summarizes the impact of the approaches with and without offset correction on TRP MU.

Table 5.1.2.2-3: Impact of TRP measurement with and without offset correction on MU

| Range Length (cm) | With Offset Correction |  | Without Offset Correction |  |
| --- | --- | --- | --- | --- |
|  | Mean TRP Error (dB) | TRP Std. Dev. (dB) | Mean TRP Error (dB) | TRP Std. Dev. (dB) |
| 20 | 0.02 | 0.13 | 0.40 | 0.26 |
| 25 | 0.03 | 0.06 | 0.24 | 0.15 |
| 28 | 0.03 | 0.04 | 0.19 | 0.11 |
| 32 | 0.03 | 0.02 | 0.14 | 0.08 |
| 43 | 0.03 | 0.02 | 0.08 | 0.04 |
| 100 | 0.04 | 0.02 | 0.01 | 0.01 |

Additionally, CDF curves for the various simulation results are presented in Figure 5.1.2.2-2 below.

Figure 5.1.2.2-1: Distribution of simulated TRP measurements with and without offset correction

Based on the submitted simulation results, the following conclusions can be drawn:

-	TBD

### 5.1.3	Manufacturer declarations

If a manufacturer declaration is used to inform or optimize a test system parameter, and the DUT is positioned in the test system according to parameters which are informed by this declaration, then the DUT is measured assuming a “white box” configuration.  If no manufacturer declaration is used, and the DUT is positioned in the test system according to common procedures, then the DUT is measured assuming a “black box” configuration.

The list of potential candidate vendor declarations and how they map to a particular test system parameter includes the following:

-	For the “white box” configuration:

-	Location of the active panels that yields the TX and RX beam peaks (applicable if the enhanced test methodology does not need to perform beam peak search/spherical coverage test cases)

-	Location of the active panels in any UL/DL test direction and the detailed locations of the panels within the DUT (applicable if the enhanced test methodology does need to perform beam peak search/spherical coverage test cases)

-	For the “black box” configuration:

-	No vendor declaration in terms of antenna panel locations is needed

Table 5.1.3-1 summarizes the path loss comparison between “white box” and “black box” configuration across IFF/DFF and NF system types.

Table 5.1.3-1: Path loss comparison between “white box” and “black box” configuration

| f (GHz) | Antenna Config. 1, 2, and 3 - BLACK BOX - (PC3 Devices: D=5cm) |  | Antenna Config. 1 and 2 - WHITE BOX - (PC3 Devices: D=5cm) |  |
| --- | --- | --- | --- | --- |
|  | IFF/DFF | NF | DFF | NF |
|  | Path Loss with 1m range length | Path Loss with 0.22m range length | Path Loss with 0.88m range length | Path Loss with 0.28m range length |
| 24.25 | 60.16 | 46.86 | 59.01 | 48.93 |
| 30 | 62.01 | 48.71 | 60.85 | 50.78 |
| 40 | 64.51 | 51.21 | 63.35 | 53.28 |
| 43.5 | 65.24 | 51.94 | 64.08 | 54.00 |
| 52.6 | 66.89 | 53.59 | 65.73 | 55.65 |

Based on the analysis shown in Table 5.1.3-1, it can be concluded that a “white box” is not deemed a feasible enhancement of the methodology.

In a NF system, the NF TX beam peak (BP) direction for an offset antenna is not necessarily the same as the FF TX BP direction; however, the knowledge of the antenna phase centre offset can be leveraged to measure at the NF BP direction as illustrated in Figure 5.1.3-1 below.  Unlike the “black box” approach with unknown antenna phase centre offset, this approach does not require a FF probe to steer and lock the beam as the knowledge of the offset together with the probe antenna pattern will allow the calculation of the optimized DUT orientation so that the DUT selects the proper beam.

Figure 5.1.3-1: Illustration of NF Testing utilizing the “black and white box” approach

The assumption for this “black & white box” testing approach is that the antenna phase centre offset for the antenna panel that corresponds to the FF beam peak is known and declared, i.e., following the “white box” approach discussed earlier. On the other hand, however, it is assumed that the geometric centre of the DUT is aligned with the centre of the QZ, i.e., following the “black box” approach. This approach would have the same advantages as the “black box” approach over the “white box” approach in terms of complexity, test time, MU, and improvements of the relaxations and is summarized in Table 5.1.3-2 below.

Table 5.1.3-2: Comparison between the “black box” and “black & white box” approaches

| Approach | Knowledge of FF BP Direction (from Meas.) | Declaration of Antenna Phase Centre Offset of Antenna yielding BP | Need for FF probes and UBF | Need for local searches around NF BP | Measurements at different Radii | Test Time Impact | Estimated maximum Improvement of Relaxation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Black Box | Yes | No | Yes | Yes | Yes (x3) | Medium (local searches & 3 different radii) | ~14dB (for 20cm range length) only considering Free Space Path Loss |
| Black & White Box | Yes | Yes | No | No | Yes (x2) | Low (2 different radii in fixed NF BP Direction) | ~14dB (for 20cm range length) only considering Free Space Path Loss |
| Black & White Box with CFFDNF [TRP only] | Yes | Yes | Yes | No | No | None | ~14dB (for 20cm range length) only considering Free Space Path Loss |

Applicablity of the “black & white box” assumption with CFFDNF to EIRP measurements is FFS.

Whether NF based solutions consider the “black box” or “black & white box” assumptions as a baseline is FFS.

### 5.1.4	Improvement of permitted methods

Table 5.1.4-1 below provides a preliminary list of potential improvement of permitted methods based on the analysis provided by one company and are applicable to the frequency range of 24.25 – 43.5 GHz.

Table 5.1.4-1: Summary of potential improvement of permitted methods by test case (24.25 – 43.5 GHz)

| Test Case | Test Metric | Regulatory related | TS 38.521-2 Test Requirements | Potential improvement |
| --- | --- | --- | --- | --- |
| 7.4 Maximum input level | EIS | No | 26dB relaxation for 24.25 ~ 29.5 GHz and 34 dB relaxation for 37 ~ 40 GHz with respect to minimun requirements. | ~ 6dB for FR2a ~10dB for FR2b |
| 7.5 Adjacent channel selectivity | EIS | Yes, for case 1. | Added relaxations for ACS Case 1: 50MHz: 1.8dB relaxation for power in transmission BW and interferer for band n260. 100MHz: 4.8dB relaxation for power in transmission BW and interferer for band n260. 200MHz and 400MHz are deemed not testable. Decision not test ACS case 2. | Similar improvements as for TC 7.4 All single carrier bandwidth could be testable 400 MHz, without relaxations up to 200 MHz |
| 6.3.2 Transmit OFF power | TRP | Yes | Relaxations for n257: 21.4dB @ 50MHz, 24.4dB @ 100MHz, 27.4dB @ 200MHz and 30.4dB @ 400MHz. Relaxations for other bands are still TBD. | ~ 10dB for FR2a and FR2b |
| 6.5.2.3 Adjacent channel leakage ratio | EIRP | Yes | Relaxation for n257, n258 and n261: 0dB, except for 200Mhz (0.5dB in one test ID) and 400MHz (between 1.5 and 3.5dB) | Improvements remove required relaxations from TC |
| 6.3.1 Minimum output power | EIRP | No | No relaxation for PC1. For other power classes, relaxation varies from 0dB to 13.5dB depending on the operating band and channel bandwidth. | ~ 10dB for FR2a and FR2b FR2a requirements testable without relaxations |

Criteria for considering permitted and non-permitted methods are TBD.

<< end text proposal >>