3GPP TSG-RAN WG4 Meeting # 97-e 												R4-2017622

Electronic Meeting, Nov. 2 – Nov .13, 2020

Agenda item:			7.9.4

Source:	Moderator (Samsung)

Title:	Email discussion summary for [97e][324] NR_eMIMO_Demod

Document for:	Information

Introduction

The scope of this email discussion, mainly focuses to finalize the detailed test parameters for identified performance requirements include demodulation and CSI, and introduce corresponding test cases into specifications

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: Discuss the details test parameters to facilitate the test case setup for requirements.

- 2nd round: Further discuss the left open issue and CR, pending on the progress on 1st round.

Topic #1: PDSCH demodulation requirements (Multi-Panel/TRP transmission schemes)

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2014248 | Apple | Draft CR for eMIMO demod requirements - General and Applicability rule |
| R4-2014556 | Intel | Proposal 1:Schedule PDSCH data in TDD special slots in multi-DCI based multi-TRP Tx test cases Proposal 2:Schedule PDSCH data in DL slots which carries TRS Tx in multi-DCI based multi-TRP Tx test cases. |
| R4-2014557 | Intel | Proposal 1: Define requirements for single-DCI multi-TRP Tx with negative TO and frequency offset (test case 1a) with following applicability rule: - If UE only supports single-DCI based multi-TRP transmission for eMBB, it should be tested with test cases 1a and 1b - If UE can support both single-DCI and multi-DCI for eMBB, it should be tested with test case 2a and 1b Proposal 2: Schedule PDSCH data in TDD special slots in single-DCI multi-TRP SDM test cases. Proposal 3: Schedule PDSCH data in DL slots which carries TRS Tx in single-DCI multi-TRP SDM test cases. |
| R4-2014558 | Intel | Proposal 1:Define performance requirements for FDM scheme A and inter-slot TDM repetition transmission schemes with the following test applicability rule: FDM scheme is skipped if UE passes the multi-DCI based multi-TRP Tx requirements and TDM scheme is skipped if UE passes URLLC slot aggregation requirements and anyone of the other multi-TRP Tx requirements. Proposal 2: Define performance requirements for FDM scheme A with wideband precoding configuration and inter-slot TDM scheme with 2 PRB precoding granularity. Proposal 3: Consider two PDSCH repetitions for test case with inter-slot TDM scheme. Proposal 4: Consider -0.5us, 2us and -0.25us, 1us TO values for requirements definition for multi-TRP repetition schemes for 15 kHz SCS and 30 kHz SCS respectively Proposal 5: Consider 200Hz and 300Hz FO values for requirements definition for multi-TRP repetition schemes for 15 kHz SCS and 30 kHz SCS respectively. Proposal 6: Use MCS 13 from MCS Table 3 for requirements definition of multi-TRP repetition schemes. Proposal 7: Define performance requirements for repetition Tx schemes with the following QCL assumption: 1000 DMRS antenna port with TCI state #1, 1001 DMRS antenna port with TCI state #2 Proposal 8: Use 1% BLER as a test metric for single-DCI based multi-TRP repetition Tx schemes performance requirements Proposal 9: Define the following test cases per each duplex mode for multi-TRP repetition schemes: - Test 1a: Single-DCI based FDM scheme A with frequency offset and positive time offset - Test 1b: Single-DCI based inter-slot TDM A with frequency offset and positive time offset Proposal 10: Adopt proposed above procedure to organize specification structure for multi-TRP Tx schemes. |
| R4-2014559 | Intel | CR to TS 38.101-4: Performance requirements single-DCI based multi-TRP Repetition Tx schemes |
| R4-2014741 | Samsung | eMBB test scope Proposal 1: eMBB test cases: Introduce following test cases for Multi-TRP/Panel transmission schemes (eMBB) to cover frequency offset, negative and positive time offset to verify proper UE processing and performance - Test 1a Single-DCI with frequency offset and negative time offset and overlapping scheduling - Test 1b Single- DCI with positive time offset and overlapping scheduling. - Test 2a Multi- DCI with frequency offset and negative time offset and non-overlapping scheduling Proposal 2: eMBB test cases: Test applicable can be introduced for above test cases: - If UE only supports single-DCI based multi-TRP transmission for eMBB, it should be tested with test case 1a and test case 1b - If UE can support both single-DCI and multi-DCI for eMBB, it should be tested test 2a and test 1b URLLC test scope Proposal 3-URRC test cases-Test scope: Introducing PDSCH demodulation requirements for transmission schemes related to URLLC operation - Test schemes: Only FDM scheme A and inter-slot TDM scheme - Test applicable rules can be applied (not our preference, can be accepted as comprise): FDM scheme is skipped if UE passes the multi-DCI based multi-TRP Tx requirements and TDM scheme is skipped if UE passes URLLC slot aggregation requirements and anyone of the other multi-TRP Tx requirements Proposal 4: URLLC test cases- test parameters: - Using option 1 to introduce test cases: - Test 1a: Single-DCI based FDM scheme A with frequency offset and negative time offset - Test 1b: Single-DCI based inter-slot TDM with positive time offset - MCS: MCS 19 (Option2) - Test metric: 1% BLER (Option 1) |
| R4-2014742 | Samsung | Simulation results summary for Rel-16 eMIMO WI |
| R4-2014743 | Samsung | Observation 2: The performance with frequency offset is degraded slightly than with no frequency offset case. Observation 3: The impact with negative time offset is more significantly than with positive time offset case. Observation 4: The FDD performance is similar with TDD performance. |
| R4-2014744 | Samsung | Observation 2: The performance of FDD is similar to TDD ones. Observation 3: The performance of ’antenna configuration 2x4’ achieves about 5dB gain than ’antenna configuration 2x2’ case. |
| R4-2014745 | Samsung | Simulation results for Single-DCI URLLC schemes |
| R4-2015128 | MTK | Simulation results on PDSCH performance requirements for multi-DCI based multi-TRP transmission |
| R4-2015648 | Huawei, HiSilicon | Proposal 1: Not introduce test 1a for Multi/Single-DCI PDSCH requirement |
| R4-2015649 | Huawei, HiSilicon | Simulation results of PDSCH requirements for Multi-DCI transmission scheme |
| R4-2015650 | Huawei, HiSilicon | Simulation results of PDSCH requirements for Single-DCI SDM scheme |
| R4-2015651 | Huawei, HiSilicon | Proposal 1: Option 1 for the number of test cases for FDM scheme A and inter-slot TDM Proposal 2: Option 2 MCS 19 for MCS configuration Proposal 3: Reuse the test metric of SNR point of 70% maximum throughput Proposal 4: Consider the possibility of reusing the parameter configuration of URLLC slot aggregation |
| R4-2015652 | Huawei, HiSilicon | Simulation Results |
| R4-2015653 | Huawei, HiSilicon | Draft CR for 38.101-4: Introduction of PDSCH requirement with Single-DCI based SDM scheme |
| R4-2015654 | Huawei, HiSilicon | DraftCR on PDSCH requirement with Multi-DCI based multi-TRP |
| R4-2015830 | Ericsson | Draft CR: PDSCH FRC for eMIMO sDCI/mDCI-based SDM transmission |
| R4-2015831 | Ericsson | Simulation results of single-DCI based SDM transmission |
| R4-2015832 | Ericsson | Proposal 1: RAN4 defines three test cases with the following applicability rule: If UE supports single-DCI based SDM transmission scheme (i.e., singleDCI-SDM-scheme) but not support multi-DCI based SDM transmission scheme (i.e., multiDCI-MultiTRP), it should be tested with test case 1a and test case 1b. If UE supports both single-DCI based SDM transmission scheme (i.e., singleDCI-SDM-scheme) and multi-DCI based SDM transmission scheme (i.e., multiDCI-MultiTRP), it should be tested test 1b and test 2a. Test 1a/1b/2a is given as follows: - If UE only supports single-DCI based multi-TRP transmission for eMBB, it should be tested with test cases 1a and 1b - If UE only supports single-DCI based multi-TRP transmission for eMBB, it should be tested with test cases 1a and 1b - If UE only supports single-DCI based multi-TRP transmission for eMBB, it should be tested with test cases 1a and 1b Proposal 2: Introduce new FRC for multi-DCI based SDM transmission requirements with 26PRB per codeword for FDD and with 53PRB per codeword for TDD. It also configures two TRS configurations every 20ms. Proposal 3: Introduce new FRC for single-DCI based SDM transmission requirements with two TRS configurations every 20ms. Proposal 4: Schedule PDSCH in the TDD special slots. Proposal 5: For multi-DCI SDM transmission, the same precoding is applied for both codewords, where PDSCH & PDSCH DMRS Precoding is configured with Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity. |
| R4-2015833 | Ericsson | Simulation results of multi-DCI based SDM transmission |
| R4-2015834 | Ericsson | Proposal 3-URRC test cases-Test scope: Introducing PDSCH demodulation requirements for transmission schemes related to URLLC operation - Test schemes: Only FDM scheme A and inter-slot TDM scheme Proposal 1: RAN4 defines PDSCH demodulation requirements with sDCI-based FDM Scheme A for UE capable of supportFDM-SchemeA. Proposal 2: FDM scheme A requirement is skipped if UE passes the multi-DCI based SDM requirements. Proposal 3: For FDM scheme A requirements, RAN4 reuses the same test setup and metric as the mDCI-based SDM transmission requirements, that is - Configure MCS 19 - Use the 70% of maximum throughput as the test metric - Set the same requirements (SNR) as multi-DCI based transmission scheme, and Configure frequency offset and negative time offset Proposal 4: RAN4 defines PDSCH demodulation requirements with sDCI-based inter-slot TDM for UE capable of supportInter-slotTDM. Proposal 5: Inter-slot TDM requirement is skipped if UE passes URLLC slot aggregation requirements, discussed in Rel-16 URLLC WI performance, and anyone of the other multi-TRP Tx requirements. Observation 1: sDCI-based inter-slot transmission shows performance gain compared with single-TRP based transmission. Observation 2: sDCI-based iner-slot transmission performance is affected by frequency offset between two TRPs, especially for higher MCS. Proposal 6: Set the same MCS (13, 16, or 19) as PDSCH slot aggregation defined in Rel-16 URLLC WI performance part. Proposal 7: Use the same test metric as PDSCH slot aggregation defined in URLLC WI, that is, 1% BLER. Proposal 8: Configure frequency offset and positive time offset. |

Open issues summary

Last RAN4 meeting agreements in WF R4-2012662 and R4-2012663

List of open issues:

- Sub-Topic 1-1: Generic test set-up

  - Issue 1-1-1: Scheduling PDSCH data in TDD special slots for single/multi-DCI

  - Issue 1-1-2: Scheduling PDSCH data in DL slots which carriers TRS Tx for single/multi-DCI

- Sub-Topic 1-2: Test parameters for Multi-DCI based multi-TRP/Panel transmission schemes (eMBB)

  - Issue 1-2-1: Number of test cases for single-DCI/multi-DCI

  - Issue 1-2-2: FRC for single /multi-DCI based SDM transmission

  - Issue 1-2-3: Precoding scheme for multi-DCI SDM

- Sub-Topic 1-3: Test parameters for Single-DCI based multi-TRP/Panel transmission schemes (URLLC)

  - Issue 1-3-1: Transmission schemes

  - Issue 1-3-2: Number of test cases

  - Issue 1-3-3: PRB precoding granularity

  - Issue 1-3-4: Antenna Port indices

  - Issue 1-3-5: timing offset among multi-panel/TRP

  - Issue 1-3-6: Frequency offset among multi-panel/TRP

  - Issue 1-3-7: Tx occasions for inter-slot TDM

  - Issue 1-3-8: Requirement setup for FDM scheme A

  - Issue 1-3-9: MCS for FDM scheme A and inter-slot TDM scheme

  - Issue 1-3-10: Test Metric for FDM scheme A and inter-slot TDM scheme

  - Issue 1-3-11: Simulation Assumption

Sub-topic 1-1: Generic test set-up

Issue 1-1-1: Scheduling PDSCH data in TDD special slots for single/multi-DCI

- Proposals

  - Option 1: Yes (Intel, Ericsson)

- Recommended WF

  - Companies’ feedback needed for above proposal

Issue 1-1-2: Scheduling PDSCH data in DL slots which carriers TRS Tx for single/multi-DCI

- Proposals

  - Option 1: Yes (Intel)

- Recommended WF

  - Companies’ feedback needed for above proposal

Sub-topic 1-2: Test parameters for Single-DCI/Multi-DCI based multi-TRP/Panel transmission schemes (eMBB)

Issue 1-2-1: Number of test cases for single-DCI/multi-DCI

- Proposals

  - Option 1: 3 test cases per duplex mode with test applicability rule (Samsung, Intel, Ericsson)

- Test 1a: Single-DCI with frequency  offset and negative time offset and overlapping scheduling

- Test 1b: Single-DCI with positive time offset, and overlapping scheduling

- Test 2a: Multi- DCI with frequency offset and negative time offset and non-overlapping scheduling

- Applicability rule

- If UE only supports single-DCI based multi-TRP transmission for eMBB, it should be tested with test case 1a and test case 1b

- If UE can support both single-DCI and multi-DCI for eMBB, it should be tested test 2a and test 1b

  - Option 2:  only 2 test cases per duplex mode (Huawei)

- Test 1b: Single-DCI with positive time offset, and overlapping scheduling

- Test 2a: Multi- DCI with frequency offset and negative time offset and non-overlapping scheduling

- Recommended WF

  - 3 Companies prefer option 1 and 1 company prefer option 2; can we agree with option 1 with test applicable rules following majority approach?

Issue 1-2-2: FRC for single /multi-DCI based SDM transmission

- Proposals

  - Option 1: Introduce new FRC for Single/multi-DCI based SDM transmission requirements with

- Multi-DCI based SDM schemes

- FDD: 26PRB per codeword

- TDD: 53PRB per codeword

- Two TRS configurations every 20ms

- Single-DCI based SDM schemes

- Two TRS configurations every 20ms

- Reference channel

- R.PDSCH.1-3.2 FDD: single-DCI based SDM schemes in FDD

- R.PDSCH.1-3.3 FDD: multi-DCI based SDM schemes in FDD

- R.PDSCH.2-3.2 TDD: single-DCI based SDM schemes in TDD

- R.PDSCH.2-3.3 TDD: multi-DCI based  SDM scheme in TDD

FDD FRCs for single DCI and multi-DCI based SDM transmissions

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-3.1 FDD | [R.PDSCH.1-3.2 FDD] | [R.PDSCH.1-3.3 FDD] |  |  |
| Channel bandwidth | MHz | 10 | 10 | 10 |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 |  |  |
| Number of allocated resource blocks | PRBs | 52 | 52 | 26 |  |  |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 |  |  |
| Allocated slots per 2 frames | Slots | 19 | 19 | 19 |  |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |  |
| MCS index |  | 19 | 19 | 19 |  |  |
| Modulation |  | 64QAM | 64QAM | 64QAM |  |  |
| Target Coding Rate |  | 0.51 | 0.51 | 0.51 |  |  |
| Number of MIMO layers |  | 2 | 2 | 2 |  |  |
| Number of DMRS REs |  | 12 | 12 | 12 |  |  |
| Overhead for TBS determination |  | 0 | 0 | 0 |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A |  |  |
| For Slots i = 10, 11 |  | 39936 | 37896 | 18960 |  |  |
| For Slots i = 1,…, 9, 12, …, 19 | Bits | 42016 | 42016 | 21000 |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A |  |  |
| For Slots i = 1,…, 19 | Bits | 24 | 24 | 24 |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A | N/A |  |  |
| For Slots i = 1,…, 19 | CBs | 5 | 5 | 3 |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A |  |  |
| For Slots i = 10, 11 | Bits | 78624 | 74800 | 37440 |  |  |
| For Slots i = 1,…, 9, 12, …, 19 | Bits | 82368 | 82368 | 41184 |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 39.707 | 39.503 | 18.216 |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

TDD FRCs for single DCI and multi-DCI based SDM transmissions

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-3.1 TDD | [R.PDSCH.2-3.2 TDD] | [R.PDSCH.2-3.3 TDD] |  |  |
| Channel bandwidth | MHz | 40 | 40 | 40 |  |  |
| Subcarrier spacing | kHz | 30 | 30 | 30 |  |  |
| Allocated resource blocks | PRBs | 106 | 106 | 53 |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 4 | 4 | 4 |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 12 | 12 |  |  |
| Allocated slots per 2 frames |  | 31 | 31 | 31 |  |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |  |
| MCS index |  | 19 | 19 | 19 |  |  |
| Modulation |  | 64QAM | 64QAM | 64QAM |  |  |
| Target Coding Rate |  | 0.51 | 0.51 | 0.51 |  |  |
| Number of MIMO layers |  | 2 | 2 | 2 |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 6 | 6 | 6 |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 12 | 12 |  |  |
| Overhead for TBS determination |  | 0 | 0 | 0 |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A |  |  |
| For Slots i = 20, 21 | Bits | 83976 | 77896 | 38936 |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 27144 | 27144 | 13576 |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 83976 | 83976 | 42014 |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 24 | 24 | 24 |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6}for i from {1,…,39} | Bits | 24 | 24 | 24 |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | 4 | 4 | 2 |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | CBs | 10 | 10 | 5 |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A |  |  |
| For Slots i = 20, 21 | Bits | 160272 | 152640 | 76320 |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 53424 | 53424 | 26712 |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 167904 | 167904 | 83952 |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 118.796 | 118.188 | 59.126 |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

- Recommended WF

  - Companies’ feedback  needed for above proposal

Issue 1-2-3: Precoding scheme for multi-DCI SDM

- Proposals

  - Option 1(Ericsson): Same precoding is applied for both codewords, where PDSCH & PDSCH DMRS Precoding is configured with Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity.

- Recommended WF

  - Companies’ feedback  needed for above proposal

Sub-topic 1-3: Test parameters for Single-DCI based multi-TRP/Panel transmission schemes (URLLC)

Issue 1-3-1: Transmission schemes

- Proposals

  - Option 1(Samsung, Intel, Ericsson, Huawei): Define performance requirement for URLLC transmission schemes with test applicability rule

- Only FDM scheme A for UE capable of supportFDM-SchemeA and inter-slot TDM scheme for UE capable of supportIntel-slotTDM

- Test applicability

- FDM scheme is skipped if UE passes the multi-DCI based multi-TRP Tx requirements

- TDM scheme is skipped if UE passes URLLC slot aggregation requirements and anyone of the other multi-TRP Tx requirements

- Recommended WF

  - All companies prefer option1, agree option 1?

Issue 1-3-2: Number of test cases

- Proposals

  - Option 1 (Samsung, Huawei):

- Test 1a: Single-DCI  based FDM scheme A with frequency offset and negative time offset

- Test 1b: Single-DCI based inter-slot TDM with positive time offset

  - Option 2 (Intel):

- Test 1a: Single-DCI based  FDM scheme A with frequency offset and positive time offset

- Test 1b: Single-DCI based inter-slot TDM A with frequency offset and positive time offset

  - Option 3 (Ericsson):

- Test 1a: Single-DCI based  FDM scheme A with frequency offset and negative time offset

- Test 1b: Single-DCI based inter-slot TDM A with frequency offset and positive time offset

- Recommended WF

  - Option 3 is the hybrid of option 1 and option 2, can we take option 3?

Issue 1-3-3: PRB precoding granularity

- Proposals

  - Option 1(Intel):

- Wideband precoding for Single-DCI based  FDM scheme A

- 2 PRB precoding granularity for Single-DCI based inter-slot TDM scheme

- Recommended WF

  - Companies’ feedback  needed for above proposal

Issue 1-3-4: Antenna Port indices

- Proposals

  - Option 1(Intel):

- 1000 DMRS antenna port with TCI state#1

- 1001 DMRS antenna port with TCI state#2

- Recommended WF

  - Agree option 1

Issue 1-3-5: timing offset among multi-panel/TRP

- Proposals

  - Option 1(Intel): Considering -0.5 us, 2us and -0.25, 1us values for requirement definition for multi-TRP repetition schemes for 15KH SCS and 30 KHz SCS

- FDD with 15KHz : 2 us

- TDD with 30 KHz: 2 us?

- Recommended WF

  - Companies’ feedback needed for above proposal

Issue 1-3-6: Frequency offset among multi-panel/TRP

- Proposals

  - Option 1(Intel):

- FDD with 15KHz: 200Hz

- TDD with 30KHz: 300Hz

- Recommended WF

  - Agree option 1

Issue 1-3-7: Tx occasions for inter-slot TDM

- Proposals

  - Option 1: two repetitions (Intel, Ericsson)

  - Option 2:  Consider the possibility of reusing the parameter configuration of URLLC slot aggregation (Huawei)

- Recommended WF

  - Agree option 1. For URLLC WI, RAN4 defined the PDSCH requirement with 2 slot aggregation?

Issue 1-3-8: Requirement setup for FDM scheme A

- Proposals

  - Option 1(Ericsson):  Set the same requirements (SNR) as multi-DCI transmission scheme

- Recommended WF

  - Companies’ feedback needed for above proposal

Issue 1-3-9: MCS for FDM scheme A and inter-slot TDM scheme

- Proposals

  - Option 1(Samsung, Huawei):  MCS 19 both FDM scheme A and inter-slot TDM scheme

  - Option 2 (Ericsson):

- FDM scheme A: MCS19

- inter-slot TDM scheme : same  MCS (13,16 or 19) defined in Rel-16 URLLC WI performance part for inter-slot TDM scheme

  - Option 3 : MCS13 from MCS Table 3 (Intel)

- Recommended WF

  - Issue 1: further clarification MCS table used?  MCS table 1 or MCS table 3?

- FDM scheme: MCS 19?

- Inter-slot TDM scheme: FFS among 13/16/19,down selection one of them

Issue 1-3-10: Test Metric for FDM scheme A and inter-slot TDM scheme

- Proposals

  - Option 1: 1% BLER (BLER is calculated after all transmission) (Intel, Samsung)

  - Option 2(Ericsson):

- 70% TP for FDM scheme A

- 1% BLER (BLER is calculated after all transmission) for inter-slot TDM scheme

  - Option 3: 70% TP (Huawei)

- Recommended WF

  - Seems option2 would be possible comprise solution as the hybrid of option 1 and option 3, can we take option 2?

Issue 1-3-11: Simulation Assumption

- Proposals

  - Option 1 (Intel):

| Parameters | Simulation cases 1-1~ 1-8 | Simulation cases 2-1~ 2-8 |
| --- | --- | --- |
| Duplex mode | FDD | TDD |
| TDD UL/DL configuration | N/A | [FR1.30-1 (7DS2U, 6D+4G+4U)] FFS whether PDSCH is scheduled in the special slots |
| Timing offset of the second TRP from the first TRP | 2 μs | 2 μs |
| Frequency offset between two TRPs | 200 Hz | 300 Hz |
|  | Common serving cell parameters |  |
| Physical Cell ID | 0 |  |
| SSB position in burst | First SSB in Slot #0 |  |
| SSB periodicity | 20ms |  |
| Transmit TRP of SSB | Only from TRP1 |  |
|  | TRS#1 (configuration for TCI state #1) |  |
| Transmit TRP | TRP#1 |  |
| CSI-RS periodicity | [20] slots | [40] slots |
| CSI-RS offset | [10] for CSI-RS resources 1 and 2 [11] for CSI-RS resources 3 and 4 | [20] for CSI-RS resources 1 and 2 [21] for CSI-RS resources 3 and 4 |
| Density | 3 | 3 |
| First subcarrier index in the PRB used for CSI-RS | k0=0 | k0=0 |
| QCL Info | TCI state #0 | TCI state #0 |
| First OFDM symbol in the PRB used for CSI-RS | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 |
|  | TRS#2 (configuration for TCI state #2) |  |
| Transmit TRP | TRP#2 |  |
| CSI-RS periodicity | [20] slots | [40] slots |
| CSI-RS offset | [10] for CSI-RS resources 1 and 2 [11] for CSI-RS resources 3 and 4 | [20] for CSI-RS resources 1 and 2 [21] for CSI-RS resources 3 and 4 |
| Density | 3 | 3 |
| First subcarrier index in the PRB used for CSI-RS | k0=1 | k0=1 |
| First OFDM symbol in the PRB used for CSI-RS | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 |
| QCL Info | TCI state #0 | TCI state #0 |
|  | TCI State configuration #0 |  |
| Type 1 QCL information | SSB index #0, QCL type C |  |
| Type 2 QCL information | N/A |  |
|  | TCI State configuration #1 |  |
| Type 1 QCL information | CSI-RS resource: TRS#1, QCL type A |  |
| Type 2 QCL information | N/A |  |
|  | TCI State configuration #2 |  |
| Type 1 QCL information | CSI-RS resource: TRS#2, QCL type A |  |
| Type 2 QCL information | N/A |  |
|  | PDCCH configuration |  |
| CORESETPoolIndex | 0 (or not configured) |  |
| Symbols for PDCCH | 0, 1 |  |
| K0 | 0 |  |
| AL | 8 (576REs) |  |
|  | PDSCH configuration |  |
| PDSCH resource mapping type | Type A |  |
| Resource allocation type | Type 1 |  |
| Antenna ports index ​ | {1000} for TCI#1 and {1001} for TCI#2 |  |
| Note 1: The same SSB (index #0) is transmitted from both TRP1 and TRP2. |  |  |

|  | FDMSchemeA | Inter-slot TDM |
| --- | --- | --- |
| DMRS configuration | Type 1, 1+1 | Type 1, 1+1 |
| Start symbol and time duration | PDSCH#1: 2, 12 PDSCH#1: 2, 12 | PDSCH#1: 2, 12 PDSCH#1: 2, 12 |
| Precoding granularity | WB | 2 |
| FDD PRB allocation | PDSCH#1: PRB#0 to PRB#25 PDSCH#2: PRB#26 to PRB#51 | PDSCH#1: PRB#0 to PRB#51 PDSCH#2: PRB#0 to PRB#51 |
| TDD PRB allocation | PDSCH#1: PRB#0 to PRB#52 PDSCH#2: PRB#53 to PRB#105 | PDSCH#1: PRB#0 to PRB#105 PDSCH#2: PRB#0 to PRB#105 |

- Recommended WF

  - Companies’ feedback needed for above proposal

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Company A | Sub-Topic 1-1: Generic test set-up - Issue 1-1-1: Scheduling PDSCH data in TDD special slots for single/multi-DCI - Issue 1-1-2: Scheduling PDSCH data in DL slots which carriers TRS Tx for single/multi-DCI Sub-Topic 1-2: Test parameters for Multi-DCI based multi-TRP/Panel transmission schemes (eMBB) - Issue 1-2-1: Number of test cases for single-DCI/multi-DCI - Issue 1-2-2: FRC for single /multi-DCI based SDM transmission - Issue 1-2-3: Precoding scheme for multi-DCI SDM Sub-Topic 1-3: Test parameters for Single-DCI based multi-TRP/Panel transmission schemes (URLLC) - Issue 1-3-1: Transmission schemes - Issue 1-3-2: Number of test cases - Issue 1-3-3: PRB precoding granularity - Issue 1-3-4: Antenna Port indices - Issue 1-3-5: timing offset among multi-panel/TRP - Issue 1-3-6: Frequency offset among multi-panel/TRP - Issue 1-3-7: Tx occasions for inter-slot TDM - Issue 1-3-8: Requirement setup for FDM scheme A - Issue 1-3-9: MCS for FDM scheme A and inter-slot TDM scheme - Issue 1-3-10: Test Metric for FDM scheme A and inter-slot TDM scheme - Issue 1-3-11: Simulation Assumption |
| Apple | Sub-Topic 1-1: Generic test set-up - Issue 1-1-1: Scheduling PDSCH data in TDD special slots for single/multi-DCI We are fine to keep the same test setup as Rel-15 tests and schedule PDSCH in special slots for TDD. - Issue 1-1-2: Scheduling PDSCH data in DL slots which carriers TRS Tx for single/multi-DCI Scheduling PDSCH in slots with TRS transmission increases the code-rate. Compared to single TRP tests we would have twice the number of REs rate-matched due to TRS from 2 TRP. Since we cannot have different TBS without changing MCS in slots with PDSCH transmission, we prefer not to schedule PDSCH in TRS slots. Sub-Topic 1-2: Test parameters for Multi-DCI based multi-TRP/Panel transmission schemes (eMBB) - Issue 1-2-1: Number of test cases for single-DCI/multi-DCI We support the recommended WF by moderator and define applicability rule. - Issue 1-2-2: FRC for single /multi-DCI based SDM transmission The FRC definition is dependent on issue 1-1-1 and 1-1-2. Also related to Rel-15 maintenance CR by Ericsson, we cannot have different TBS in slots with TRS transmission without changing MCS. - Issue 1-2-3: Precoding scheme for multi-DCI SDM Firstly, would like to understand if this is for single DCI SDM or multi-DCI transmission scheme. In our opinion the random precoder for each TRP should be independently generated. Sub-Topic 1-3: Test parameters for Single-DCI based multi-TRP/Panel transmission schemes (URLLC) Has it already been agreed to define requirements with multi-TRP single DCI URLLC schemes? We prefer not to define requirements for this case. |
| Ericsson | Sub-Topic 1-1: Generic test set-up - Issue 1-1-1: Scheduling PDSCH data in TDD special slots for single/multi-DCI Support Option 1 - Issue 1-1-2: Scheduling PDSCH data in DL slots which carriers TRS Tx for single/multi-DCI Support Option 1 Sub-Topic 1-2: Test parameters for Multi-DCI based multi-TRP/Panel transmission schemes (eMBB) - Issue 1-2-1: Number of test cases for single-DCI/multi-DCI Support Option 1. It is a good compromise to considering the test coverage (positive/negative time offset between two antenna ports) and number of test cases. However we are also ok to reduce the test case further, like option 2. - Issue 1-2-2: FRC for single /multi-DCI based SDM transmission Support Option 1. For FRC, since we don’t consider TRS to derive TBS, we need to set the same TBS value for both slots where TRS is transmitted and TRS is not transmitted. - Issue 1-2-3: Precoding scheme for multi-DCI SDM Support Option 1. We suggest to capture this text in the draft CR for sDCI-based SDM transmission, if it is agreed. Sub-Topic 1-3: Test parameters for Single-DCI based multi-TRP/Panel transmission schemes (URLLC) - Issue 1-3-1: Transmission schemes Support Option 1. - Issue 1-3-2: Number of test cases Support Option 3. For sDCI-based FDM, we prefer to use the same configuration as mDCI-based SDM. Therefore we propose to consider the frequency offset and negative time offset. For inter-slot TDM, our simulation result shows the performance is sensitive to frequency offset. We therefore want to verify the performance of inter-slot TDM with frequency offset. - Issue 1-3-3: PRB precoding granularity Support Option 1. - Issue 1-3-4: Antenna Port indices Support Option 1 - Issue 1-3-5: timing offset among multi-panel/TRP We prefer the same time offset setting as sDCI/mDCI-based SDM transmission, that is, 2us for FDD SCS=15kHz and 1us for TDD SCS=30kHz. - Issue 1-3-6: Frequency offset among multi-panel/TRP Support the moderator’s recommendation. - Issue 1-3-7: Tx occasions for inter-slot TDM I think options 1 and 2 are identical because PDSCH demod with URLLC WI uses repetition=2. - Issue 1-3-8: Requirement setup for FDM scheme A Support Option 1 - Issue 1-3-9: MCS for FDM scheme A and inter-slot TDM scheme Option 2. Considering the applicability rule that FDM scheme A is tested if UE is not capable of multi-DCI monitoring and inter-slot TDD is tested if UE does not pass URLLC slot aggregation requirements, we would like to set the same MCS as the corresponding test cases. - Issue 1-3-10: Test Metric for FDM scheme A and inter-slot TDM scheme Option 2. Considering the applicability rule that FDM scheme A is tested if UE is not capable of multi-DCI monitoring and inter-slot TDD is tested if UE does not pass URLLC slot aggregation requirements, we would like to set the same metric as the corresponding test cases. - Issue 1-3-11: Simulation Assumption Timing offset of TDD should be 1us. Schedule PDSCH in the special slots. |
| Huawei | Sub-Topic 1-1: Generic test set-up - Issue 1-1-1: Scheduling PDSCH data in TDD special slots for single/multi-DCI Given that no degradation on performance has been observed, it is ok to schedule PDSCH data in the special slot. - Issue 1-1-2: Scheduling PDSCH data in DL slots which carriers TRS Tx for single/multi-DCI OK with us. Sub-Topic 1-2: Test parameters for Multi-DCI based multi-TRP/Panel transmission schemes (eMBB) - Issue 1-2-1: Number of test cases for single-DCI/multi-DCI Support option 2. Two test cases are enough for verifying the right compensation for time offset and frequency offset. For time offset, one compensation algorithm will be used. So, either positive or negative time offset is the same thing to the testing. For frequency offset, it can be added into test 1b for coverage. In this case, we propose the following compromising option 2a: Only 2 test cases per duplex mode - Test 1b: Single-DCI with frequency offset and positive time offset and overlapping scheduling. - Test 2a: Multi-DCI with frequency offset and negative time offsets and non-overlapping scheduling. - Issue 1-2-2: FRC for single /multi-DCI based SDM transmission Decide after issue 1-1-1 and 1-1-2 are settled. - Issue 1-2-3: Precoding scheme for multi-DCI SDM We don’t have strong view. Seems like it does not affect the performance requirement. Sub-Topic 1-3: Test parameters for Single-DCI based multi-TRP/Panel transmission schemes (URLLC) - Issue 1-3-1: Transmission schemes Agree with recommended WF. - Issue 1-3-2: Number of test cases No strong view. Recommended WF is also fine for us. - Issue 1-3-3: PRB precoding granularity Option 1 is fine for us. - Issue 1-3-4: Antenna Port indices Agree with recommended WF. - Issue 1-3-5: timing offset among multi-panel/TRP Agree with option 1. 15kHz SCS: 2us, -0.5us. 30kHz SCS: 1us, -0.25us. - Issue 1-3-6: Frequency offset among multi-panel/TRP Agree with recommended WF. - Issue 1-3-7: Tx occasions for inter-slot TDM Agree with recommended WF. - Issue 1-3-8: Requirement setup for FDM scheme A Further check the results and then decide. - Issue 1-3-9: MCS for FDM scheme A and inter-slot TDM scheme We prefer to reuse the MCS as multi/single-DCI transmission schemes for eMBB. - Issue 1-3-10: Test Metric for FDM scheme A and inter-slot TDM scheme Support option 2. Back to the RAN4 #94-e meeting, while companies were discussing the test scope of Multi-TRP/Panel transmission, there was an agreement related to the test metric of URLLC multi-TRP transmission schemes saying that: - No PDSCH requirements for URLLC multi-TRP transmission schemes with reliability transmission with lower BLER test metric (e.g., lower than 1% BLER) in Rel-16 NR eMIMO WI. which indicated that lower BLER test metric will not be considered for the requirement definition of URLLC multi-TRP transmission scheme. Then companies began to discuss whether to define PDSCH requirements for URLLC multi-TRP transmission schemes with test metric @70% maximum TP, - Whether to define PDSCH requirements for URLLC multi-TRP transmission schemes with test metric @70% maximum TP - Option 1: Yes - Option 2: Deprioritize - Option 3: No. which means that only 70% max TP is within the discussion scope. Besides, eMIMO is not within the scope of URLLC WI, so it is reasonable to differentiate from it. Issue 1-3-11: Simulation Assumption Further decide after above parameter are settled. |
| Intel | Sub-Topic 1-1: Generic test set-up - Issue 1-1-1: Scheduling PDSCH data in TDD special slots for single/multi-DCI Support Option 1 since there is no performance impact. - Issue 1-1-2: Scheduling PDSCH data in DL slots which carriers TRS Tx for single/multi-DCI Even that effective code rate in PDSCH slots which carry TRS Tx is higher than in other slots, based on our link-level results there is no performance difference if we consider PDSCH scheduling or not. Therefore, we support Option 1. Sub-Topic 1-2: Test parameters for Multi-DCI based multi-TRP/Panel transmission schemes (eMBB) - Issue 1-2-1: Number of test cases for single-DCI/multi-DCI From test coverage perspective we should ensure that robustness to both time and frequency offsets will be tested for both Single-DCI and Multi-DCI schemes. In this case we support option 1 or proposed option 2a by Huawei. - Issue 1-2-2: FRC for single /multi-DCI based SDM transmission Agree with proposed FRCs. - Issue 1-2-3: Precoding scheme for multi-DCI SDM We agree that it is beneficial to capture precoding generation procedure in specification. However, we do not think that same PMI among two PDSCHs is a typical configuration for multi-TRP Tx. In real field UE will report PMI for each PDSCH and due to different propagation conditions more likely it will be different PMIs. In this case we suggest considering independent random PMI generation for each TRP. Similar approach should be adopted for other multi-TRP TX schemes and addressed in corresponding CRs. Sub-Topic 1-3: Test parameters for Single-DCI based multi-TRP/Panel transmission schemes (URLLC) - Issue 1-3-1: Transmission schemes Support Option 1 and recommended WF. - Issue 1-3-2: Number of test cases As compromise we agree with recommended WF (Option 3). - Issue 1-3-3: PRB precoding granularity Support Option 1. - Issue 1-3-4: Antenna Port indices Support Option 1. - Issue 1-3-5: timing offset among multi-panel/TRP In proposed Table with recommended simulation assumptions there is a typo regarding TO for TDD. Our intention is to use same values as were agreed for eMBB schemes. In this case we support option with 2us;-0;5us for FDD and 1us;-0.25us for TDD. - Issue 1-3-6: Frequency offset among multi-panel/TRP Support Option 1. - Issue 1-3-7: Tx occasions for inter-slot TDM Agree with recommended WF. - Issue 1-3-8: Requirement setup for FDM scheme A In multi-DCI Tx schemes each codeword is coded independently, when is FDM scheme A it is a single codeword. Different TBS sizes will lead to different coding gains and different demodulation performances. The main intention of Option 1 is to reduce the simulation efforts. However, simulation result collection campaign is required to identify the exact difference. Therefore, we cannot accept Option 1. - Issue 1-3-9: MCS for FDM scheme A and inter-slot TDM scheme As compromise we agree with recommended WF. Regarding MCS table we suggest considering Low SE table (Table 3) which is more typical configuration for URLLC scenario. - Issue 1-3-10: Test Metric for FDM scheme A and inter-slot TDM scheme Options with 1% BLER is fully align with previous agreement which preclude discussions regarding options with (lower) less than 1% BLER. To decide appropriate test metric, we should not rely on applicability rule and make prioritizations for some schemes under another. FDMA scheme A is not a subset of multi-DCI scheme - it is another UE feature. Also, it will be strange to have different test metrics for URLLC repetition schemes which were designed by RAN1 under single motivation to ensure reliable packet transmission. Considering that simulation results collection is needed for repetition schemes (there are concerns regarding reusing SNR point) we do not see reasons to consider 70% Throughput as an appropriate test metric. Support Option 1 with 1% BLER for each scheme. Issue 1-3-11: Simulation Assumption Agree with comment from Ericsson that: Timing offset of TDD should be 1us. Schedule PDSCH in the special slots. |
| MediaTek | Sub-Topic 1-2: Test parameters for Multi-DCI based multi-TRP/Panel transmission schemes (eMBB) - Issue 1-2-1: Number of test cases for single-DCI/multi-DCI Support the recommended WF. Sub-Topic 1-3: Test parameters for Single-DCI based multi-TRP/Panel transmission schemes (URLLC) - Issue 1-3-1: Transmission schemes Support option 1. - Issue 1-3-2: Number of test cases Support option 1 and option 3. - Issue 1-3-10: Test Metric for FDM scheme A and inter-slot TDM scheme Support option 1. |
| Samsung | Issue 1-1-1: Scheduling PDSCH data in TDD special slots for single/multi-DCI We are with option 1, to align the Rel-15 test cases Issue 1-1-2: Scheduling PDSCH data in DL slots which carriers TRS Tx for single/multi-DCI We are with option 1, Issue 1-2-1: Number of test cases for single-DCI/multi-DCI We are ok with the recommend WF. Based on the agreed UE feature list, support single-DCI and multi-DCI transmission schemes are separate UE features. In order to ensure test coverage, we should introduce separate test cases based on UE feature list Also consideirng the performance impact with negative/postive time offset refer to receiver FFT window is unsymetrical and we cann’t exclude the possibility negeative timing offset irrespective UE receiver FFT window ajustment stragecy consideirng UE mobility i.e. UE move to the centre among two tranmistted TPs. It’s important to cover both negtive timing offset, postive time offset and frequency offset during test cases to verify proper UE processing irrespective UE capability. If we only introduce test 1b and test 1a, there are test coverage issue for UE which only support single-DCI based on transmission schemes as the test cases didn’t cover negative time offset. Also from UE test effort aspect, pending on UE capalbility, each UE only need to pass 2 test cases from 3 test cases if test 1a introduced which means no addtional test effort from UE aspect. Issue 1-2-2: FRC for single /multi-DCI based SDM transmission We are ok with the recommend WF. Issue 1-2-3: Precoding scheme for multi-DCI SDM From demodultion perspecive, the peformance with common PMI and independent PMI for each TRP show minior difference. With common PMI can simplify the TE implemention. While , in the real field UE, it can not guarantee the same PMI will be reported for each TRP consideirng the indepenpent channel conditions. In that sense,we slightly prefer with independent PMI generatlion for multi-TRP Tx schemes. Issue 1-3-1: Transmission schemes We are ok with recommed WF To Apple: Regarding to define the PDSCH requirement with single-DCI based, we have discussed for past three meeting, The main concerns is whether there is different UE bahavior that is not tested in other test cases. URLLC based on transmission schemes and eMBB based on transmission schemes target to different deployment and usage scenarios. Furthermore from UE processing aspect, there are special scheduling and processing for URLLC operation related schemes compared eMBB operation i.e. mini-slot scheduling, CW combining within slots and across slots. Compared to URLLC test cases, these transmission schemes also required to support multi-TCI states with different QCL information associated with same TBS which transmitted from different TRPs. Based on above analysis, the UE function/behaviour can be partially verified in the already agreed URLLC test cases or multi-TRP transmission test cases for eMBB transmission from UE processing aspect; meanwhile there are still delta compared what already agreed test cases. As a whole independent feature itself, the UE behaviour hasn’t been covered by existing test cases. During the discussion in the last meeting, one compromise solution accepted by companies to define the URLLC transmission schemes with test applicability rule to reduce the test effort, if UE can pass thee multi-DCI Tx scheme or can pass the URLLC slot aggregation requirements. Issue 1-3-2: Number of test cases We are ok with recommed WF. Meanwhile, the timing offset and frequencey offset should be set to align with test cases defined for eMBB as - Test 1a: Single-DCI based FDM scheme A with frequency offset and negative time offset - FDD: Frequency offset 200Hz, Timing offset: -0.5us - TDD: Frequency offset 300Hz, Timing offset: -0.25us - Test 1b: Single-DCI based inter-slot TDM A with frequency offset and positive time offset - FDD: Frequency offset 200Hz, Timing offset: 2us - TDD: Frequency offset 300Hz, Timing offset: 1us Issue 1-3-3: PRB precoding granularity We are ok with option1 Issue 1-3-4: Antenna Port indices We are ok with option1 Issue 1-3-5: timing offset among multi-panel/TRP We prefer to use the same timing offset to align the test case define in eMBB as 1 us for TDD with 30KHz SCS. Issue 1-3-6: Frequency offset among multi-panel/TRP We are ok with opiton 1, which is aligned the FO simulation assumption for eMBB Issue 1-3-7: Tx occasions for inter-slot TDM We are ok with recommended WF Issue 1-3-8: Requirement setup for FDM scheme A We understanding the motivation is to reduce the simulation result. FDM scheme A is the single codeword with combination the same information for each layer, which is different with mult-DCI Tx schemes. The demodualtion performance should be different. We prefer to define the requirement based on the simulation results at this stage. Issue 1-3-9: MCS for FDM scheme A and inter-slot TDM scheme We are ok with the recommended WF. For URLLC WI, the MCS 13, MCS16 and MCS19 with table MCS table 3 are considered for PDSCH requirement with high reliability. The MCS selection is based on high or equal to -4dB for find 4Rx requirement defintion. Based on current results, the SNR with MCS 13 or MCS 16 is very lower, even with 2 dB IM. In that sense, for sake the progress, we suggest to define the MCS 19 for both FDM scheme A and inter-slot TDM Meanwhile, MCS 19 is also used to define requirement for eMBB. Considering the applicability rule FDM scheme A is tested if UE is not capable of multi-DCI and inter-slot TDD is tested if UE does not pass URLLC slot aggregation. For sake of the progress, we suggest to define the MCS 19 (table 3) for both two schemes. Issue 1-3-10: Test Metric for FDM scheme A and inter-slot TDM scheme We are ok with the recommended WF. The intention of previously agreement for ”No PDSCH requirements for URLLC multi-TRP transmission schemes with reliability transmission with lower BLER test metric” is want to decouple the Rel-16 URLLC WI, where it is targeting 10^-5 BLER. From our side, whether to define 70% or 1% BLER, we are open to discussion. Cconsidering these transmission schemes more suitable for URLLC, we slightly prefer with 1% BLERConsidering we have the test abaiblity rule with URLLC schemes that FDM scheme A is tested if UE is not capable of multi-DCI monitoring and inter-slot TDD is tested if UE does not pass URLLC slot aggregation requirements. Considering the existing test metric for multi-DCI is 70% and requirement under slot aggregation is 1% BLER, in that sense, if we apply the test ability rule, set the same metric as the corresponding test cases is more appropriate. Issue 1-3-11: Simulation Assumption As commented in issue 1-3-5, the timing offset for TDD should be 1us if considering postive timing offset. |
| Qualcomm | Sub-Topic 1-1: Generic test set-up - Issue 1-1-1: Scheduling PDSCH data in TDD special slots for single/multi-DCI - Support option 1 - Issue 1-1-2: Scheduling PDSCH data in DL slots which carriers TRS Tx for single/multi-DCI - Support option 1 Sub-Topic 1-2: Test parameters for Multi-DCI based multi-TRP/Panel transmission schemes (eMBB) - Issue 1-2-1: Number of test cases for single-DCI/multi-DCI - Support recommended WF - Issue 1-2-2: FRC for single /multi-DCI based SDM transmission - Revisit after issues in sub-topic 1-1 are settled. - Issue 1-2-3: Precoding scheme for multi-DCI SDM - We’d like to get clarification regarding the proposed option 1. - Does it apply for mutli-DCI SDM non-overlapping case? - Each TRP has an independent randomly generated precoder? Sub-Topic 1-3: Test parameters for Single-DCI based multi-TRP/Panel transmission schemes (URLLC) Similar to Apple’s comment. We prefer not to define requirements for multi-TRP URLLC schemes given previous RAN4 agreements. - Issue 1-3-1: Transmission schemes - Issue 1-3-2: Number of test cases - Issue 1-3-3: PRB precoding granularity - Issue 1-3-4: Antenna Port indices - Issue 1-3-5: timing offset among multi-panel/TRP - Issue 1-3-6: Frequency offset among multi-panel/TRP - Issue 1-3-7: Tx occasions for inter-slot TDM - Issue 1-3-8: Requirement setup for FDM scheme A - Issue 1-3-9: MCS for FDM scheme A and inter-slot TDM scheme - Issue 1-3-10: Test Metric for FDM scheme A and inter-slot TDM scheme Issue 1-3-11: Simulation Assumption |

CRs/TPs comments collection

Major close-to-finalize Wis and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going Wis, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2014559 (Intel) CR to TS 38.101-4: Performance requirements single-DCI based multi-TRP Repetition Tx schemes | Apple: Have we already agreed to introduce requirements with single DCI URLLC transmission schemes? Intel: There was agreement to introduce such requirements, but exact schemes are still open. |
|  | Huawei: Proposed change affects should not be void. |
|  | Company C |
| R4-2015653 (Huawei) draftCR on PDSCH requirement with Single-DCI based SDM scheme | Apple: Coversheet - Proposed change affects: Should be ME The section number needs to be updated based on agreement of Issue 3-1-1. In Test parameter tables: Timing offset of the second TRP from the first TRP (Note 2): If only one value is used for tests, it should be specified instead of positive and negative TO. If 2 values are used, we prefer to specify in testcase. Avoid specifying parameters in both Test parameters and Minimum requirements table. Frequency offset between two TRPs (Note 2). Incorrect reference to Note 2. Need to specify in Test parameters table as its fixed for all tests and need not be duplicated in Minimum requirements table. The number of tests needs to updated based on agreement on Issue 1-2-1 |
|  | Ericsson: Format of test parameter table should be aligned with other requirements. |
|  | Intel: We should align TRS configuration between CR for repetition schemes and CRs for eMBB schemes. In our understanding QCL assumptions for CSI-RS is defined per each CSI-RS resource not per resource set. In this case we can configure 8 CSI-RS resources from which 1-4 are transmitted from TRP#1 with TCI#1 and 5-8 are transmitted from TRP#2 with TCI#2. |
|  | Samsung: the section tile and number should be updated based on structure of specification discussion |
| R4-2015654 (Huawei) draftCR on PDSCH requirement with Multi-DCI based SDM scheme | Apple: Coversheet - Proposed change affects: Should be ME The section number needs to be updated based on agreement of Issue 3-1-1. Same comments as R4-2015653 on TO/FO parameter. The number of tests needs to updated based on agreement on Issue 1-2-1 |
|  | Ericsson: Same comment as R4-2015653. Format of test parameter table should be aligned with other requirements. |
|  | Intel: Same comment as on R4-2015653. |
|  | Samsung: the section tile and number should be updated based on structure of specification discussion |
| R4-2015830 (Ericsson) Draft CR: PDSCH FRC for eMIMO sDCI/mDCI-based SDM transmission | Apple Pending discussion and agreement on Issue 1-1-2. For slots with TRS transmission, FFS if we should schedule PDSCH or reduced TBS can be used without changing MCS. |
|  | Company B |
|  | Company C |
| R4-2014248 (Apple) Draft CR for eMIMO demod requirements - General and Applicability rule | Huawei: The category should be ‘B’. |
|  | Samsung: UE featrure should be aligned with RAN1 eMIMO terminology. Multi DCI based multi-TRP support-> Multi DCI based multi-TRP SingleDCI based SDM transmission for multi-TRP support-> SingleDCI based SDM scheme |
|  | Company C |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |
| Sub-topic 1-1 | Issue 1-1-1: Scheduling PDSCH data in TDD special slots for single/multi-DCI Tentative agreements: Scheduling PDSCH data in TDD special slots for single/multi-DCI Recommendations for 2nd round: N,A Issue 1-1-2: Scheduling PDSCH data in DL slots which carriers TRS Tx for single/multi-DCI Candidate options: - Proposals - Option 1: Yes (Intel, Ericsson, Samsung, Huawei, Qualcomm ) - Option 2: No (Apple) Recommendations for 2nd round: - Discuss in GTW, |
| Sub-topic 1-2 | Issue 1-2-1: Number of test cases for single-DCI/multi-DCI Tentative agreements: - Proposals - Option 1: 3 test cases per duplex mode with test applicability rule (Samsung, Intel, Ericsson, MTK, Qualcomm, Apple) - Test 1a: Single-DCI with frequency offset and negative time offset and overlapping scheduling - Test 1b: Single-DCI with positive time offset, and overlapping scheduling - Test 2a: Multi- DCI with frequency offset and negative time offset and non-overlapping scheduling - Applicability rule - If UE only supports single-DCI based multi-TRP transmission for eMBB, it should be tested with test case 1a and test case 1b - If UE can support both single-DCI and multi-DCI for eMBB, it should be tested test 2a and test 1b - Option 2a: only 2 test cases per duplex mode (Huawei, Intel) - Test 1b: Single-DCI with frequency and positive time offset, and overlapping scheduling - Test 2a: Multi- DCI with frequency offset and negative time offset and non-overlapping scheduling Recommendations for 2nd round: - Discuss in GTW Agreement from GTW session in 1st round - Test 1a: Single-DCI with frequency offset and negative time offset and overlapping scheduling - Test 1b: Single-DCI with positive time offset, and overlapping scheduling - Test 2a: Multi- DCI with frequency offset and negative time offset and non-overlapping scheduling - Applicability rule: - If UE only supports single-DCI based multi-TRP transmission for eMBB, it should be tested with test case 1a and test case 1b - If UE can support both single-DCI and multi-DCI for eMBB, it should be tested test 2a and test 1b - If UE only support multi-DCI transmission schemes for eMBB, it should be test 2a Issue 1-2-2: FRC for single /multi-DCI based SDM transmission (Pending on the Issue 1-1-2 ) Candidate options: - Proposals - Option 1(Ericsson, samsung, Intel): Introduce new FRC for Single/multi-DCI based SDM transmission requirements with - Multi-DCI based SDM schemes - FDD: 26PRB per codeword - TDD: 53PRB per codeword - Two TRS configurations every 20ms - Single-DCI based SDM schemes - Two TRS configurations every 20ms - Reference channel - R.PDSCH.1-3.2 FDD: single-DCI based SDM schemes in FDD - R.PDSCH.1-3.3 FDD: multi-DCI based SDM schemes in FDD - R.PDSCH.2-3.2 TDD: single-DCI based SDM schemes in TDD - R.PDSCH.2-3.3 TDD: multi-DCI based SDM scheme in TDD Recommendations for 2nd round: N,A Issue 1-2-3: Precoding scheme for multi-DCI SDM Candidate options: - Proposals - Option 1(Ericsson): Same precoding is applied for both codewords, where PDSCH & PDSCH DMRS Precoding is configured with Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity. - Option 2(Intel, Samsung, Qualcomm,Apple): Independent precoding generation is applied for both codewords, where PDSCH & PDSCH DMRS Precoding is configured with Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity. Recommendations for 2nd round: - Discuss in GTW |
| Sub-topic 1-3 | Issue 1-3-1: Transmission schemes Candidate options: - Proposals - Option 1(Samsung, Intel, Ericsson, Huawei, MTK): Define performance requirement for URLLC transmission schemes with test applicability rule - Only FDM scheme A for UE capable of supportFDM-SchemeA and inter-slot TDM scheme for UE capable of supportIntel-slotTDM - Test applicability - FDM scheme is skipped if UE passes the multi-DCI based multi-TRP Tx requirements - TDM scheme is skipped if UE passes URLLC slot aggregation requirements and anyone of the other multi-TRP Tx requirements - Option 2(Apple, Qualcomm): No to define requirement for URLLC transmission schemes Recommendations for 2nd round: - Discuss in GTW, Agreement from GTW session in 1st round - Define performance requirement for URLLC transmission schemes with test applicability rule - Only FDM scheme A for UE capable of supportFDM-SchemeA and inter-slot TDM scheme for UE capable of supportIntel-slotTDM - Test applicability - FDM scheme A is skipped if UE passes the multi-DCI based multi-TRP Tx requirements - Inter-slot TDM scheme is skipped if UE passes URLLC slot aggregation requirements and anyone of the other multi-TRP Tx requirements Issue 1-3-2: Number of test cases (pending on issue 1-3-1) Tentative agreements: - Test 1a: Single-DCI based FDM scheme A with frequency offset and negative time offset - Test 1b: Single-DCI based inter-slot TDM with frequency offset and positive time offset Recommendations for 2nd round: N,A Issue 1-3-3: PRB precoding granularity(pending on issue 1-3-1) Tentative agreements: - Wideband precoding for Single-DCI based FDM scheme A - 2 PRB precoding granularity for Single-DCI based inter-slot TDM scheme Recommendations for 2nd round: N,A Issue 1-3-4: Antenna Port indices(pending on issue 1-3-1) Tentative agreements: - 1000 DMRS antenna port with TCI state#1 - 1001 DMRS antenna port with TCI state#2 Recommendations for 2nd round: N,A Issue 1-3-5: timing offset among multi-panel/TRP(pending on issue 1-3-1) Tentative agreements: - Positive timing offset - FDD with 15KHz: 2us - TDD with 30KHz: 1us - Negative timing offset - FDD with 15KHz: -0.5us - TDD with 30KHz: -0.25us Recommendations for 2nd round: N,A Issue 1-3-6: Frequency offset among multi-panel/TRP(pending on issue 1-3-1) Tentative agreements: - FDD with 15KHz: 200Hz - TDD with 30KHz: 300Hz Recommendations for 2nd round: N,A Issue 1-3-7: Tx occasions for inter-slot TDM (pending on issue 1-3-1 ) Tentative agreements: - two repetitions Recommendations for 2nd round: N,A Issue 1-3-8: Requirement setup for FDM scheme A(pending on issue 1-3-1 ) Candidate options: - Proposals - Option 1(Ericsson): Set the same requirements (SNR) as multi-DCI transmission scheme - Option 2(Intel, Samsung Huawei): Set the requirement (SNR) based on the simulation result collection Recommendations for 2nd round: - Discuss in GTW Issue 1-3-9: MCS for FDM scheme A and inter-slot TDM scheme (pending on issue 1-3-1 ) Candidate options: - Proposals - Option 1(Huawei,) - MCS 19 (with table 1)for both FDM scheme and inter-slot TDM scheme - Option 2(Ericsson. Samsung) : - FDM scheme A: MCS 19 (table 1) - Inter-slot TDM scheme: FFS from MCS13,16, 19 (table 3) - Option 3 (Intel): - FDM scheme A: MCS 19 (table 3) - Inter-slot TDM scheme: FFS from MCS13,16, 19 (table 3) Recommendations for 2nd round: - Discuss in GTW, Issue 1-3-10: Test Metric for FDM scheme A and inter-slot TDM scheme Candidate options: - Option 1: 1% BLER (BLER is calculated after all transmission) (Intel, Samsung, MTK) - Option 2(Ericsson, Intel, Samsung): - 70% TP for FDM scheme A - 1% BLER (BLER is calculated after all transmission) for inter-slot TDM scheme - Option 3: 70% TP (Huawei) Recommendations for 2nd round: - Discuss in GTW, |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | WF for NR eMIMO PDSCH requirement | Intel |
| #2 | Simulation assumption for PDSCH requirement with single-DCI and Multi-DCI transmission schemes | Ericsson, Intel |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2014559 | To be revised |
| R4-2015653 | To be revised |
| R4-2015654 | To be revised |
| R4-2015830 | To be revised |
| R4-2014248 | To be revised |

Discussion on 2nd round (if applicable)

Issue 1-1-2: Scheduling PDSCH data in DL slots which carriers TRS Tx for single/multi-DCI

- Proposals

  - Option 1: Yes (Intel, Ericsson, Samsung, Huawei, Qualcomm )

  - Option 2: No (Apple)

- Recommended WF

  - Option 1. Following the Rel-15 UE demod set up?

[Ericsson] Support the recommended WF (Option 1)

[Intel] Support the recommended WF (Option 1)

[Apple] We are fine to go with majority view and support option 1

[Huawei]: We are fine with the recommended WF.

Issue 1-2-2: FRC for single /multi-DCI based SDM transmission (Pending on the Issue 1-1-2 )

- Proposals

  - Option 1(Ericsson, samsung, Intel): Introduce new FRC for Single/multi-DCI based SDM transmission requirements with

- Multi-DCI based SDM schemes

- FDD: 26PRB per codeword

- TDD: 53PRB per codeword

- Two TRS configurations every 20ms

- Single-DCI based SDM schemes

- Two TRS configurations every 20ms

- Reference channel

- R.PDSCH.1-3.2 FDD: single-DCI based SDM schemes in FDD

- R.PDSCH.1-3.3 FDD: multi-DCI based SDM schemes in FDD

- R.PDSCH.2-3.2 TDD: single-DCI based SDM schemes in TDD

- R.PDSCH.2-3.3 TDD: multi-DCI based  SDM scheme in TDD

- Recommended WF

  - Option 1?

  - Encouraged companies to check the value in the FRC

  - Ericsson: Please update the corresponding CR for FRC table based on 1st round attentive agreement

[Ericsson] We have uploaded the FRC in the inbox. In the revision, we assume to keep the same MCS19 for all the slots and PDSCH is transmitted in the slots where TRS is transmitted.

https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_97_e/Inbox/Drafts/%5B97e%5D%5B324%5D%20NR_eMIMO_Demod/CR/draft%20R4-2017641%20eMIMO%20draft%20CR%20demod%20FRC%20rev1.docx

[Apple] What is multi-DCI based SDM scheme?

In the CR, FRC for multi-DCI scheme is incorrect. It only captures transmission from single TRP and TP calculated from single TRP transmission. The PDSCH allocation and FRC should be for both TRPs. For single DCI SDM scheme, we have one layer transmission from each TRP, but the FRC suggests 2 layer transmission. We must have a way to identify that these are for multi-TRP transmission.

[Apple]

Thanks for all your efforts for the CRs. We have a few comments on the FRCs for mTRP:

1. For multi-DCI scheme. The FRC is specified per TRP. But the allocation for each TRP is not captured anywhere.

2. For single DCI SDM scheme, the combined FRC is specified, would number of MIMO layers be interpreted per TRP or both combined? We have 1 layer transmission per TRP.

3. For FDM Scheme FRC is specified for 2TRP, does 2 layers here mean combined across both TRP or 2 layer transmission per TRP? We have 2 layer transmission from each TRP.

4. For inter slot TDM scheme. The TDM allocation is not captured anywhere. The FRC is specified for each TRP, but that suggests 1 TRP transmits in all slots, which is incorrect.

The FRCs at least for single DCI schemes don’t correctly capture the intended transmission scheme.

Artyom, For FDM scheme the table titles should be Table 5.2.2.1.13-3: Minimum performance for Rank 2

We also need to discuss where we capture the test applicability rules agreed. I checked 36.101 and test applicability are captured with the test case and not in the general and applicability section.

I will update the applicability section to reflect the correct section numbers and share shortly.

[Ericsson] As I mentioned before, my preference for FRC is to be independent of transmission antenna. That is, this FRC can be used not only for multi-TRP transmission scheme but also for the non-multi-TRP transmission schemes.

I understand we should clarify how FRC is mapped to each TRP. I think it is specified in section 5.

Issue 1-2-3: Precoding scheme for multi-DCI SDM

- Proposals

  - Option 1(Ericsson): Same precoding is applied for both codewords, where PDSCH & PDSCH DMRS Precoding is configured with Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity.

  - Option 2(Intel, Samsung, Qualcomm, Apple):  Independent precoding generation is applied for both codewords, where PDSCH & PDSCH DMRS Precoding is configured with Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity.

- Recommended WF

  - Option 2?  The current PDSCH requirement is based on 2Tx (Ports), the codebook indices i1, i2 combination only available for number of port large than 4 antenna ports

- Independent precoding generation is applied for both codewords for both single-DCI/multi-DCI based multi-TRP transmission, where PDSCH & PDSCH DMRS Precoding is configured with Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity?

  - Same precoding generation principle should be applied for both single-DCI/multi-DCI with multi-TRP transmission.  Capture precoding generation procedure in the CR for PDSCH requirement, encourage companies to check the wording.

[Ericsson] We are ok with Option 2 for multi-DCI based transmission. For single-DCI based SDM/FDM transmission, however, it is single CW, so we think the same precoding should be applied.

[Samsung] we are ok with the recommend WF, To Ericsson, we agree with you, it is single CW for single-DCI based SDM/FDM. During the assumption, we only consider rank2 for this SDM. layer 1 is transmitted from TP1, layer 2 is transmitted from TP2. In that sense, independent procoding generation is more reasonable to reflect the real UE field., we think it should be applied for both single-DCI/multi-DCI, no matter single CW or two CWs.

[Intel] We have similar view as Samsung. From PMI reporting perspective, UE will report PMI for each link independently since different non-collocated CSI-RS resources will be configured from each TRP. In this case to optimize performance for each link TRPs will define Tx PMI independently.

Issue 1-3-8: Requirement setup for FDM scheme A

- Proposals

  - Option 1(Ericsson):  Set the same requirements (SNR) as multi-DCI transmission scheme

  - Option 2(Intel, Samsung Huawei): Set the requirement  (SNR) based on the simulation result collection

- Recommended WF

  - Option 2?

[Ericsson] We are ok with Option 2.

Issue 1-3-9: MCS for FDM scheme A and inter-slot TDM scheme

- Proposals

- Option 1(Huawei,)

- MCS 19 (with table 1)for both FDM scheme and inter-slot TDM scheme

- Option 2(Ericsson. Samsung) :

- FDM scheme A: MCS 19 (table 1)

- Inter-slot TDM scheme:  FFS from MCS13,16, 19 (table 3)

- Option 3 (Intel):

- FDM scheme A: MCS 19 (table 3)

- Inter-slot TDM scheme:  FFS from MCS13,16, 19 (table 3)

- Recommended WF

  - Refer the simulation results (R4-2015628) and agreements of “Higher or equal to -4 dB for final 4 Rx requirement definition (average ideal SNR alignment result + IM)” in URLLC WI for PDSCH requirement with high reliability 1% BLER for PUSCH aggregation level n2. MCS19 (table 3) as the conclusion as MCS19 is the only option meets the agreements.

  - MCS 19 (table 3) for both FDM scheme A and inter-slot TDM scheme?

  - To intel: if agreed to introduce MCS 19 (table 3) for PDSCH requirement with FDM scheme A and inter-slot TDM scheme, the corresponding CR for FRC should be introduced , please cooperate with Ericsson how to capture the FRC with MCS 19 (table 3)

Ericsson] We prefer MCS 19 with Table 1 for FDM Scheme A and MCS19 with Table 3 for Inter-slot TDM (Option 2).

RAN4 agreed that the applicability that:
- FDM scheme is skipped if UE passes the multi-DCI based multi-TRP Tx requirements
- TDM scheme is skipped if UE passes URLLC slot aggregation requirements and anyone of the other multi-TRP Tx requirements
Based on this applicability rule, we prefer to configure the same MCS between FDMSchemeA and multi-DCI SDM transmission, and between URLLC and inter-slot TDM.

[Intel] We are fine to consider MCS 19 for both repetition schemes. However, we prefer to consider MCS Table 3 for both. Although we have applicability rule between FDM scheme A and multi-DCI based scheme, we cannot reuse the same SNR point for them. Applicability rule just indicate the more-less similar receive processing not performance. In this case, the alignment of test setups is not a goal. It is more reasonable to consider typical configuration for each scheme (like MCS table 3 for URLLC related schemes).  Different MCS tables as well test metrics will not affect the rationality of the defined applicability rule. 
Support MCS 19 from MCS Table 3 for each scheme.

[Huawei]: We confirm that MCS table 3 is for URLLC related. But if we choose MCS 6 or bigger in table 3, the modulation order and target code rate are the same as table 1 except the MCS index, which degrade the meaning of choosing table 3 because we didn’t use the specific MCS for URLLC. Instead, we can choose lower MCS in table 1.

[Samsung]: we agree with intel, the test applicablity rule is just to indicate that UE willl apply the similar receiver. Just like other WI, i,e HST,different requirements are defined for different Tx , such  single Tap, SFN, we donot need to align the test setup for different Tx scheme.

To huawei: MCS 19 in table 3 is 16QAM, and while MCS19 in table 1 is 64QAM

[Huawei]: To Samsung, Yes we know the specific modulation order for MCS 19. Our point is that if we could look at the table 3 from MCS 6, those modulation order and target code rate are just the same as table 1 after moving down 6 units. So if we choose a specific MCS larger than 6 in table 3, we could always find a same modulation order and target code rate in table 1 with only different on MCS. Choosing MCS lower than 6 is the meaning of using table 3 for URLLC related. Otherwise we could use table 1 with MCS lower than 19 to be instead.

[Intel]: As we commented before, our preference is to have most typical test configuration for each transmission scheme. Different simulation assumptions do not affect applicability rules. As Samsung pointed out in HST WI we have applicability rule among test cases with different MCS/Rank. We also understand Huawei point that we can find same combination of modulation order/code rate in MCS Table 1 as in MCS Table 3. But what is a purpose to do this? MCS 3 table was defined for reliability critical scenarios, same as multi-TRP repetition schemes. Why we should consider non-typical configuration during the test? Do companies worry that if we consider another MCS/Rank or test metric UE will fail the test? We do not really understand motivation to align test setup with other test if anyway simulation campaign is needed since provided performance is different among all schemes.

To move forward we can compromise to hybrid Option 2. However, we do not think it is reasonable to assume different test metrics. It will bring confusions for non RAN4 people.

Considering our compromise, we’d like to encourage other companies to follow moderator recommendation and compromise on Option 3 in issue 1-3-9: MCS 19 in MCS Table 3 for both Tx schemes.

[Intel]: To move forward we highlighted in yellow possible agreements. Please double check.

Regarding MIMO Rank Tx, we can compromise to Ericsson suggestion assuming Rank 2 for FDM scheme A if MCS Table 3 will be used for both schemes. Otherwise we cannot accept Rank 2 and 70% Throughput test metric because it will be quite non-typical configuration for repetition scheme.

As for precoding generation at current stage we propose Option 2 based on concerns raised in discussion below for Issue 1-2-3. Response from Ericsson and other companies is required.

Issue 1-3-10: Test Metric for FDM scheme A and inter-slot TDM scheme

- Proposals

  - Option 1: 1% BLER (BLER is calculated after all transmission) (Intel, Samsung, MTK)

  - Option 2(Ericsson, Samsung, Huawei (compromise)):

- 70% TP for FDM scheme A

- 1% BLER (BLER is calculated after all transmission) for inter-slot TDM scheme

  - Option 3: 70% TP (Huawei)

- Recommended WF

  - Refer the simulation results (R4-2015628) and agreements of “Higher or equal to -4 dB for final 4 Rx requirement definition (average ideal SNR alignment result + IM)” in URLLC WI for PDSCH requirement with high reliability 1% BLER. MCS19 (table 3) as the conclusion as MCS19 is the only option meets the agreements.

  - Option with 1% BLER for both FDM scheme A and inter-slot TDM scheme (BLER is calculated after all transmission)?

[Ericsson] We prefer option 2.

Same as Issue 1-3-9, we prefer to use the same metrics between FDMSchemeA and multi-DCI SDM transmission, and between URLLC and inter-slot TDM.

[Intel] Support recommended WF (Option 1). First, as moderator mentioned it will allow to avoid problems with small SNR values (less than -4dB). 1% BLER requires higher SNR than 70% Throughput metric. In addition, as we mentioned above, we do not need to align test setups between different Tx schemes even that we have applicability rule between them. Different test setups will not affect assumption that if UE has passed requirements for multi-DCI based Tx scheme it can pass FDM scheme A requirements.

[Huawei]: For FDM 2a scheme, it is not affected by the potential concern of SNR value less than -4dB. So 1% BLER is similar to 70% max TP ratio but results in higher SNR, which only relaxes the requirement. For inter-slot TDM, it might have such concern. Thus, we can compromise and accept option 2.

Issue 1-3-11: Simulation Assumption

- Proposals

  - Option 1 (Intel)

| - Parameters | Simulation cases 1-1~ 1-8 | Simulation cases 2-1~ 2-8 |
| --- | --- | --- |
| Duplex mode | FDD | TDD |
| TDD UL/DL configuration | N/A | [FR1.30-1 (7DS2U, 6D+4G+4U)] PDSCH is scheduled in the special slots |
| Timing offset of the second TRP from the first TRP | 2 μs | 1 μs |
| Frequency offset between two TRPs | 200 Hz | 300 Hz |
|  | Common serving cell parameters |  |
| Physical Cell ID | 0 |  |
| SSB position in burst | First SSB in Slot #0 |  |
| SSB periodicity | 20ms |  |
| Transmit TRP of SSB | Only from TRP1 |  |
|  | TRS#1 (configuration for TCI state #1) |  |
| Transmit TRP | TRP#1 |  |
| CSI-RS periodicity | [20] slots | [40] slots |
| CSI-RS offset | [10] for CSI-RS resources 1 and 2 [11] for CSI-RS resources 3 and 4 | [20] for CSI-RS resources 1 and 2 [21] for CSI-RS resources 3 and 4 |
| Density | 3 | 3 |
| First subcarrier index in the PRB used for CSI-RS | k0=0 | k0=0 |
| QCL Info | TCI state #0 | TCI state #0 |
| First OFDM symbol in the PRB used for CSI-RS | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 |
|  | TRS#2 (configuration for TCI state #2) |  |
| Transmit TRP | TRP#2 |  |
| CSI-RS periodicity | [20] slots | [40] slots |
| CSI-RS offset | [10] for CSI-RS resources 1 and 2 [11] for CSI-RS resources 3 and 4 | [20] for CSI-RS resources 1 and 2 [21] for CSI-RS resources 3 and 4 |
| Density | 3 | 3 |
| First subcarrier index in the PRB used for CSI-RS | k0=1 | k0=1 |
| First OFDM symbol in the PRB used for CSI-RS | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 |
| QCL Info | TCI state #0 | TCI state #0 |
|  | TCI State configuration #0 |  |
| Type 1 QCL information | SSB index #0, QCL type C |  |
| Type 2 QCL information | N/A |  |
|  | TCI State configuration #1 |  |
| Type 1 QCL information | CSI-RS resource: TRS#1, QCL type A |  |
| Type 2 QCL information | N/A |  |
|  | TCI State configuration #2 |  |
| Type 1 QCL information | CSI-RS resource: TRS#2, QCL type A |  |
| Type 2 QCL information | N/A |  |
|  | PDCCH configuration |  |
| CORESETPoolIndex | 0 (or not configured) |  |
| Symbols for PDCCH | 0, 1 |  |
| K0 | 0 |  |
| AL | 8 (576REs) |  |
|  | PDSCH configuration |  |
| PDSCH resource mapping type | Type A |  |
| Resource allocation type | Type 1 |  |
| Antenna ports index ​ | {1000} for TCI#1 and {1001} for TCI#2 |  |
| Note 1: The same SSB (index #0) is transmitted from both TRP1 and TRP2. |  |  |

|  | FDMSchemeA | Inter-slot TDM |
| --- | --- | --- |
| DMRS configuration | Type 1, 1+1 | Type 1, 1+1 |
| Start symbol and time duration | PDSCH#1: 2, 12 PDSCH#1: 2, 12 | PDSCH#1: 2, 12 PDSCH#1: 2, 12 |
| Precoding granularity | WB | 2 |
| FDD PRB allocation | PDSCH#1: PRB#0 to PRB#25 PDSCH#2: PRB#26 to PRB#51 | PDSCH#1: PRB#0 to PRB#51 PDSCH#2: PRB#0 to PRB#51 |
| TDD PRB allocation | PDSCH#1: PRB#0 to PRB#52 PDSCH#2: PRB#53 to PRB#105 | PDSCH#1: PRB#0 to PRB#105 PDSCH#2: PRB#0 to PRB#105 |

- Recommended WF

  - To intel: Please cooperate with Ericsson for contribution preparation about simulation assumption for PDSCH with single-DCI and multi-DCI transmission schemes (included both agreed eMBB and URLLC test cases)

| Company | Comments |
| --- | --- |
| Company A | Sub-Topic 1-1: Generic test set-up - Issue 1-1-2: Scheduling PDSCH data in DL slots which carriers TRS Tx for single/multi-DCI Sub-Topic 1-2: Test parameters for Multi-DCI based multi-TRP/Panel transmission schemes (eMBB) - Issue 1-2-2: FRC for single /multi-DCI based SDM transmission - Issue 1-2-3: Precoding scheme for multi-DCI SDM Sub-Topic 1-3: Test parameters for Single-DCI based multi-TRP/Panel transmission schemes (URLLC) - Issue 1-3-8: Requirement setup for FDM scheme A - Issue 1-3-9: MCS for FDM scheme A and inter-slot TDM scheme - Issue 1-3-10: Test Metric for FDM scheme A and inter-slot TDM scheme - Issue 1-3-11: Simulation Assumption |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2017529 WF for NR eMIMO PDSCH requirement | Agreeable |
| R4-2017530 Simulation assumption for PDSCH requirement | Agreeable |
| R4-2017601 Draft CR for eMIMO demod requirements - General and Applicability rule | Agreeable |
| R4-2017641 Draft CR: PDSCH FRC for eMIMO sDCI/mDCI-based SDM transmission | Considering there is no consensus about how to the FRC mapping to two TRP, suggested to postpone and further discussion in the next meeting postponed |
| R4-2017532 DraftCR for 38.101-4: Introduction of PDSCH requirement with Single-DCI based SDM scheme | Agreeable |
| R4-2017533 DraftCR for 38.101-4: Introduction of PDSCH requirement with Multi-DCI based transmission scheme | Agreeable |
| R4-2017534 CR to TS 38.101-4: Performance requirements single-DCI based multi-TRP Repetition Tx schemes | Agreeable |

Topic #2: CSI requirements(Rel-16 TypeII codebook)

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2014249 | Apple | Observation 1: With SU-MIMO setup performance with correctly reported Type II PMI is significantly better than incorrect Type II PMI reporting. Observation 2: With SU-MIMO setup performance of Type II PMI is better than Type I PMI. Proposal 1: Define PMI reporting requirements in Rel-16 with SU-MIMO test setup for Type II and enhanced Type II codebook. |
| R4-2014740 | Samsung | Observation 1: With SU-MIMO set-up, the Rel-16 Enhanced Type II could achieve significant gain over Rel-15 Type II and Rel-15 Type I codebook with following PMI. Observation 2: With MU-MIMO Set-up, the performance of Type II codebook even worse than Type I codebook based on the agreed test parameters. Observation 3: With SU-MIMO set-up, the throughput ratio among following PMI and random under Rel-16 Type II codebook is quite huge at 90% relative throughput. Proposal1: Overall Test set-up: - Introduce Rel-15 Type II PMI test cases under SU-MIMO test set-up in Rel-16 timeframe. - Further study and define proper performance requirements if needed under MU-MIMO scenarios in Rel-17 performance enhancement WI. Proposal 2: In order to achieve test point and test requirements under SU-MIMO test set-up: - Using 95% relative throughput with following PMI as test point - Restrict random PMI to random selected within Rel-15 Type I codebook set |
| R4-2014747 | Samsung | Draft CR for introduction of Rel-16 Type II PMI test cases |
| R4-2014949 | Nokia | Observation 1. eType II PMI is primarily intended to enhance MU-MIMO throughput by providing a much more accurate representation of the strongest channel eigenvectors than Type I SP PMI. This allows the gNB to steer the beams of co-schedule UEs in each other’s null space with less residual interference. Observation 2. SU-MIMO throughput is less sensitive than MU-MIMO to PMI inaccuracies because MU-MIMO throughput is limited by interference between co-scheduled UEs. Observation 3. A DUT could in practice be able to report a eType II PMI that does not represent well the channel main eigenvectors and still pass an SU-MIMO test for eType II, because the throughput difference between Type I SP and eType II is not large enough for SU-MIMO transmission. Observation 4. On a MU-MIMO test setup, the TE does not necessarily need to encode any data or control signal for the co-scheduled UE. The data sequence for the co-scheduled UE may be generated as a random sequence of QAM symbols and processed by the MU-MIMO precoder calculated for the co-scheduled UE. Observation 5. The precoder calculation, x_b, for the co-scheduled UE described in the WF document results in reduced computational complexity at the TE: about 3N_t complex multiplications and additions in total, where N_t is the number of transmit antennas, and no advanced processing techniques are needed. Observation 6. The comparison of the Type I with the eType II throughput provides an efficient mean of asserting the improved accuracy of the PMI reported with eType II codebook. Proposal 1. RAN4 to adopt the MU-MIMO test setup described in the WF and clarify the simplification options for the co-scheduled UE generation. Further clarify that the TE does not need to encode any data or control signal for the co-scheduled UE. As an example, the data sequence for the co-scheduled UE may be generated as a random sequence of QAM symbols and processed by the simplified MU-MIMO precoder calculated for the co-scheduled UE. Proposal 2. Further clarify that the precoder calculation for the co-scheduled UE boils down to the following simple procedure in case of rank 1 test Proposal 3. RAN4 to adopt random PMI generation for the co-scheduled UE. Further clarify that, for rank 1 test, the PMI for the co-scheduled UE, w_b, is a vector of size N_t×1, with elements drawn from a complex Gaussian distribution of mean 0 and variance 1. Proposal 4. RAN4 to adopt Option 1 for the test metric, i.e., the throughput ratio between following PMI for Type I SP and eType II |
| R4-2015646 | Huawei, HiSilicon | Observation 1: For SU-MIMO test setup, the performance of Follow PMI for Type II has an obvious gain over Follow PMI for Type I single panel Proposal 1: Option 3 for the test setup of (e) Type II codebook based PMI reporting test Proposal 2: Consider proposed simulation assumption for the PDSCH requirement under the test setup of MU-MIMO |
| R4-2015647 | Huawei, HiSilicon | Simulation results for SU-MIMO eType II codebook based PMI reporting test |
| R4-2016033 | R&S | Proposal 1: RAN4 agrees to use an SU-MIMO based test setup for type II PMI reporting for Rel-15 and Rel-16 requirements. Proposal 2: RAN4 further investigates how to enhance the PMI type II test using the SU-MIMO test setup. |
| R4-2016101 | Ericsson | Simulation results for Rel-16 Type II codebook |
| R4-2016102 | Ericsson | Observation 1: The proposed SU-MIMO test cannot be used for Type II CSI reporting since the performance benefit of Type II feedback is not visible. This is due to that SU-MIMO doesn’t take advantage of the rich channel feedback of Type II reporting Observation 2: Type I codebook is a subset of the Type II codebook, the “Type I subset” Observation 3: It is difficult to guarantee a minimum performance and benchmark when employing a random type II PMI value to the gNB precoder Observation 4: There is practically no performance difference when ‘High’ correlation matrix is employed in the SU-MIMO test Observation 5: 70% throughput mark shows more gain with Type II codebook over the 90% throughput mark for custom correlation and medium correlation. Observation 6: The subband size does not distinguish performance to any significant degree Observation 7: The gain when calculated by comparing follow Type II PMI with random Type II PMI has high variance arising from the stochastical behaviour and uncertainty of random PMI precoder selection Proposal 1: Use 70% throughput mark as reference throughput for gain requirement Proposal 2: Configure subband size 4 for FDD and 8 for TDD. Proposal 3: Use Type II follow PMI divided by Type I follow PMI to guarantee performance benefit of Type II codebooks Proposal 4: Configure medium or custom correlation Proposal 5: use MCS11 Rank1 for MU-MIMO testing Proposal 6: use custom correlation for MU-MIMO PMI testing |
| R4-2016429 | Qualcomm | Observation 1: UE reports the same precoder for both SU-MIMO and MU-MIMO test setups Observation 2: There is a significant difference in PMI ratio and SNR needed to reach 90% of peak throughput between eType 2 and Type 1 PMI reporting. Proposal 1: Use SU-MIMO test setup for defining Enhanced Type II PMI reporting tests. |

Open issues summary

Last RAN4 meeting agreements captured in WF R4-2012761

List of open issues:

- Sub-Topic 2-1: Test set-up SU-MIMO VS. MU-MIMO

  - Issue 2-1-1: SU-MIMO VS MU-MIMO Setup

- Sub-Topic 2-2: Test parameters for SU-MIMO option

  - Issue 2-2-1: MIMO Correlation

  - Issue 2-2-2: Test point for gain requirements

  - Issue 2-2-3: Test metric

- Sub-Topic 2-3: Test parameters for MU-MIMO option

  - Issue 2-3-1: Simplification of MU-MIMO scheduling

  - Issue 2-3-2: Precoder generation in TE for DUT and co-scheduled UEs

  - Issue 2-3-3: Sub-band Size

  - Issue 2-3-4: Channel Model

  - Issue 2-3-5: MIMO Correlation

  - Issue 2-3-6: MCS and Rank

  - Issue 2-3-7: Test metric

Sub-topic 2-1: Test setup (SU-MIMO vs MU-MIMO)

Issue 2-1-1: SU-MIMO VS MU-MIMO Setup

- Proposals

  - Option 1: SU-MIMO Set-up (Apple, R&S, Huawei, Qualcomm, Samsung)

- Option 1a: Using SU-MIMO set-up to introduce PMI test cases meanwhile  a MU-MIMO setup based demodulation test with test metric of either follow PMI based or random PMI based throughput can be introduced (Huawei)

- Option 1b: Introduce Type II PMI test cases under SU-MIMO test set-up in Rel-16 timeframe. Further study and define proper performance requirements if needed under MU-MIMO scenarios in Rel-17 performance enhancement WI (Samsung)

  - Option 2: MU-MIMO Set-up (Ericsson, Nokia)

- Recommended WF

5 companies preferred SU-MIMO set-up, and 2 companies prefer option2. And one TE vendor provides feedback the current proposal MU-MIMO setup is not feasible

The performance under SU-MIMO and MU-MIMO performance with Rel-16 Type II and Type I codebook summarized in following tables (5 companies provided results for SU-MIMO set-up, only 2 company provide results for MU-MIMO scenario)

FDD 16x2 Medium -64 QAM Rank2-with SU-MIMO set-up

| Company | Rel16 Enhanced Type II PMI |  | Rel15 Enhanced Type II PMI |  | Type I PMI |  | TP ratio eType II/Type I |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNR in dB at 90% of peak throughput | TP ratio | SNR in dB at 90% of peak throughput | TP ratio | SNR in dB at 90% of peak throughput | TP ratio |  |
| Samsung | 6.3 | 10.33 | 12.6 | 2.9 | 15.0 | 2.75 | 2.1 |
| Qualcomm | 8.45 | 5.76 | 10.44 | 4.10 | 13.84 | 2.10 | 2.75 |
| Ericsson | 11.3 | 3.81 | 12.1 | 2.82 |  |  | 1.12/1.32 (70% point) |
| Huawei |  |  | 9.82 |  |  |  |  |
| Apple | 11.28 | 4.69 | 12.16 | 3.57 | 14.5 | 2.4 |  |

FDD 16x2 Medium –MCS 7 Rank1-with SU-MIMO set-up

| Company | Rel16 Enhanced Type II PMI |  | Rel15 Enhanced Type II PMI |  |
| --- | --- | --- | --- | --- |
|  | SNR in dB at 90% of peak throughput | Gain Type II/Type I | SNR in dB at 90% of peak throughput | Gain Type II/Type I |
| Samsung (Option 1: Xb using random PMI) | -2.5 | 0.99 | -4.3 | 0.99 |
| Ericsson (not mentioned) | -2.81 | 1.17 | -2.67 | 2.82 |

Major concern from the proponent of option 2 is that option 1 with SU-MIMO set-up, test cases cannot discriminate UE behaviour to follow eType II codebook as Type I codebook is the sub-set of eType II codebook.

From the simulation results from all the companies’ results, we can see that Type II show enough performance gap over than Type I, which is enough to discriminate UE behaviour to ensure proper UE processing.

This issue has been discussed over 3 meeting cycles without agreements, and this is the basic set-up we started to define eType II requirement in Rel-16. As rapporteur, we do encourage companies comprised to a concrete proposal and make decision in this meeting.

As recommended WF:

  - Introduce Rel-16 Type II codebook requirements with SU-MIMO set-up under the condition that with proper test parameters to ensure enough performance difference over than Type I

  - Further study and define proper performance requirements if needed under MU-MIMO scenario in Rel-17 performance enhancement WI.

Sub-topic 2-2: Test parameters for SU-MIMO option

Issue 2-2-1: MIMO Correlation

- Proposals

  - Option 1: XP (custom)low ( Ericsson)

  - Option 2: XP Medium (Previous agreement)

- Recommended WF

  - Agreed option 2.  Option 2 is the agreement in the last meeting for SU-MIMO test setup

Issue 2-2-2: Test point for gain requirements

- Proposals

  - Option 1: 70% of TP (Ericsson)

  - Option 2: 95% of TP (Samsung)

  - Option 3: 90% of TP

- Recommended WF

  - Companies’ feedback needed for above proposal

Issue 2-2-3: Test Metric

- Proposals

  - Option 1: The ratio of following PMI with enhanced type II codebook and type I single panel codebook (Ericsson)

  - Option 2: The ratio of following PMI with enhanced type II codebook and random PMI (Previous agreements)

- Recommended WF

  - Following the agreement in RAN4#94b-e meeting in WF-2005530. The test metric is the ratio of following PMI and random PMI. Based on the simulation results. The gain is feasible with this test metric.

Sub-topic 2-3: Test parameters for MU-MIMO option

Issue 2-3-1: Simplification of MU-MIMO scheduling

- Proposals

  - Option 1 (Nokia): Simplified MU-MIMO test setup with 1 real UE (DUT) where a dummy co-scheduled UE is modelled a through random PMI, TE does not need to encode any data ort control signal for the co-schedule UE. Data sequence for the co-scheduled UE may be generated as random sequence of QAM symbols and processed by the simplified MU-MIMO precoder calculated for the co-scheduled UE

- Recommended WF

  - N.A

Issue 2-3-2: Precoder generation in TE for DUT and co-scheduled UEs

- Proposals

  - Option 1: Using ZF to generate co-schedule UE precoder with rank 1 test clarification  (Nokia)

W=WaXbH where Wa is the orthogonalized and normalised Wa and XbH is the normalized projection of the co-scheduled PMI on the null space of Wa XbH=I-WaWaHWb diagq-1/2

- Initialisation: wa is an Nt×1 unit-norm vector (wa=1) reconstructed from the PMI reported by the DUT. - Step 1) Generate a random vector wb of size Nt×1, with elements drawn from a complex Gaussian distribution of mean 0 and variance 1. - Step 2) Calculate the quantities: pb=wbHwb, and pab=waHwb - Step 3) xb=(wb-pabwa)/pb-pab2

- Recommended WF

  - N.A

Issue 2-3-3: Sub-band Size

- Proposals

  - Option 1: (Ericsson)

- 4 for FDD with 15kHz SCS, 10MHz CBW

- 8 for TDD with 30kHz SCS, 40MHz CBW

- Recommended WF

  - Companies’ feedback needed for above proposal

Issue 2-3-4: Channel Model

- Proposals

  - Option 1: TDLC300-5 (Ericsson)

- Recommended WF:

  - Companies’ feedback needed for above proposal

Issue 2-3-5: MIMO Correlation

- Proposals

  - Option 1: XP (custom)low (Ericsson)

- Recommended WF:

  - Companies’ feedback needed for above proposal

Issue 2-3-6: MCS and Rank

- Proposals

  - Option 1: MCS 11 (64QAM Table), Rank 1 (Ericsson)

- Recommended WF:

  - Companies’ feedback needed for above proposal

Issue 2-3-7: Test metric

- Proposals

  - Option 1 :  The ratio of following PMI with following PMI with enhanced type II codebook and type I single panel codebook (Ericsson, Nokia)

- Recommended WF:

  - Companies’ feedback needed for above proposals

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Company | Sub-Topic 2-1: Test set-up SU-MIMO VS. MU-MIMO - Issue 2-1-1: SU-MIMO VS MU-MIMO Setup Sub-Topic 2-2: Test parameters for SU-MIMO option - Issue 2-2-1: MIMO Correlation - Issue 2-2-2: Test point for gain requirements - Issue 2-2-3: Test metric Sub-Topic 2-3: Test parameters for MU-MIMO option - Issue 2-3-1: Simplification of MU-MIMO scheduling - Issue 2-3-2: Precoder generation in TE for DUT and co-scheduled UEs - Issue 2-3-3: Sub-band Size - Issue 2-3-4: Channel Model - Issue 2-3-5: MIMO Correlation - Issue 2-3-6: MCS and Rank - Issue 2-3-7: Test metric |
| Apple | Sub-Topic 2-1: Test set-up SU-MIMO VS. MU-MIMO - Issue 2-1-1: SU-MIMO VS MU-MIMO Setup We agree with the recommended way forward from moderator. Sub-Topic 2-2: Test parameters for SU-MIMO option - Issue 2-2-1: MIMO Correlation XP_medium is already agreed in last meeting and also have good performance and prefer not to change it. - Issue 2-2-2: Test point for gain requirements We prefer to use same test point as Type I codebook – 90% max TP - Issue 2-2-3: Test metric We prefer to use same test metric as previously agreed for SU-MIMO setup. TP ratio of follow PMI vs random PMi |
| Ericsson | Sub-Topic 2-1: Test set-up SU-MIMO VS. MU-MIMO - Issue 2-1-1, 2-2-1, 2-2-2, 2-2-3 We can agree to use the SU-MIMO based setup if and only if we distinguish the performance over Type I PMI reporting. When we create these requirements, companies’ have simulated and shown performance gains over Type I codebook with different simulations. Yet if we measure the throughput over random PMI we do not capture those performance gains into the specification. Furthermore, with simulations coming from different companies the requirements will create a minimum requirement based on all simulations from all companies. E.g., if one company shows ~6dB gain over Type I with Type II codebook it will not be reflected in the specification when another company may only show ~2dB gain. The point of simulations is to align a minimum gain requirement. Considering the 6dB gain may not be used as the sole value when creating requirements. Thus, the only way we see that the performance may be distinguished for the Type II family of codebooks is to measure the gain over the Type I codebook. Thereby ensuring that the UE has a different implementation of the two codebooks and ensuring a performance gain as such. In our simulations we see that there is slightly more performance gain when tweaking the correlation matrix according to our alpha, beta, and gamma values. However, there may also be sufficient performance gains when medium correlation is configured. It is also questionable whether these performance gains observed with Type II codebook will be negligible if we do not ensure that there is a different implementation for the Type II codebook compared to the Type I codebook. Furthermore, as we highlight in our paper, the performance under ‘random’ PMI is very diverging amongst companies’ and it poorly reflects a test metric to highlight Type II implementation. ‘Random’ PMI feedback showcases solely that performance will be very poor if the gNB randomly selects a precoder to employ when precoding the transmission. Indeed this should be the case for large antenna arrays that the codebook will have a large amount of poor PMIs to select from. If we measure against the Type I codebook. We have a very good baseline to create requirements which will suitably reflect a performance gain when employing more advanced codebooks. Also, the expected performance for the Type I codebook is much more stable and will create a requirement where the benchmark reference guarantees that the Type II codebook performs as expected. Sub-Topic 2-3: Test parameters for MU-MIMO option For the MU-MIMO approach we see that the conditions and limitations of the previous WF causes the performance of Type II codebook to degrade. As a result, the performance gain previously employed when the MU-MIMO environment considered zero-forcing accurate PMI reports both from the DUT, and the co-scheduled UE are not attainable. As such, for MU-MIMO performance, we may need to look into Rel-17 and study how to setup a test which is suitable from both a performance perspective. But also, from a feasibility perspective. |
| Huawei | Sub-Topic 2-1: Test set-up SU-MIMO VS. MU-MIMO - Issue 2-1-1: SU-MIMO VS MU-MIMO Setup In our paper, we have listed four reasons to support SU-MIMO for Type II and eType II PMI reporting test in Rel-16. Four reasons for us to support using SU-MIMO for (e) Type II codebook PMI reporting test: - Significant gain of Type II over Type I under SU-MIMO test setup has been observed - SU-MIMO is simple and wildly used for years - After several meetings discussion, parameter configurations for SU-MIMO test setup are all settled - The goal of testing Type II codebook based PMI reporting can be reached So, our preference is to use SU-MIMO test setup for Type II and eType II codebook based PMI testing. For MU-MIMO test setup, it has its own advantages in testing and we also see a wild demand on it, which makes it hard to be ruled out. So if we are going to have MU-MIMO as test setup, our preference is to have it in a demodulation test case. The thing is that there will be a new topic called Advance Receiver in Rel-17 which will mainly focus on the interference elimination on UE receiver. And it is agreed that MU-MIMO will be introduced as a special scenario and discuss the evaluation of performance requirements. In this case, we think MU-MIMO also can be further introduced at that time. To summarize, we prefer to define Type II and eType II codebook using SU-MIMO test setup, and define a new demodulation test case using MU-MIMO test setup only if MU-MIMO test setup is agreed to be introduced in Rel-16. Update and comments to Samsung: Ensuring a robust testing will also need to ensure the performance gains for the Rel-15 Type II codebook. Given that the discussion on PMI reporting requirements for Type II codebooks, we must ensure that the performance gains can be observed for both Rel-15 and Rel-16 Type II. The throughput margin may be sufficient for Rel-16 eTypeII codebook. But looking into the results we see very small margin between the best Type I results and the worst Rel-15 Type II results 0.07: Company; Rel16 Enhanced Type II PMI; Rel15 Enhanced Type II PMI; Type I PMI; TP ratio eType II/Type I. SNR in dB at 90% of peak throughput; TP ratio; SNR in dB at 90% of peak throughput; TP ratio; SNR in dB at 90% of peak throughput; TP ratio. Samsung; 6.3; 10.33; 12.6; 2.9; 15.0; 2.75; 2.1. Qualcomm; 8.45; 5.76; 10.44; 4.10; 13.84; 2.10; 2.75. Ericsson; 11.3; 3.81; 12.1; 2.82; 1.12/1.32 (70% point). Huawei; 9.82. Apple; 11.28; 4.69; 12.16; 3.57; 14.5; 2.4. Furthermore, for 4Rx antennas the margin may be even smaller. As observed in our discussion paper for 4Rx scenario the throughput difference is smaller than in 2Rx conditions. Sub-Topic 2-2: Test parameters for SU-MIMO option - Issue 2-2-1: MIMO Correlation Support option 2 and keep previous agreement. - Issue 2-2-2: Test point for gain requirements Depends on the issue 2-2-3. - Issue 2-2-3: Test metric Since now companies have different views on how to implement ‘Random PMI’ for (e) Type II codebook based PMI reporting tests, the throughput ratio by using the agreed test metric ‘Follow PMI versus Random PMI’ is not stable. Based on the simulation results submitted by different companies, the performance requirement will suffer from the difficulty of alignment for TP ratio of ‘Follow PMI versus Random PMI’ due to big gaps observed. In this case, we think that option 1, which refers to the ratio of following PMI with enhanced type II codebook and type I single panel codebook, can be considered since it can reflect the enhancement and also ensure the right reporting of Rel-16 Type II codebook if a reasonable gain (TP ratio of following PMI with enhanced type II codebook and type I single panel codebook) can be reached. Sub-Topic 2-3: Test parameters for MU-MIMO option Pending for the issue 2-1-1. |
| Nokia/NSB | Sub-Topic 2-1: Test set-up SU-MIMO VS. MU-MIMO - Issue 2-1-1: SU-MIMO VS MU-MIMO Setup We are ok with the recommended WF provided the test metric is robust enough to differentiate eType II from Type I SP. We also support further studying CSI requirements for the MU-MIMO scenario in Rel-17, with the aim at addressing certain limitations of the current MU-MIMO test setup, for example by increasing the number of co-scheduled UEs and/or the transmission rank, and the feasibility of the test. Sub-Topic 2-2: Test parameters for SU-MIMO option - Issue 2-2-1: MIMO Correlation We are open to consider the proposed custom correlation if it shows consistently better differentiation of eType II codebook against Type I. - Issue 2-2-3: Test metric We prefer Option 1. It is more robust to set performance requirements for a new codebook type against a well-established baseline, i.e., Type I SP, particularly because the new codebook structure is designed as an extension of the baseline codebook. Conversely, comparing against a random PMI made sense for Type I SP CSI requirements because there was no other codebook reference available at that time. Sub-Topic 2-3: Test parameters for MU-MIMO option Pending decision on Issue 2-1-1. |
| Samsung | Issue 2-1-1: SU-MIMO VS MU-MIMO Setup We are ok with recommend WF. As we summarized in our paper, both options have pros and cons - Option 1: Test set-up with one target UE scheduled a.k.a SU-MIMO test set-up  Pros: Widely used in existing LTE and NR PMI test cases, the feasibility already been verified; proceed the work quickly since RAN4 has lot of experience  Cons: not close to the real network scenario of scheduling Type II codebook, not reflect the advantage of Type II codebook over than Type I codebook under Multi-user co-scheduling scenario - Option 2: Test set-up with co-scheduled interference UE a.k.a MU-MIMO set-up  Pros: Close to type II codebook usage scenario  Cons: More complexity, the detailed test set-up still unclear and the feasibility from performance requirements and test aspect need to be further confirmed; bring uncertainty of completing Type II codebook test cases in Rel-16 timeframe The main concern for SU-MIMO test set-up was whether we can discriminate UE performance with eType II over than Type I sine Type I is subset of eType II we should guarantee UE which only implement Type I can’t pass eType II test cases. Based on the summarized result in below tables, we can see that type II show enough performance gap over than type I. We think the gain is enough to discriminate UE behavior to ensure proper UE processing. In that sense, it is feasible to use SU-MIMO test setup to ensure UE report Rel-16 type II codebook other than Rel-15 type I. On the other side, we observed only 2 companies submit results for MU-MIMO set-up. And from our results, under the proposed MU-MIMO set-up, eType II results even worse than Type I. There are many uncertainties need to be further evaluated/studied for MU-MIMO scenario before RAN4 start to define proper performance requirements i.e. receiver assumption, co-scheduled UE generation, test feasibility. With these uncertainties, it’s not realistic to introduce requirements under MU-MIMO in Rel-16 timeframe. Meanwhile we also realized that evaluation performance on MU-MIMO performance was one of the work objectives in approved Rel-17 performance enhancement WI. Regarding how to guarantee that introducing proper test requirements under SU-MIMO set-up to guarantee the discrimination of eTypeII over than Type I i.e. incorrect UE which employ Type I feedback should not pass the test cases. From our understanding, it’ still possible to use the agreed test metric (TP ratio of following PMI and Random PMI with eType II at 90% to discriminate UE performance. Based on the results summarized table as Company; Rel16 Enhanced Type II PMI; Rel15 Enhanced Type II PMI; Type I PMI; TP ratio eType II/Type I. SNR in dB at 90% of peak throughput; TP ratio; SNR in dB at 90% of peak throughput; TP ratio; SNR in dB at 90% of peak throughput; TP ratio. Samsung; 6.3; 10.33; 12.6; 2.9; 15.0; 2.75; 2.1. Qualcomm; 8.45; 5.76; 10.44; 4.10; 13.84; 2.10; 2.75. Ericsson; 11.3; 3.81; 12.1; 2.82; 1.12/1.32 (70% point). Huawei; 9.82. Apple; 11.28; 4.69; 12.16; 3.57; 14.5; 2.4. The minimum TP ratio with eType II following PMI/random PMI from companies is 3.81 and the maximum TP ratio with type I following PMI and random PMI is 2.75. If we introduce test requirements as following PMI/random PMI among [2.75 -3.81] i.e. 3.0 we still can ensure all UEs which follow eType II codebook can pass the test cases meanwhile if UE use Type I codebook for reporting, all UEs will fail the test In that sense, we think it’s possible using the agreed test metric following PMI/random PMI under SU-MIMO test set-up under XP medium correlation channel Meanwhile, we have observed that the performance under random PMI is very diverging, it may poorly reflect the test metric to justify the type II implementation. We may need further align the way of doing random PMI to avoid uncertainty and unexpected results brought by infinite random parameters We suggest to further clarify these issues to align the random PMI generation - Beam steering approach Whether to enable two beam steer direction with 2 beam direction for both type II and type I codebook simulation? - Beam randomization Whether to select the possible beams under the RRC configuration for following PMI? How to generate the main beam index and the remaining 2L-1 beams as well as the related amplitude and phase coefficient? Issue 2-2-1: MIMO Correlation We are ok with recommend WF Issue 2-2-3: Test Metric We are ok with recommend WF. Based on the companies result, it is feasible to define the test metric with following PMI/random PMI under XP medium correlation channel. While as mentioned there is different implementation of random PMI, it may poorly reflect the test metric to justify the type II implementation, we suggest to clarify the simulation assumption to ensure enough performance difference over type I under this metric. ### Sub-topic 2-3: Test parameters for MU-MIMO option For MU-MIMO setup, only 2 companies submit results. As shown in our result, under the proposed MU-MIMO set-up, eType II results even worse than Type I. There are many uncertainties need to be further evaluated/studied for MU-MIMO scenario before RAN4 start to define proper performance requirements i.e. receiver assumption, co-scheduled UE generation, test feasibility. With these uncertainties, it’s not realistic to introduce requirements under MU-MIMO in Rel-16 timeframe. Meanwhile we also realized that evaluation performance on MU-MIMO performance was one of the work objectives in approved Rel-17 performance enhancement WI. Therefore, we propose to further study and define proper performance requirements if needed under MU-MIMO scenario in Rel-17 performance enhancement WI. |
| Vodafone | We echo the same views as Nokia and Ericsson. I.e., we are ok with the recommended WF provided the test metric is robust enough to differentiate eType II from Type I SP, and a minimum requirement should be set based on all simulations from all companies. |
| Qualcomm | Sub-Topic 2-1: Test set-up SU-MIMO VS. MU-MIMO - Issue 2-1-1: SU-MIMO VS MU-MIMO Setup - Support option 1b Sub-Topic 2-2: Test parameters for SU-MIMO option - Issue 2-2-1: MIMO Correlation - Option 2: XP medium - Issue 2-2-2: Test point for gain requirements - Option 3: 90% TP - Issue 2-2-3: Test metric - Option 2 Sub-Topic 2-3: Test parameters for MU-MIMO option We don’t support using the MU-MIMO set-up at this point. - Issue 2-3-1: Simplification of MU-MIMO scheduling - Issue 2-3-2: Precoder generation in TE for DUT and co-scheduled UEs - Issue 2-3-3: Sub-band Size - Issue 2-3-4: Channel Model - Issue 2-3-5: MIMO Correlation - Issue 2-3-6: MCS and Rank - Issue 2-3-7: Test metric |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2014747 (Samsung) Draft CR for introduction of Rel-16 Type II PMI test cases | Company A |
|  | Ericsson: May need to be revised based on the ongoing discussion. |
|  | Huawei: Proposed change affects should be ‘ME’. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |
| Sub-topic 2-1 and 2-3 | Issue 2-1-1: SU-MIMO VS MU-MIMO Setup Tentative agreements: - Introduce Rel-16 Type II codebook requirements with SU-MIMO set-up under the condition that with proper test parameters to ensure enough performance difference over than Type I - Further study and define proper performance requirements if needed under MU-MIMO scenario in Rel-17 performance enhancement WI. Recommendations for 2nd round: Further check the open issue in sub-topic 2-2 with test point and test metric to ensure enough performance different over the type I |
| Sub-topic 2-2 | Issue 2-2-1: MIMO Correlation Tentative agreements: - XP Medium Recommendations for 2nd round: N.A Issue 2-2-2: Test point for gain requirements Candidate options: - Proposals - Option 1: 70% of TP (Ericsson) - Option 2: 90% of TP (baseline) Recommendations for 2nd round: Further check the open issue in sub-topic 2-2 with test point and test metric to ensure enough performance different over the type I Issue 2-2-3: Test Metric Candidate options: - Proposals - Option 1: The ratio of following PMI with enhanced type II codebook and type I single panel codebook (Ericsson, Nokia, Vodafone, Huawei) - Option 2: The ratio of following PMI with enhanced type II codebook and random PMI (baseline) Recommendations for 2nd round: Further check the open issue in sub-topic 2-2 with test point and test metric to ensure enough performance different over the type I |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | Way forward on PMI reporting requirement for NR eMIMO | Ericsson, Samsung |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2014747 | To be revised |

Discussion on 2nd round (if applicable)

Agreement in GTW in 1st round:

Introduce Rel-16 eType II codebook requirements with SU-MIMO set-up under the condition that with proper test parameters, test metric/ test requirements and test procedure to ensure enough performance difference over than Type I i.e. UE which employ Type I reporting will fail in the test case

- Test metric : Further check results based on following candidate options during 2nd round

  - Option 1: Following PMI/Random PMI

  - Option 2: Following with eType II codebook / following PMI with Type I codebook

Further study and define proper performance requirements if needed under MU-MIMO scenario in Rel-17 performance enhancement WI.

The same agreements applied for Rel-15 Type II test case(s).

[Qualcomm]:

One question regarding the options below. Do we expect to make a decision in this meeting? Thanks

•Test metric : Further check results based on following candidate options during 2nd round

•Option 1: Following PMI/Random PMI

•Option 2: Following with eType II codebook / following PMI with Type I codebook

[Samsung]:

Regarding the new option as

•Option 3: Following eTypeII codebook / random PMI with Type I

we can not accept it. The motivation of option 2 is discriminate the UE behaviour over Type I. if we go with option 3, how we justify the eType II implementation?

Ericsson:

[Qualcomm]: You were included in the offline discussions regarding the motivation behind including that option, but in short here are the motivations:

since the problematic seems to be the volatility of the results from different companies when comparing against Random Type II benchmark, in our view Random Type I benchmark can provide a more robust metric and we don’t expect to see huge variations when using that as a reference; Regarding discerning UEs using type I, the Gain provided by eType II codebook is expected to be larger than the gain provided by Type I, and that will reflect on the requirement. If the Gain is considered sufficient, how the UE generated the report should not be target of the test;

I’d like to propose to keep the WF as in latest version, including Option 3. Since the discussion is very open and we’ll need a new round of results, I don’t see the need to restrict the options to 1 and 2

[Samsung]: From our side, we donot prefer to keep many options.

We agree with you the intention with random type I PMI as benchmark can avoid the volatility of random eType II PMI implementation

As you commented, with the improved of implementation type I with no change eType II implementation can also result in the relative gain is marginal.

In that sense, can we agree to narrow down option 3 for test metric of eType II codebook requirement, if sufficient gain can be observed with option 3?

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2 …. |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2017531 WF on PMI reporting requirement for NR eMIMO | Treated on the GTW session. Revised to R4-2017678 The revised version should be agreeable |
| R4-2017535 Draft CR for introduction of Rel-15 Type II PMI test cases | Agreeable |

Topic #3: Structure for specfications

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2014558 | Intel | Proposal 10: Adopt proposed procedure to organize specification structure for multi-TRP Tx schemes - 5.2.2.1.5 Minimum requirements for PDSCH 0.001% BLER - 5.2.2.1.6 Minimum requirements for PDSCH repetitions over multiple slots - 5.2.2.1.7 Minimum requirements for PDSCH Mapping Type B and UE processing capability 2 - 5.2.2.1.8 Minimum requirements for PDSCH pre-emption - 5.2.2.1.9 Minimum requirements for PDSCH HST-SFN - 5.2.2.1.10 Minimum requirements for PDSCH HST-DPS - 5.2.2.1.11 Minimum requirements for PDSCH with multi-DCI based Tx scheme - 5.2.2.1.12 Minimum requirements for PDSCH with single-DCI based SDM scheme - 5.2.2.1.13 Minimum requirements for PDSCH with single-DCI based FDM Scheme A - 5.2.2.1.14 Minimum requirements for PDSCH with single-DCI based Inter-slot TDM scheme |

Open issues summary

Sub-topic 3-1: Structure for specification for demodulation and CSI

Issue 3-1-1: specification for demodulation and CSI

Considering only one official meeting for NR eMIMO WI, the CR work split is proceed to finalization of performance requirement on schedule as follows

|  | Responsibility |
| --- | --- |
| CR on general and applicability rule | Apple |
| CR on PDSCH requirement with Single-DCI based SDM scheme | Huawei |
| CR on PDSCH requirement with Multi-DCI based multi-TRP | Huawei |
| CR on PDSCH requirement with Single-DCI based FDMScheme A (if agreed) | Intel |
| CR on PDSCH requirement with Single-DCI based inter-slot TDM scheme (if agreed) | Intel |
| CR on PMI reporting requirement based on Regular eType-II | [Samsung] |
| CR on Annex (FRC if any) | Ericsson |

The volunteer companies have submitted the corresponding CR in this meeting. Regarding how to capture eMIMO feature into specification, both adding a separate section or continuous section are feasible.

Considering the current progress of HST and URLLC WI, it was agreed to use the same section for specifying PDSCH demodulation requirement for different WI. Based on discussions on previous meetings it was agreed to introduce 4 new clauses for URLLC performance requirements. Two new clauses are needed to define HST-SFN and HST-DPS requirement.

For example, the following is the existing specification for 2Rx with FDD for PDSCH demodulation requirements and CSI requirement

In this sub-topic, the section numbers will be discussed and allocated. According to the agreements, the new features should be added following the existing requirements. Companies provide views for 2Rx FDD, then 2Rx TDD and 4Rx FDD/TDD will the agreement

- Proposals

  - Option 1(Intel):

- PDSCH demodulation requirements

- 5.2.2.1.5 Minimum requirements for PDSCH 0.001% BLER

- 5.2.2.1.6 Minimum requirements for PDSCH repetitions over multiple slots

- 5.2.2.1.7 Minimum requirements for PDSCH Mapping Type B and UE processing capability 2

- 5.2.2.1.8 Minimum requirements for PDSCH pre-emption

- 5.2.2.1.9 Minimum requirements for PDSCH HST-SFN

- 5.2.2.1.10 Minimum requirements for PDSCH HST-DPS

- 5.2.2.1.11 Minimum requirements for PDSCH with multi-DCI based Tx scheme

- 5.2.2.1.12 Minimum requirements for PDSCH with single-DCI based SDM scheme

- 5.2.2.1.13 Minimum requirements for PDSCH with single-DCI based FDM Scheme A

- 5.2.2.1.14 Minimum requirements for PDSCH with single-DCI based Inter-slot TDM scheme

- Reporting of Precoding Matrix Indicator (PMI)

- 6.3.2.1.5	Multiple PMI with 16TX TypeII Codebook

- 6.3.2.1.6	Multiple PMI with 16TX Enhanced Type II Codebook

- Recommended WF

  - Agree option 1?

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Company A | Sub-Topic 3-1: Structure for specification of demodulation and CSI - Issue 3-1-1: Specification of demodulation and CSI |
| Apple | Sub-Topic 3-1: Structure for specification of demodulation and CSI - Issue 3-1-1: Specification of demodulation and CSI We are fine with the proposed structure from Intel |
| Huawei | Sub-Topic 3-1: Structure for specification of demodulation and CSI - Issue 3-1-1: Specification of demodulation and CSI OK with option 1. |
| Samsung | - Issue 3-1-1: Specification of demodulation and CSI We are ok with option1 for minor changes to align with RAN1 eMIMO feature terminology - 5.2.2.1.11 Minimum requirements for PDSCH with multi-DCI based multi-TRP - 5.2.2.1.12 Minimum requirements for PDSCH with single-DCI based SDM scheme - 5.2.2.1.13 Minimum requirements for PDSCH with single-DCI based FDM Scheme A - 5.2.2.1.14 Minimum requirements for PDSCH with single-DCI based Inter-slot TDM scheme |
| Qualcomm | Sub-Topic 3-1: Structure for specification of demodulation and CSI - Issue 3-1-1: Specification of demodulation and CSI The proposed structure looks reasonable. May be revised according to resolution of topics 1 and 2. |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
|  | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |
|  | - PDSCH demodulation requirements - 5.2.2.1.5 Minimum requirements for PDSCH 0.001% BLER - 5.2.2.1.6 Minimum requirements for PDSCH repetitions over multiple slots - 5.2.2.1.7 Minimum requirements for PDSCH Mapping Type B and UE processing capability 2 - 5.2.2.1.8 Minimum requirements for PDSCH pre-emption - 5.2.2.1.9 Minimum requirements for PDSCH HST-SFN - 5.2.2.1.10 Minimum requirements for PDSCH HST-DPS - 5.2.2.1.11 Minimum requirements for PDSCH with multi-DCI based multi-TRP - 5.2.2.1.12 Minimum requirements for PDSCH with single-DCI based SDM scheme - 5.2.2.1.13 Minimum requirements for PDSCH with single-DCI based FDM Scheme A - 5.2.2.1.14 Minimum requirements for PDSCH with single-DCI based Inter-slot TDM scheme - Reporting of Precoding Matrix Indicator (PMI) - 6.3.2.1.5 Multiple PMI with 16TX TypeII Codebook - 6.3.2.1.6 Multiple PMI with 16TX Enhanced Type II Codebook |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |