3GPP TSG-RAN WG4 Meeting #97-e	R4-2017645

# Electronic Meeting, 2- 13 Nov, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 5695 | rev | - | Current version: | 15.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Mirror CR: Updates to Rel-15 LTE V2X performance requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_V2X-Perf |  |  |  |  |  | Date: |  |  | 2020-11-9 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - The square bracket of SNR point @ 10% BLER for soft buffer test requirement in Table 14.7-2 is still existing. - For PSCCH/PSSCH decoding test, this test can’t verify the maximum number of bits per TTI and it is verified on soft buffer test. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the square bracket of SNR point @ 10% BLER for soft buffer test requirement in Table 14.7-2 - Delete the sentence “and the maximum number of bits” for PSCCH/PSSCH decoding test. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Performance requirements for soft buffer is confusing - The purpose of PSCCH/PSSCH decoding test is wrong. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.7, 14.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

14.7	Soft buffer test

The purpose of this test is to verify the maximum number of bits per TTI supported by the V2X UE.

The minimum requirement is specified in Table 14.7-2 with the test parameters specified in Table 14.7-1.

Table 14.7-1: Test Parameters

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Communication resource pool configuration |  |  | As specified in Table A.9-4 (Configuration #4-V2X) |  |
| at antenna port |  | dBm/15kHz | -98 |  |
| Active cell(s) |  |  | None |  |
| V2X Tx format (SL-V2X-TxProfileList-r15) |  |  | N/A | SL-V2X-TxProfile-r15 = rel15 |
| Active Sidelink UE(s) |  |  | Sidelink UE i, 0 ≤ i ≤ 14 |  |
| Sidelink UE i, 0 ≤ i ≤ 14 | Sidelink Transmissions |  | PSCCH + PSSCH |  |
|  | Time gap between initial transmission and retransmission | Subframe | 15 |  |
|  | Timing offset (Note 1) | s | 0 |  |
|  | Frequency offset (Note 2) | Hz | 0 |  |
|  | Synchronization source |  | GNSS or GNSS-equivalent |  |
|  | Antenna configuration |  | 1x2 |  |
| Note 1: Time offset of Sidelink UE receive signal with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. Note 3: 15 sidelink UEs should transmit one by one circularly for every subframe. |  |  |  |  |

Table 14.7-2: Minimum performance

| Test num. | Bandwidth | PSSCH Reference channel | Propagation condition | Reference value |  | ue-CategorySL-C-RX |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | PSSCH BLER (%) | SNR (dB) of PSSCH |  |
| 1 | 20 MHz | CD.11 | AWGN | 5 | 8.0 | SL-C-RX Category 2 |
| 2 | 20 MHz | CD.15 | AWGN | 5 | 12.0 | SL-C-RX Category 3 |

14.8	PSCCH/PSSCH decoding capability test

The purpose of this test is to verify the maximum number of Sidelink processes supported by the V2X UE.

The minimum requirements are specified in Table 14.8-3 with the test parameters specified in Table 14.8-1 and Table 14.8-2 according to UE capability.

Table 14.8-1: Test Parameters

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Communication resource pool configuration |  |  | As specified in Table A.9-5 (Configuration #5-V2X) |
| Active cell(s) |  |  | None |
| Sidelink UE i, 0 ≤ i ≤ 9 | Sidelink Transmissions |  | PSCCH + PSSCH |
|  | Timing offset (Note 1) | s | 0 |
|  | Frequency offset (Note 2) | Hz | 0 |
|  | Synchronization source |  | GNSS or GNSS-equivalent |
|  | Propagation Channel |  | Static propagation condition No external noise sources are applied |
|  | Antenna configuration |  | 1x2 |
|  | PSSCH RMC |  | CD.12 |
| Note 1: Time offset of Sidelink UE receive signal with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. Note 3: Cyclic shift for PSCCH DMRS is randomly selected between {0, 3, 6, 9} for each PSCCH transmission. |  |  |  |

Table 14.8-2: Test Parameters

| Parameter |  | Unit | Test 2 |
| --- | --- | --- | --- |
| Communication resource pool configuration |  |  | As specified in Table A.9-5 (Configuration #5-V2X) |
| Active cell(s) |  |  | None |
| Sidelink UE i, 0 ≤ i ≤ 19 | Sidelink Transmissions |  | PSCCH + PSSCH |
|  | Timing offset (Note 1) | s | 0 |
|  | Frequency offset (Note 2) | Hz | 0 |
|  | Synchronization source |  | GNSS or GNSS-equivalent |
|  | Propagation Channel |  | Static propagation condition No external noise sources are applied |
|  | Antenna configuration |  | 1x2 |
|  | PSSCH RMC |  | CD.10 |
| Note 1: Time offset of Sidelink UE receive signal with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. Note 3: Cyclic shift for PSCCH DMRS is randomly selected between {0, 3, 6, 9} for each PSCCH transmission. |  |  |  |

Table 14.8-3: Minimum performance

| Test number | Bandwidth | PSCCH Reference channel | at antenna port (dBm/15kHz) | Reference value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Probability of missed PSCCH (%) |
| 1 | 20 MHz | CC.8 | -85 | 1 |
| 2 | 20 MHz | CC.8 | -85 | 1 |

<End of change>