3GPP TSG-RAN WG4 Meeting#98-e	R4- 2100170

Online, 25th Jan – 5th Feb, 2021

Source: 	Qualcomm Incorporated

Title: 	Views on URLLC High BLER Demodulation Test Cases

Agenda item:	7.8.1.2.1

Document for:	Discussion

- Introduction

In the last meeting, WF [1] listed different options for testing URLLC features. In this paper, we discuss our views on test cases for URLLC features.

- High Reliability Requirements

In WF [1], it was agreed to choose the MCS for the FR2 tests for high reliability requirements based on evaluation results. We observed below results from our simulations for FR2 TDD with 2 slot aggregation.

Table 1: FR2 Slot Aggregation Simulation Results

| Case | SNR in dB at 1e-2 BLER | Impairment SNR in dB | Requirement SNR in dB |
| --- | --- | --- | --- |
| MCS 13 | -5.64 | -3.64 | -3.14 |
| MCS 16 | -3.11 | -1.11 | -0.61 |
| MCS 19 | -1.36 | 0.64 | 1.14 |

Based on above results, it can be noticed that requirement SNR is greater than -1dB for MCS 16 and 19. Therefore, we propose the following.

Proposal 1: Define high reliability high BLER FR2 slot aggregation tests with MCS 16 or 19 in Low SE MCS Table.

- DL Pre-emption Requirements

Based on WF [1], below are our simulation results for FDD.

Table 2: FDD DL Pre-emption Simulation Results

| Preemption Probability | Antenna Configuration | MCS | SNR (dB) at 70% peak thpt with buffer flushing | SNR (dB) at 70% peak thpt without buffer flushing | Gain (dB) |
| --- | --- | --- | --- | --- | --- |
| 10% | 2x2 | 4 | -1.99 | -1.03 | 0.96 |
| 10% | 2x4 | 4 | -4.92 | -4.07 | 0.85 |
| 10% | 2x2 | 13 | 5.77 | 7.23 | 1.46 |
| 10% | 2x4 | 13 | 2.40 | 3.53 | 1.13 |
| 10% | 2x2 | 16 | 8.57 | 10.40 | 1.83 |
| 10% | 2x4 | 16 | 5.07 | 6.49 | 1.42 |
| 20% | 2x2 | 4 | -1.09 | 0.52 | 1.61 |
| 20% | 2x4 | 4 | -4.18 | -2.64 | 1.54 |
| 20% | 2x2 | 13 | 6.88 | 11.08 | 4.2 |
| 20% | 2x4 | 13 | 3.27 | 7.38 | 4.11 |
| 20% | 2x2 | 16 | 9.73 | NA | NA |
| 20% | 2x4 | 16 | 5.94 | NA | NA |

Based on above simulations, 10% pre-emption probability with MCS4 or MCS13 has big enough loss to define the requirements for DL pre-emption. Therefore, we propose the following.

Proposal 2: Define DL pre-emption test for eMBB with 10% pre-emption probability and MCS 4.

In case of TDD, we see an issue with current RAN4 simulation assumptions. Currently, it is assumed that there are 8 HARQ processes and slot format is 7D1S2U. Pre-emption scheduling is assumed to be fixed and it is also assumed that UE sends NACK on the slots with pre-emption without trying to decode it. Based on these assumptions, pre-emption HARQ timeline looks as in Figure 1.

Figure 1: HARQ Timeline of DL pre-emption in case of TDD

In the above figure, red slot is being pre-empted and each pre-emption has the same HARQ process ID. As a result, all 4 transmissions are NACK’ed based on current RAN4 assumptions. In this case, there will be no performance difference whether UE does the pre-emption or not. Therefore, we observe and propose the following.

Observation 1: There is no performance difference between correct pre-emption or no pre-emption in case of TDD based on current simulation assumptions.

Proposal 3: Use 10 HARQ processes in case of TDD DL Pre-emption tests.

- FR2 Mapping Type B Test

Based on simulation assumptions in [2], below are the simulation results for FR2 Type B test case.

Table 3: Simulation result for FR2 Type B Test Case

| Test | Alignment SNR (dB) at 70% of peak throughput | Impairment SNR (dB) |
| --- | --- | --- |
| FR2, 2x2, MCS4 | -0.40 | 1.60 |

- Conclusions

This paper discusses the testability of certain URLLC features and test cases. Following has been observed and proposed:

Proposal 1: Define high reliability high BLER FR2 slot aggregation tests with MCS 16 or 19 in Low SE MCS Table.

Proposal 2: Define DL pre-emption test for eMBB with 10% pre-emption probability and MCS 4.

Observation 1: There is no performance difference between correct pre-emption or no pre-emption in case of TDD based on current simulation assumptions.

Proposal 3: Use 10 HARQ processes in case of TDD DL Pre-emption tests.

# References

[1] R4-2017509, “WF on URLLC UE performance requirements with higher BLER”, 3GPP TSG RAN WG4 Meeting #97-e, Online, Nov 2 - Nov 13, 2020.

[2] R4-2017510, “Simulation assumptions for NR URLLC UE FR2 performance requirements test cases”, 3GPP TSG RAN WG4 Meeting #97-e, Online, Nov 2 - Nov 13, 2020.