3GPP TSG-RAN WG4 Meeting # 98-e 	R4-2100367

Electronic Meeting, Jan. 25-Feb. 5, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.809 | CR | Draft CR | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TR 38.809: IAB-MT EVM measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Core |  |  |  |  |  | Date: |  |  | 2020-12-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | IAB-MT EVM measurement process decision background is missing in the TR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | IAB-MT EVM measurement process decision background is added in the TR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There’s no EVM measurement process decision background in the TR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.2.2, 9.6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of the changes >

#### 7.5.2.2	Error Vector Magnitude

EVM performance is the SNR performance of the transmitted signal. In order to have the same link performance, IAB-MT output signal quality should have the same performance as UE then guarantee the link quality. UE requirements are reused by IAB-MT EVM requirements. The difference is that BPSK EVM requirement is removed considering BPSK modulation is not likely to be used by the backhaul link. As IAB-MT is part of IAB node which is a network node, the principle of EVM frame structure for IAB-MT measurement can reuse BS EVM frame structure. IAB-MT Tx EVM measurement is the measurement for UL signals, R15 FR1 UE EVM measurement process is taken as reference. Because EVM is SNR performance and there’s no SNR difference for different physical channels/signals, IAB-MT Tx EVM measurement can only measure PUSCH physical channel. The descriptions of other physical channels/signals in UE EVM measurement process are removed for IAB spec.

< Next change >

9.6.2.2	Error Vector Magnitude

IAB-MT OTA EVM requirement analysis is the same as the conducted requirement in 7.5.2.2. IAB-MT type1-O EVM requirements should be the same with conducted requirements. IAB-MT type2-O EVM requirement reuses UE FR2 EVM requirements with the exception that BPSK requirement is removed. As IAB-MT is part of IAB node which is a network node, the principle of EVM frame structure for IAB-MT measurement can reuse BS EVM frame structure. Based on the same background as 7.5.2.2, R15 FR2 UE EVM measurement process is taken as reference and the descriptions of physical channels/signals other than PUSCH in UE EVM measurement process are removed for IAB spec.

< End of the changes >