3GPP TSG-RAN WG4 Meeting # 98-e 	R4-2100371

Electronic Meeting, Jan. 25-Feb. 5, 2021

Title: 	Discussion on IAB-MT test model

Source: 	CATT

Agenda item:	7.4.3.2.2

Document for:	Approval

- Introduction

This contribution provides our further study of IAB-MT test model. Some detail proposals are provided for discussion.

- Discussion

The followings are the agreements in the last RAN4 meeting.

IAB-MT test models:

IAB-MT tests models will be introduced for UL operation, regarding the detailed parameters need to be included in Test models will be further discussed.

- We will further compare the UE test models (uplink RMC) and BS Test models to narrow down and simplify the necessary information

Test model WF:

Test model design takes BS TMs as baseline (regarding the amount of specified details) and further discussion is concentrates on what, if anything, needs to be added from UE test models

For the FR1 and FR2, detail go through is needed to see how to design the IAB-MT test model.

There’s a general issue need to be discussed and decided that if all of the physical channels PUSCH, PUCCH, PRACH, SRS should be tested. Our view is that for RF test, PUSCH test may be sufficient.

Proposal 1: Only PUSCH is tested for the RF test.

2.1 FR1 IAB-MT test models

2.1.1 General configuration and parameters

For the configurations of TDD for IAB-MT, comparing BS and UE configuration can be done to see the solution. The following table is copied from TS 38.141.

==============

Table 4.9.2.2-1: Configurations of TDD for BS type 1-C and BS type 1-H test models

| Field name | Value |  |  |
| --- | --- | --- | --- |
| referenceSubcarrierSpacing (kHz) | 15 | 30 | 60 |
| Periodicity (ms) for dl-UL-TransmissionPeriodicity | 5 | 5 | 5 |
| nrofDownlinkSlots | 3 | 7 | 14 |
| nrofDownlinkSymbols | 10 | 6 | 12 |
| nrofUplinkSlots | 1 | 2 | 4 |
| nrofUplinkSymbols | 2 | 4 | 8 |

=================

The following is copied from TS 38.521,

===============

TDD slot patterns defined for reference sensitivity tests will be used for UL RMCs defined below.

===============

Table A.3.3.1-1: Additional reference channels parameters for TDD

| Parameter |  | Value |  |  |
| --- | --- | --- | --- | --- |
|  |  | SCS 15 kHz (µ=0) | SCS 30 kHz (µ=1) | SCS 60 kHz (µ=2) |
| TDD Slot Configuration pattern (Note 1) |  | DDDSU | 7DS2U | 14DS1S24U |
| Special Slot Configuration (Note 2) |  | 10D+2G+2U | 6D+4G+4U | S1=12D+2G, S2=6G+8U |
| referenceSubcarrierSpacing |  | 15 kHz | 30 kHz | 60 kHz |
| UL-DL configuration | dl-UL-TransmissionPeriodicity | 5 ms | 5 ms | 5 ms |
|  | nrofDownlinkSlots | 3 | 7 | 14 |
|  | nrofDownlinkSymbols | 10 | 6 | 12 |
|  | nrofUplinkSlot | 1 | 2 | 4 |
|  | nrofUplinkSymbols | 2 | 4 | 8 |
| Number of HARQ Processes |  | 8 | 8 | 16 |
| The number of slots between PDSCH and corresponding HARQ-ACK information (Note 3) |  | K1 = 4 if mod(i,5) = 0 K1 = 3 if mod(i,5) = 1 K1 = 2 if mod(i,5) = 2 where i is slot index per frame; i = {0,…,9} | K1 = 8 if mod(i,10) = 0 K1 = 7 if mod(i,10) = 1 K1 = 6 if mod(i,10) = 2 K1 = 5 if mod(i,10) = 3 K1 = 4 if mod(i,10) = 4 K1 = 3 if mod(i,10) = 5 K1 = 2 if mod(i,10) = 6 where i is slot index per frame; i = {0,…,19} | K1 = 13 if mod(i,20) = 2 K1 = 12 if mod(i,20) = 3 K1 = 11 if mod(i,20) = 4 K1 = 10 if mod(i,20) = 5 K1 = 9 if mod(i,20) = 6 K1 = 8 if mod(i,20) = 7 K1 = 7 if mod(i,20) = 8 K1 = 6 if mod(i,20) = 9 K1 = 6 if mod(i,20) = 10 K1 = 6 if mod(i,20) = 11 K1 = 6 if mod(i,20) = 12 K1 = 6 if mod(i,20) = 13 where i is slot index per frame; i = {0,…,39} |
| NOTE 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. NOTE 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. NOTE 3: i is the slot index per frame. |  |  |  |  |

======================

It can be seen that the TDD UL-DL pattern for UE test is the same as BS except the special slot configuration is provided in the UE test configuration. So this part can be common for IAB-DU and IAB-MU.

Proposal 2: The configurations of TDD for IAB-MT reuse the same configuration table as BS test, i.e. leave the special slot configuration in test implementation.

For the common physical channel parameters, BS parameters can’t be reused because the DL signal should be changed to UL signal. TS 38.521 has the details of the parameters in A.2. However, the information in TS 38.521 is redundant, for example that every CBW is tested for each SCS. There’re two ways to treat this part, first is that extract the useful information from TS 38.521, second is that refer to TS 38.521 with the clarification that only the information for the tested CBW information is used. Our understanding is that referring TS 38.521 can be a choice to make the IAB test spec cleaner. Extracting the information from

Proposal 3: IAB-MT common physical channel parameters can refer TS 38.521 with the clarification that only the information for the used CBW is referred.

2.1.2 Test models

As it was agreed that IAB-MT follows BS structure, BS test models can be reviewed one by one to see if there need some modification for IAB-MT. The test model for NB-IoT is not needed, a total of 8 test models are reviewed and summarized in the following table.

Table 1: FR1 test models

| BS Test Models | Necessary for IAB-DU? | Necessary for IAB-MT? | Parameters change for IAB-MT |
| --- | --- | --- | --- |
| NR-FR1-TM1.1 | yes | yes | # of PRBs PUSCH: NRB Modulation PUSCH: QPSK Waveform: DFT-s-OFDM and CP-OFDM |
| NR-FR1-TM1.2 | yes | no (no boosting function for IAB-MT) | - |
| NR-FR1-TM2 | yes | Depends on how to test power dynamic range | If it’s needed, changing PDSCH to PUSCH and adding DFT-s-OFDM waveform. |
| NR-FR1-TM2a | yes | Depends on how to test power dynamic range | If it’s needed, changing PDSCH to PUSCH and adding DFT-s-OFDM waveform. |
| NR-FR1-TM3.1 | yes | Yes The model needs to be reviewed for frequency error | Using BS approach that IAB-MT NR-FR1-TM1.1 modulation is changed to 64QAM. |
| NR-FR1-TM3.1a | yes | Yes The model needs to be reviewed for frequency error | Using BS approach that IAB-MT NR-FR1-TM1.1 modulation is changed to 256QAM. |
| NR-FR1-TM3.2 | yes | No (no boosting for IAB-MT) | - |
| NR-FR1-TM3.3 | yes | No (no boosting for IAB-MT) | - |

Observation 1: NR-FR1-TM1.1, NR-FR1-TM3.1 and NR-FR1-TM3.1a are needed for IAB-MT, some modifications are needed to adapt to UL signal.

There’s no power control requirement for BS, so the model for IAB-MT power control need to be designed after the decision of power control.

Observation 2: The test models for IAB-MT power control and frequency error need to be reviewed after the decision of the two tests.

2.1.3 Data content of Physical channels and Signals

The data content of UL signal can’t reuse BS spec, mapping of uplink physical channels and signals to physical resources for UE test is described in TS 38.521 Table G.1-1. However, it’s similar with common physical channel parameters, there’re some redundant information. Dealing with this can be the same as physical channel parameters that referring this table with some information that only the useful information is used.

Observation 3: FR1 IAB-MT physical channel and signals mapping can refer TS 38.521-1 with the clarification that only the information for the used physical channels is referred.

2.2 FR2 IAB-MT test models

2.2.1 General configuration and parameters

For the general configuration, after comparing with TS 38.521-2 and TS 38.141-2, there’re some differences for the TDD pattern if Tx signals are tested.

The followings are copied from TS 38.141-2,

==========================

Table 4.9.2.2-1: Configurations of TDD for BS type 2-O test models

| Field name | Value |  |
| --- | --- | --- |
| referenceSubcarrierSpacing (kHz) | 60 | 120 |
| Periodicity (ms) for dl-UL-TransmissionPeriodicity | 1.25 | 1.25 |
| nrofDownlinkSlots | 3 | 7 |
| nrofDownlinkSymbols | 10 | 6 |
| nrofUplinkSlots | 1 | 2 |
| nrofUplinkSymbols | 2 | 4 |

=======================

The followings are copied from TS 38.521-2,

=======================

For UL RMCs defined below, TDD slot pattern defined in Table A.2.3-1 will be used for the requirements requiring at least one sub frame (1ms) for the measurement period. For other requirements, TDD slot patterns defined for reference sensitivity tests in Table A.3.3.1-1 will be used.

Table A.2.3-1: Additional reference channels parameters for TDD

| Parameter |  | Value |  |
| --- | --- | --- | --- |
|  |  | SCS 60 kHz (µ=2) | SCS 120 kHz (µ=3) |
| TDD Slot Configuration pattern (Note 1) |  | DDDSUUUU | 7DS8U |
| Special Slot Configuration (Note 2) |  | S=4D+6G+4U | S=12D+2G |
| UL-DL configuration | referenceSubcarrierSpacing | 60 kHz | 120 kHz |
|  | dl-UL-TransmissionPeriodicity | 2 ms | 2 ms |
|  | nrofDownlinkSlots | 3 | 7 |
|  | nrofDownlinkSymbols | 4 | 12 |
|  | nrofUplinkSlot | 4 | 8 |
|  | nrofUplinkSymbols | 0 | 0 |
|  | UL slot numbers | mod(slot index, 40) = {36,…,39} | mod(slot index, 80) = {72,…,79} |
| NOTE 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. NOTE 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. |  |  |  |

===================

It can be seen the UE FR2 Tx test TDD slot configuration pattern is not the same as BS. The Tx time duration for UE is 1 ms which is near BS Tx time duration. From that aspect, it may be reasonable that using UE TDD pattern for IAB-MT test model. The views from TE vendors is needed that if the Tx time duration is too small when BS TDD patern is used. More discussion is needed to make the decision.

Observation 4: For FR2, TDD pattern for UE Tx and BS Tx are different. It may need some discussion if UE TDD reference parameters can be reused for IAB-MT.

Similar with FR1, we have the following observation,

Observation 5: FR2 IAB-MT common physical channel parameters can refer TS 38.521-2 with the clarification that only the information for the used CBW is referred.

2.2.2 Test models

Very similar with FR1, BS FR2 test models can be reviewed to see if the model is necessary and if any modification is needed.

Table 1: FR2 test models

| BS Test Models | Necessary for IAB-DU? | Necessary for IAB-MT? | Parameters change for IAB-MT |
| --- | --- | --- | --- |
| NR-FR2-TM1.1 | yes | yes | # of PRBs PUSCH: NRB Modulation PUSCH: QPSK Waveform: DFT-s-OFDM and CP-OFDM |
| NR-FR2-TM2 | yes | Depends on how to test power dynamic range | If it’s needed, changing PDSCH to PUSCH and adding DFT-s-OFDM waveform. |
| NR-FR2-TM2a | yes | Depends on how to test power dynamic range | If it’s needed, changing PDSCH to PUSCH and adding DFT-s-OFDM waveform. |
| NR-FR2-TM3.1 | yes | Yes The model needs to be reviewed for frequency error | Using BS approach that NR-FR2-TM1.1 modulation is changed to 64QAM. |
| NR-FR2-TM3.1a | yes | Yes The model needs to be reviewed for frequency error | Using BS approach that NR-FR2-TM1.1 modulation is changed to 256QAM. |

2.2.3 Data content of Physical channels and Signals

The observation for this part is very similar as FR1,

Observation 6: FR2 IAB-MT physical channel and signals mapping can refer TS 38.521-2 with the clarification that only the information for the used physical channels is referred.

- Conclusion

We discuss IAB-MT test model and provides the following proposals and observations,

Proposal 1: Only PUSCH is tested for the RF test.

Proposal 2: The configurations of TDD for IAB-MT reuse the same configuration table as BS test, i.e. leave the special slot configuration in test implementation.

Proposal 3: IAB-MT common physical channel parameters can refer TS 38.521 with the clarification that only the information for the used CBW is referred.

Observation 1: NR-FR1-TM1.1, NR-FR1-TM3.1 and NR-FR1-TM3.1a are needed for IAB-MT, some modifications are needed to adapt to UL signal.

Observation 2: The test models for IAB-MT power control and frequency error need to be reviewed after the decision of the two tests.

Observation 3: FR1 IAB-MT physical channel and signals mapping can refer TS 38.521-1 with the clarification that only the information for the used physical channels is referred.

Observation 4: For FR2, TDD pattern for UE Tx and BS Tx are different. It may need some discussion if UE TDD reference parameters can be reused for IAB-MT.

Observation 5: FR2 IAB-MT common physical channel parameters can refer TS 38.521-2 with the clarification that only the information for the used CBW is referred.

Observation 6: FR2 IAB-MT physical channel and signals mapping can refer TS 38.521-2 with the clarification that only the information for the used physical channels is referred.

Reference

[1] R4-2017490, “WF on dynamic range, power control (LA) and frequency error for IAB-MT”