3GPP TSG-RAN4 Meeting #98-e	R4-2100431

# Electronic meeting, Jan. 25 – Feb. 5, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1473 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on performance requirement for CSI-SINR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CSIRS_L3meas-Perf |  |  |  |  |  | Date: |  |  | 2021-1-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The performance requirements for CSI-SINR L3 measurement need to be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the performance requirements for CSI-SINR L3 measurement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance requirements for CSI-SINR L3 measurement are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.12, 10.1.12.2, 10.1.13, 10.1.13.2, 10.1.14, 10.1.14.2, 10.1.15, 10.1.15.2, 10.1.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

10	Measurement Performance requirements

10.1	NR measurements

10.1.12	Intra-frequency SINR accuracy requirements for FR1

10.1.12.2	Intra-frequency CSI-SINR accuracy requirements

10.1.12.2.1	Absolute CSI-SINR Accuracy

Unless otherwise specified, the requirements for absolute accuracy of CSI-SINR in this clause apply to the intra-frequency measurement defined in 9.10.2.1 in FR1.

The accuracy requirements in Table 10.1.12.2.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for associated SSB.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.8 for a corresponding Band for each relevant CSI-RS.

-	The configuration of CSI-RS resource is {D=3 with PRB≥48}.

Table 10.1.12.2.1-1: CSI-SINR Intra frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  | Note 3 | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| 3.0 | 4 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| 6.5 | 8 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The requirements apply for CSi-RS Ês/Iot ≤ 10 dB. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

<End of Change 1>

<Start of Change 2>

10.1.13	Intra-frequency SINR accuracy requirements for FR2

10.1.13.2	Intra-frequency CSI-SINR accuracy requirements

10.1.13.2.1	Absolute CSI-SINR Accuracy

Unless otherwise specified, the requirements for absolute accuracy of CSI-SINR in this clause apply to the intra-frequency measurement defined in 9.10.2.1 in FR2.

The accuracy requirements in Table 10.1.13.2.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.8 for a corresponding Band for each relevant CSI-RS.

-	The configuration of CSI-RS resource is {D=3 with PRB≥48}.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.13.2.1-1: CSI-SINR Intra frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSi-RS Ês/Iot | Io Note 2 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 1 |  | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |
| 3 | 4 | ≥-3 | Same value as CSI_RP in Table B.2.8-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| 6.5 | 8 | ≥-6 |  |  |  |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the CSI-RS Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. Note 4: The requirements apply for CSi-RS Ês/Iot ≤ 10 dB. |  |  |  |  |  |

<End of Change 2>

<Start of Change 3>

10.1.14	Inter-frequency SINR accuracy requirements for FR1

10.1.14.2	Inter-frequency CSI-SINR accuracy requirements

10.1.14.2.1	Absolute CSI-SINR Accuracy

Unless otherwise specified, the requirements for absolute accuracy of CSI-SINR in this clause apply to the inter-frequency measurement defined in 9.10.3.1 in FR1.

The accuracy requirements in Table 10.1.14.2.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for associated SSB.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.9 for a corresponding Band for each relevant CSI-RS.

-	The configuration of CSI-RS resource is {D=3 with PRB≥48}.

Table 10.1.14.2.1-1: CSI-SINR Inter frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| 3.0 | 4 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| 6.5 | 8 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The requirements apply for CSI-RS Ês/Iot ≤ 10 dB. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

10.1.14.2.2	Relative CSI-SINR Accuracy

The relative accuracy of CSI-SINR is defined as the CSI-SINR measured from one cell compared to the CSI-SINR measured from another cell with the same center frequency, or between any two CSI-SINR levels measured on the same cell in FR1.

The accuracy requirements in Table 10.1.14.2.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for the associated SSB.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.9 for a corresponding Band for each relevant CSI-RS.

-	The configuration of CSI-RS resource is {D=3 with PRB≥48}.

Table 10.1.14.2.2-1: CSI-SINR Inter frequency relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  | Note 2,4 | NR operating band groups Note 5 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 120 kHz | SCSSSB = 240 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| 2.5 | 3 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| 4 | 4 | -6 | Note 3 | Note 3 | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter CSI-RS Ês/Iot is the minimum CSI-RS Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: The requirements apply for CSI-RS Ês/Iot ≤ 10 dB. NOTE 5: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

<End of Change 3>

<Start of Change 4>

10.1.15	Inter-frequency SINR accuracy requirements for FR2

10.1.15.2	Inter-frequency CSI-SINR accuracy requirements

10.1.15.2.1	Absolute CSI-SINR Accuracy

Unless otherwise specified, the requirements for absolute accuracy of CSI-SINR in this clause apply to the inter-frequency measurement defined in 9.10.3.1 in FR2.

The accuracy requirements in Table 10.1.15.2.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for associated SSB.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.9 for a corresponding Band for each relevant CSI-RS.

-	The configuration of CSI-RS resource is {D=3 with PRB≥48}.

Table 10.1.15.2.1-1: CSI-SINR Inter frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 2 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSCSI-RS Note 1 |  | dBm/BWChannel |
|  |  |  | SCSCSI-RS = 120kHz | SCSCSI-RS = 240kHz |  |
| 3 | 4 | ≥-3 | Same value as CSI_RP in Table B.2.9-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| 6.5 | 8 | ≥-4 |  |  |  |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the CSI-RS Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. Note 4: The requirements apply for CSI-RS Ês/Iot ≤ 10 dB. |  |  |  |  |  |

10.1.15.2.2	Relative CSI-SINR Accuracy

The relative accuracy of CSI-SINR is defined as the CSI-SINR measured from one cell compared to the CSI-SINR measured from another cell with the same center frequency, or between any two CSI-SINR levels measured on the same cell in FR1.

The accuracy requirements in Table 10.1.15.2.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for the associated SSB.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.9 for a corresponding Band for each relevant CSI-RS.

-	The configuration of CSI-RS resource is {D=3 with PRB≥48}.

Table 10.1.15.2.2-1: CSI-SINR Inter frequency relative accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 2 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSCSI-RS Note 1 |  | dBm/BWChannel |
|  |  |  | SCSCSI-RS = 120kHz | SCSCSI-RS = 240kHz |  |
| 2.5 | 3 | ≥-3 | Same value as CSI_RP in Table B.2.9-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| 4 | 4 | ≥-6 |  |  |  |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: The parameter CSI-RS Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. Note 4: In the test cases, the CSI-RS Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. Note 5: The requirements apply for CSI-RS Ês/Iot ≤ 10 dB. |  |  |  |  |  |

<End of Change 4>

<Start of Change 5>

10.1.16	SINR report mapping

#### 10.1.16.1	SS-SINR and CSI-SINR measurement report mapping

The reporting range of SS-SINR and CSI-SINR is defined from -23 dB to 40 dB with 0.5 dB resolution. The mapping of measured quantity is defined in Table 10.1.16.1-1. The range in the signalling may be larger than the guaranteed accuracy range.

The reporting range of differential SS-SINR and CSI-SINR for L1 reporting is defined from -15 dB to 0 dB with 1 dB resolution.

The mapping of measured quantity is defined in Table 10.1.16.1-2. The range in the signalling may be larger than the guaranteed accuracy range.

Table 10.1.16.1-1: SS-SINR and CSI-RSRP measurement report mapping

| Reported value | Measured quantity value (L3 SS-SINR) | Measured quantity value (L1 SS-SINR and L1 CSI-SINR) | Measured quantity value (L3 CSI-SINR) | Unit |
| --- | --- | --- | --- | --- |
| SINR_0 | SS-SINR<-23 | SINR<-23 | CSI-SINR<-23 | dB |
| SINR_1 | -23≤ SS-SINR<-22.5 | -23≤SINR<-22.5 | -23≤ CSI-SINR<-22.5 | dB |
| SINR_2 | -22.5≤ SS-SINR<-22 | -22.5≤SINR<-22 | -22.5≤ CSI-SINR<-22 | dB |
| SINR_3 | -22≤ SS-SINR<-21.5 | -22≤SINR<-21.5 | -22≤ CSI-SINR<-21.5 | dB |
| SINR_4 | -21.5≤ SS-SINR<-21 | -21.5≤SINR<-21 | -21.5≤ CSI-SINR<-21 | dB |
| .. | .. | .. | .. | … |
| SINR_123 | 38≤ SS-SINR<38.5 | 38≤SINR<38.5 | 38≤ CSI-SINR<38.5 | dB |
| SINR_124 | 38.5≤ SS-SINR<39 | 38.5≤SINR<39 | 38.5≤ CSI-SINR<39 | dB |
| SINR_125 | 39≤ SS-SINR<39.5 | 39≤SINR<39.5 | 39≤ CSI-SINR<39.5 | dB |
| SINR_126 | 39.5≤ SS-SINR<40 | 39.5≤SINR<40 | 39.5≤ CSI-SINR<40 | dB |
| SINR_127 | 40≤ SS-SINR | 40≤SINR | 40≤ CSI-SINR | dB |

Table 10.1.16.1-2: Differential SS-SINR and CSI-SINR measurement (for L1 reporting and L3 reporting) report mapping

| Reported value | Measured quantity value (difference in measured SINR from largest SINR) | Unit |
| --- | --- | --- |
| DIFFSINR_0 | 0≥ΔSINR>-1 | dB |
| DIFFSINR_1 | -1≥ΔSINR>-2 | dB |
| DIFFSINR_2 | -2≥ΔSINR>-3 | dB |
| DIFFSINR_3 | -3≥ΔSINR>-4 | dB |
| DIFFSINR_4 | -4≥ΔSINR>-5 | dB |
| DIFFSINR_5 | -5≥ΔSINR>-6 | dB |
| DIFFSINR_6 | -6≥ΔSINR>-7 | dB |
| DIFFSINR_7 | -7≥ΔSINR>-8 | dB |
| DIFFSINR_8 | -8≥ΔSINR>-9 | dB |
| DIFFSINR_9 | -9≥ΔSINR>-10 | dB |
| DIFFSINR_10 | -10≥ΔSINR>-11 | dB |
| DIFFSINR_11 | -11≥ΔSINR>-12 | dB |
| DIFFSINR_12 | -12≥ΔSINR>-13 | dB |
| DIFFSINR_13 | -13≥ΔSINR>-14 | dB |
| DIFFSINR_14 | -14≥ΔSINR>-15 | dB |
| DIFFSINR_15 | -15≥ΔSINR | dB |

<End of Change 5>