3GPP TSG-RAN4 Meeting #98-e	                                                                 R4-2100432

Electronic meeting, Jan. 25 – Feb. 5, 2021

Title:	Discussion on test case for CSI-RS based measurement

Source:	CATT

Agenda Item:	7.14.2.2.1

Document for:	Discussion

1 Introduction

In RAN4#97e meeting, the test case list for CSI-RS based measurement was agreed in [1]. And all the CRs for test cases are endorsed. The structure for test cases is decided basically, but there are still some parameters for test cases to be decided in this meeting. The candidate options are captured in [2] as below.

- Whether to introduce test case for FDD mode - Option 1: No - Option 2: Yes - The values of DRX cycle - Option 1: 40ms for FR1, 320ms for FR2 - Option 2: 40ms for FR1, 640ms for FR2 - Option 3: Others.

In this paper, we have some discussions on these parameters and give our proposals.

2 Discussion

Based on the discussions in our companion paper [3], the performance of CSI-RS based measurement will degrade as the timing offset increases due to the single FFT assumption. And only one set of requirements when timing offset between measurement timing and the target is smaller than CP is proposed. Then the 2 cells in test case should be synchronized and the timing offset can be 3us.

To verify the accuracy requirements for case 1 discussed in [3], the timing offset should be smaller than CP even for test case in FDD mode. There may be no difference for TDD mode and FDD mode if the timing offset is the same. So it may be not necessary to introduce test cases for FDD mode. But to guarantee the full coverage of test cases, we are also fine to define the FDD test cases.

Proposal 1: We have no strong view on whether to introduce test cases for FDD mode. But if they are introduced, the timing offset should also be 3us which is the same as TDD mode.

For DRX cycle, it was agreed that long DRX is used for FR1 and short DRX is used for FR2. For the value of DRX, option 1 which should be 320ms for FR1 and 40ms for FR2 is slightly preferred.

Proposal 2: The DRX cycle for test case is 320ms for FR1 and 40ms for FR2.

3 Summary

In this paper, the test cases for CSI-RS based measurement are discussed, and the following proposals are given：

Proposal 1: We have no strong view on whether to introduce test cases for FDD mode. But if they are introduced, the timing offset should also be 3us which is the same as TDD mode.

Proposal 2: The DRX cycle for test case is 320ms for FR1 and 40ms for FR2.

4 References

- R4-2017229, Work plan for performance part of CSI-RS based L3 measurements, CATT

- R4-2017367, WF on performance requirements of CSI-RS based L3 measurement, CATT

- R4-2100426, Discussion on performance requirement for CSI-RSRP, CATT