3GPP TSG-RAN WG4 Meeting #98-e                                                R4-2100478

Electronic Meeting, 25 Jan. – 05 Feb., 2021

Title: 	Initial performance evaluation for RLM/BFD relaxation factor

Source: 	CATT

Agenda item:	11.9.2

Document for:	Discussion

Introduction

In this contribution, the initial simulation results are provided for RLM/BFD relaxation factor.

Simulation assumption

| UE speed | 3km/h, 30km/h |
| --- | --- |
| Relaxation factor (X) | 2, 4 |
| DRX cycle length | 20ms |
| Carrier frequency | 4GHz |
| SCS | 15kHz SCS for SSB |
| Simulation bandwidth | 20MHz |
| RLM RS | SSB |
| Channel Model | TDL-B |

Simulation results

The simulation results of RLM/BFD relaxation factor are shown as below:

| 3km/h | K=2 | K=4 |
| --- | --- | --- |
| 5% | -1.89 | -2.62 |
| 95% | 1.96 | 2.55 |

| 30km/h | K=2 | K=4 |
| --- | --- | --- |
| 5% | -4.46 | -5.23 |
| 95% | 4.6 | 5.35 |