3GPP TSG-RAN WG4 Meeting#98e                          				           				R4-2100488

Electronic meeting, 25 Jan. - 5 Feb., 2021

Source: 	CATT

Title: 	Further discussion on UE parameters for 6.425-7.125GHz, 7.025-7.125GHz and 10.0-10.5GHz

Agenda Item:	10.1.1

Document for:	Discussion

- Introduction

In RAN#87-e meeting, a SI on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz was approved in [1]. There were extensive discussions on this topic and a WF was approved in [2, 3, 4]. Companies are encouraged to have further inputs on how to reply the parameters regarding the open issue in [4].

This paper further presents our views on UE parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz.

- Discussion

According to the agreed WF [4], the following UE parameters are still open for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz.

- ACLR/ACS pending further check according to indoor simulations

- Out of band emissions

ACLR

The following ACLR and ACS value have been agreed [4] for UE operating in Rural/Macro suburban/Macro urban/Small cell outdoor/Micro urban scenarios. Whether they apply to the small cell indoor/indoor urban scenario is FFS. According to the simulation results in [5, 6], the required ACIR for all uplink and downlink cases does not exceeds 20dB to ensure both 5% average T-put loss and 5% cell edge T-put loss. Hence the following ACLR/ACS value can be confirmed for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz bands.

Table 2-1 ACLR/ACS

| Frequency range | UE ACLR | UE ACS |
| --- | --- | --- |
| 6.425-7.025GHz, 7.025-7.125GHz | 26 | 32 |
| 10.0-10.5GHz | 24 | 31 |

Proposal 1: The ACLR/ACS value in Table 2-1 can be confirmed for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz bands.

Out of band emissions

We agree to reuse the out of band emission in TS 38.101-1 for 6.425-7.125 GHz and 10.0-10.5 GHz bands.

Proposal 2: reuse the out of band emission in TS 38.101-1 for 6.425-7.125 GHz and 10.0-10.5 GHz bands.

- Conclusion

This contribution further discussed the open UE parameters for the frequency range 6.425-7.125GHz and 10.0-10.5GHz and gave our proposals.

Proposal 1: The ACLR/ACS value in Table 2-1 can be confirmed for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz bands.

Proposal 2: reuse the out of band emission in TS 38.101-1 for 6.425-7.125 GHz and 10.0-10.5 GHz bands.

References

[1]	RP-200513,	New SI proposal: Study on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz	Ericsson, Huawei, HiSilicon

[2] R4-2005733, WF on BS and UE IMT parameters for the SI on 6.425-7.125GHz and 10.0-10.5GHz, CATT

[3] R4-2009151, WF on BS and UE IMT parameters for the SI on 6.425-7.125GHz and 10.0-10.5GHz, CATT

[4] R4-2015809, WF on BS and UE parameters for 6.425-7.125 and 10.0-10.5 GHz, Nokia, CATT, Ericsson, ZTE, Huawei, Qualcomm

[5] R4-2014459, Simulation results for 6425-7125MHz and 10-10.5GHz-uplink, CATT

[6] R4-2014458, Simulation results for 6425-7125MHz and 10-10.5GHz-downlink, CATT