3GPP TSG-RAN WG4 Meeting # 98-e 			R4-2100567

Electronic Meeting, Jan. 25-Feb. 5, 2021

Source:	Sony, Ericsson

Title:	Peak EIRP and EIRP spherical coverage for PC3 for n262

Agenda item:	9.25.1.1

Document for:	Approval

- Background

The work item introducing the new FR2 47GHz band has been discussed for a couple of RAN4 meetings [3], [4]. In RAN4 #97-e a “WF on UE RF requirement of n262” was approved [5]. Another “WF on multi-band relaxation of n262” was also approved [6].

Remaining items listed in the Status Report to TSG [2] are:

- UE RF core requirement

  - Minimum Peak EIRP

  - REFSENS

  - EIRP spherical coverage

  - EIS spherical coverage

  - MPR

  - Beam correspondence

  - Min output power

  - Multiband relaxations

- BS RF conformance

  - MU/TT

In this contribution we provide our inputs to Minimum Peak EIRP and EIRP spherical coverage. Multiband relaxations are discussed in [8] and REFSENS and EIS spherical coverage for PC3 for n262is discussed in [7]

- Transmitter, PC 3

## Minimum peak EIRP

In RAN4 #97-e 6 companies provided input to min peak EIRP in the range 12.7 to 18 dBm. In the discussion this was narrowed down to two values where the WF [5] says: “Companies are encouraged to provide a view if 16.5 dBm or 16.1 dBm can be specified or not”.  Our detailed analysis is found in [9] where we conclude min peak EIRP of 18 dBm. From our horizon no new facts has been revealed but we can accept 16.5 dBm or 16.1 dBm. However, we believe the value should be averaged over linear scale from in mathematic correctness. Therefore, 16.5 dBm is preferred.

Proposal 1	In light of discussion in RAN4 #97-e we propose min peak EIRP to be 16.5 dBm for PC3: n262

## Spherical coverage PC3

The spherical coverage of an antenna in this frequency range depends on many factors, including, but not limited to, the surface current distribution, the material of the back cover and its separation distance to the antennas, other components around the antenna panel etc. During the discussion of spherical coverage for the previously defined FR2 frequency bands, it was observed that impact from all the factors does not monotonously change with frequency but rather changes periodically (see e.g. [10]). Therefore, it cannot be concluded that the n262 band must be worse than e.g., n259 band in terms of spherical coverage. The total impact (peak and spherical coverage) of the back cover and components around the antenna panel, on the other hand, will increase with frequency.

Observation 1	The spherical coverage performance (delta between peak and 50% EIRP) depends on many factors, and it cannot be concluded that the n262 band must be worse than, e.g., n259 in terms of spherical coverage.

To understand the spherical coverage performance at 47 GHz, a full phone model simulation has been carried out, where the phone model is enclosed with metal bezel and back/front glass. Two antenna panels with 4*1 patch array are used in the simulation which face towards the front and rear side of the phone, respectively.

Figure 1. Simulated gain 47 GHz (n262) and 41 GHz (n259) GHz.

The simulated CDF EIRP spherical performance is shown in  for 47 GHz, where the results at 41 GHz (n262) has also been provided as reference value. Comparing the two frequencies a slightly degradation of spherical coverage in higher frequency is observed. However, the specific value is highly implementation dependent, and is in the range between 0 – 1 dB.  Considering the already tight link budget of the cellular network in higher frequency, it is reasonable to further confine the allowed performance degradation at higher frequency, and thus we think the spherical coverage requirement for n262 should be at most 0.5 dB worse than n259.

Proposal 2	The spherical coverage requirement for n262 should be no more than 0.5 dB worse than requirement for n259.

- Conclusion

In this contribution we have discussed minimum peak EIRP and spherical coverage requirement for PC3: n262. The following observations and proposal are made:

Observation 1	The spherical coverage performance (delta between peak and 50% EIRP) depends on many factors, and it cannot be concluded that the n262 band must be worse than, e.g., n259 in terms of spherical coverage.

Proposal 1	In light of discussion in RAN4 #97-e we propose min peak EIRP to be 16.5 dBm for PC3: n262

Proposal 2	The spherical coverage requirement for n262 should be no more than 0.5 dB worse than requirement for n259.

- References

- RP-201232, 	“New WID: introduction of NR 47GHz band, RAN #88-e, June 2020

- RP-202204,	“Status Report to TSG”, Rapporteur (Nokia)

- R4-2011873,	“Email discussion summary for [96e][135] NR_47GHz_Band”, Moderator (Nokia)

- R4-2016971,	“Email discussion summary for [97e][130] NR_47GHz_Band”, Moderator (Nokia)

- R4-2016879, 	“WF on UE RF requirement of n262”, Qualcomm, Nokia, Sony

- R4-2016880,	“WF on multi-band relaxation of n262”, Apple

- R4-2100569,	“REFSENS and EIS spherical coverage for PC3 for n262”, Sony, Ericsson

- R4-2100568,	“Multiband relaxations for PC3 for n262”, Sony, Ericsson

- R4-2015855,	“Link budget for PC3 for n262”, Sony, Ericsson

- R4-1815845,	“UE Spherical EIRP for the 28GHz and 39GHz bands”, Sony