3GPP TSG-RAN WG4 Meeting # 98-e	R4-2100656

# Electronic Meeting, 25 Jan. - 5 Feb., 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | - | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for PSSCH demodulation requirements for NR V2X |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Perf |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to Big draft CR approach, draft CR for PSSCH demodulation requirements were submitted based on RAN4#97-e meeting agreements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The PSSCH demodulation requirements have been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance requirements will be incompleted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----- << Start of Change 1>> -----

11	V2X Requirements

11.1	Demodulation performance requirements (conducted requirements)

This clause specifies the performance requirements for the sidelink physical channels specified for V2X sidelink.

The definition of frequency ranges (FR1 and FR2) are specified in table 5.1-1 of TS 38.101-3 [8].

11.1.1	General

11.1.2	PSSCH performance requirements

The purpose of the requirements in this subclause is to verify the PSSCH for V2X demodulation performance with a single active PSSCH link.

The minimum requirements are specified in Table 11.1.2-2 with the test parameters specified in Table 11.1.2-1. In this test scenario, GNSS or GNSS-equivalent synchronization source is used and sidelink UE 1 transmits PSCCH and PSSCH.

Table 11.1.2-1: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Resource pool configuration |  |  | As specified in Table A.y-1 (Configuration #1-V2X) |
| NOC at antenna port |  | dBm/15kHz | -98 |
| Active cell(s) |  |  | None |
| Sidelink UE 1 | Sidelink transmissions |  | PSCCH + PSSCH + PSFCH |
|  | Timing offset (Note 1) |  | CP/2-12Ts |
|  | Frequency offset (Note 2) | Hz | +600 |
|  | Synchronization |  | GNSS or GNSS-equivalent |
|  | Antenna configuration |  | 1x2 |
| Note 1: Time offset of sidelink UE receive signal with respect to GNSS referring timing. Note 2: Frequency offset of sidelink UE with respect to GNSS reference frequency. |  |  |  |

Table 11.1.2-2: Minimum performance

| Test num. | Bandwidth (MHz) | PSSCH Reference channel | Propagation condition | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | PSSCH BLER (%) | SNR(dB) of PSSCH |
| 1 | 20 | CD.1 | TBD | 10% | TBD |
| 2 | 20 | CD.2 | TBD |  | TBD |

----- << End of Change 1>> -----

----- << Start of Change 2>> -----

A.x	V2X reference measurement channels

A.x.1	General

The transport block size (TBS) determination procedure is described in clause 8.1.3 of TS 38.214 [12].

A.x.2	Reference measurement channels for PSSCH performance requirements

Table A.x.2-1: PSSCH Reference Channel for V2X

| Parameter |  | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  |  | CD.1 | CD.2 |  |  |
| Channel bandwidth |  | MHz | 20 | 20 |  |  |
| Subcarrier spacing |  | kHz | 30 | 30 |  |  |
| Allocated resource blocks |  | RB | 20 | TBD |  |  |
| CP-OFDM symbols for slot with PSFCH(Note 1) |  |  | 9 | TBD |  |  |
| CP-OFDM symbols for slot without PSFCH |  |  | [12] | TBD |  |  |
| DMRS symbols for slot with PSFCH |  |  | 3 | TBD |  |  |
| DMRS symbols for slot without PSFCH |  |  | [4] | TBD |  |  |
| Modulation order |  |  | QPSK | TBD |  |  |
| MCS index |  |  | 4 | TBD |  |  |
| 2nd stage SCI format 2-A configuration | Payloads | Bits | TBD | TBD |  |  |
|  | α |  | TBD | TBD |  |  |
|  | βoffset |  | TBD | TBD |  |  |
|  | γ | Bits | TBD | TBD |  |  |
| Transport Block Size for slot with PSFCH |  | Bits | TBD | TBD |  |  |
| Transport Block Size for slot without PSFCH |  | Bits | TBD | TBD |  |  |
| Transport block CRC |  | Bits | 24 | TBD |  |  |
| Maximum number of HARQ transmissions |  |  | 1 | TBD |  |  |
| Binary Channel Bits for slots with PSFCH |  |  | TBD | TBD |  |  |
| Binary Channel Bits for slots without PSFCH |  | Bits | TBD | TBD |  |  |
| PSFCH resource period |  | Slot | [4] | TBD |  |  |
| MinTimeGapPSFCH |  | Slot | [3] | TBD |  |  |
| Max. Throughput averaged over 1 frames |  | Mbps |  |  |  |  |
| Note 1: OFDM symbols is for PSCCH/PSSCH transmission not including first symbol (AGC) and PSFCH symbols. |  |  |  |  |  |  |

----- << End of Change 2>> -----

----- << Start of Change 3>> -----

A.y	V2X reference resource pool configurations

Table A.y-1 V2X sidelink communication resource pool for PSSCH tests (Configuration #1-V2X)

| Information Element |  |  | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | CD.1 | CD.2 |
| SL-ResourcePool-r16 | sl-PSCCH-Config-r16 | sl-TimeResourcePSCCH-r16 | 2 | n2 |
|  |  | sl-FreqResourcePSCCH-r16 | n10 | n10 |
|  | sl-SyncAllowed-r16 |  | gnss-Sync-r16 | gnss-Sync-r16 |
|  | sl-SubchannelSize-r16 |  | TBD | TBD |
|  | sl-TimeResource-r16 |  | 160 | 160 |
|  | sl-StartRB-Subchannel-r16 |  | 0 | 0 |
|  | sl-NumSubchannel-r16 |  | TBD | TBD |
|  | sl-Additional-MCS-Table-r16 |  | Not presented | Not presented |

----- << End of Change 3>> -----