3GPP TSG-RAN WG4 Meeting #98-e		                               R4-2100705

Electronic Meeting, 25 Jan – 05 Feb, 2021

Source: 	Murata Manufacturing Co Ltd.

Title: 	REFSENS of n25 for 45MHz channel bandwidth

Agenda Item:	9.22.3	UE RF requirements	[NR_FR1_35MHz_45MHz_BW-Core]

Document for:	Approval

Introduction

In the WF [1], [2], more investigation and discussion were encouraged about n25 operating for 45MHz channel bandwidth (CBW). In this contribution, we provide measurement results of Rx band noise (RxBN) for 30, 35, 40 and 45MHz UL and 45MHz DL (UL/DL asymmetric operation), MSD analysis and REFSENS specifications proposal for n25.

Discussion

WF Review

In the WF [1], [2], the following conditions were agreed for n25 for 45MHz CBW.

◇WF R4-2016863

- Option1: UL BW is limited to 40 MHz

  - No need for NS_03 and associated SEM requirements for 35MHz

  - Low MSD can be specified using current 40MHz UL configuration:

    - Option 1a: Worst case 40MHz UL closest to DL

    - Option 1b: Best case 40MHz UL furthest to DL

    - Option 1c: both 1b and 1c are specified

  - Limited impact to band combinations

- Option2: 45MHz UL BW is supported

  - Need for NS_03 and associated SEM requirements

  - Large MSD with further reduced UL configuration are specified which does not help the link throughput

  - Potential additional work for band combinations with n25 UL

◇WF R4-2016864

- Consider asymmetric UL/DL for n8, n71, n25

- Keep 35MHz for n8, n71 on the DL

- Keep 45MHz for n25 on the DL

- Configuration table is already in place

  - Possibly introduce BCS1 for affected bands

- Re-evaluate REFSENS for the following with agreed UL configuration

  - 20MHz UL/35MHz DL for n8 (Skyworks has provided)

  - 20MHz UL/35MHz DL for n71 (Skyworks has provided)

  - 45MHz DL only for n25 with UL restricted to ≤ 40MHz

  - 35MHz UL/35MHz DL, 45MHz UL/45MHz DL for n3 (Qualcomm, Murata has provided)

  - 35MHz UL/35MHz DL for n25 (Qualcomm, Murata has provided)

- Channel location(s) of asymmetric UL (Worst Case/Best Case) DL cases can be studied for MSD test points

- Other companies encouraged to bring measurements and analysis

Power Amplifier Calibration and Test Waveforms

PA conditions:

The power amplifier (PA) is calibrated to meet 1 dB MPR for Power Class 3 using a 100 RB QPSK DFT-s-OFDM SCS 15kHz with RBs allocated at the channel edge.

Test conditions:

- QPSK DFT-s-OFDM SCS 15kHz with the UL RBs be located as close as possible to the DL operating band but confined within the transmission bandwidth configuration for the CBW;

- Image and Local Oscillator (LO) rejection is set to -28 dBc;

- Counter-IM3 (C-IM3) set to -60 dBc;

- Counter-IM5 (C-IM5) set to -70 dBc;

Measurement Results

Rx band noise for 30, 35, 40 and 45MHz UL and 45MHz DL are measured. For 30MHz, resource block is configured 48RBs that is current UL configuration. For other UL bandwidth, resource block is configured 40RBs.

The measurement results are shown in Table 2-1.

Table 2-1 n25 RxBN measurement results

|  | Tx freq. | Rx freq. | Tx/Rx BW | Tx/Rx Gap | RB alloca. | RxBN | Delta (a, b) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | (MHz) | (MHz) | (MHz) | (MHz) | (-) | (dBm/BW) | (dB) |
| Option 1a | 1895 | 1972.5 | 40/45 | 35 | 40@176 | -27.8 | - |
| Option 1b | 1870 | 1952.5 | 40/45 | 40 | 40@176 | -34.6 | 6.8 |
| Addition 1a | 1897.5 | 1972.5 | 35/45 | 35 | 40@148 | -34.3 | - |
| Addition 1b | 1867.5 | 1952.5 | 35/45 | 45 | 40@148 | -37.7 | 3.4 |
| Addition 2a | 1900 | 1972.5 | 30/45 | 35 | 48@112 | -36.0 | - |
| Addition 2b | 1865 | 1952.5 | 30/45 | 50 | 48@112 | -38.7 | 2.7 |
| Option 2a | 1892.5 | 1972.5 | 45/45 | 35 | 40@203 | -20.8 | - |
| Option 2b | 1872.5 | 1952.5 | 45/45 | 35 | 40@203 | -20.9 | 0.1 |

※Option1a, 1b referenced WF [1] Option1.

Additon1a, 1b, 2a, 2b set the UL BW to 35 and 30MHz with reference to WF [2].

Option2a, 2b referenced WF [1] Option2.

Observation 1:

When the UL bandwidth is reduced, RxBN is significantly improved even Tx-Rx Gap is 35MHz which is same gap for 45MHz UL bandwidth.

Observation 2:

The difference between condition “a” and “b” (i.e. worst case and best case) gets smaller and smaller along with reducing UL bandwidth.

MSD Estimation

The link-budget calculation assumptions are listed in table 3-1 and MSD analysis is shown in table 3-2.

Table 3-1 Typical receiver performance parameters for MSD analysis

| Parameter | Value | Unit |
| --- | --- | --- |
| Antenna isolation | 10 | dB |
| DPX isolation from TX to RX | 50 | dB |
| DPX attenuation at RX band | 40 | dB |
| Diversity receiver filter TX rejection | 40 | dB |
| Front-end loss | 4 | dB |
| Thermal noise at RX ANT port | -164 | dBm/Hz |
| Transceiver effective phase noise due to wider BW | -144 | dBc/Hz |
| SNR requirement for QPSK | -1 | dB |

Table 3-2 MSD analysis for n25

| Parameter | Unit | UL / DL BW |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 40/45MHz |  | 35/45MHz |  | 30/45MHz |  | 45/45MHz |  |
|  |  | Option 1a | Option 1b | Addition 1a | Addition 1b | Addition 2a | Addition 2b | Option 2a | Option 2b |
| UL Config. (SCS=15kHz) | - | 40@176 | 48@176 | 40@148 | 40@148 | 48@112 | 48@112 | 40@203 | 40@203 |
| De-sens. | dB | 4.8 | 10.2 | 3.0 | 5.0 | 2.6 | 4.0 | 16.8 | 16.9 |
| REFSENS | dBm | -77.4 | -82.8 | -82.6 | -84.6 | -83.6 | -85.0 | -70.7 | -70.8 |

Considering the advantages of 40MHz described in Option 1 of WF [1], we think 40MHz is a reasonable UL BW. Based on the estimated result of Option 1a, we make the following proposal.

Proposal 1:

UL BW is limited 40MHz for 45MHz channel bandwidth operation. The REFSENS specifications and UL configurations are shown yellow highlighted in table 7.3.2-1 and table 7.3.2-3, respectively.

Table 7.3.2-1: Two antenna port reference sensitivity QPSK PREFSENS

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| n25 | 15 | -96.5 | -93.3 | -91.5 | -90.3 | -89.3 | -82.2 | -85.4 | -79.5 | -77.4 |  | FDD |
|  | 30 |  | -93.6 | -91.6 | -90.5 | -89.4 | -82.3 | -85.5 | -79.6 | -77.5 |  |  |
|  | 60 |  | -94 | -91.9 | -90.7 | -89.6 | -82.4 | -85.6 | -79.7 | -77.6 |  |  |
| NOTE x: UL BW is limited to 40MHz, and UL channel shall be closest to DL channel within operating band. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2-3: Uplink configuration for reference sensitivity

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| n25 | 15 | 25 | 50 | 50 | 50 | 50 | 48 | 40 | 40 | 40 |  | FDD |
|  | 30 |  | 24 | 24 | 24 | 24 | 24 | 20 | 20 | 20 |  |  |
|  | 60 |  | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |  |  |

*Blue letters are the results for 35MHz channel bandwidth operation proposed from Murata in RAN4#97e [3].

Conclusion

In this contribution, we provide measurement results of Rx band noise (RxBN) for 30, 35, 40 and 45MHz UL and 45MHz DL (UL/DL asymmetric operation), MSD analysis and REFSENS specifications proposal for n25.

Observation 1:

When the UL bandwidth is reduced, RxBN is improved significantly even Tx-Rx Gap is 35MHz which is same gap for 45MHz UL bandwidth.

Observation 2:

The difference between condition “a” and “b” (i.e. worst case and best case) gets smaller and smaller along with reducing UL bandwidth.

Proposal 1:

UL BW is limited 40MHz for 45MHz channel bandwidth operation. The REFSENS and UL configurations are shown yellow highlighted in table 7.3.2-1 and table 7.3.2-3, respectively.

Table 7.3.2-1: Two antenna port reference sensitivity QPSK PREFSENS

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| n25 | 15 | -96.5 | -93.3 | -91.5 | -90.3 | -89.3 | -82.2 | -85.4 | -79.5 | -77.4 |  | FDD |
|  | 30 |  | -93.6 | -91.6 | -90.5 | -89.4 | -82.3 | -85.5 | -79.6 | -77.5 |  |  |
|  | 60 |  | -94 | -91.9 | -90.7 | -89.6 | -82.4 | -85.6 | -79.7 | -77.6 |  |  |
| NOTE x: UL BW is limited to 40MHz, and UL channel shall be closest to DL channel within operating band. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2-3: Uplink configuration for reference sensitivity

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| n25 | 15 | 25 | 50 | 50 | 50 | 50 | 48 | 40 | 40 | 40 |  | FDD |
|  | 30 |  | 24 | 24 | 24 | 24 | 24 | 20 | 20 | 20 |  |  |
|  | 60 |  | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |  |  |

※Blue letters are the results for 35MHz channel bandwidth operation proposed from Murata in RAN4#97e [3].

References

[1]		R4-2016863, WF on general aspects for UE RF requirements, Skyworks Solutions Inc., Apple

[2]		R4-2016864, WF on 35MHz 45MHz REFSENS AMPR, Qualcomm, Skyworks Solutions Inc., […]

[3] 	R4-2015432, REFSENS of n3, n8, n25 and n71 for new channel bandwidth, Murata Manufacturing Co Ltd.