3GPP TSG-RAN WG4 Meeting #98-e                                                            R4-2100964

Online, 25th Jan - 5th Feb, 2021

Source:	Samsung, KDDI

Title:	TP for TR 38.717-03-02: CA_n3-n18-n41

Agenda item:			9.11.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.717-03-02 to include CA_n3A-n18A-n41(2A) according to the request in [1].

2. Reference

- RP-202201, Revised WID on Rel-17 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with 2 bands UL.

# 3. Text Proposal

<Start of Text Proposal>

5.1.x	CA_n3-n18-n41

5.1.x.1	Operating bands for CA

Table 5.1.x.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n3-n18-n41 | n3 | 1710MHz | – | 1780MHz | 1805MHz | – | 1880MHz | FDD |
|  | n18 | 703MHz | – | 748MHz | 758MHz | – | 803MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

5.1.x.2	Channel bandwidths per operating band for CA

Table 5.1.x.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA Configuration | UL Config | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3A-n18A-n41A | CA_n3A-n41A CA_n3A-n18A CA_n18A-n41A | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in Table 5.3.5-1 of TS 38.101-1 [3] and in Table 5.3.5-1 of TS 38.101-2 when operating in single carrier mode.

5.1.x.3	Co-existence studies

Co-existence studies of CA_n3-n18-n41 with 2UL are already covered in the constituent fall-back modes, it can get that:

- IMD2 of band n18 UL and band n41 UL falling to band n3 DL

- IMD2 and IMD3 of band n3 UL and band n41 UL falling to band n18 DL.

- IMD2 and IMD3 of band n3 UL and band n18 L falling to band n41 DL. IMD2 has no issue with KDDI band.

5.1.x.4	REFSENS requirements

Table 5.1.x.4-1 shows the required MSD levels for the CA configuration, its value can reuse the value of DC_18A-41A_n3A in TS 38.101-3 and CA_3A-5A-7A in TS 36.101.

Table 5.1.x.4-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3A-n18A-n41A | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |  |
|  | n3 | 1725 | 5 | 25 | 1820 | N/A | FDD | N/A |  |
|  | n41 | 2630 | 5 | 25 | 2630 | 16.0 | TDD | IMD3 |  |
|  | n18 | 820 | 5 | 25 | 865 | 28.9 | FDD | IMD21 |  |
|  | n3 | 1765 | 5 | 25 | 1860 | N/A | FDD | N/A |  |
|  | n41 | 2630 | 5 | 25 | 2630 | N/A | TDD | N/A |  |
|  | n18 | 830 | 5 | 25 | 875 | 19.0 | FDD | IMD3 |  |
|  | n3 | 1725 | 5 | 25 | 1820 | N/A | FDD | N/A |  |
|  | n41 | 2670 | 5 | 25 | 2670 | N/A | TDD | N/A |  |
|  | n3 | 1755 | 5 | 25 | 1850 | 28.8 | FDD | IMD2 |  |
|  | n41 | 2670 | 5 | 25 | 2670 | N/A | TDD | N/A |  |
|  | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |  |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |  |

<End of Text Proposal>