3GPP TSG-RAN WG4 Meeting #98-e		R4-2101247

E-meeting, 25 January – 5 February, 2021

Agenda item:	7.8.1.2.2

Source:	Intel Corporation

Title:	Simulation results for URLLC BS demodulation requirements

Document for:	Information

# Introduction

In the previous RAN4 meeting way forward on NR URLLC BS performance requirements [1] and paper with simulation assumptions [2] were approved. In this paper we provide alignment and impairment simulation results for agreed test cases.

# Simulation results

## FR1 high reliability requirements

In Figure 1 we provide simulation results for different channel bandwidth/SCS combinations and PUSCH mapping types. Summaries of alignment and impairment results are presented in Table 1 and Table 2, respectively.

| 5 MHz, 15 kHz | 10 MHz, 15 kHz |
| --- | --- |
| 10 MHz, 30 kHz | 40 MHz, 30 kHz |
| Figure 1. PUSCH results for FR1 high reliability requirements. |  |

Table 1. Summary of alignment simulation results for FR1 high reliability requirements

|  | 5 MHz, 15 kHz |  | 10 MHz, 15 kHz |  | 10 MHz, 30 kHz |  | 40 MHz, 30 kHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FDD | TDD | FDD | TDD | FDD | TDD | FDD | TDD |
| PUSCH mapping Type A | -12.8 | -13.0 | -13.5 | -13.6 | -13.6 | -13.6 | -14.3 | -14.3 |
| PUSCH mapping Type B | -12.7 | -12.9 | -13.4 | -13.5 | -13.5 | -13.5 | -14.2 | -14.3 |

Table 2. Summary of impairment simulation results for FR1 high reliability requirements

|  | 5 MHz, 15 kHz |  | 10 MHz, 15 kHz |  | 10 MHz, 30 kHz |  | 40 MHz, 30 kHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FDD | TDD | FDD | TDD | FDD | TDD | FDD | TDD |
| PUSCH mapping Type A | -11.3 | -11.5 | -12.0 | -12.1 | -12.1 | -12.1 | -12.8 | -12.8 |
| PUSCH mapping Type B | -11.2 | -11.4 | -11.9 | -12.0 | -12.0 | -12.0 | -12.7 | -12.8 |

## FR1 PUSCH mapping Type B requirements

In Figure 2 we provide simulation results for different channel bandwidth/SCS combinations. Summary of alignment and impairment results are presented in Table 3.

| 5 MHz, 15 kHz | 10 MHz, 15 kHz |
| --- | --- |
| 10 MHz, 30 kHz | 40 MHz, 30 kHz |
| Figure 2. PUSCH results for FR1 mapping Type B requirements. |  |

Table 3. Summary of results for FR1 mapping Type B requirements.

|  | 5 MHz, 15 kHz | 10 MHz, 15 kHz | 10 MHz, 30 kHz | 40 MHz, 30 kHz |
| --- | --- | --- | --- | --- |
| Alignment results | -1.5 | -2.0 | -1.7 | -2.0 |
| Impairment results | 0.0 | 0.5 | 0.2 | 0.5 |

## FR2 high reliability requirements

In Figure 3 we provide simulation results for different channel bandwidth/SCS combinations. Summary of alignment and impairment results are presented in Table 4.

| 50 MHz, 60 kHz | 100 MHz, 60 kHz |
| --- | --- |
| 50 MHz, 120 kHz | 100 MHz, 120 kHz |
| Figure 3. PUSCH results for FR2 high reliability requirements. |  |

Table 4. Summary of results for FR2 high reliability requirements.

|  | 50 MHz, 60 kHz | 100 MHz, 60 kHz | 50 MHz, 120 kHz | 100 MHz, 120 kHz |
| --- | --- | --- | --- | --- |
| Alignment results | -14.1 | -14.5 | -13.3 | -14.1 |
| Impairment results | -12.6 | -13.0 | -11.8 | -12.6 |

## FR2 PUSCH mapping Type B requirements

In Figure 4 we provide simulation results for different channel bandwidth/SCS combinations. Summary of alignment and impairment results are presented in Table 5.

| 50 MHz, 60 kHz | 100 MHz, 60 kHz |
| --- | --- |
| 50 MHz, 120 kHz | 100 MHz, 120 kHz |
| Figure 4. PUSCH results for FR2 mapping Type B requirements. |  |

Table 5. Summary of results for FR2 mapping Type B requirements.

|  | 50 MHz, 60 kHz | 100 MHz, 60 kHz | 50 MHz, 120 kHz | 100 MHz, 120 kHz |
| --- | --- | --- | --- | --- |
| Alignment results | -6.5 | -6.9 | -6.2 | -7.0 |
| Impairment results | -5.0 | -5.4 | -4.7 | -5.5 |

# Conclusion

In this paper we provided alignment and impairment results for URLLC BS demodulation requirements with higher BLER.

# References

- R4-2017675 “Way forward on NR URLLC BS performance requirements”, Huawei, HiSilicon, RAN4 #97-e, November 2020.

- R4-2017526 “Simulation assumption for URLLC FR2 BS performance requirement with higher BLER” , Huawei, HiSilicon, RAN4 #97-e, November 2020.