3GPP TSG-RAN WG4 Meeting #98e		R4-2101258

Electronic, 25th January – 5th February, 2021

Agenda item:	7.9.4.2.1

Source:	Intel Corporation

Title:	Simulation results for single-DCI based multi-TRP Repetition Tx schemes

Document for:	Discussion

# Introduction

In RAN4 #97e meeting WF on PDSCH demodulation requirements and general aspects of NR eMIMO Rel-16 was agreed[1]. All test parameters for single-DCI based repetition Tx schemes were finalized. In this paper we provide simulation results for corresponding scenarios.

# Discussion

## Simulation results

In Figures 1-2 simulation results for the agreed single-DCI based repetition multi-TRP Tx scenarios are provided for FDD and TDD duplex modes for the test case 1a and 2b. The simulation assumptions can be found in [2]. Demodulation performance summary is provided in Table 1 for ideal and impairment results as well.

| FDM scheme A |  |
| --- | --- |
| Figure 1. Demodulation performance for FDM scheme A |  |

| Inter-slot TDM scheme |  |
| --- | --- |
| Figure 2. Demodulation performance for Inter-slot TDM scheme |  |

Table 1. Demodulation performance at 70% @max achievable throughput

|  | Duplex mode | Antenna configuration | Ideal results, dB | Impairment results, dB |
| --- | --- | --- | --- | --- |
| FDM scheme A SNR at 70% @ max Throughput | FDD | 2x2 | 15.1 | 17.1 |
|  |  | 2x4 | 8.1 | 10.1 |
|  | TDD | 2x2 | 14.8 | 16.8 |
|  |  | 2x4 | 7.7 | 9.7 |
| Inter-slot TDM scheme SNR at 1% BLER | FDD | 2x2 | 0.5 | 2.5 |
|  |  | 2x4 | -3.1 | -1.1 |
|  | TDD | 2x2 | 0.7 | 2.7 |
|  |  | 2x4 | -2.9 | -0.9 |

# Conclusion

In this paper we have provided simulation results to define performance requirements for single-DCI based Repetition Tx schemes.

# References

- R4-2017529 “WF on NR eMIMO PDSCH requirement”, Intel, RAN4 #97-e, November 2020

- R4-2017530 “Simulation assumption for PDSCH requirement with single-DCI and Multi-DCI transmission schemes”, Ericsson, Intel, RAN4 #97-e, November 2020