3GPP TSG-RAN WG4 Meeting #98-e	R4-2101323

# Electronic Meeting,  Jan. 25th  – Feb. 5th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0150 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for 38.101-4: Applicability for PMI reporting test of eType II codebook |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ eMIMO-Perf |  |  |  |  |  | Date: |  |  | 2021-1-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There is no applicability for eType II codebook of NR PMI reporting test |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduces the applicability for eType II codebook of NR PMI requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Applicability for PMI reporting test of eType II codebook is not completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of First change>

CSI reporting requirements (Conducted requirements)

6.1	General

This clause includes conducted requirements for the reporting of channel state information (CSI).

6.1.1	Applicability of requirements

6.1.1.1	General

The minimum performance requirements are applicable to all FR1 operating bands defined in TS 38.101-1 [6].

The minimum performance requirements in Clause 6 are mandatary for UE supporting NR operation, except test cases listed in Clause 6.1.1.3, 6.1.1.4.

6.1.1.2	Applicability of requirements for different number of RX antenna ports

The number of RX antenna ports for different RF operating bands is up to UE declaration.

The UE shall support 2 or 4 RX antenna ports for different RF operating bands. The operating bands, where 4 RX antenna ports shall be the baseline, are defined in clause 7.2 of TS 38.101-1 [6]. The UE requirements applicability for UEs with different number of RX antenna ports is defined in Table 6.1.1.2-1.

Table 6.1.1.2-1: Requirements applicability

| Supported RX antenna ports | Test type | Test list |
| --- | --- | --- |
| UE supports only 2RX | CQI | All tests in Clause 6.2.2 |
|  | PMI | All tests in Clause 6.3.2 |
|  | RI | All tests in Clause 6.4.2 |
| UE supports only 4RX or both 2RX and 4RX | CQI | All tests in Clause 6.2.3 |
|  | PMI | All tests in Clause 6.3.3 |
|  | RI | All tests in Clause 6.4.3 |

6.1.1.3	Applicability of requirements for optional UE features

The performance requirements in Table 6.1.1.3-1 shall apply for UEs which support optional UE features only.

Table 6.1.1.3-1: Requirements applicability for optional UE features

| UE feature/capability [14] | Test type |  |  | Test list |  | Applicability notes |
| --- | --- | --- | --- | --- | --- | --- |
| Supported maximum number of ports across all configured NZP-CSI-RS resources per CC (maxConfigNumberPortsAcrossNZP-CSI-RS-PerCC) |  | FR1 FDD | PMI |  | Clause 6.3.2.1.3 Clause 6.3.2.1.4 Clause 6.3.2.1.5 Clause 6.3.3.1.3 Clause 6.3.3.1.4 Clause 6.3.3.1.5 | The requirements apply only in case that UE supports Type I single panel with Tx ports larger than 8 or Type II codebook |
|  |  | FR1 TDD | PMI |  | Clause 6.3.2.2.3 Clause 6.3.2.2.4 Cluase 6.3.2.2.5 Clause 6.3.3.2.3 Clause 6.3.3.2.4 Cluase 6.3.2.2.5 Cluase 6.3.3.2.5 |  |
| Supported additional codebooks (codebookParametersAddition-r16) |  | FR1 FDD | PMI |  | Clause 6.3.2.1.6 Clause 6.3.3.1.6 | The requirements apply only in case that UE supports eType II codebook |
|  |  | FR1 TDD | PMI |  | Cluase 6.3.2.2.6 Cluase 6.3.3.2.6 |  |

<End of First change