3GPP TSG-RAN WG4 Meeting #98-e	R4-2101348

# Electronic Meeting, Jan. 25th – Feb. 5th, 2021

Title: 	Simulation results on NR-U PUCCH performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	7.1.7.4.3

Document for:	Discussion

Background

At the last meeting, RAN 4 agreed to introduce requirements for PUCCH with PRB-Interlaced resource allocation [1]. In this paper, we provide our initial simulation results.

Simulation results

Simulation results are shown in Figure 2-1~Figure 2-4 and related required SNR values are summarized in Table 2-1.The simulation assumptions derived from [1] are shown in appendix.

Figure 2-1: Simulation results PF0

Figure 2-2: Simulation results PF1

Figure 2-3: Simulation results PF2

Figure 2-4: Simulation results PF3

Table 2-1: Summary of simulation results for NR-U PUCCH.

| Test metric (SNR Db @ 1% of BLER or 1% of ACK miss or 0.1%of NACK) |  | PF0 | PF1 |  | PF2 |  | PF3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ACK miss | ACK miss | NACK2ACK | 4 bits (ACK miss) | 22bits (UCI BLER) | 4 bits | 16bits |  |
|  |  |  |  |  |  |  | 4 symbols (ACK miss) | 4symbols (UCI BLER) | 14 symbols (UCI BLER) |
| TDLA30-10 | 15kHz | -4.6 | -17.2 | -16.7 | -0.1 | 1.3 | -3.2 | -2.8 | -7.5 |
|  | 30kHz | -3.9 | -17.1 | -16.3 | 0.70 | 1.9 | -2.6 | -2.2 | -7.2 |
| TDLC300-100 | 15kHz | -6.2 | -16.0 | -15.3 | -1.5 | -0.4 | -5.1 | -3.9 | -9.0 |
|  | 30kHz | -6.1 | -15.1 | -14.6 | -1.3 | 0.2 | -5 | -3.6 | -8.7 |

Conclusion

In this paper, we provide our simulation results for NR-U PUCCH performance requirements.

Reference

[1]   R4-2017647 Way Forward on NR-U PUCCH demodulation requirements. Ericsson

Appendix

Table 5-1: Simulation assumptions for PF0

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of symbol | 1 |
| Intra-slot frequency hopping | N/A |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 |
| Antenna configuration | 1T2R |
| Channel bandwidth | 20MHz |
| SCS | 15kHz and 30kHz |
| Number of interlaces | 1 |
| Interlace index | 0 Note1 |
| Propagation conditions | Option 1: TDLC300-100 low Option 2: TDLA30-10 |
| Test metric | FFS |
| Note 1: RBs 0, 10, 20,…,100 are allocated for 15kHz if agreed and RBs 0,5,10,…,50 are allocated for 30kHz. |  |

Table 5-2: Simulation assumptions for PF1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of symbols | 14 |
| Intra-slot frequency hopping | N/A |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Antenna configuration | 1T2R |
| Channel bandwidth | 20MHz |
| SCS | 15kHz and 30kHz |
| Number of interlaces | 1 |
| Interlace index | 0 Note1 |
| Propagation conditions | Option 1: TDLC300-100 low Option 2: TDLA30-10 |
| Test metric | FFS |
| Note 1: RBs 0, 10, 20,…,100 are allocated for 15kHz if agreed and RBs 0,5,10,…,50 are allocated for 30kHz. |  |

Table 5-3: Simulation assumptions for PF2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| Intra-slot frequency hopping | N/A |
| Number of symbols | 1 |
| The number of UCI information bits | Option 1: 4 bits or 22 bits Option 2: 22 bits |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |
| Antenna configuration | 1T2R |
| Channel bandwidth | 20MHz |
| SCS | 15kHz and 30kHz |
| Number of interlaces | 1 |
| Interlace index | 0 Note 1 |
| Propagation conditions | Option 1: TDLC300-100 low Option 2: TDLA30-10 |
| OCC-Length-r16 | Option 1: Not configured Option 2: n2 |
| Test metric | SNR @ ProbUCI block BLER=10-2 |
| Note 1: RBs 0, 10, 20,…,100 are allocated for 15kHz and RBs 0,5,10,…,50 are allocated for 30kHz. |  |

Table 5-4: Simulation assumptions for PF3

| Parameter | Test 1 |
| --- | --- |
| Modulation order | QPSK |
| Intra-slot frequency hopping | N/A |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Additional DM-RS configuration | No additional DM-RS |
| Number of symbols | Option 1: both 4 and 14 Option 2: 4 Option 3: 14 |
| The number of UCI information bits | Option 1: 16 Option 2: 16 and 4 (4 for 14 os) |
| Channel bandwidth | 20MHz |
| SCS | 15kHz and 30kHz |
| Antenna configuration | 1T2R |
| Number of interlaces | 1 |
| Interlace index | 0Note 1 |
| Propagation conditions | Option 1: TDLC300-100 low Option 2: TDLA30-10 |
| Index of OCC | n0 |
| Length of OCC | Option 1: n1 Option 2: n2 |
| Cyclic shift index for DMRS | 0 |
| Test metric | SNR @ UCI block BLER=10-2 |
| Note 1: RBs 0, 10, 20,…,90 are allocated for 15kHz and RBs 0,5,10,…,45 are allocated for 30kHz. Note 2: The UCI information does not contain CSI part 2. |  |