3GPP TSG-RAN WG4 Meeting #98-e	R4-2101354

# Electronic Meeting, Jan. 25th – Feb. 5th, 2021

Title: 	Discussions and simulation results on PSCCH requirements

Source: 	Huawei, HiSilicon

Agenda item:	7.3.6.2.2

Document for:	Discussion

Background

At last meeting, simulation assumptions and test scope for PSCCH were discussed. In this paper, we provide our simulation results and further discussions based on WF [1]

Simulation results

Simulation results for PSSCH are shown in Figure 2-1 and Table 2-1. Corresponding simulation assumptions are shown in Table 6-1.

Figure 2-1: Simulation results for PSCCH

Table 2-1 Summary of PSCCH simulation results

|  | SNR@1% BLER |  |
| --- | --- | --- |
|  | Ideal results | Impairment results |
| 26bits | 3.0dB | 4.0dB |
| 24bits | 3.0dB | 4.0dB |

Discussions

The only open issue is payload size, we propose to use 10 PRBs sub-channel size in [2]. Therefore, the bit number indicating frequency resource should be 4 and the payload size should be 26.

Proposal 1: Use payload size 26 bits for PSCCH test.

Conclusion

In this contribution, we provide our simulation results and further discussions on PSCCH test. The observations and proposals are:

Proposal 1: Use payload size 26 bits for PSCCH test.

Reference

[1]   R4-2017687 (WF on V2X single link demodulation). LG Electronics

[2]   R4-2101352 Discussions on PSSCH requirements

Appendix

Table 6-1: Simulation assumption for PSCCH demodulation performance

| Parameters | Unit | Value |
| --- | --- | --- |
| Test |  | CCH_Test1 |
| Synchronization source |  | GNSS |
| Propagation condition |  | TDLA30-1400 |
| Channel bandwidth | MHz | 20 |
| Allocated resource blocks |  | 10 |
| Subcarrier spacing | kHz | 30 |
| Timing offset |  | CP/2-12Ts |
| Frequency offset | Hz | 600 |
| The number of PSCCH symbols |  | 2 |
| Modulation order |  | QPSK |
| Payload size (without CRC) | Bits | Option 1: 26 Option 2: 24 |
| Transport block CRC | Bits | 24 |
| Binary Channel Bits | Bits |  |
| PSCCH performance metric (Note 1) |  | SNRPSCCH@1%BLER |
| Note 1: The performance metric is used for PSCCH simulation evaluation. Note 2: OCC index i (in TS 38.211) for PSCCH DMRS is randomly selected between {0, 1, 2} for each PSCCH transmission. Note 3: Two propagation conditions are listed as options for alignment purpose, one of the option will be chosen for defining requirement |  |  |