3GPP TSG-RAN WG4 Meeting #98-e	R4-2101356

# Electronic Meeting, Jan. 25th – Feb. 5th, 2021

Title: 	Discussions on PSFCH requirements

Source: 	Huawei, HiSilicon

Agenda item:	7.3.6.2.4

Document for:	Discussion

Background

At last meeting, simulation assumptions and test scope for PSFCH were discussed. In this paper, we provide our simulation results.

Simulation results

Simulation results for PSFCH are shown in Figure 2-1 and Table 2-1. Corresponding simulation assumptions derived from [1] are shown in Table 4-1

.

Figure 2-1: Simulation results for PSFCH

Table 2-1 Summary of simulation results for PSFCH

|  | SNR@ Pr(NACK miss)=1% and Pr(DTX to NACK)=1% |  |
| --- | --- | --- |
|  | Ideal results | Impairment results |
| TDLA-30-180 | 6.86 dB | 7.86dB |

Reference

[1]   R4-2017687(WF on V2X single link demodulation). LG Electronics

Appendix

Table 4-1 Simulation assumption for PSFCH demodulation performance

| Parameters | Unit | Value |
| --- | --- | --- |
| Test |  | FCH_Test1 |
| Synchronization source |  | GNSS |
| Propagation condition |  | TDLA30-180 |
| Channel bandwidth | MHz | 20 |
| Allocated resource blocks |  | 1 |
| Subcarrier spacing | kHz | 30 |
| Timing offset |  | CP/2-12Ts |
| Frequency offset | Hz | 600 |
| The number of PSFCH symbols |  | 2 |
| Number of information bits | bit | 1 |
| PSFCH period | Slot | 1 |
| Cyclic shift pair index |  | 1 |
| PSFCH performance metric (Note 1) |  | SNR@ Pr(NACK miss) =1% and [Pr(DTX to NACK)<1%] |
| Note 1: The performance metric is used for PSFCH simulation assumption. Note 2: The 1st PSFCH which is used for AGC doesn’t need to be considered in performance evaluation. |  |  |